# 3GPP TSG-RAN WG3 Meeting #122	R3-237290

# Chicago, USA, 13 - 17 November 2023

# Agenda Item:	12.2.1

# Source:	ZTE, Samsung, CMCC, China Telecom, China Unicom

# Title:	(TP to 38.300 BLCR) Stage2 description update for AI/ML RAN

# Document for:	Discussion

- Introduction

In this contribution, we further provide some updates on the current 38.300 BLCR to finalize the stage2 description for AI/ML function.

- Discussion

Regarding the stage2 description, several agreements are not reflected into the current 38.300 BLCR[1].

Reviewing the information used to support AI/ML based use cases in TR37.817, predicted/current resource status, UE performance feedback, and predicted UE trajectory can be used in all three AI/ML use cases. During WI discussion, measured UE trajectory & cell-level predicted UE trajectory are considered as the unified feature of Mobility Optimization use case.However, in the current stage2 description, it states that the measured UE trajectory is used for all use cases.

Proposal 1: Revise the stage2 description that the measured UE trajectory is used for Mobility Optimization use case.

In addition, legacy information (e.g, resource status information) can also be used for all use cases to support AI/ML function. And during the previous discussion, it was agreed that legacy information that are used to support AI/ML are transferred via existing legacy procedures.

Legacy information that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures).

Proposal 2: Add the text in TS38.300 to reflect the agreement that legacy information used to support AI/ML are transferred via existing legacy procedures.

The same principle should also be applied for CU/DU split architecture. The gNB-CU needs to collect the resource status information from the gNB-DU via legacy procedure to perform AI/ML function, and the gNB-CU-CP needs to collect the resource status information from the gNB-CU-UP via legacy procedure to perform AI/ML function. Following is the agreement related to resource status information over F1/E1 interface:

The location for resource status prediction in split architecture:

For current resource status input data from gNB DU, the resource status prediction function is located in gNB CU-CP.

For current resource status input data from gNB CU-UP, the resource status prediction function is located in gNB CU-CP.

Proposal 3: Add the text in TS38.401 to reflect the agreement that legacy information used to support AI/ML are transferred via existing legacy procedures.

With respect to the energy cost, the definition of the energy cost, as well as the agreement that the mapping rules “energy consumption - energy cost” is up to operator should be captured into the stage2 description.

EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

Proposal 4: Capture the definition of the energy cost into the stage2 description.

- Conclusion

Proposal 1: Revise the stage2 description that the measured UE trajectory is used for Mobility Optimization use case.

Proposal 2: Add the text in 38300 to reflect the agreement that legacy information used to support AI/ML are transferred via existing legacy procedures.

Proposal 3: Capture the definition of the energy cost into the stage2 description.

- Reference

- R3-235075, (BL CR to 38.300) AI/ML for NG-RAN, CMCC

- Annex - TP to 38.300 BLCR

<<<<<< Start of CHANGE >>>>>>

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

******Unchanged Text Omitted******

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

<<<<<<Next CHANGE >>>>>>

X.X	AI/ML for NG-RAN

X.X.1	General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using AI and ML techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2	Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

Signalling procedures used for the exchange of  information to support AI/ML function are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of the AI/ML function does not apply to ng-eNBs.

For the deployments of RAN intelligence, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

Resource Status Reporting Initiation & Resource Status Reporting procedures are used to retrieve resource status information to support AI/ML function.

Following information can be configured to be reported and used for all use cases:

-	Predcited Resource Status Information

-  UE performance feedback

Following information can be configured be reported and used for Mobility Optimization use case:

-  Measured UE trajectory

The reporting is configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory, as well as UE performance feedback, is triggered via the Handover Preparation procedure.

Cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Request message to provide information for e.g. subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the next one hop target NG-RAN node.

The metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced and can be exchanged with the neighbouring NG-RAN nodes upon request. EC provides a representation of the energy consumption at an NG-RAN node, which is an index normalized by mapping rules provided by OAM.

<<<<<< End of CHANGE >>>>>>