3GPP TSG-RAN WG3 Meeting #122	R3-237311

Chicago, USA, 13 - 17 November 2023

Agenda Item:	21.2

Source:	ZTE

Title:	Discussion on NR Redcap enhancement with MT-SDT

Document for:	Approval

- Introduction

In the last RAN3 #121bis meeting, RAN3 sent an LS [R3-235765] to SA2/CT4, as below.

1. Overall Description: RAN3 thanks SA2 for the LS (R3-235020) and CT4 for the LS (R3-235005) on “INACTIVE eDRX above 10.24sec and SDT”. Based on the following SA2 feedback: SA2 discussed the issue and considers it’s technically feasible for UPF/SMF to provide the DL data size of the first downlink packet to NG-RAN via AMF. SA2 also discussed if more accurate data size is needed besides the first packet, but no consensus could be reached. SA2 asks RAN3 to provide view regarding if indicating the first downlink packet data size by 5GC is sufficient for NG-RAN to decide the use of MT-SDT.. RAN3 answer: - RAN3 discussed the solution recommended by SA2 and the input from CT4 and prefers the possibility of AMF providing the most recent buffered data size per QFI at the end of the long eDRX cycle in the RAN PAGING REQUEST message, which can provide accurate information to NG-RAN for applying MT-SDT during paging decision. In case the provision of such information is deemed too complex from CT4 and SA2 point of views, RAN3 will consider the possibility of less optimized alternatives at next meeting. 2. Actions: To SA2, CT4 ACTION: RAN3 kindly asks SA2 and CT4 to provide feedback on the possibility of AMF providing the most recent buffered data size per QFI at the end of the long eDRX cycle in the RAN PAGING REQUEST message.

For the above LS, SA2 has agreed to send a reply LS [R3-237151/ S2-2311359] to CT4/RAN3, as below.

1. Overall Description: SA2 thanks RAN3 for their reply LS (R3-235765/ S2-2311527) on “INACTIVE eDRX above 10.24sec and SDT”. Based on the following RAN3 feedback: - RAN3 discussed the solution recommended by SA2 and the input from CT4 and prefers the possibility of AMF providing the most recent buffered data size per QFI at the end of the long eDRX cycle in the RAN PAGING REQUEST message, which can provide accurate information to NG-RAN for applying MT-SDT during paging decision. In case the provision of such information is deemed too complex from CT4 and SA2 point of views, RAN3 will consider the possibility of less optimized alternatives at next meeting.. SA2 answer: SA2 discussed potential solutions and would like to provide the following feedback: - SA2 has concluded that providing accurate information to NG-RAN regarding the most recent cumulative data size of data buffered per QFI in the UPF has some impact on the N4 interface. Namely, the N4 procedures today (as SA2 understands them) allow the UPF to send the Downlink Data Notification only upon arrival of the first packet, which is insufficient for providing accurate information on the cumulative data size of data buffered per QFI at the UPF at the end of the long eDRX cycle. SA2 would like to kindly ask CT4 to indicate whether SA2’s understanding of the N4 procedures today is correct and to provide feedback on the feasibility of N4 procedure enhancements that would allow for more accurate provision of the cumulative data size of the data buffered per QFI at the UPF, while taking into account the cost of such enhancements in term of specification impact and the incurred additional N4 signalling load. 2. Actions: To CT4 ACTION: SA2 kindly asks to indicate whether SA2’s understanding of the N4 procedures today is correct and to provide feedback on the feasibility of N4 procedure enhancement that would allow for more accurate provision of the cumulative data size of the data buffered per QFI at the UPF, while taking into account the cost of such enhancements in term of specification impact and the incurred additional N4 signalling load. To RAN3 ACTION: SA2 kindly asks RAN3 the take the information provided above into account.

In this paper, we will address the issue on INACTIVE eDRX above 10.24sec and SDT.

- Discussion

  - Provide the cumulative data size of the data buffered per QFI from 5GC

In the last RAN3 #121bis, RAN3 has discussed four options on the table.

Providing list of data size per QFI - Opt1: Do nothing - Opt2: - Opt3: the list of data size per QFI which arrived at the time of eDRX timer expired - Opt4: total data size of packets which arrived at the time of eDRX timer expired

Currently, SA2/CT4 agreed to provide option 2 (i.e., the data size of the first packet of the highest PPI QFI). However, RAN3 thinks option 2 is not sufficient and prefers option 3, then sent an LS [R3-235765] as above to SA2/CT4.

Based on this LS, SA2 asked CT4 to evaluate whether option 3 is feasible, seen in [R3-237151/ S2-2311359].

Observation 1: RAN3 prefers to have the most recent buffered data size per QFI, but it will introduce additional N4 signalling load.

However, from RAN’s point of view, in the following cases, it is useless to provide DL data size to RAN.

- RAN has not configured SDT bearers.

In other word, if RAN has not configured SDT, RAN will not trigger MT-SDT paging, it does not need to receive DL data size.

- RAN has received DL data packet for non-SDT QFI.

RAN has received NGAP: RAN paging request message including DL data/signalling for one or more QFIs, but if at least QFI is non-SDT, RAN will trigger MT-SDT paging, it also does not need to receive DL data size.

Observation 2: It is useless to provide DL data size to RAN, in case of either SDT bearer not configured or non-SDT’s DL data arrival.

Observation 3: It is benefit to provide the MT-SDT assistant information to 5GC, to reduce additional N4 signalling load.

  - Provide MT-SDT assistant message to 5GC

In the R18 MT-SDT WID, RAN3 also made the following progress.

Figure 1: MT-SDT procedure initiation

Refer to R3-233555 “(BLCR to 38.401) Introduction on MT-SDT”, in step 1, the CU-CP shall notify of MT-SDT bearers to the CU-UP, then upon reception of DL data, the CU-UP shall, in step 3, send the sum of MT-SDT data via DL data Notification message.

Observation 4: In E1 interface, the gNB-CU-UP has acquired MT-SDT bearer information then it will calculate the data size for MT-SDT bearer only, which can decrease its work load.

For the similar reason, RAN shall provide MT-assistant information to 5GC, then in some cases (e.g., SDT bearer not configured, non-SDT’s DL data arrival), 5GC will not provide DL data size to RAN.

Figure 3: CN based MT Communication handling with SDT procedure

At step 1, upon reception of NG: MT communication handling request message, the 5GC initiates CN based MT communication handling procedure, when UE reachable, the 5GC shall, in step 3, sends NG DL data Notification message including PPI, ARP, PDU session ID and QFI, then the RAN can trigger Uu Paging.

If SA2 agrees to send Data Size to RAN, then RAN can decide to trigger MT-SDT paging. However, currently, the 5GC has no idea of SDT information.

Observation 5: In NG interface, the 5GC has not acquired MT-SDT bearer information then will calculate the data size for all kind of bearers, which will increase its work load.

Compared figure 2 to figure 3, the procedures and flows are very similar, although the former is in E1 interface and the latter is in NG interface.

Proposal 1: Introduce MT-SDT assistant information by NGAP MT Communication Handing request message.

Figure 2: CN based MT Communication handling with MT-SDT assistant information

- Conclusion

Based on the above analysis, we have the following proposals.

Observation 1: RAN3 prefers to have the most recent buffered data size per QFI, but it will introduce additional N4 signalling load.

Observation 2: It is useless to provide DL data size to RAN, in case of either SDT bearer not configured or non-SDT’s DL data arrival.

Observation 3: It is benefit to provide the MT-SDT assistant information to 5GC, to reduce additional N4 signalling load.

Observation 4: In E1 interface, the gNB-CU-UP has acquired MT-SDT bearer information then it will calculate the data size for MT-SDT bearer only, which can decrease its work load.

Observation 5: In NG interface, the 5GC has not acquired MT-SDT bearer information then will calculate the data size for all kind of bearers, which will increase its work load.

Proposal 1: Introduce MT-SDT assistant information by NGAP MT Communication Handing request message.

Proposal 2: Agree to send LS to SA2/CT4.

- Draft LS to SA2/CT4

1. Overall Description:

RAN3 thanks SA2 for the LS (R3-237151/ S2-2311359) on “INACTIVE eDRX above 10.24sec and SDT”.

RAN3 prefers the possibility of AMF providing the most recent buffered data size per QFI at the end of the long eDRX cycle in the RAN PAGING REQUEST message, which can provide accurate information to NG-RAN for applying MT-SDT during paging decision.

RAN3 understands that it will introduce the cost of such enhancements in term of specification impact and the incurred additional N4 signalling load.

In order to reduce such N4 signalling load, RAN3 will provide the SDT type QoS flow indicator list within MT-SDT assistant information to AMF by NGAP: RAN Paging Request message.

RAN3 wishes to receive DL data size, in case that RAN sends MT-SDT assistant information to 5GC and the arrival of DL data is for pure SDT bearer.

2. Actions:

To SA2, CT4

ACTION: 	RAN3 kindly asks SA2 and CT4 to take the above information into account.