3GPP TSG-RAN WG3 Meeting #122	R3-237321

# Chicago, USA, 13-17 November 2023

Agenda Item:	16.4

Source:	Ericsson

Title:	(TPs for SL Relay to TS 38.401 and TS 38.473) Multi-path for Sidelink Relay

Document for:	Discussion, Decision

# 1	Introduction

In RAN3#121 meeting, the following progress has been made for the support of Multipath for Sidelink Relay.

RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay in Rel-18.

For the intra-DU case, the gNB-DU should take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE.

For the direct path release in intra-DU and inter-DU case, the legacy PCell change procedure from the direct path to the indirect path can be reused.

For the indirect path release,

- In intra-DU case, since both paths are connected to same gNB-DU, the UE CONTEXT MODIFCATION REQUEST message can be used to release the indirect path related configuration;

- In inter-DU case, the gNB-CU initiates the UE Context Release procedure towards the gNB-DU associated with the indirect path.

The DL RRC message is transmitted via the primary path. FFS on TP.

The indirect path addition procedure and signalling for Scenario 1 can be reused for Scenario 2 with the following difference:

- The ID of the relay UE should be included into the F1AP UE CONTEXT MODIFICATION REQUEST message for indirect path addition; FFS on which ID of relay UE is used in F1AP.

- gNB-CU does not indicate gNB-DU to provide the PC5 Relay RLC channel configuration;

- The PC5 connection establishment can be skipped.

FFS on whether to inform the gNB-DU of whether the SRB to be setup/modified is of multi-path split SRB type.

Open issues from last meeting

In this contribution, we would provide further analysis.

# 2	Discussion

RAN3 has reached a consensus to use the existing IEs to accommodate Scenario 2. However, an open points pertains to the determination of the target Relay UE ID in the Path Addition Information IE that should be checked.

#### 9.3.1.x1	Path Addition Information

This IE provides information for path addition.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Path Addition Information | M |  |  |  |
| >Indirect Path Addition |  |  |  |  |
| >>Target Relay UE ID | M |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the targetRelayUE-Identity contained in the SL-PathSwitchConfig IE, defined in TS 38.331 [8] |
| >>Remote UE Local ID | M |  | 9.3.1.267 |  |
| >>T420-like (FFS on the name) | M |  | FFS | FFS |
| >Direct Path Addition |  |  | NULL |  |
| >N3C Indirect Path Addition |  |  |  |  |
| >>Target Relay UE ID | M |  | FFS | FFS |

In line with RAN2's consensus, C-RNTI is used to indicate the target Relay UE ID in case of N3C. The IE design is given in the running RRC CR as follows.

N3C-IndirectPathAddChange

The IE N3C-IndirectPathAddChange indicates the N3C indirect path related configuration used by N3C remote UE.

N3C-IndirectPathAddChange information element

-- ASN1START

-- TAG-N3C-INDIRECTPATHADDCHANGE-START

N3C-IndirectPathAddChange-r18 ::=  SEQUENCE {

n3c-RelayIdentification-r18          SEQUENCE {

n3c-CellGlobalId-r18                 SEQUENCE {

n3c-PLMN-Id-18                        PLMN-Identity,

n3c-CellIdentity-r18                  CellIdentity

},

n3c-C-RNTI-r18                       RNTI-Value

}                                                OPTIONAL, -- Cond N3CIndirectPathAddChange

...

}

-- TAG-N3C-INDIRECTPATHADDCHANGE-STOP

-- ASN1STOP

N3C-IndirectPathAddChange field descriptions

n3c-RelayIdentification Indicates the NCGI and C-RNTI of N3C relay UE.

| Conditional Presence | Explanation |
| --- | --- |
| N3CIndirectPathAddChange | The field is mandatory present in case of N3C indirect path addition/change. Otherwise, it is absent. |

- To support Scenario 2, the target Relay UE ID points to C-RNTI.

Furthermore, to address the remaining issues in the signaling flows, the TPs should be updated to include the agreed-upon changes. For instance, it's important to note that the DL RRC MESSAGE TRANSFER message is always sent on the primary path. While this doesn't need to be documented for intra-DU case in the current BL CR to TS 38.401, it does require an update for inter-DU scenario.

- Agree the attached TPs to TS 38.401 and TS 38.473.

# 3	Conclusion

In this paper we propose the followings.

Proposal 1	To support Scenario 2, the target Relay UE ID points to C-RNTI.

Proposal 2	Agree the attached TPs to TS 38.401 and TS 38.473.

# 4	Reference

R3-235772, Summary of Offline Discussion on SL relay enhancement, LG (moderator)

# 5	Annex

- TP to TS 38.401:

### 8.xx.2	Inter-DU indirect path addition on top of direct path

The signalling flow for inter-DU indirect path addition is shown in Figure 8.xx.2-1.

Figure 8.xx.2-1 Signalling procedure of inter-DU indirect path addition on top of direct path

- If the remote UE is connected with the relay UE using sidelink, the Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

In case that the remote UE is connected with the relay UE using non-3GPP link, the remote UE reports at least C-RNTI of the relay UE and the cell ID of the serving relay UE.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2). Mode 1 resource configuration cannot be configured for remote UE in inter-gNB-DU multi-path relay in this release.

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message on the direct path for remote UE by including the RRCReconfiguration message to gNB-DU1. If the remote UE using the sidelink has no PC5 connection with the target relay UE, the contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s). If the remote UE is using non-3GPP link, the contents in the RRCReconfiguration message may include at least indirect path addition configuration, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	If the remote UE using the sidelink has no PC5 connection with the target relay UE, the remote UE establishes PC5 connection with relay UE. If the remote UE is using non-3GPP link, this step is skipped.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU1. In case duplicate SRB1 is configured, the remote UE sends the RRCReconfigurationComplete message to the gNB via both direct path and indirect path. Otherwise, the remote UE sends the RRCReconfigurationComplete message to the gNB via direct path.

Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

- TP to TS 38.473:

#### 9.3.1.x1	Path Addition Information

This IE provides information for path addition.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Path Addition Information | M |  |  |  |
| >Indirect Path Addition |  |  |  |  |
| >>Target Relay UE ID | M |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the targetRelayUE-Identity contained in the SL-PathSwitchConfig IE, defined in TS 38.331 [8] |
| >>Remote UE Local ID | M |  | 9.3.1.267 |  |
| >>T420-like (FFS on the name) | M |  | FFS | FFS |
| >Direct Path Addition |  |  | NULL |  |
| >N3C Indirect Path Addition |  |  |  |  |
| >>Target Relay UE ID | M |  | C-RNTI 9.3.1.32 |  |

<<<<<< NEXT CHANGE >>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

******* skip unchanged *******

#### -- N

N3CIndirectPathAddition::= SEQUENCE {

targetRelayUEID			C-RNTI,

iE-Extensions			ProtocolExtensionContainer { { N3CIndirectPathAddition-ExtIEs } }		OPTIONAL,

...

}

N3CIndirectPathAddition-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NA-Resource-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofHSNASlots)) OF NA-Resource-Configuration-Item

NA-Resource-Configuration-Item ::= SEQUENCE {

nADownlink					NADownlink 	    OPTIONAL,

nAUplink					NAUplink 	    OPTIONAL,

nAFlexible					NAFlexible 	    OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { NA-Resource-Configuration-Item-ExtIEs} } OPTIONAL

}

NA-Resource-Configuration-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NADownlink ::= ENUMERATED { true, false, ...}

NAFlexible ::= ENUMERATED { true, false, ...}

NAUplink ::= ENUMERATED { true, false, ...}