3GPP TSG-RAN WG3 Meeting #121bis	R3-237367

Xiamen, China, 9th – 13th October, 2023

Agenda Item:	23.2.3

Source:	Huawei

Title:	(TP BL 38.xxx) Discussion on CPP, Bandwidth Aggregation and Redcap Positioning

Document for:	Approval

# 1	Introduction

This paper discusses remaining issues on Carrier Phase Positioning, Bandwidth aggregation and Redcap Positioning.

# 2	Discussion

## 2.1 Carrier Phase Positioning

To support UL CPP for Rel-18, UL RSCP may be reported together with gNB Rx-Tx time difference measurement or UL-RTOA measurement. According to L1 parameter list, the phase quality can also be reported, which includes phase quality index and phase quality resolution:

| Carrier Phase quality information reported by gNB to LMF. Includes: - phase quality index - phase quality resolution | Includes: - phase quality index - phase quality resolution Value Ranges up to RAN4. |  |  |
| --- | --- | --- | --- |

Measurement Quality IE is designed to report the quality of the measurement, and therefore can be enhanced to support the reporting of phase quality.

RAN2 sent LSs [1-2] to RAN1 consulting on CPP related issues. RAN3 can wait for the reply LS to address other related issues.

Proposal 1: Enhance Measurement Quality IE to support the gNB to report the associated phase quality of carrier phase measurements. Wait for RAN1 LS on other issues for CPP.

## 2.2 Bandwidth Aggregation

### 2.2.1 PRS Bandwidth Aggregation

The following information was provided by RAN1 parameter List [1]:

Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. • FFS whether the link is for all TRPs or per TRP basis • FFS whether the link is per PRS resource set basis or per PRS resource basis. Agreement For PRS bandwidth aggregation across PFLs, support • Option 2: Per TRP basis and per PRS resource set basis. o For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. o It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied. Agreement For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following: • A request to indicate UE which two or three PFLs to be used for performing joint measurement • A new ReportingGranularityfactor smaller than 0 which can be applicable at least when the LMF requests aggregated measurements o Support at least the values of k={-1,-2}  FFS other values e.g. -3, -4, -5, -6 o Send RAN4 an LS to confirm the feasibility

| Indication of DL PRS resource sets in the two or three DL PFLs that are linked for DL PRS BW aggregation from the NG-RAN node to the LMF | TBD Up to three NR-DL-PRS-ResourceSetID values |  | Per TRP Example: in PRS Configuration (as in 9.2.44) in PRS CONFIGURATION RESPONSE message |
| --- | --- | --- | --- |

The following agreements were made during RAN2-123bis:

Agreements: For DL PRS bandwidth aggregation across PFLs, RAN2 to consider following signalling enhancements (subject to the details of the RAN1 parameter list): − Include the joint measurement indication and the aggregated PRSs resource PRS sets IDs across PFLs in LPP RequestLocationInformation and ProvideAssistanceData messages. − Include the PFL aggregation indication and the aggregated measurement results with PRS resource sets ID in LPP ProvideLocationInformation message. Agreements: Enhance the PRS configuration assistance data provided in NR-DL-PRS-AssistanceData IE in the Provide Assistance Data message for DL-TDOA and multi-RTT positioning as follows: - indicate the DL PRS resource sets IDs from two or three different PFLs that are linked for DL PRS BW aggregation that UE needs to use for the joint measurement (FFS if multiple combinations of linked PFLs can be indicated, e.g., 2+2 and others). - extend the NR-DL-TDOA-ReportConfig IE and add a new timingReportingGranularityFactor-Ext-r18 field with values {-1, -2}. Other values FFS. - introduce a new NR-Multi-RTT-ReportConfig-Ext-r18 IE add a new timingReportingGranularityFactor-Ext-r18 field with values {-1, -2}. Other values FFS. Extend the NR-DL-TDOA-SignalMeasurementInformation IE and add a new field to indicate whether the reported RSTD/RSRP/RSRPP measurement is a joint measurement or not. Extend the NR-Multi-RTT-SignalMeasurementInformation IE and add a new field to indicate whether the reported UE Rx-Tx time difference/RSRP/RSRPP measurement is a joint measurement or not.

For PRS Aggregation, enhancement is needed to indicate the aggregation information. In RAN2 running CR [4], it is assumed that multiple combinations of linked PFLs can be indicated. For better alignment, it is proposed to align with RAN2 encoding for DL PRS and introduce a new IE in TRP Information Response message to indicate the aggregation information for PRS configuration of each TRP.

Proposal 2: Introduce a new IE in TRP Information Response message to indicate the aggregation information for PRS per TRP.

### 2.2.2 SRS Bandwidth Aggregation

The following information was included in L1 parameter list for SRS bandwidth aggregation:

Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked • SRS resources are per BWP per carrier configuration • FFS whether the link is per SRS resource set basis or per SRS resource basis. Agreement For SRS bandwidth aggregation across two or three carriers, support • Option 2: Per SRS resource set basis. o Support new signaling to indicate which SRS resource sets across carriers are linked. o It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied.

| Indication of the SRS for positioning resource sets in the two or three carriers that are linked for SRS for positioning BW aggregation from the gNB to the LMF | Indication of SRS for positioning resource sets in each of the Indicated 2 or 3 carriers. |  | In SRS Confgiuration in POSITIONING INFORMATION RESPONSE message |
| --- | --- | --- | --- |

According to RAN1, SRS Configuration IE in Positioning Information Response message should be enhanced to indicate SRS aggregation information. In current BL CR [2], aggregation ID is used to indicate the linked SRS resources, but we think an explicit resource set ID list should be more straightforward. Similar to PRS aggregation, the frequency information of the carriers should also be provided for SRS aggregation. The following agreements were made by RAN1 that the frequency information for SRS bandwidth aggregation in RRC_INACTIVE may include point A, offset to carrier, etc.

Agreement To support intra-band contiguous SRS bandwidth aggregation for UE in RRC_INACTIVE state, frequency information (e.g. point A, offset to carrier) of one or two additional carriers with respective SRS configurations should be provided to the UE, where the newly introduced carrier(s) and the carrier of the initial BWP should be intra-band contiguous carriers.

Proposal 3: The gNB indicates the linked resource sets in SRS Configuration IE in Positioning Information Response message using explicit resource set ID list with frequency information. Aggregation ID is not needed.

To support SRS bandwidth aggregation, the current bandwidth values in Requested SRS Transmission Characteristics IE should not be adequate and extension is needed.

Meanwhile, we also think the bandwidth aggregation indication, i.e. Bandwidth Aggregation Request Information IE, in Positioning Information Request message in BL CR is not needed, because the NG-RAN node can take the bandwidth information into account to decide whether perform SRS aggregation.

Proposal 4: Extend bandwidth values in Requested SRS Transmission Characteristics IE to support SRS bandwidth aggregation.

Proposal 5: Remove Bandwidth Aggregation Request Information IE in Positioning Information Request message from BL CR.

The following information is provided in [1] on the aggregated SRS measurements:

Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs Agreement For SRS bandwidth aggregation across carriers, support - Single RSRP or RSRPP is reported - FFS: the single RSRP/RSRPP is based on aggregated SRS resources across aggregated carriers - The used SRS resource IDs for the aggregated measurement are shared for RSRP/RSRPP and/or timing measurement results

| Request from LMF to NG-RAN node for the UL positioning measurement from aggregated SRS resources across multiple CCs for UL-TDOA and/or Multi-RTT. | requested |  | For each UL-RTOA and/or gNB Rx-Tx time difference measurement in TRP Measurement Quantities in Measurement Request message. Whether this indication may be common to multiple measurements is up to RAN3. |
| --- | --- | --- | --- |
| Indicates whether the reported UL-TDOA or gNB Rx-Tx time difference measurement is based on processing of SRS for positioning resources across aggregated carriers. | enabled |  | For each UL-RTOA or gNB Rx-Tx time difference measurement in TRP Measurement Result message. Whether this indication may be common to multiple measurements is up to RAN3. |
| SRS resource IDs for the aggregated measurement which are used for RSRP/RSRPP and/or timing measurement results . | SRS resource IDs. |  | For each UL-RTOA or gNB Rx-Tx time difference measurement in TRP Measurement Result message. Whether this indication may be common to multiple measurements is up to RAN3. |

According to RAN1, LMF should be able to request aggregated measurements in Measurement Request message, e.g. indication of aggregated measurements or indication of aggregated carriers for the measurements.  There is a Note in [2] that RAN3 can decide whether the indication is common to multiple measurements, which means there are two options:

Option1. Provide SRS aggregation information per requested TRP measurement quantity.

Option 2. Provide a common SRS aggregation information per Measurement Request.

From our understanding, both options are feasible from RAN1 perspective. We slightly prefer option 2 considering the signaling overhead.

Proposal 6: Provide a common SRS aggregation information per Measurement Request, which can either be an indicator or explicit resource ID list.

For measurement reporting, the SRS resource IDs for aggregated measurement should be provided with Measurement Results.

Proposal 7: Enhance TRP Measurement Result IE to indicate aggregated resource IDs for the reported measurements.

A remaining issue is the activation/deactivation of semi-persistence and aperiodic SRS, which are supported for SRS aggregation. The following agreements have been made by RAN1:

Agreement Support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. - FFS the details Agreement For SRS bandwidth aggregation between SRS in two or three carriers, the aggregated SRS resources are of the same SRS resource-Type. Agreement For SRS bandwidth aggregation across two or three carriers, support - Option 2: Per SRS resource set basis. - Support new signaling to indicate which SRS resource sets across carriers are linked. - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied. Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility. Agreement With regard to aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, support both Option 2 and Option1. - Option 2: Support to use a DCI format 0_3 or 1_3 for multi-cell PDSCH/PUSCH scheduling to trigger SRS resources for bandwidth aggregation in multiple CCs. - Option 1: Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers, as a separate UE capability. - Reuse Rel-17 DCI framework without modification. - If a single DCI indicates transmission of an aperiodic positioning SRS resource set, UE transmits aperiodic positioning SRS resource sets across all linked carriers for bandwidth aggregation.

According to the above, if SRS bandwidth aggregation is configured for the UE, the NW should be enabled to activate all or only a part of the aggregated carriers. This means when LMF sends Positioning Activation/Deactivation message, enhancements are needed so that the LMF is able to indicate the activation/deactivation of the resource set(s) across the aggregated carriers or indicate the specific SRS resources set(s) to be activated/deactivated.

Proposal 8: Enhance Positioning Activation/Deactivation messages to support LMF to flexibly activate/deactivate the aggregated carriers.

In addition, the LS [8] is received from RAN1 with the measurement report mapping tables for gNB Rx-Tx and UL-RTOA measurements, which are aligned with the values in BL CRs [6-7], therefore it is proposed to confirm it and remove Editor’s Notes.

Proposal 9: Confirm the measurement report mapping values for k=-1 and k=-2 for gNB Rx-Tx time difference and UL-RTOA.

## 2.3 Redcap

The following agreements were made during RAN1-114:

Agreement SRS for positioning with Tx hopping can be configured outside of the active UL BWP - The configuration may include SCS, CP size and bandwidth (position and size), which can use a SCS, CP size and bandwidth different from the UL active BWP Agreement For SRS Tx hopping, the configuration parameters values are: - For the hop bandwidth common to all hops - Configuration re-uses C_SRS - The values of C_SRS in legacy SRS for positioning such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively when B_SRS equal 0. - For the starting RB of the first hop in time domain: - Configuration re-uses the IE freqDomainShift - The range is {0,268} RBs - For the single overlap common to all hops for the SRS resource - The value can be 0,1,2,4 RBs - Note: This is a new IE - For the starting slot offset and starting symbol for the SRS resource with tx hopping (first hop in time) - The value range is {0,1,2…, nrof slot in periodicity} in slots for the slot offset - Note: this is for the periodic [and semi-persistent] SRS - Starting symbol: {0,1,2,…13} in symbol - Starting slot reuses the SRS-PeriodicityAndOffset IE - Starting symbol reuses the starting position startPosition in the IE resourceMapping - The starting slot offset and symbol for each of the hops following the first hop in time, - FFS: The value range is {0,1,2…, nrof slot in periodicity} in slots for the slot offset - Note: this is for the periodic [and semi-persistent] SRS - Starting symbol: {0,1,2,…13} in symbol - this is a new IE - The number of consecutive symbols in a hop common to all hops - Values are 1,2,4,8 and 12 symbols - Configuration re-uses the IE nrofsymbols in resourcemapping - The number of hops - Values are 2,3,4,5,6 - This is a new IE

In order to provide the SRS Tx hopping information, several new IEs should be introduced (as highlighted). Therefore, it is proposed to enhance SRS Configuration IE to support SRS Tx frequency hopping and the encoding structure can be aligned with current RRC running CR [6].

Meanwhile, Positioning Information Request message may need to be enhanced accordingly so that the LMF can request the gNB to configure SRS frequency hopping. The detail information can be decided when RAN1 agreements are stable.

Proposal 10: It is proposed to enhance SRS Configuration IE based on RAN1 agreements to support SRS Tx frequency hopping and enhance Positioning Information Request message so that the LMF can request for SRS frequency hopping.

The following agreements were made during RAN1-113 on reporting of measurements based on frequency hopping:

Agreement The previous agreement is updated as follows: Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One measurement where a measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together

The measure gNBs may need to support to report the frequency hopping information along with the measurements, e.g., whether is measurement is performed based on a single hop or multiple hops. Besides, the LMF should be able to request the gNB to take measurements based on UL SRS hopping. Therefore, it is proposed to enhance Measurement request/response messages to support measurements based on frequency hopping. However, further RAN1 inputs are expected.

Proposal 11: Enhance Measurement Request/Response messages to support the UL SRS measurement based on frequency hopping. Wait RAN1 for details.

A draft TP is provided in the Annex based on current BL CRs [6-7], which can be used for way forward.

# 3	Conclusion

This document proposes:

Carrier Phase Positioning:

- Proposal 1: Enhance Measurement Quality IE to support the gNB to report the associated phase quality of carrier phase measurements. Wait for RAN1 LS on other issues for CPP.

Bandwidth Aggregation:

- Proposal 2: Introduce a new IE in TRP Information Response message to indicate the aggregation information for PRS per TRP.

- Proposal 3: The gNB indicates the linked resource sets in SRS Configuration IE in Positioning Information Response message using explicit resource set ID list with frequency information. Aggregation ID is not needed.

- Proposal 4: Extend bandwidth values in Requested SRS Transmission Characteristics IE to support SRS bandwidth aggregation.

- Proposal 5: Remove Bandwidth Aggregation Request Information IE in Positioning Information Request message from BL CR.

- Proposal 6: Provide a common SRS aggregation information per Measurement Request, which can either be an indicator or explicit resource ID list.

- Proposal 7: Enhance TRP Measurement Result IE to indicate aggregated resource IDs for the reported measurements

- Proposal 8: Enhance Positioning Activation/Deactivation messages to support LMF to flexibly activate/deactivate the aggregated carriers.

- Proposal 9: Confirm the measurement report mapping values for k=-1 and k=-2 for gNB Rx-Tx time difference and UL-RTOA.

Redcap;

- Proposal 10: It is proposed to enhance SRS Configuration IE based on RAN1 agreements to support SRS Tx frequency hopping and enhance Positioning Information Request message so that the LMF can request for SRS frequency hopping.

- Proposal 11: Enhance Measurement Request/Response messages to support the UL SRS measurement based on frequency hopping. Wait RAN1 for details.

If the proposals are fine, Huawei is volunteer to provide a TP for the BL CR according to the tabular shown in this contribution.

# 4	References

- R1-2310692, Rel-18 higher layer parameter list

- R2-2311565, Reply LS on R1-2308644 for CPP

- R2-2311391, LS on request for clarifications on RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning

- R2-2311398, LPP running CR for bandwidth aggregation

- R2-2310862, Rapporteur CR for Redcap Positioning RRC Changes

- R3-235986, BL CR for NRPPa

- R3-235988, Support of NR Positioning Enhancements

- R3-237149, LS on report mapping for positioning measurements with PRS_SRS bandwidth aggregation

# Annex A. TP for 38.455 on Bandwidth Aggregation

## 8.2	Location Information Transfer Procedures

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message. If the NG-RAN decides to perform SRS aggregation based on the Bandwidth IE in Requested SRS Transmission Characteristics IE, the NG-RAN node shall, if supported, includes Linked Resource Set List IE in the SRS Configuration IE in POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If the Time Window Information for SRS IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, configure the UE to start transmitting its UL SRS transmission at the indicated time instance.

If the LPHAP Assistance Information IE within the Requested SRS Transmission Characteristics IE  is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for configuring SRS transmissions for the UE in the indicated validty area, and it shall include SRS Configuration IE the SFN initialisation time IE and the LPHA P Validity Area Cells IE in the POSITIONING INFORMATION RESPONSE message.

Editor’s note:  If the SRS Configuration could be re-use is FFS.

Editor’s note: If the recommended validity area is not modified by the gNB, whether to include LPHAP Validity Area Cells in the Response message is FFS.

Editor’s note:  the procedural text needs further refinement.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.9	Positioning Activation

#### 8.2.9.1	General

The Positioning Activation procedure is initiated by the LMF to request the NG-RAN node to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Positioning Activation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the NG-RAN node.

For semi-persistent UL SRS, the POSITIONING ACTIVATION REQUEST message includes an indication of the UL SRS resource set to be activated and may include the spatial relation for the semi-persistent UL SRS resource to be activated. For aperiodic UL SRS, if the SRS Resource Trigger IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the value of this IE into account when triggering aperiodic SRS transmission by the UE. If Activate All IE is included in POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall, if supported, activate all resource sets linked with the indicated SRS resource set ID across the aggregated carriers.  If Activated Resource Set IE is included in POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall, if supported, activate the resources set indicated by the IE.

If the Activation Time IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the indicated value as the LMF’s requested time for activation of the UE’s SRS transmission.

Following successful activation of UL SRS transmission in the UE, the NG-RAN node shall respond with a POSITIONING ACTIVATION RESPONSE message. If the POSITIONING ACTIVATION RESPONSE message includes the System Frame Number and/or the Slot Number IEs, the LMF shall consider that the respective information indicates the activation time of SRS transmission by the UE.

#### 8.2.9.3	Unsuccessful Operation

Figure 8.2.9.3-1: Positioning Activation procedure, unsuccessful operation

If the NG-RAN node is unable to activate UL SRS transmission in the UE, it shall respond with a POSITIONING ACTIVATION FAILURE message.

If the NG-RAN node is unable to trigger the aperiodic SRS transmission with the indicated SRS Resource Trigger IE, it shall respond with a POSITIONING ACTIVATION FAILURE message with an appropriate cause value.

#### 8.2.9.4	Abnormal Conditions

Void.

### 8.2.10	Positioning Deactivation

#### 8.2.10.1	General

The Positioning Deactivation procedure is initiated by the LMF to indicate to the NG-RAN node that UL SRS transmission should be deactivated in the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Positioning Deactivation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DEACTIVATION message to the NG-RAN node. This message shall include an indication of the UL SRS resource set to be deactivated or release all the related resources.

#### 8.2.10.3	Unsuccessful Operation

Not Applicable.

#### 8.2.10.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If Aggregated Measurement Requested IE is included the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the SRS based on SRS aggregation and report the corresponding aggregated measurements. If Aggregated Resources IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall measure the SRS from the indicated aggregated resources and report the corresponding aggregated measurements.

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

#### 8.5.1.3	Unsuccessful Operation

Figure 8.5.1.3.1: Measurement procedure. Unsuccessful operation.

If the NG-RAN node cannot configure any of the requested measurements for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, it shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.

#### 8.5.1.4	Abnormal Conditions

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message but the NG-RAN node is unable to provide the measurement results within the indicated time, the NG-RAN node shall, if supported, respond with a MEASUREMENT FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Location Information Transfer Procedures

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - | - |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | YES | ignore |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED(true,…) |  | - | - |
| >>SRS Resource Trigger | O |  | 9.2.35 |  | - | - |
| Activation Time | O |  | Relative Time 1900 9.2.36 | Indicates the start time when the SRS activation is requested | YES | ignore |
| CHOICE Positioning SRS Aggregation Activation | O |  |  |  | YES | ignore |
| >Activate All | M |  | ENUMERATED(true,…) |  | YES | ignore |
| >Activated Resource Set | M |  | 9.2.x |  | YES | ignore |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.20	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Abort Transmission | M |  |  |  | YES | Ignore |
| >Deactivate SRS Resource Set |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |
| >Release ALL |  |  | NULL | the NG-RAN node configures UE to stop transmitting SRS for the positioning purpose |  |  |
| CHOICE Aggregation Deactivation | O |  |  |  | YES | ignore |
| >Deactivate All | M |  | ENUMERATED(true,…) |  | YES | ignore |
| >Deactivated Resource Set | M |  | 9.2.x |  | YES | ignore |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >Timing Reporting Granularity Factor Extended | O |  | FFS | FFS |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.2.x2 |  | YES | ignore |
| CHOICE Positioning SRS Aggregation Information | O |  |  |  | YES | ignore |
| >Aggregated Measurement Requested | M |  | ENUMERATED(true,…) |  | YES | ignore |
| >Aggregated Resources | M |  | 9.2.x |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

Editor’s note: whether the IE should be extendable, cover the previous value and the coding could start by (-1...) need further check.

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 9.2	Information Element definitions

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>NR CGI | M |  | 9.2.9 |  | - |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>PRS Configuration | M |  | 9.2.44 |  | - |  |
| >>SSB Information | M |  | 9.2.54 |  | - |  |
| >>SFN Initialisation Time | M |  | Relative Time 1900 9.2.36 |  | - |  |
| >>Spatial Direction Information | M |  | 9.2.45 |  | - |  |
| >>Geographical Coordinates | M |  | 9.2.46 |  | - |  |
| >>TRP type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp…) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information | M |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association | M |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information | M |  | 9.2.82 |  | YES | reject |
| PRS Aggregation Information |  | 0..1 |  |  | YES | ignore |
| > PRS Aggregation Combination |  | 1..<maxnoPRSBWAggregation> |  |  | YES | ignore |
| >>PRS Aggregation List | M |  | 9.2.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |
| maxnoPRSBWAggregation | Maximum no of combinations of PRS resource sets across the positoning frequency layers available for PRS bandwidth aggregation of a TRP. Value FFS. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ..., 60mHz, 140mHz, 160mHz, 200mHz, 240mHz, 300mHz) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…, 300mHz, 600mHz, 850mHz, 1200mHz) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| LPHAP Assistance Information | O |  | 9.2.A1 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

Editor’s note: the detail information in the Bandwidth Aggregation Request Information is FFS.

Editor’s note: The candidate values of bandwidth is FFS.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.32	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  |
| >Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Resource Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  |
| >semi-persistent |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >aperiodic |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  |
| Linked Resource Set List | O |  | 9.2.x |  |

Editor’s note: It is FFS if the SRS Resource Set are also impacted by SRS Bandwidth aggregation.

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| >kminus1 | M |  | INTEGER (0..3940097) | TS 38.133 [16] | YES | ignore |
| >kminus2 | M |  | INTEGER (0..7880193) | TS 38.133 [16] | YES | ignore |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.2.85 |  | YES | ignore |

### 9.2.40	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| >kminus1 | M |  | INTEGER (0..3940097) | TS 38.133 [16] | YES | ignore |
| >kminus2 | M |  | INTEGER (0..7880193) | TS 38.133 [16] | YES | ignore |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP TEG Information | O |  | 9.2.80 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.41	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 | M |  | INTEGER(0..16351) |  | - |  |
| >>k1 | M |  | INTEGER(0..8176) |  | - |  |
| >>k2 | M |  | INTEGER(0..4088) |  | - |  |
| >>k3 | M |  | INTEGER(0..2044) |  | - |  |
| >>k4 | M |  | INTEGER(0..1022) |  | - |  |
| >>k5 | M |  | INTEGER(0..511) |  | - |  |
| >kminus1 | M |  | INTEGER (0..32701) |  | YES | ignore |
| >kminus2 | M |  | INTEGER (0..65401) |  | YES | ignore |
| >Path Quality | O |  | Measurement Quality 9.2.43 |  | - |  |
| >Multiple UL-AoA | O |  | 9.2.71 |  | YES | ignore |
| >Path Power | O |  | UL SRS-RSRPP 9.2.72 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.74	Extended Additional Path List

This IE contains the extended additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxNoPathExtended> |  |  |  |  |
| >CHOICE Relative Path Delay | M |  |  |  |  |  |
| >>k0 | M |  | INTEGER(0..16351) |  |  |  |
| >>k1 | M |  | INTEGER(0..8176) |  |  |  |
| >>k2 | M |  | INTEGER(0..4088) |  |  |  |
| >>k3 | M |  | INTEGER(0..2044) |  |  |  |
| >>k4 | M |  | INTEGER(0..1022) |  |  |  |
| >>k5 | M |  | INTEGER(0..511) |  |  |  |
| >>kminus1 | M |  | INTEGER (0..32701) |  | YES | ignore |
| >>kminus2 | M |  | INTEGER (0..65401) |  | YES | ignore |
| >Path Quality | O |  | Measurement Quality 9.2.43 |  |  |  |
| >Multiple UL-AoA | O |  | 9.2.71 |  |  |  |
| >Path Power | O |  | UL SRS-RSRPP 9.2.72 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxNoPathExtended | Maximum no. of additional path measurement. Value is 8. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.73	SRS Resource type

This IE contains the SRS resource type.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Reference Signal | M |  |  |  |  |  |
| >SRS |  |  |  |  |  |  |
| >>SRS Resource ID | M |  | INTEGER(0..63) |  |  |  |
| >Positioning SRS |  |  |  |  |  |  |
| >>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |  |  |
| >>Aggregated Resources | O | 1..<maxnoAggregatedCarriers> |  |  | YES | ignore |
| >>>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | YES | ignore |
| >>>Point A | O |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>Offset to Carrier | O |  | INTEGER(0..2199,…) |  | YES | ignore |
| SRS Port Index | O |  | ENUMERATED(id1000, id1001, id1002, id1003, …) | This IE may be present if the SRS Resource ID IE is present, and is ignored otherwise. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoAggregatedCarriers | Maximum no of aggregated carriers. Value is 3. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.x	SRS Aggregation

This information element is used to indicated aggreagted SRS resource sets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Aggregation List |  | 1 |  |  |
| >SRS Aggregation Item |  | 1..< maxnoAggregatedCarrier> |  |  |
| >>Resource Set ID | O |  | INTEGER(0..15) |  |
| >>Point A | O |  | INTEGER (0..3279165) |  |
| >>Offset to Carrier | O |  | INTEGER(0..2199,…) |  |

### 9.2.y	PRS Aggregation

This information element is used to indicated aggreagted PRS resource sets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PRS Aggregation List |  | 1 |  |  |
| >PRS Aggregation Item |  | 1..<maxnoaggregatedCarriers> |  |  |
| >>PRS frequency | O |  | INTEGER (0..3279165) |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |

<<<<<<<<<<<<<<<<<<<End of Change >>>>>>>>>>>>>>>>>>>>

# Annex A2. TP for 38.473 on Bandwidth Aggregation

<<<<<<<<<<<<<<<<<<<Start of Change >>>>>>>>>>>>>>>>>>>>

### 8.13.3	Positioning Measurement

#### 8.13.3.1	General

The purpose of the Positioning Measurement procedure is to allow the gNB-CU to request one or more TRPs in the gNB-DU to perform and report positioning measurements. The procedure uses non-UE-associated signalling.

#### 8.13.3.2	Successful Operation

Figure 8.13.3.2-1: Positioning Measurement procedure: successful operation

The gNB-CU initiates the procedure by sending a POSITIONING MEASUREMENT REQUEST message to the gNB-DU, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The gNB-DU node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the gNB-DU shall reply with the POSITIONING MEASUREMENT RESPONSE message including the Positioning Measurement Response List IE.

If the Positioning Report Characteristics IE is set to "OnDemand", the gNB-DU shall return the corresponding measurement results in the Positioning Measurement Result List IE in the POSITIONING MEASUREMENT RESPONSE message, and the gNB-CU shall consider that this reporting has been terminated by the gNB-DU.

If the Measurement Beam Information Request IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU node shall include the Measurement Beam Information IE in the Positioning Measurement Result IE of the POSITIONING MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the Measurement Result IE in the POSITIONING MEASUREMENT RESPONSE message, the gNB-CU may use it for further signalling. If the Measurement Quality IE includes the Zenith Quality IE, the gNB-CU may use it for further signalling.

If the System Frame Number IE and/or the Slot Number IE are included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Measurement Characteristics Request Indicator IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the POSITIONING MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU may take it into account as the number of SRS measurement time occasions for a measurement instance.

If Aggregated Measurement Requested IE is included the MEASUREMENT REQUEST message, the gNB-DU shall, if supported, measure the SRS based on SRS aggregation and report the corresponding aggregated measurements. If Aggregated Resources IE is included in the MEASUREMENT REQUEST message, the gNB-DU shall measure the SRS from the indicated aggregated resources and report the corresponding aggregated measurements.

Interaction with the Positioning Measurement Report procedure:

If the Positioning Report Characteristics IE is set to "Periodic", the gNB-DU shall initiate the corresponding measurements, and it shall reply with the POSITIONING MEASUREMENT RESPONSE message without including any measurement results in the message. The gNB-DU shall then periodically initiate the Positioning Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, return the corresponding measurement results in the POSITIONING MEASUREMENT RESPONSE message within the indicated time.

If the Positioning Report Characteristics IE is set to "Periodic" and the Positioning Measurement Amount IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, take it into account for sending the POSITIONING MEASUREMENT REPORT message.

#### 8.13.3.3	Unsuccessful Operation

Figure 8.13.3.3-1: Positioning Measurement procedure: unsuccessful operation

If the gNB-DU is unable to configure any of the requested positioning measurements for any of the TRPs in the TRP Measurement Request List IE of the POSITIONING MEASUREMENT REQUEST message, it shall respond with a POSITIONING MEASUREMENT FAILURE message.

#### 8.13.3.4	Abnormal Conditions

If the gNB-DU receives a POSITIONING MEASUREMENT REQUEST message containing an LMF Measurement ID corresponding to an ongoing positioning measurement, it shall consider the procedure as failed and initiate local error handling.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.13.9	Positioning Information Exchange

#### 8.13.9.1	General

The Positioning Information Exchange procedure is initiated by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals and to retrieve the SRS configuration from the gNB-DU. The procedure uses UE-associated signalling.

#### 8.13.9.2	Successful Operation

Figure 8.13.9.2-1: Positioning Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the gNB-DU.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message. If the SRS Positioning INACTIVE Query Indication IE is also included in the POSITIONING INFORMATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, include the SRS-PosRRC-InactiveConfig IE in the POSITIONING INFORMATION RESPONSE message. If the gNB-DU decides to perform SRS aggregation based on the Bandwidth IE in Requested SRS Transmission Characteristics IE, the gNB-DU node shall, if supported, includes Linked Resource Set List IE in the SRS Configuration IE in POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the gNB-DU shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the Time Window Information for SRS IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU shall, if supported, configure the UE to start transmitting its UL SRS transmission at the indicated time instance.

If the LPHAP Assistance Information IE within the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU shall, if supported, take this information into account for configuring SRS transmissions for the UE in the indicated validity area, and shall include the SRS-PosRRC-InactiveValidityAreaConfig IE, the SFN initialisation time IE and the LPHAP Validity Area Cells IE in the POSITIONING INFORMATION RESPONSE message.

Editor’s note:  the procedural text and IE details are FFS

Editor’s Note: If the recommended validity area is not modified by the gNB-DU, whether to include LPHAP Validity Area Cells is FFS.

Interaction with the UE Context Modification Required (gNB-DU initiated) procedure:

The UE Context Modification Required (gNB-DU initiated) procedure may be performed before the POSITIONING INFORMATION RESPONSE message.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.13.10	Positioning Activation

#### 8.13.10.1	General

The Positioning Activation procedure is initiated by the gNB-CU to request the gNB-DU to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE. The procedure uses UE-associated signalling.

#### 8.13.10.2	Successful Operation

Figure 8.13.10.2-1: Positioning Activation procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the gNB-DU.

For semi-persistent UL SRS, the POSITIONING ACTIVATION REQUEST message includes an indication of the UL SRS resource set to be activated, and may include the spatial relation for the semi-persistent UL SRS resource to be activated. For aperiodic UL SRS, if the SRS Resource Trigger IE is included in the POSITIONING ACTIVATION REQUEST message, the gNB-DU shall take the value of this IE into account when triggering aperiodic SRS transmission by the UE. If Activate All IE is included in POSITIONING ACTIVATION REQUEST message, the gNB-DU shall, if supported, activate all resource sets linked with the indicated SRS resource set ID across the aggregated carriers.  If Activated Resource Set IE is included in POSITIONING ACTIVATION REQUEST message, the gNB-DU shall, if supported, activate the resources set indicated by the IE.

If the Activation Time IE is included in the POSITIONING ACTIVATION REQUEST message, the gNB-DU shall take the indicated value as the requested time for activation of the UE’s SRS transmission.

Following successful activation of UL SRS transmission in the UE, the gNB-DU shall respond with a POSITIONING ACTIVATION RESPONSE message. If the POSITIONING ACTIVATION RESPONSE message includes the System Frame Number and/or the Slot Number IEs, the gNB-CU shall consider that the respective information indicates the activation time of SRS transmission by the UE.

#### 8.13.10.3	Unsuccessful Operation

Figure 8.13.10.3-1: Positioning Activation procedure, unsuccessful operation

If the gNB-DU is unable to activate UL SRS transmission in the UE, it shall respond with a POSITIONING ACTIVATION FAILURE message.

If the gNB-DU is unable to trigger the aperiodic SRS transmission with the indicated SRS Resource Trigger IE, it shall respond with a POSITIONING ACTIVATION FAILURE message with an appropriate cause value

#### 8.13.10.4	Abnormal Conditions

Void.

### 8.13.11	Positioning Deactivation

#### 8.13.11.1	General

The Positioning Deactivation procedure is initiated by the gNB-CU to indicate to the gNB-DU node that UL SRS transmission should be deactivated in the UE. The procedure uses UE-associated signalling.

#### 8.13.11.2	Successful Operation

Figure 8.13.11.2-1: Positioning Deactivation procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING DEACTIVATION message to the gNB-DU, including an indication of the UL SRS resources to be deactivated.

#### 8.13.11.3	Unsuccessful Operation

Not Applicable.

#### 8.13.11.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.12	Messages for Positioning Procedures

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.3.1.197 |  |  |  |
| >>Search Window Information | O |  | 9.3.1.204 |  |  |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.3.1.238 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, and 30min are not applicable. | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| >Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >>Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, Multiple UL AoA, UL SRS-RSRPP, UL-RSCP) |  |  | - |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. |  |  |
| >>Timing Reporting Granularity Factor Extended | O |  | FFS | FFS |  |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) | This IE is ignored when the Measurement characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.3.1.242 | This IE is ignored when the Positioning Report Characteristics IE is set to “Periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.3.1.254 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4,…) |  | YES | ignore |
| Positioning Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Positioning Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.3.1.x4 |  | YES | ignore |
| CHOICE Positioning SRS Aggregation Information | O |  |  |  | YES | ignore |
| >Aggregated Measurement Requested | M |  | ENUMERATED(true,…) |  | YES | ignore |
| >Aggregated Resources | M |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Positioning Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Positioning Measurement Periodicity IE is set to the value "extended". |

Editor’s note: whether the Timing Reporting Granularity Factor Extended IE should be extendable, cover the previous value and the coding could start by (-1...) need further check.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.16	POSITIONING ACTIVATION REQUEST

This message is sent by the gNB-CU to cause the gNB-DU to activate/trigger UL SRS transmission by the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.3.1.180 |  | - | - |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.3.1.181 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - | - |
| >>Spatial Relation Information per SRS Resource | O |  | 9.3.1.210 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED (true, …) |  | - | - |
| >>SRS Resource Trigger | O |  | 9.3.1.182 |  | - | - |
| Activation Time | O |  | Relative Time 1900 9.3.1.183 | Indicates the start time when the SRS activation is requested | YES | ignore |
| CHOICE Positioning SRS Aggregation Activation | O |  |  |  | YES | ignore |
| >Activate All | M |  | ENUMERATED(true,…) |  | YES | ignore |
| >Activated Resource Set | M |  | 9.3.1.x |  | YES | ignore |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.19	POSITIONING DEACTIVATION

This message is sent by the gNB-CU to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE Abort Transmission | M |  |  |  | YES | ignore |
| >SRS Resource Set ID deactivation |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.3.1.180 |  | - |  |
| >Release ALL |  |  | NULL |  |  |  |
| CHOICE Aggregation Deactivation | O |  |  |  |  |  |
| >Deactivate All | M |  | ENUMERATED(true,…) |  | YES | ignore |
| >Deactivated Resource Set | M |  | 9.3.1.x |  | YES | ignore |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.1	Radio Network Layer Related IEs

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.176	TRP Information

The TRP Information IE contains information for one TRP within a gNB-DU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.3.1.197 |  | - |  |
| TRP Information Type Response List |  | 1 |  |  | - |  |
| >TRP Information Type Response Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  | - |  |
| >>CHOICE TRP Information Type Response Item | M |  |  |  | - |  |
| >>>NR PCI |  |  |  |  |  |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>>NR CGI |  |  |  |  |  |  |
| >>>>NR CGI |  |  | 9.3.1.12 |  | - |  |
| >>>NR ARFCN |  |  |  |  |  |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>PRS Configuration |  |  |  |  |  |  |
| >>>>PRS Configuration | M |  | 9.3.1.177 |  | - |  |
| >>>SSB Information |  |  |  |  |  |  |
| >>>>SSB Information | M |  | 9.3.1.202 |  | - |  |
| >>>SFN Initialisation Time |  |  |  |  |  |  |
| >>>>SFN Initialisation Time | M |  | Relative Time 1900 9.3.1.183 |  | - |  |
| >>>Spatial Direction Information |  |  |  |  |  |  |
| >>>>Spatial Direction Information | M |  | 9.3.1.179 |  | - |  |
| >>>Geographical Coordinates |  |  |  |  |  |  |
| >>>>Geographical Coordinates | M |  | 9.3.1.184 |  | - |  |
| >>>TRP Type |  |  |  |  |  |  |
| >>>>TRP Type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp…) | TS 38.305 [42] | YES | reject |
| >>>On-demand PRS TRP Information |  |  |  |  |  |  |
| >>>>On-demand PRS TRP Information | M |  | 9.3.1.240 |  | YES | reject |
| >>>TRP Tx TEG Association |  |  |  |  |  |  |
| >>>>TRP Tx TEG Association | M |  | 9.3.1.252 |  | YES | reject |
| >>>TRP Beam Antenna |  |  |  |  |  |  |
| >>>>TRP Beam Antenna Information | M |  | 9.3.1.256 |  | YES | reject |
| PRS Aggregation Information |  | 0..1 |  |  | YES | ignore |
| > PRS Aggregation Combination |  | 1..<maxnoPRSBWAggregation> |  |  | YES | ignore |
| >>PRS Aggregation List | M |  | 9.3.1.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.168	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 |  |  |  |  |  |  |
| >>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] | - |  |
| >k1 |  |  |  |  |  |  |
| >>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] | - |  |
| >k2 |  |  |  |  |  |  |
| >>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] | - |  |
| >k3 |  |  |  |  |  |  |
| >>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] | - |  |
| >k4 |  |  |  |  |  |  |
| >>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] | - |  |
| >k5 |  |  |  |  |  |  |
| >>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] | - |  |
| >kminus1 |  |  |  |  |  |  |
| >>kminus1 | M |  | INTEGER (0.. 3940097) | TS 38.133 [38] | YES | ignore |
| >kminus2 |  |  |  |  |  |  |
| >>kminus2 | M |  | INTEGER (0.. 7880193) | TS 38.133 [38] | YES | ignore |
| Additional Path List | O |  | 9.3.1.169 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.3.1.248 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.3.1.280 |  | YES | ignore |

#### 9.3.1.169	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnoofPath> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 |  |  |  |  |  |  |
| >>>k0 | M |  | INTEGER(0..16351) |  | - |  |
| >>k1 |  |  |  |  |  |  |
| >>>k1 | M |  | INTEGER(0..8176) |  | - |  |
| >>k2 |  |  |  |  |  |  |
| >>>k2 | M |  | INTEGER(0..4088) |  | - |  |
| >>k3 |  |  |  |  |  |  |
| >>>k3 | M |  | INTEGER(0..2044) |  | - |  |
| >>k4 |  |  |  |  |  |  |
| >>>k4 | M |  | INTEGER(0..1022) |  | - |  |
| >>k5 |  |  |  |  |  |  |
| >>>k5 | M |  | INTEGER(0..511) |  | - |  |
| >kminus1 |  |  |  |  |  |  |
| >>kminus1 | M |  | INTEGER (0.. 32701) | TS 38.133 [38] | YES | ignore |
| >kminus2 |  |  |  |  |  |  |
| >>kminus2 | M |  | INTEGER (0.. 65401) | TS 38.133 [38] | YES | ignore |
| >Path Quality | O |  | TRP Measurement Quality 9.3.1.172 |  | - |  |
| >Multiple UL AoA | O |  | 9.3.1.245 |  | YES | ignore |
| >Path Power | O |  | UL SRS-RSRPP 9.3.1.246 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPath | Maximum no. of additional path measurements. Value is 2. |

#### 9.3.1.170	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 |  |  |  |  |  |  |
| >>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] | - |  |
| >k1 |  |  |  |  |  |  |
| >>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] | - |  |
| >k2 |  |  |  |  |  |  |
| >>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] | - |  |
| >k3 |  |  |  |  |  |  |
| >>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] | - |  |
| >k4 |  |  |  |  |  |  |
| >>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] | - |  |
| >k5 |  |  |  |  |  |  |
| >>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] | - |  |
| >kminus1 |  |  |  |  |  |  |
| >>kminus1 | M |  | INTEGER (0.. 3940097) | TS 38.133 [38] | YES | ignore |
| >kminus2 |  |  |  |  |  |  |
| >>kminus2 | M |  | INTEGER (0.. 7880193) | TS 38.133 [38] | YES | ignore |
| Additional Path List | O |  | 9.3.1.169 |  | - |  |
| Extended Additional Path List | O |  | 9.3.1.248 |  | YES | ignore |
| TRP TEG Information | O |  | 9.3.1.253 |  | YES | ignore |

#### 9.3.1.171	Time Stamp

This information element contains the time stamp associated with the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  | - |  |
| CHOICE Slot Index | M |  |  |  | - |  |
| >SCS-15 |  |  |  |  |  |  |
| >>SCS-15 | M |  | INTEGER(0..9) |  | - |  |
| >SCS-30 |  |  |  |  |  |  |
| >>SCS-30 | M |  | INTEGER(0..19) |  | - |  |
| >SCS-60 |  |  |  |  |  |  |
| >>SCS-60 | M |  | INTEGER(0..39) |  | - |  |
| >SCS-120 |  |  |  |  |  |  |
| >>SCS-120 | M |  | INTEGER(0..79) |  | - |  |
| >SCS-480 |  |  |  |  |  |  |
| >>SCS-480 | M |  | INTEGER(0..319) |  | YES | reject |
| >SCS-960 |  |  |  |  | - |  |
| >>SCS-960 | M |  | INTEGER(0..639) |  | YES | reject |
| Measurement Time | O |  | Relative Time 1900 9.3.1.183 |  | - |  |
| Symbol Index [FFS] | O |  | INTEGER(0..13) |  | YES | FFS |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.175	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE for positioning purposes.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth SRS | M |  |  |  | - |  |
| >FR1 |  |  |  |  | - |  |
| >>FR1 Bandwidth | M |  | ENUMERATED (5, 10, 20, 40, 50, 80, 100, ...) |  | - |  |
| >FR2 |  |  |  |  | - |  |
| >>FR2 Bandwidth | M |  | ENUMERATED (50, 100, 200, 400,…,800,1600, 2000) |  | - |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.3.1.181 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.3.1.201 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.3.1.210 |  | YES | ignore |
| SSB Information | O |  | 9.3.1.202 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| LPHAP Assistance Information | O |  | 9.3.1.x6 |  | YES | ignore |

Editor Notes: the detail information in the Bandwidth Aggregation Request Information is FFS.

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.196	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  |
| >Positioning SRS Resource ID | M |  | INTEGER (0..63, ...) |  |
| CHOICE Resource Type | M |  |  |  |
| >Periodic |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  |
| >Semi-persistent |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >Aperiodic |  |  |  |  |
| >>SRS Resource Trigger List | M |  | INTEGER(1..3) |  |
| Linked Resource Set List | O |  | 9.3.1.x |  |

Editor Notes: It is FFS if the SRS Resource Set are also impacted by SRS Bandwidth aggregation.

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.248	Extended Additional Path List

This IE contains the extended additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..< maxNoPathExtended> |  |  |
| >CHOICE Relative Path Delay | M |  |  |  |
| >>k0 |  |  |  |  |
| >>>k0 | M |  | INTEGER(0..16351) |  |
| >>k1 |  |  |  |  |
| >>>k1 | M |  | INTEGER(0..8176) |  |
| >>k2 |  |  |  |  |
| >>>k2 | M |  | INTEGER(0..4088) |  |
| >>k3 |  |  |  |  |
| >>>k3 | M |  | INTEGER(0..2044) |  |
| >>k4 |  |  |  |  |
| >>>k4 | M |  | INTEGER(0..1022) |  |
| >>k5 |  |  |  |  |
| >>>k5 | M |  | INTEGER(0..511) |  |
| >kminus1 |  |  |  |  |
| >>kminus1 | M |  | INTEGER (0.. 32701) | TS 38.133 [38] |
| >kminus2 |  |  |  |  |
| >>kminus2 | M |  | INTEGER (0.. 65401) | TS 38.133 [38] |
| >Path Quality | O |  | TRP Measurement Quality 9.3.1.172 |  |
| >Path Power | O |  | UL SRS-RSRPP 9.3.1.246 |  |
| >Multiple UL AoA | O |  | 9.3.1.245 |  |

| Range bound | Explanation |
| --- | --- |
| maxNoPathExtended | Maximum no. of additional path measurement. Value is 8. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.247	SRS Resource type

This IE contains the SRS resource type.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Reference Signal | M |  |  |  |  |  |
| >SRS |  |  |  |  |  |  |
| >>SRS Resource ID | M |  | INTEGER(0..63) |  |  |  |
| >Positioning SRS |  |  |  |  |  |  |
| >>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |  |  |
| >>Aggregated Resources | O | 1..<maxnoAggregatedCarriers> |  |  | YES | ignore |
| >>>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | YES | ignore |
| >>>Point A | O |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>Offset to Carrier | O |  | INTEGER(0..2199,…) |  | YES | ignore |
| SRS Port Index | O |  | ENUMERATED (id1000, id1001, id1002, id1003, …) | This IE may be present if the SRS Resource ID IE is present, and is ignored otherwise. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoAggregatedCarriers | Maximum no of aggregated carriers. Value is 3. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	SRS Aggregation

This information element is used to indicated aggreagted SRS resource sets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Aggregation List |  | 1 |  |  |
| >SRS Aggregation Item |  | 1..< maxnoAggregatedCarrier> |  |  |
| >>Resource Set ID | O |  | INTEGER(0..15) |  |
| >>Point A | O |  | INTEGER (0..3279165) |  |
| >>Offset to Carrier | O |  | INTEGER(0..2199,…) |  |

#### 9.3.1.y	PRS Aggregation

This information element is used to indicated aggreagted PRS resource sets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PRS Aggregation List |  | 1 |  |  |
| >PRS Aggregation Item |  | 1..<maxnoaggregatedCarriers> |  |  |
| >>PRS frequency | O |  | INTEGER (0..3279165) |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |

<<<<<<<<<<<<<<<<<<<End of Change >>>>>>>>>>>>>>>>>>>>

# Annex B1. TP for 38.455 on UL CPP

<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

## 9.2	Information Element definitions

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.41	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 | M |  | INTEGER(0..16351) |  | - |  |
| >>k1 | M |  | INTEGER(0..8176) |  | - |  |
| >>k2 | M |  | INTEGER(0..4088) |  | - |  |
| >>k3 | M |  | INTEGER(0..2044) |  | - |  |
| >>k4 | M |  | INTEGER(0..1022) |  | - |  |
| >>k5 | M |  | INTEGER(0..511) |  | - |  |
| >kminus1 | M |  | INTEGER (0..32701) |  | YES | ignore |
| >kminus2 | M |  | INTEGER (0..65401) |  | YES | ignore |
| >Path Quality | O |  | Measurement Quality 9.2.43 |  | - |  |
| >Multiple UL-AoA | O |  | 9.2.71 |  | YES | ignore |
| >Path Power | O |  | UL SRS-RSRPP 9.2.72 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |

Editor’s note: The values of k and the value range of the granularity factor are FFS.

Editor’s note: FFS whether the UL RSCP for additional paths should be reported.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.43	Measurement Quality

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  |  |  | Corresponds to information provided in NR-TimingQuality IE as defined in TS 37.355 [14] |
| >>Measurement Quality | M |  | INTEGER(0..31) |  |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) |  |
| >Angle Measurement Quality |  |  |  |  |
| >>Azimuth Quality | M |  | INTEGER(0..255) |  |
| >>Zenith Quality | O |  | INTEGER(0..255) |  |
| >>Resolution | M |  | ENUMERATED (0.1deg, …) |  |
| >Phase Quality |  |  |  | Corresponds to information provided in NR-PhaseQuality IE as defined in TS 37.355 [14] |
| >>Phase Quality Index | M |  | FFS |  |
| >>Phase Quality Resolution | M |  | FFS |  |

<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# Annex B2. TP for 38.473 on UL CPP

<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.169	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnoofPath> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 |  |  |  |  |  |  |
| >>>k0 | M |  | INTEGER(0..16351) |  | - |  |
| >>k1 |  |  |  |  |  |  |
| >>>k1 | M |  | INTEGER(0..8176) |  | - |  |
| >>k2 |  |  |  |  |  |  |
| >>>k2 | M |  | INTEGER(0..4088) |  | - |  |
| >>k3 |  |  |  |  |  |  |
| >>>k3 | M |  | INTEGER(0..2044) |  | - |  |
| >>k4 |  |  |  |  |  |  |
| >>>k4 | M |  | INTEGER(0..1022) |  | - |  |
| >>k5 |  |  |  |  |  |  |
| >>>k5 | M |  | INTEGER(0..511) |  | - |  |
| >kminus1 |  |  |  |  |  |  |
| >>kminus1 | M |  | INTEGER (0.. 32701) | TS 38.133 [38] | YES | ignore |
| >kminus2 |  |  |  |  |  |  |
| >>kminus2 | M |  | INTEGER (0.. 65401) | TS 38.133 [38] | YES | ignore |
| >Path Quality | O |  | TRP Measurement Quality 9.3.1.172 |  | - |  |
| >Multiple UL AoA | O |  | 9.3.1.245 |  | YES | ignore |
| >Path Power | O |  | UL SRS-RSRPP 9.3.1.246 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPath | Maximum no. of additional path measurements. Value is 2. |

Editor’s Notes: The values of k and the value range of the granularity factor are FFS.

Editor’s note: FFS whether the UL RSCP for additional paths should be reported.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.172	TRP Measurement Quality

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE TRP Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  |  |  |  |
| >>Measurement Quality | M |  | INTEGER(0..31) | TS 37.355 [39] |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) | TS 37.355 [39] |
| >Angle Measurement Quality |  |  |  |  |
| >> Azimuth Quality | M |  | INTEGER(0..255) |  |
| >> Zenith Quality | O |  | INTEGER(0..255) |  |
| >>Resolution | M |  | ENUMERATED (0.1deg, …) |  |
| >Phase Measurement Quality |  |  |  | TS 37.355 [39] |
| >>Phase Quality Index | M |  | FFS |  |
| >>Phase Quality Resolution | M |  | FFS |  |

<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# Annex C1. TP for 38.455 on Redcap Positioning

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 9.1.4	Messages for Measurement Information Transfer Procedures

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >Timing Reporting Granularity Factor Extended | O |  | FFS | FFS |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.2.x2 |  | YES | ignore |
| Frequency Hopping Requested (FFS) | O |  |  |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

Editor’s note: whether the IE should be extendable, cover the previous value and the coding could start by (-1...) need further check.

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 9.2	Information Element definitions

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ...) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request Information[FFS] | O |  | ENUMERATED(true, …) [FFS] |  | YES | ignore |
| LPHAP Assistance Information | O |  | 9.2.A1 |  | YES | ignore |
| Frequency Hopping Request Information | O |  | FFS |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

Editor’s note: the detail information in the Bandwidth Aggregation Request Information is FFS.

Editor’s note: It is FFS if the Measurement procedure needs to indicate explicit SRS aggregation ID.

Editor’s note: whether and what Frequency Hopping Request Information is needed wait further RAN1 input.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.30	Positioning SRS Resource

This information element contains the SRS resource for positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Transmission Comb | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  |
| >Comb Eight |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(n1,n2,n4, n8, n12} |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  |
| CHOICE Resource Type Positioning | M |  |  |  |
| >periodic |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >semi-persistent |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot 1, slot 2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >aperiodic |  |  |  |  |
| >>slot offset | M |  | INTEGER(0..32) |  |
| Sequence ID | M |  | INTEGER(0..65535) |  |
| CHOICE Spatial Relation Positioning | O |  |  |  |
| >SSB |  |  |  |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  |
| >>SSB index | O |  | INTEGER(0..63) |  |
| >PRS |  |  |  |  |
| >>PRS ID | M |  | INTEGER(0..255) |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>PRS Resource ID | O |  | INTEGER(0..63) |  |
| SRS Hopping Info | O |  |  |  |
| >Overlap | M |  | ENUMERATED(RB0, RB1, RB2, RB4, …) |  |
| >Number of Hops | M |  | ENUMERATED(2, 3, 4, 5, 6, …) |  |
| >Hop Start Position List | M | 1..<MaxnoHops> |  |  |
| >>CHOICE Slot Index | M |  | INTEGER(0..79) |  |
| >>Start Symbol | M |  | INTEGER(0..13) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of Hops minus one. Value is 5. |

Editor’s note: the detail information in SRS Hopping Info needs further RAN1 input.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Z-AoA | M |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA | M |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP | M |  | 9.2.72 |  | YES | reject |
| >>UL RSCP | M |  | 9.2.x3 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >CHOICE Frequency Hopping Indicator | O |  | FFS |  | YES | ignore |
| >>Single Hop | M |  |  |  | YES | reject |
| >>Multiple Hops | M |  |  |  | YES | reject |

Editor’s note: the detail information in Frequency Hopping Indicator needs further RAN1 input.

<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# Annex C2. TP for 38.473 on Redcap Positioning

<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 9.2.12	Messages for Positioning Procedures

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.3.1.197 |  |  |  |
| >>Search Window Information | O |  | 9.3.1.204 |  |  |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.3.1.238 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, and 30min are not applicable. | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| >Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >>Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, Multiple UL AoA, UL SRS-RSRPP, UL-RSCP) |  |  | - |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. |  |  |
| >>Timing Reporting Granularity Factor Extended | O |  | FFS | FFS |  |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) | This IE is ignored when the Measurement characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.3.1.242 | This IE is ignored when the Positioning Report Characteristics IE is set to “Periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.3.1.254 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4,…) |  | YES | ignore |
| Positioning Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Positioning Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.3.1.x4 |  | YES | ignore |
| Frequency Hopping Requested (FFS) | O |  |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Positioning Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Positioning Measurement Periodicity IE is set to the value "extended". |

Editor’s note: whether the Timing Reporting Granularity Factor Extended IE should be extendable, cover the previous value and the coding could start by (-1...) need further check.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.166	Positioning Measurement Result

The purpose of this information element is to provide the measurement result(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning Measured Result Item |  | 1 .. <maxnoofPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  |  |  |  |  |
| >>>UL Angle of Arrival | M |  | 9.3.1.167 |  | - |  |
| >>UL SRS-RSRP |  |  |  |  |  |  |
| >>>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA |  |  |  |  |  |  |
| >>>UL RTOA | M |  | UL RTOA Measurement 9.3.1.168 |  | - |  |
| >>gNB Rx-Tx Time Difference |  |  |  |  |  |  |
| >>>gNB Rx-Tx Time Difference | M |  | 9.3.1.170 |  | - |  |
| >>Zenith Angle of Arrival Information |  |  |  |  |  |  |
| >>>Zenith Angle of Arrival Information | M |  | 9.3.1.239 |  | YES | reject |
| >>Multiple UL AoA |  |  |  |  |  |  |
| >>>Multiple UL AoA | M |  | 9.3.1.245 |  | YES | reject |
| >>UL SRS-RSRPP |  |  |  |  |  |  |
| >>>UL SRS-RSRPP | M |  | 9.3.1.246 |  | YES | reject |
| >>UL RSCP |  |  |  |  |  |  |
| >>>UL RSCP | M |  | 9.3.1.x5 |  | YES | reject |
| >Time Stamp | M |  | 9.3.1.171 |  | - |  |
| >Measurement Quality | O |  | TRP Measurement Quality 9.3.1.172 |  | - |  |
| >Measurement Beam Information | O |  | 9.3.1.173 |  | - |  |
| >ARP ID | O |  | 9.3.1.244 |  | YES | ignore |
| >SRS Resource type | O |  | 9.3.1.247 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.3.1.249 |  | Yes | ignore |
| >CHOICE Frequency Hopping Indicator | O |  | FFS |  | YES | ignore |
| >>Single Hop | M |  |  |  | YES | reject |
| >>Multiple Hops | M |  |  |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |

Editor’s note: the detail information in Frequency Hopping Indicator needs further RAN1 input.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.175	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE for positioning purposes.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth SRS | M |  |  |  | - |  |
| >FR1 |  |  |  |  | - |  |
| >>FR1 Bandwidth | M |  | ENUMERATED (5, 10, 20, 40, 50, 80, 100, ...) |  | - |  |
| >FR2 |  |  |  |  | - |  |
| >>FR2 Bandwidth | M |  | ENUMERATED (50, 100, 200, 400,…,800,1600, 2000) |  | - |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.3.1.181 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.3.1.201 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.3.1.210 |  | YES | ignore |
| SSB Information | O |  | 9.3.1.202 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request Information (FFS) | O |  | ENUMERATED(true, …) [FFS] |  | YES | ignore |
| LPHAP Assistance Information | O |  | 9.3.1.x6 |  | YES | ignore |
| Frequency Hopping Request Information | O |  | FFS |  | YES | ignore |

Editor Notes: the detail information in the Bandwidth Aggregation Request Information is FFS.

Editor Notes: It is FFS if the Measurement procedure need to indicate explicit SRS aggregation ID

Editor’s note: whether and what Frequency Hopping Request Information is needed wait further RAN1 input.

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.194	Positioning SRS Resource

This information element contains the SRS resource for positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |
| CHOICE Transmission Comb Positioning | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  |
| >Comb Eight |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(1,2,4,8,12) |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  |
| CHOICE Resource Type Positioning | M |  |  |  |
| >Periodic |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >Semi-persistent |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot20480, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >Aperiodic |  |  |  |  |
| >>Slot offset | M |  | INTEGER(0..32) |  |
| Sequence ID | M |  | INTEGER(0..65535) |  |
| CHOICE Spatial Relation Positioning | O |  |  |  |
| >SSB |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  |
| >>SSB index | O |  | INTEGER(0..63) |  |
| >PRS |  |  |  |  |
| >>PRS ID | M |  | INTEGER(0..255) |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>PRS Resource ID | O |  | INTEGER(0..63) |  |
| SRS Hopping Info | O |  |  |  |
| >Overlap | M |  | ENUMERATED(RB0, RB1, RB2, RB4, …) |  |
| >Number of Hops | M |  | ENUMERATED(2, 3, 4, 5, 6, …) |  |
| >Hop Start Position List | M | 1..<MaxnoHops> |  |  |
| >>CHOICE Slot Index | M |  | INTEGER(0..79) |  |
| >>Start Symbol | M |  | INTEGER(0..13) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of Hops minus one. Value is 5. |

Editor’s note: the detail information in SRS Hopping Info needs further RAN1 input.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>