3GPP TSG-RAN WG3 Meeting #122	R3-237374

Chicago, USA, 13th – 17th November, 2023

Title: 	E-CID for Network UE location Verification

Source: 	Huawei

Agenda item:	18.2

Document Type:	Discussion

# 1. Introduction

This contribution is continuing the discussion on NTN IoT Network UE location Verification also based on RAN decision:

RAN#101 endorses that E-CID method could be further analyzed and enhanced in RAN3, if needed for NTN scenarios (NR-NTN and IoT-NTN).

The E-CID is a positioning method limited to RAN3 protocols; NRPPa for NR NTN and LPPa for IoT NTN, without any UE impact.

# 2. Discussion

From our understanding the Network UE location Verification, could apply to IoT NTN based on SA2 specification.

In NR NTN, RAN1 reach agreement on multi-RTT method Positioning, with possibility of resolving the ambiguity with E-CID. But we should notice also that E-CID could be used as standalone method for the Positioning without any RAN1 Impact.

In IoT-NTN, there is as far as we know, no RAN1 agreement on Multi-RTT, either no positioning method identified to for Network UE location Verification in EPS. However, The E-CID is a positioning method limited to RAN3 protocols; LPPa for IoT NTN, without any UE impact, which can provide the UE rough UE location based on Mapped Cell Id. That the reason we suggest then to enhance E-CID with the Mapped Cell Information like in [1] in LPPa protocol.

We should also noticed that even if the for NTN, the serving cell identity, provided from the MME to the E-SMLC, is a Mapped Cell ID, nothing preclude that the UE moved and the mapped cell provided during the request of MME to the E-SMLC is same as the current UE Mapped cell.

We propose then like in [1] to include the mapped cell ID Information element as request of E-CID and reporting of E-CID.

# 3. Conclusion

We propose RAN3 to discuss and agreed to include the mapped cell ID Information element as request of E-CID and reporting of E-CID as described in R3-235493 [2].

# 4. Reference

[1] R3-237454, Discussion on E-CID method for NTN, THALES, Huawei, Ericsson

[2] R3-237426, Mapped Cell ID Introduction for E-CID and IoT NTN Ericsson, Huawei, Thales