3GPP TSG-RAN WG3 Meeting #122	R3-237381

Chicago, US, November 13 – 17, 2023

Title: 	(TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Energy Saving use case

Source: 	Huawei

Agenda item:	12.2.2.3

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3#121bis meeting the discussion on how to support the inference-based Energy Saving (ES) use case was mainly focused on the issues raised by SA5 in their Reply LS to RAN3 in [1] – a reply to the LS sent by RAN3 to SA5 in [2] after RAN3#120 meeting (May 2023) – and the following agreements were reached [3][4]:

RAN3 agree that ng-eNBs (E-UTRA nodes connected to 5GC) are not in the scope of the Rel-18 AI/ML for NG-RAN WI, and provide feedback on this to SA5 in the Reply LS.

The area where the “Energy Consumption – to Energy Cost” mapping rule is applicable is up to the Operator.

Eventually, RAN3 agree the Reply LS to SA5 in [5] with further indications and clarifications on how to define the “Energy Consumption – to – Energy Cost” mapping rule.

In this paper, we discuss about the remaining open issues for the inference-based ES use case, i.e., the FFS on the Energy Cost IE semantics description in the latest BLCR in [6] as well as the interpretation of Energy Cost (EC) at the source NG-RAN node side.

# 2. Discussion

## Background

The Rel-18 AI/ML for NG-RAN Work Item aims at enabling inference-based use cases including Load Balancing (LB), Mobility Enhancements (ME) and Energy Saving (ES). For the latter use case RAN3 introduced the metric of Energy Cost (EC) in RAN3#119 (February 2023):

Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs.

Adopt the below Option-3a and exchange Energy Cost (EC) upon request over the Xn interface.

The metric of Energy Cost (EC) exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). EC is a value at gNB level.

In RAN3#119bis-e meeting (April 2023) further details on the EC metric were discussed and RAN3 achieved consensus on a Working Assumption (WA) concerning the meaning, configuration and usage of the EC as follows:

WA: If the Energy Cost is encoded as an index (0, ..Max), representing energy consumption on a linear scale, it is agreed that the OAM configures rules to a NG-RAN node to determine how to normalize the values of the EC. The rules shall be the same at least for all neighboring NG-RAN nodes within the area where a request on EC reporting is triggered by a source NG-RAN node.

It is agreed that the Energy Cost is a node level parameter. Further EC granularities are out of scope of Rel18.

RAN3 then revised the above WA in RAN3#120 meeting (May 2023) and eventually achieved the agreement as below:

EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

Finally, in RAN3#121 (August 2023) the encoding of the EC in the RAN3 specifications was agreed:

Define the Energy Cost IE as an INTEGER (0..10000,…), it can be revisited based on reply from SA5.

and the above agreements have been reflected in the latest version of the BLCR to TS 38.423 in [6] where, within the tabular of the DATA COLLECTION UPDATE message, in the Semantics description of the Energy Cost IE, it is specified that the EC is the node level measured Energy Consumption index, with value 0 (10000) corresponding to minimum (maximum) measured Energy Consumption value, and the Energy Consumption is measured on a linear scale.

#### 9.1.3.FF	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| *** skip unchanged parts *** |  |  |  |  |  |  |
| Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. Energy Consumption is measured on a linear scale. (FFS) | YES | FFS |

In the above reported tabular, however, there is still an FFS that needs to be addressed, which refers to the use of a linear scale to measure the NG-RAN node’s Energy Consumption. In our understanding, and based on the content of the Reply LS that RAN3 sent in the last meeting to SA5 in [5], such aspect is out of RAN3 scope because it is part of the definition of the “Energy Consumption – to – Energy Cost” mapping rule, which is up to the Operator.

Observation 1: It is up to the Operator to define the “Energy Consumption – to Energy Cost” mapping rule, hence it is out of RAN3 scope to decide in which manner the NG-RAN node’s Energy Consumption values are measured.

Hence, we propose to solve the FFS in the semantics description of the Energy Cost IE as follows:

#### 9.1.3.FF	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| *** skip unchanged parts *** |  |  |  |  |  |  |
| Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | YES | FFS |

Proposal 1: RAN3 to agree the TP to the BLCR for TS 38.423 in the Annex which solves the FFS in the semantics description of the Energy Cost IE in the DATA COLLECTION UPDATE message.

## Interpretation of EC at the source NG-RAN node side

In RAN3#119bis-e meeting [7] there was a discussion on how to interpret EC values, i.e., whether they represent either the absolute EC values or the relative (i.e. "delta") EC values, as follows:

Which of the following two options to be selected for inferred and measured EC definition:

1)	Inferred EC represents the node level EC value assuming that an additional load is served; Measured EC represents the actual node level EC value, e.g. after an additional load is transferred

2)	Inferred EC represents the delta increase of the EC value assuming that an additional load is served; Measured EC represents the delta increase of the EC value after an additional load is transferred

Note that the above FFS concerns both the inferred and the measured EC values, but considering the RAN3 agreement from RAN3#121 on the fact that the inferred EC is not pursued in Rel-18 we focus on the measured EC metric (see highlighted text in green above). It should be recalled that, in RAN3#119bis-e [7][8], there was a common understanding in RAN3 that the case of “Inferred EC represents the delta increase of the EC value assuming that an additional load is served; Measured EC represents the actual node level EC value” is not considered.

In the last RAN3 meeting, the contribution in [9] further discussed about the correct interpretation of the EC metric at the source NG-RAN node (NG-RAN node 1 in the BLCR to TS 38.423) and proposed two options for discussion:

- Option 1: NG-RAN node 1 may ask NG-RAN node 2 to report Energy Cost before and after an offloading action and NG-RAN node 1 will take the difference of the two to determine the corresponding Energy Cost of the offloading.

- Option 2: NG-RAN node 1 may request Energy Cost reporting from NG-RAN node 2. NG-RAN node 2 may subsequently calculate the delta increase of the Energy Cost corresponding to the offloaded traffic by NG-RAN node 1 by comparing the Energy Cost before and after the offloading action and may send the (delta) difference to NG-RAN node 1.

According to the proponents, Option 1 might be subject to “pollution” from other concurrent offloading actions, in the sense that NG-RAN node 1 will not be able to determine if the reported Energy Cost value by the neighbouring NG-RAN node 2 corresponds only to the offloading action of NG-RAN node 1 or by actions taken also by other neighbouring nodes. For Option 1 to become meaningful, the reporting node NG-RAN node 2 would also need to indicate (together with the reported Energy Cost to NG-RAN node 1) some additional information about other concurrent offloading actions that could possibly pollute the reported Energy Cost measurement, which could even further complexify the solution in case Energy Cost is reported periodically.

Hence, in their view, Option 2 is simpler since it allows NG-RAN node 2 to isolate and report only the proportion of the Energy Cost that corresponds to an offloading action of the requesting NG-RAN node: the reported value may more accurately reflect impacts of a certain offloading action to the reported Energy Cost value and requires less signalling since it is up to the target node receiving the offloaded traffic to calculate the difference.

Although we acknowledge the issue in raised in [9], we think that the absolute node-level EC reported after an inference-based offloading action being polluted by other AI/ML-based offloading actions from other neighbouring nodes is kind of a corner case. By referring to Figure 1 and to the discussion from the last meeting on the definition of the “Energy Consumption – to – Energy Cost” mapping rule, we think it is reasonable to assume that the AI/ML functionality is configured by the Operator to be activated in some areas of its network, e.g., in business areas where traffic offloading within a certain area can be performed in specific hours of the day from a single NG-RAN node – which provides additional network capacity and has AI/ML capabilities – towards a limited set of neighbour NG-RAN nodes in the coverage layer (without AI/ML capabilities) within that business area.

Figure 1 – Energy Cost reporting within adjacent areas of the Operator’s network.

In the above Figure 1, the Operator has configured two adjacent areas of its own network to be provided with distinct mapping rules (Mapping rule #1 for Area1 and Mapping rule #2 for Area2); gNB3 (gNB5) provides additional network capacity in Area1 (Area2) and, by means of its own AI/ML capabilities, it can perform inference-based ES actions (e.g., traffic offloading) towards the gNBs within Area1 (Area2) and then it can be switched-off during non-business hours. Considering that inferring a proper traffic offloading action by gNB3 (gNB5) to be then performed towards its neighbour NG-RAN nodes gNB1/gNB2/gNB4 (gNB6/gNB7/gNB8) requires a preliminary exchange of the EC values from such neighbour NG-RAN nodes gNB1/gNB2/gNB4 (gNB6/gNB7/gNB8) to assess the current (i.e., pre-offloading) status of the energy consumption of Area1 (Area2), it should be clear from Figure 1 that NG-RAN nodes in Area2 (Area1) should never report their EC values to gNB3 (gNB5) because, as per Operator’s configuration, they use Mapping rule #2 (Mapping rule #1) which is unknown to gNB3 (gNB5) and could lead to misinterpretation of the reported EC values at gNB3 (gNB5).

In addition to that, a reasonable configuration of Area1 (Area2) by the Operator should consider the availability of the Xn interface among the involved NG-RAN nodes, hence it could be foreseen that NG-RAN nodes in Area1 (Area2) have no Xn interface with gNB5 (gNB3), hence making the pollution issue even less likely to happen in real scenarios.

Observation 2: The absolute node-level EC reported after an inference-based offloading action being polluted by other inference-based offloading actions from other neighbouring nodes is a corner case that can be dealt with by proper Operator’s configuration.

Based on the above, for the sake of keeping the specification work at a reasonable complexity level, for the case the delta information needs to be determined, the source NG-RAN node could compute the delta value internally by comparing the measured absolute EC values from the (target) neighbour NG-RAN node reported before and after the ES action is performed (i.e., Option 1).

Proposal 2: The measured EC represents a mapped value corresponding to an absolute actual node-level energy consumption value, not to a delta increase of the energy consumption value after an ES action is performed.

For the sake of clarity, concerning the following FFS from RAN3#119bis-e [7]:

Whether this EC can only be reported after HO needs to be further discussed.

it should be clear enough from past RAN3 discussions that the measured EC can also be reported before a handover or, in general, before any ES action is performed. However, after an ES action has been performed, the source NG-RAN node which performed the ES action could be interested in evaluating whether such ES action provided the expected improvement (i.e., reduction) of the overall EC: to this end, the source NG-RAN node may ask neighbour NG-RAN nodes to provide the new (i.e., after the ES action) measured EC values so it can now re-assess the overall EC and determine whether such action was accurate and beneficial in terms of ES. If necessary, the source NG-RAN node could use such feedback information for re-training purposes.

Proposal 3: The current, measured EC from neighbour NG-RAN nodes can be reported before and/or after an ES action (e.g., handover).

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

Observation 1: It is up to the Operator to define the “Energy Consumption – to Energy Cost” mapping rule, hence it is out of RAN3 scope to decide in which manner the NG-RAN node’s Energy Consumption values are measured.

Observation 2: The absolute node-level EC reported after an inference-based offloading action being polluted by other inference-based offloading actions from other neighbouring nodes is a corner case that can be dealt with by proper Operator’s configuration.

Proposal 1: RAN3 to agree the TP to the BLCR for TS 38.423 in the Annex which solves the FFS in the semantics description of the Energy Cost IE in the DATA COLLECTION UPDATE message.

Proposal 2: The measured EC represents a mapped value corresponding to an absolute actual node-level energy consumption value, not to a delta increase of the energy consumption value after an ES action is performed.

Proposal 3: The current, measured EC from neighbour NG-RAN nodes can be reported before and/or after an ES action (e.g., handover).

# 4. Reference

- R3-235031, (S5-235777) Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index, Huawei

- R3-233515, LS on AI/ML for NG-RAN Energy Saving Energy Cost index, CMCC

- 3GPP RAN3#121bis Chairman’s notes (RAN3_121bis_agenda_20231013_EOM1.doc)

- R3-235835, Summary of Offline Discussion on CB: # AIRAN4_ES, Huawei

- R3-235743, Reply LS on Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index, Huawei

- R3-235955, (BLCR to 38.423) for AI/ML for NG-RAN, Ericsson, Nokia, Nokia Shanghai Bell

- 3GPP RAN3#119bis-e Chairman’s notes (RAN3_119bis-e_agenda_202304026_EOM1.doc)

- R3-232028, SoD on CB: # AIRAN4_ES, Ericsson

- R3-235613, (TP to TS 38.423) Energy Efficiency aspects related to SA5 LS and Energy Cost Description, Nokia, Nokia Shanghai Bell, Orange

# 5. Annex: TP to 38.423 (based on BLCR in R3-235955)

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.FF	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Info Result List |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | FFS |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 | – |  |
| >>UE Performance | O |  | 9.2.3.Y |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.P1 | It contains information about cells that a UE has connected to. | – |  |
| Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | YES | FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>