3GPP TSG-RAN WG3 Meeting #122	R3-237397

Chicago, U.S., 13th – 17th November 2023

Agenda Item:	15.3

Source:	Ericsson

Title:	Finalising Support for RRC_INACTIVE state

Document for:	Discussions & Approval

# 1	Introduction

This TP aims at finalising the open items on support of rRRC_INACTIVE state.

# 2	Discussion

## 2.1	Resolving the Editor’s Note in F1AP BL CR in the MULTICAST GROUP PAGING message

The F1AP BL CR currently contains the following Editor’s Note:

| Indication for Multicast RRC_INACTIVE Reception | O |  | ENUMERATED (true, ...) | Editor’s Note: This IE follows the RAN2 decision to Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE. IE name, codepoint name, and semantics are FFS. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

The latest RRC BL CR contains a new Rel-18 extension of the Paging message (as part of the pagingGroupListthe following new field description

inactiveReceptionAllowed

Indicates whether the UE with a valid PTM configuration for a TMGI in the PagingGroupList stays in RRC_INACTIVE to receive the corresponding MBS multicast session.

it is proposed to align IE name and semantics accordingly and to remove the Editor’s Note.

Proposal 1:	It is proposed to rename in the F1AP BL CR in the MULTICAST GROUP PAGING message the new IE to “InactiveReceptionAllowed”, and to replace the Editor’s Note by the semantic description “Corresponds to information contained the inactiveReceptionAllowed as specified in TS 38.331 [8].”

## 2.2	Control of the content of the MBSMulticastConfiguration message

Last meeting we started to include further details regarding the MBSMulticastConfiguration message, which is sent on MCCH (see RRC BL CR).

The current status of the F1AP BL CR is that common parts are FFS and session-specific parts (in MBS-SessionInfoListMulticast).

In order to configure common parts two approaches were proposed:

-	incorporate the respective parts into the Multicast Context procedures (although the common parts are related to all sessions)

-	incorporate the respective parts into a new procedure.

For the sake of progress, we are fine with a new procedure and propose the following:

Proposal 2:	Define a new procedure to configure common parts (related to all sessions) of the MCCH: Multicast Common Configuration, a class 1 procedure.

Proposal 3:	The Multicast Common Configuration contains per cell MCCH configuration parameters: The mbs-NeighbourCellList-r18 and the thresholdMBS-List-r18.

Further, it is important to allow the gNB-CU to control sending the MBSMulticastConfiguration message on MCCH, i.e. to be able to turn it on and off. This is one of the aspects we have discusse previously and it reflects well the overall concept in Rel-18.

Proposal 4:	F1AP shall enable the gNB-CU to control the broadcast of the MBSMulticastConfiguration message on MCCH, i.e. to be able to turn it on and off.

## 2.3	On RAN2 plans to define a “PDCP COUNT synchronized” indication to UEs for neighbouring cells

Latest RAN2 stage-2 running CR contains the following statements:

The gNB may provide an indication on cell PDCP COUNT synchronization for an MBS session with PTM configuration in RRCRelease message, once indicated by the gNB, all cells within the RNA are synchronized for PDCP COUNT MRBs of the corresponding MBS service. Upon reselection to a cell indicated as synchronized in terms of PDCP COUNT value, the UE doesn’t initialize the PDCP state variables. Otherwise, the UE initializes the PDCP state variables as defined in TS 38.323 [8].

Editor’s Note:	FFS how the UE is indicated about cells being synchronized (i.e. what information the network needs to provide to the UE).

We have so far assumed that within a rather large area, it is ensured that the cells are synchronized in terms PDCP COUNT. Given the additions in the RAN2 stage-2 running CR we assume only a small/local area will be configured to enable UEs’ reception of multicast data in RRC_INACTIVE, we do not believe that any particular protocol support would be needed, for sure not among cells served by the same gNB. Inter-gNB scenarios will probably not change the size of “synchronisation areas” and can be regarded as static information.

Observation 5:	W.r.t. RAN2 plans to define a “PDCP COUNT synchronized” indication to UEs it is not expected that any specific RAN3 protocol work is necessary.

## 2.4	Further open items in F1AP

### #1 FFSs in the Multicast CU to DU RRC Information (9.3.1.x1)

We have checked the references to RRC and have sufficiently discussed the structure and believe that the overall FFS can be removed.

Also the FFS on the presence of the Multicast DU to CU Cell List can be replaced by “0..1” to denote an optional IE.

Proposal 6:	F1AP BL CR: Remove the Editor’s Note in 9.3.1.x1 and replace the FFS on the Range of the MBS Multicast Cell List by “0..1”..

### #2 Editor’s Note on FFSs in the Multicast CU to DU RRC Information (9.3.1.x11)

There is an EN on whether the MBS Multifast Configuration Notification IE, which is sent in the Multicast DU to CU RRC Information IE within the MULTICAST CONTEXT NOTIFICATION INDICATION message if the MBSMulticastConfiguration message has changed its contents. (Editor’s Note: whether both Multicast Configuration related IEs could be also encoded in a CHOICE structure could be considered in future discussions.)

The “choice” structure which is applied if the content of the MBSMulticastConfiguration message is requested by the gNB-CU (at setup or modification of the MC context), in case the information is not available.

In principle, the gNB-CU controls the broadcast of the MBSMulticastConfiguration message, so it should be aware whether it is sent. However, there could be gNB-DU related cases which would make it necessary to also apply the “choice” structure to the MBS Multicast Configuration Notification IE.

Proposal 7:	F1AP BL CR: Introduce a new IE type for the MBS Multicast Configuration Notification IE in 9.3.1.x11, equivalent to the MBS Multicast Configuration Response Information IE type definition, in order to allow the gNB-DU to report removal of the MBSMulticastConfiguration message for gNB-DU generated reasons.

# 3	Conclusions

We have looked at some open items and conclude on the following observations and proposals:

Proposal 1:	It is proposed to rename in the F1AP BL CR in the MULTICAST GROUP PAGING message the new IE to “InactiveReceptionAllowed”, and to replace the Editor’s Note by the semantic description “Corresponds to information contained the inactiveReceptionAllowed as specified in TS 38.331 [8].”

Proposal 2:	Define a new procedure to configure common parts (related to all sessions) of the MCCH: Multicast Common Configuration, a class 1 procedure.

Proposal 3:	The Multicast Common Configuration contains per cell MCCH configuration parameters: The mbs-NeighbourCellList-r18 and the thresholdMBS-List-r18.

Proposal 4:	F1AP shall enable the gNB-CU to control the broadcast of the MBSMulticastConfiguration message on MCCH, i.e. to be able to turn it on and off.

Observation 5:	W.r.t. RAN2 plans to define a “PDCP COUNT synchronized” indication to UEs it is not expected that any specific RAN3 protocol work is necessary.

Proposal 6:	F1AP BL CR: Remove the Editor’s Note in 9.3.1.x1 and replace the FFS on the Range of the MBS Multicast Cell List by “0..1”..

Proposal 7:	F1AP BL CR: Introduce a new IE type for the MBS Multicast Configuration Notification IE in 9.3.1.x11, equivalent to the MBS Multicast Configuration Response Information IE type definition, in order to allow the gNB-DU to report removal of the MBSMulticastConfiguration message for gNB-DU generated reasons.

Final Proposal:	Agree on the text proposal shown in the next chapter for F1AP.

# 4	Text Proposal for the F1AP BL CR (latest available in R3-235967)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| MBS Context Notification | MBS CONTEXT NOTIFICATION INDICATION | MBS CONTEXT NOTIFICATION CONFIRM | MBS CONTEXT NOTIFICATION REFUSE |
| Multicast Common Configuration | MULTICAST COMMON CONFIGURATION REQUEST | MULTICAST COMMON CONFIGURATION RESPONSE | MULTICAST COMMON CONFIGURATION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.14	NR MBS Procedures

### 8.14.1	Broadcast Context Setup

#### 8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

If

-	either the MBS Service Area IE was included in the BROADCAST CONTEXT SETUP REQUEST message,

-	or the the MBS Service Area IE was not included in the BROADCAST CONTEXT SETUP REQUEST message and the gNB-DU was not able to establish MBS Session Resources in all cells served by the gNB-DU,

the Broadcast Area Scope IE shall be included in the BROADCAST CONTEXT SETUP RESPONSE message to indicate the cells where MBS Session resources have been successfully established in the gNB-DU.

If the Associated Session ID IE is contained in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.401 [4]. If the gNB-DU decides to not establish F1-U tunnel towards the gNB-CU it shall include the F1-U tunnel Not Established IE set to "true" in the BROADCAST CONTEXT SETUP RESPONSE message.

#### 8.14.1.3	Unsuccessful Operation

Figure 8.14.1.3-1: Broadcast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the requested MBS session context it shall consider the procedure as failed and reply with the BROADCAST CONTEXT SETUP FAILURE message.

#### 8.14.1.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.14.6	Multicast Context Setup

#### 8.14.6.1	General

The purpose of the Multicast Context Setup procedure is to establish an MBS Session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

#### 8.14.6.2	Successful Operation

Figure 8.14.6.2-1: Multicast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending MULTICAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the multicast MBS Session context, it replies to the gNB-CU with MULTICAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the MULTICAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the MULTICAST CONTEXT SETUP RSPONSE message, the result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Multicast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Multicast MRB Failed To Be Setup List IE;

If the Multicast MRB Failed To Setup List IE is contained in the MULTICAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Multicast MRB(s) failed to be setup with an appropriate cause value for each Multicast MRB failed to setup.

If the MULTICAST CONTEXT SETUP REQUEST message contains the MBS Multicast Configuration Request IE in the Multicast CU to DU RRC Information IE set to "query" and

-	if the gNB-DU is able to provide information about the requested resources, the gNB-DU shall, if supported, include the MBS Multicast Configuration IE in the MBS Multicast Configuration Response Information IE in the Multicast DU to CU RRC Information IE,

-	else if the gNB-DU is not able to provide information about the requested resources, the gNB-DU shall, if supported, include the MBS Multicast Configuration not available IE in the MBS Multicast Configuration Response Information IE in the Multicast DU to CU RRC Information IE set to "not available".

Interaction with the Multicast Distribution Context Setup procedure:

Upon reception of the MULTICAST CONTEXT SETUP REQUEST procedure, the gNB-DU shall trigger either per cell or per MBS Area Session ID or for the whole gNB-DU the Multicast Distribution Context Setup procedure to establish per cell or per MBS Area Session ID or the the whole gNB DU per accepted MRB a shared F1-U tunnel.

#### 8.14.6.3	Unsuccessful Operation

Figure 8.14.6.3-1: Multicast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the MBS session context it shall consider the procedure as failed and reply with the MULTICAST CONTEXT SETUP FAILURE message.

#### 8.14.6.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.14.9	Multicast Context Modification

#### 8.14.9.1	General

The purpose of the Multicast Context Modification procedure is to modify an established MBS session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

#### 8.14.9.2	Successful Operation

Figure 8.14.9.2-1: Multicast Context Modification procedure. Successful operation

The MULTICAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the MULTICAST CONTEXT MODIFICATION RESPONSE message.

If the Multicast MRB To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Multicast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Multicast MRB Failed To Be Setup List IE;

If the Multicast MRB Failed To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicated the resource for the failure with an appropriate cause value for each MRB failed to be setup.

If the Multicast MRB To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Multicast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Multicast MRB Failed To Be Modified List IE;

If the Multicast MRB Failed To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Multicast MRB(s) failed to be modified with an appropriate cause value for each Multicast MRB failed to modify.

If the MULTICAST CONTEXT MODIFICATION REQUEST message contains the MBS Multicast Configuration Request IE in the Multicast CU to DU RRC Information IE set to "query" and

-	if the gNB-DU is able to provide information about the requested resources, the gNB-DU shall, if supported, include the MBS Multicast Configuration IE in the MBS Multicast Configuration Response Information IE in the Multicast DU to CU RRC Information IE,

-	else if the gNB-DU is not able to provide information about the requested resources, the gNB-DU shall, if supported, include the MBS Multicast Configuration not available IE in the MBS Multicast Configuration Response Information IE in the Multicast DU to CU RRC Information IE set to "not available".

#### 8.14.9.3	Unsuccessful Operation

Figure 8.14.9.3-1: Multicast Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the multicast context can be successfully performed, the gNB-DU shall respond with the MULTICAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.14.9.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.14.A	Multicast Context Notification

#### 8.14.A.1	General

The purpose of the Multicast Context Notification is to inform the gNB-CU about changes in the multicast context configuration during an active multicast MBS session.

The procedure uses MBS-associated signalling.

#### 8.14.A.2	Successful Operation

Figure 8.14.A.2-1: Multicast Context Notification. Successful operation

The gNB-DU initiates the procedure by sending the MULTICAST CONTEXT NOTIFICATION INDICATION message to the gNB-CU.

If the MULTICAST CONTEXT NOTIFICATION INDICATION message contains the MBS Multicast Configuration Notification IE within the Multicast DU to CU RRC Information IE, the gNB-CU shall replace, for the respective cell, the Multicast Configuration Information previously received with information received in the MBS Multicast Configuration Notification IE.

If the gNB-CU is able to execute the requested functions, the gNB-CU shall respond with the MULTICAST CONTEXT NOTIFICATION CONFIRM message to the gNB-DU.

#### 8.14.A.3	Unsuccessful Operation

Figure 8.14.A.3-1: Multicast Context Notification. Unsuccessful operation

If the gNB-CU is not able to execute the requested functions, the gNB-CU shall respond with the MULTICAST CONTEXT NOTIFICATION REFUSE message to the gNB-DU.

#### 8.14.A.4	Abnormal Conditions

Not applicable.

### 8.14.C	Multicast Common Configuration

#### 8.14.C.1	General

The purpose of the Multicast Common Configuration procedure is to allow the gNB-CU to control the configuration of items common to all multicast contexts in the gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.14.C.2	Successful Operation

Figure 8.14.C.2-1: Multicast Common Configuration. Successful operation

The gNB-CU initiates the procedure by sending the MULTICAST COMMON CONFIGURATION REQUEST message to the gNB-DU.

If the the Multicast CU to DU Common RRC Information IE is included in the MULTICAST COMMON CONFIGURATION REQUEST message and contains the Multicast Common CU2DU Cell List IE, the gNB-DU shall for each indicated NR CGI

-	if the Multicast Common CU2DU Cell Info Setup or Update IE is included, start or continue sending the MBSMulticastConfiguration message on MCCH.

-	if the Update MBS Multicast Neighbour Cell List IE is included, update the MBSMulticastConfiguration message on MCCH accordingly.

-	if the Remove MBS Multicast Neighbour Cell List IE is included, remove the mbs-NeighbourCellList from the MBSMulticastConfiguration message on MCCH.

-	if the Update ThresholdMBS-List IE is included update the MBSMulticastConfiguration message on MCCH accordingly.

-	if the Remove ThresholdMBS-List IE is included, remove the thresholdMBS-List from the MBSMulticastConfiguration message on MCCH.

-	if the Stop MBSMulticastConfiguration IE set to “stop” is included, stop sending the MBSMulticastConfiguration message on MCCH.

If the gNB-DU is able to execute the requested functions, the gNB-DU shall respond with the MULTICAST COMMON CONFIGURATION RESPONSE message to the gNB-CU.

#### 8.14.C.3	Unsuccessful Operation

Figure 8.14.C.3-1: Multicast Context Notification. Unsuccessful operation

If the gNB-DU is not able to execute the requested functions, the gNB-DU shall respond with the MULTICAST COMMON CONFIGURATION RESPONSE message to the gNB-CU.

#### 8.14.C.4	Abnormal Conditions

If the the Multicast CU to DU Common RRC Information IE is included in the MULTICAST COMMON CONFIGURATION REQUEST message and contains the Multicast Common CU2DU Cell List IE, the gNB-DU shall for each indicated NR CGI

-	if the Stop MBSMulticastConfiguration IE set to “stop” is included although the MBSMulticastConfiguration message is not sent on MCCH or

-	if the Remove MBS Multicast Neighbour Cell List IE is included, although the mbs-NeighbourCellList is not contained in the MBSMulticastConfiguration message on MCCH or

-	if the Remove ThresholdMBS-List IE is included, although the thresholdMBS-List is not contained in the MBSMulticastConfiguration message on MCCH

ignore these requests.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.13	Broadcast Context Management messages

#### 9.2.13.1	BROADCAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a broadcast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Associated Session ID | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| F1-U Tunnel Not Established | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.14.1	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to multicast group page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| UE Identity List for Paging |  | 0..1 |  |  | YES | ignore |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >>UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.40 |  | - |  |
| MC Paging Cell List |  | 0..1 |  |  | YES | ignore |
| >MC Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| InactiveReceptionAllowed | O |  | ENUMERATED (true, ...) | Corresponds to information contained in the inactiveReceptionAllowed as specified in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for multicast group paging. Value is 4096. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

#### 9.2.14.2	MULTICAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a multicast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Multicast MRBs To Be Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRBs to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast CU to DU RRC Information | O |  | 9.3.1.x1 |  | YES | reject |
| MBS Multicast Session State | O |  | 9.3.1.x2 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.14.3	MULTICAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | Reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast DU to CU RRC Information | O |  | 9.3.1.x11 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.14.7	MULTICAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify multicast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| Multicast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | O |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast CU to DU RRC Information | O |  | 9.3.1.x1 |  | YES | reject |
| MBS Multicast Session State | O |  | 9.3.1.x2 |  | YES | reject |

#### 9.2.14.8	MULTICAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Multicast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast DU to CU RRC Information | O |  | 9.3.1.x11 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.14.B1	MULTICAST CONTEXT NOTIFICATION INDICATION

This message is sent by the gNB-DU to notify the gNB-CU about changes of the multicast context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast DU to CU RRC Information | O |  | 9.3.1.x11 |  | YES | reject |

#### 9.2.14.B2	MULTICAST CONTEXT NOTIFICATION CONFIRM

This message is sent by the gNB-CU to notify the gNB-DU to confirm the execution of the requested functions.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.14.B3	MULTICAST CONTEXT NOTIFICATION REFUSE

This message is sent by the gNB-CU to notify the gNB-DU that the execution of the requested functions was not successful.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.14.D1	MULTICAST COMMON CONFIGURATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure common items in the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Multicast CU to DU Common RRC Information | O |  | 9.3.1.x14 |  | YES | reject |

#### 9.2.14.D2	MULTICAST CONTEXT NOTIFICATION CONFIRM

This message is sent by the gNB-DU to notify the gNB-CU to confirm the execution of the requested functions.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.14.D3	MULTICAST CONTEXT NOTIFICATION REFUSE

This message is sent by the gNB-DU to notify the gNB-CU that the execution of the requested functions was not successful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x 	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	Multicast CU to DU RRC Information

This IE indicates the multicast specific CU to DU RRC Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Multicast Cell List |  | 0..1 |  |  |
| >MBS Multicast Cell Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  |
| >>RRC Multicast MTCH Neighbour Cell Information | O |  | OCTET STRING | Includes the mtch-NeighbourCell in the MBS-SessionInfoListMulticast IE as specified in TS 38.331 [8]. |
| >>ThresholdIndex | O |  | INTEGER (0..maxnoofThresholdMBS) | Corresponds to the thresholdIndex as specified in TS 38.331 [8]. Editor’s Note: to be aligned with running RRC CR. |
| >>MBS Multicast Configuration Request | O |  | ENUMERATED (query, ...) |  |
| MBS Multicast MRB List |  | 0..1 |  |  |
| >MBS Multicast MRB Item |  | 1 .. <maxnoofMRBs> |  |  |
| >>MRB ID | M |  | 9.3.1.224 |  |
| >>MRB PDCP Config Broadcast | M |  | OCTET STRING | Includes the MRB-PDCP-ConfigBroadcast IE, as defined in TS 38.331 [8]. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofMRBs | Maximum no. MRBs allowed to be setup for one MBS session, the maximum value is 32. |
| maxnoofThresholdMBS | Maximum no. thresholds configured in a cell. Value is 64. Editor’s Note: value is FFS. |

#### 9.3.1.x11	Multicast DU to CU RRC Information

This IE indicates the multicast specific DU to CU RRC Information.

Editor’s Note:	IE names, their presence, ranges, semantics and references to field/IE/messages names in TS 38.331 are FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Multicast Cell List |  | 0..1 |  |  |
| >MBS Multicast Cell Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  |
| >>MBS Multicast Configuration Response Information | O |  | 9.3.1.x12 |  |
| >>MBS Multicast Configuration Notification | O |  | 9.3.1.x13 |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.3.1.x12	MBS Multicast Configuration Response Information

This IE contains information on the gNB-DU’s response to the requested multicast configuration for reception RRC_INACTIVE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Multicast Configuration Response Information | O |  |  |  |
| >MBS Multicast Configuration available |  |  |  |  |
| >>MBS Multicast Configuration | M |  | OCTET STRING | Includes the MBSMulticastConfiguration message as defined in TS 38.331 [8]. |
| >MBS Multicast Configuration not available |  |  |  |  |
| >>MBS Multicast Configuration not available | M |  | ENUMERATED (not available, ...) |  |

#### 9.3.1.x13	MBS Multicast Configuration Notification

This IE contains information on the gNB-DU’s notification of MBS Multicast Configuration information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Multicast Configuration Notification Information | O |  |  |  |
| >MBS Multicast Configuration changed |  |  |  |  |
| >>MBS Multicast Configuration | M |  | OCTET STRING | Includes the MBSMulticastConfiguration message as defined in TS 38.331 [8]. |
| >MBS Multicast Configuration removed |  |  |  |  |
| >>MBS Multicast Configuration removed | M |  | ENUMERATED (removed, ...) |  |

#### 9.3.1.x14	Multicast CU to DU Common RRC Information

This IE indicates the multicast specific CU to DU RRC Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multicast Common CU2DU Cell List |  | 0..1 |  |  |
| >Multicast COMMON CU2DU Cell Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  |
| >>CHOICE Multicast Common CU2DU Cell Information | M |  |  |  |
| >>>Common CU2DU Cell Info Setup or Update |  |  |  |  |
| >>>CHOICE MBS Multicast Neighbour Cell List Item | O |  |  |  |
| >>>>>Update MBS Multicast Neighbour Cell List |  |  | OCTET STRING | Includes the mbs-NeighbourCellList-r18, as specified in TS 38.331[8] |
| >>>>>Remove MBS Multicast Neighbour Cell List |  |  | NULL |  |
| >>>>CHOICE ThresholdMBS-List Item | O |  |  |  |
| >>>>>Update ThresholdMBS-List |  |  | OCTET STRING | Includes thresholdMBS-List as specified in TS 38.331[8] |
| >>>>>Remove ThresholdMBS-List |  |  | NULL |  |
| >>>Common CU2DU Cell info Stop |  |  |  |  |
| >>>>Stop MBSMulticastConfiguration | O |  | ENUMERATED (stop, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.3.1.x2	MBS Multicast Session State

This IE indicates the state of MBS Multicast Session. The MBS Multicast Session state is defined in TS 23.247 [x3].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Multicast Session State | M |  | ENUMERATED (active, inactive, …) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

F1SetupResponse,

F1SetupFailure,

GNBDUConfigurationUpdate,

GNBDUConfigurationUpdateAcknowledge,

GNBDUConfigurationUpdateFailure,

GNBCUConfigurationUpdate,

GNBCUConfigurationUpdateAcknowledge,

GNBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

GNBDUResourceCoordinationRequest,

GNBDUResourceCoordinationResponse,

PrivateMessage,

UEInactivityNotification,

InitialULRRCMessageTransfer,

SystemInformationDeliveryCommand,

Paging,

Notify,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

PWSCancelRequest,

PWSCancelResponse,

PWSRestartIndication,

PWSFailureIndication,

GNBDUStatusIndication,

RRCDeliveryReport,

UEContextModificationRefuse,

F1RemovalRequest,

F1RemovalResponse,

F1RemovalFailure,

NetworkAccessRateReduction,

TraceStart,

DeactivateTrace,

DUCURadioInformationTransfer,

CUDURadioInformationTransfer,

BAPMappingConfiguration,

BAPMappingConfigurationAcknowledge,

BAPMappingConfigurationFailure,

GNBDUResourceConfiguration,

GNBDUResourceConfigurationAcknowledge,

GNBDUResourceConfigurationFailure,

IABTNLAddressRequest,

IABTNLAddressResponse,

IABTNLAddressFailure,

IABUPConfigurationUpdateRequest,

IABUPConfigurationUpdateResponse,

IABUPConfigurationUpdateFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

AccessAndMobilityIndication,

ReferenceTimeInformationReportingControl,

ReferenceTimeInformationReport,

AccessSuccess,

CellTrafficTrace,

PositioningMeasurementRequest,

PositioningMeasurementResponse,

PositioningMeasurementFailure,

PositioningAssistanceInformationControl,

PositioningAssistanceInformationFeedback,

PositioningMeasurementReport,

PositioningMeasurementAbort,

PositioningMeasurementFailureIndication,

PositioningMeasurementUpdate,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PositioningInformationUpdate,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

BroadcastContextSetupRequest,

BroadcastContextSetupResponse,

BroadcastContextSetupFailure,

BroadcastContextReleaseCommand,

BroadcastContextReleaseComplete,

BroadcastContextReleaseRequest,

BroadcastContextModificationRequest,

BroadcastContextModificationResponse,

BroadcastContextModificationFailure,

MulticastGroupPaging,

MulticastContextSetupRequest,

MulticastContextSetupResponse,

MulticastContextSetupFailure,

MulticastContextReleaseCommand,

MulticastContextReleaseComplete,

MulticastContextReleaseRequest,

MulticastContextModificationRequest,

MulticastContextModificationResponse,

MulticastContextModificationFailure,

MulticastDistributionSetupRequest,

MulticastDistributionSetupResponse,

MulticastDistributionSetupFailure,

MulticastDistributionReleaseCommand,

MulticastDistributionReleaseComplete,

PDCMeasurementInitiationRequest,

PDCMeasurementInitiationResponse,

PDCMeasurementInitiationFailure,

PDCMeasurementReport,

PDCMeasurementTerminationCommand,

PDCMeasurementFailureIndication,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

PosSystemInformationDeliveryCommand,

MulticastContextNotificationIndication,

MulticastContextNotificationConfirm,

MulticastContextNotificationRefuse,

MulticastCommonConfigurationRequest,

MulticastCommonConfigurationResponse,

MulticastCommonConfigurationFailure

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementInitiationRequest,

id-PDCMeasurementInitiationResponse,

id-PDCMeasurementInitiationFailure,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand,

id-MulticastContextNotification,

id-MulticastCommonConfiguration

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

F1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

choice-extension	ProtocolIE-SingleContainer { { F1AP-PDU-ExtIEs} }

}

F1AP-PDU-ExtIEs F1AP-PROTOCOL-IES ::= { -- this extension is not used

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURES F1AP-ELEMENTARY-PROCEDURE ::= {

F1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

F1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

f1Setup							|

gNBDUConfigurationUpdate		|

gNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning				|

pWSCancel					|

gNBDUResourceCoordination		|

f1Removal						|

bAPMappingConfiguration			|

gNBDUResourceConfiguration		|

iABTNLAddressAllocation			|

iABUPConfigurationUpdate		|

resourceStatusReportingInitiation	|

positioningMeasurementExchange	|

tRPInformationExchange			|

positioningInformationExchange	|

positioningActivation			|

e-CIDMeasurementInitiation		|

broadcastContextSetup			|

broadcastContextRelease			|

broadcastContextModification|

multicastContextSetup			|

multicastContextRelease		|

multicastContextModification	|

multicastDistributionSetup		|

multicastDistributionRelease	|

pDCMeasurementInitiation		|

pRSConfigurationExchange		|

measurementPreconfiguration		|

multicastContextNotification	|

multicastCommonConfiguration	,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

qoEInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransfer

PROCEDURE CODE			id-QoEInformationTransfer

CRITICALITY				ignore

}

posSystemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PosSystemInformationDeliveryCommand

PROCEDURE CODE			id-PosSystemInformationDeliveryCommand

CRITICALITY				ignore

}

multicastContextNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextNotificationIndication

SUCCESSFUL OUTCOME		MulticastContextNotificationConfirm

UNSUCCESSFUL OUTCOME	MulticastContextNotificationRefuse

PROCEDURE CODE			id-MulticastContextNotification

CRITICALITY				reject

}

multicastCommonConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastCommonConfigurationRequest

SUCCESSFUL OUTCOME		MulticastCommonConfigurationResponse

UNSUCCESSFUL OUTCOME	MulticastCommonConfigurationFailure

PROCEDURE CODE			id-MulticastCommonConfiguration

CRITICALITY				reject

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

InactiveReceptionAllowed,

MulticastCU2DURRCInfo,

MulticastDU2CURRCInfo,

MBSMulticastSessionState,

F1UTunnelNotEstablished,

MulticastCU2DUCommonRRCInfo

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-InactiveReceptionAllowed,

id-MulticastCU2DURRCInfo,

id-MulticastDU2CURRCInfo,

id-MBSMulticastSessionState,

id-F1UTunnelNotEstablished,

id-MulticastCU2DUCommonRRCInfo,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofChildIABNodes,

maxnoofServedCellsIAB,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofNeighbourNodeCellsIAB,

maxnoofMRBsforUE,

maxnoofServingCellMOs

FROM F1AP-Constants;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Multicast Group Paging PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Multicast Group Paging

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastGroupPagingIEs}},

...

}

MulticastGroupPagingIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID					CRITICALITY reject	TYPE MBS-Session-ID							PRESENCE mandatory	}|

{ ID id-UEIdentity-List-For-Paging-List	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-List		PRESENCE optional		}|

{ ID id-MC-PagingCell-List				CRITICALITY ignore	TYPE MC-PagingCell-list						PRESENCE optional		}|

{ ID id-InactiveReceptionAllowed	CRITICALITY ignore	TYPE InactiveReceptionAllowed		PRESENCE optional		},

...

}

UEIdentity-List-For-Paging-List	 ::= SEQUENCE (SIZE(1.. maxnoofUEIDforPaging)) OF ProtocolIE-SingleContainer { { UEIdentity-List-For-Paging-ItemIEs } }

UEIdentity-List-For-Paging-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentity-List-For-Paging-Item	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-Item 			PRESENCE optional }	,

...

}

MC-PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF ProtocolIE-SingleContainer { { MC-PagingCell-ItemIEs } }

MC-PagingCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MC-PagingCell-Item		CRITICALITY ignore	TYPE MC-PagingCell-Item			PRESENCE mandatory}	,

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP REQUEST

--

-- **************************************************************

MulticastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupRequestIEs}},

...

}

MulticastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory  }|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID						PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea					CRITICALITY reject TYPE	MBS-ServiceArea							PRESENCE optional   }|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI								PRESENCE mandatory  }|

{ ID id-MulticastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	MulticastMRBs-ToBeSetup-List		PRESENCE mandatory  }|

{ ID id-MulticastCU2DURRCInfo			CRITICALITY reject	TYPE	MulticastCU2DURRCInfo				PRESENCE optional   }|

{ ID id-MBSMulticastSessionState		CRITICALITY reject	TYPE	MBSMulticastSessionState			PRESENCE optional   },

...

}

MulticastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetup-ItemIEs} }

MulticastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	MulticastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

MulticastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupResponseIEs}},

...

}

MulticastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MulticastMRBs-Setup-List			CRITICALITY reject TYPE MulticastMRBs-Setup-List					PRESENCE mandatory	}|

{ ID id-MulticastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetup-List 	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-MulticastDU2CURRCInfo				CRITICALITY reject TYPE MulticastDU2CURRCInfo						PRESENCE optional	},

...

}

MulticastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Setup-ItemIEs} }

MulticastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetup-ItemIEs} }

MulticastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Setup-Item			CRITICALITY reject	TYPE MulticastMRBs-Setup-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE MulticastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP FAILURE

--

-- **************************************************************

MulticastContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupFailureIEs}},

...

}

MulticastContextSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY ignore	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE optional	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE COMMAND

--

-- **************************************************************

MulticastContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseCommandIEs}},

...

}

MulticastContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE COMPLETE

--

-- **************************************************************

MulticastContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseCompleteIEs}},

...

}

MulticastContextReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT RELEASE REQUEST ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE REQUEST

--

-- **************************************************************

MulticastContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseRequestIEs}},

...

}

MulticastContextReleaseRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

MulticastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationRequestIEs}},

...

}

MulticastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeModified-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-List	PRESENCE optional  }|

{ ID id-MulticastCU2DURRCInfo				CRITICALITY reject	TYPE	MulticastCU2DURRCInfo					PRESENCE optional  }|

{ ID id-MBSMulticastSessionState			CRITICALITY reject	TYPE MBSMulticastSessionState				PRESENCE optional  },

...

}

MulticastMRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetupMod-ItemIEs} }

MulticastMRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeModified-ItemIEs} }

MulticastMRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeModified-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeReleased-ItemIEs} }

MulticastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

MulticastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationResponseIEs}},

...

}

MulticastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-MulticastMRBs-SetupMod-List			CRITICALITY reject TYPE MulticastMRBs-SetupMod-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-Modified-List			CRITICALITY reject TYPE MulticastMRBs-Modified-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeModified-List	PRESENCE optional  }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional  }|

{ ID id-MulticastDU2CURRCInfo					CRITICALITY reject TYPE MulticastDU2CURRCInfo							PRESENCE optional	},

...

}

MulticastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-SetupMod-ItemIEs} }

MulticastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE MulticastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetupMod-ItemIEs} }

MulticastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE MulticastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Modified-ItemIEs } }

MulticastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Modified-Item				CRITICALITY		reject	TYPE MulticastMRBs-Modified-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeModified-ItemIEs} }

MulticastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE MulticastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

MulticastContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationFailureIEs}},

...

}

MulticastContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT NOTIFICATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT NOTIFICATION INDICATION

--

-- **************************************************************

MulticastContextNotificationIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastContextNotificationIndicationIEs}},

...

}

MulticastContextNotificationIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MulticastDU2CURRCInfo				CRITICALITY reject	TYPE MulticastDU2CURRCInfo						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT NOTIFICATION CONFIRM

--

-- **************************************************************

MulticastContextNotificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastContextNotificationConfirmIEs}},

...

}

MulticastContextNotificationConfirmIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT NOTIFICATION REFUSE

--

-- **************************************************************

MulticastContextNotificationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastContextNotificationRefuseIEs}},

...

}

MulticastContextNotificationRefuseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- MULTICAST COMMON CONFIGURATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST COMMON CONFIGURATION REQUEST

--

-- **************************************************************

MulticastCommonConfigurationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastCommonConfigurationRequestIEs}},

...

}

MulticastCommonConfigurationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastCU2DUCommonRRCInfo				CRITICALITY reject	TYPE MulticastCU2DUCommonRRCInfo						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST COMMON CONFIGURATION RESPONSE

--

-- **************************************************************

MulticastCommonConfigurationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastCommonConfigurationResponseIEs}},

...

}

MulticastCommonConfigurationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST COMMON CONFIGURATION FAILURE

--

-- **************************************************************

MulticastCommonConfigurationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastCommonConfigurationFailureIEs}},

...

}

MulticastCommonConfigurationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- MULTICAST DISTRIBUTION SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofMRBs,

maxNrofBWPs,

maxnoofThresholdMBS

FROM F1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM F1AP-CommonDataTypes

ProtocolExtensionContainer{},

F1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

#### -- A

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

IngressNonF1terminatingTopologyIndicator ::= ENUMERATED {true, ...}

IntendedTDD-DL-ULConfig-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

InactiveReceptionAllowed ::= ENUMERATED {true, ...}

IPHeaderInformation ::= SEQUENCE {

destinationIABTNLAddress			IABTNLAddress,

dsInformationList					DSInformationList	OPTIONAL,

iPv6FlowLabel						BIT STRING (SIZE (20))	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { IPHeaderInformation-ItemExtIEs} } OPTIONAL,

...

}

IPHeaderInformation-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBSMulticastF1UContextDescriptor ::= SEQUENCE {

multicastF1UContextReferenceF1		MulticastF1UContextReferenceF1,

mc-F1UCtxtusage 	ENUMERATED {ptm, ptp, ptp-retransmission, ptp-forwarding, ...},

mbsAreaSession						MBS-Area-Session-ID				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}} OPTIONAL,

...

}

MBSMulticastF1UContextDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastSessionState ::= ENUMERATED {active, inactive, ...}

MulticastCU2DURRCInfo		::= SEQUENCE {

mBS-Multicast-CU2DU-Cell-List		MBS-Multicast-CU2DU-Cell-List		OPTIONAL,

mBS-Multicast-MRB-List		MBS-Multicast-MRB-List						OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastCU2DURRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastCU2DURRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-CU2DU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Multicast-CU2DU-Cell-Item

MBS-Multicast-CU2DU-Cell-Item ::= SEQUENCE {

nRCGI								NRCGI,

multicast-mtch-neighbourCell		OCTET STRING		OPTIONAL,

thresholdIndex						INTEGER (0..maxnoofThresholdMBS)		OPTIONAL,

mbsMulticastConfigurationRequest	ENUMERATED {query, ...}				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-CU2DU-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-CU2DU-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Multicast-MRB-Item

MBS-Multicast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastDU2CURRCInfo		::= SEQUENCE {

mBS-Multicast-DU2CU-Cell-List		MBS-Multicast-DU2CU-Cell-List			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastDU2CURRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastDU2CURRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-DU2CU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Multicast-DU2CU-Cell-Item

MBS-Multicast-DU2CU-Cell-Item ::= SEQUENCE {

nRCGI									NRCGI,

mbsMulticastConfigurationResponseInfo	MBSMulticastConfigurationResponseInfo		OPTIONAL,

mbsMulticastConfigurationNotification	MBSMulticastConfigurationNotificationInfo								OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-DU2CU-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-DU2CU-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationResponseInfo ::= CHOICE {

mbsMulticastConfiguration-available			OCTET STRING,

mbsMulticastConfiguration-notavailable		ENUMERATED {not-available, ...},

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastConfigurationResponseInfo-ExtIEs} }

}

MBSMulticastConfigurationResponseInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MBSMulticastConfigurationNotificationInfo::= CHOICE {

mbsMulticastConfiguration-changed			OCTET STRING,

mbsMulticastConfiguration-removed		ENUMERATED {removed, ...},

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastConfigurationNotificationInfo-ExtIEs} }

}

MBSMulticastConfigurationNotificationInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastCU2DUCommonRRCInfo		::= SEQUENCE {

multicastCommonCU2DU-Cell-List		MulticastCommonCU2DU-Cell-List			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {MulticastCU2DUCommonRRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastCU2DUCommonRRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCommonCU2DU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF MulticastCommonCU2DU-Cell-List-Item

MulticastCommonCU2DU-Cell-List-Item ::= SEQUENCE {

nRCGI									NRCGI,

multicastCommonCU2DUCellInfo			MulticastCommonCU2DUCellInfo		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {MulticastCommonCU2DU-Cell-List-Item-ExtIEs} } OPTIONAL,

...

}

MulticastCommonCU2DU-Cell-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCommonCU2DUCellInfo ::= CHOICE {

commonCU2DUCellinfoSetupOrUpdate			MulticastCommonCU2DUCellinfoSetupOrUpdate,

commonCU2DUCellinfoRemove					ENUMERATED {stop, ...},

choice-extension		ProtocolIE-SingleContainer { {MulticastCommonCU2DUCellInfo-ExtIEs} }

}

MulticastCommonCU2DUCellInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastCommonCU2DUCellinfoSetupOrUpdate ::= SEQUENCE {

multicastNeighbourCellListItem		MulticastNeighbourCellListItem			OPTIONAL,

thresholdMBSListItem				ThresholdMBSListItem					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {MulticastCommonCU2DUCellinfoSetupOrUpdate-ExtIEs} } OPTIONAL,

...

}

MulticastCommonCU2DUCellinfoSetupOrUpdate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastNeighbourCellListItem ::= CHOICE {

updateMBSMulticastNeighbourCellList			OCTET STRING,

removeMBSMulticastNeighbourCellList			NULL,

choice-extension		ProtocolIE-SingleContainer { {MulticastNeighbourCellListItem-ExtIEs} }

}

MulticastNeighbourCellListItem-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ThresholdMBSListItem ::= CHOICE {

updateThresholdMBSList						OCTET STRING,

removeThresholdMBSList						NULL,

choice-extension		ProtocolIE-SingleContainer { {ThresholdMBSListItem-ExtIEs} }

}

ThresholdMBSListItem-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastF1UContext-ToBeSetup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-DU      UPTransportLayerInformation,

mbsProgressInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-ToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

id-MulticastContextNotification 			ProcedureCode ::= 100 -- to be allocated

id-MulticastCommonConfiguration				ProcedureCode ::= 101 -- to be allocated

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList                                 		ProtocolIE-ID ::= 707

id-AssociatedSessionID								ProtocolIE-ID ::= 994 -- to be allocated

id-InactiveReceptionAllowed					ProtocolIE-ID ::= 999 -- to be allocated

id-MulticastCU2DURRCInfo							ProtocolIE-ID ::= 998 -- to be allocated

id-MBSMulticastSessionState							ProtocolIE-ID ::= 997 -- to be allocated

id-F1UTunnelNotEstablished                       	ProtocolIE-ID ::= 996 -- to be allocated

id-MulticastDU2CURRCInfo							ProtocolIE-ID ::= 995 -- to be allocated

id-MulticastCU2DUCommonRRCInfo						ProtocolIE-ID ::= 994 -- to be allocated

END

-- ASN1STOP