3GPP TSG-RAN WG3 #122						               										R3-237436

13th – 17th Nov 2023

Chicago, USA

Agenda item:	17.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.455) Support UE location verification in NR NTN

Document for:	Discussion & decision

# 1 Introduction

This contribution discusses the potential required RAN3 enhancements to support UE location verification.

# 2 Discussion

Previous RAN3 meetings failed to reach agreement on following:

Whether the NTN related configuration information, e.g., Ephemeris info of the satellites, Association between the satellite and TRP ID are dynamic information？

P1: Ephemeris info of the satellites should be configured to the LMF via OAM.

P2: Association between the satellite and TRP ID should also be configured to LMF via OAM.

P3: The TRP related information exchange procedure can be applicable to NTN without any standard impact?

In current Terrestrial Network, the location of TRP is fixed. In NTN, the TRP is on the satellite, and its location keeps changing in NGSO. This brings some issues:

- Issue #1: how can LMF know the location of the TRP/Satellite when a measurement is performed

Option 1: similar solution as Mobile IAB

The similar issue was discussed in Rel-18 mobile IAB that the UE may be positioned using the TRP associated with the mobile IAB (i.e. mobile TRP). The location of mobile TRP may be dynamic. In last meeting, RAN3 agreed the BL CR for TS38.455/TS38.473 to report the location of mobile TRP via the TRP Measurement Result IE, e.g. when the NG-RAN node report the measurement report. We believe this can also be reused for NTN. When the NTN gNB report the UE’s measurement, the NG-RAN can also report the location of the TRP/Satellite.

This option allows a common LMF to be developed for both mobile IAB and NTN, and does not require the LMF to know the satellite information.

Option 2: LMF is configured with the satellite information

Ericsson/CATT R3-233311/R3-232799 propose:

For NTN, the LMF receives from the OAM the satellite related information (described in TS 38.300 [x]), as well as the association between the TRP(s) and the satellite(s).

Option 2 also help to solve the mirror point issue. By knowing the satellite ephemeris information, the LMF can know what direction the neighbors are placed on Earth (so the beam direction would need to be known). So we prefer Option 2.

Proposal 1: LMF is configured with the satellite information

There may be a small issue here. The satellite can be changed for a UE, but no handover for the stationary UE, as the serving cell stays identical. The LMF may only be able to know the rough period for the serving window of a satellite, but it may not know exactly which satellite is serving the UE. For example, in case of soft feeder link switch, the change of satellite may be happened during a short period. LMF may not know exactly when the satellite switch is performed. LMF may assume the UE position is using Satellite#1, but it is switched to Satellite#2.  So it may seems necessary for gNB inform LMF the satellite ID, indicating which satellite has performed measurement. This may require enhancement to TS 38.455 TRP Measurement Result IE.

Proposal 2: Add Satellite ID in the TRP Measurement Result for TS38.455.

The proposed TP can be found in Annex – TP for TS38.455.

This is also related to RAN2 LS ([3]), which asked following scenarios:

Scenario 1: Handover between TN and NTN where LMF needs to be aware that UE capabilities may be different.

Scenario 2: Satellite switch with PCI unchanged where SRS and PRS configurations remain the same, but LMF would need to have updated satellite information.

Scenario 3: Intra-gNB handover

Continuity for Scenario 1 is not needed. The location verification is only applicable for NTN. When the UE is HO from NTN to TN, the location verification can be stopped since the TN cell ID is enough to meet the regulatory requirements.

Scenario 2 was described above. It may cause issue since LMF does not know the satellite switch.

Scenario 3 maybe same as TN, where an intra-gNB HO can happen during the position.

- Issue #2: Altitude

Huawei ([2]) discussed a potential issue for the Altitude IE.

| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N £ a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| --- | --- | --- | --- | --- |

We noticed that the maximal altitude provided by GAD is ~32 km (32,768 m) where some satellite are generally reach an orbital above e.g. 400 km to 2000 Km for (LEO), 8 000 to 20 000 for NGSO and 36 000 km for GEO.

It seems then it is not possible for the LMF to get the correct Altitude of the TRP to provide positioning. We hence propose to correct this issue by introducing a NTN Access Point Position based on ECEF. The X, Y, Z are currently aligned with the RRC e.g. EphemerisInfo IE . The ANNEX proposes some draft CR to reflect the change in LPPa and NRPPa.

We do not foreseen impact on F1 due to transparent payload usage in current release, but similar change could be implemented in F1 to align the specifications…. It is up to RAN3 to discuss and decide.

In case Option 2 (LMF is configured with the satellite ephemeris information), this issue may be vanished. The LMF will not initiate the TRP Information Exchange procedure or other NRPPa procedure to retrieve the location of TRP/satellite.

Proposal 3: In case Proposal 1 is agreed, there is no need to enhance the Altitude IE in NRPPa.

- Issue #3: mirror-image issue

When multi-RTT position method is used for location verification, there is a mirror-image issue.

Figure 1: multi-RTT based positioning with single satellite

RAN#101 endorses that E-CID method could be further analyzed and enhanced in RAN3, if needed for NTN scenarios (NR-NTN and IoT-NTN).

RAN2#121 reached following agreement:

Agreements:

- In order to resolve the mirror point ambiguity issue, the network relies on the legacy signaling and procedure to configure NTN UE to measure and report neighbor cells or reference signals/beams. No spec changes to radio interface are needed from RAN2 perspective. Unclear if changes are needed to other interfaces, NRPPa protocol (RAN2 will no longer discuss this)

The LMF knows the satellite ephemeris, and the footprint of each satellite beam/Uu cell. The LMF can request the UE for which the location verification is performed to do RSRP measurements for certain neighboring cells. In Figure 2, the network verified location procedure is intended to prevent the UE to maliciously report itself being in the below part of the cell (e.g. mirror location B) although it being in the upper part (e.g. actual Location A), thus being allowed to access services not available in the country covered by the upper part of the cell. In order to detect this, neighbour cell measurements can be applied, as i.e. satellite beam 7 (SB7) would provide a higher RSRP than i.e. SB10, thus allowing the LMF to detect the actual side in which the UE is.

Country border

Country border

Figure 2 UE placement (red triangle) with respect to a country border (blue line)

The UE measures the serving cell and neighbour cells. This still works in case UE measures neighboring cells. It may be a new requirement to UE, i.e. to measure particular neighbor cells.

The UE provides the measurement result indicating the related cell (e.g. PCI, CGI, etc). By combining the received measurement result and the knowledge of satellite beam information, the LMF is able to differentiate the UE’s actual location and the mirror-location.

One argument is the LMF may not know the PCI. But this may be not an issue. The LMF can initiate TRP Information Exchange procedure to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node. The detailed information includes nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geocoordinates, trp type, ondemand prs, trp tx teg, beam antenna info.

TS38.305 defines following information that can be transferred from gNB to LMF:

Information

UL Angle of Arrival (azimuth and elevation)

Cell Portion ID

NR Measurement Results List:

- SS Reference signal received power (SS-RSRP)

- SS Reference Signal Received Quality (SS-RSRQ)

- CSI Reference signal received power (CSI-RSRP)

- CSI Reference Signal Received Quality (CSI-RSRQ)

- NR Cell Global Identifier /Physical Cell ID

- Timing Advance (TADV)

Table: Information that may be transferred from gNB to the LMF

Based on the above information, the LMF can know the neighboring cell for the UE. This can address the mirror-image issue.

Observation 1: by measuring the neighbour cell, the LMF is able to differentiate the UE’s actual location from the mirror location.

In current E-CID position method, the LMF only request the UE to perform specific measurement, as defined in TS 37.355 (copied as below).

NR-ECID-RequestLocationInformation-r16 ::= SEQUENCE {

requestedMeasurements-r16		BIT STRING {	ssrsrpReq		(0),

ssrsrqReq		(1),

csirsrpReq		(2),

csirsrqReq		(3)} (SIZE(1..8)),

...

}

-- ASN1STOP

In case of legacy procedures, LMF provides assistance data to UE which can be a list of PRS reference signal configuration for each TRP (i.e. gNB). UE uses the PRS assistance data to perform PRS based positioning measurements. However, as measurement based on the provided PRS config for a list of TRPs is up to UE implementation this solution may not be sufficient to achieve the desired behaviour for network verified location, as the UE in this case should report at least a cell on each side of the country border (i.e. SB6, and a worse cell i.e. SB10) as this would indicate to the LMF which side the UE is closest to. Thus, we propose to have a new information element in the LPP for network verified UE location, which is then used to indicate to the UE which cells specifically to measure and report. Such an information element may also reduce overhead in respect to reporting as the report will contain fewer entities. Another option is for gNB to configure the UE to measure specific neighbor cells. This may be in RAN2 scope.

Proposal 4: RAN3/RAN2 to specify means for how to configure the UE to measure specific neighboring cells.

Mapped Cell ID

Last meeting discussed whether the gNB provide the Mapped Cell ID to LMF, e.g. : Include mapped Cell ID in Measurement Quantities Value IE for E-CID MEASUREMENT INITIATION REQUEST NRRPa message and Define a new Mapped Cell ID IE in E-CID Measurement Result information element. However, the justification is unclear due to following reasons:

- LMF already know the Mapped Cell ID from AMF, when AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF.

During the UE’s registration procedure, the gNB determines a Mapped Cell ID, and provide the Mapped Cell ID to AMF. In case the AMF determine there is a need to perform a location verification, the AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. The service operation includes the serving cell identity, which is the Mapped Cell ID received from the gNB.  So when the LMF starts the UE position procedure, the LMF know the Mapped Cell ID determined by the gNB.

Since the position procedure may takes 30-second to 1-minute, there is an argument that the Mapped Cell ID may be changed during this period. Considering the potential velocity of the UE, it is rare that the Mapped Cell ID is changed during this 30-second to 1-min period. In case a UE is on the airplane, the UE may move 10KM during a 1-min period. But this may be similar to TN, e.g. position a high-speed UE.

Observation: when the AMF invoke the Nlmf_Location_DetermineLocation service operation towards the LMF , the AMF provides the Mapped Cell ID received from the gNB to the LMF.

- Even in case the gNB re-determine a Mapped cell ID during the measurement, it may not help to address the mirror-point issues.

It is worthy to first review how gNB determine the Mapped Cell ID.

- Option 1: gNB determine the Mapped Cell ID based on the UE location reported from the UE. Since the UE reported location is not-trustable and awaiting for verification, the gNB cannot use the UE reported location to determine the Mapped Cell ID. So this option is not applicable.

- Option 2: gNB determine the Mapped Cell ID based on its implementation. Since it is possible that UE just power up and get connected, so the determination should not rely on the history information. One possibility is to use the satellite beam information. There are two further options.

+ Option 2a: the footprint of a satellite beam is small, and only covers location A, but not mirror location B. By using the satellite beam information, the gNB is able to determine a correct/reliable Mapped Cell ID. However, since the determined Mapped Cell ID is reliable, there is no need to perform any further location verification.

Figure 3 Different Mapped Cell ID for Location A and mirror Location B

+ Option 2b: the footprint of a satellite beam is large and covers both location A and mirror location B. By using the satellite beam information, the gNB is not able to determine a correct Mapped Cell ID that can differentiate location A from  Mirror Location B. In this case, providing an incorrect Mapped Cell ID to LMF is useless to the LMF.

Figure 4 Same Mapped Cell ID for Location A and mirror Location B

In a summary, it is proposed the gNB first guess the Mapped Cell ID based on its implementation, and provide the “Guess Mapped Cell ID” to LMF. The LMF then use the received “Guess Mapped Cell ID” to verify the UE’s actual location. So it is questionable whether this verification is correct, since the verification is based on a “Guess Mapped Cell ID”.

Based on the above analysis, it is questionable on what kind of issue can be solved, and what benefit can be added by added, by introducing the Mapped Cell ID in NRPPa.

Observation 2: it is unclear on the specific issues to be addressed and benefit by introducing the Mapped Cell ID in NRPPa.

Proposal 5: discuss and clarify the scenario/benefit for introducing Mapped Cell ID in NRPPa.

# 4 Conclusions

In this contribution, we analyzed the support for location verification in NRT NTN. Our proposals are:

Proposal 1: LMF is configured with the satellite information

Proposal 2: Add Satellite ID in the TRP Measurement Result for TS38.455.

The proposed TP can be found in Annex – TP for TS38.455.

Proposal 3: In case Proposal 1 is agreed, there is no need to enhance the Altitude IE in NRPPa.

Proposal 4: RAN3/RAN2 to specify means for how to configure the UE to measure specific neighboring cells.

Proposal 5: discuss and clarify the scenario/benefit for introducing Mapped Cell ID in NRPPa.

# References

- TS23.273

- R3-232997, Altitude correction for the NTN TRP (Huawei)

- R3-237138/R2-2311324, LS on NW verified UE location failure during cell change

# Annex – TP for TS38.455

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:"NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[4]	Void.

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[8]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[9]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[10]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.455: " Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 37.355: " Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[15]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[16]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[17]	3GPP TS 36:214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer (PHY); Measurements".

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[19]	3GPP TS 38.215: "NR; Physical layer (PHY); Measurements".

[x] 		3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Z-AoA | M |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA | M |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP | M |  | 9.2.72 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >NTN Node ID (name FFS) | O |  | 9.2.x |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

### 9.2.x	NTN Node ID

This IE is used to uniquely identify an NTN node which host the TRP in Non-Terrestrial Network.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NTN Node ID | M |  | OCTET STRING |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

id-ResultEUTRA,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoPosMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxPRS-ResourceSets,

maxPRS-ResourcesPerSet,

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource,

maxnoofULAoAs,

maxNoPathExtended,

maxnoARPs,

maxnoTRPTEGs,

maxnoUETEGs,

maxFreqLayers,

maxnoPRSTRPs,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

id-Cell-ID,

id-TRPInformationTypeItem,

id-SrsFrequency,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-PRS-Resource-ID,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoA,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-ARPLocationInfo,

id-ARP-ID,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRP-Rx-TEGInformation,

id-TRPBeamAntennaInformation,

id-NR-TADV,

id-pathPower,

id-SRSPortIndex,

id-UETxTimingErrorMargin,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-StartRBHopping,

id-StartRBIndex,

id-transmissionCombn8,

id-NTNNodeID

FROM NRPPA-Constants

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

NR-PRS-Beam-InformationItem ::= SEQUENCE {

pRSresourceSetID 	PRS-Resource-Set-ID,

pRSAngle 		SEQUENCE (SIZE(1..maxPRS-ResourcesPerSet)) OF PRSAngleItem,

iE-Extensions	ProtocolExtensionContainer { { NR-PRS-Beam-InformationItem-ExtIEs} } OPTIONAL,

...

}

NR-PRS-Beam-InformationItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NR-TADV ::= INTEGER (0.. 7690)

NTNNodeID	::= OCTET STRING

NumberOfAntennaPorts-EUTRA ::= ENUMERATED {

n1-or-n2,

n4,

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSResourcetype	CRITICALITY ignore EXTENSION SRSResourcetype PRESENCE optional}|

{ ID id-ARP-ID		CRITICALITY ignore EXTENSION ARP-ID 	PRESENCE optional}|

{ ID id-LoS-NLoSInformation		CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional }|

{ ID id-NTNNodeID				CRITICALITY ignore EXTENSION NTNNodeID 					PRESENCE reject },

...

}

TrpMeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOAMeasurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension							ProtocolIE-Single-Container { { TrpMeasuredResultsValue-ExtIEs } }

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-StartRBHopping												ProtocolIE-ID ::= 109

id-StartRBIndex													ProtocolIE-ID ::= 110

id-transmissionCombn8											ProtocolIE-ID ::= 111

id-NTNNodeID													ProtocolIE-ID ::= a

END

-- ASN1STOP