3GPP TSG-RAN WG3 Meeting #122	R3-237448

Chicago, USA, 13 – 17 November 2023

Agenda Item:	11.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.413) Handling of failed MBS QMC activation

Document for:	Text Proposal

# 1	Introduction

In this paper we discuss the scenario of failed MBS QMC activation.

# 2	Discussion

Independently of RAN3’s choice of UE-based or CN-based solution, we believe it is needed to consider the following scenario:

- The AMF requests configuration of one or more s-based QMC sessions during Initial Context Setup or UE Context Modification procedures intended to run when the UE is in RRC_IDLE,

- but the gNB is unable to transfer the corresponding QMC configuration container to the UE before UE context release.

In this case the AMF should be informed and hence be enabled to retransmit the full configuration information again (including the XML-encoded QMC configuration container) to the NG-RAN the next time the UE connects.

Fig. 1: Failed MBS QMC configuration, with reporting of failure information to the AMF.

Details of the impacted steps are as follows:

Step 1: The AMF sends s-based MBS QMC Configuration to the gNB for a given UE (during Initial Context Setup or using UE Context Modification procedure).

Step 2/3: The MBS QMC configuration can’t be sent to the UE before UE Context Release (e.g. due to RLF or other reason leading to short call duration).

Step 4: The gNB informs the AMF that the QMC configuration was not transferred to the UE. This is needed at least for the MBS QMC configuration, where the AMF would not retransmit the XML-encoded QMC configuration container to the gNB when the UE reconnects from RRC_IDLE.

Steps 5-8: The full MBS QMC configuration, including the XML-encoded QMC configuration container, is sent to the UE next time the UE connects to the network.

Proposal: RAN3 to discuss scenario for failed s-based QMC configuration towards the UE, and agree solution as described in Figure 1.

A simple NGAP TP for the CN-based solution can be found in annex of this paper.

# 3	Conclusion

We have made the following proposal:

Proposal: RAN3 to discuss scenario for failed s-based QMC configuration towards the UE, and agree solution as described in Figure 1.

# References

[1]		R3-235992, BLCR to TS 38.413

# Annex	- TP for BLCR to 38.413

The TP is based on [3].

<<< start of changes >>>

### 8.3.3	UE Context Release (AMF initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the AMF to order the release of the UE-associated logical NG-connection due to various reasons, e.g., completion of a transaction between the UE and the 5GC, or release of the old UE-associated logical NG-connection when the UE has initiated the establishment of a new UE-associated logical NG-connection, etc. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE context release: successful operation

The AMF initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the NG-RAN node.

The UE CONTEXT RELEASE COMMAND message shall contain both the AMF UE NGAP ID IE and the RAN UE NGAP ID IE if available, otherwise the message shall contain the AMF UE NGAP ID IE.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the NG-RAN node shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message.

If the PDU Session Resource List IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the User Location Information IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the Information on Recommended Cells and RAN Nodes for Paging IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall, if supported, store it and may use it for subsequent paging.

For each PDU session for which the Secondary RAT Usage Information IE is included in the PDU Session Resource Release Response Transfer IE, the SMF shall handle this information as specified in TS 23.502 [10].

If the Paging Assistance Data for CE Capable UE IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall, if supported, store it and use it for subsequent paging, as specified in TS 23.502 [10].

If the QMC Configuration Failure Information IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall, if supported, take it into account for QMC.

#### 8.3.3.3	Unsuccessful Operation

Not applicable.

#### 8.3.3.4	Abnormal Conditions

If the UE Context Release procedure is not initiated towards the NG-RAN node before the expiry of the timer TNGRELOCOverall, the NG-RAN node shall request the AMF to release the UE context.

If the UE returns to the NG-RAN node before the reception of the UE CONTEXT RELEASE COMMAND message or the expiry of the timer TNGRELOCOverall, the NG-RAN node shall stop the timer TNGRELOCOverall and continue to serve the UE.

<<< next change >>>

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the NG-RAN node to confirm the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| PDU Session Resource List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session Resource Release Response Transfer | O |  | OCTET STRING | Containing the PDU Session Resource Release Response Transfer IE specified in subclause 9.3.4.21. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | Ignore |
| QMC Configuration Failure Information | O |  | 9.3.1.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<<< next change >>>

#### 9.3.1.x1	QMC Configuration Failure Information

This IE contains information about QMC configurations for MBS broadcast that was received by the gNB from the AMF for this UE but were not transferred to the UE before UE context release.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Configuration Failure Information List |  | 1 |  |  |
| >QMC Configuration Failure Information Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [45]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-bytes Octet String. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of UE application layer measurements. Value is 16. |

<<< end of changes >>>