3GPP TSG-RAN WG3 Meeting #122	R3-237460

Chicago, USA, 13th – 17th November, 2023

Agenda item:	24.2

Source:	Qualcomm Incorporated

Title:	Open issues on NES techniques

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further details related to the following objective in the Rel-18 NES WID (RP-223450) based on the discussion in the last meeting:

Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3].

# Discussion

## Cell DTX/DRX

WA: Support the exchange of the Cell DTX/DRX configuration over Xn

FFS on the activation/deactivation status exchange

There were multiple options discussed last meeting on whether/what cell DTX/DRX information can be exchanged over F1/Xn and is reiterated below:

- Option 1: Exchange the cell DTX/DRX configuration as specified in RRC (i.e, the CellDTXDRX-Config). This includes the initial activation status as decided by RRC (i.e., the cellDTXDRXactivationStatus-r18)

- Option 2: Option 1 + Dynamic (de)activation status as per L1/L2 signaling.

  - This would mean the (de)activation status has to be sent from gNB-DU to gNB-CU over F1 and then over Xn each time a L1/L2 signaling is sent to the UE(s) for cell DTX/DRX (de)activation.

- Option 3: Cell DTX/DRX related probabilistic information (e.g., onDuration usage probability)

  - We don’t think a probabilistic information can help in making a deterministic decision at the receiver and is more of an assistance. For example, a receiver can’t really do much knowing that the cell was transmitting/receiving X% of the time without knowing the exact onDuration or cell DTX/DRX configuration of neighbors. For example, a receiver can’t do much knowing without knowing the neighbor cell DTX/DRX configurations and just knowing if the neighbor cell was transmitting 40% or 60% of the time.

We think at the very least, we need to support semi-static exchange of cell DTX/DRX configuration. This can be useful for different purposes so that the nodes know the neighbor’s cell DTX/DRX configurations and can decide to do interference management or coordinated scheduling. Also, this is useful in case of DC, where the cell DTX/DRX configurations between MN and SN can be coordinated.

Observation 1: The knowledge of neighbor’s cell DTX/DRX configurations (or SN’s cell DTX/DRX configuration at MN or vice-versa in case of NR-DC) can be beneficial for multiple purposes e.g., interference management, coordinated scheduling

Proposal 1:  Exchange cell DTX/DRX configuration as specified in RRC (i.e., the CellDTXDRX-Config) over Xn so that the NG-RAN node(s) can have the knowledge of neighbor cells’ cell DTX/DRX configurations (or for MN-SN coordination).

Proposal 2: There is no need to exchange dynamic cell (de)activation status as this would mean sending this (de)activation status over F1 and then over Xn each time there is a L1/L2 signaling for cell DTX/DRX activation.

Proposal 3: There is no need to exchange probabilistic information related to cell DTX/DRX (e.g., onDuration usage probability) over Xn as a receiver can’t do much knowing this information without knowing the neighbor cell DTX/DRX configurations (e.g., it can’t change its cell DTX/DRX configuration).

## Paging enhancements

RAN2 sent a reply LS to RAN3

No RAN2 impact is identified but it was understood that paging may fail as UE assumes same paging message to be sent on all beams. Regardless it is RAN2 understanding that UE behavior does not need to be changed, as the potential paging failure could be handled by NW implementation e.g., by paging on all the beams in subsequent paging.

From RAN2 reply, it can be understood that the NES benefits would be smaller if gNB doesn’t select the right set of UEs. In such a case, the initial paging attempt might fail on the UE side and the gNB would have to send another page on a wider area (e.g., the whole cell). But if the gNB selects the right set of UEs e.g., only those UE which it is confident to be stationary, then we can save network energy by paging in specific beam rather than paging blindly in the whole cell. Therefore with this understanding that gNB will select right set of UEs, we propose the following:

Proposal 4: Confirm the support for paging enhancements for RRC_INACTIVE UEs.

Regarding whether to support paging enhancements for RRC_IDLE UEs, we think the solution is very similar to RRC_IDLE and can be easily extended. We therefore also propose the following:

Proposal 5: Support paging enhancements for RRC_IDLE UEs and send reply LS to SA2 on the questions they asked.

# Conclusion

Cell DTX/DRX

Observation 1: The knowledge of neighbor’s cell DTX/DRX configurations (or SN’s cell DTX/DRX configuration at MN or vice-versa in case of NR-DC) can be beneficial for multiple purposes e.g., interference management, coordinated scheduling.

Proposal 1:  Exchange cell DTX/DRX configuration as specified in RRC (i.e., the CellDTXDRX-Config) over Xn so that the NG-RAN node(s) can have the knowledge of neighbor cells’ cell DTX/DRX configurations (or for MN-SN coordination).

Proposal 2: There is no need to exchange dynamic cell (de)activation status as this would mean sending this (de)activation status over F1 and then over Xn each time there is a L1/L2 signaling for cell DTX/DRX activation.

Proposal 3: There is no need to exchange probabilistic information related to cell DTX/DRX (e.g., onDuration usage probability) over Xn as a receiver can’t do much knowing this information without knowing the neighbor cell DTX/DRX configurations (e.g., it can’t change its cell DTX/DRX configuration).

Paging enhancements

Proposal 4: Confirm the support for paging enhancements for RRC_INACTIVE UEs.

Proposal 5: Support paging enhancements for RRC_IDLE UEs and send reply LS to SA2 on the questions they asked.

# 4. References