3GPP TSG-RAN WG3 Meeting # 122	R3-237466

Chicago, USA,  13th  - 17th November, 2023

Agenda item:	14.2

Source:	Google

Title:	(TP for L1L2Mob BLCR for TS 38.473) Reference configuration and Target Configuration ID in LTM

Document for:	Discussion & Decision

# Introduction

This paper discusses the resolution of introducing reference configuration and target configuration ID in LTM considering the CU-DU split and provides a TP to the TS 38.473 BLCR for LTM.

# Discussion

## LTM Reference Configuration

RAN3#121bis discussions and chairman’s note on reference configuration

Reference configuration: The CU decides from which node (source DU or candidate DU) to retrieve the low layer configuration for the purpose of generating the reference configuration. The source (candidate) DU generates the low layer configuration to the CU based on the request from CU, the CU does not modify it The CU generates/decides the reference configuration and sends the low layer part reference configuration (CellGroupConfig) to the candidate DUs. The DU provides either the delta config or the full low layer configuration if reference config is received from CU. The DU provides the full lower layer configuration if the reference configuration is not received from the CU

The above agreements are also captured in the LTM F1AP BLCR as below.

If the LTM Reference Configuration IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration. Editor’s note: FFS whether to reuse existing IE to signal LTM reference configuration

If the LTM Reference Configuration IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration. FFS whether to reuse existing IE to signal LTM reference configuration Editor’s note: FFS on the LTM Reference Configuration remains.

It remains in FFS whether to reuse existing IE (e.g., CellGroupConfig) to signal the LTM reference configuration.

The current specification captures only the handling of the received CG-ConfigInfo and the HandoverPreparationInformation at the gNB-DU as below. It is worth noting that the CellGroupConfig was introduced to the CU to DU RRC Information for the sake of RRC state transition from inactive to connected state and its use case was not overlapped with the mobility or DC scenario.

Observation 1:  CellGroupConfig was introduced to the CU to DU RRC Information for the sake of supporting delta configuration during RRC state transition from inactive to connected state and it was not overlapped with the mobility or DC scenario.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8]. If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE, or at least one BH RLC channel is set up for IAB-MT. If the HandoverPreparationInformation IE containing the sidelink related UE information is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8]. If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

Some points to be highlighted

- The lower layer LTM reference configuration can be also from a candidate cell (i.e., not only from source) either in intra- or inter-DU case.

- The elements in the CU to DU RRC Information IE are optional and there seems to be no restriction whether they can appear together

- CG-ConfigInfo (including optional sourceConfigSCG) has to be signaled to the gNB-DU at SN as it is DC operation and the SCG LTM is also supported in Rel-18.

- It is not clear whether the HandoverPreparationInformation (including optional/conditional sourceConfig) will be signaled to the gNB-DU at MN during LTM

- There were no explicit specification text when and how the CellGroupConfig is handled if received at the gNB-DU

Therefore, if the CellGroupConfig is reused for LTM reference configuration, some further specification texts are still needed and the examples are as follows.

UE Context Setup procedure - If the received CU to DU RRC Information IE does not include source or reference/candidate cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed. … For LTM operation, if the CellGroupConfig IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration. UE Context Modification procedure - For LTM operation, if the CellGroupConfig IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

Although both options (introducing the LTM Reference Configuration IE or reusing the CellGroupConfig in the CU to DU RRC Information) are viable, we prefer introducing the LTM Reference Configuration IE as the purpose and scenario is different from the original RRC state transition and some specification impact would be necessary anyway as shown above.

Proposal 1:  Use the LTM Reference Configuration IE as in the BLCR and remove the FFSs as in the TP.

If the Request for Lower Layer Configuration IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message and set to "true", the gNB-DU shall, if supported, provide the lower layer configuration in the CellGroupConfig IE in the UE CONTEXT SETUP RESPONSE message.

One additional issue in this topic is that whether the Request for Lower Layer Configuration IE is necessary in the UE Context Setup Request message or the UE Context Modification Request message as in the LTM preparation the gNB-DU shall always generate the lower layer configuration (i.e., CellGroupConfig) with or without the IE during the LTM preparation. If it is to be kept, it is proposed to add in the semantics in the tabular clarifying that the IE is used for “Request the lower layer configuration for generating the ltm-ReferenceConfiguration as defined in TS 38.331 [8].” Otherwise, this IE may not be needed.

Proposal 2:  Clarify in the semantics of the Request for Lower Layer Configuration IE that it is for the gNB-CU to generate the ltm-ReferenceConfiguration.

## Configuration ID in LTM Command

In LTM TS 38.300 Running CR, it is captured:

“The gNB decides to execute cell switch to a target cell, and transmits a MAC CE triggering cell switch by including the candidate configuration index of the target cell. The UE switches to the target cell and applies the configuration indicated by candidate configuration index.”

It is also captured in the LTM TS 38.321 Running CR that the LTM Cell Switch Command MAC CE contains:

“Target Configuration ID: indicates the index of candidate target configuration to apply for LTM cell switch, corresponding to [ltm-CandidateId] as specified in TS 38.331 [5]. The length of the field is 3 bits;”

As this MAC CE has to be signalled by the gNB-DU while the LTM configuration ID is assigned by the gNB-CU and included in the LTM-Candidate IE when configuring the LTM-Config to the UE as in the TS 38.331 Running CR. To enable the gNB-DU to include a correct Target Configuration ID in the LTM Command after receiving the L1L2 measurement reports from the UE, the gNB-DU needs to associate the Target Configuration ID with the Requested Target Cell ID and the (lower layer) candidate configuration.

LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId, ltm-SSB-Config-r18, LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration), OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R //skip unrelated parts ... }

We have the following agreement in RAN3#121bis but how the gNB-DU obtains the LTM configuration ID is yet to be determined.

LTM Configuration ID: The gNB-DU needs to have the LTM configuration ID from the CU to include it in the LTM cell switch command.

During LTM offline discussion, it was mentioned that the UE would report SSBRI (SS/PBCH Block Resource indicator or SSB Resource Indicator) in the L1 measurement report but not the LTM configuration ID.

Agreement in RAN1#114 (August 2023) SSBRI among configured candidate cells is included for each L1-RSRP report - The bit size of SSBRI is log2KsSSBwhere KsSSB is the number of configured SSBs in the corresponding resource set for the report - The following format is used for reporting CSI report number; CSI fields. CSI report #n; SSBRI #1 as in Table 6.3.1.1.2-6, if reported. SSBRI #2 as in Table 6.3.1.1.2-6, if reported. :. SSBRI #L*M as in Table 6.3.1.1.2-6, if reported. RSRP #1 as in Table 6.3.1.1.2-6, if reported. Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported. :. Differential RSRP #L*M as in Table 6.3.1.1.2-6, if reported.

The above agreement can be referred to TS38.212v18.0.0 and TS38.214v18.0.0, respectively, as below.

The bitwidth for CRI, SSBRI, RSRP, differential RSRP, and CapabilityIndex are provided in Table 6.3.1.1.2-6. Table 6.3.1.1.2-6: CRI, SSBRI, RSRP, and CapabilityIndex Field; Bitwidth. CRI. SSBRI. RSRP; 7. Differential RSRP; 4. CapabilityIndex; 2. where is the number of CSI-RS resources in the corresponding resource set, and is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP'.

#### 5.2.1.2 Resource settings … Each LTM CSI Resource Setting [LTM-CSI-ResourceConfig] contains configuration of a [LTM-CSI-SSB-ResourceSet] which comprises of a list of Z ≥ 1 SS/PBCH blocks indices (given by [LTM-csi-SSB-ResourceList]) and a list of Z [PCI indices] (given by [LTM-CandidateId-list]) referring to cells associated with the SS/PBCH block indices. The UE determines the time domain behavior of a SS/PBCH block from ssb-Periodicity and ssb-PositionsInBurst and the frequency domain behavior of a SS/PBCH block is determined by the higher layer parameters subcarrierspacing, ssbFrequency. ##### 5.2.1.4.1 Resource Setting configuration … If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting. - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters. …

Observation: The UE reports SSBRI in the L1 measurement report.

It was also indicated that the RAN2 RRC running CR (i.e., R2-2310885) had captured some data structure in the LTM-CSI-ResourceConfig > LTM-CSI-SSB-ResourseSet so that the gNB-DU may be able to utilize to find the association between the configuration IDs and the SSBRIs if the LTM-CSI-ResourceConfig is received and if there is a direct mapping.

In the last version of LTM running CR (R2-2310885) before Xiamen meeting, it was as below and in the offline comment the ltm-CSI-SSB-ResourceSetID-r18 was recognized as the SSBRI and there could be a mapping with a target cell during the LTM preparation as it is for one LTM candidate cell configuration.

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for an LTM candidate cell configuration.

…

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceSetId-r18             LTM-CSI-SSB-ResourceSetId-r18,

ltm-CandidateId-r18                       LTM-CandidateId-r18

ltm-CSI-SSB-ResourceList-r18              SEQUENCE (SIZE (1..maxNrofLtmCSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

}

LTM-CSI-SSB-ResourceSetId-r18 ::=           INTEGER (0..maxNrofLtmCSI-SSB-ResourceSets-r18-1)

Editor’s Note: FFS whether to implement the SSB resource sets according to what is stated in the RAN1 excel.

However, after checking internally, with the above RAN1 specification texts and agreement, the SSBRI should actually be referred to or associated with the SSB-index rather than the ltm-CSI-SSB-ResourceSetId-r18 in the RRC running CR. In the current version (R2-2311604, post meeting), it has also been updated to as below to reflect RAN1’s original design and now it is for one or more LTM candidate cell configurations.

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations

…

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceSetId-r18             LTM-CSI-SSB-ResourceSetId-r18,

ltm-CSI-SSB-ResourceList-r18              SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18                   SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18

}

LTM-CSI-SSB-ResourceSet field descriptions

ltm-CandidateIdList This field Indicates the LTM candidate cell IDs related to the SSBs in the ltm-CSI-SSB-ResourceList. The list has the same number of entries as ltm-CSI-SSB-ResourceList.

However, SSB-index values of different candidate cells can be the same, i.e., the same SSB-index value can be associated to different LTM candidate IDs. Therefore, the source gNB-DU cannot uniquely identify a LTM candidate ID according to a SSB-index/SSBRI indicated in a lower layer measurement report (i.e., CSI report). Further, for the inter-DU mobility, the source gNB-DU also needs to know the target cell from other gNB-DU and provide the NR CGI of the target cell in the LTM Cell Change Notification message to the CU and then to the other gNB-DU upon LTM execution.

Observation: The gNB-DU cannot only rely on the LTM-CSI-ResourceConfig IE to associate the LTM configuration ID and the target cell.

Therefore, it is proposed that the gNB-CU adds in the UE Context Setup/Modification Request message a LTM Configuration ID IE along with the SpCell ID IE and the LTM-CSI-ResourceConfig IE for the gNB-DU to establish the corresponding association during the LTM preparation.

Proposal 3: The gNB-CU includes a LTM Configuration ID IE in the UE Context Setup Request message and the UE Context Modification Request message.

As for the inter-gNB DU LTM, the source gNB-DU needs to be informed of the prepared LTM IDs and target Cell IDs at the other candidate gNB-DUs. Similarly, the candidate gNB-DU needs to be informed of the prepared LTM IDs and target cell IDs at the source gNB-DU. Therefore, it is proposed to add a list for adding or modifying LTM ID assignment and a list for releasing LTM ID assignment in the UE Context Modification Request message so that the gNB-CU can use the UE context modification procedure to signal the LTM ID assignment to the source/candidate gNB-DU.

Proposal 4: The gNB-CU uses the UE Context Modification procedure to the source/candidate gNB-DU to signal the LTM ID assignment.

# Conclusion

LTM Reference Configuration –

Proposal 1:  Use the LTM Reference Configuration IE as in the BLCR and remove the FFSs as in the TP.

Proposal 2:  Clarify in the semantics of the Request for Lower Layer Configuration IE that it is for the gNB-CU to generate the ltm-ReferenceConfiguration.

Configuration ID in LTM Command -

Observation: The UE reports SSBRI in the L1 measurement report.

Observation: The gNB-DU cannot only rely on the LTM-CSI-ResourceConfig IE to associate the LTM configuration ID and the target cell.

Proposal 3: The gNB-CU includes a LTM Configuration ID IE in the UE Context Setup Request message and the UE Context Modification Request message.

Proposal 4: The gNB-CU uses the UE Context Modification procedure to the source/candidate gNB-DU to signal the LTM ID assignment.

# References

[1] R3-235971, (BLCR to 38.473) Additions for L1/L2 triggered mobility, Ericsson, Huawei, Nokia, Nokia Shanghai Bell, Intel Corporation, ZTE

[2] R2-2311604, RRC running CR for LTM, Ericsson

[3] R2-2311595, 38.321 running CR for introduction of NR further mobility enhancements, Huawei, HiSilicon

[4] 3GPP TS 38.212v18.0.0

[5] 3GPP TS 38.214v18.0.0

# TP for TS 38.473 BL CR

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

<<<<<< Skip Unchanged >>>>

If the LTM Indicator IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message and set to "true", the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT SETUP RESPONSE message.

If the Request for Lower Layer Configuration IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message and set to "true", the gNB-DU shall, if supported, provide the lower layer configuration in the CellGroupConfig IE in the UE CONTEXT SETUP RESPONSE message.

If the Source gNB-DU ID is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use this information to identify the source gNB-DU.

If the CSI Resource Configuration in the LTM Information to be Setup IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig for the requested LTM candidate cell.

If the Request for RACH Configuration IE is contained in the Early Sync Information Request IE included in the UE CONTEXT SETUP REQUEST message and set to "true", the gNB-DU shall, if supported, include the RACH Configuration IE for early TA acquisition (early UL synchronisation), in the UE CONTEXT SETUP RESPONSE message.

If the Early Sync Information IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early sync information from accepted LTM candidate cell in the candidate gNB-DU.

If the LTM Reference Configuration IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

<<<<<< NEXT CHANGE >>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

<<<<<< Skip Unchanged >>>>

If the LTM Indicator IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message and set to "true", the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the CSI Resource Configuration IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig for the requested LTM candidate cell.

If the Request for Lower Layer Configuration IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message and set to "true", the gNB-DU shall, if supported, include the CellGroupConfig IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration.

If the Request for RACH Configuration IE is contained in the Early Sync Information Request IE included in the UE CONTEXT MODIFICATION REQUEST message and set to "true", the gNB-DU shall, if supported, take it into account for early TA acquisition, and shall include the RACH Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Early Sync Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the Early Sync Information List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the early sync information is from the source cell and candidate cells.

If the LTM Reference Configuration IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the LTM ID Assignment To Be Added or Modified List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, add or modify the LTM configuration IDs and configured candidate cells in the list.

If the LTM ID Assignment To Be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the LTM configuration IDs and configured candidate cells in the list.

<<<<<< NEXT CHANGE >>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction : gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,…) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| LTM Information to be Setup |  | 0..1 |  |  | YES | reject |
| >LTM indicator | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1.. maxnoofLTMCells) | Includes the ltm-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Request for Lower Layer Configuration | O |  | ENUMERATED (true, …) | Request the lower layer configuration for generating the ltm-ReferenceConfiguration as defined in TS 38.331 [8]. |  |  |
| >LTM Reference Configuration | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Source gNB-DU ID | O |  | 9.3.1.9 |  | - |  |
| >CSI Resource Configuration (FFS) | O |  | OCTET STRING | Includes the LTM-CSI-ResourceConfig IE as defined in TS 38.331 [8]. |  |  |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to “CHO-replace”. |

<<<<<< NEXT CHANGE >>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, …) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| LTM Information to be Modified (details FFS) |  | 0..1 |  |  | YES | ignore |
| >LTM indicator | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1.. maxnoofLTMCells) | Includes the ltm-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Request for Lower Layer Configuration | O |  | ENUMERATED (true, …) | Request the lower layer configuration for generating the ltm-ReferenceConfiguration as defined in TS 38.331 [8]. | - |  |
| >LTM Reference Configuration | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >CSI Resource Configuration | O |  | OCTET STRING | Includes the LTM-CSI-ResourceConfig IE as defined in TS 38.331 [8]. | - |  |
| Early Sync Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>RACH Configuration | O |  |  |  |  |  |
| >>Joint or DL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |
| >>UL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-UL-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| LTM Cells to be Released List |  | 0..1 |  |  | YES | ignore |
| >LTM Cells to be Released Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| LTM ID Assignment to be Added or Modified List |  | 0..1 |  |  |  |  |
| >LTM ID Assignment to be Added or Modified Item IEs |  | 1 .. <maxnoofLTMCells> |  |  |  |  |
| >>LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM Configuration ID | M |  | INTEGER (1.. maxnoofLTMCells) | Includes the ltm-CandidateId IE, as defined in TS 38.331 [8]. | YES | ignore |
| LTM ID Assignment to be Released List |  | 0..1 |  |  |  |  |
| >LTM ID Assignment to be Released Item IEs |  | 1 .. <maxnoofLTMCells> |  |  |  |  |
| >>LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM Configuration ID | M |  | INTEGER (1.. maxnoofLTMCells) | Includes the ltm-CandidateId IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is FFS. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

<<<<<< NEXT CHANGE >>>>

<<<<<< SKIP UNCHANGED >>>>

-- **************************************************************

--

#### -- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	TYPE LowerLayerPresenceStatusChange				PRESENCE optional	}|

{ ID id-BHChannels-ToBeSetupMod-List			CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeModified-List			CRITICALITY reject	TYPE BHChannels-ToBeModified-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeReleased-List			CRITICALITY reject	TYPE BHChannels-ToBeReleased-List					PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetupMod-List				CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeModified-List				CRITICALITY reject	TYPE SLDRBs-ToBeModified-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeReleased-List				CRITICALITY reject	TYPE SLDRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-ConditionalIntraDUMobilityInformation	CRITICALITY reject	TYPE ConditionalIntraDUMobilityInformation		PRESENCE optional}|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore	TYPE SCGIndicator										PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentTwoCarrierListInfo		CRITICALITY ignore	TYPE UplinkTxDirectCurrentTwoCarrierListInfo	PRESENCE optional	}|

{ ID id-IABConditionalRRCMessageDeliveryIndication				CRITICALITY reject	TYPE IABConditionalRRCMessageDeliveryIndication							PRESENCE optional	}|

{ ID id-F1CTransferPathNRDC							CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional }|

{ ID id-MDTPollutedMeasurementIndicator				CRITICALITY ignore	TYPE MDTPollutedMeasurementIndicator				PRESENCE optional }|

{ ID id-SCGActivationRequest						CRITICALITY ignore	TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CG-SDTQueryIndication						CRITICALITY ignore	TYPE CG-SDTQueryIndication							PRESENCE optional	}|

{ ID id-FiveG-ProSeAuthorized						CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional }|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-FiveG-ProSePC5LinkAMBR						CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-UpdatedRemoteUELocalID						CRITICALITY ignore	TYPE RemoteUELocalID				 					PRESENCE optional	}|

{ ID id-UuRLCChannelToBeSetupList					CRITICALITY reject	TYPE UuRLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelToBeModifiedList				CRITICALITY reject	TYPE UuRLCChannelToBeModifiedList					PRESENCE optional}|

{ ID id-UuRLCChannelToBeReleasedList				CRITICALITY reject	TYPE UuRLCChannelToBeReleasedList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeSetupList					CRITICALITY reject	TYPE PC5RLCChannelToBeSetupList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeModifiedList				CRITICALITY reject	TYPE PC5RLCChannelToBeModifiedList				PRESENCE optional}|

{ ID id-PC5RLCChannelToBeReleasedList				CRITICALITY reject	TYPE PC5RLCChannelToBeReleasedList				PRESENCE optional}|

{ ID id-PathSwitchConfiguration						CRITICALITY ignore	TYPE PathSwitchConfiguration				 		PRESENCE optional	}|

{ ID id-GNBDUUESliceMaximumBitRateList				CRITICALITY ignore	TYPE GNBDUUESliceMaximumBitRateList				PRESENCE optional }|

{ ID id-MulticastMBSSessionSetupList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-MulticastMBSSessionRemoveList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-UE-MulticastMRBs-ToBeSetup-atModify-List	CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-atModify-List	PRESENCE optional	}|

{ ID id-UE-MulticastMRBs-ToBeReleased-List			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-List			PRESENCE optional	}|

{ ID id-SLDRXCycleList								CRITICALITY ignore  TYPE SLDRXCycleList 										PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList		CRITICALITY ignore	TYPE MDTPLMNModificationList						PRESENCE optional }|

{ ID id-SDTBearerConfigurationQueryIndication		CRITICALITY ignore	TYPE SDTBearerConfigurationQueryIndication		PRESENCE optional }|

{ ID id-DAPS-HO-Status								CRITICALITY ignore	TYPE DAPS-HO-Status										PRESENCE optional }|

{ ID id-ServingCellMO-List							CRITICALITY ignore	TYPE ServingCellMO-List								PRESENCE optional	}|

{ ID id-UlTxDirectCurrentMoreCarrierInformation	    CRITICALITY ignore	TYPE UlTxDirectCurrentMoreCarrierInformation	PRESENCE optional	}|

{ ID id-CPACMCGInformation						CRITICALITY ignore	TYPE CPACMCGInformation									PRESENCE optional }|

{ ID id-LTMInformation-ToBeModified					CRITICALITY ignore	TYPE LTMInformation-ToBeModified					PRESENCE optional	}|

{ ID id-LTMCells-ToBeReleased-List					CRITICALITY ignore	TYPE LTMCells-ToBeReleased-List					PRESENCE optional	}|

{ ID id-LTMIDAssignment-ToBeAddedModified-List		CRITICALITY ignore	TYPE LTMIDAssignment-ToBeAddedModifiedList		PRESENCE optional	}|

{ ID id-LTMIDAssignment-ToBeReleased-List			CRITICALITY ignore	TYPE LTMIDAssignment-ToBeReleasedList		PRESENCE optional	},

...

}

SCell-ToBeSetupMod-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetupMod-ItemIEs} }

SCell-ToBeRemoved-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeRemoved-ItemIEs} }

SRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetupMod-ItemIEs} }

BHChannels-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetupMod-ItemIEs} }

DRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeModified-ItemIEs} }

BHChannels-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeModified-ItemIEs} }

SRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeReleased-ItemIEs} }

DRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeReleased-ItemIEs} }

BHChannels-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeReleased-ItemIEs} }

UE-MulticastMRBs-ToBeSetup-atModify-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeSetup-atModify-ItemIEs} }

UE-MulticastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeReleased-ItemIEs} }

SCell-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetupMod-Item			CRITICALITY ignore	TYPE SCell-ToBeSetupMod-Item			PRESENCE mandatory	},

...

}

SCell-ToBeRemoved-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeRemoved-Item			CRITICALITY ignore	TYPE SCell-ToBeRemoved-Item			PRESENCE mandatory	},

...

}

SRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE DRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeModified-Item		CRITICALITY reject	TYPE DRBs-ToBeModified-Item			PRESENCE mandatory},

...

}

SRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeReleased-Item	CRITICALITY reject	TYPE SRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

DRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeReleased-Item		CRITICALITY reject	TYPE DRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetupMod-Item		CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeModified-Item		CRITICALITY reject	TYPE BHChannels-ToBeModified-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeReleased-Item		CRITICALITY reject	TYPE BHChannels-ToBeReleased-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetupMod-ItemIEs} }

SLDRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeModified-ItemIEs} }

SLDRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeReleased-ItemIEs} }

SLDRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeModified-Item		CRITICALITY reject	TYPE SLDRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeReleased-Item		CRITICALITY reject	TYPE SLDRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeSetup-atModify-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeSetup-atModify-Item	CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-atModify-Item			PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

LTMCells-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF ProtocolIE-SingleContainer { { LTMCells-ToBeReleased-ItemIEs} }

LTMCells-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-LTMCells-ToBeReleased-Item		CRITICALITY reject	TYPE LTMCells-ToBeReleased-Item		PRESENCE mandatory},

...

}

LTMIDAssignment-ToBeAddedModified-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF ProtocolIE-SingleContainer { { LTMIDAssignment-ToBeAddedModified-ItemIEs} }

LTMIDAssignment-ToBeAddedModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-LTMIDAssignment-ToBeAddedModified-Item		CRITICALITY reject	TYPE LTMIDAssignment-ToBeAddedModified-Item		PRESENCE mandatory},

...

}

LTMIDAssignment-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF ProtocolIE-SingleContainer { { LTMIDAssignment-ToBeReleased-ItemIEs} }

LTMIDAssignment-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-LTMIDAssignment-ToBeReleased-Item		CRITICALITY reject	TYPE LTMIDAssignment-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

### 9.4.5	Information Element Definitions

#### -- L

< skip unchanged part>

LTMCells-Required-ToBeReleased-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { LTMCells-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

LTMCells-Required-ToBeReleased-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMCells-ToBeReleased-Item ::= SEQUENCE {

nRCGI				NRCGI,

lTMConfigurationID	LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer { { LTMCells-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

LTMCells-ToBeReleased-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMInformation-ToBeSetup ::= SEQUENCE {

lTMIndicator						LTMIndicator										OPTIONAL,

lTMConfigurationID					LTMConfigurationID,

requestforLowerLayerConfiguration 	RequestforLowerLayerConfiguration					OPTIONAL,

lTMReferenceConfiguration		LTMReferenceConfiguration							OPTIONAL,

sourceGNB-DU-ID 					GNB-DU-ID											OPTIONAL,

cSIResourceConfiguration (FFS)		CSIResourceConfiguration							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-ToBeSetup-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-ToBeSetup-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMInformation-ToBeModified (FFS)::= SEQUENCE {

lTMIndicator						LTMIndicator											OPTIONAL,

lTMConfigurationID					LTMConfigurationID,

requestforLowerLayerConfiguration	RequestforLowerLayerConfiguration						OPTIONAL,

lTMReferenceConfiguration		LTMReferenceConfiguration								OPTIONAL,

cSIResourceConfiguration 			CSIResourceConfiguration								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-ToBeModified-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-ToBeModified-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMIndicator ::= ENUMERATED {true, ...}

LTMRequestforRACHConfiguration ::= ENUMERATED {true, ...}

LTMRACHConfiguration ::= {} (FFS)

LTMLowerLayerConfig ::= SEQUENCE {

lTMRACHConfiguration 			LTMRACHConfiguration 				OPTIONAL,

rSConfig 						RSConfig							OPTIONAL,

tCIStateConfig 					TCIStateConfig						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { LTMLowerLayerConfig-ExtIEs} }	OPTIONAL

}

LTMLowerLayerConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

LTMLowerLayerConfig-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTMLowerLayerConfig-Item

LTMLowerLayerConfig-Item ::= SEQUENCE {

nRCGI				NRCGI,

lTMLowerLayerConfig	LTMLowerLayerConfig					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMLowerLayerConfig-Item-ExtIEs } }	OPTIONAL,

...

}

LTMLowerLayerConfig-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMLowerLayerConfig-Mod-Resp-Item::= SEQUENCE {

nRCGI				NRCGI,

rSConfig			RSConfig					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMLowerLayerConfig-Mod-Resp-Item-ExtIEs } }	OPTIONAL,

...

}

LTMLowerLayerConfig-Mod-Resp-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMIDAssignment-ToBeAddedModified-Item ::= SEQUENCE {

LTMCellID							NRCGI,

lTMConfigurationID					LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer { { LTMIDAssignment-ToBeAddedModified-ExtIEs} }	OPTIONAL

}

LTMIDAssignment-ToBeAddedModified-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMIDAssignment-ToBeReleased-Item ::= SEQUENCE {

LTMCellID							NRCGI,

lTMConfigurationID					LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer { { LTMIDAssignment-ToBeReleased-ExtIEs} }	OPTIONAL

}

LTMIDAssignment-ToBeReleased-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<< NEXT CHANGE, SKIP UNCHANGED >>>>

LTMConfigurationID	 ::= INTEGER (1.. maxnoofLTMCells)

<<<<<< END OF CHANGE >>>>