3GPP TSG-RAN3 Meeting #122	R3-237523

# Chicago, USA, 13th November – 17th November 2023

Agenda Item:	8.1

Source:	Ericsson

Title:		Support of Sub One Second Report Interval following CT4 LS

Document for:	Discussion

# Introduction

This paper describes the impacts on NRPPa following the LS from CT4 in [1].

# Discussion

The LS from CT4 states the following:

1. Overall Description: CT4 has agreed following CRs from Rel-17 at CT4#115e, which specified support of periodic deferred location reporting with interval less than 1 second: - GMLC SBI interface (CR0102 of TS 29.515) - AMF SBI interface (CR0892 of TS 29.518) - LMF SBI interface (CR0165 of TS 29.572) - MAP interface (CR1267 of TS 29.002). The CRs were approved at CT#100 and implemented in 3GPP June 2023 public release. The agreed CRs may have LPP and/or NRPPA impacts. 2. Actions: To RAN WG2 and RAN WG3 group: ACTION: CT4 kindly asks RAN2 and RAN3 to take the above information into account and proceed to update the related specifications if needed.

The following CRs have been agreed in CT4, where we can find more details on the background:

| WG TDoc | WG decisn | TSG decisn | Spec | CR | rev | cat | Rel | init vers | Title | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C4-231503 | agreed | approved | 29.515 | 0102 | 1 | F | Rel-17 | 17.6.0 | Missing finer periodicities than 1s and an infinite reporting amount | Ngmlc update |
| C4-231504 | agreed | approved | 29.515 | 0103 | 1 | A | Rel-18 | 18.1.0 | Missing finer periodicities than 1s and an infinite reporting amount | Ngmlc update |
| C4-231505 | agreed | approved | 29.518 | 0892 | 1 | F | Rel-17 | 17.9.0 | Missing finer periodicities than 1s and an infinite reporting amount | Namf update |
| C4-231506 | agreed | approved | 29.518 | 0893 | 1 | A | Rel-18 | 18.1.0 | Missing finer periodicities than 1s and an infinite reporting amount | Namf update |
| C4-231507 | agreed | approved | 29.572 | 0165 | 1 | F | Rel-17 | 17.7.0 | Missing finer periodicities than 1s and an infinite reporting amount | Nlmf update |
| C4-231508 | agreed | approved | 29.572 | 0166 | 1 | A | Rel-18 | 18.1.0 | Missing finer periodicities than 1s and an infinite reporting amount | Nlmf update |
| C4-232529 | agreed | approved | 29.002 | 1267 | 1 | F | Rel-17 | 17.2.0 | Missing finer periodicities than 1s and an infinite reporting amount | Mobile Application Part (MAP) update |

The purpose of these CRs from CT4 is to reflect the changes introduced by RAN2 and RAN3 in LPP and NRPPa for Rel-17 Latency improvement to have finer than 1s configuration of timestamps and scheduled location time.

In fact, in NRPPa it is already possible to configure reporting intervals finer than 1 second. For example, in multi-RTT, a LMF can indicate the measurement unit less than a second in the Response Time IE:

### 9.2.68	Response Time

This information element contains the response time of the measurement results reporting.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time | M |  | INTEGER(1..128,…) |  |
| Time Unit | M |  | ENUMERATED (second, ten-seconds, ten-milliseconds, …) |  |

Observation 1: NRPPa already supports measurement report with response time finer than 1s, as well as Nnef, Nlmf, Ngmlc and the Mobile Application Part (MAP)

Moreover, CT4 notices there is a misalignment between CT4 and with LPP and NRPPa where a UE can be configured to report an infinite amount of location information, while GMLC is limited in its reporting amount.

Especially when smaller interval (less than 1 second) is requested, more than 8639999 reports can be generated during the allowed duration (8639999 seconds). This was addressed by CT4 in their specs by adding a new ReportingInfiniteInd IE as described below:

##### 6.1.6.2.24	Type: PeriodicEventInfo

Table 6.1.6.2.24-1: Definition of type PeriodicEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reportingAmount | ReportingAmount | M | 1 | Number of event reports |
| reportingInterval | ReportingInterval | M | 1 | Interval of event reports |
| reportingInfiniteInd | boolean | O | 0..1 | When present, this IE shall be set to the value true, indicating that as many reports as possible to be generated during allowed duration (8639999 seconds). When this IE is present in a request, the reportingAmount should be set to the largest possible value (see NOTE) for backward compatibility consideration. An LMF supporting this IE shall ignore the reportingAmount IE and an LMF not supporting this IE will use reportingAmount IE to generate indicated number of reports as legacy. When this IE is present in a response, it indicates that this IE is supported and accepted by the LMF. |
| reportingIntervalMs | ReportingIntervalMs | O | 0..1 | When present, this IE shall indicate the Interval of event reports in milliseconds. When this IE is present in a request, the reportingInterval is set ot 1 for backward compatible consideration. An LMF supporting this IE shall ignore the reportingInterval IE; an LMF not supporting this IE will use reportingInterval IE to generate report with minimal interval as legacy, i.e. 1 second. When this IE is present in a response, it indicates that this IE is supported by the LMF and it shall indicate the value accepted by the LMF. |
| NOTE: reportingAmount x reportingInterval shall not exceed 8639999 (99 days, 23 hours, 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP. |  |  |  |  |

In RAN3 specs however, we notice that a UE can be configured to report an infinite number of periodic reporting when the Reporting Amount IE is set to "0", but the Reporting Interval is on seconds. CT4 have added a new IE of reporting interval in ms:

| ReportingIntervalMs | integer | Event reporting periodic interval in milliseconds. Minimum: 1. Maximum: 999. |
| --- | --- | --- |

### 9.2.70	UE Reporting Information

This IE contains the UE Reporting Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Amount | M |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | Value 0 represents an infinite number of periodic reporting |
| Reporting Interval | M |  | ENUMERATED (none, 1, 2, 4, 8, 10, 16, 20, 32, 64) | Unit: seconds |

Observation 2: In NRPPa reporting amount for a configured UE already supports infinite reporting, but not sub-ms reporting Interval.

To align with CT4, it is needed to add a new IE Reporting Interval in Milliseconds to the UE Reporting Information

| Reporting Interval in Milliseconds | O |  | INTEGER (1..999) | Unit: milliseconds |
| --- | --- | --- | --- | --- |

Proposal 1: To align with CT4, the UE configured Reporting Interval should be extended to milliseconds values.

This change affects NRPPa only. A CR to NRPPa is provided in [2].

# Conclusion

Observation 1: NRPPa already supports measurement report with response time finer than 1s, as well as Nnef, Nlmf, Ngmlc and the Mobile Application Part (MAP)

Observation 2: In NRPPa reporting amount for a configured UE already supports infinite reporting, but not sub-ms reporting Interval.

Proposal 1: To align with CT4, the UE configured Reporting Interval should be extended to milliseconds values.

# References

- R3-237135 LS on Sub One Second Report Period for Deferred Location over SBI

- R3-237524, CR to NRPPa on addition of milliseconds reporting interval