3GPP TSG-RAN3 Meeting #122	R3-237536

# Chicago, USA, November 13 – 17, 2023

Agenda item:	23.2.1

Source:	Ericsson, Xiaomi, Samsung

Title:	(TP to TS 38.423) Clarification on Ranging and Sidelink Positioning Service Information

Document for:	Other

# 1	Introduction

This TP is to clarify the relationship between RSPP and V2X and Prose.

According to SA2’s specification in TS 23.586, the Ranging/SL positioning service information can only be used at least one of the services for V2X and ProSe is authorized, it is proposed to add semantics descriptions i.e. “This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services.” in the Ranging and Sidelink Positioning Service Information to clarify the relationship between Ranging/Sidelink Positioning Service and V2X/ProSe service.

# 2	TP for TS 38.423 BL CR Ranging and Sidelink Positioning Service Information

----------------------------------------------<Start of change>----------------------------------------------

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| <<<<<omit the unchanged parts>>>>> |  |  |  |  |  |  |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.x1 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| <<<<<omit the unchanged parts>>>>> |  |  |  |  |  |  |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.x1 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |