3GPP TSG-RAN WG3 Meeting #122	R3-237574

Chicago, USA, 13 – 17 November, 2023

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion related to the incoming LS from SA2 on the support of AI/ML for air interface and NG-RAN in RAN

Document for:	Discussion and Decision

# 1	Introduction

SA2 sent an LS to RAN groups  (RAN, RAN1, RAN2 and RAN3) to request feedback on whether there is any requirement for SA2 to support AI/ML for air interface and NG-RAN in RAN [1]. This request was in the context of a Rel.19 draft “SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)” which has been under discussion in working task (WT) #1 (S2-2310034).

The objective of WT #1 relevant to RAN3 is WT1.4:

-	WT#1: AI/ML cross-domain coordination aspects

-	Study enhancements to support AI enabled RAN based on conclusions of the RAN study. The WT will discuss whether and how to support the cross domain (i.e. UE, RAN, 5GC, OAM and AF) collaborative AI/ML mechanisms to support the UE, the RAN, the 5GC and the AF for the aspects described by the work tasks below. The WT will also discuss interaction/coordination with RAN to support the AI enabled RAN framework:

-	WT1.1 – Study enhancements to UE data collection framework. Study whether and how to enhance UE data collection framework to meet requirements for RAN AI support for air interface operation (for RAN). This includes identifying what benefit can be achieved from enhanced UE data collection for 5GC, and the potential impacts on the 5G framework, including potential enhancements to policy control. Regarding the radio related data collected from UE or RAN, e.g, channel status information and beam information, the WT will also discuss the data leakage from the operator's domain which should be avoided.

-	WT1.2 – Study 5GC support for AI/ML model and information sharing with the UE. Study whether (and how) to support model transfer/delivery to the UE according to RAN1/RAN2 considerations, including potential enhancements to policy control. Whether and what entities or functions transfer the AI/ML model or information to the UE will be studied as part of the work. This WT will also discuss the data leakage from the operator's domain which should be avoided.

-	WT1.3: Study whether and how to support the alignment of model identification and model management between SA2 and RAN. Work will be based on the possible requirements defined by RAN1 and RAN2.

-	WT1.4: Study whether and how to support interaction/coordination with RAN3 to support the AI enabled NG-RAN framework (i.e. AI/ML for NG-RAN in Rel-18). Work will be based on possible requirements from RAN3.

-	WT1.5: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning.

NOTE A: The work will not modify the architectural principle that a service-based architecture only applies for 5GC.

NOTE B: Whether SA2 will study WT1 and the content of WT1 will depend on and follow RAN study and conclusions. WT1 and associated TUs will be revised to align to RAN study conclusions, when RAN reaches such conclusions.

NOTE C: Further alignment with SA5 for the AI/ML Functional framework may be required.

NOTE D: security aspects are in the scope of SA3, however architectural aspects related to security enhancements will be discussed in this WT.

NOTE E: The model management will follow the framework as defined by RAN.

In this contribution, we provide our views to the LS from [1].

# 2	Discussion

RAN3 has been working on the AI/ML for NG-RAN work item in Rel.18. The current work assumes that no changes to the current architecture are needed. RAN3 agreed in Rel.18 that the existing MDT framework is used as baseline for data collection from the UE. In addition the current work focuses on RAN aspects and no agreement with impact on the core network was taken. Rel.19 discussions for a possible Rel.19 study or work item are still under discussion in RAN. Therefore, RAN3 has today no requirements for SA2 with respect to supporting AI/ML for NG-RAN at this point.

Proposal: RAN3 would like to inform SA2 that RAN3 has not identified any requirements towards SA2 with respect to supporting AI/ML for NG-RAN.

We have submitted a draft response LS to SA2 to this meeting in [3].

# 3	Conclusion

In this contribution, we make the following proposal:

Proposal: RAN3 would like to inform SA2 that RAN3 has not identified any requirements towards SA2 with respect to supporting AI/ML for NG-RAN.

# References

[1]	S2-2311921 - R3-237155, LS on AI/ML Core Network Enhancements

# [2]	S2-2310034, New SID on Artificial Intelligence (AI)/Machine Learning (ML) for 5GS Service to enable 5GC and Air interface Intelligence

# [3]	R3-237575, [Draft] Response LS to SA2 LS on AI/ML Core Network enhancements