3GPP TSG-RAN WG3 Meeting #122	R3-237576

Chicago, USA, 13 – 17 November, 2023

Agenda item:	12.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.300) Stage 2 Updates

Document for:	Text Proposal

# 1	Introduction

In RAN3 #121 we agreed the following with respect to UE Trajectory Information:

The Measured UE Trajectory reporting is one-time report.

The conditions triggering the one-time reporting are at least:

- Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE);

- Condition B: UE leaves the target node;

- Condition C: Expiration of the report time duration;

- Condition D: The configured number of hand-overs within the same target node is reached.

Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message.

We also captured the following open point:

If the UE moves to Idle/Inactive or it hands over to another cell during the Reporting Duration, collection of UE Performance Feedback is terminated. Whether an explicit or implicit indication on the latter events or any other events is signalled to the requesting node needs further discussions.

In this contribution, we proceed to capture Stage 2 text related to agreements on UE Performance measurement collection termination and UE Trajectory reporting termination.

# 2	Discussion

In this contribution, we provide stage 2 text to earlier agreements related to the UE Performance measurement collection termination and related to UE Trajectory reporting termination.

Proposal: Discuss and agree the TP for BL CR to TS 38.300 provided in the Annex.

# 3	Conclusion

In this contribution we make the following proposal:

Proposal: Discuss and agree the TP for BL CR to TS 38.300 provided in the Annex.

# References

[1]		R3-235953, Draft CR to 38.300 on AI/ML for NG-RAN

# Annex	- TP to BL CR for TS 38.300

This TP is based on [1].

<<< start of changes >>>

## X.X	AI/ML for NG-RAN

### X.X.1	General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

### X.X.2	Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

Signalling procedures used for the exchange of AI/ML related information are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of the AI/ML function does not apply to ng-eNBs.

For the deployments of RAN intelligence, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

The predicted Resource Status Information, measured UE trajectory as well as the UE performance feedback can be configured to be reported and used for all use cases. The reporting is configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory as well as UE performance feedback is triggered via the Handover Preparation procedure.

The reporting of UE Trajectory for a handed over UE is terminated and is reported in one-shot when any of the following conditions is satisfied:

- UE transits to RRC_IDLE or RRC_INACTIVE;

- UE is handed over to a different NG-RAN node;

- Expiration of Reporting Time Duration configured for the measurement;

- Number of handovers configured for the measurement is equal to the number of times a UE has been handed over within the reporting NG-RAN node.

The collection of measured UE performance feedback for a given UE is terminated when any of the following conditions is satisfied:

- UE transits to RRC_IDLE or RRC_INACTIVE;

- UE is handed over to a different NG-RAN node.

Cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Request message to provide information for e.g. subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the next one hop target NG-RAN node.

The metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced and can be exchanged with the neighbouring NG-RAN nodes upon request.

<<< end of changes >>>