3GPP TSG-RAN WG3 Meeting #122	R3-237645

Chicago, IL, USA, November 13th – 17th, 2023

Agenda item:	14.2

Source: 	LG Electronics Inc.

Title: 	Almost complete discussions on LTM (TPs for TS 38.473 and TS 38.401)

Document for:	Discussion and Decision

# Introduction

For LTM, RAN3-121bis has progressed as follows [1]:

Reference configuration

The CU decides from which node (source DU or candidate DU) to retrieve the low layer configuration for the purpose of generating the reference configuration.

The source (candidate) DU generates the low layer configuration to the CU based on the request from CU, the CU does not modify it

The CU generates/decides the reference configuration and sends the low layer part reference configuration (CellGroupConfig) to the candidate DUs.

The DU provides either the delta config or the full low layer configuration if reference config is received from CU.

The DU provides the full lower layer configuration if the reference configuration is not received from the CU

LTM Configuration ID

The gNB-DU needs to have the LTM configuration ID from the CU to include it in the LTM cell switch command.

Whether F1AP signaling needed is to be continued

RS Configuration and the signaling

Option 2, UE associated signaling (UE Context Setup/Modification) is adopted to retrieve the RS configuration from the gNB-DU.

No need for the gNB-CU to explicitly request the candidate(source) gNB-DU to provide the RS configuration during context setup/modification.

FFS on whether the RS configuration can be retrieved before step2.

TCI state configuration

WA: No need for the gNB-CU to explicitly request the candidate (source) gNB-DU to provide the TCI state configuration.

Selected beam transfer

Option 3, class 1 procedure is ruled out.

The selected beam information is the TCI State ID.

Option 1: Two new class 2 procedures with different name.

Option 2: same new class 2 procedure (LTM cell switch notification)

Down selection between option 1 and 2 at next meeting.

TA acquisition

CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU.

Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

The following information are agreed to be carried in the TA info transfer message:

- Source DU ID

- TA value

- Preamble index,

- Candidate cell ID.

- RO information (i.e. RA-RNTI)

To be continued on:

- one or two messages.

- Dedicated message name for dedicated purpose or Generic message name

- Whether and when the source DU forwards the valid TA values to target DU via CU for subsequent LTM

UE based TA measurement

For intra-DU LTM, if the gNB-CU decides to use UE based TA measurement, it may explicitly request the source gNB-DU to provide the configuration for UE based TA measurement via UE Context Modification Request message.

To be continued. FFS on the details of the UE based TA measurement configuration.

Execution of L1/L2 triggered mobility cell switch

RAN3 to move forward with approach 1.

LTM and legacy HO coexistence

In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

SCG Release

To be continued in next meeting.

Proposals to F1AP

Enhance the TCI state configuration IE to a list for one candidate cell in both UE Context Setup response message and UE Context Modification request message.

Add TCI State ID IE for each TCI state in both UE Context Setup request message and UE Context Modification request message.

Further checking is needed.

In this contribution, we discuss on some (most of) remaining issues and provide the corresponding TPs.

# Discussion

## LTM Configuration ID

This is related to the following:

LTM Configuration ID

The gNB-DU needs to have the LTM configuration ID from the CU to include it in the LTM cell switch command.

Whether F1AP signaling needed is to be continued

The purpose of LTM Configuration ID is to identify each prepared LTM candidate cell configuration with the UE. As we already agreed, it should be assigned by CU and configured to each DU to be included in its LTM cell switch command (MAC CE). According to RAN2’s running CR for TS 38.331 [2], its value range is the same as the maximum number of LTM candidate cell configurations (i.e. maxNrofCellsLTM-r18 = 8).

Currently in the F1AP BL CR [3], the UE CTXT MOD REQ is missing to include the list of all the admitted candidate cells for each DU to execute LTM accordingly. This should be provided via UE CTXT MOD REQ > LTM Information to be Modified IE. Together with this, the corresponding LTM Configuration ID assigned for each prepared LTM candidate configuration should also be provided.

While UE CTXT MOD REQ is the baseline, especially toward the S-DU, we think there is no need to use UE CTXT MOD procedure. After LTM preparation, the class-2 DL RRC MESSAGE TRANSFER is anyway sent to the S-DU which carries LTM RRC configuration to the UE. Toward the S-DU, this can be enhanced to configure the list of all the admitted candidate cells and their LTM Configuration IDs.

Proposal 1: After LTM preparation, CU should configure the list of all the admitted candidate cells to all the involved DUs in order for each DU to execute LTM accordingly, together with the corresponding LTM Configuration ID assigned by CU. For such configuration,

- To S-DU, enhance DL RRC MESSAGE TRANSFER.

- To C-DU, enhance UE CTXT MOD REQ.

## RS configuration and the signalling

This is related to the following:

RS Configuration and the signaling

Option 2, UE associated signaling(UE Context Setup/Modification) is adopted to retrieve the RS configuration from the gNB-DU.

No need for the gNB-CU to explicitly request the candidate(source) gNB-DU to provide the RS configuration during context setup/modification.

FFS on whether the RS configuration can be retrieved before step2.

Regarding FFS, we understand the intention to reduce latency for Inter-DU LTM preparation phase, but overall we think it is not appropriate to retrieve RS resource configuration of candidate cells before admission control with DUs. Before admission control, CU doesn't know which candidate cells (among which CU initiates preparation toward DUs) will be admitted by DUs and properly prepared for LTM. If this is allowed, then CU may end up sending RS resource configurations of non-admitted cells in the UE CTXT SETUP REQ to a C-DU, and the generated CSI reporting configuration in the UE CTXT SETUP RESP may include MR for non-admitted cells toward the UE. Additional round-trip with all the DUs may be required (i.e. the update of CellGroupConfig in order to fix the CSI reporting configuration within), so losing the original intention of reducing preparation latency.

Proposal 2: Do not allow CU to retrieve the RS resource configuration of candidate cells before admission control with DUs, which may necessitate additional round-trip with all the DUs (to update CellGroupConfig in order to fix the CSI reporting configuration within) and thus lose the original intention of reducing Inter-DU LTM preparation latency.

- Remove the placeholder “CSI Resource Configuration (FFS)” IE in UE CTXT SETUP REQ > LTM Information to be Setup IE in F1AP BL CR [3].

- Revert the steps 7-8 in inter-DU LTM flow in TS 38.401 BL CR [4] back to mandatory.

## Selected beam transfer

This is related to the following:

Selected beam transfer

Option 3, class 1 procedure is ruled out.

The selected beam information is the TCI State ID.

Option 1: Two new class 2 procedures with different name.

Option 2: same new class 2 procedure (LTM cell switch notification)

Down selection between option 1 and 2 at next meeting.

We prefer Option 2. There seems no strong need to separate UL and DL given that the main usage of this UE-associated class-2 message (LTM Cell Switch Notification) is to convey information from the current serving DU to CU (also to the target DU in case of inter-DU LTM). It is not expected that upstream and downstream are evolved drastically in a different way.

Proposal 3: For selected beam transfer, Option 2 is preferred, given that the main usage of this UE-associated class-2 message (LTM Cell Switch Notification) is to convey information from the current serving DU to CU (also to the target DU in case of inter-DU LTM) and it is not expected that upstream and downstream are evolved drastically in a different way.

## TA acquisition

Regarding the following:

TA acquisition

CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU.

Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

The following information are agreed to be carried in the TA info transfer message:

- Source DU ID

- TA value

- Preamble index,

- Candidate cell ID.

- RO information (i.e. RA-RNTI)

To be continued on:

- one or two messages.

- Dedicated message name for dedicated purpose or Generic message name

- Whether and when the source DU forwards the valid TA values to target DU via CU for subsequent LTM

First, we prefer one message. As similarly to the selected beam transfer discussed above, the main usage of this non-UE-associated class-2 message is to convey information from one DU to another DU via CU, and it is not expected that upstream and downstream are evolved drastically in a different way.

We also prefer the current name “TA Information Notify”, which sounds generic enough to serve any purpose related to the notification of TA information.

Proposal 4: For TA acquisition, the current class-2 message name “TA Information Notify” sounds generic enough to serve any purpose related to the notification of TA information. Also it is not preferred to separate this class-2 into UL/DL as they are not expected to evolve drastically different.

With respect to forwarding valid TA values, we think it is beneficial to share the already known/obtained TA values of some candidate cells between the involved DUs. The current progress assumes that the UL TA values for some candidate cells obtained through the early TA acquisition procedure can only be used by the current serving DU who triggered that procedure, and cannot be re-used by other serving DU(s) in the subsequent LTM operations. For example, even though the S-DU obtained valid UL TA values of all the candidate cells lying in other C-DUs and they are still valid upon serving cell change to one of them (new serving DU), currently there is no other option but for the new serving DU to trigger the early TA acquisition procedures again to obtain those UL TA values, which is costly and inefficient.

As a result, we think that, upon successful serving cell change across DUs, CU who receives ACCESS SUCCESS from the new serving DU should be able to request the previous serving DU to forward the stored valid UL TA values to the new serving DU.

In fact, the forwarding part (from the previous serving DU to the new serving DU) looks possible to leverage the existing TA Information Notify mechanism and no stage-3 change is expected. The CU’s request part (triggering the pervious serving DU to forward to new serving DU) requires additional changes, but we think this can be worked out by enhancing the same TA Information Notify signalling.

Proposal 5: In case of successful serving cell change across DUs, allow CU (upon receiving ACCESS SUCCESS from the new serving DU) to request the previous serving DU to forward the stored valid UL TA values to the new serving DU.

- With respect to forwarding the stored valid UL TA values from the previous serving DU to the new serving DU, re-use the existing TA Information Notify mechanism (no stage-3 change).

- With respect to CU’s triggering the pervious serving DU to forward to the new serving DU, enhance the TA Information Notify message.

One last thing regarding TA acquisition is about the following agreements made in the last meeting:

CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU.

Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

The part of requesting CFRA resources to a C-DU has been already reflected into F1AP BL CR [3] through the Early Sync Information Request IE in UE CTXT SETUP/MOD REQ messages. But the source DU ID is missing in UE CTXT MOD REQ, while in UE CTXT SETUP REQ, it is wrongly placed under the LTM Information to be Setup IE. The right place should be under the Early Sync Information Request IE in both UE CTXT SETUP/MOD REQ messages.

Moreover, we think that the “Source DU ID” should be a list, rather than a single ID. The above agreements were achieved for a C-DU to allocate/reserve different CFRA resources that can be used by other DUs, which may include S-DU as well as other C-DU. For example, if inter-DU LTM is to be prepared with S-DU and two C-DUs, then CFRA resources of a specific candidate cell in a C-DU may have to be allocated differently for the usage by S-DU and by the other C-DU. In this case, two different CFRA resources (with S-DU ID and other C-DU ID) shall be requested.

Proposal 6: Fix the mis-placed/missing “Source DU ID” to be under the Early Sync Information Request IE in both UE CTXT SETUP/MOD REQ messages, and re-define it as a list, so that CU can request allocating non-conflicting CFRA resources of a specific cell in a C-DU to be used by all the other DUs (i.e. more than one – including S-DU and other C-DUs).

## UE based TA measurement

Regarding the following:

UE based TA measurement

For intra-DU LTM, if the gNB-CU decides to use UE based TA measurement, it may explicitly request the source gNB-DU to provide the configuration for UE based TA measurement via UE Context Modification Request message.

To be continued. FFS on the details of the UE based TA measurement configuration.

It was agreed that a UE (who is capable of UE based TA measurement) may be configured by NW to perform UE based TA measurement from the current serving cell toward a candidate cell, where the UE derives TA based on Rx timing difference between the current serving cell and the candidate cell, together with the current TA value (of the current serving cell).

If a UE is configured such for a candidate cell, it was also agreed that the UE applies the measured TA value and performs RACH-less LTM upon serving cell switch to that candidate cell. In this case, it is expected that early TA acquisition for such candidate cells is not triggered from the current serving DU for the purpose of RACH-less LTM. However, currently during LTM preparation, the serving DU has no idea whether the UE has been configured for UE based TA measurement or not and for which candidate cells.

Of course, during preparation, a DU may be requested by CU to prepare the lower-layer configuration for UE based TA measurement per candidate cell basis (e.g. reference signal configuration to measure the received signal time difference between the current serving cell and candidate cell). But such lower-layer configuration request from CU cannot be translated that the UE shall always be configured so – whether to configure the UE based TA measurement and for which candidate cells are totally up to CU to decide.

Moreover, as serving cell changes, depending on in which cell the UE is currently served, CU may decide to reconfigure the UE based TA measurement per candidate cell basis. For example, once a serving cell is changed from S-DU to one of candidate cells in a C-DU, CU may reconfigure UE based TA measurement for some of other candidate cells in that C-DU (e.g. those cells are collocated to the current new serving cell and all operated in FR1 that could result in the UE’s DL TAE between the serving cell and candidate cell within 260ns according to RAN4’s agreement [5]). In this case, early TA acquisition is unnecessary for those candidate cells in that C-DU, for which currently there is no mechanism for CU to inform this to a DU.

Based on the above reasonings, we propose the following:

Proposal 7: For UE-based TA measurement, allow CU to request the lower-layer configuration from DU for UE based TA measurement per candidate cell basis during LTM preparation (e.g. reference signal configuration to measure the received signal time difference between the current serving cell and candidate cell).

- Enhance the UE CTXT SETUP/MOD REQ to include such request.

Proposal 8: Once CU decides to configure the UE based TA measurement and for which candidate cells, after LTM RRC configuration to the UE, CU notifies the current serving DU which candidate cells are applicable for UE-based TA measurement via UE CTXT MOD REQ:

- If the UE based TA measurement is configured by RRC (pending RAN2), the current serving DU, once notified by CU of which candidate cells, may skip early TA acquisition for those candidate cells.

- If the UE based TA measurement is configured by LTM cell switch command to the UE (pending RAN2), the current serving DU, once notified by CU of which candidate cells, may not only skip early TA acquisition for those candidate cells, but also configure via LTM cell switch command when such cell is triggered for mobility.

## Others (corrections/editorials/clean-up)

- The terminology, L1/L2 triggered mobility, is still used in some F1AP procedural decriptions. Replace by the agreed abbreviation of “LTM”.

- In Stage-2 TS 38.401 BL CR [4], the Inter-DU LTM call flow doesn’t provide any context on how candidate cells are parallely prepared with S-DU. The steps 5-6 with S-DU is basically for the post information-sharing process to deliver collected RS configuration, TCI state configuration and RACH configuration after all the candidate cells are prepared across DUs.

- In the Inter-DU LTM call flow in [4], we have the following FFS for Steps 7-8:

FFS on step 7 and 8 on whether should be parallel or single.

Both could be made to work but “single” requires to enhance the UE CTXT MOD RESP message to carry the list of updated CellGroupConfig for the candidate cells admitted by C-DU. On the other hand, “parallel” just requires some stage-2 update. Considering this is the last meeting, we propose to go with “parallel”.

- In UE CTXT MOD REQ, the reference of the CSI Resource Configuration IE should be fixed to LTM-CSI-ResourceConfigToAddModList. According to RAN2 [2], the current LTM-CSI-ResourceConfig reference contains a ground of one or more CSI resources for “one” candidate cell configuration. To give the information related to CSI resources for all the prepared candidate cell configurations by one-shot, it should be the upper-level IE, which is LTM-CSI-ResourceConfigToAddModList.

| LTM Information to be Modified (details FFS) |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM indicator | O |  | ENUMERATED (true, …) |  | - |  |
| >Request for Lower Layer Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM Reference Configuration (FFS) | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >CSI Resource Configuration | O |  | OCTET STRING | Includes the LTM-CSI-ResourceConfig IE as defined in TS 38.331 [8]. | - |  |

- ACCESS SUCCESS: While no one disputes on the usage of ACCESS SUCCES, but currently it is described for conditional mobility with auto-cancellataion (as originally designed). On the other hand, LTM continues after execution. So, update is necessary.

- LTM CELL CHANGE NOTIFICATION: The procedural descriptions is too detailed and not accurate.

- TA INFORMATION NOTIFY: The procedural descriptions is decribed from C-DU to S-DU, but considering subsequent LTM, it could be from any DU to any other DU.

Proposal 9: Fix/update the following:

- Replace “L1/L2 triggered mobility” by “LTM” in F1AP descriptions.

- For Inter-DU LTM call flow, add a note that candidate cells preparation with S-DU follows the Steps 3-4 of Intra-DU LTM call flow before Step 5.

- Regarding FFS on Step 7-8 whether “parallel” or “single” in Inter-DU LTM flow, RAN3 to go with “parallel” (doesn’t require major stage-3 changes) and update stage-2 accordingly.

- Fix the CSI Resource Configuration IE reference in UE CTXT MOD REQ to LTM-CSI-ResourceConfigToAddModList.

- ACCESS SUCCESS: Update the F1AP description, tailored for LTM.

- LTM CELL CHANGE NOTIFICATION: Fix the F1AP desciprtion and make it more generic.

- TA INFORMATION NOTIFY: Fix the F1AP description from any DU to any other DU (not just from C-DU to S-DU).

# Conclusion

Based on the discussion in the present contribution we propose:

LTM Configuration ID

Proposal 1: After LTM preparation, CU should configure the list of all the admitted candidate cells to all the involved DUs in order for each DU to execute LTM accordingly, together with the corresponding LTM Configuration ID assigned by CU. For such configuration,

- To S-DU, enhance DL RRC MESSAGE TRANSFER.

- To C-DU, enhance UE CTXT MOD REQ.

RS configuration and the signalling

Proposal 2: Do not allow CU to retrieve the RS resource configuration of candidate cells before admission control with DUs, which may necessitate additional round-trip with all the DUs (to update CellGroupConfig in order to fix the CSI reporting configuration within) and thus lose the original intention of reducing Inter-DU LTM preparation latency.

- Remove the placeholder “CSI Resource Configuration (FFS)” IE in UE CTXT SETUP REQ > LTM Information to be Setup IE in F1AP BL CR [3].

- Revert the steps 7-8 in inter-DU LTM flow in TS 38.401 BL CR [4] back to mandatory.

Selected beam transfer

Proposal 3: For selected beam transfer, Option 2 is preferred, given that the main usage of this UE-associated class-2 message (LTM Cell Switch Notification) is to convey information from the current serving DU to CU (also to the target DU in case of inter-DU LTM) and it is not expected that upstream and downstream are evolved drastically in a different way.

TA acquisition

Proposal 4: For TA acquisition, the current class-2 message name “TA Information Notify” sounds generic enough to serve any purpose related to the notification of TA information. Also it is not preferred to separate this class-2 into UL/DL as they are not expected to evolve drastically different.

Proposal 5: In case of successful serving cell change across DUs, allow CU (upon receiving ACCESS SUCCESS from the new serving DU) to request the previous serving DU to forward the stored valid UL TA values to the new serving DU.

- With respect to forwarding the stored valid UL TA values from the previous serving DU to the new serving DU, re-use the existing TA Information Notify mechanism (no stage-3 change).

- With respect to CU’s triggering the pervious serving DU to forward to the new serving DU, enhance the TA Information Notify message.

Proposal 6: Fix the mis-placed/missing “Source DU ID” to be under the Early Sync Information Request IE in both UE CTXT SETUP/MOD REQ messages, and re-define it as a list, so that CU can request allocating non-conflicting CFRA resources of a specific cell in a C-DU to be used by all the other DUs (i.e. more than one – including S-DU and other C-DUs).

UE based TA measurement

Proposal 7: For UE-based TA measurement, allow CU to request the lower-layer configuration from DU for UE based TA measurement per candidate cell basis during LTM preparation (e.g. reference signal configuration to measure the received signal time difference between the current serving cell and candidate cell).

- Enhance the UE CTXT SETUP/MOD REQ to include such request.

Proposal 8: Once CU decides to configure the UE based TA measurement and for which candidate cells, after LTM RRC configuration to the UE, CU notifies the current serving DU which candidate cells are applicable for UE-based TA measurement via UE CTXT MOD REQ:

- If the UE based TA measurement is configured by RRC (pending RAN2), the current serving DU, once notified by CU of which candidate cells, may skip early TA acquisition for those candidate cells.

- If the UE based TA measurement is configured by LTM cell switch command to the UE (pending RAN2), the current serving DU, once notified by CU of which candidate cells, may not only skip early TA acquisition for those candidate cells, but also configure via LTM cell switch command when such cell is triggered for mobility.

Others (corrections/editorials/clean-up)

Proposal 9: Fix/update the following:

- Replace “L1/L2 triggered mobility” by “LTM” in F1AP descriptions.

- For Inter-DU LTM call flow, add a note that candidate cells preparation with S-DU follows the Steps 3-4 of Intra-DU LTM call flow before Step 5.

- Regarding FFS on Step 7-8 whether “parallel” or “single” in Inter-DU LTM flow, RAN3 to go with “parallel” (doesn’t require major stage-3 changes) and update stage-2 accordingly.

- Fix the CSI Resource Configuration IE reference in UE CTXT MOD REQ to LTM-CSI-ResourceConfigToAddModList.

- ACCESS SUCCESS: Update the F1AP description, tailored for LTM.

- LTM CELL CHANGE NOTIFICATION: Fix the F1AP desciprtion and make it more generic.

- TA INFORMATION NOTIFY: Fix the F1AP description from any DU to any other DU (not just from C-DU to S-DU).

Based on the proposals above, the TP for F1AP and 38.401 BL CRs [3][4] can be found in Section 5 and 6, respectively.

Proposal 10: Agree the TPs in Section 5 and Section 6.

# Reference

[1] R3-235764, “Summary of Offline Discussion: CB: # MoblityEnh_LTM”, Huawei

[2] R2-2310885, “RRC running CR for LTM”, Ericsson

[3] R3-235971, “(BL CR to 38.473) Additions for L1/L2 triggered mobility”, Ericsson, Huawei, Nokia, Nokia Shanghai Bell, Intel Corporation, ZTE

[4] R3-235968, “(BL CR to 38.401) for L1L2Mob”, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, ZTE

[5] R4-2314455, Reply LS on beam application time and UE based TA measurement for LTM, RAN4

# TP for NR feMob2 for TS 38.473

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the case of RACH based SDT procedure, the CellGroupConfig IE shall be ignored by the gNB-CU.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the LTM Indicator IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message and set to "true", the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT SETUP RESPONSE message.

If the Request for Lower Layer Configuration IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message and set to "true", the gNB-DU shall, if supported, provide the lower layer configuration in the CellGroupConfig IE in the UE CONTEXT SETUP RESPONSE message.

If the Request for UE based TA Measurement Lower Layer Configuration IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message and set to "true", the gNB-DU shall, if supported, provide the lower layer configuration for the UE based TA measurement for the candidate cell identified by the SpCell ID IE in the UE based TA Measurement Configuration IE in the UE CONTEXT SETUP RESPONSE message.

If the Request for RACH Configuration Lists IE is contained in the Early Sync Information Request IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the RACH Configuration IE for each requested gNB-DU indicated by the Other Candidate gNB-DU ID IE for early TA acquisition (early UL synchronisation), in the RACH Configuration List IE of the UE CONTEXT SETUP RESPONSE message.

If the LTM Reference Configuration IE is contained in the LTM Information to be Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

Editor’s note: FFS whether to reuse existing IE to signal LTM reference configuration

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message. If the Conditional Inter-DU Mobility Information IE or the LTM Indicator IE was included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

#### 8.3.1.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value. If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT SETUP REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

In case of "CHO-replace" when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT SETUP REQUEST message was not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore this candidate cell.

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message and set to "true", the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the CSI Resource Configuration IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig for the requested LTM candidate cell.

If the Request for Lower Layer Configuration IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message and set to "true", the gNB-DU shall, if supported, include the CellGroupConfig IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration.

If the Request for UE based TA Measurement Lower Layer Configuration IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message and set to "true", the gNB-DU shall, if supported, provide the lower layer configuration for the UE based TA measurement for the candidate cell identified by the included SpCell ID IE in the UE based TA Measurement Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the LTM Cells Prepared List IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the LTM has been prepared for the UE and use the included information for LTM execution.

If the UE based TA Measurement LTM Cells List IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the included candidate cells are applicable for UE based TA measurement.

If the Request for RACH Configuration Lists IE is contained in the Early Sync Information Request IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the RACH Configuration IE associated with the candidate cell identified by the included the SpCell ID IE for each requested gNB-DU indicated by the Other Candidate gNB-DU ID IE for early TA acquisition (early UL synchronisation), in the RACH Configuration List IE of the UE CONTEXT MODIFICATION RESPONSE message.

If the Early Sync Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the LTM Reference Configuration IE is contained in the LTM Information to be Modified IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration. FFS whether to reuse existing IE to signal LTM reference configuration

Editor’s note: FFS on the LTM Reference Configuration remains.

- 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the UE context can be successfully performed, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. If the Conditional Intra-DU Mobility Information IE was included in the UE CONTEXT MODIFICATION REQUEST message and set to "CHO-initiation", the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT MODIFICATION REQUEST message, it shall reply with the UE CONTEXT MODIFICATION FAILURE message.

If the Conditional Intra-DU Mobility Information IE was included and set to "CHO-initiation" or "CHO-replace" or if the LTM Indicator IE was included, but the SpCell ID IE was not included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

If the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.3.4.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT MODIFICATION REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated candidate cells.

If more than one of the following IEs, i.e., the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a logical error.

If one or more LTM cells in the LTM Cells To Be Released List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated LTM cells.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.8	Access Success

#### 8.3.8.1	General

The purpose of the Access Success procedure is to enable the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Access Success procedure. Successful operation.

The gNB-DU initiates the procedure by sending a ACCESS SUCCESS message.

Conditional handover or conditional PSCell addition or conditional PSCell change

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU and consider all the other CHO preparations or conditional PSCell addition or conditional PSCell change preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU as cancelled.

LTM

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU.

Interaction with other procedure:

The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate gNB-DUs for this UE, if any.

#### 8.3.8.3	Abnormal Conditions

If the ACCESS SUCCESS message refers to a context that does not exist, the gNB-CU shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.x LTM Cell Change Notification

#### 8.3.x.1	General

The purpose of the LTM Cell Change Notification procedure is to enable the source gNB-DU to inform the gNB-CU about the initiation of LTM cell switch command to the UE. This procedure is also used to transfer the cell switch related information from the source gNB-DU to the gNB-CU, and from the gNB-CU to the target gNB-DU (in case of inter-gNB-DU LTM).

The procedure uses UE-associated signalling.

#### 8.3.x.2	Successful Operation

##### 8.3.x.2.1	LTM Cell Change Notification from the gNB-DU

Figure 8.3.x.2-1: LTM Cell Change Notification procedure initiated from the gNB-DU. Successful operation.

The source gNB-DU initiates the procedure by sending an LTM CELL CHANGE NOTIFICATION message.

Upon reception of the LTM CELL CHANGE NOTIFICATION message, the gNB-CU shall consider that LTM was commanded to the UE where the target cell is indicated by the included NR CGI IE.

##### 8.3.x.2.2	LTM Cell Change Notification from the gNB-CU

Figure 8.3.x.2-2: LTM Cell Change Notification procedure initiated from the gNB-CU. Successful operation.

The gNB-CU initiates the procedure by sending an LTM CELL CHANGE NOTIFICATION message.

Upon reception of the LTM CELL CHANGE NOTIFICATION message, the target gNB-DU shall consider thatLTM was commanded to the UE toward the target cell ID included, with the indicated TCI state if included.

#### 8.3.x.3	Abnormal Conditions

If the LTM CELL CHANGE NOTIFICATION message refers to a context that does not exist, the gNB-CU or the target gNB-DU shall ignore the message.

### 8.3.y TA Information Notify

#### 8.3.y.1	General

The purpose of the TA Information Notify procedure is to enable the gNB-DU to send the TA information to the gNB-CU. This procedure is also used by the gNB-CU to forward the received TA information to another gNB-DU, or to request the gNB-DU to forward the TA information to another gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.3.y.2	Successful Operation

##### 8.3.y.2.1	TA Information Notify from the gNB-DU

Figure 8.3.y.2-1: TA Information Notify procedure initiated from the gNB-DU. Successful operation.

The gNB-DU initiates the procedure by sending a TA Information Notify message.

Upon reception of the TA Information Notify message, the gNB-CU shall consider that the each information in the TA Information List IE is the TA information from the candidate cell indicated by the included NR CGI IE and needs to be forwarded to the gNB-DU indicated by the Source gNB-DU ID IE.

##### 8.3.y.2.2	TA Information Notify from the gNB-CU

Figure 8.3.y.2-2: TA Information Notify procedure initiated from the gNB-CU. Successful operation.

The gNB-CU initiates the procedure by sending a TA Information Notify message.

Upon reception of the TA Information Notify message, the gNB-DU shall consider that each information included in the TA Information List IE is the TA information from the candidate cell indicated by the included NR CGI IE.

If the TA Information Sharing Request IE is included in the TA INFORMATION NOTIFY message, the gNB-DU shall, if supported, initiate the TA Information Notify procedure to the gNB-CU to deliver the stored valid TA information of LTM candidate cells for the UE identified by the included gNB-CU UE F1AP ID IE and the gNB-CU UE F1AP ID IE toward the gNB-DU identified by the included Target gNB-DU ID IE.

#### 8.3.y.3	Abnormal Conditions

Not applicable.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.4.2	DL RRC Message Transfer

#### 8.4.2.1	General

The purpose of the DL RRC Message Transfer procedure is to transfer an RRC message.

The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful operation

Figure 8.4.2.2-1: DL RRC Message Transfer procedure

The gNB-CU initiates the procedure by sending a DL RRC MESSAGE TRANSFER message. If a UE-associated logical F1-connection exists, the DL RRC MESSAGE TRANSFER message shall contain the gNB-DU UE F1AP ID IE, which should be used by gNB-DU to lookup the stored UE context. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established at reception of the DL RRC MESSAGE TRANSFER message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the DL RRC MESSAGE TRANSFER contains the SRB Mapping Info IE, the gNB-DU shall, if supported, use it for the Remote UE’s SRB0 or SRB1 transfer.

If the DL RRC MESSAGE TRANSFER message contains the LTM Cells Prepared List IE, the gNB-DU shall, if supported, consider that the LTM has been prepared for the UE and use the included information for LTM execution.

Interactions with UE Context Release Request procedure:

If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the gNB-DU may trigger the UE Context Release Request procedure, as specified in TS 38.401 [4].

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| ServingCellMO List |  | 0..1 |  | For NCD-SSBs | YES | ignore |
| >ServingCellMO Item IEs |  | 1 .. <maxnoofServingCellMOs> |  |  | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64) |  | - |  |
| >>SSB frequency | M |  | INTEGER (0..3279165) | ARFCN | - |  |
| LTM Information to be Setup |  | 0..1 |  |  | YES | reject |
| >LTM indicator | O |  | ENUMERATED (true, …) |  | - |  |
| >Request for Lower Layer Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM Reference Configuration (FFS) | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Request for UE based TA Measurement Lower Layer Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration List |  | 0.. <maxnoofLTMCellsMinusOne> |  |  | - |  |
| >>Other Candidate gNB-DU ID |  |  | 9.3.1.9 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofLTMCellsMinusOne | Maximum no. of Cells configured for LTM allowed towards one UE minus 1. The value is 7. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT SETUP REQUEST message. | YES | reject |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >RACH Configuration List |  | 0.. <maxnoofLTMCellsMinusOne> |  | This information is used for LTM early TA acquisition (details are FFS). | - |  |
| >>Other Candidate gNB-DU ID | M |  | 9.3.1.9 |  | - |  |
| >>RACH Configuration | M |  | OCTET STRING | Corresponds to the EarlyUL-SyncConfig IE as defined in TS 38.331 [8]. (FFS) | - |  |
| >Joint or DL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |
| >UL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-UL-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >RS Configuration | O |  | OCTET STRING | Includes the LTM-CSI-ResourceConfig IE as defined in TS 38.331 [8]. | - |  |
| >UE based TA Measurement Configuration | O |  | OCTET STRING | FFS on reference | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxNrofBWPs | Maximum number of BWPs per serving cell, the maximum value is 8. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofLTMCellsMinusOne | Maximum no. of Cells configured for LTM allowed towards one UE minus 1. The value is 7. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| CPAC MCG Information |  | 0..1 |  | This IE is used at the MN for MCG configuration as specified in TS 37.340 [7] for CPAC. | YES | ignore |
| >CPAC Trigger | M |  | ENUMERATED (CPAC-preparation, CPAC-executed, …) |  | - | - |
| >PSCell ID | M |  | NR CGI 9.3.1.12 | The PSCell corresponding to the included CG-Config IE at CPAC-preparation or the selected PSCell by the UE at CPAC-executed. | - | - |
| LTM Information to be Modified (details FFS) |  | 0..1 |  |  | YES | ignore |
| >LTM indicator | O |  | ENUMERATED (true, …) |  | - |  |
| >Request for Lower Layer Configuration | O |  | ENUMERATED (true, …) | The request is for the SpCell ID IE included. | - |  |
| >LTM Reference Configuration (FFS) | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >CSI Resource Configuration | O |  | OCTET STRING | Includes the LTM-CSI-ResourceConfigToAddModList IE as defined in TS 38.331 [8]. | - |  |
| >Request for UE based TA Measurement Lower Layer Configuration | O |  | ENUMERATED (true, …) | The request is for the SpCell ID IE included. | - |  |
| >LTM Cells Prepared List |  | 0 .. <maxnoofLTMCells> |  | List of all the prepared candidate cells for LTM | YES | reject |
| >>LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >>LTM Configuration ID | M |  | INTEGER (1..8, …) |  | - | - |
| >UE based TA Measurement LTM Cells List |  | 0 .. <maxnoofLTMCells> |  | List of all candidate cells applicable for UE based TA measurement | YES | reject |
| >>LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| Early Sync Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>RACH Configuration | O |  | OCTET STRING | Corresponds to the EarlyUL-SyncConfig IE as defined in TS 38.331 [8]. (FFS) | - |  |
| >>Joint or DL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |
| >>UL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-UL-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |
| Early Sync Information Request |  | 0..1 |  | The request is for the SpCell ID IE included. | YES | ignore |
| >Request for RACH Configuration List |  | 0.. <maxnoofLTMCellsMinusOne> |  |  | - |  |
| >>Other Candidate gNB-DU ID |  |  | 9.3.1.9 |  | - |  |
| LTM Cells to be Released List |  | 0..1 |  |  | YES | ignore |
| >LTM Cells to be Released Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxnoofLTMCellsMinusOne | Maximum no. of Cells configured for LTM allowed towards one UE minus 1. The value is 7. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell or PSCell ID in the CPAC MCG Information IE indicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |
| ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >RS Configuration | O |  |  | Includes the LTM-CSI-ResourceConfig IE as defined in TS 38.331 [8]. |  |  |
| >UE based TA Measurement Configuration | O |  | OCTET STRING | FFS on reference | - |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >RACH Configuration List |  | 0.. <maxnoofLTMCellsMinusOne> |  | This information is used for LTM early TA acquisition (details are FFS). | - |  |
| >>Other Candidate gNB-DU ID | M |  | 9.3.1.9 |  | - |  |
| >>RACH Configuration | M |  | OCTET STRING | Corresponds to the EarlyUL-SyncConfig IE as defined in TS 38.331 [8]. (FFS) | - |  |
| >Joint or DL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |
| >UL TCI State | O |  | OCTET STRING | Includes the CandidateTCI-UL-States as defined in 6.3.2 of TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxNrofBWPs | Maximum number of BWPs per serving cell, the maximum value is 8. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 32. |
| maxnoofLTMCellsMinusOne | Maximum no. of Cells configured for LTM allowed towards one UE minus 1. The value is 7. |

#### 9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change or LTM.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.x	LTM CELL CHANGE NOTIFICATION

This message is sent by the source gNB-DU to inform the gNB-CU about the initiation of the LTM cell switch  command to the UE. The message is also used to transfer the cell switch related information from the source gNB-DU to the gNB-CU, and from the gNB-CU to the target gNB-DU (in case of inter-gNB-DU LTM).

The procedure uses UE-associated signalling.

Direction: gNB-DU  gNB-CU

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NRCGI 9.3.1.12 |  | YES | reject |
| TCI State ID | O |  | INTEGER (0..FFS-1) | Corresponds to the CandidateTCI-StatesId IE as defined in TS 38.331 [8]. | YES | ignore |

#### 9.2.2.y	TA Information Notify

This message is sent by the gNB-DU to inform the gNB-CU about TA information. This message is also sent by the gNB-CU to forward the received TA information to another gNB-DU, or to request the gNB-DU to forward the TA information to another gNB-DU.

Direction: gNB-DU  gNB-CU

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) (FFS) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>TA Related UE Information (FFS) | M |  | OCTET STRING | Corresponds to the RACH-ConfigDedicated IE as defined in TS 38.331 [8]. | - |  |
| >>Source gNB-DU ID | M |  | 9.3.1.9 |  | - |  |
| TA Information Sharing Request |  | 0..1 |  |  | YES | ignore |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >Target gNB-DU ID | M |  | 9.3.1.9 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is FFS. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.3	RRC Message Transfer messages

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.3.2	DL RRC MESSAGE TRANSFER

This message is sent by the gNB-CU to transfer the layer 3 message to the gNB-DU over the F1 interface.

Direction: gNB-CU gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| Execute Duplication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| RRC-Container | M |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| UE Context not retrievable | O |  | ENUMERATED (true, ...) |  | YES | reject |
| Redirected RRC message | O |  | RRC Container 9.3.1.6 | Includes the UL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| PLMN Assistance Info for Network Sharing | O |  | PLMN Identity 9.3.1.14 |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the Remote UE’s SRB0 or SRB1. | YES | ignore |
| LTM Cells Prepared List |  | 0 .. <maxnoofLTMCells> |  | List of all the prepared candidate cells for LTM | YES | reject |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >LTM Configuration ID | M |  | INTEGER (1..8, …) |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

# TP for NR feMob2 for TS 38.401

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID. The gNB-CU requests PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or for the purpose of configuring the UE based TA measurement to the UE.

- If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations (e.g., TCI state configuration, RACH configuration, UE based TA measurement configuration, and the RS configuration) for the accepted target candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration and the cell ID(s) of all the prepared candidate cell(s) with their corresponding LTM configuration ID(s) assigned.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

Note 1: If the UE based TA measurement is applicable for some candidate cells, the gNB-CU may notify to the gNB-DU via the UE Context Modification procedure. Early synchronization may be skipped for those candidate cells.

- Early synchronization is performed as specified in TS 38.300 [2].

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI stage ID.

- FFS: How the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: Inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to initiate LTM configuration.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID. The gNB-CU may indicate the other gNB-DU ID(s), and request their respective PRACH resources from the Candidate gNB-DU. The gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or for the purpose of configuring the UE based TA measurement to the UE.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration (e.g., TCI state configuration,  RACH configuration, UE based TA measurement configuration, and the RS configuration) for the accepted target candidate cell.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected RS configuration, TCI state configuration and RACH configuration for the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which may include the RS configuration of the source cell (FFS), prepared candidate cells (FFS) and the generated CSI report configuration (FFS).

Note 1: If there are some candidate cells to be prepared with the source gNB-DU, the Steps 3-4 in 8.2.1.X	Intra-gNB-DU LTM may precede before Step 5.

7. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message for each accepted candidate cell to each candidate gNB-DU(s), where one of which contains the cell ID(s) of all the prepared candidate cell(s) with their corresponding LTM configuration ID(s) assigned, and the associated RS configuration for each candidate cell in other candidate gNB-DU(s). The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

Note: The candidate cell may be the same cell as source cell.

8. The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated low layer configuration in which contains the generated CSI report configuration.

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration and the cell ID(s) of all the prepared candidate cell(s) with their corresponding LTM configuration ID(s) assigned.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

Note 2: If the UE based TA measurement is applicable for some candidate cells, the gNB-CU may notify to the gNB-DU(s) via the UE Context Modification procedure. Early synchronization may be skipped for those candidate cells.

13. Early synchronization is performed as specified in TS 38.300 [2].

14. The candidate gNB-DU sends the TA, the associated CFRA resource information, the candidate cell ID and the source gNB-DU ID to the source gNB-DU via the gNB-CU.

15. The UE sends the lower layer measurement result to the source gNB-DU.

16. The source gNB-DU decides to execute LTM to a candidate target cell.

17. The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

18. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI state ID.

19. The gNB-CU sends the target cell ID and the TCI state ID to the target gNB-DU.

20. FFS:  how the target gNB-DU detects the UE access.

21. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

Note 3: Upon receiving the ACCESS SUCCESS message, the gNB-CU may request the previous serving gNB-DU to forward the stored valid TA information of the UE’s candidate cells to the new serving gNB-DU, via the TA Information Notify procedure.

22. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

23. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.