3GPP TSG-RAN WG3 Meeting #122	R3-237677

Chicago, USA, Nov 13- Nov 17, 2023

Title: 	(TP for TS38.425) Support for XR PDU Set Handling

Source: 	CMCC, Nokia, Nokia Shanghai Bell, Huawei, ZTE

Agenda item:	25.2.1

Document Type:	Other

1. Introduction

This contribution provides the TP to TS 38.425 for NR XR based on the discussion paper.

- Text Proposal to TS 38.425 BLCR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs.

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

The corresponding node shall detect whether an NR-U packet was lost and memorise the respective sequence number after it has declared the respective NR-U packet as being "lost".

The corresponding node shall transfer the remaining NR PDCP PDUs towards the UE and memorise the highest NR PDCP PDU sequence number of the NR PDCP PDU that was successfully delivered (as defined in TS 36.322 [6] and TS 38.322 [7]) in sequence towards the UE (in case RLC AM is used) and the highest NR PDCP PDU sequence number of the NR PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered. The DL DATA DELIVERY STATUS sent as a response to a specific DL report NR PDCP PDU SN shall be sent only when all PDCP PDU SNs up to this DL report NR PDCP PDU have been successfully delivered in-sequence.

If the Request OutOfSeq Report is set to 1, the corresponding node shall, if supported, include the NR PDCP PDU sequence number successfully delivered out of sequence in the DL DATA DELIVERY STATUS to the node hosting the NR PDCP entity.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost NR-U packets assist the node hosting the NR PDCP entity in avoiding NR PDCP HFN de-synchronisation. If a deployment decides to not use the Transfer of Downlink User Data procedure, NR PDCP HFN synchronization should be ensured by other means.

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

If the corresponding indicators set to 1, the node hosting the NR PDCP entity indicates to the corresponding node to consider PDU Set QoS handling as specified in TS 23.501 [8].

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

#### 5.4.1.2	Unsuccessful operation

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  |  | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  |  | PDU Set Flag |  | DL Discard Blocks | DL Flush | Report polling | 1 |
| End of PDU Set | End of Data Burst |  | Spare |  | Request OutofSeq Report | Report Delivered |  | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Spare |  |  |  |  |  |  | PSI |  |  |  | 0 or 1 |
| Spare |  |  |  | QFI |  |  |  |  |  |  | 0 or 1 |
| PSSN |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| PSSN |  |  |  | PSN |  |  |  |  |  |  |  |
| PSSize |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

NOTE 1:	In this section, information elements are also applicable to E-UTRA PDCP unless specified otherwise. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP unless specified otherwise.

*// skip unchanged part //*

#### 5.5.3.a	PDU Set Flag

Description: This parameter indicates the presence of PDU Set parameters. The PDU Set parameters include the End PDU Set Indicator, End Data Burst Indicator, PDU Set Importance, QoS Flow Identifier, PDU size, PDU Set Sequence Number and PDU Sequence Number within a PDU Set.

Value range: {0= PDU Set parameters not present, 1= PDU Set parameters present}.

Field length: 1 bit.

#### 5.5.3.b	End of PDU Set

Description: This parameter indicates whether the PDU is the last PDU of the PDU Set.

Value range: {0= PDU is not the final PDU of the PDU Set, 1= PDU is the final PDU of the PDU Set}.

Field length: 1 bit.

#### 5.5.3.c	End of Data Burst

Description: This parameter indicates whether the PDU is the last PDU of the Data Burst.

Value range: {0= PDU is not the final PDU of the Data Burst, 1= PDU is the final PDU of the Data Burst}.

Field length: 1 bit.

5.5.3.d	PDU Set Importance (PSI)

Description: This parameter indicates the importance of the current PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance. PDU Set with the highest importance PDU Set is indicated by 0 and the lowest importance PDU Set is indicated by 15.

Value range: {0..15}.

Field length: 4 bits.

5.5.3.e	PDU Set Sequence Number (PSSN)

Description: This parameter indicates the sequence number of the PDU Set to which the current PDU belongs acting as an identifier for the PDU Set.

Value range: {0..210-1}.

Field length: 10 bits.

5.5.3.f	PDU Sequence Number within a PDU Set (PSN)

Description: This parameter indicates the sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..26-1}.

Field length: 6 bits.

5.5.3.g	PDU Set Size (PSSize)

Description: This parameter indicates the total size of all PDUs of the PDU Set to which the current PDU belongs.

Value range: {0..224-1}.

Field length: 3 octets.

5.5.3.h	QoS Flow Identifier (QFI)

Description: This parameter indicates the the QoS Flow Identifier of the QoS flow to which the user data belongs.

Value range: {0..26-1}.

Field length: 6 bits.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>