# 3GPP TSG-RAN WG3 Meeting #122	R3-237793

# Chicago, USA, 13 - 17 November 2023

# Agenda Item:	12.2.1

# Source:	ZTE, Samsung, CMCC, China Telecom, China Unicom, Huawei, 	Lenovo, Nokia, Nokia Shanghai Bell, NEC, Qualcomm Incorporated, 	Ericsson, CATT ,LGE

# Title:	(TP to 38.300 BLCR) Stage2 description update for AI/ML RAN

# Document for:	Text Proposal

- Introduction

In this contribution, we further provide some updates on the current 38.300 BLCR to finalize the stage2 description for AI/ML function.

- Annex - TP to 38.300 BLCR

<<<<<< Start of CHANGE >>>>>>

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

******Unchanged Text Omitted******

EC					Energy Cost

******Unchanged Text Omitted******

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

<<<<<<Next CHANGE >>>>>>

X.X	Support of AI/ML for NG-RAN

X.X.1	General

Support of AI/ML for NG-RAN, as a RAN internal function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2	Mechanisms and Principles

Support of AI/ML in NG-RAN requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results).

Signalling procedures used for the exchange of information to support AI/ML in NG-RAN, are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of AI/ML in NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML in NG-RAN, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information

-	UE performance feedback

-	Measured UE trajectory

-	Energy Cost (EC)

The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover execution.

Cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for e.g. subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

<<<<<< End of CHANGE >>>>>>