- 3GPP TSG-RAN WG3 Meeting #122                                                            R3-237796

- Chicago, USA, 13 - 17 November 2023

Agenda Item:	12.2.2.2

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # AIML2_LB

Document for:	Approval

# Introduction

During the online discussion, following are the agreements and left issues:

Measurement Collection periodicity does not need to be introduced in Rel-18, can be further discussed in R19.

Not introducing a new indicator in Data Collection Update message to indicate the UE performance feedback reporting termination in R18.

Revised the common understanding as follows: “For one pair of measurement IDs, the frequency at which the data collection update messages that include UE performance feedback are signaled is determined by the current reporting periodicity IE”?

Introduce New Cause value?

Introduce the new codepoint like “update”/ “modify” type in Registration Request IE, allowing the modification of the configuration of the requested information?

CB: # AIML2_LB

- Discuss the open issues above

- Provide TPs if agreeable

(moderator - ZTE)

Summary of offline disc R3-237796

# For the Chairman’s Notes

Proposal 1: No consensus on the cause value due to timing issues in R18.

Proposal 2: Define the Measurement Collection Duration as INTEGER (1..5000, ...) per millisecond.

Proposal 3: Change the name “Average Packet Loss” IE to “Average Packet Loss DL” IE.

Proposal 4: Introduce Average Packet Delay UL IE and Average Packet Delay DL IE within Average Packet Delay IE. These IE are defined as INTERGE (0..10000).

Proposal 5: Predicted Resource Status IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. And remove the FFS on the details of Predicted Resource Status IE.

- Proposal 6: Remove the FFS related to the actions of the NG-RAN node2 and the corresponding FFS in the tabular for the encoding of the Registration Request IE.

Proposal 7: Cell To Report List IE in the DATA COLLECTION REQUEST message is not used for requesting UE performance feedback.

Proposal 8: Move the bits of UE trajectory and UE performance to Node Measurement Initiation Result List IE.

Other remaining FFS:

- Replace the FFS in the criticality of UE associated Info Result List in DATA COLLECTION UPDATE message to “ignore”

- Set the maximum value of maxnoofUEReports to 16 and remove FFS.

- Keep the name of UE Assistant Identifier, and remove FFS.

- Remove all Editor’s notes

- Correct the description of maxnoofUEReports as follows:Maximum no. UEs for which information can be reported by a NG-RAN node

ZTE provides the TP R3-237939 for Measurement Collection Duration, and resolving all remaining FFS.

Nokia provides the TP R3-237940 to capture P3&P4.

Samsung provides the TP R3-237941 to capture P8.

# Discussion

## The trigger of DATA COLLECTION UPDATE message

For one pair of measurement IDs, the frequency at which the data collection update messages are signaled is determined by the current reporting periodicity IE.

In R3-237292, it is proposed to clarify the scenarios where measured UE trajectory and UE performance feedback are configured in a single DATA COLLECTION UPDATE message.

Scenario 1: Measured UE trajectory is configured for one-time reporting, and UE performance feedback is also configured for one-time reporting with different collection duration.

For this scenario, there maybe two understanding on the trigger mechanism of DATA COLLECTION UPDATE message：

- Understanding 1: Two separate DATA COLLECTION UPDATE messages are used—one for measured UE trajectory and the other for UE performance feedback. Each message is triggered after the respective collection duration for measured UE trajectory or UE performance feedback has passed. As shown in Figure 2a.

Figure 2a. Understanding 1: Two separate DATA COLLECTION UPDATE messages

- Understanding 2: One single DATA COLLECTION UPDATE message. The measurement with short collection duration should wait for the measurement with long duration, which means the DATA COLLECTION UPDATE message is triggered after the long collection duration, involving the measured UE trajectory and UE performance feedback. As shown in Figure 2b.

Figure 2b. Understanding 2: One single DATA COLLECTION UPDATE message

Q1: Companies are invited to provide their views on which understanding is correct.

| Company | Understanding 1, 2? | Comments |
| --- | --- | --- |
| CMCC | 1 | UE trajectory collection termination can be triggered by 4 different conditions, if one of the conditions is triggered, the measured UE trajectory should be reported immediately through a Update message. No need to wait for the report duration timeout. We can also accept it as up to implementation. |
| Qualcomm | Both 1 and 2 | We think both the options should be allowed. For option 1, we support CATT proposal to support multiple measurement ID pairs in HO Request message. |

Scenario 2: Measured UE trajectory is configured for one-time reporting, while UE performance feedback is configured for periodic reporting with different collection duration.

For this scenario, there maybe two understanding on the trigger mechanism of DATA COLLECTION UPDATE message：

- Understanding 1: DATA COLLECTION UPDATE message that includes UE performance feedback strictly follows reporting periodicity, while DATA COLLECTION UPDATE message that includes measured UE trajectory is triggered when its collection duration has passed, without considering the periodicity of UE performance feedback, as shown in Figure 3a.

Figure 3a. Understanding1 for the scenario2

- Understanding 2: DATA COLLECTION UPDATE message that includes UE performance feedback strictly follows reporting periodicity, and the target node will not report the measured UE trajectory until the next reporting period to send an DATA COLLECTION UPDATE message, strictly to the previously agreed common understanding, as shown in Figure 3b.

Figure 3b. Understanding2 for the scenario2

Q2: Companies are invited to provide their views on which understanding is correct.

| Company | Understanding 1, 2? | Comments |
| --- | --- | --- |
| CMCC | 1 | We have already agreed that UE trajectory report is one-time report, it is higher priority than periodical report. The Report should be sent through Update message as soon as the collection terminates. No need to wait until the periodicity time to send the Update message for it. And also the same reason with Q1, the UE trajectory should be reported when the any of the 4 conditions is satisfied. If we restrict the UE trajectory report is sent on the next reporting period as Understanding 2, it’s conflict with the agreed 4 trigger conditions of UE trajectory reporting. So, not need to restrict the measured UE trajectory report with the reporting period, it can be send with separate Update message. |
| Qualcomm | Both 1 and 2 | Same comment as above |

## Cause value

It was agreed that including failure measurement per node or per cell in the response message to support partial reporting mechanism, and each failure measurement is indicated with the corresponding cause value. Currently, it was agreed to reuse the existing cause value, e.g., “Measurement not Supported for The Object” and “Measurement Temporarily not Available”. Besides, RAN3 agreed to introduce cause values indicating failure due to timing issues.

Agree to introduce cause value(s) indicating failures due to timing issues. Further discussions are needed on which timing issues to address.

Following are cause value proposed by companies:

Option 1: Measurement not supported with requested prediction time

Option 2: Measurement not available with requested prediction time

Option 3: Measurement not supported with requested reporting periodicity

Option 4: Measurement not available with requested reporting periodicity

Option 5: Measurement not Supported For The Requested Timing Configuration

Option 6: Measurement Temporarily not Available For The Requested Timing Configuration

Note: 5&6 are the simplified version on cause value.

No consensus on the cause value due to timing issues in R18.

Q3: Companies are invited to provide their views on options are acceptable.

| Company | Understanding 1, 2? | Comments |
| --- | --- | --- |
| CMCC | Option 1 | No strong view, option 1 can be accepted. |
| Qualcomm | No strong view | We think option1 should be supported for Rel-18 as bare minimum. But we are open to discuss more cause values too. |

## UE performance collection configuration

Following options are proposed by companies on defining the UE performance collection configuration.

Option 1: ENUMERATED TYPE

#### 9.2.3.N	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement Collection Duration | O |  | ENUMERATED(1s, 5s, 10s, 20s, 30s, 40s ,50s, 60s, ...) | Time duration in seconds, starting from successful handover execution, during which UE performance feedback is collected. |

Option 2: INTEGER TYPE

Option2 is accepted.

#### 9.2.3.N	UE Performance Feedback Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement Collection Duration | M |  | INTEGER(1..5000,…) | Time duration starting at successful handover execution within which the UE performance measurements are collected by NG-RAN node2. Unit: Millisecond |

Q4: Companies are invited to provide their views on the IE type of measurement collection duration. Details of value can be discussed during offline discussion.

| Company | ENUMERATED or INTEGER | Comments |
| --- | --- | --- |
| CMCC | Integer | No strong view, but think Integer may be more flexible. |
| Qualcomm | Enumerated | No strong view. |

Discuss offline:

Procedural Text Description for condition of termination of UE performance feedback collection:

If the UE Performance Collection Configuration IE in the DATA COLLECTION REQUEST message is present, it indicates the configuration for the collection of UE performance. The NG-RAN node2 shall take it into account when collecting the UE performance. Collection of the UE performance by the NG-RAN node2 shall be terminated either at the expiration of the collection duration or if the UE moves to RRC_IDLE or RRC_INACTIVE or if the UE is handed over to another cell.

If the UE Performance Collection Configuration IE (FFS on the IE name) is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

- the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration IE;

- UE moves to RRC_INACTIVE or RRC_IDLE state;

-     UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

## Remaining FFS issue

### Packet Delay IE

The IE type and reference for the packet delay is still FFS. Following are the options proposed to resolve FFS.

Option 1: The Average Packet Delay for UE performance refers to the existing IE Packet Delay Budget.

#### 9.2.3.12	Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Delay Budget | M |  | INTEGER (0..1023, ...) | Upper bound value for the delay that a packet may experience expressed in units of 0.5ms. |

Option 2: Define the Packet Delay IE, as an INTEGER (0..10000, ...) in milliseconds.

#### 9.2.3.Y	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | 9.2.3.4 |  |
| Average UE Throughput UL | O |  | 9.2.3.4 |  |
| Average Packet Delay | O |  |  |  |
| Average Packet Loss DL | O |  | 9.2.3.11 |  |

Option 3: Define Packet Delay in the UL and DL

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | As specified in TS38.314 [x]. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | As specified in TS38.314 [x]. Unit: 0.1 millisecond |

Change the name “Average Packet Loss” IE to “Average Packet Loss DL” IE

Option3 is accepted for Average Packet Delay IE.

Q5: Companies are invited to provide their views on options above.

| Company | Option1 or Option2 | Comments |
| --- | --- | --- |
| CMCC | Option 1 | No strong view, but think the current one can be re-used. |
| Qulcomm | Option 1 | Same view as CMCC |

### Predicted Resource Status IE

There is still FFS related to the details of Predicted Radio Resource Status IE. Currently, we have reached an agreement that the predicted resource status is limited to cell level. However, Predicted Radio Resource Status IE in the BLCR refers to the existing Radio Resource Status IE, which includes PRBs per SSB, per slice and per MIMO.

-	the SSB area the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

Moderator thinks currently we only discuss PRBs per cell level.

Option 1: Introduce a new IE for predicted radio resource status.

Option 2: Add the following semantic description:

This IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE.

Option 2 is accepted.

Q7: Companies are invited to provide their views on options above.

| Company | Option1 or Option2 | Comments |
| --- | --- | --- |
| CMCC | Option 2 | No strong view |
| Qualcomm | Option 2 |  |

### Registration Request Type

Following are the proposals for the registration type:

- Remove the FFS related to the actions of the NG-RAN node2 and the corresponding FFS in the tabular for the encoding of the Registration Request IE.

- Introduce the “modify”/ “update” type in Registration Request IE, allowing the modification of the configuration of the requested information.

- Introduce the “add” type in Registration Request IE, to improve the signaling efficiency, when the reporting periodicities of new cells and existing ones are same.

Q7: Companies are invited to provide their views on proposals for registration type above.

| Company | a,b,c | Comments |
| --- | --- | --- |
| CMCC | b | Data Collection Request is a new introduced message, the content of the Data Collection Request message is not necessary to all follow the legacy procedure (e.g. add). And ‘Update’ can include the function of ‘Add’. So we suggest to introduce ‘modify’ or ‘update’. |
| Qualcomm | b | It is easier to update any reporting periodicity changes or collection duration or addition and removal of cells/UEs, rather than deleting the whole message and creating a new one. It then causes burden at both sender and receiver to restart the whole measurement collection just for a small change in like say reporting periodicity. Hence we prefer to this flexibility. |

### Failed Report Characteristics of UE trajectory and UE performance

In R3-237323, it is stated that “Node measurement initiation result” and “Per cell measurement initiation result” are used to inform the requesting node which measurement item(s) it can not provide. However, for the UE performance feedback and measured UE trajectory, they does not belong to node level or cell level ones. If the requested node can not feedback them, in current structure, the requested node should indicate the failure for all the cells.

Therefore it is proposed to extend the existing node level initiation result to support the requested node to inform the failed UE performance feedback and measured UE trajectory.

- 9.1.3.DD	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| > Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss, Sixth Bit = Measured UE Trajectory. Other bits shall be ignored by the NG-RAN node1. | – |  |

Cell List IE in the DATA COLLECTION REQUEST message is not used for requesting UE performance feedback.

Move the bits of UE trajectory and UE performance to Node Measurement Initiation Result List IE.

Q8: Companies are invited to provide their views on proposals above.

| Company | Yes/No | Comments |
| --- | --- | --- |
| CMCC | No | UE trajectory failure can be leveled up to node level, but UE performance feedback failure should be kept in Cell level. No need to introduce UE trajectory and UE performance measurement to node level, it can be achieved by the current mechanism. As the UE performance feedback can be configured just before the handover, the requesting node can request the prediction information first and send a separate request message after the requesting node inferred, the requesting node may know the target cell, and it can be used to configure the UE performance feedback and UE trajectory. |
| Qualcomm | Yes | At the time of Data Collection procedure initiation, the target does not have any information on the UE or which cell the UE will belong to. Hence providing failure at the node level is the only possibility, |

### Other FFS can be easily agreed

P1: Replace the FFS in the criticality of UE Associated Info Result List in DATA COLLECTION UPDATE message to “ignore”.

P2:  Set the maximum value of maxnoofUEReports to 16.

P3: Rename UE Assistant Identifier as NG-RAN node 1 UE XnAP ID in DATA COLLECTION UPDATE message?

Keep the name of UE Assistant Identifier.

P4: Remove all Editor’s notes.

P5: Correct the description of maxnoofUEReports.

Maximum no. UEs for which information can be reported by a NG-RAN node. Value is 16.

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-237256 | Discussion on remaining open issues in UE performance for NG-RAN AI/ML (Qualcomm Incorporated) | discussion |
| --- | --- | --- |
| R3-237292 | Futher discussion on remaining issues for Load Balancing (ZTE) | discussion |
| R3-237293 | (TP to 38.423 BLCR) AIML based Load Balancing (ZTE) | other |
| R3-237577 | (TP to BL CR for TS 38.423) Cause Value Discussion and Other Open Issues (Nokia, Nokia Shanghai Bell) | other |
| R3-237578 | (TP to BL CR for TS 38.423) UE Performance Data Collection (Nokia, Nokia Shanghai Bell) | other |
| R3-237172 | Discussion on UE performance feedback (CATT) | discussion |
| R3-237173 | TP to BLCR for 38.423 on AL/ML load balance and FFS clear up (CATT) | other |
| R3-237238 | RAN AI/ML on UE Performance Feedback (NEC) | discussion |
| R3-237239 | (TP to TS 38.423 on RAN AI/ML) on UE Performance Feedback (NEC) | other |
| R3-237579 | Discussion on Predicted Radio Resource Status (Nokia, Nokia Shanghai Bell, BT, Orange) | discussion |
| R3-237580 | (TP to BL CR for TS 38.423) Predicted Radio Resource Status (Nokia, Nokia Shanghai Bell, BT, Orange) | other |
| R3-237323 | (TP to TS 38.423 BLCR) Failed Report Characteristics of UE Performance Feedback and Measured UE Trajectory (Samsung) | other |
| R3-237324 | (TP to TS 38.423 BLCR) Support of AI/ML for LB (Samsung) | other |
| R3-237379 | (TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Load Balancing use case (Huawei) | other |
| R3-237408 | Left issues related to load balancing use case (Lenovo) | discussion |
| R3-237409 | (TP for TS38.423 BLCR) On UE related feedback collection (Lenovo) | other |
| R3-237423 | Discussion on the open issue for AI/ML load balancing (LG Electronics Inc.) | discussion |
| R3-237424 | (TP for NR_AIML_NGRAN-Core BL CR for TS 38.423) Discussion on the open issue for AI/ML load balancing (LG Electronics Inc.) | other |
| R3-237481 | Periodic UE Performance Feedback measurements collection and reporting (Ericsson, Deutsche Telekom, InterDigital) | discussion |
| R3-237482 | (TP for AIML BL CR for TS 38.423) Periodic UE Performance Feedback measurements collection and reporting (Ericsson, Deutsche Telekom, InterDigital) | other |
| R3-237483 | Cause values for Data Collection Reporting Initiation (Ericsson, Deutsche Telekom, AT&T, InterDigital) | discussion |
| R3-237484 | (TP for AIML for BLCR to TS38.423) Cause values for Data Collection Reporting Initiation (Ericsson, Deutsche Telekom, AT&T, InterDigital) | other |
| R3-237485 | Data Collection Reporting clean-up (Ericsson, Deutsche Telekom, AT&T, InterDigital) | discussion |
| R3-237486 | (TP for AIML for BLCR to TS38423) Data Collection Reporting clean-up (Ericsson, Deutsche Telekom, AT&T, InterDigital) | other |
| R3-237673 | Discussion on AIML remaining issues (CMCC) | discussion |
| R3-237674 | (TP for 38.423)Procedure for Load Balancing (CMCC) | other |