3GPP TSG-RAN WG3 Meeting #122	R3-237805

13th – 17th Nov 2023, Chicago,USA

Agenda item:	26.3

Source: 	ZTE, CATT, China Telecom, Nokia, Nokia Shanghai bell

Title: 	(TPs for BLCRs for TS38.300)RAN support Network Slicing Area of Service not mapping to existing TA

Document for:   Discussion

# Introduction

This TP for BLCR for TS38.300 is to update stage 2 in Annex for slicing Area of Service not mapping to existing TA.

# Annex  - TP for TS 38.300

START OF CHANGE

- 16.3.1	General Principles and Requirements

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Access control

-	By means of the unified access control (see clause 7.4), operator-defined access categories can be used to enable differentiated handling for different slices. NG-RAN may broadcast barring control information (i.e. a list of barring parameters associated with operator-defined access categories) to minimize the impact of congested slices.

Slice Availability

-	Some slices may be available only in part of the network. The NG-RAN supported S-NSSAI(s), NSAG(s) and NSAG related information such as NSAG associated Cell Reselection Priority and/or NSAG associated RACH resources are configured by OAM. Awareness in the NG-RAN of the slices supported in the cells of its neighbours may be beneficial for inter-frequency mobility in connected mode. In order to support the NSAG, the NG-RAN provides the AMF with the NSAG information per TA in the appropriate NG interface management procedures, as specified in TS 38.413 [26]. Awareness in the NG-RAN of the NSAG information supported in the list(s) of neighbour cells may be configured by OAM, or exchanged with neighbour NG-RAN nodes.It is assumed that the slice availability does not change within the UE's registration area (see TS 23.501 [3]).

-	The NG-RAN and the 5GC are responsible to handle a service request for a slice that may or may not be available in a given area. Admission or rejection of access to a slice may depend by factors such as support for the slice, availability of resources, support of the requested service by NG-RAN.

-	The NG-RAN may be signalled with the Partially Allowed NSSAI from the AMF as specified in TS 23.501 [3]. The NG-RAN may decide to use the Partially Allowed NSSAI for mobility decision.

-    The NG-RAN may support Network Slices with Network Slice Area of Service not matching deployed Tracking Areas as specified in TS 23.501 [3]. NG-RAN cells are outside Area of Service may be configured with zero resources of pertained Slice(s). The pertained Slice(s) cannot use any dedicated, prioritized nor any shared resources of that cell. Awareness of zero resources configured for a slice in the list(s) of neighbour cells may be exchanged with neighbour NG-RAN nodes for mobility reasons.

END OF CHANGE