3GPP TSG-RAN WG3 #123bis	R3-242159

Changsha, 15 – 19 Apr. 2024

Agenda Item:	12.2

Source:	NTTDOCOMO, Qualcomm, Ericsson, ZTE, Xiaomi, Huawei, Nokia, Nokia Shanghai Bell, Samsung

Title:	TP for TR 38.799 on WAB general requirements and architecture

Document for:	Agreement

Introduction

This document provides a TP for WAB general requirements and architecture.

Discussion

Proposal 1: RAN3 to agree the TP for TR 38.799 WAB general requirements and architecture in ANNEX.

ANNEX (WAB TP for TR 38.799)

Start of Change

- X	Wireless Access and Backhaul (WAB)

- X.1	General

- The study is based on the following requirements:

- The WAB-node includes a gNB component (WAB-gNB) and an MT component (WAB-MT).

- The WAB-gNB is based on the gNB functionality specified in TS 38.300 [x] and TS 38.401 [y].

- The CU-DU split of the WAB-gNB is not considered in this study.

- The WAB-MT supports at least a subset of UE functionalities.

- The NR Uu is used for the radio link between WAB-gNB and the served UEs.

- The NR Uu radio link between the WAB-gNB and the served UEs does not use NTN.

- The study focuses on NR-Uu backhaul.

- In-band scenario for access and backhaul is not precluded to be studied.

- The study precludes the scenario where the access and the backhaul are in-band meanwhile the backhaul uses NTN.

- The study focuses on the use of WAB-MT´s PDU session via NR Uu as backhaul of WAB-gNB. Other options for the backhaul (including non-3GPP radio technology) are not precluded but are not a part of the study.

- A WAB-gNB cannot serve WAB-MT(s).

- The study includes a scenario where the WAB-gNB and the WAB-MT connect to the same PLMN or to different PLMNs.

- The WAB-MT may connect to a public PLMN or an SNPN.

- The WAB-gNB may connect to a public PLMN or an SNPN.

- Legacy UEs can connect to the WAB-gNB. There are no WAB-specific enhancements for UEs that connect to the WAB-gNB.

- X.2	WAB Architecture

Figure X.Y-1 shows an example of WAB architecture for 5GS when the WAB-gNB’s NG traffic is transported via PDU session backhaul.

Figure X.Y-1: The WAB architecture example for 5GS when the WAB-gNB traffic is transported via PDU session backhaul

Figure X.Y-2 shows an example of WAB architecture for 5GS when the WAB-gNB’s NG traffic is transported via non-3GPP backhaul:

Figure X.Y-2: The WAB architecture example for 5GS when the WAB-gNB traffic is transported via non-3GPP backhaul

Figure X.Y-3 shows protocol stack examples of Control plane and User plane transport for a UE connected to the network via a WAB-node.

Figure X.Y-3: Protocol stack examples of Control plane and User plane transport for UEs connected via WAB-node

End of Change