3GPP TSG-RAN WG3 Meeting #123bis	R3-242192

# Changsha, China,  15 - 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | Draft | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of XR enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add support of NR-DC related enhancements for XR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support for XR, and ECN Marking and congestion information exposure in NR-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancement on NR-DC for XR is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.x (new), 13.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1269 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

## 13.x	eXtended Reality Services

### 13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

### 13.x.3	Discard

For MN terminated SCG bearer, the MN notifies SN whether the UL PSI based SDU discarding is (re)configured/released for uplink discarding via XnAP signaling.

For SN terminated MCG bearer, the SN notifies MN whether the UL PSI based SDU discarding is (re)configured/released for uplink discarding via XnAP signaling.

Editor Note: FFS for split bearer and DL PSI discard

### 13.y	ECN marking for L4S and congestion information exposure

The ECN marking for L4S and congestion information exposure as described in TS 38.300 [3] is extended to address the NR-DC operation.

For ECN marking for L4S at gNB or UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking request at gNB or ECN marking request at UPF to SN and SN reports the status indication back to the MN.

For congestion reporting from gNB to UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the congestion information request to SN and SN reports the status indication back to the MN.

-----------------End of the Change-------------------