3GPP TSG-RAN3 Meeting #123	R3-240060

Athens, Greece, 26 Feb - 1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Forwarding of the SPR due to T304 to source SN in case of SN initiated PSCell change/CPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SPR due to T304 may be triggered in different cases. It may be caused by some RACH issues in target side but also by wrong mobility thresholds in source side or even both. In the latter two cases the SPR generated due to T304 can help initiating node to detect sub-optimal successful PSCell change/CPC and possibly optimize the mobility thresholds when needed. Currently the part dealing with forwarding the SPR generated due to T304 to source SN for SN initiated PSCell change/CPC is missing in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add stage 2 that in case of SN initiated PSCell change/CPC the SPR reported due to T304 may be forwarded also to source SN which may help to detect sub-optimal successful PSCell change/CPC in initiating node which may be caused by this node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case the change is not accepted, the operators lose the possibility to detect sub-optimal successful PSCell change/CPC in initiating node in case SPR is generated due to T304. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.18.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 10.18.14	Successful PSCell Change Report

The objective of Successful PSCell change Report (SPR) is to detect sub-optimal successful PSCell change/CPC or successful PSCell addition/CPA.

For analysis of such sub-optimal successful PSCell change/CPC and successful PSCell addition/CPA, the UE may collect SPR based on the triggers configured by the network, if received, and makes the SPR available to the network as specified in TS 38.331 [4].

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger for SPR and performs root cause analysis. In case of SN initiated PSCell change/CPC the SPR reported due to T304 may be forwarded also to source SN (initiating node). The information contained in the SPR may help initiating node (MN or SN) to detect sub-optimal successful PSCell change/CPC which may be caused by this node.

For SN-initiated PSCell change/CPC, the source SN decides the T310/T312 triggers for SPR and is responsible for SPR related optimizations e.g., to optimize PSCell change/CPC configuration or associated mobility thresholds or adjust T310/T312 timer values.

For MN-initiated PSCell change/CPC, the MN decides the T310/T312 triggers for SPR. MN may optimize PSCell change/CPC configuration or associated mobility thresholds or both. Source SN may optimize lower layer issues e.g., adjust T310/T312 timer values.

The SPR can be fetched from the UE by the MN only while the UE is still connected to the MN, or by a node different from the MN that sent the SPR configuration to the UE if the UE is not connected to the MN anymore. In case the SPR is retrieved in a node different from the MN that sent the SPR configuration to the UE, the SPR is first forwarded to that MN, which then forwards it to the respective SN(s) which should perform the SPR optimization.

End of Change