3GPP TSG-3 Meeting #123	 R3-240073

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0318 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Initation of TMM procedure for support of mIAB authorization status change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mobile_IAB |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR refers to the scenario where an mIAB-node’s RRC-terminating and F1-termianting IAB-donor-CUs are different, and whose authorization status is changed to “not authorized”. According to clause 8.9.14, the RRC-terminating IAB-donor-CU receives the authorization status update from the AMF and forwards it to the F1-terminating IAB-donor-CU via the Xn IAB TRANSPORT MIGRATION MODIFICATION REQUEST so that the F1-terminating IAB-donor-CU can perform an orderly release of F1. This procedure implicitly assumes that the RRC-terminating IAB-donor-CU can initiate the IAB Transport Migration Modification procedure since the F1-terminating IAB-donor-CU has initiated the IAB Transport Migration Management procedure in opposite direction before. A problem arises in case the mIAB-DU has not carried any traffic before the authorization status change occurs. In this case, the F1-terminating IAB-donor-CU may not have used the IAB Transport Migration Management procedure, and it would therefore be inconsistent with usual procedures, if the RRC-terminating IAB-donor-CU initiated the IAB Transport Migration Modification to send the authorization status change. The following two options can be considered to address this issue: Option 1: The RRC-terminating IAB-donor-CU does not send the status update but immediately removes BH support. In this case, it would not be possible to perform an orderly F1 release, i.e., the F1-terminating IAB-donor-CU would have to derive from the expiration of application-layer timers that F1 support has discontinued. This is not an elegant approach to F1 removal. Option 2: The F1-terminating IAB-donor-CU initates the IAB Transprot Migration Management procedure right after F1 Setup even if it the mIAB-DU doesn’t have any traffic. In this case, it can announce the priority requirement is has for F1-C (and potentially non-F1) traffic. Option 2 can be captured as a Note in section 8.9.14 (Mobile IAB-node authorization). |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note was added to section 8.9.14 on Mobile IAB-node authorization that the F1-terminating IAB-donor-CU should intitiate the IAB Transport Migration Management procedure toward the RRC-terminating IAB-donor-CU at least once after F1 Setup, so that the RRC-terminating IAB-donor-CU can intiate the IAB Transport Migration Modification procedure when it has to send an authorization status change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case authorization status is changed to “un-authorized” before the mIAB-DU carries any traffic, it would not be possible to perform an orderly F1 release, i.e., the F1-terminating IAB-donor-CU would have to derive from the expiration of application-layer timers that F1 support has discontinued. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Begin of Changes ---

### 8.9.14	Mobile IAB-node authorization

During the mobile IAB-node integration procedure, the RRC-terminating IAB-donor-CU receives the authorization status of the mobile IAB-node from the 5GC. If the authorization status is “not authorized”, the RRC-terminating IAB-donor-CU will neither establish any backhaul resources nor allocate any BAP address, TNL address or default BAP configuration for this mobile IAB-node. If the authorization status for the mobile IAB-node changes, the 5GC sends an updated authorization status to the RRC-terminating IAB-donor-CU.

In case the mobile IAB-MT and its co-located mobile IAB-DU connect to same IAB-donor-CU, and the updated authorization status received from the 5GC is “not authorized”, the IAB-donor-CU will perform the following actions in this order: it will attempt to hand over the UEs served by the mobile IAB-node to other cell(s), release the F1 interface towards the mobile IAB-DU, and release all backhaul resources (including the BAP address, TNL address and default BAP reconfiguration) for this mobile IAB-node.

In case the mobile IAB-MT and its co-located mobile IAB-DU connect to different IAB-donor-CUs, the RRC-terminating IAB-donor sends the updated authorization status to the F1-terminating IAB-donor-CU via the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message. The F1-terminating IAB-donor-CU confirms the reception of the updated authorization status via the IAB TRANSPORT MIGRATION MODIFICATION RESPONSE message.

NOTE 1: 	In absence of Xn connectivity between the RRC-terminating IAB-donor-CU and the F1-terminating IAB-donor-CU, the passing of the authorization status is left up to implementation.

NOTE 2:	The F1-terminating IAB-donor-CU should intitiate the IAB Transport Migration Management procedure toward the RRC-terminating IAB-donor-CU at least once after F1 Setup, so that the RRC-terminating IAB-donor-CU can intiate the IAB Transport Migration Modification procedure when it has to send an authorization status change.

If the updated authorization status for the mobile IAB-node is “not authorized”, the F1-terminating IAB-donor, attempts to hand over the UEs served by the mobile IAB-node to other cell(s), and then releases the F1 interface towards the mobile IAB-DU. After that, the F1-terminating IAB-donor requests from the RRC-terminating IAB-donor the release of all the offloaded traffic via the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message. The RRC-terminating IAB-donor releases the offloaded traffic and all backhaul resources (including BAP address, TNL address and default BAP reconfiguration) for this mobile IAB-node. The RRC-terminating IAB-donor may send an indication which indicates that the mobile IAB-MT can be deregistered, to AMF.

If the authorization status is changed back from “not authorized” to “authorized”, the phase 2 and phase 3 of the mobile IAB-node integration procedure as defined in clause 8.12.3 are carried out.

--- End of Changes ---