3GPP TSG-RAN WG3 Meeting #123	R3-240128

# Athens, Greece, 26th February– 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1055 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for R18 Assistance Information for QoE Measurement IE exchanges during NG handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R18, the Assistance Information for QoE Measurement IE is introduced in QMC Configuration Information IE of INITIAL CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message to indicate the suggested priority of the application layer measurement configuration from AMF. From the current specification, for NG handover, the Assistance Information for QoE Measurement IE is already included in QMC Configuration Information IE of Source NG-RAN Node to Target NG-RAN Node Transparent Container in HANDOVER REQUEST message and HANDOVER REQUIRED message. However, there is no text description about the Assistance Information for QoE Measurement IE for mobility over NG handover. From our point of view, the Assistance Information for QoE Measurement IE transmission between two NG-RANs should be supported in RRC_CONNECTED/RRC_INACTIVE/RRC_IDLE states since it is helpful to the new serving NG-RAN memory management for storing application layer measurement report. For the RRC_CONNECTED states, the Assistance Information for QoE Measurement IE should be exchanged between two NG-RAN via Xn/NG interfaces. For RRC_INACTIVE/RRC_IDLE states, the UE needs to store Assistance Information for QoE Measurement IE when UE changes from RRC_CONNECTED to RRC_INACTIVE/RRC_IDLE and then send the Assistance Information for QoE Measurement IE to the new serving gNB when the UE enters RRC_ CONNECTED state. Therefore, we suggest adding the NG-RAN behavior descriptions for the NG interface to support the Assistance Information for QoE Measurement IE transmission during the NG handover between NG-RANs. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) as it only adds corresponding procedure text for the Assistance Information for QoE Measurement IE in HANDOVER REQUEST message and HANDOVER REQUIRED message that is optional presence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the RAN behavior descriptions if Assistance Information for QoE Measurement IE is included in QMC Configuration Information IE of Source NG-RAN Node to Target NG-RAN Node Transparent Container in HANDOVER REQUEST message and HANDOVER REQUIRED message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no support for Assistance Information for QoE Measurement IE exchange during NG handover, and the target NG-RAN may be unable to optimize its memory management for storing application layer measurement reports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2, 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change part

8.4.1	Handover Preparation

8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the 5GC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE. The procedure uses UE-associated signalling.

8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

If the Index to RAT/Frequency Selection Priority IE is contained in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE, the target NG-RAN node shall store the content of the received Index to RAT/Frequency Selection Priority IE in the UE context and use it as defined in TS 23.501 [9].

If the DAPS Request Information IE is included for a DRB in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE within the HANDOVER REQUIRED message, it indicates that the request concerns a DAPS Handover for that DRB, as described in TS 38.300 [8].

If the Assistance Information for QoE Measurement IE is included in the UE Application Layer Measurement Configuration Information IE within the QMC Configuration Information IE, the NG-RAN node may take it into account for controlling the UE reporting when the NG-RAN node is overloaded, as described in TS 38.300 [8].

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

Next change

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

If the HANDOVER REQUEST message contains within the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE the Time Based Handover Information IE, the target NG-RAN node may use this information to allocate necessary resources for the incoming handover.

If the Candidate Relay UE Information List IE is included in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE within the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to configure the path switch to indirect path as specified in TS 38.300 [8].

If the Assistance Information for QoE Measurement IE is included in the UE Application Layer Measurement Configuration Information IE within the QMC Configuration Information IE, the NG-RAN node may take it into account for controlling the UE reporting when the NG-RAN node is overloaded, as described in TS 38.300 [8].

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

End of change part