3GPP TSG-RAN3 Meeting #123	R3-240132

# Athens, GR, Feb 27th – Mar 1st , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1257 | rev | - | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SL destination transmission over F1 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For NR V2X, when CU receiving the SUI reporting from UE, CU will based on the reporting order of the destinations within TxResourceReqList perform sequenced indexing from destination identity to destination index. Later UE will use the sequenced destination index for BSR reporting. While currently CU will not forward the ordered destinations towards DU, so that DU cannot understand the reported BSR correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In A.I. 8.3.1.2, add descrption of forwarding mechanism of TxResourceReqList from CU to DU within UE CONTEXT SETUP REQUEST message - In A.I. 8.3.4.2, add description of forwarding mechanism of TxResourceReqList from CU to DU within UE CONTEXT MODIFICATION REQUEST message - In A.I. 9.2.2, add relevant IE in UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message - In A.I. 9.4 Add relevant parameters Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) on the sidelink UE destination ID reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DU cannot understand BSR reported via NR V2X UE so that cannot allocate resource correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.4.2, 9.2.2.1,9.2.2.7, 9.4.4 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

8	F1AP procedures

8.3	UE Context Management procedures

8.3.1	UE Context Setup

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB. If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE as follows:

-	if the IP to layer2 Traffic Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the IP to layer2 Mapping Info To Add IE, if present, for the egress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the IP to layer2 Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored for the mapping of IP traffic to layer 2, as specified in TS 38.340 [30].

-	if the BAP layer BH RLC channel Mapping Info IE is included, the gNB-DU shall store the mapping information  contained in the BAP layer BH RLC channel Mapping Info To Add IE, if present, for the egress or ingress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the BAP layer BH RLC channel Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored when forwarding traffic on BAP sublayer, as specified in TS 38.340 [30].

If two UL UP TNL Information IEs are included in UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT SETUP RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT SETUP RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating CA based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when activating/deactivating CA based PDCP duplication for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. If DC Based Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

If UL PDCP SN length IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall, if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is available at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT SETUP REQUEST. The gNB-DU may use it for RRM purposes.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs, SRBs and BH RLC channels in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to Setup List IE.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15].

For NG-RAN operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE, or at least one BH RLC channel is set up for IAB-MT. If the HandoverPreparationInformation IE containing the sidelink related UE information is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall deduce that changes to the measurements configuration need to be applied. If the measObjectToAddModList IE is included in the MeasConfig IE, then the frequencies added in such IE are to be activated. Then the gNB-DU shall decide if measurement gaps are needed or not and, if needed, the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. If the measObjectToRemoveList IE is included in the MeasConfig IE, the gNB-DU shall ignore it. If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT SETUP REQUEST message shall be ignored. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

For sidelink operation, the NR V2X destination identity IE in TxResourceReqList IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE via SidelinkUEInformationNR message; if the TxResourceReqList IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the Masked IMEISV IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT SETUP REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT SETUP RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the ServCellInfoList IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall take it into account to generate the content of inter-node RRC message, i.e., CG-Config or CG-ConfigInfo, as described in TS 38.331 [8].

If the Full Configuration IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the Notification Control IE is included in the DRB to Be Setup List IE contained in the UE CONTEXT SETUP REQUEST message and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

The gNB-DU shall store the received gNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the UE CONTEXT SETUP REQUEST message contains the QoS Flow Mapping Indication IE, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the UE CONTEXT SETUP REQUEST message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the RAN UE ID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store and replace any previous information received.

If the Trace Activation IE is included in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, initiate the requested trace function as described in TS 32.422 [29].

In particular, the gNB-DU shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [29];

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT Only", initiate the requested MDT session as described in TS 32.422 [29] and the gNB-DU shall ignore Interfaces To Trace IE, and Trace Depth IE. If the Management Based MDT PLMN List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [29].

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the UE CONTEXT SETUP REQUEST message contains the Configured BAP Address IE, the gNB-DU shall, if supported, store this BAP address configured for the corresponding child IAB-node and use it as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the NR V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services.

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the Conditional Inter-DU Mobility Information IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Target gNB-DU UE F1AP ID IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall replace the existing prepared conditional handover or conditional PSCell change identified by the Target gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Serving NID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and may take it into account for UE context establishment.

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

Next change

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message and the indicated SCell(s) are already setup, the gNB-DU shall replace any previously received value. If the SCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the SCell To Be Removed List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of SCells to be removed.

If the DRX Cycle IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the provided value from the gNB-CU. If the DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall release DRX configuration.

If the SRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4], and replace any previously received value. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB if the value is set to be "true", or delete the RLC entity of secondary path if the value is set to be "false". If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB.

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Modified Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall release the BH RLC channels in the list.

If two UL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT MODIFICATION RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating CA based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. Otherwise, the gNB-DU shall regard that DC based PDCP duplication is de-configured for this DRB id the value is set to be "false", and it should stop PDCP duplication activation/deactivation by MAC CE. If DC Based Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the UE Context Modification procedure, the gNB-DU shall consider that the CA based PDCP duplication for the concerned DRB is de-configured. If such UE Context Modification procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the UL Configuration IE in DRB to Be Setup Item IE or DRB to Be Modified Item IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the RRC Reconfiguration Complete Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider the ongoing reconfiguration procedure involving changes of the L1/L2 configuration at the gNB-DU signalled to the gNB-CU via the CellGroupConfig IE for MR-DC operation or standalone operation has been successfully performed when such IE is set to ‘true’; otherwise (when such IE is set to ‘failure’), the gNB-DU shall consider the ongoing reconfiguration procedure has been failed and it shall continue to use the old L1/L2 configuration.

If DL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If UL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If the RLC Failure Indication IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU should consider that the RLC entity indicated by such IE needs to be re-established when the CA-based packet duplication is active, and the gNB-DU may include the Associated SCell List IE in UE CONTEXT MODIFICATION RESPONSE by containing a list of SCell(s) associated with the RLC entity indicated by the RLC Failure Indication IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE. If the UE CONTEXT MODIFICATION REQUEST message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the Transmission Action Indicator IE, the gNB-DU shall stop or restart (if already stopped) data transmission for the UE, according to the value of this IE. It is up to gNB-DU implementation when to stop or restart the UE scheduling.

For EN-DC operation, if the DRB to Be Setup List IE is present in the UE CONTEXT MODIFICATION REQUEST message the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15]. For NG-RAN operation, the gNB-CU shall include the DRB Information IE in the UE CONTEXT MODIFICATION REQUEST message.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall deduce that changes to the measurements’ configuration need to be applied. The gNB-DU shall take the received info, e.g. the measObjectToAddModList IE, and/or the measObjectToRemoveList IE into account, when generating measurement gap and when deciding if a measurement gap is needed or not. If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For DC operation, if the gNB-CU includes the CG-Config IE in the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may initiate low layer parameters coordination taking this information into account.

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION REQUEST message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT MODIFICTION REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT MODIFICATION REQUEST message shall , if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is modified at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT MODIFICATION REQUEST. The gNB-DU may use it for RRM purposes.

Only one of the Uplink TxDirectCurrentList Information IE and Uplink TxDirectCurrentTwoCarrierList Information IE IE may be contained in the UE CONTEXT MODIFICATION REQUEST message. If the UE CONTEXT MODIFICATION REQUEST message contains one of the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE, the gNB-DU may take that into account when selecting L1 configuration.

For sidelink operation, the NR V2X destination identity IE in TxResourceReqList IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE via SidelinkUEInformationNR message; if the TxResourceReqList IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT MODIFICATION RESPONSE message, the result for all the requested or modified DRBs, SRBs and BH RLC Channels in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to be Setup List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed to be Modified List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to be Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of successfully modified SRBs with logical channel identities for primary path shall be included in the SRB Modified List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of BH RLC channels which are successfully modified shall be included in the BH RLC Channel Modified List IE;

-	A list of BH RLC channels which failed to be modified shall be included in the BH RLC Channel Failed to be Modified List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to be Setup List IE;

-	A list of SL DRBs which are successfully modified shall be included in the SL DRB Modified List IE;

-	A list of SL DRBs which failed to be modified shall be included in the SL DRB Failed to be Modified List IE.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Modified List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30]. Otherwise, if the BAP Control PDU Channel IE is not present for any BH RLC channel, any available BH RLC channel can be used to transmit BAP Control PDUs as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the ServCellInfoList IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall take it into account to generate the content of inter-node message, i.e., CG-Config or CG-ConfigInfo, as described in TS 38.331 [8].

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP MODIFICATION REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the C-RNTI IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT MODIFICATION RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

The UE Context Modify Procedure is not used to configure SRB0.

If in the UE CONTEXT MODIFICATION REQUEST, the Notification Control IE is included in the DRB to Be Setup List IE or the DRB to Be Modified List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the received UL PDU Session Aggregate Maximum Bit Rate and use it as specified in TS 23.501 [21].

If the gNB-DU UE Aggregate Maximum Bit Rate Uplink IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall:

-	replace the previously provided gNB-DU UE Aggregate Maximum Bit Rate Uplink with the new received gNB-DU UE Aggregate Maximum Bit Rate Uplink;

-	use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink for non-GBR Bearers for the concerned UE.

The gNB-DU UE Aggregate Maximum Bit Rate Uplink IE shall be sent in the UE CONTEXT MODIFICATION REQUEST if DRB to Be Setup List IE is included and the gNB-CU has not previously sent it. The gNB-DU shall store and use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink IE.

If the RLC Status IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the CellGroupConfig IE in the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Bearer Type Change IE is included in DRB to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall either reset the lower layers or generate a new LCID for the affected bearer as specified in TS 37.340 [7].

For NE-DC operation, if NeedforGap IE is included in the UE CONTEXT MODIFICATION REQUEST message,the gNB-DU shall generate measurement gap for the SeNB.

If the QoS Flow Mapping Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Lower Layer presence status change IE set to "suspend lower layers" is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall keep all lower layer configuration for UEs, and not transmit or receive data from UE.

If the Lower Layer presence status change IE set to "resume lower layers" is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the previously stored lower layer configuration for the UE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the NR V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the NR V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the LTE V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE LTE Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE NR Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

If the PC5 Link Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE PC5 Link Aggregate Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-initiation", the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-replace", the gNB-DU shall replace the existing prepared conditional mobility identified by the gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-cancel", the gNB-DU shall consider that the gNB-CU is about to remove any reference to, and release any resources previously reserved for the candidate cells associated to the UE-associated signalling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE. If the Candidate Cells To Be Cancelled List IE is also included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that only the resources reserved for the cells identified by the included NR CGIs are about to be released by the gNB-CU.

If the Transmission Stop Indicator IE is included within the DRB to Be Modified Item IE in the UE CONTEXT MODIFICATION REQUEST message and set to “true”, the gNB-DU shall, if supported, stop the data transmission for the DRB. It is up to gNB-DU implementation when to stop the UE scheduling for that DRB.

If the SCG Indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and it is set to “released”, the gNB-DU shall, if supported, deduce that an SCG is removed.

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the Location Measurement Information IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring measurement gaps for the UE.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

Next change

9	Elements for F1AP Communication

9.2	Message Functional Definition and Content

9.2.2	UE Context Management messages

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS Information | M |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| Configured BAP Address | O |  | 9.3.1.111 | The BAP address configured for the corresponding child IAB-node. | YES | reject |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier |  |  | 9.3.1.121 |  | - |  |
| >>RLC mode | M |  | 9.3.1.27 |  | - |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.151 |  | YES | ignore |
| Serving NID | O |  | 9.3.1.155 |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | Reject |
| TxResourceReqList |  | 0..1 |  |  | YES | reject |
| >TxResourceReq Item IEs |  | 1 .. <maxnoofSLDestIDs> |  |  | EACH | reject |
| >>NR V2X destination identity | M |  | INTEGER (1..32) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSLDestIDs | Maximum no. of destination identity allowed to be maintained within one UE, the maximum value is 32. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

Next change

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| DAPS HO status | O |  | ENUMERATED(initiation, …) | This IE is used if DAPS HO is initiated. | YES | ignore |
| Uplink TxDirectCurrentTwoCarrierList Information | O |  | 9.3.1.283 |  | YES | ignore |
| TxResourceReq To Be Modified List |  | 0..1 |  |  | YES | reject |
| >TxResourceReq To Be Modified Item IEs |  | 1 .. <maxnoofSLDestIDs> |  |  | EACH | reject |
| >>NR V2X destination identity | M |  | INTEGER (1..32) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofSLDestIDs | Maximum no. of destination identity allowed to be maintained within one UE, the maximum value is 32 |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

Next change

9.4	Message and Information Element Abstract Syntax (with ASN.1)

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Allowed-to-be-Deactivated-List-Item,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

EUTRANQoS,

ExecuteDuplication,

FullConfiguration,

GNB-CU-MBS-F1AP-ID,

GNB-CU-UE-F1AP-ID,

GNB-DU-MBS-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

LowerLayerPresenceStatusChange,

MBS-Area-Session-ID,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

MulticastMRBs-Setup-Item,

MulticastMRBs-FailedToBeSetup-Item,

MulticastMRBs-ToBeSetupMod-Item,

MulticastMRBs-ToBeModified-Item,

MulticastMRBs-ToBeReleased-Item,

MulticastMRBs-SetupMod-Item,

MulticastMRBs-FailedToBeSetupMod-Item,

MulticastMRBs-Modified-Item,

MulticastMRBs-FailedToBeModified-Item,

BroadcastAreaScope,

NetworkControlledRepeaterAuthorized,

NotificationControl,

NRCGI,

NRPCI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

RequestedSRSTransmissionCharacteristics,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCContainer-RRCSetupComplete,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

SDT-Volume-Threshold,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SNSSAI,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

SupportedUETypeList,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

UEIdentity-List-For-Paging-Item,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

NotificationInformation,

TraceActivation,

TraceID,

Neighbour-Cell-Information-Item,

SymbolAllocInSlot,

NumDLULSymbols,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Address-Info,

BHChannels-ToBeSetup-Item,

BHChannels-Setup-Item,

BHChannels-FailedToBeSetup-Item,

BHChannels-ToBeModified-Item,

BHChannels-ToBeReleased-Item,

BHChannels-ToBeSetupMod-Item,

BHChannels-FailedToBeModified-Item,

BHChannels-FailedToBeSetupMod-Item,

BHChannels-Modified-Item,

BHChannels-SetupMod-Item,

BHChannels-Required-ToBeReleased-Item,

BAPAddress,

BAPPathID,

BAPRoutingID,

BH-Routing-Information-Added-List-Item,

BH-Routing-Information-Removed-List-Item,

Child-Nodes-List,

Child-Nodes-List-Item,

Child-Node-Cells-List,

Child-Node-Cells-List-Item,

Activated-Cells-to-be-Updated-List,

Activated-Cells-to-be-Updated-List-Item,

UL-BH-Non-UP-Traffic-Mapping,

IABTNLAddressesRequested,

IABIPv6RequestType,

IAB-TNL-Addresses-To-Remove-Item,

IABTNLAddress,

IAB-Allocated-TNL-Address-Item,

IABv4AddressesRequested,

TrafficMappingInfo,

UL-UP-TNL-Information-to-Update-List-Item,

UL-UP-TNL-Address-to-Update-List-Item,

DL-UP-TNL-Address-to-Update-List-Item,

NRV2XServicesAuthorized,

LTEV2XServicesAuthorized,

NRUESidelinkAggregateMaximumBitrate,

LTEUESidelinkAggregateMaximumBitrate,

SLDRBs-SetupMod-Item,

SLDRBs-ModifiedConf-Item,

SLDRBID,

SLDRBs-FailedToBeModified-Item,

SLDRBs-FailedToBeSetup-Item,

SLDRBs-FailedToBeSetupMod-Item,

SLDRBs-Modified-Item,

SLDRBs-Required-ToBeModified-Item,

SLDRBs-Required-ToBeReleased-Item,

SLDRBs-Setup-Item,

SLDRBs-ToBeModified-Item,

SLDRBs-ToBeReleased-Item,

SLDRBs-ToBeSetup-Item,

SLDRBs-ToBeSetupMod-Item,

GNBCUMeasurementID,

GNBDUMeasurementID,

RegistrationRequest,

ReportCharacteristics,

CellToReportList,

HardwareLoadIndicator,

CellMeasurementResultList,

ReportingPeriodicity,

TNLCapacityIndicator,

RAReportList,

RLFReportInformationList,

ReportingRequestType,

TimeReferenceInformation,

ConditionalInterDUMobilityInformation,

ConditionalIntraDUMobilityInformation,

TargetCellList,

MDTPLMNList,

PrivacyIndicator,

TransportLayerAddress,

URI-address,

NID,

PosAssistance-Information,

PosBroadcast,

PositioningBroadcastCells,

RoutingID,

PosAssistanceInformationFailureList,

PosMeasurementQuantities,

PosMeasurementResultList,

PosReportCharacteristics,

TRPInformationTypeItem,

TRPInformationItem,

LMF-MeasurementID,

RAN-MeasurementID,

SDT-Termination-Request,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

SRSConfiguration,

TRPList,

E-CID-MeasurementQuantities,

MeasurementPeriodicity,

E-CID-MeasurementResult,

Cell-Portion-ID,

LMF-UE-MeasurementID,

RAN-UE-MeasurementID,

RelativeTime1900,

SystemFrameNumber,

SlotNumber,

AbortTransmission,

TRP-MeasurementRequestList,

MeasurementBeamInfoRequest,

E-CID-ReportCharacteristics,

Extended-GNB-CU-Name,

Extended-GNB-DU-Name,

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

SuccessfulHOReportInformationList,

Coverage-Modification-Notification,

CCO-Assistance-Information,

CellsForSON-List,

IABCongestionIndication,

IABConditionalRRCMessageDeliveryIndication,

F1CTransferPathNRDC,

BufferSizeThresh,

IAB-TNL-Addresses-Exception,

BAP-Header-Rewriting-Added-List-Item,

Re-routingEnableIndicator,

NonF1terminatingTopologyIndicator,

EgressNonF1terminatingTopologyIndicator,

IngressNonF1terminatingTopologyIndicator,

Neighbour-Node-Cells-List,

Neighbour-Node-Cells-List-Item,

NA-Resource-Configuration-List,

NA-Resource-Configuration-Item,

Serving-Cells-List,

Serving-Cells-List-Item,

RBSetConfiguration,

PDCMeasurementPeriodicity,

PDCMeasurementQuantities,

PDCMeasurementResult,

PDCReportType,

RAN-UE-PDC-MeasID,

SCGActivationRequest,

SCGActivationStatus,

TRP-MeasurementUpdateList,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

TRP-PRS-Info-List,

PRS-Measurement-Info-List,

PRSConfigRequestType,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

UEReportingInformation,

PosConextRevIndication,

NRRedCapUEIndication,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

QoEInformation,

CG-SDTQueryIndication,

CG-SDTKeptIndicator,

CG-SDTSessionInfo,

SDTInformation,

FiveG-ProSeAuthorized,

UuRLCChannelToBeSetupList,

UuRLCChannelToBeModifiedList,

UuRLCChannelToBeReleasedList,

UuRLCChannelSetupList,

UuRLCChannelFailedToBeSetupList,

UuRLCChannelModifiedList,

UuRLCChannelFailedToBeModifiedList,

UuRLCChannelRequiredToBeModifiedList,

UuRLCChannelRequiredToBeReleasedList,

PC5RLCChannelToBeSetupList,

PC5RLCChannelToBeModifiedList,

PC5RLCChannelToBeReleasedList,

PC5RLCChannelSetupList,

PC5RLCChannelFailedToBeSetupList,

PC5RLCChannelFailedToBeModifiedList,

PC5RLCChannelRequiredToBeModifiedList,

PC5RLCChannelRequiredToBeReleasedList,

PC5RLCChannelModifiedList,

RemoteUELocalID,

PathSwitchConfiguration,

SidelinkRelayConfiguration,

PagingCause,

PEIPSAssistanceInfo,

UEPagingCapability,

GNBDUUESliceMaximumBitRateList,

MDTPollutedMeasurementIndicator,

UE-MulticastMRBs-ConfirmedToBeModified-Item,

UE-MulticastMRBs-RequiredToBeModified-Item,

UE-MulticastMRBs-RequiredToBeReleased-Item,

UE-MulticastMRBs-Setup-Item,

UE-MulticastMRBs-Setupnew-Item,

UE-MulticastMRBs-ToBeReleased-Item,

UE-MulticastMRBs-ToBeSetup-Item,

UE-MulticastMRBs-ToBeSetup-atModify-Item,

PosMeasurementAmount,

BAP-Header-Rewriting-Removed-List-Item,

SLDRXCycleList,

MDTPLMNModificationList,

ActivationRequestType,

PosMeasGapPreConfigList,

PosMeasurementPeriodicityNR-AoA,

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

ServingCellMO-List-Item,

ServingCellMO-encoded-in-CGC-List,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

DedicatedSIDeliveryIndication,

Configured-BWP-List,

MT-SDT-Information,

LTMInformation-Setup,

LTMConfigurationIDMappingList,	LTMInformation-Modify,

LTMCells-ToBeReleased-List,	LTMConfiguration,

EarlySyncInformation-Request,

EarlySyncInformation,

EarlySyncInformation-List,

LTMCellSwitchInformation,

TAInformation-List,

DeactivationIndication,

RAReportIndicationList,

SuccessfulPSCellChangeReportInformationList,

PathAdditionInformation,

RANTSSRequestType,

RANTimingSynchronisationStatusInfo,

GlobalGNB-ID,

Activated-Cells-Mapping-List-Item,

RRC-Terminating-IAB-Donor-Related-Info,

NCGI-to-be-Updated-List-Item,

Mobile-IAB-MTUserLocationInformation,

TAI,

IndicationMCInactiveReception,

MulticastCU2DURRCInfo,

MulticastDU2CURRCInfo,

MBSMulticastSessionReceptionState,

F1UTunnelNotEstablished,

MulticastCU2DUCommonRRCInfo,

NRA2XServicesAuthorized,

LTEA2XServicesAuthorized,

NReRedCapUEIndication,

NRPaginglongeDRXInformationforRRCINACTIVE,

Cells-With-SSBs-Activated-List,

Recommended-SSBs-for-Paging-List,

TxResourceReq-Item,

TxResourceReq-ToBeModified-Item,

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

id-BroadcastMRBs-SetupMod-Item,

id-BroadcastMRBs-ToBeModified-List,

id-BroadcastMRBs-ToBeModified-Item,

id-BroadcastMRBs-ToBeReleased-List,

id-BroadcastMRBs-ToBeReleased-Item,

id-BroadcastMRBs-ToBeSetup-List,

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-Cells-Allowed-to-be-Deactivated-List,

id-Cells-Allowed-to-be-Deactivated-List-Item,

id-Cells-With-SSBs-Activated-List,

id-Recommended-SSBs-for-Paging-List,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-MBS-F1AP-ID,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-MBS-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-Extended-GNB-CU-Name,

id-Extended-GNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-MBS-Area-Session-ID,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

id-MulticastMRBs-FailedToBeSetupMod-Item,

id-MulticastMRBs-Modified-List,

id-MulticastMRBs-Modified-Item,

id-MulticastMRBs-Setup-List,

id-MulticastMRBs-Setup-Item,

id-MulticastMRBs-SetupMod-List,

id-MulticastMRBs-SetupMod-Item,

id-MulticastMRBs-ToBeModified-List,

id-MulticastMRBs-ToBeModified-Item,

id-MulticastMRBs-ToBeReleased-List,

id-MulticastMRBs-ToBeReleased-Item,

id-MulticastMRBs-ToBeSetup-List,

id-MulticastMRBs-ToBeSetup-Item,

id-MulticastMRBs-ToBeSetupMod-List,

id-MulticastMRBs-ToBeSetupMod-Item,

id-MulticastF1UContext-ToBeSetup-List,

id-MulticastF1UContext-ToBeSetup-Item,

id-MulticastF1UContext-Setup-List,

id-MulticastF1UContext-Setup-Item,

id-MulticastF1UContext-FailedToBeSetup-List,

id-MulticastF1UContext-FailedToBeSetup-Item,

id-BroadcastAreaScope,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-RequestedSRSTransmissionCharacteristics,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCContainer-RRCSetupComplete,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SDT-Termination-Request,

id-SDT-Volume-Threshold,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SNSSAI,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-SupportedUETypeList,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-UEIdentity-List-For-Paging-List,

id-UEIdentity-List-For-Paging-Item,

id-UE-MulticastMRBs-ConfirmedToBeModified-List,

id-UE-MulticastMRBs-ConfirmedToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeModified-List,

id-UE-MulticastMRBs-RequiredToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeReleased-List,

id-UE-MulticastMRBs-RequiredToBeReleased-Item,

id-UE-MulticastMRBs-Setup-List,

id-UE-MulticastMRBs-Setup-Item,

id-UE-MulticastMRBs-Setupnew-List,

id-UE-MulticastMRBs-Setupnew-Item,

id-UE-MulticastMRBs-ToBeReleased-List,

id-UE-MulticastMRBs-ToBeReleased-Item,

id-UE-MulticastMRBs-ToBeSetup-atModify-List,

id-UE-MulticastMRBs-ToBeSetup-atModify-Item,

id-UE-MulticastMRBs-ToBeSetup-List,

id-UE-MulticastMRBs-ToBeSetup-Item,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-NotificationInformation,

id-TraceActivation,

id-TraceID,

id-Neighbour-Cell-Information-List,

id-Neighbour-Cell-Information-Item,

id-SymbolAllocInSlot,

id-NumDLULSymbols,

id-AdditionalRRMPriorityIndex,

id-DUCURadioInformationType,

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Address-Info,

id-BHChannels-ToBeSetup-List,

id-BHChannels-ToBeSetup-Item,

id-BHChannels-Setup-List,

id-BHChannels-Setup-Item,

id-BHChannels-ToBeModified-Item,

id-BHChannels-ToBeModified-List,

id-BHChannels-ToBeReleased-Item,

id-BHChannels-ToBeReleased-List,

id-BHChannels-ToBeSetupMod-Item,

id-BHChannels-ToBeSetupMod-List,

id-BHChannels-FailedToBeSetup-Item,

id-BHChannels-FailedToBeSetup-List,

id-BHChannels-FailedToBeModified-Item,

id-BHChannels-FailedToBeModified-List,

id-BHChannels-FailedToBeSetupMod-Item,

id-BHChannels-FailedToBeSetupMod-List,

id-BHChannels-Modified-Item,

id-BHChannels-Modified-List,

id-BHChannels-SetupMod-Item,

id-BHChannels-SetupMod-List,

id-BHChannels-Required-ToBeReleased-Item,

id-BHChannels-Required-ToBeReleased-List,

id-BAPAddress,

id-ConfiguredBAPAddress,

id-BH-Routing-Information-Added-List,

id-BH-Routing-Information-Added-List-Item,

id-BH-Routing-Information-Removed-List,

id-BH-Routing-Information-Removed-List-Item,

id-UL-BH-Non-UP-Traffic-Mapping,

id-Child-Nodes-List,

id-Activated-Cells-to-be-Updated-List,

id-IABIPv6RequestType,

id-IAB-TNL-Addresses-To-Remove-List,

id-IAB-TNL-Addresses-To-Remove-Item,

id-IAB-Allocated-TNL-Address-List,

id-IAB-Allocated-TNL-Address-Item,

id-IABv4AddressesRequested,

id-TrafficMappingInformation,

id-UL-UP-TNL-Information-to-Update-List,

id-UL-UP-TNL-Information-to-Update-List-Item,

id-UL-UP-TNL-Address-to-Update-List,

id-UL-UP-TNL-Address-to-Update-List-Item,

id-DL-UP-TNL-Address-to-Update-List,

id-DL-UP-TNL-Address-to-Update-List-Item,

id-NRV2XServicesAuthorized,

id-LTEV2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrate,

id-LTEUESidelinkAggregateMaximumBitrate,

id-PC5LinkAMBR,

id-SLDRBs-FailedToBeModified-Item,

id-SLDRBs-FailedToBeModified-List,

id-SLDRBs-FailedToBeSetup-Item,

id-SLDRBs-FailedToBeSetup-List,

id-SLDRBs-Modified-Item,

id-SLDRBs-Modified-List,

id-SLDRBs-Required-ToBeModified-Item,

id-SLDRBs-Required-ToBeModified-List,

id-SLDRBs-Required-ToBeReleased-Item,

id-SLDRBs-Required-ToBeReleased-List,

id-SLDRBs-Setup-Item,

id-SLDRBs-Setup-List,

id-SLDRBs-ToBeModified-Item,

id-SLDRBs-ToBeModified-List,

id-SLDRBs-ToBeReleased-Item,

id-SLDRBs-ToBeReleased-List,

id-SLDRBs-ToBeSetup-Item,

id-SLDRBs-ToBeSetup-List,

id-SLDRBs-ToBeSetupMod-Item,

id-SLDRBs-ToBeSetupMod-List,

id-SLDRBs-SetupMod-List,

id-SLDRBs-FailedToBeSetupMod-List,

id-SLDRBs-SetupMod-Item,

id-SLDRBs-FailedToBeSetupMod-Item,

id-SLDRBs-ModifiedConf-List,

id-SLDRBs-ModifiedConf-Item,

id-gNBCUMeasurementID,

id-gNBDUMeasurementID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReportList,

id-CellMeasurementResultList,

id-HardwareLoadIndicator,

id-ReportingPeriodicity,

id-TNLCapacityIndicator,

id-RAReportList,

id-RLFReportInformationList,

id-ReportingRequestType,

id-TimeReferenceInformation,

id-ConditionalInterDUMobilityInformation,

id-ConditionalIntraDUMobilityInformation,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-TraceCollectionEntityIPAddress,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityURI,

id-ServingNID,

id-PosAssistance-Information,

id-PosBroadcast,

id-PositioningBroadcastCells,

id-RoutingID,

id-PosAssistanceInformationFailureList,

id-PosMeasurementQuantities,

id-PosMeasurementResultList,

id-PosMeasurementPeriodicity,

id-PosReportCharacteristics,

id-TRPInformationTypeListTRPReq,

id-TRPInformationTypeItem,

id-TRPInformationListTRPResp,

id-TRPInformationItem,

id-LMF-MeasurementID,

id-RAN-MeasurementID,

id-SRSType,

id-ActivationTime,

id-AbortTransmission,

id-SRSConfiguration,

id-TRPList,

id-E-CID-MeasurementQuantities,

id-E-CID-MeasurementPeriodicity,

id-E-CID-MeasurementResult,

id-Cell-Portion-ID,

id-LMF-UE-MeasurementID,

id-RAN-UE-MeasurementID,

id-SFNInitialisationTime,

id-SystemFrameNumber,

id-SlotNumber,

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-PosMeasurementPeriodicityExtended,

id-SuccessfulHOReportInformationList,

id-Coverage-Modification-Notification,

id-CCO-Assistance-Information,

id-CellsForSON-List,

id-IABCongestionIndication,

id-IABConditionalRRCMessageDeliveryIndication,

id-F1CTransferPathNRDC,

id-BufferSizeThresh,

id-IAB-TNL-Addresses-Exception,

id-BAP-Header-Rewriting-Added-List,

id-BAP-Header-Rewriting-Added-List-Item,

id-Re-routingEnableIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-IngressNonF1terminatingTopologyIndicator,

id-Neighbour-Node-Cells-List,

id-Serving-Cells-List,

id-MDTPollutedMeasurementIndicator,

id-PDCMeasurementPeriodicity,

id-PDCMeasurementQuantities,

id-PDCMeasurementResult,

id-PDCReportType,

id-RAN-UE-PDC-MeasID,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-TRP-MeasurementUpdateList,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-TRP-PRS-Info-List,

id-PRS-Measurement-Info-List,

id-PRSConfigRequestType,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-UEReportingInformation,

id-PosConextRevIndication,

id-NRRedCapUEIndication,

id-RANUEPagingDRX,

id-CNUEPagingDRX,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-QoEInformation,

id-CG-SDTQueryIndication,

id-CG-SDTKeptIndicator,

id-CG-SDTSessionInfoOld,

id-SDTInformation,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5LinkAMBR,

id-FiveG-ProSeUEPC5AggregateMaximumBitrate,

id-UuRLCChannelToBeSetupList,

id-UuRLCChannelToBeModifiedList,

id-UuRLCChannelToBeReleasedList,

id-UuRLCChannelSetupList,

id-UuRLCChannelFailedToBeSetupList,

id-UuRLCChannelModifiedList,

id-UuRLCChannelFailedToBeModifiedList,

id-UuRLCChannelRequiredToBeModifiedList,

id-UuRLCChannelRequiredToBeReleasedList,

id-PC5RLCChannelToBeSetupList,

id-PC5RLCChannelToBeModifiedList,

id-PC5RLCChannelToBeReleasedList,

id-PC5RLCChannelSetupList,

id-PC5RLCChannelFailedToBeSetupList,

id-PC5RLCChannelModifiedList,

id-PC5RLCChannelFailedToBeModifiedList,

id-PC5RLCChannelRequiredToBeModifiedList,

id-PC5RLCChannelRequiredToBeReleasedList,

id-SidelinkRelayConfiguration,

id-UpdatedRemoteUELocalID,

id-PathSwitchConfiguration,

id-PagingCause,

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-PosMeasurementAmount,

id-BAP-Header-Rewriting-Removed-List,

id-BAP-Header-Rewriting-Removed-List-Item,

id-SLDRXCycleList,

id-ManagementBasedMDTPLMNModificationList,

id-ActivationRequestType,

id-PosMeasGapPreConfigList,

id-PosMeasurementPeriodicityNR-AoA,

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-ServingCellMO-List,

id-ServingCellMO-List-Item,

id-ServingCellMO-encoded-in-CGC-List,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-DedicatedSIDeliveryIndication,

id-Configured-BWP-List,

id-NetworkControlledRepeaterAuthorized,

id-MT-SDT-Information,

id-LTMInformation-Setup,	id-LTMConfigurationIDMappingList,

id-LTMInformation-Modify,

id-LTMCells-ToBeReleased-List,

id-LTMConfiguration,

id-EarlySyncInformation-Request,

id-EarlySyncInformation,

id-EarlySyncInformation-List,

id-LTMCellSwitchInformation,

id-TAInformation-List,

id-Source-gNB-DU-ID,

id-DeactivationIndication,

id-RAReportIndicationList,

id-SuccessfulPSCellChangeReportInformationList,

id-PathAdditionInformation,

id-RANTSSRequestType,

id-RANTimingSynchronisationStatusInfo,

id-Target-gNB-ID,

id-Target-gNB-IP-address,

id-Target-SeGW-IP-address,

id-Activated-Cells-Mapping-List,

id-Activated-Cells-Mapping-List-Item,

id-F1SetupOutcome,

id-RRC-Terminating-IAB-Donor-Related-Info,

id-RRC-Terminating-IAB-Donor-gNB-ID,

id-NCGI-to-be-Updated-List,

id-NCGI-to-be-Updated-List-Item,

id-Mobile-IAB-MTUserLocationInformation,

id-IndicationMCInactiveReception,

id-MulticastCU2DURRCInfo,

id-MulticastDU2CURRCInfo,

id-MBSMulticastSessionReceptionState,

id-F1UTunnelNotEstablished,

id-MulticastCU2DUCommonRRCInfo,

id-NRA2XServicesAuthorized,

id-LTEA2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrateForA2X,

id-LTEUESidelinkAggregateMaximumBitrateForA2X,

id-NReRedCapUEIndication,

id-NRPaginglongeDRXInformationforRRCINACTIVE,

id-TxResourceReqList,

id-TxResourceReq-Item,

id-TxresourceReq-ToBeSetupMod-List,

id-TxResourceReq-ToBeModified-Item,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofChildIABNodes,

maxnoofServedCellsIAB,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofNeighbourNodeCellsIAB,

maxnoofMRBsforUE,

maxnoofServingCellMOs,

maxnoofLTMCells,

maxnoofSLDestIDs

FROM F1AP-Constants;

<text omitted>

-- **************************************************************

--

#### -- UE Context Setup ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List								PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE conditional }|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-new-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID											PRESENCE optional	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-BHChannels-ToBeSetup-List				CRITICALITY reject	TYPE BHChannels-ToBeSetup-List						PRESENCE optional	}|

{ ID id-ConfiguredBAPAddress					CRITICALITY reject	TYPE BAPAddress											PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetup-List					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-List							PRESENCE optional	}|

{ ID id-ConditionalInterDUMobilityInformation	CRITICALITY reject	TYPE ConditionalInterDUMobilityInformation		PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE 		MDTPLMNList									PRESENCE optional }|

{ ID id-ServingNID								CRITICALITY reject	TYPE NID												PRESENCE optional }|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-TxResourceReqList						CRITICALITY reject	TYPE TxResourceReqList										PRESENCE optional },

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF ProtocolIE-SingleContainer { { Candidate-SpCell-ItemIEs} }

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetup-ItemIEs} }

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetup-ItemIEs} }

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetup-ItemIEs} }

BHChannels-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetup-ItemIEs} }

SLDRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetup-ItemIEs} }

TxResourceReqList ::= SEQUENCE (SIZE(1..maxnoofSLDestIDs)) OF ProtocolIE-SingleContainer	{ { TxResourceReq-ItemIEs } }

Candidate-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Candidate-SpCell-Item					CRITICALITY ignore	TYPE Candidate-SpCell-Item						PRESENCE mandatory	},

...

}

SCell-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetup-Item						CRITICALITY ignore	TYPE SCell-ToBeSetup-Item					PRESENCE mandatory	},

...

}

SRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetup-Item		CRITICALITY reject		TYPE SRBs-ToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetup-Item					CRITICALITY reject	TYPE DRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

BHChannels-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetup-Item					CRITICALITY reject	TYPE BHChannels-ToBeSetup-Item					PRESENCE mandatory},

...

}

SLDRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetup-Item					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

TxresourceReq-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TxResourceReq-Item				CRITICALITY reject	TYPE	TxResourceReq-Item					PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	TYPE LowerLayerPresenceStatusChange				PRESENCE optional	}|

{ ID id-BHChannels-ToBeSetupMod-List			CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeModified-List			CRITICALITY reject	TYPE BHChannels-ToBeModified-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeReleased-List			CRITICALITY reject	TYPE BHChannels-ToBeReleased-List					PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetupMod-List				CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeModified-List				CRITICALITY reject	TYPE SLDRBs-ToBeModified-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeReleased-List				CRITICALITY reject	TYPE SLDRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-ConditionalIntraDUMobilityInformation	CRITICALITY reject	TYPE ConditionalIntraDUMobilityInformation		PRESENCE optional}|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore	TYPE SCGIndicator										PRESENCE optional	}|

{ ID id-DAPS-HO-Status							CRITICALITY ignore	TYPE DAPS-HO-Status										PRESENCE optional }|

{ ID id-UplinkTxDirectCurrentTwoCarrierListInfo	CRITICALITY ignore	TYPE UplinkTxDirectCurrentTwoCarrierListInfo	PRESENCE optional	}|

{ ID id-TxResourceReq-ToBeModified-List			CRITICALITY reject	TYPE TxResourceReq-ToBeModified List				PRESENCE optional },

...

}

SCell-ToBeSetupMod-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetupMod-ItemIEs} }

SCell-ToBeRemoved-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeRemoved-ItemIEs} }

SRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetupMod-ItemIEs} }

BHChannels-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetupMod-ItemIEs} }

DRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeModified-ItemIEs} }

BHChannels-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeModified-ItemIEs} }

SRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeReleased-ItemIEs} }

DRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeReleased-ItemIEs} }

BHChannels-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeReleased-ItemIEs} }

TxResourceReq-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofSLDestIDs)) OF ProtocolIE-SingleContainer { { TxResourceReq-ToBeModified-ItemIEs} }

SCell-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetupMod-Item			CRITICALITY ignore	TYPE SCell-ToBeSetupMod-Item			PRESENCE mandatory	},

...

}

SCell-ToBeRemoved-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeRemoved-Item			CRITICALITY ignore	TYPE SCell-ToBeRemoved-Item			PRESENCE mandatory	},

...

}

SRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE DRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeModified-Item		CRITICALITY reject	TYPE DRBs-ToBeModified-Item			PRESENCE mandatory},

...

}

SRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeReleased-Item	CRITICALITY reject	TYPE SRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

DRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeReleased-Item		CRITICALITY reject	TYPE DRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetupMod-Item		CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeModified-Item		CRITICALITY reject	TYPE BHChannels-ToBeModified-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeReleased-Item		CRITICALITY reject	TYPE BHChannels-ToBeReleased-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetupMod-ItemIEs} }

SLDRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeModified-ItemIEs} }

SLDRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeReleased-ItemIEs} }

SLDRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeModified-Item		CRITICALITY reject	TYPE SLDRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeReleased-Item		CRITICALITY reject	TYPE SLDRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

TxResourceReq-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TxResourceReq-ToBeModified-Item				CRITICALITY reject	TYPE	TxResourceReq-ToBeModified-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

<text omitted>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-areaScope,

id-IntendedTDD-DL-ULConfig,

id-QosMonitoringRequest,

id-BHInfo,

id-IAB-Info-IAB-DU,

id-IAB-Info-IAB-donor-CU,

id-IAB-Barred,

id-SIB12-message,

id-SIB13-message,

id-SIB14-message,

id-UEAssistanceInformationEUTRA,

id-SL-PHY-MAC-RLC-Config,

id-SL-ConfigDedicatedEUTRA-Info,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRPRACHConfig,

id-TDD-UL-DLConfigCommonNR,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-TSCTrafficCharacteristics,

id-AdditionalPDCPDuplicationTNL-List,

id-RLCDuplicationInformation,

id-AdditionalDuplicationIndication,

id-mdtConfiguration,

id-TraceCollectionEntityURI,

id-NID,

id-NPNSupportInfo,

id-NPNBroadcastInformation,

id-AvailableSNPN-ID-List,

id-SIB10-message,

id-RequestedP-MaxFR2,

id-DLCarrierList,

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-PDCPTerminatingNodeDLTNLAddrInfo,

id-ENBDLTNLAddress,

id-PRS-Resource-ID,

id-LocationMeasurementInformation,

id-InterFrequencyConfig-NoGap,

id-NeedForGapsInfoNR,

id-ConfigRestrictInfoDAPS,

id-L571Info,

id-L1151Info,

id-ServCellInfoList,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxCellingNBDU,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofslots,

maxnoofNonUPTrafficMappings,

maxnoofServingCells,

maxnoofServedCellsIAB,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofSymbols,

maxnoofDUFSlots,

maxnoofHSNASlots,

maxnoofEgressLinks,

maxnoofMappingEntries,

maxnoofDSInfo,

maxnoofQoSParaSets,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofPhysicalResourceBlocks-1,

maxnoofPRACHconfigs,

maxnoofRACHReports,

maxnoofRLFReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationState,

maxnoofCHOcells,

maxnoofMDTPLMNs,

maxnoofCAGsupported,

maxnoofNIDsupported,

maxnoofExtSliceItems,

maxnoofPosMeas,

maxnoofTRPInfoTypes,

maxnoofSRSTriggerStates,

maxnoofSpatialRelations,

maxnoBcastCell,

maxnoofTRPs,

maxnoofAngleInfo,

maxnooflcs-gcs-translation,

maxnoofPath,

maxnoofMeasE-CID,

maxnoofSSBs,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofPosSITypes

FROM F1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM F1AP-CommonDataTypes

ProtocolExtensionContainer{},

F1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

<text omitted>

#### -- T

FiveGS-TAC ::= OCTET STRING (SIZE(3))

Configured-EPS-TAC ::= OCTET STRING (SIZE(2))

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {

target-cell								NRCGI,

iE-Extensions							ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL

}

TargetCellList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TDD-Info ::= SEQUENCE {

nRFreqInfo							NRFreqInfo,

transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-IntendedTDD-DL-ULConfig	CRITICALITY ignore	EXTENSION	IntendedTDD-DL-ULConfig	PRESENCE optional}|

{ID id-TDD-UL-DLConfigCommonNR	CRITICALITY ignore	EXTENSION TDD-UL-DLConfigCommonNR	PRESENCE optional }|

{ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList				PRESENCE optional },

...

}

TDD-UL-DLConfigCommonNR ::= OCTET STRING

TimeReferenceInformation ::= SEQUENCE {

referenceTime					ReferenceTime,

referenceSFN					ReferenceSFN,

uncertainty						Uncertainty,

timeInformationType				TimeInformationType,

iE-Extensions		ProtocolExtensionContainer { {TimeReferenceInformation-ExtIEs} }	OPTIONAL

}

TimeReferenceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeInformationType ::= ENUMERATED {localClock}

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL

}

TimeStamp-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-SingleContainer { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TimingMeasurementQuality ::= SEQUENCE {

measurementQuality		INTEGER(0..31),

resolution				ENUMERATED{m0dot1, m1, m10, m30, ...},

iE-Extensions		ProtocolExtensionContainer { { TimingMeasurementQuality-ExtIEs} }	OPTIONAL

}

TimingMeasurementQuality-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TNLCapacityIndicator::= SEQUENCE {

dLTNLOfferedCapacity		INTEGER (1.. 16777216,...),

dLTNLAvailableCapacity		INTEGER (0.. 100,...),

uLTNLOfferedCapacity		INTEGER (1.. 16777216,...),

uLTNLAvailableCapacity		INTEGER (0.. 100,...),

iE-Extensions	ProtocolExtensionContainer { { TNLCapacityIndicator-ExtIEs} } OPTIONAL

}

TNLCapacityIndicator-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TraceActivation ::= SEQUENCE {

traceID								TraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL

}

TraceActivation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-mdtConfiguration	CRITICALITY ignore	EXTENSION	MDTConfiguration		PRESENCE optional}|

{ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URI-address		PRESENCE optional	},

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TraceID ::= OCTET STRING (SIZE(8))

TrafficMappingInfo	::= CHOICE {

iPtolayer2TrafficMappingInfo					IPtolayer2TrafficMappingInfo,

bAPlayerBHRLCchannelMappingInfo					BAPlayerBHRLCchannelMappingInfo,

choice-extension								ProtocolIE-SingleContainer { { TrafficMappingInfo-ExtIEs} }

}

TrafficMappingInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TransportLayerAddress		::= BIT STRING (SIZE(1..160, ...))

TransactionID				::= INTEGER (0..255, ...)

Transmission-Bandwidth ::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { { Transmission-Bandwidth-ExtIEs} } OPTIONAL,

...

}

Transmission-Bandwidth-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionComb ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

choice-extension				ProtocolIE-SingleContainer { { TransmissionComb-ExtIEs} }

}

TransmissionComb-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TransmissionCombPos ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

n8    SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5)

},

choice-extension				ProtocolIE-SingleContainer { { TransmissionCombPos-ExtIEs} }

}

TransmissionCombPos-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TransmissionStopIndicator ::= ENUMERATED {true, ... }

Transport-UP-Layer-Address-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Address-Info-To-Add-Item

Transport-UP-Layer-Address-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Address-Info-To-Add-ItemExtIEs } }	OPTIONAL

}

Transport-UP-Layer-Address-Info-To-Add-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Address-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Address-Info-To-Remove-Item

Transport-UP-Layer-Address-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Address-Info-To-Remove-ItemExtIEs } }	OPTIONAL

}

Transport-UP-Layer-Address-Info-To-Remove-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionActionIndicator ::= ENUMERATED {stop, ..., restart }

TRPID ::= INTEGER (0.. maxnoofTRPs, ...)

TRPInformation ::= SEQUENCE {

tRPID							TRPID,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL

}

TRPInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPInformationItem ::= SEQUENCE {

tRPInformation					TRPInformation,

iE-Extensions					ProtocolExtensionContainer { { TRPInformationItem-ExtIEs } }	OPTIONAL

}

TRPInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPInformationTypeItem ::= ENUMERATED {

nrPCI,

nG-RAN-CGI,

arfcn,

pRSConfig,

sSBConfig,

sFNInitTime,

spatialDirectInfo,

geoCoord,

...,

trp-type

}

TRPInformationTypeResponseList ::= SEQUENCE (SIZE(1.. maxnoofTRPInfoTypes)) OF TRPInformationTypeResponseItem

TRPInformationTypeResponseItem ::= CHOICE {

pCI-NR								NRPCI,

nG-RAN-CGI							NRCGI,

nRARFCN								INTEGER (0..maxNRARFCN),

pRSConfiguration					PRSConfiguration,

sSBinformation						SSBInformation,

sFNInitialisationTime				RelativeTime1900,

spatialDirectionInformation			SpatialDirectionInformation,

geographicalCoordinates				GeographicalCoordinates,

choice-extension					ProtocolIE-SingleContainer { { TRPInformationTypeResponseItem-ExtIEs} }

}

TRPInformationTypeResponseItem-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TRPType		CRITICALITY reject TYPE TRPType		PRESENCE mandatory },

...

}

TRPList ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF TRPListItem

TRPListItem ::= SEQUENCE {

tRPID							TRPID,

iE-Extensions					ProtocolExtensionContainer { { TRPListItem-ExtIEs } }	OPTIONAL

}

TRPListItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuality ::= SEQUENCE {

tRPmeasurementQuality-Item 	TRPMeasurementQuality-Item,

iE-Extensions				ProtocolExtensionContainer { {TRPMeasurementQuality-ExtIEs} } OPTIONAL

}

TRPMeasurementQuality-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuality-Item ::= CHOICE {

timingMeasurementQuality	TimingMeasurementQuality,

angleMeasurementQuality		AngleMeasurementQuality,

choice-extension			ProtocolIE-SingleContainer { { TRPMeasurementQuality-Item-ExtIEs } }

}

TRPMeasurementQuality-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRP-MeasurementRequestList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementRequestItem

TRP-MeasurementRequestItem ::= SEQUENCE {

tRPID							TRPID,

search-window-information		Search-window-information	OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { TRP-MeasurementRequestItem-ExtIEs } } OPTIONAL

}

TRP-MeasurementRequestItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRCGI	CRITICALITY ignore EXTENSION NRCGI		PRESENCE optional },

...

}

TRPPositionDefinitionType ::= CHOICE {

direct		TRPPositionDirect,

referenced	TRPPositionReferenced,

choice-extension							ProtocolIE-SingleContainer { { TRPPositionDefinitionType-ExtIEs } }

}

TRPPositionDefinitionType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRPPositionDirect ::= SEQUENCE {

accuracy	TRPPositionDirectAccuracy,

iE-extensions		ProtocolExtensionContainer { { TRPPositionDirect-ExtIEs } }	OPTIONAL

}

TRPPositionDirect-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPPositionDirectAccuracy ::= CHOICE {

tRPPosition				AccessPointPosition,

tRPHAposition			NGRANHighAccuracyAccessPointPosition,

choice-extension		ProtocolIE-SingleContainer { { TRPPositionDirectAccuracy-ExtIEs } }

}

TRPPositionDirectAccuracy-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRPPositionReferenced ::= SEQUENCE {

referencePoint					ReferencePoint,

referencePointType				TRPReferencePointType,

iE-extensions					ProtocolExtensionContainer { { TRPPositionReferenced-ExtIEs } } 	OPTIONAL

}

TRPPositionReferenced-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPReferencePointType ::= CHOICE {

tRPPositionRelativeGeodetic			RelativeGeodeticLocation,

tRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-SingleContainer { { TRPReferencePointType-ExtIEs } }

}

TRPReferencePointType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Transport-Layer-Address-Info ::= SEQUENCE {

transport-UP-Layer-Address-Info-To-Add-List		Transport-UP-Layer-Address-Info-To-Add-List							OPTIONAL,

transport-UP-Layer-Address-Info-To-Remove-List	Transport-UP-Layer-Address-Info-To-Remove-List					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs } }								OPTIONAL

}

Transport-Layer-Address-Info-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPType ::= ENUMERATED {

prsOnlyTP,

srsOnlyRP,

tp,

rp,

trp,

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime													OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDL		TSCAssistanceInformation								OPTIONAL,

tSCAssistanceInformationUL		TSCAssistanceInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} }	OPTIONAL,

...

}

TSCTrafficCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TxResourceReq-Item ::= SEQUENCE {

nR-V2X-destination-identity			NR-V2X-destination-identity,

iE-Extensions		ProtocolExtensionContainer { { TxResourceReq-Item-ExtIEs} }	OPTIONAL,

...

}

TxResourceReq-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TxResourceReqToBeModified-Item ::= SEQUENCE {

nR-V2X-destination-identity			NR-V2X-destination-identity,

iE-Extensions		ProtocolExtensionContainer { { TxResourceReqToBeModified-Item-ExtIEs} }	OPTIONAL,

...

}

TxResourceReq-ToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-V2X-destination-identity	::=	INTEGER (1..32)

Next change

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRACHReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofPosSITypes						INTEGER ::= 32

maxnoofSLDestIDs						INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container	ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-NotficationControl								ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 						ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 						ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-MeasGapSharingConfig								ProtocolIE-ID ::= 238

id-systemInformationAreaID							ProtocolIE-ID ::= 239

id-areaScope										ProtocolIE-ID ::= 240

id-RRCContainer-RRCSetupComplete					ProtocolIE-ID ::= 241

id-TraceActivation									ProtocolIE-ID ::= 242

id-TraceID											ProtocolIE-ID ::= 243

id-Neighbour-Cell-Information-List					ProtocolIE-ID ::= 244

id-SymbolAllocInSlot								ProtocolIE-ID ::= 246

id-NumDLULSymbols									ProtocolIE-ID ::= 247

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 248

id-DUCURadioInformationType							ProtocolIE-ID ::= 249

id-CUDURadioInformationType 						ProtocolIE-ID ::= 250

id-AggressorgNBSetID								ProtocolIE-ID ::= 251

id-VictimgNBSetID									ProtocolIE-ID ::= 252

id-LowerLayerPresenceStatusChange					ProtocolIE-ID ::= 253

id-Transport-Layer-Address-Info						ProtocolIE-ID ::= 254

id-Neighbour-Cell-Information-Item					ProtocolIE-ID ::= 255

id-IntendedTDD-DL-ULConfig							ProtocolIE-ID ::= 256

id-QosMonitoringRequest								ProtocolIE-ID ::= 257

id-BHChannels-ToBeSetup-List						ProtocolIE-ID ::= 258

id-BHChannels-ToBeSetup-Item						ProtocolIE-ID ::= 259

id-BHChannels-Setup-List							ProtocolIE-ID ::= 260

id-BHChannels-Setup-Item							ProtocolIE-ID ::= 261

id-BHChannels-ToBeModified-Item						ProtocolIE-ID ::= 262

id-BHChannels-ToBeModified-List						ProtocolIE-ID ::= 263

id-BHChannels-ToBeReleased-Item						ProtocolIE-ID ::= 264

id-BHChannels-ToBeReleased-List						ProtocolIE-ID ::= 265

id-BHChannels-ToBeSetupMod-Item						ProtocolIE-ID ::= 266

id-BHChannels-ToBeSetupMod-List						ProtocolIE-ID ::= 267

id-BHChannels-FailedToBeModified-Item				ProtocolIE-ID ::= 268

id-BHChannels-FailedToBeModified-List				ProtocolIE-ID ::= 269

id-BHChannels-FailedToBeSetupMod-Item				ProtocolIE-ID ::= 270

id-BHChannels-FailedToBeSetupMod-List				ProtocolIE-ID ::= 271

id-BHChannels-Modified-Item							ProtocolIE-ID ::= 272

id-BHChannels-Modified-List							ProtocolIE-ID ::= 273

id-BHChannels-SetupMod-Item							ProtocolIE-ID ::= 274

id-BHChannels-SetupMod-List							ProtocolIE-ID ::= 275

id-BHChannels-Required-ToBeReleased-Item			ProtocolIE-ID ::= 276

id-BHChannels-Required-ToBeReleased-List			ProtocolIE-ID ::= 277

id-BHChannels-FailedToBeSetup-Item					ProtocolIE-ID ::= 278

id-BHChannels-FailedToBeSetup-List					ProtocolIE-ID ::= 279

id-BHInfo											ProtocolIE-ID ::= 280

id-BAPAddress										ProtocolIE-ID ::= 281

id-ConfiguredBAPAddress								ProtocolIE-ID ::= 282

id-BH-Routing-Information-Added-List				ProtocolIE-ID ::= 283

id-BH-Routing-Information-Added-List-Item			ProtocolIE-ID ::= 284

id-BH-Routing-Information-Removed-List				ProtocolIE-ID ::= 285

id-BH-Routing-Information-Removed-List-Item			ProtocolIE-ID ::= 286

id-UL-BH-Non-UP-Traffic-Mapping						ProtocolIE-ID ::= 287

id-Activated-Cells-to-be-Updated-List				ProtocolIE-ID ::= 288

id-Child-Nodes-List									ProtocolIE-ID ::= 289

id-IAB-Info-IAB-DU									ProtocolIE-ID ::= 290

id-IAB-Info-IAB-donor-CU							ProtocolIE-ID ::= 291

id-IAB-TNL-Addresses-To-Remove-List					ProtocolIE-ID ::= 292

id-IAB-TNL-Addresses-To-Remove-Item					ProtocolIE-ID ::= 293

id-IAB-Allocated-TNL-Address-List					ProtocolIE-ID ::= 294

id-IAB-Allocated-TNL-Address-Item					ProtocolIE-ID ::= 295

id-IABIPv6RequestType								ProtocolIE-ID ::= 296

id-IABv4AddressesRequested							ProtocolIE-ID ::= 297

id-IAB-Barred										ProtocolIE-ID ::= 298

id-TrafficMappingInformation						ProtocolIE-ID ::= 299

id-UL-UP-TNL-Information-to-Update-List				ProtocolIE-ID ::= 300

id-UL-UP-TNL-Information-to-Update-List-Item		ProtocolIE-ID ::= 301

id-UL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 302

id-UL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 303

id-DL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 304

id-DL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 305

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= 306

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= 307

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 308

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 309

id-SIB12-message									ProtocolIE-ID ::= 310

id-SIB13-message									ProtocolIE-ID ::= 311

id-SIB14-message									ProtocolIE-ID ::= 312

id-SLDRBs-FailedToBeModified-Item					ProtocolIE-ID ::= 313

id-SLDRBs-FailedToBeModified-List					ProtocolIE-ID ::= 314

id-SLDRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 315

id-SLDRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 316

id-SLDRBs-Modified-Item								ProtocolIE-ID ::= 317

id-SLDRBs-Modified-List								ProtocolIE-ID ::= 318

id-SLDRBs-Required-ToBeModified-Item				ProtocolIE-ID ::= 319

id-SLDRBs-Required-ToBeModified-List				ProtocolIE-ID ::= 320

id-SLDRBs-Required-ToBeReleased-Item				ProtocolIE-ID ::= 321

id-SLDRBs-Required-ToBeReleased-List				ProtocolIE-ID ::= 322

id-SLDRBs-Setup-Item								ProtocolIE-ID ::= 323

id-SLDRBs-Setup-List								ProtocolIE-ID ::= 324

id-SLDRBs-ToBeModified-Item							ProtocolIE-ID ::= 325

id-SLDRBs-ToBeModified-List							ProtocolIE-ID ::= 326

id-SLDRBs-ToBeReleased-Item							ProtocolIE-ID ::= 327

id-SLDRBs-ToBeReleased-List							ProtocolIE-ID ::= 328

id-SLDRBs-ToBeSetup-Item							ProtocolIE-ID ::= 329

id-SLDRBs-ToBeSetup-List							ProtocolIE-ID ::= 330

id-SLDRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 331

id-SLDRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 332

id-SLDRBs-SetupMod-List								ProtocolIE-ID ::= 333

id-SLDRBs-FailedToBeSetupMod-List					ProtocolIE-ID ::= 334

id-SLDRBs-SetupMod-Item								ProtocolIE-ID ::= 335

id-SLDRBs-FailedToBeSetupMod-Item					ProtocolIE-ID ::= 336

id-SLDRBs-ModifiedConf-List							ProtocolIE-ID ::= 337

id-SLDRBs-ModifiedConf-Item							ProtocolIE-ID ::= 338

id-UEAssistanceInformationEUTRA						ProtocolIE-ID ::= 339

id-PC5LinkAMBR										ProtocolIE-ID ::= 340

id-SL-PHY-MAC-RLC-Config							ProtocolIE-ID ::= 341

id-SL-ConfigDedicatedEUTRA-Info						ProtocolIE-ID ::= 342

id-AlternativeQoSParaSetList						ProtocolIE-ID ::= 343

id-CurrentQoSParaSetIndex							ProtocolIE-ID ::= 344

id-gNBCUMeasurementID								ProtocolIE-ID ::= 345

id-gNBDUMeasurementID								ProtocolIE-ID ::= 346

id-RegistrationRequest								ProtocolIE-ID ::= 347

id-ReportCharacteristics							ProtocolIE-ID ::= 348

id-CellToReportList									ProtocolIE-ID ::= 349

id-CellMeasurementResultList						ProtocolIE-ID ::= 350

id-HardwareLoadIndicator							ProtocolIE-ID ::= 351

id-ReportingPeriodicity								ProtocolIE-ID ::= 352

id-TNLCapacityIndicator								ProtocolIE-ID ::= 353

id-CarrierList										ProtocolIE-ID ::= 354

id-ULCarrierList									ProtocolIE-ID ::= 355

id-FrequencyShift7p5khz								ProtocolIE-ID ::= 356

id-SSB-PositionsInBurst								ProtocolIE-ID ::= 357

id-NRPRACHConfig									ProtocolIE-ID ::= 358

id-RACHReportInformationList						ProtocolIE-ID ::= 359

id-RLFReportInformationList							ProtocolIE-ID ::= 360

id-TDD-UL-DLConfigCommonNR							ProtocolIE-ID ::= 361

id-CNPacketDelayBudgetDownlink						ProtocolIE-ID ::= 362

id-ExtendedPacketDelayBudget						ProtocolIE-ID ::= 363

id-TSCTrafficCharacteristics						ProtocolIE-ID ::= 364

id-ReportingRequestType								ProtocolIE-ID ::= 365

id-TimeReferenceInformation							ProtocolIE-ID ::= 366

id-CNPacketDelayBudgetUplink						ProtocolIE-ID ::= 369

id-AdditionalPDCPDuplicationTNL-List				ProtocolIE-ID ::= 370

id-RLCDuplicationInformation						ProtocolIE-ID ::= 371

id-AdditionalDuplicationIndication					ProtocolIE-ID ::= 372

id-ConditionalInterDUMobilityInformation			ProtocolIE-ID ::= 373

id-ConditionalIntraDUMobilityInformation			ProtocolIE-ID ::= 374

id-targetCellsToCancel								ProtocolIE-ID ::= 375

id-requestedTargetCellGlobalID						ProtocolIE-ID ::= 376

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 377

id-TraceCollectionEntityIPAddress 					ProtocolIE-ID ::= 378

id-PrivacyIndicator									ProtocolIE-ID ::= 379

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 380

id-mdtConfiguration									ProtocolIE-ID ::= 381

id-ServingNID										ProtocolIE-ID ::= 382

id-NPNBroadcastInformation							ProtocolIE-ID ::= 383

id-NPNSupportInfo									ProtocolIE-ID ::= 384

id-NID												ProtocolIE-ID ::= 385

id-AvailableSNPN-ID-List							ProtocolIE-ID ::= 386

id-SIB10-message									ProtocolIE-ID ::= 387

id-DLCarrierList									ProtocolIE-ID ::= 389

id-ExtendedTAISliceSupportList					ProtocolIE-ID ::= 390

id-RequestedSRSTransmissionCharacteristics			ProtocolIE-ID ::= 391

id-PosAssistance-Information						ProtocolIE-ID ::= 392

id-PosBroadcast										ProtocolIE-ID ::= 393

id-RoutingID										ProtocolIE-ID ::= 394

id-PosAssistanceInformationFailureList				ProtocolIE-ID ::= 395

id-PosMeasurementQuantities							ProtocolIE-ID ::= 396

id-PosMeasurementResultList							ProtocolIE-ID ::= 397

id-TRPInformationTypeListTRPReq						ProtocolIE-ID ::= 398

id-TRPInformationTypeItem							ProtocolIE-ID ::= 399

id-TRPInformationListTRPResp						ProtocolIE-ID ::= 400

id-TRPInformationItem								ProtocolIE-ID ::= 401

id-LMF-MeasurementID								ProtocolIE-ID ::= 402

id-SRSType											ProtocolIE-ID ::= 403

id-ActivationTime									ProtocolIE-ID ::= 404

id-AbortTransmission								ProtocolIE-ID ::= 405

id-PositioningBroadcastCells						ProtocolIE-ID ::= 406

id-SRSConfiguration									ProtocolIE-ID ::= 407

id-PosReportCharacteristics							ProtocolIE-ID ::= 408

id-PosMeasurementPeriodicity						ProtocolIE-ID ::= 409

id-TRPList											ProtocolIE-ID ::= 410

id-RAN-MeasurementID								ProtocolIE-ID ::= 411

id-LMF-UE-MeasurementID								ProtocolIE-ID ::= 412

id-RAN-UE-MeasurementID								ProtocolIE-ID ::= 413

id-E-CID-MeasurementQuantities						ProtocolIE-ID ::= 414

id-E-CID-MeasurementQuantities-Item					ProtocolIE-ID ::= 415

id-E-CID-MeasurementPeriodicity						ProtocolIE-ID ::= 416

id-E-CID-MeasurementResult							ProtocolIE-ID ::= 417

id-Cell-Portion-ID									ProtocolIE-ID ::= 418

id-SFNInitialisationTime							ProtocolIE-ID ::= 419

id-SystemFrameNumber								ProtocolIE-ID ::= 420

id-SlotNumber										ProtocolIE-ID ::= 421

id-TRP-MeasurementRequestList						ProtocolIE-ID ::= 422

id-MeasurementBeamInfoRequest						ProtocolIE-ID ::= 423

id-E-CID-ReportCharacteristics						ProtocolIE-ID ::= 424

id-ConfiguredTACIndication							ProtocolIE-ID ::= 425

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 426

id-Extended-GNB-DU-Name								ProtocolIE-ID ::= 427

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-PDCPTerminatingNodeDLTNLAddrInfo					ProtocolIE-ID ::= 436

id-ENBDLTNLAddress									ProtocolIE-ID ::= 437

id-PosMeasurementPeriodicityExtended				ProtocolIE-ID ::= 438

id-PRS-Resource-ID									ProtocolIE-ID ::= 439

id-LocationMeasurementInformation					ProtocolIE-ID ::= 440

id-InterFrequencyConfig-NoGap						ProtocolIE-ID ::= 651

id-L571Info											ProtocolIE-ID ::= 659

id-L1151Info										ProtocolIE-ID ::= 660

id-NeedForGapsInfoNR								ProtocolIE-ID ::= 665

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-ServCellInfoList                                 		ProtocolIE-ID ::= 707

id-TxResourceReqList								ProtocolIE-ID ::= [assigned by rapp]

id-TxResourceReq-Item								ProtocolIE-ID ::= [assigned by rapp]

id-TxResourceReq-ToBeSetupMod-List					ProtocolIE-ID ::= [assigned by rapp]

id-TxResourceReq-ToBeModified-Item					ProtocolIE-ID ::=  [assigned by rapp]

END

-- ASN1STOP

End of the change