3GPP TSG-RAN WG3 #123		R3-240180

Athens, Greece, 26th Feb – 1th Mar 2024

Source:	CATT

Title:	Discussion on QoE measurement continuity for mobility

Agenda Item:	9.1.2.1

Document for:	Discussion

- Introduction

On QoE measurement collection continuity for mobility, there are still some issues need further discuss.

In this contribution, we will provide some analysis on intra-system inter-RAT handover issue and MBS delivery mode.

- Discussion

2.1 intra-system inter-RAT handover

In last RAN3 meeting, the agreement on On QoE measurement collection continuity for intra-system inter-RAT handover is captured in TS 38.300[1] as below:

QoE measurement collection continuity for intra-system inter-RAT handover is supported. For the handover from NR to LTE, only one QoE measurement can be continued when the UE connects to the target LTE node. The source gNB decides which QoE measurement to keep and sends the information about this QoE measurement to the target ng-eNB.

In last RAN2 meeting[2], the following agreements were achieved.

For HO from LTE/5GC to NR, UE should release all LTE QoE configurations and apply NR QoE configuration if received. How UE releases old QoE configurations can be handled in CR phase.

For HO from NR to LTE/5GC, UE should release all NR QoE configurations and apply LTE QoE configuration if received. How UE releases old QoE configurations can be handled in CR phase.

For the handover from NR to LTE, the source NR node may not able to decide which QoE measurement can be continued. So, RAN3 agree to keep only one QoE measurement may be not feasible.

RAN2 decide to release all NR QoE configurations by NR node and then configure LTE QoE configuration by LTE node. We think this solution is reasonable. It is proposed to take RAN2 agreement as the solution on QoE measurement collection continuity for intra-system inter-RAT handover. Capture in 38.300 that the UE should release all the QoE configurations from the source RAT in intra-system inter-RAT handover.

Proposal 1: Capture in 38.300 that the UE should release all the QoE configurations from the source RAT in intra-system inter-RAT handover.

2.2 MBS delivery mode

The related RAN3 agreement is captured in TS 38.300[1] as below:

QoE measurements for ongoing sessions should be continued when switching between MBS multicast and unicast transmission modes.

There is another case on QoE Measurement Continuity when UE moving to a cell not providing the MBS broadcast which is captured in TS 38.300[1] as below:

##### 16.10.6.5.1 Service Continuity in RRC_IDLE or RRC_INACTIVE Mobility procedures for MBS reception allow the UE to start or continue receiving MBS service(s) when changing cells. The gNB may indicate in the MCCH the list of neighbour cells providing the same MBS broadcast service(s) as provided in the serving cell. This allows the UE, e.g., to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. To avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies, the UE is made aware of which frequency is providing which MBS broadcast services via PTM, through User Service Description (USD), as defined in TS 26.346 [46], or the combination of the following: - USD; - SIB21, as defined in clause 7.3.1. NOTE: UE can request unicast reception of the service after moving to a cell not providing the MBS broadcast service(s) using PTM transmission.

We think QoE measurements for ongoing sessions should also be continued when switching between MBS broadcast and unicast transmission modes for the same reason as switching between multicast and unicast delivery mode.

Proposal 2: Capture the case of QoE measurement continuity when switching between MBS broadcast and unicast transmission modes in 38.300.

- Conclusion

According to the analysis in section 2, we have the proposals as below:

Proposal 1: Capture in 38.300 that the UE should release all the QoE configurations from the source RAT in intra-system inter-RAT handover.

Proposal 2: Capture the case of QoE measurement continuity when switching between MBS broadcast and unicast transmission modes in 38.300.

- Reference

- 3GPP RAN, TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.0.0 (2023-12)

- Minutes report, RAN2 Chairman, RAN2#124