3GPP TSG-RAN WG3 Meeting #123                                                                    R3-240184

Athens, Greece, Feb 26th – Mar 1st, 2023

Source:	CATT

Title:	Leftover issues on cell DTXDRX for Rel-18 NES

Agenda Item:	9.1.12.1

Document for:	Discussion and Decision

1 Introduction

After the gNB determines to use cell DTX/DRX, the CU should initiate handover to some UEs that are not suitable after the cell DTX/DRX is activated. However, it is DU to activate the cell DTX/DRX. Hence, it’s necessary that the CU and the DU coordinate on cell DTX/DRX activation. RAN3 has not discussed the issue. Furthermore, it’s not clear which node (CU or DU) to determine using the cell DTX/DRX to NES cell, which should be discussed in the first place.

In this contribution, we provide analysis and proposals on leftover issues for cell DTX/DRX. We also provide the CR to Rel-18 TS38.473 corresponding to our proposals.

2 Discussion

Assumed steps on cell DTX/DRX activation or deactivation in our opinion:

Step 1: The gNB determines to use or stop using cell DTX/DRX for a specific cell

The gNB can determine applying cell DTX/DRX if entering NES status for a specific cell is required. Or the gNB can decide stop to apply cell DTX/DRX for leaving NES status.

Step 2: If using cell DTXDRX, the gNB-CU carries out necessary handovers or remove serving cell for UE

If using cell DTX/DRX for energy saving, the CU needs to remove the UEs from the NES cell which are not suitable to be served by the cell, e.g., the legacy UEs, UEs with higher priority traffic and UEs not capable of cell DTX/DRX. The CU should carry out handover to the unsuitable UEs. In addition, the CU may configure NES specific CHO to those capable UEs. Or, the CU can release the NES cell for the UEs who are using the NES cell as SCell.

Step 3: The gNB-DU activates or deactivates cell DTX/DRX

For entering or leave the NES status using cell DTX/DRX, the DU needs to activate or deactivate cell DTX/DRX accordingly. Two approaches of activation/deactivation signalling are introduced to Rel-18 by RAN2, i.e., signalling based on RRC configuration message or based on L1 signal (DCI (2_9)). The DU further need to send L1 signal (DCI (2_9)) to trigger NES CHO if the CU has configured NES-specific CHO event to UE(s).

Note that, for entering NES status, the DU should not activate cell DTX/DRX until the CU removing the unsuitable UEs (e.g., completing handovers for the those UEs), and the DU should not send L1 signal to trigger NES CHO until the CU configures the NES CHO to UEs.

Issue 1: How gNB determines whether to use cell DTXDRX

In step 1, one leftover issue is who (DU or CU) determines to use cell DTX/DRX if entering NES is required for a specific cell or stop to use cell DTX/DRX if leaving NES status is required.

In case of cell switch off for energy saving, if a cell does not serve any UE the gNB can turn the cell off. Due to the CU controls serving cell configuration to UE, it is the CU to decide whether to switch a cell off finally. According to the Rel-18 specs, cell DTXDRX is per cell configured and activated. Different to cell switch-off for energy saving, gNB decides whether to send a cell to NES status using cell DTXDRX technique based on the resource usage of the cell. If the resource usage is low enough and most of the UEs served by the cell support cell DTXDRX, the gNB can apply cell DTX/DRX.

Both CU and DU have the UE capability information, but difference is, DU knows the cell capacity, and it takes charge of DL/UL scheduling for each cell based on QoS of each UE traffic. DU can control and thus be aware of the resource usage status of each cell. Therefore, the DU has sufficient knowledge on determining whether the condition is fulfilled to enter NES state using cell DTX/DRX. But CU does not have the adequate information to derive the resource usage. For CU to do the determination on whether to use cell DTX/DRX, DU reporting the resource usage status to CU is required, which is a lot of signal consumption and not necessary in our view.

Furthermore, the cell DTX/DRX configuration and activation/deactivation is performed by DU. As concluded by RAN2, gNB is allowed to configure cell DTX to UEs with C-DRX configuration. To achieve a best efficiency of network energy saving, the gNB should ensures at least partial overlapping between C-DRX on-duration and cell DTX/DRX on-duration, and the alignment between cell DTX and C-DRX is up to implementation.

Clause 15.4.2.3 of TS 38.300 v18.0.0[1]:

Once the gNB recognizes there is an emergency call or public safety related service (e.g. MPS or MCS), the network should ensure that there is no impact to that service (e.g. it may release or deactivate cell DTX/DRX configuration). The network should also ensure that there is at least partial overlapping between UE's connected mode DRX on-duration and cell DTX/DRX active duration, i.e. the UE's connected mode DRX periodicity is a multiple of cell DTX/DRX periodicity or vice versa.

According to current spec, C-DRX is configured by gNB-DU and DU well knows the parameters configured for C-DRX. So, it is DU to determine whether to configure or activate cell DTX/DRX and further decide whether/how to align the cell DTX with the C-DRX of UEs. The CU cannot implement the configuration of cell DTX/DRX, the configuration of C-DRX, nor the alignment between cell DTX and C-DRX. If the CU decides whether to use cell DTX/DRX, the feasibility of its determination should be confirmed by DU, that will lead to a redundant step.

Observation 1: gNB-DU has the sufficient information to decide using cell DTX/DRX or not. If gNB-CU decides using cell DTX/DRX, gN-DU reporting the resource usage status to CU is required, and gNB-CU should confirm the decision with gNB-DU.

gNB-DU deciding applying cell DTX/DRX for network energy saving is preferred.

Proposal 1: The gNB-DU decides whether to apply cell DTX/DRX for network energy saving.

Issue 2: Coordination between CU and DU on cell DTX/DRX

In step 2, after the gNB decides to apply cell DTX/DRX to a specific cell, the CU should carry out necessary handovers or remove the configuration of the NES cell to the unsuitable UEs. The CU needs to initiate handover to remove the unsuitable UEs from the NES cell if the NES cell serves as PCell of those UEs. The CU can configure NES specific CHO to those UEs supporting NES CHO, after that the DU should send a L1 signal to enable the CHO execution.

Clause 15.4.2.4 of TS 38.300 v18.0.0[1]:

The same principle as described in 9.2.3.4 applies to conditional handover in case the source cell is using a network energy saving solution (e.g., the cell is activating cell DTX/DRX or turning off), unless hereunder specified. In this case, the following additional triggering conditions are supported, upon which UE may use NES-specific CHO event for executing CHO to a candidate cell, as defined in TS 38.331 [12]: - The UE may be notified via DCI to enable CHO conditions(s) configured with NES event indication.

For those not support NES CHO, the CU can initiate the legacy handover procedure. If the NES cell serves as SCell of the unsuitable UE, the CU can release the NES cell to the unsuitable UEs.

When the DU decides to apply cell DTX/DRX for network energy saving, it should indicate the requirement of cell DTX/DRX activation to the CU for carrying out the legacy handovers, configure NES CHO or releasing the NES cell to the unsuitable UEs before the cell DTX/DRX is activated. The DU can further indicate the cell DTX/DRX type (i.e., only DTX, only DRX or both DTX and DRX) required to be applied. Alternatively, the DU decides to stop applying cell DTXDRX, it can indicate requirement of cell DTX/DRX deactivation to the CU. The DU can send the requirement of cell DTX/DRX activation/deactivation via a GNB-DU CONFIGURATION UPDATE message.

Proposal 2: The gNB-DU sends the requirement of cell DTX/DRX activation or deactivation to the gNB-CU via GNB-DU CONFIGURATION UPDATE message. The gNB-CU can initiate handovers or release the NES cell to the unsuitable UEs if the requirement of cell DTX/DRX activation is received from the gNB-DU.

In step 3, the DU should not activate cell DTX/DRX before the CU completing the necessary handovers or releasing the NES cell to UEs. And the DU should not send a L1 signal for triggering NES CHO before the CU configuring NES CHO to UEs. Therefore, it is necessary the CU send an indication to allow the DU activate the cell DTX/DRX after it completing the legacy handover, configuring NES CHO and releasing the NES cell to those unsuitable UEs. Since the handover procedures take time, the CU should send the indication to allow the DU activate cell DTX/DRX via a GNB-CU CONFIGURATION UPDATE message.

Alternatively, if the CU receives the requirement of cell DTX/DRX deactivation from the DU, it can confirm the reception of requirement to the CU. Since the reception does not take much time, the CU can indicate the confirmation via the GNB-DU CONFIGURATION UPDATE ACKNOWLEGDE message, then the DU can deactivate cell DTX/DRX.

Proposal 3: The gNB-CU sends an indication to the gNB-DU for allowing cell DTXDRX activation via GNB-CU CONFIGURATION UPDATE message.

Issue 3: How to configure/activate cell DTX/DRX to UE

In step 3, when the DU receives the indication of allowing activation of cell DTX/DRX from the CU, it can assume that all the unsuitable UEs have been handed over, released the NES cell or configured with NES CHO by the CU. So, the DU can activate the cell DTX/DRX or send L1 signal to trigger NES CHO (if necessary).

The remaining UEs with the NES cell configured include the UEs need to apply cell DTX/DRX and the UEs configured with NES CHO waiting for L1 signal trigger. In principle, the DU does not need to configure/activate cell DTX/DRX for the UEs which are configured with NES CHO.

RAN2 defined two types of UE capability on cell DTX/DRX[2]. One is for the cell DTX/DRX operation with only RRC signal based activation/deactivation, the other is for the cell DTX/DRX operation for both RRC and L1 based activation/deactivation:

nes-CellDTX-DRX-r18 Indicates whether the UE supports cell DTX and/or DRX operation by RRC configuration. The supported number of cell DTX/DRX patterns per cell group is 2, regardless of each pattern is for cell DTX only, cell DRX only, or both.

nes-CellDTX-DRX-DCI2-9-r18 Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9. A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18.

In our view, the DU can configure/activate cell DTX/DRX to all the UEs supporting cell DTX/DRX, i.e., the UEs with either of the two cell DTX/DRX capabilities. To prevent the DU configure/activate cell DTX/DRX to UEs which apply NES CHO, the CU can remove the UEs supporting cell DTX/DRX but not suitable anymore (e.g., UEs with high traffic priority) from the NES cell via the legacy handover procedure instead of NES CHO.

Observation 2: When the gNB-DU receives the indication from the gNB-CU for allowing activation of cell DTXDRX, it can configure/activate cell DTX/DRX to all the remaining UEs supporting cell DTXDRX.

Issue 4: Whether to send L1 signal to trigger NES CHO

The DU should decide whether to send L1 signal for triggering NES CHO, when it receives the indication of allowing activation of cell DTX/DRX from the CU. Per above analysis, all the remaining UEs supporting cell DTX/DRX should apply cell DTX/DRX. So, if there are remaining UEs not supporting cell DTX/DRX, the DU should know the CU has configured NES-specific CHO event to those UEs, then the DU can send L1 signal to trigger NES CHO.

Observation 3: When gNB-DU receives the indication of allowing cell DTX/DRX activation from the gNB-CU, it can send L1 signal to trigger NES CHO if there are remaining UEs not supporting cell DTX/DRX.

The overall procedures on cell DTX/DRX activation and deactivation:

- Cell DTX/DRX activation                                                            b)   Cell DTX/DRX deactivation

3 Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: gNB-DU has the sufficient information to decide using cell DTX/DRX or not. If gNB-CU decides using cell DTXDRX, gN-DU reporting the resource usage status to CU is required, and gNB-CU should confirm the decision with gNB-DU.

Proposal 1: The gNB-DU decides whether to apply cell DTX/DRX for network energy saving.

Proposal 2: The gNB-DU sends the requirement of cell DTXDRX activation or deactivation to the gNB-CU via GNB-DU CONFIGURATION UPDATE message. The gNB-CU can initiate handovers or release the NES cell to the unsuitable UEs if the requirement of cell DTXDRX activation is received from the gNB-DU.

Proposal 3: The gNB-CU sends an indication to the gNB-DU for allowing cell DTX/DRX activation via GNB-CU CONFIGURATION UPDATE message.

Observation 2: When the gNB-DU receives the indication from the gNB-CU for allowing activation of cell DTXDRX, it can configure/activate cell DTXDRX to all the remaining UEs supporting cell DTXDRX.

Observation 3: When gNB-DU receives the indication of allowing cell DTXDRX activation from the gNB-CU, it can send L1 signal to trigger NES CHO if there are remaining UEs not supporting cell DTXDRX.

4 Reference

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.0.0 (2023-12)

- 3GPP TS 38.306, User Equipment (UE) radio access capabilities, v18.0.0 (2023-12)

- R3-240185, Cell DTXDRX activation deactivation NR_38.473_CR1270r0_(Rel-18), CATT, RAN3#123