3GPP TSG-RAN3 Meeting #123	R3-240199

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1064 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support redcap with SDT in TS38.413 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 has agreed to support redcap with SDT in Rel-18, seen incoming LS R3-240026. This CR is used to enhance NGAP: RAN paging request message to support redcap with SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance NGAP: RAN paging request message to support redcap with SDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function (i.e., redcap with SDT) cannot be supported in Rel-18 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.63, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------------------------Start of Change------------------------------------------------------

9.3.3.63	Paging Policy Differentiation

This IE provides paging policy differentiation information for a UE in RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session ID | M |  | 9.3.1.50 |  |
| Paging Policy Differentiation List |  | 0..1 |  |  |
| >Paging Policy Differentiation Item |  | 1..<maxnoofQoSFlows> |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |
| >>Paging Policy Indicator | O |  | INTEGER (0..7, …) | Used for paging policy differentiation as specified in TS 23.501 [9]). |
| >>Allocation and Retention Priority | O |  | 9.3.1.19 |  |
| >>5QI | O |  | INTEGER (0..255, …) | Indicates the 5QI associated with the PDU session |
| >>DL Data Size | O |  | INTEGER (1.. 96000, ...) | Unit: Byte. |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

---------------------------------------------------------Next of Change------------------------------------------------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Skip unrelated part>

PagingOrigin ::= ENUMERATED {

non-3gpp,

...

}

PagingPolicyDifferentiation ::= SEQUENCE {

pDUSessionID						PDUSessionID,

pagingPolicyDifferentiation-list	PagingPolicyDifferentiation-list	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PagingPolicyDifferentiation-ExtIEs} }	OPTIONAL,

...

}

PagingPolicyDifferentiation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingPolicyDifferentiation-list ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF PagingPolicyDifferentiation-Item

PagingPolicyDifferentiation-Item ::= SEQUENCE {

qosFlowIdentifier					QosFlowIdentifier,

pagingPolicyIndicator				PagingPolicyIndicator				OPTIONAL,

allocationAndRetentionPriority		AllocationAndRetentionPriority		OPTIONAL,

fiveQI								FiveQI								OPTIONAL,

dl-DataSize			 				INTEGER (0..96000, ...)				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PagingPolicyDifferentiation-Item-ExtIEs} }	OPTIONAL,

...

}

PagingPolicyDifferentiation-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

---------------------------------------------------------End of Change------------------------------------------------------