3GPP TSG-RAN WG3 Meeting #123	R3-240221

Athens, GR, 26th Feb. – 1st Mar. 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optimization on handling of DL non-SDT in case of RA-SDT without UE anchor relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 (DL-Non-SDT) |  |  |  |  |  |  | Date: |  | 2024-01-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | On how to optimize the signalling procedure for the DL non-SDT data arrival in case of RA-SDT without UE anchor relocation, RAN3 discussed and suggested to optimize the procedure as below, and send the LS to confirm with RAN2 (R2-2309436/R3-234589): When DL non-SDT data arrives at last serving gNB during ongoing SDT data transfer and last serving gNB decides to triggers UE transition into RRC_CONNECTED state, including a new indicator in the RRCRelease message could be one solution to allow the SDT UE transiting into RRC_INACTIVE state and UE automatically initiates a new RRCResumeRequest, then the gNB could move the UE into RRC_CONNECTED state by sending RRCResume message. This would avoid sending unnecessary RAN Paging messages in the cells of the RNA and could reduce the delay to trigger transition into RRC_CONNECTED state. In the LS reply (R2-2313759/R3-240015), RAN2 agreed the benefits of the proposed solution from RAN3. The solution is feasible from RAN2 perspective and has small impacts on RAN2 spec, and RAN2 decided to agree on the related CR for TS 38.331 and 38.306. As above, the stage 2 texts in TS 38.300 should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the texts on handling of the DL non-SDT data arrival in case of RA-SDT without UE anchor relocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures texts on handling of the DL non-SDT data arrival in case of RA-SDT without UE anchor relocation is not optimized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2. The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides not to relocate the full UE context for SDT.

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address. The UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB. Then UL/DL GTP-U tunnels are established for DRBs configured for SDT, if any, and the UL SDT data and/or signalling, if any, are forwarded to the last serving gNB, and then delivered to the core network.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers it to the UE.

NOTE 1a:	 In case DL non-SDT data or DL non-SDT signalling arrives, or UE assistance information (i.e. UL non-SDT data arrival indication) is received from the UE, the last serving gNB terminates the SDT procedure by sending the RRCRelease message with an indicator to allow the SDT UE transiting into RRC_INACTIVE state and automatically initiates a new RRCResumeRequest, then UE is transited from RRC_INACTIVE to RRC_CONNECTED state as specified in 9.2.2.4.1.

NOTE 1b:	 The last serving gNB may terminate the SDT procedure and direct the UE to continue in RRC_INACTIVE state by sending the RRCRelease message based on (e.g. large size of) DL SDT data or DL SDT signalling.

6.	The receiving gNB detects the end of SDT session and sends the RETRIEVE UE CONTEXT CONFIRM message including whether this is a "normal" end of SDT transaction or a radio link problem, or large SDT volume from BSR.

7.	Upon receiving the RETRIEVE UE CONTEXT CONFIRM message and deciding to terminate the SDT, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The receiving gNB shall release the established partial UE context.

NOTE 2:	Void.

NOTE 3:	Void.

8. The receiving gNB sends the RRCRelease message to the UE.

NOTE 4:	Void.

NOTE 5:	The last serving gNB may terminate the SDT procedure and direct the UE to continue in RRC_INACTIVE state by sending the RRCRelease message upon receiving the indication about large uplink SDT data from the BSR from the receiving gNB in step 6.

9.	The UE moves to RRC_INACTIVE state if the suspend indication is included in the RRCRelease message. Or else, the UE moves to RRC_IDLE state.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>