3GPP TSG-RAN WG3 #123	R3-240232

Athens, Greece, 26 Feb. – 1 Mar. 2024

Title: 	Discussion on Multicast CU to DU Common RRC Information

Source: 	Samsung

Agenda item:	9.1.6.2.

Document Type:	Discussion

1. Introduction

The documents discuss the small corrections in F1 for the MBS Multicast CU to DU Common RRC Information in MULTICAST COMMON CONFIGURATION REQUEST message.

2. Discussion

The purpose of the MULTICAST COMMON CONFIGURATION REQUEST message is used by the gNB-CU to send the configuration which is common to all multicast contexts to the gNB-DU. The message includes Multicast CU to DU Common RRC Information IE carrying Multicast Common CU2DU Cell List IE.

This is a complex nested structure with two layers of choice, as shown in below. This structure can be used to establish, update, or delete information indicated by a certain IE. E.g. if No CU2DU Cell Info provided is included in the first layer of choice, it means the previously configured common CU2DP Cell Info is released.

However, in the ASN.1, the first layer of choice is missing. As showed in the left part. MulticastCommonCu2DUCellInformaiton is included in the MulticastCommonCU2DUCell-Item together with NRCGI. Below this IE, there should be a Choice type, where a choice is Common CU2DU Cell Info Setup or Update. Another choice is No CU2DU Cell Info provided. But in ASN.1, below this IE, define a sequence of MBS Multicast Neighboring Cell List Item and ThreshouldMBS List Item. i.e. the information contained in the blue background box in the left figure are missing in the ASN.1 in the right figure.

Figure 1

#### TS38.473 V18.0.0 #### 9.3.1.314 Multicast CU to DU Common RRC Information This IE includes multicast specific CU to DU common RRC information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Multicast Common CU2DU Cell List; 0..1. >Multicast COMMON CU2DU Cell Item; 1 .. <maxCellingNBDU>. >>NR CGI; M; NR CGI 9.3.1.12. >>CHOICE Multicast Common CU2DU Cell Information; M. >>>Common CU2DU Cell Info Setup or Update. >>>>CHOICE MBS Multicast Neighbour Cell List Item; O. >>>>>MBS Multicast Neighbour Cell List Information provided; Update MBS Multicast Neighbour Cell List Information 9.3.1.315. >>>>>No MBS Multicast Neighbour Cell List provided; NULL. >>>>CHOICE ThresholdMBS-List Item; O. >>>>>ThresholdMBS-List Information provided; Update ThresholdMBS-List Information 9.3.1.316. >>>>>No ThresholdMBS-List provided; NULL. >>>No CU2DU Cell info Provided; NULL.

The misalignment between tabular and ASN.1 should be corrected by changing the ASN.1. Since No CU2DU Cell Info provided IE is used when the gNB-CU would like to release the previous configuration. If one NR CGI is sent together with this IE, means the cell deactivates the reception in RRC_Inactive.

Proposal 1: To agree the CR to correct ASN.1 for Multicast CU to DU Common RRC Information.

Another small correction is the tabular name in 9.2.14.19, it should be “MULTICAST COMMON CONFIGURATION RESPONSE’ instead of “MULTICAST COMMON CONFIGURATION CONFIRM”.

Proposal 2: To correct the name to MULTICAST COMMON CONFIGURATION RESPONSE in section 9.2.14.19.

3 Proposals

This documents discuss the misalignment between tabular and ASN.1 and proposed to correct the ASN.1

Proposal 1: To agree the CR to correct ASN.1 for Multicast CU to DU Common RRC Information.

Proposal 2: To correct the name to MULTICAST COMMON CONFIGURATION RESPONSE in section 9.2.14.19.

4. Reference

[1] TS38.473 V18.0.0