3GPP TSG RAN3 Meeting#123                                   R3-240241

# Athens, Greece, 26 February - 1 March, 2024

Agenda item:	9.1.2.1

Source: 	Samsung

Title: 	Discussion on corrections to QMC support in NR-DC

Document for:	Discussion and decision

Introduction

Last RAN3 meeting concluded the R18 QoE WI. And this meeting aims at discussing corrections to R18 QoE WI, with potential open issue as follows,

e.g., stage2 and stage3 text refinement, including potential corrections on MN-SN coordination in NR-DC.

In this contribution, we further discuss corrections on MN-SN coordination in NR-DC.

Discussion

2.1 Correction on removing the redundancy

Last RAN3 meeting concluded the R18 QoE WI, and the latest specs have captured corresponding enhancement.

In XnAP v18.0.0, however, our understanding is that the current version is a little redundant in some places, and we would like to ask RAN3 to further consider such redundancy, like what we commented last meeting, i.e. the place where the QMC configuration related coordination is performed between MN and SN, which is specified as follows,

9.2.3.197	QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node, or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used to request from the S-NG-RAN node to indicate whether it is interested in receiving further RVQoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (true, ...) | This IE indicates the preferred SRB for receiving further RAN Visible QoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| >>>>>>>>>>>>>>>>>>Similar parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |

9.2.3.198	QMC Coordination Response

This IE contains the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, or the information that the S-NG-RAN node needs to provide to the S-NG-RAN node in response to the QMC Coordination Request.

| IE/Group Name |  | Presence | Range | IE type and reference | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>>>>>>>>>>>>>>>>>Similar parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |
| SN to MN QMC Coordination Response List |  |  | 0..1 |  |  |  |
| >SN to MN QMC Coordination Response Item |  |  | 1..<maxnoofUEAppLayerMeas> |  |  |  |
| >>QoE Reference |  | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |  |
| >>QoE Reporting Path Response |  | O |  | ENUMERATED (accepted, rejected, …) |  |  |
| >>RVQoE Reporting Path Response |  | O |  | ENUMERATED (accepted, rejected, …) |  |  |
| >>Further RAN Visible QoE Interest Response |  | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the S-NG-RAN node is interested in receiving further RAN Visible QoE reports. |  |
| >>Further RAN Visible QoE Reporting Path |  | O |  | ENUMERATED (srb4, srb5, ...) | This IE is to indicate the preferred path for further RAN Visible QoE reportring. |  |
| >>Preferred RAN Visible QoE Configuration |  | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the S-NG-RAN node. |  |

However, we think the three pairs of the captured IEs marked with yellow background is a little redundant. Note that all these three pairs of IEs are used for RVQoE configuring node to initiate the coordination with non-RVQoE configuring node, and we believe there’s possibility to simplify the stg3 details without sacrificing the readability.

- Further RAN Visible QoE Reporting Path IE is used for the requesting node to indicate the preferred SRB for receiving further RVQoE reports. Our understanding is that the function of such IE can already been achieved by the RAN Visible QoE Reporting Path Request IE since both IEs serve for similar purpose. So the Further RAN Visible QoE Reporting Path IE in the QMC Coordination Request and Further RAN Visible QoE Reporting Path in the QMC Coordination Response are not needed and can be removed.

- For Further RAN Visible QoE Interest Inquiry IE and Current RAN Visible QoE Configuration IE in the QMC Coordination Request, we think only to keep one of them is enough. In our understanding, the peer node does not need to know the current RVQoE configuration; instead, if the peer node is interested in receiving RVQoE report, it is enough to provide the preferred RVQoE configuration as the response. In addition, the presence of the Preferred RVQoE Configuration IE is enough to implicitly indicate that the peer node is interested in further RVQoE reporting. So the Current RAN Visible QoE Configuration IE in the QMC Coordination Request and Further RAN Visible QoE Interest Response IE in the QMC Coordination Response are not needed and can be removed.

As a result, it is enough to only keep Further RAN Visible QoE Interest Inquiry IE in the QMC Coordination Request Item, and RAN Visible QoE Configuration Preference IE in the QMC Coordination Response Item.

Proposal 1: Remove Further RAN Visible QoE Reporting Path IE and Current RAN Visible QoE Configuration IE in the QMC Coordination Request in XnAP spec.

Proposal 2: Remove Further RAN Visible QoE Interest Response IE and Further RAN Visible QoE Reporting Path IE in the QMC Coordination Response in XnAP spec.

2.2 Correction on introduction of the new information

Another aspect to discuss is the introduction of the new information that are deemed required. In order to have a better understanding on whether there are something more needed to be specified over XnAP, we need to look into what have specified in RRC spec.

In terms of the QoE configuration, RAN2 has introduced three RRC parameters in QoE configuration, namely,

-       reportingSRB: This parameter indicates which SRB (SRB4 or SRB5) UE should use for QoE reporting.

-       rrc-SegAllowedSRB4: This parameter indicates whether NW allows RRC segmentation on SRB4

-       rrc-SegAllowedSRB5: This parameter indicates whether NW allows RRC segmentation on SRB5

AppLayerMeasConfig information element

-- ASN1START

-- TAG-APPLAYERMEASCONFIG-START

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowedSRB4-r17               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...,

[[

rrc-SegAllowedSRB5-r18               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

idleInactiveReportAllowed-r18        ENUMERATED {enabled}                                                       OPTIONAL  -- Need R

]]

}

MeasConfigAppLayer-r17 ::=           SEQUENCE {

measConfigAppLayerId-r17             MeasConfigAppLayerId-r17,

measConfigAppLayerContainer-r17      OCTET STRING (SIZE (1..8000))                                              OPTIONAL, -- Need N

serviceType-r17                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}   OPTIONAL, -- Need M

pauseReporting-r17                   BOOLEAN                                                                    OPTIONAL, -- Need M

transmissionOfSessionStartStop-r17   BOOLEAN                                                                    OPTIONAL, -- Need M

ran-VisibleParameters-r17            SetupRelease {RAN-VisibleParameters-r17}                         OPTIONAL, -- Cond ServiceType

...,

[[

reportingSRB-r18                     ENUMERATED {srb4, srb5, spare2, spare1}                                    OPTIONAL, -- Need M

appLayerMeasPriority-r18             INTEGER (1..16)                                                            OPTIONAL, -- Need M

appLayerIdleInactiveConfig-r18       AppLayerIdleInactiveConfig-r18                                             OPTIONAL  -- Need M

]]

}

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity-r17           ENUMERATED {ms120, ms240, ms480, ms640, ms1024}                            OPTIONAL, -- Need S

numberOfBufferLevelEntries-r17       INTEGER (1..8)                                                             OPTIONAL, -- Need R

reportPlayoutDelayForMediaStartup-r17 BOOLEAN                                                                   OPTIONAL, -- Need M

...,

[[

ran-VisibleReportingSRB-r18          ENUMERATED {srb4, srb5, spare2, spare1}                                    OPTIONAL  -- Need M

]]

}

-- TAG-APPLAYERMEASCONFIG-STOP

-- ASN1STOP

Remember that MN and SN negotiate reportingSRB value via QoE Reporting Path Request / QoE Reporting Path Response in XnAP.

Then let’s further consider the following cases,

Case 1: For those MN configured QoE configuration, MN can configure a QoE configuration to UE with reportingSRB=SRB5. Then, SN can forward QoE report to MCE directly.

Case 2: For those SN configured QoE, configuration, SN can configure a QoE configuration to UE with reportingSRB=SRB4. Then, MN can forward QoE report to MCE directly.

By considering Case 1 and Case 2, then the question becomes,

- How MN knows/sets rrc-SegAllowedSRB5 in Case 1 (and How SN knows/sets rrc-SegAllowedSRB4 in Case 2)?

In our understanding, the coordination between MN and SN is needed.

For example, in Case 1,

1) MN asks SN whether SN allows RRC segmentation explicitly or implicitly, and

2) SN responses whether SRB5 allows RRC segmentation or not explicitly, and

3) MN sets rrc-SegAllowedSRB5 based on SN’s response.

Proposal 3: The QoE configuring node explicitly/implicitly asks the non QoE configuring node whether the non QoE configuring node allows RRC segmentation in QMC Coordination Request over XnAP.

Proposal 4: The non-QoE configuring node explicitly responses whether it allows RRC segmentation or not in QMC Coordination Response over XnAP.

In addition, in terms of the QoE reporting, when UE sends QoE report to gNB, UE can include QoE session status information to indicate whether QoE session has started or ended, which is given in RRC spec as follows,

MeasReportAppLayer-r17 ::=            SEQUENCE {

measConfigAppLayerId-r17              MeasConfigAppLayerId-r17,

measReportAppLayerContainer-r17       OCTET STRING                                                             OPTIONAL,

appLayerSessionStatus-r17             ENUMERATED {start, stop}                                                 OPTIONAL,

ran-VisibleMeasurements-r17           RAN-VisibleMeasurements-r17                                              OPTIONAL

}

And this indication is primarily introduced to be used for MDT alignment by gNB.

Then let’s consider the following case,

If MN is the QoE configuring node, and the reporting SRB is SRB5 after the QMC coordination, the UE send the appLayerSessionStatus indication to SN via SRB5. However, the appLayerSessionStatus indication cannot be transferred from SN to MN according to current XnAP spec (Note that only QoE report and RVQoE report can be transferred between MN and SN via RRC TRANSFER message).

Therefore, MN is not aware of the QoE session status, and cannot configure the UE with MDT measurements that are to be aligned with the QoE measurements.

To solve this issue, we propose to transfer QoE session status information between MN and SN via e.g. XnAP RRC TRANSFER message.

Proposal 5: Transfer QoE session status information between MN and SN over XnAP.

Conclusion

In this paper, we provides further considerations for QoE on support of NR-DC. The following proposals are provided,

Proposal 1: Remove Further RAN Visible QoE Reporting Path IE and Current RAN Visible QoE Configuration IE in the QMC Coordination Request in XnAP spec.

Proposal 2: Remove Further RAN Visible QoE Interest Response IE and Further RAN Visible QoE Reporting Path IE in the QMC Coordination Response in XnAP spec.

Proposal 3: The QoE configuring node explicitly/implicitly asks the non QoE configuring node whether the non QoE configuring node allows RRC segmentation in QMC Coordination Request over XnAP.

Proposal 4: The non-QoE configuring node explicitly responses whether it allows RRC segmentation or not in QMC Coordination Response over XnAP.

Proposal 5: Transfer QoE session status information between MN and SN over XnAP.

References