3GPP TSG-RAN WG3 Meeting #123	R3-240294

# Athens, Greece, 26th Feb – 1st Mar, 2024

Agenda item:	23.3

Source:	Xiaomi

Title:	(TP for 38.455) Support of Sidelink Positioning

Document for:	Discussion and Decision

1	Introduction

For SL-PRS resource allocation, RAN3 had discussed whether LMF can request SL-PRS resource for a UE and send LS [1] to RAN2 to confirm it. On the other hand, RAN1 sends a LS [2] to RAN2/3 as below.

The following agreement was reached in RAN1 related to the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration for scheme 1

Agreement Send an LS to RAN2 and RAN3 with the following: - From RAN1 perspective, for scheme 1, it is important for the following request to be specified: - a gNB is able to receive a request from either LMF or UE for SL-PRS bandwidth - Action to RAN2 and RAN3 to consider how to specify support for such request, if not already specified.

From RAN1 perspective, the LMF can request at least the SL-PRS bandwidth for the UE. In this paper, we’d like to discuss how to support the LMF request and providing the TP in the Annex.

2	Discussion

According to RAN1/2 discussion, there’re two types of resource pools that can be used for SL-PRS transmission, including:

- Shared resource pools, that can be used for R18 SL-PRS and legacy SL data transmission,

- Dedicated resource pools, that can be used for R18 SL-PRS transmission.

According to RAN2 agreement below, both shared resource pools and dedicated resource pools can be provided to UE.

Confirm that dedicated/shared RP can be configured at the same time.

Leave the resource pool selection to UE implementation among resource pools allowing SL-PRS transmission when resource selection is triggered for SL-PRS transmission.

The resource pools can be used by multiple UEs, how to select the received resource pools is UE implementation.

In our understanding, the LMF can provide the requested SL-PRS bandwidth to the serving gNB according to the SL positioning requirements, the serving gNB can consider it to configure proper SL-PRS resources pool(s) to the UE, and which resource pools are configured to UE are gNB implementation.

We think the NRPPa positioning information procedure can be used for LMF to request the SL-PRS bandwidth. Whether other information is needed depends on RAN2 discussion, we can wait for RAN2’s reply.

For dedicated resource pool, according to RAN1 agreement below, the SL-PRS bandwidth is same as resource pool bandwidth.

Agreement

For a dedicated resource pool, where SL PRS bandwidth is same as resource pool bandwidth, the following interpretation applies: SL PRS bandwidth corresponds to all PRBs of the resource pool bandwidth.

The RB numbers for a dedicated resource pool can be found in TS 38.331 as below:

#### –	SL-PRS-ResourcePool

The IE SL-PRS-ResourcePool specifies the configuration information for NR sidelink PRS dedicated resource pool.

SL-PRS-ResourcePool information element

-- ASN1START

-- TAG-SL-PRS-RESOURCEPOOL-START

SL-PRS-ResourcePool-r18 ::=                  SEQUENCE {

sl-PRS-PSCCH-Config-r18                      SetupRelease { SL-PSCCH-ConfigDedicatedSL-PRS-RP-r18}           OPTIONAL,   -- Need M

sl-StartRB-SubchannelDedicatedSL-PRS-RP-r18  INTEGER (0..265)                                                OPTIONAL,   -- Need M

sl-RB-Number-r18                             INTEGER (10..275)                                               OPTIONAL,   -- Need M

For shared resource pool, the SL-PRS bandwidth is same as SL resource pool, the RB numbers are also INTEGER (10..275).

SL-ResourcePool information element

-- ASN1START

-- TAG-SL-RESOURCEPOOL-START

SL-ResourcePool-r16 ::=            SEQUENCE {

sl-PSCCH-Config-r16                SetupRelease { SL-PSCCH-Config-r16 }                                  OPTIONAL,   -- Need M

<<<<<<omit the irrelevant part>>>>>

sl-FilterCoefficient-r16           FilterCoefficient                                                     OPTIONAL,   -- Need M

sl-RB-Number-r16                   INTEGER (10..275)                                                     OPTIONAL,   -- Need M

with above discussion, it is proposed to introduce a new Requested SL-PRS Transmission Information IE in NRPPa/F1AP Positioning Information Request message, and the new IE includes Requested SL-PRS Bandwidth to represent the request bandwidth for a UE. FFS on other information.

Proposal 1, RAN3 agree to introduce a new Requested SL-PRS Transmission Information IE in NRPPa/F1AP Positioning Information Request message for SL-PRS bandwidth request initiated by LMF.

Proposal 2, RAN3 agree the TP for NRPPa in the Annex.

3	Conclusion

In this contribution, we had the following observations and proposals.

Proposal 1, RAN3 agree to introduce a new Requested SL-PRS Transmission Information IE in NRPPa/F1AP Positioning Information Request message for SL-PRS bandwidth request initiated by LMF.

Proposal 2, RAN3 agree the TP for NRPPa in the Annex.

4	References

[1] R3-237860, LS on LMF involvement in SL-PRS resource allocation, RAN3

[2] R3-240009, LS on the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration, RAN1

5		Annex (TP for BL CR NRPPa)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

CG-SDT	Configured Grant Small Data Transmission

CID	Cell-ID (positioning method)

DL-PRS	Downlink Positioning Reference Signal

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

LMF	Location Management Function

LPP	LTE Positioning Protocol

MSAS	Multi-functional Satellite Augmentation System

NavIC	NAVigation with Indian Constellation

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference of Arrival

posSIB	Positioning SIB

PRS	Positioning Reference Signal (for E-UTRA)

QZSS	Quasi-Zenith Satellite System

RSRP	Reference Signal Received Power

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SBAS	Space Based Augmentation System

SL-PRS	Sidelink Positioning Reference Signal

SRS	Sounding Reference Signal

TEG	Timing Error group

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL SRS-RSRPP	UL SRS reference signal received path power

WAAS	Wide Area Augmentation System

Z-AoA			Zenith Angles of Arrival

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If the Time Window Information for SRS IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, configure the UE to start transmitting its UL SRS transmission at the indicated time instance.

If the Positioning Validity Area Cell List IE within the Requested SRS Transmission Characteristics IE  is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for configuring SRS transmissions for the UE in the indicated validty area, and it shall include SRS Configuration IE the SFN initialisation time IE and the Positioning Validity Area Cell List IE in the POSITIONING INFORMATION RESPONSE message.

If the Requested SL-PRS Transmission Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SL-PRS resource pools for the UE as defined in TS 38.331[13].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Requested SL-PRS Transmission Information | O |  | 9.2.x7 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.x7	Requested SL-PRS Transmission Information

This IE contains the requested SL-PRS transmission information for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested SL-PRS Bandwidth | M |  | INTEGER (10..275) | RB numbers for requested SL-PRS bandwidth |
| FFS |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for NRPPa

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

<<<<<<<<<<<<<<<<<<unchanged parts are skipped>>>>>>>>>>>>>>>>>>>>>>

UE-TEG-ReportingPeriodicity,

MeasurementPeriodicityNR-AoA,

SRSTransmissionStatus,

CGI-NR,

TimeWindowInformation-SRS-List,

TimeWindowInformation-Measurement-List,

PosValidityAreaCellList,

SRSNewCellIdentity,

SRSReservationRequestType,

RequestedSL-PRSTransmissionInformation

<<<<<<<<<<<<<<<<<<unchanged parts are skipped>>>>>>>>>>>>>>>>>>>>>>

id-UE-TEG-ReportingPeriodicity,

id-MeasurementPeriodicityNR-AoA,

id-SRSTransmissionStatus,

id-NewNRCGI,

id-TimeWindowInformation-SRS-List,

id-TimeWindowInformation-Measurement-List,

id-PosValidityAreaCellList,

id-SRSReservationRequestType,

id-RequestedSL-PRSTransmissionInformation

<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### -- POSITIONING INFORMATION REQUEST

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationRequest-IEs}},

...

}

PositioningInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional	}|

{ ID id-UEReportingInformation					CRITICALITY ignore	TYPE UEReportingInformation							PRESENCE optional	}|

{ ID id-UE-TEG-Info-Request						CRITICALITY ignore	TYPE UE-TEG-Info-Request								PRESENCE optional	}|

{ ID id-UE-TEG-ReportingPeriodicity				CRITICALITY reject	TYPE UE-TEG-ReportingPeriodicity					PRESENCE conditional	}|

{ ID id-TimeWindowInformation-SRS-List			CRITICALITY reject	TYPE TimeWindowInformation-SRS-List				PRESENCE optional	}|

{ ID id-RequestedSL-PRSTransmissionInformation	CRITICALITY ignore	TYPE RequestedSL-PRSTransmissionInformation		PRESENCE optional	},

-- The IE shall be present if the UE TEG Info Request IE is set to “periodic”

...

}

<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

RequestedSL-PRSTransmissionInformation ::= SEQUENCE {

requestedSL-PRSBandwidth	INTEGER (10..275),

iE-Extensions				ProtocolExtensionContainer { { RequestedSL-PRSTransmissionInformation-ExtIEs} } OPTIONAL,

...

}

RequestedSL-PRSTransmissionInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

id-SRSPortIndex													ProtocolIE-ID ::= 100

id-procedure-code-101-not-to-be-used							ProtocolIE-ID ::= 101

id-procedure-code-102-not-to-be-used							ProtocolIE-ID ::= 102

id-procedure-code-103-not-to-be-used							ProtocolIE-ID ::= 103

id-UETxTimingErrorMargin										ProtocolIE-ID ::= 104

id-MeasurementPeriodicityNR-AoA									ProtocolIE-ID ::= 105

id-SRSTransmissionStatus										ProtocolIE-ID ::= 106

id-nrofSymbolsExtended											ProtocolIE-ID ::= 107

id-repetitionFactorExtended										ProtocolIE-ID ::= 108

id-StartRBHopping												ProtocolIE-ID ::= 109

id-StartRBIndex													ProtocolIE-ID ::= 110

id-transmissionCombn8											ProtocolIE-ID ::= 111

id-ExtendedResourceSymbolOffset									ProtocolIE-ID ::= 112

id-NewNRCGI														ProtocolIE-ID ::= 113

id-Mobile-TRP-LocationInformation								ProtocolIE-ID ::= 114

id-Mobile-IAB-MT-UE-ID											ProtocolIE-ID ::= 115

id-MobileAccessPointLocation									ProtocolIE-ID ::= 116

id-CommonTAParameters											ProtocolIE-ID ::= 117

id-Bandwidth-Aggregation-Request-Information					ProtocolIE-ID ::= xx1

id-PosSRSResourceSet-Aggregation-List							ProtocolIE-ID ::= xx2

id-ReportingGranularitykminus1									ProtocolIE-ID ::= xx3

id-ReportingGranularitykminus2									ProtocolIE-ID ::= xx4

id-TimingReportingGranularityFactorExtended						ProtocolIE-ID ::= xx5

id-TimeWindowInformation-SRS-List								ProtocolIE-ID ::= xx6

id-TimeWindowInformation-Measurement-List						ProtocolIE-ID ::= xx7

id-UL-RSCPMeas													ProtocolIE-ID ::= xx8

id-SymbolIndex													ProtocolIE-ID ::= xx9

id-PosValidityAreaCellList 										ProtocolIE-ID ::= x10

id-SRSReservationRequestType									ProtocolIE-ID ::= x11

id-TransmissionCombPos											ProtocolIE-ID ::= x12

id-ResourceMapping												ProtocolIE-ID ::= x13

id-FreqDomainShift												ProtocolIE-ID ::= x14

id-C-SRS														ProtocolIE-ID ::= x15

id-ResourceTypePos												ProtocolIE-ID ::= x16

id-SequenceIDPos												ProtocolIE-ID ::= x17

id-PRSBWAggregationRequestInfo									ProtocolIE-ID ::= x18

id-AggregatedPosSRSResourceID-List								ProtocolIE-ID ::= x19

id-AggregatedPRSResourceSetList									ProtocolIE-ID ::= x20

id-TRPPhaseQuality												ProtocolIE-ID ::= x21

id-SRSNewCellIdentity											ProtocolIE-ID ::= x22

id-RequestedSL-PRSTransmissionInformation						ProtocolIE-ID ::= x23

<<<<<<<<<<<<<<<<<<<< Change End>>>>>>>>>>>>>>>>>>>>