3GPP TSG-RAN WG3 Meeting #123	R3-240328

# Athens, Greece, 26 February – 01 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Draft | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup of Stage 2 for Timing Resiliency and URLLC enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC-NR-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The stage 2 description of the TRS_URLLC-NR features contains errors that require cleanup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - §16.8.2.x: It is clarified that there are two alternatives for TSCTSF to receive RAN TSS information. Also, the description of RAN TSS reporting towards the CN/UE is cleaned up and aligned with event triggers defined in TS 38.401. - §16.17: The current description is missing the PDU Session Resource release procedure. However, rather than add it, the establishment and modification procedures are instead removed (i.e. reduced detail). - §18.1: The clock quality control information is stored in the UE context, and when the UE performs MO-SDT, the UE cannot indicate that it performs MO-SDT to retrieve clock quality information. Therefore, full UE context relocation is needed to enable the receiving gNB to provide clock quality information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 description of the TRS_URLLC-NR features remains unclear or contains errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.8.2.1, 16.8.2.2, 16.8.2.3, 16.8.3, 16.17, 18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | n/a |  |  |  |  |  |  |  |  |

start of changes

### 16.8.2	Network timing synchronization monitoring

#### 16.8.2.1	General

While time synchronization service is offered by the 5GS, the network timing synchronization status of the gNB may change. The gNB detects timing synchronization degradation or improvement locally and informs the consumer of the information as follows:

-	TSCTSF may receive information about timing synchronization status from the gNB via the AMF based on node-level reporting configuration or via OAM;

-	UE may receive clock quality information from the gNB based on UE-level clock quality reporting control information.

#### 16.8.2.2	Network timing synchronization monitoring towards CN

The gNB may receive a request for RAN timing synchronization status (TSS) information from the AMF as specified in TS 23.501 [3]. The RAN TSS information includes one or more of the following attributes: synchronization state, traceable to UTC, traceable to GNSS, clock frequency stability, clock accuracy, parent time source, as defined in Table 5.27.1.12-1 in TS 23.501 [3].

Based on the gNB’s capabilities of reporting timing synchronisation status, the gNB accepts or rejects the request from the AMF. The gNB is pre-configured with a threshold for each RAN TSS attribute it supports. When the gNB detects a primary source event as defined in TS 38.401 [4], a timing synchronisation status report is triggered towards the AMF to provide the current RAN timing synchronisation status information.

#### 16.8.2.3	Network timing synchronization monitoring towards UE

The gNB may receive clock quality reporting control information for a UE from the AMF as specified in TS 23.501 [3]. The clock quality reporting control information contains the clock quality detail level (i.e., “metrics” or “acceptable/not acceptable indication”) and clock quality acceptance criteria for the UE (if the clock quality detail level equals “acceptable/not acceptable indication”). Based on the clock quality reporting control information, the gNB determines what clock quality information to provide the UE:

-	If the clock quality detail level equals “clock quality metrics”, the gNB provides clock quality metrics supported by the gNB to the UE, i.e., one or more of the RAN TSS attributes.

-	If the clock quality detail level equals “acceptable/not acceptable indication”, the gNB indicates “acceptable” to the UE if the gNB’s timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates “not acceptable” to the UE. Clock quality acceptance criteria is defined based on one or more of the RAN TSS attributes.

To provide clock quality information to the UE:

-	For a UE in the RRC CONNECTED state, the gNB uses unicast RRC signalling. The RRC signalling includes Event ID and clock quality information.

-	For a UE that is not in the RRC_CONNECTED state, the UE first needs to establish or resume the RRC connection to receive the clock quality information from the gNB via unicast RRC signalling. The gNB broadcasts Event ID in SIB9 to notify its timing synchronization status. Event ID or gNB ID change serves as a notification for the Ues reading the SIB information that the RAN timing synchronization status information has changed.

The following figure describes the signalling procedure of gNB reporting clock quality information to a UE:

Figure 16.8.2.3-1: Signalling procedure of gNB reporting clock quality information to a UE

0.	The gNB is pre-configured with a threshold for each RAN TSS attribute it supports as described in TS 23.501 [3]. When there is a primary source event as defined in TS 38.401 [4], the gNB may detect a change on its timing synchronization state (e.g., degradation, failure, recovery).

1.	The gNB notifies a change on its timing synchronization operation using Event ID in SIB9. The Event ID scope is local to gNB.

2.	The UE in RRC_INACTIVE or RRC_IDLE determines if there is clock quality information update available at the gNB based on SIB9 information. For a UE in RRC_CONNECTED state, steps 2-3 can be skipped.

3.	If there is a RAN timing synchronization status update available, the UE’s RRC layer indicates this to the NAS layer which may request the RRC layer to initiate RRC Setup or RRC Resume procedure.

4.	The gNB determines clock quality information reporting to the UE (e.g., metrics or “acceptable/not acceptable”).

5.	The gNB sends the clock quality information to the UE via unicast RRC signalling.

### 16.8.3	RAN feedback for adaptation of Burst Arrival Time and Periodicity

The NG-RAN may support the proactive feedback and reactive feedback mechanisms as specified in TS 23.501 [3]. The NG-RAN can provide the feedback in order to align the arrival of the traffic bursts with the next expected transmission opportunity over the air interface in each direction (i.e. DL or UL) for a QoS flow.

next change

## 16.17	Support for TSN enabled Transport Network

The NG-RAN may support the TSN enabled Transport Network as specified in TS 23.501 [3].

next change

## 18.1	Support of SDT procedure over RACH

For SDT procedure over RACH, if the UE accesses a gNB other than the last serving gNB, the UL SDT data/signalling is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. Other SDT assistance information (e.g., single packet, multiple packets) may also be provided by the receiving gNB to help the decision of UE context relocation. If the UE is configured with the clock quality control information, the last serving gNB performs full UE context relocation to enable the receiving gNB to provide clock quality information.

If the last serving gNB decides not to relocate the full UE context, it transfers a partial UE context containing SDT RLC context information necessary for the receiving gNB to handle SDT via the Partial UE Context Transfer procedure.

Then, in case SDT is used for user data over DRBs, UL/DL tunnels are established for DRBs configured for SDT between the receiving gNB and the last serving gNB. The PDCP PDU of UL/DL data is transferred over the tunnels, until the last serving gNB terminates the SDT session and directs the UE to continue in RRC_INACTIVE by sending the RRCRelease message.

Or in case SDT is used for signalling, SRB PDCP PDUs are transferred between the receiving gNB and the last serving gNB via the XnAP RRC Transfer procedure, until the last serving gNB terminates the SDT session and directs the UE to continue in RRC_INACTIVE by sending the RRCRelease message.

During the SDT session, in case the receiving gNB detects that no more packets are to be transmitted, or radio link problem is detected, the receiving gNB may also request to terminate the SDT session to the last serving gNB via the UE Context Retrieve Confirmation procedure.

end of changes