3GPP TSG-RAN3 Meeting #123	R3-240434

# Athens, Greece, 26th Feb – 1st Mar 2024

Agenda item:	9.1.8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the support for HO with location-based trigger condition

Document for:	Discussion & decision

# 1 Introduction

This contribution discusses the RAN3 impact to support handover with location-based trigger condition.

# 2 HO with Location based trigger condition

This issue applies to both NR NTN and IoT NTN. The below analysis uses NR NTN NG-based HO as an example, and the analysis also apply to IoT NTN S1-based HO.

Handover with Time-based trigger condition and Location-based trigger condition are specified in Stage-2 TS 38.300. RAN3 Stage-3 specification TS38.413 has been updated to support HO with Time-based trigger condition. More specifically, the enhancement include:

- Transfer the Time Based Handover Information to target RAN.

- Allow Uplink RAN Early Status Transfer procedure and Downlink RAN Early Status Transfer procedure to support early data forwarding and discarding.

However, both Stage-2 and Stage-3 specifications do not describe how the HO with Location-based trigger condition is supported in NG-based HO. So it is unclear how the HO with Location-based trigger condition is supported in NG-based HO.

Observation 1: it is unclear how the HO with Location-based trigger condition is supported in NG-based HO.

In both time-based trigger condition and location-based trigger condition, the UE may remain connected with source RAN node for a period of time after the UE receive the HOCommand, until the time-based trigger condition or location-based trigger condition (and measurement-based trigger condition) is met. RAN3 has agreed the early data forwarding/discard can be used in time-based trigger condition to avoid packet loss/over-buffering in the target RAN node. Since the data forwarding is same in HO with time-based trigger condition and HO with location-based trigger condition, the previously agreed enhancement to Early Data Forwarding for HO with Time-based trigger condition is also applicable to HO with Location-based trigger condition.

Observation 2: the data forwarding issue in NG-based HO with Time-based trigger condition, and NG-based HO with Location-based trigger condition are the same. So the early data forwarding adopted in NG-based HO with time-based trigger condition is also applicable to NG-based HO with location-based trigger condition.

In current NG-based HO signaling, there is no indication to tell target gNB that source gNB is initiating a HO with Location-based Trigger Condition. When source gNB uses NG-based HO with Location-based trigger condition and initiate the NG-based HO procedure, the NG-based HO signaling is same as normal NG-based HO signaling.   Upon the reception of the HO signaling, which is same as normal NG-based HO signaling, target gNB will incorrectly consider the received NGAP HANDOVER REQUEST message is normal NG-based HO.

Observation 3: without new indication, target gNB cannot know source gNB is initiating a NG-based HO with Location-based trigger condition, and consider the incoming HO is a normal NG-HO.

This can cause issue later, i.e. when target gNB receives a further DOWNLINK RAN EARLY STATUS TRANSFER message. Current Uplink/Downlink RAN Early Status Transfer procedure is only used for DAPS HO and HO with time-based trigger condition, rather to be used in normal NG-based HO.  In case source gNB initiates the Early Status Transfer procedure, target gNB may treat the received DOWNLINK RAN EARLY STATUS TRANSFER message as a logical error, since the received DOWNLINK RAN EARLY STATUS TRANSFER message is only used for DAPS or HO with time-based trigger condition, but current state in the target gNB is “normal HO”. The handling of logical error is defined in TS 38.413 Section 10.4 (copied as below)

10.4 Logical Error

Logical error situations occur when a message is comprehended correctly, but the information contained within the message is not valid (i.e., semantic error), or describes a procedure which is not compatible with the state of the receiver.

Observation 4: without knowing source gNB is initiating a NG-HO with Location-based trigger condition, target gNB may treat the further DOWNLINK RAN EARLY STATUS TRANSFER message as a logical error.

To address the issue, Option 1 is to introduce the HO with Location-based Trigger Condition in Stage-3 (TS 38.413).

- add a new IE in Source NG-RAN Node to Target NG-RAN Node Transparent Container IE, to inform target gNB that source gNB is using Location-based trigger condition.

- clarify the Uplink RAN Early Status Transfer procedure and Downlink RAN Early Status Transfer procedure are applicable to HO with Location-based trigger condition.

When the target gNB receives the HANDOVER REQUEST includes the new indication, it reserves the radio resource and be prepared for the incoming HO that may happen at any time.  From the perspective of resource allocation in target gNB, it is similar to a HO with Time-Based Trigger Condition that Handover Window Start IE is set to value “0” and the Handover Window Duration IE is set to the maximum value.

Observation 5: From the perspective of resource allocation in target gNB, the NG-HO with Location-based Trigger Condition is similar to a NG-HO with Time-based Trigger Condition that the Handover Window Start IE is set to value "0" and the Handover Window Duration IE is set to the maximum value.

Based on this finding, another Option (i.e. Option 2) is to clarify the “HO with Location-based Trigger Condition” is treated as “HO with Time-based Trigger Condition” in Stage-2, and not affect Stage-3. For example, source gNB uses the location information to determine a rough or longer timing window, and use it in the NGAP HO signaling. In other words, no matter whether the Uu interface uses time-based trigger condition or location-based trigger condition, the NGAP Source NG-RAN Node to Target NG-RAN Node Transparent Container IE signaling contains the Time Based Handover Information IE. This can be captured in Stage-2. For example,

When a location-based trigger condition is used in NG-C based handover, the source gNB should estimate the handover window parameters and signal them to target NG-RAN node, as the handover with time-based trigger condition.

The reason to use “should” in above text is to avoid any potential IOT issue. In case using “may”, source gNB can choose to NOT include the Timing Window in the HO signaling, thus cause target gNB to treat the HO as a normal NG-based HO and the issues described above can happen.

In a summary, Option 2 is simple and avoid Stage-3 changes. We propose to adopt Option 2.

Proposal 1: Clarify the source gNB includes HO Timing Window information in the NGAP signaling, to support NG-based HO with Location-based trigger condition.

In addition, there is an issue in current TS38.300, which states:

When a time-based trigger condition is used, the source gNB may signal the corresponding parameters to a single target gNB via the Source NG-RAN Node to Target NG-RAN Node Transparent Container in an NG-C based handover, see TS 23.502 [22]. The source gNB signals the corresponding CHO configuration to the UE in the RRC Reconfiguration message during handover execution.

This “may” needs to be changed to “should”. Otherwise, the target gNB cannot know the HO is for a HO with Time-based Trigger Condition.

Proposal 2: in case of NG-HO with Time-based Trigger Condition, Source gNB should signal the HO Timing Window information to target gNB.

The draft TS38.300 CR can be found at ([3]).

Proposal 3: In case the proposals/CR is agreed, similar CR is needed for IoT NTN. We volunteer to draft a corresponding TS 36.300 CR.

# 4 Conclusions

In this contribution, we analyzed how to support NG-based HO with Location-based Trigger Condition. Our proposals are:

Observation 1: it is unclear how the HO with Location-based trigger condition is supported in NG-based HO.

Observation 2: the data forwarding issue is same in NG-based HO with Time-based trigger condition, and NG-based HO with Location-based trigger condition are the same. So the early data forwarding adopted in NG-based HO with time-based trigger condition is also applicable toNG-based HO with location-based trigger condition.

Observation 3: without new indication, target gNB cannot know source gNB is initiating a NG-HO with Location-based trigger condition, and consider the incoming HO is for a normal NG-HO.

Observation 4: without knowing source gNB is initiating a NG-HO with Location-based trigger condition, target gNB may treat the further DOWNLINK RAN EARLY STATUS TRANSFER message as a logical error.

Observation 5: From the perspective of resource allocation in target gNB, the NG-HO with Location-based Trigger Condition is similar to a NG-HO with Time-based Trigger Condition that the Handover Window Start IE is set to value "0" and the Handover Window Duration IE is set to the maximum value.

Proposal 1: Clarify the source gNB includes HO Timing Window information in the NGAP signaling, to support NG-based HO with Location-based trigger condition.

Proposal 2: in case of NG-HO with Time-based Trigger Condition, Source gNB should signal the HO Timing Window information to target gNB.

Proposal 3: In case the proposals/CR is agreed, similar CR is needed for IoT NTN. We volunteer to draft a corresponding TS 36.300 CR.

The draft TS38.300 CR can be found at ([3]).

# References

- 3GPP TS 38.300

- 3GPP TS 38.413

- R3-240435, CR for TS38.300.