3GPP TSG-RAN WG3 Meeting #123 	R3-240440

# Athens, Greece, 26 February – 01 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1314 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Context identification after successful cell switch and correction to S-CPAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core, NR_Mob_enh-Core, LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Cell RNTI can identify a UE context and a RRC connection. It is also used to help validate the UE in case of RRC connection resumption, RNA update, or RRC connection re-establishment as described in TS 38.331 and TS 33.501. For conditional mobility like LTM, a base station could prepare multiple configurations and assign corresponding C-RNTIs to be used. Once the UE fulfills a condition or is commanded to apply one of the prepared configuration, a corresponding C-RNTI shall be applied for that cell group. It is noticed that the C-RNTI IE is optional in the UE Context Setup Response message and the UE Context Modification Response message. However, as only receiving side behavior is described for C-RNTI and captured in the current TS 38.473 for the UE context response messages (i.e., during the preparation phase), it is not clear, upon execution, whether and how the CU determines the C-RNTI to use from the multiple C-RNTIs for the UE. - This issue was brought up (R3-237468) during the discussion of Rel-18 LTM but some companies would like to discuss its impact also to the conditional mobility in earlier releases. Although this issue might have existed since the introduction of conditional mobility, it is more profound when the prepared configurations can be kept for subsequent mobility (i.e., LTM or subsequent CPAC). - S-CPAC: It is incorrect that, upon the reception of Access Success, the subsequent CPAC preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU is considered as cancelled by the gNB-CU |  |  |  |  |  |  |  |  |
| Summary of change: |  | - gNB-DU provides the applied C-RNTI in the Access Success message. - S-CPAC: Upon reception of Access Success, the subsequent CPAC preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU is NOT considered as cancelled by the gNB-CU. The other LTM preparations or subsequent CPAC preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU are kept for subsequent mobility. Imapct analysis: This CR has isolated impact on Access Success procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | After execution of conditional mobility (legacy, LTM, or subsequent CPAC), without a clear guidance how the C-RNTI is determined, the CU might lose track of the exact C-RNTI and fail in validating a UE during an RRC connection re-establishment or a RRC state transitions. The subsequent CPAC preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU is considered as cancelled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.8.2, 9.2.2.14, 9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.3.8	Access Success

#### 8.3.8.1	General

The purpose of the Access Success procedure is to enable the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change or LTM or subsequent CPAC. The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Access Success procedure. Successful operation.

The gNB-DU initiates the procedure by sending a ACCESS SUCCESS message.

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU and consider all the other CHO preparations or conditional PSCell addition or conditional PSCell change accepted for this UE under the same UE-associated signaling connection in this gNB-DU as cancelled. Upon reception of the ACCESS SUCCESS message, the gNB-CU shall, if supported, consider that the UE context corresponding to the included C-RNTI IE is active. The other LTM preparations or subsequent CPAC preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU are kept for subsequent mobility.

Interaction with other procedure:

The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate gNB-DUs for this UE, if any.

#### 8.3.8.3	Abnormal Conditions

If the ACCESS SUCCESS message refers to a context that does not exist, the gNB-CU shall ignore the message.

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change or LTM or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI applied by the UE | YES | ignore |

<<<<<< NEXT CHANGE >>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<<<<<< SKIP UNCHANGED >>>>>>

-- **************************************************************

--

#### -- Access Success

--

-- **************************************************************

AccessSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ AccessSuccessIEs}},

...

}

AccessSuccessIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-NRCGI								CRITICALITY reject	TYPE NRCGI									PRESENCE mandatory	}|

{ ID id-C-RNTI								CRITICALITY ignore	TYPE C-RNTI									PRESENCE optional	},

...

}

<<<<<< END OF CHANGE >>>>