3GPP TSG-RAN WG3 Meeting #123	R3-240487

# Athens, Greece, 26 Feb-1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0352 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mobile IAB for RLF recovery and integration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The current mobile IAB-node RLF recovery chapter simply refers to the IAB inter-CU backhaul RLF recovery. The IAB inter-CU backhaul RLF recovery scenario just support two donor-CUs, but if the mobile IAB-MT and the IAB-DU connects to different IAB-donors before RLF, there exist three donor-CUs and the legacy RLF recovery procedure is not applicable. - In the mobile IAB-node integration procedure, after the F1 is set up and before the mobile IAB-node start serving UEs, the TMM procedure is not mandatory according to the current spec. If there is no UE access this mobile IAB-node, the TMM procedure may not be performed, then the QoS for F1-C and non-F1 traffic cannot be guaranteed. Furthermore, if no Transport Migration Management procedure is initialized after the F1 being set up, and if the MT’s CU receives an updated mobile IAB authorization status from core network, it cannot initialize a Transport Migration Modification procedure to inform the updated authorization status to DU’s CU because the MT’s CU has no clue on which CU is the F1-terminating IAB-donor-CU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add descriptions of the mobile IAB-node RLF recovery procedure to support the scenario where the new CU for the RLF recovery is different from the F1 terminating CU. - Change the TMM procedure after F1 setup in mobile IAB-node integration as a mandatory step. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, which will impact the mobile IAB RLF recovery and mobile IAB-node integration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The mobile IAB-node RLF recovery feature is not supported in the decoupled case. - If no UE connects to the mobile IAB after integration, the F1 terminating CU also cannot be informed the latest mobile IAB authorization status, causing the mobile IAB-DU may wrongly remain activated when the mobile IAB-node becomes not authorized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.3, 8.23.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

8.12.3	Mobile IAB-node integration

During the integration, the mobile IAB-MT and the mobile IAB-DU can connect to the same IAB-donor or to different IAB-donors. The procedure for the latter case is shown in Figure 8.12.3-1.

Figure 8.12.3-1: Decoupled mobile IAB-node integration procedure

Phase 1: Equivalent procedure to Phase 1 of the IAB-node integration in SA mode in clause 8.12.1, where the mobile IAB-node and the RRC-terminating IAB-donor correspond to IAB-node 2 and the IAB-donor, respectively. The mobile IAB-node selects the parent node based on a mobile-IAB-specific over-the-air indication (transmitted in SIB1). The mobile IAB-MT includes a mobile-IAB-node-specific indication in the RRCSetupComplete message to assist the RRC-terminating IAB-donor in selecting an AMF supporting mobile IAB.

Phase 2-1: Same as Phase 2-1 of procedure in clause 8.12.1.

Phase 2-2: Same as Phase 2-2 of procedure in clause 8.12.1.

Phase 3: Mobile IAB-DU part setup. In this phase, the mobile IAB-DU is configured via OAM. The configured information includes, e.g., the information of the F1-terminating IAB-donor-CU, to enable the mobile IAB-DU to initiate the establishment of F1-C to the F1-terminating IAB-donor-CU.  The mobile IAB-DU initiates the TNL establishment, and F1 setup (as defined in clause 8.5) with the F1-terminating IAB-donor-CU using the default BAP routing ID and default BH RLC channel configured by the RRC-terminating IAB-donor-CU in Phase 2-1 for upstream traffic. During the F1 setup, the mobile IAB-DU includes the gNB ID of the RRC-terminating IAB-donor-CU and the BAP address of the co-located mobile IAB-MT in the F1 SETUP REQUEST message. The mobile IAB-node determines this gNB ID based on the over-the-air broadcast (SIB1) by the RRC-terminating IAB-donor.

After the F1 is set up, the mobile IAB-node can start serving UEs. In addition, the F1-terminating IAB-donor-CU should initiate the IAB Transport Migration Management procedure, which at least includes the context for traffic related to the F1-C/non-F1 traffic, towards the RRC-terminating IAB-donor-CU as defined in clause 8.17.3.1.

Next Change

8.23.4	Mobile IAB-node RLF recovery

When the mobile IAB-MT detects backhaul RLF, the mobile IAB-MT can perform inter-CU backhaul RLF recovery procedure to another parent node underneath a different IAB-donor-CU. The procedure is the same as steps 1-18 of IAB inter-CU backhaul RLF recovery procedure described in clause 8.17.4 where the mobile IAB-node corresponds to the Recovery IAB-node.

In case mobile IAB-MT and the mobile IAB-DU connects to different IAB-donors before the backhaul RLF, the procedure for the backhaul RLF recovery in such decoupled case is shown in Figure 8.23.4-X.

Figure 8.23.4-X: Decoupled mobile IAB-node backhaul RLF recovery procedure

- Steps 1-17 of the backhaul RLF recovery procedure in clause 8.17.4 are performed to conduct backhaul RLF recovery of the mobile IAB-MT from the initial parent IAB-node to the new parent IAB-node. In these steps, the mobile IAB-node corresponds to the recovery IAB-node in clause 8.17.4, the initial and new RRC terminating IAB-donor-CUs correspond to the initial and new IAB-donor-CUs of clause 8.17.4, respectively.

- Same as step 14 of the topology adaptation procedure in clause 8.17.3.1, the backhaul resources are configured along the new path for the mobile IAB-DU’s F1-C traffic. In this step, the mobile IAB-node corresponds to the migrating IAB-node in clause 8.17.3.1, the new RRC-terminating IAB-donor-CU corresponds to the target IAB-donor-CU in clause 8.17.3.1, and the new path of the mobile IAB-node corresponds to the target path in clause 8.17.3.1.

- Step 15 of the topology adaptation procedure in clause 8.17.3.1 is performed to redirect the mobile IAB-DU’s F1-C to the new path, and report the new F1-U TNL information to the F1-terminating IAB-donor-CU. In this step, the mobile IAB-node corresponds to the migrating IAB-node in clause 8.17.3.1, the F1-terminating IAB-donor-CU corresponds to the source IAB-donor-CU in clause 8.17.3.1.

In addition, during this step, the mobile IAB-DU also passes to the F1-terminating IAB-donor-CU the gNB ID  of the new RRC-terminating IAB-donor-CU and the mobile IAB-node’s BAP address allocated by the new RRC-terminating IAB-donor-CU.

- Same as step 16-20 of the topology adaptation procedure in clause 8.17.3.1, where the F1-terminating IAB-donor-CU initiates IAB Transport Migration Management procedure to the new RRC-terminating IAB-donor-CU to provide the context of the offloaded traffic, the backhaul related configurations along the new path can be re-configured by the new RRC-terminating IAB-donor-CU, and the F1-U connections of the mobile IAB-node are migrated to the new path. In this step, the mobile IAB-node corresponds to the migrating IAB-node in clause 8.17.3.1, the F1-terminating IAB-donor-CU corresponds to the source IAB-donor-CU in clause 8.17.3.1, and the new RRC-terminating IAB-donor-CU corresponds to the target IAB-donor-CU in clause 8.17.3.1.

End of Change