3GPP TSG-RAN WG3 Meeting #123	R3-240490

# Athens, Greece, 26 Feb-1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0127 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mobile TRP location information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For acquiring the mobile TRP’s location information to calculate the UE location, the Mobile TRP Location Information IE is reported from the NG-RAN node to the LMF in the TRP Information IE and some measurement result report IEs, i.e., E-CID Measurement Result IE and TRP Measurement Result IE. According to the current spec, the Mobile TRP Location Information IE contains three child IEs: Location Information, Velocity Information, and Location Time Stamp, which are all optional. In TS38.473, the Location information is mandatory in the Mobile TRP Location Information IE. So there is misalignment for the IE design in the two specifications. We tend to think the “Location information” should be set as mandatory, for the following two reasons: - The Mobile TRP Location Information IE is designed to transfer at least the location information, which should not be optional. - Moreover, if all the child IEs of the Mobile TRP Location Information IE are optional, it looks possible that the content of the parent IE will be nothing, such abnormal case should be avoided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the presence of the “Location Information” IE in the “Mobile TRP Location Information” IE to “Mandatory” in the tabular and ASN.1. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, which will impact the mobile TRP location information report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobile TRP location information may not be reported to the LMF, so that the UE location may not be correctly calculated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.88, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

9.2.88	Mobile TRP Location Information

This IE contains location information for one mobile TRP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Location Information | M |  | OCTET STRING | Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14] |
| Velocity Information | O |  | OCTET STRING | Velocity of the mobile TRP, Includes the velocityEstimate IE as defined in TS 37.355 [14] |
| Location Time stamp | O |  | Time Stamp 9.2.42 | Indicates the time when the mobile TRP location information is generated. |

Next Change

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<

Mobile-TRP-LocationInformation ::= SEQUENCE {

location-Information			OCTET STRING	,

velocity-Information			OCTET STRING	OPTIONAL,

location-time-stamp				TimeStamp	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Mobile-TRP-LocationInformation-ExtIEs} } OPTIONAL,

...

}

Mobile-TRP-LocationInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

End of Change