3GPP TSG-RAN WG3 Meeting #123	R3-240541

# Athens, Greece, 26th Feb – 1st Mar, 2024

Agenda Item:	23.2

Source:	Huawei

Title:	(TP to BL 38.423, etc) Discussion on SL Positioning

Document for:

# 1	Introduction

This paper discusses the remaining issues for Rel-18 SL positioning based on the incoming LS.

# 2	Discussion on SL-PRS bandwidth

A LS is received from RAN1 with the following agreement related to scheme 1 resource allocation [1]:

Agreement Send an LS to RAN2 and RAN3 with the following: - From RAN1 perspective, for scheme 1, it is important for the following request to be specified: - a gNB is able to receive a request from either LMF or UE for SL-PRS bandwidth - Action to RAN2 and RAN3 to consider how to specify support for such request, if not already specified.

There were some discussions on LMF involvement for resource allocation in RAN3 during previous meetings but no agreement was reached so far. For Rel-18, scheme 1 resource allocation consists of three types:

- Dynamic grant: the NG-RAN dynamically allocate resources to the UE via PDCCH

- Configured grant type 1: RRC provides the configured sidelink grant for NR sidelink positioning.

- Configured grant type 2: RRC defines the periodicity of the configured sidelink grant while PDCCH can signal and activate the configured sidelink grant for sidelink positioning.

All the above methods are initiated by request from the UE. According to the LS, either the UE or the LMF should be enabled to send SL-PRS bandwidth to a gNB for SL-PRS configuration request. From UE perspective, it means the bandwidth information should be carried in RRC or MAC CE for the UE to request for SL-PRS configuration.  While for LMF, it could mean a new resource allocation method initiated by LMF, similar to legacy UL SRS resource allocation. Therefore, the existing Positioning Information Exchange procedures may be enhanced to support the new feature. While for the delivery of SL-PRS configurations from the gNB to the UE, the current mechanism is reused by gNB, i.e. via RRC and/or PDCCH

Observation 1: According to RAN1, LMF could send SL-PRS bandwidth to the serving gNB for scheme 1 resource allocation.

However, there is no whole picture for how the mechanism works from RAN3 perspective.  A potential use case for LMF-initiated resource allocation might be when the LMF decides sidelink positioning methods, the LMF could request the gNB to configure the SL-PRS transmissions in the UE(s) with expected bandwidth. However, the overall procedure is not clear e.g. how LMF select the bandwidth, how LMF knows the gNB decision, … and benefit to have a second way to set-up the SL-PRS is not clear.

Observation 2: The stage 2 procedure for LMF-initiated SL-PRS request is not clear.

Therefore, we only propose the following with the purpose to align with the incoming RAN1 LS and keep all the decision on the SL-PRS configuration in the gNB:

Proposal 1: Enhance Positioning Information Request message to carry SL-PRS bandwidth information for SL-PRS configuration with LMF involvement and keep all the decision on the SL-PRS configuration in the gNB.

# 3	BL CR Clean-up

We noticed remaining FFS related to SL-PRS in last BL CR related to NG-AP (Coverpage only ??!!?) [1] and Xn AP [2].

The FFS are:

- Related to the TP on “Ranging and SL Positioning Services”subclause 8.2.1.2

- Same subclause 8.2.4.2

- The new RSPP Transport QoS Parameters IE

We correct also few editorials.

Proposal 2: We propose to remove the SL-Pos FFS in NG-AP an Xn-AP BL CR by agreeing the TP proposal in Annex A and Annex B. It is including few editorials.

# 4	Conclusion

Observation 1: According to RAN1, LMF could send SL-PRS bandwidth to the serving gNB for scheme 1 resource allocation.

Observation 2: The stage 2 procedure for LMF-initiated SL-PRS request is not clear.

Proposal 1: Enhance Positioning Information Request message to carry SL-PRS bandwidth information for SL-PRS configuration with LMF involvement and keep all the decision on the SL-PRS configuration in the gNB.

Proposal 2: We propose to remove the SL-Pos FFS in NG-AP an Xn-AP BL CR by agreeing the TP proposal in Annex A and Annex B. It is including few editorials.

# 5	References

[1] R3-240009, R1-2312630, LS on the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration

[2] R3-238098, (BL CR to 38.413) Support of NR Positioning Enhancements

[3] R3-238099, (BL CR to 38.423) Support of NR Positioning Enhancements

# 6	Annex A: TP BL CR NG-AP (cover page)

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

| Reason for change: | The CR aims at introducing functionally support for Expanded and Improved NR Positioning. |
| --- | --- |
| Summary of change: | RAN3#119bise: - SL Positioning and Ranging service authorization information is signalled in the following messages: NG: - INITIAL CONTEXT SETUP REQUEST - UE CONTEXT MODIFICATION REQUEST - HANDOVER REQUEST - PATH SWITCH REQUEST ACKNOWLEDGE RAN3#120 (Incheon) - SLPP/RSPP Transport QoS parameters are introduced with auhtorization information RAN3#121bis (Xiamen) - Add semantic description for the clarification of Ranging and Sidelink Positioning Service Information - New procedure LPHAP Information Transfer RAN3#122 (Chicago) - Add semantics descriptions i.e. “This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services.” in the Ranging and Sidelink Positioning Service Information. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

If the Ranging and Sidelink Positioning Service Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, update the Ranging and Sidelink Positioning service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE, within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# 7	Annex A: TP BL CR XN-AP

# 4	General

<<<<<<<<<<<<<<<<<<<< No Change >>>>>>>>>>>>>>>>>>>>

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

5G ProSe:

-	If the 5G ProSe Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

- 	If the 5G ProSe PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.304 [48].

Ranging and SL Positioning Services:

- If the Ranging and Sidelink Positioning Authorized IE, within the Ranging and Sidelink Positioning Services Information IE, is included in the HANDOVER REQUEST message and set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the Ranging and Sidelink Positioning services.

If the PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287 [38].

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

5G ProSe:

-	If the 5G ProSe Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the UE Context Information - Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

- 	If the 5G ProSe PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it as defined in TS 23.304 [48].

Ranging and SL Positioning Services:

- If the Ranging and Sidelink Positioning Authorized IE, within the Ranging and Sidelink Positioning Services Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message and set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the Ranging and Sidelink Positioning services.

If the PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it as defined in TS 23.287[38].

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x2	RSPP Transport QoS Parameters

This IE provides information on the RSPP Transport QoS Parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RSPP Transport QoS Flow List |  | 1 |  |  |
| >RSPP Transport QoS Flow Item |  | 1..<maxnoofRSPPQoSFlows> |  |  |
| >>PQI | M |  | INTEGER (0..255, …) | PQI is a special 5QI as specified in TS 23.501 [9]. |
| >>RSPP Transport Bit Rates |  | 0..1 |  | Only applies for GBR QoS flows. |
| >>>Guaranteed Flow Bit Rate | M |  | Bit Rate 9.2.3.4 | Guaranteed Bit Rate for the RSPP QoS flow. Details in TS 23.501 [9]. |
| >>>Maximum Flow Bit Rate | M |  | Bit Rate 9.2.3.4 | Maximum Bit Rate for the RSPP QoS flow. Details in TS 23.501 [9]. |
| >>Range | O |  | ENUMERATED (m50, m80, m180, m200, m350, m400, m500, m700, m1000, …) | Only applies for groupcast. |
| RSPP Transport Link Aggregate Bit Rates | O |  | Bit Rate 9.2.3.4 | Only applies for Non-GBR QoS flows. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRSPPQoSFlows | Maximum no. of RSPP QoS flows allowed towards one UE for NR Ranging and Positioning sidelink communication, the maximum value is 2048. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>