3GPP TSG-RAN WG3 #123								R3-240549

Athens, Greece, 26th Feb – 1st Mar 2024

Agenda Item:	9.3.2

Source:	China Telecom,ZTE,CATT

Title: 	Discussion on RAN3 Impact of ATG

Document for:	Discussion

Introduction

The core part of Air-to-ground (ATG) WI led by RAN4 was completed in RAN#102 meeting[2]. The following enhancements were introduced in TS38.331[3]:

- cellBarredATG in SIB1

- New SIB for ATG: SIB22

- Location information for both serving cell and neighbour cells

Since there was not any TUs allocated in RAN3, the RAN3 impact need to be discussed. Therefore, this contribution aims to provide our views on RAN3 impact of ATG.

Discussion

Air-to-ground (ATG) network using ground-based cell towers that send signals up to an aircraft’s antenna(s) of onboard ATG terminal. In practical deployment, one 5G base station may configure one ATG cell and multiple terrestrial network cells. From our point of view, ATG cell may also support split architecture. If majority companies agreed with this view, the F1 impact obviously exists as the new SIB22 had been introduced. The following information in SIB22 include ATG-config, ATG-NeighCellConfig and ATG-cellReselectionSet :

SIB22-r18 ::=                         SEQUENCE {

atg-Config-r18                        ATG-Config-r18                                     OPTIONAL,     -- Need R

hs-ATG-cellReselectionSet-r18         ENUMERATED {true}                                  OPTIONAL,     -- Need R

atg-NeighCellConfigList-r18           ATG-NeighCellConfigList-r18                        OPTIONAL,     -- Need R

lateNonCriticalExtension              OCTET STRING                                       OPTIONAL,

...

}

ATG-NeighCellConfigList-r18 ::=       SEQUENCE (SIZE(1..maxCellATG-r18))  OF ATG-NeighCellConfig-r18

ATG-NeighCellConfig-r18 ::=           SEQUENCE {

atg-gNB-Location-r18                  ReferenceLocation-r17                              OPTIONAL,     -- Need R

heightgNB-r18                         INTEGER (-16384..16383)                          OPTIONAL,     -- Need R

carrierFreq-r18                       ARFCN-ValueNR                                      OPTIONAL,     -- Need R

physCellId-r18                        PhysCellId                                         OPTIONAL,     -- Need R

...

}

It is obvious that the above information in SIB22 shall be generated by gNB-DU. Therefore, it is need to add new SIB22 in gNB-DU System Information IE in TS38.473. In addition, SIB22 is also missing in TS38.470.

Proposal 1: The split architecture is supported in ATG network.

Proposal 2: to add SIB22 in TS38.473 and TS38.470.

Proposal 3: to agree the corresponding CRs [4]-[5].

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: The split architecture is supported in ATG network.

Proposal 2: to add SIB22 in TS38.473 and TS38.470.

Proposal 3: to agree the corresponding CRs [4]-[5].

Reference

- RP-233444 WI summary of R18 ATG

- RP-233443 Status report for WI on Air-to-ground network for NR

- RP-233916 Air-to-ground network for NR

- R3-240410, Correction on the missing SIBs for MBS and ATG in TS 38.470

- R3-24xxxx, Correction on the missing SIBs for MBS and ATG in TS 38.473