3GPP TSG-RAN3 Meeting # 123	R3-240577

# Athens, Greece,  26-02-2024 – 02-03-2024

Agenda Item:	23.3

Source:	Ericsson

Title:	Support of RedCap Positioning

Document for:	Discussion, Other

# 1	Introduction

This document describes the NRPPa impacts to support RedCap positioning.

The F1AP impacts are expected to mirror them.

# 2	Discussion - SRS Transmission Frequency Hopping

In RAN1’s new LS on new physical layer parameters [1], there are new rows for the NRPPa impacts regarding RedCap positioning. We copy the relevant columns below:

| SRS for Positioning with Tx Frequency Hopping for RedCap UEs | Bandwidth | Existing | In addition to current description in 38.455: For a RedCap UE with Tx FH capability, a request on bandwidth for SRS for positioning from the LMF to the serving gNB that exceeds RedCap UE bandwidth implies configuration of SRS for positioning with Tx FH configuration | Ref.: Bandwidth in 9.2.27 (38.455) | Bandwidth in (9.2.27) Requested SRS Transmission Characteristics |  | Agreement SRS for positioning with Tx hopping can be configured outside of the active UL BWP - The configuration may include SCS, CP size and bandwidth (position and size), which can use a SCS, CP size and bandwidth different from the UL active BWP Agreement For SRS Tx hopping, the configuration includes: • a hop bandwidth common to all hops o FFS: possible values • a single overlap value can be configured for all hops for the SRS resource o FFS: possible values • The starting slot offset and starting symbol for the SRS resource with tx hopping (first hop) o FFS: possible values • the starting slot offset and symbol for each of the hops following the first hop, o Note Up to ran2 to design signaling of the starting position for each hop, i.e. how the SRS resource configuration signaling indicates the starting slot offset and starting symbol for the hops following the first hop o FFS: possible values • The number of consecutive symbols in a hop common to all hops o FFS: possible values • The number of hops o FFS: possible values • UE does not expect to be configured for any hops across slot boundaries, i.e.the starting position + duration of a hop cannot exceed a slot duration • FFS: whether/how special handling for the last hop overlap Agreement For the values of the starting slot offset for each of the hops following the first hop in time: - Alt1: the value range can be {0,1,2…, nrof slot in periodicity -1} in slots for the slot offset. - The value range slot offset for each hop applies to both the periodic and semi-persistent SRS. - The periodicity in PeriodicityandOffset configured for each hop for a SRS resource with Tx hopping must be the same. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SRS for Positioning with Tx Frequency Hopping for RedCap UEs |  | New | SRS config with SRS hopping from the serving gNB to the LMF | Contains the following parameters: - SCS, CP size and bandwidth (position and size) of frequency region (reuse Uplink Channel BW-PerSCS-List) - a hop bandwidth common to all hops: Ref. parameter c-SRS (INTEGER (0 .. 63)) such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively when B_SRS equal 0 - a single overlap value for all hops for the SRS resource: ENUMERATED {0, 1, 2, 4} RBs - starting slot offset and starting symbol for the SRS resource with tx hopping (first hop) for periodic and SP-SRS: slot offset: INTEGER (0,1,2…, nrof slot in periodicity) in slots (reuse the SRS-PeriodicityAndOffset IE); starting symbol: INTEGER (0, ...13) (reuse startPosition in IE resourceMapping) - starting slot offset and symbol for each of the hops following the first hop for periodic and SP-SRS: slot offset: INTEGER (0,1,2…, nrof slot in periodicity -1) in slots (new IE); starting symbol: INTEGER (0, ...13) (new IE) - starting slot offset for the SRS resource with tx hopping (first hop and subsequent hops) for aperiodic SRS: INTEGER (1 ..32) in slots relative to the slot containing the DCI triggering the SRS for positioning with tx hopping - number of consecutive symbols in a hop common to all hops: ENUMERATED {1,2,4,8, 12} symbols (re-use the IE nrofsymbols in resourcemapping) - number of hops: ENUMERATED {2,3,4,5,6} (new IE) |  | In SRS Configuration in POSITIONING INFORMATION RESPONSE | Agreement SRS for positioning with Tx hopping can be configured outside of the active UL BWP - The configuration may include SCS, CP size and bandwidth (position and size), which can use a SCS, CP size and bandwidth different from the UL active BWP Agreement For SRS Tx hopping, the configuration includes: • a hop bandwidth common to all hops o FFS: possible values • a single overlap value can be configured for all hops for the SRS resource o FFS: possible values • The starting slot offset and starting symbol for the SRS resource with tx hopping (first hop) o FFS: possible values • the starting slot offset and symbol for each of the hops following the first hop, o Note Up to ran2 to design signaling of the starting position for each hop, i.e. how the SRS resource configuration signaling indicates the starting slot offset and starting symbol for the hops following the first hop o FFS: possible values • The number of consecutive symbols in a hop common to all hops o FFS: possible values • The number of hops o FFS: possible values • UE does not expect to be configured for any hops across slot boundaries, i.e.the starting position + duration of a hop cannot exceed a slot duration • FFS: whether/how special handling for the last hop overlap Agreement For the values of the starting slot offset for each of the hops following the first hop in time: - Alt1: the value range can be {0,1,2…, nrof slot in periodicity -1} in slots for the slot offset. - The value range slot offset for each hop applies to both the periodic and semi-persistent SRS. - The periodicity in PeriodicityandOffset configured for each hop for a SRS resource with Tx hopping must be the same. Agreement SRS for positioning with Tx hopping can be configured to be periodic, aperiodic or semi-persistent - The mechanism for aperiodic and semi-persistent SRS for positioning (DCI triggered and MAC-CE activation/deactivation, respectively) can be re-used - For aperiodic SRS, § The UE is configured with the slot offset for each hop, relative to the slot containing the DCI triggering the SRS for positioning with tx hopping • Note: all the hops are within 32 slots from the DCI triggering the SRS for positioning with tx hopping |
| SRS for Positioning with Tx Frequency Hopping |  | New | SRS config with SRS hopping from the LMF to a gNB to perform measurements | Contains the following parameters: - SCS, CP size and bandwidth (position and size) of frequency region (reuse Uplink Channel BW-PerSCS-List) - a hop bandwidth common to all hops: Ref. parameter c-SRS (INTEGER (0 .. 63)) such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively when B_SRS equal 0 - a single overlap value for all hops for the SRS resource: ENUMERATED {0, 1, 2, 4} RBs - starting slot offset and starting symbol for the SRS resource with tx hopping (first hop) for periodic and SP-SRS: slot offset: INTEGER (0,1,2…, nrof slot in periodicity) in slots (reuse the SRS-PeriodicityAndOffset IE); starting symbol: INTEGER (0, ...13) (reuse startPosition in IE resourceMapping) - starting slot offset and symbol for each of the hops following the first hop for periodic and SP-SRS: slot offset: INTEGER (0,1,2…, nrof slot in periodicity -1) in slots (new IE); starting symbol: INTEGER (0, ...13) (new IE) - starting slot offset for the SRS resource with tx hopping (first hop and subsequent hops) for aperiodic SRS: INTEGER (1 ..32) in slots relative to the slot containing the DCI triggering the SRS for positioning with tx hopping - number of consecutive symbols in a hop common to all hops: ENUMERATED {1,2,4,8, 12} symbols (re-use the IE nrofsymbols in resourcemapping) - number of hops: ENUMERATED {2,3,4,5,6} (new IE) |  | In SRS Configuration in (9.1.4.1) MEASUREMENT REQUEST | Agreement SRS for positioning with Tx hopping can be configured outside of the active UL BWP - The configuration may include SCS, CP size and bandwidth (position and size), which can use a SCS, CP size and bandwidth different from the UL active BWP Agreement For SRS Tx hopping, the configuration includes: • a hop bandwidth common to all hops o FFS: possible values • a single overlap value can be configured for all hops for the SRS resource o FFS: possible values • The starting slot offset and starting symbol for the SRS resource with tx hopping (first hop) o FFS: possible values • the starting slot offset and symbol for each of the hops following the first hop, o Note Up to ran2 to design signaling of the starting position for each hop, i.e. how the SRS resource configuration signaling indicates the starting slot offset and starting symbol for the hops following the first hop o FFS: possible values • The number of consecutive symbols in a hop common to all hops o FFS: possible values • The number of hops o FFS: possible values • UE does not expect to be configured for any hops across slot boundaries, i.e.the starting position + duration of a hop cannot exceed a slot duration • FFS: whether/how special handling for the last hop overlap Agreement For the values of the starting slot offset for each of the hops following the first hop in time: - Alt1: the value range can be {0,1,2…, nrof slot in periodicity -1} in slots for the slot offset. - The value range slot offset for each hop applies to both the periodic and semi-persistent SRS. - The periodicity in PeriodicityandOffset configured for each hop for a SRS resource with Tx hopping must be the same. Agreement SRS for positioning with Tx hopping can be configured to be periodic, aperiodic or semi-persistent - The mechanism for aperiodic and semi-persistent SRS for positioning (DCI triggered and MAC-CE activation/deactivation, respectively) can be re-used - For aperiodic SRS, § The UE is configured with the slot offset for each hop, relative to the slot containing the DCI triggering the SRS for positioning with tx hopping • Note: all the hops are within 32 slots from the DCI triggering the SRS for positioning with tx hopping |
| SRS for Positioning with Tx Frequency Hopping for RedCap UEs | reportedSRS-measurementBasedOnSingleOrMultihopRx | New | Indicates that the reported measurement is based on receiving single or multiple hops of UL SRS for positioning | {single hop, multiple hops} |  | In TRP Measurement Result | Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: • A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning • One [or more] measurements where each a measurement is associated with one received hop • FFS: indication of how many received hops / which received hops where used in the measurement report. • Note: no new measurement definition is introduced in RAN1 • FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together Agreement For measurements based on DL PRS with Rx frequency hopping or UL SRS with Tx hopping: - UE/gNB can report either a single-hop or multi-hops measurement. - Indication of which of a single-hop or multi-hops measurement is optionally reported. o Note: mapping of the indicator to performance requirement(s), or impact to performance requirement(s), is up to RAN4 |

Based on the above input from RAN1, the IEs in NRPPa that require update consist of

- 9.2.27 Requested SRS Transmission Characteristics

- 9.2.28 SRS Resource Configuration within the 9.2.30 Positioning SRS Resource

- 9.2.37 TRP Measurement Result

The impacts to NRPPa can be translated as below:

### 9.2.27 Requested SRS Transmission Characteristics This IE contains the requested SRS configuration for the UE. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Skipped rows unaffected. SRS Frequency; O; INTEGER(0..3279165); NR ARFCN The carrier frequency of SRS transmission bandwidth.; YES; ignore. Overlapped SRS Frequency Hopping. >SCS; ENUMERATED(khz15, khz30, khz60, khz120). >Hop Bandwidth; M; ENUMRATED(prb104, prb48, prb132, prb64). >Overlap; M; ENUMERATED(rb0, rb1, rb2, rb4). >Number of symbols per hop; M; ENUMERATED(1,2,4,8, 12). >Number of hops; M; ENUMERATED(2,3,4,5,6). ### 9.2.30 Positioning SRS Resource This information element contains the SRS resource for positioning. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Skipped rows unaffected. CHOICE Spatial Relation Positioning; O. >SSB. >> NR PCI; M; INTEGER (0..1007). >>SSB index; O; INTEGER(0..63). >PRS. >>PRS ID; M; INTEGER(0..255). >>PRS Resource Set ID; M; INTEGER(0..7). >>PRS Resource ID; O; INTEGER(0..63). Overlapped frequency hopping; O. >Overlap; M; ENUMERATED(rb0, rb1, rb2, rb4). >Hop List; 1..<maxnohop-1>. >>Hop ID; M; INTEGER(0..6). >>Aperiodic; O. >>>Slot offset; M; INTEGER (1..32). >>>Start symbol; M; INTEGER (0..13). >>Semi-persistent; O. >>>Periodicity; M; ENUMERATED(slot 1, slot 2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot128, slot160, slot256, slot320, slot512, slot640, slot1280, slot2560, slot5120, slot10240, slot20480, slot40960, slot81920,…). >>>Offset; M; INTEGER(0..81919,…). >>Periodic; O. >>>Periodicity; M; ENUMERATED(slot 1, slot 2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot128, slot160, slot256, slot320, slot512, slot640, slot1280, slot2560, slot5120, slot10240, slot20480, slot40960, slot81920,…). >>>Offset; M; INTEGER(0..81919,…). Range bound; Explanation. maxnohop; Maximum no of hops for partially overlapped SRS frequency hopping. Value is 6.. ### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>. >CHOICE Measured Results Value; M. >>UL Angle of Arrival; M; 9.2.38; -. >>UL SRS-RSRP; M; INTEGER (0..126); -. >>UL RTOA; M; 9.2.39; -. >>gNB Rx-Tx Time Difference; M; 9.2.40; -. >>Z-AoA; M; 9.2.67; YES; reject. >>Multiple UL-AoA; M; 9.2.71; YES; reject. >>UL SRS-RSRPP; M; 9.2.72; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -. >Measurement Beam Information; O; 9.2.57; -. >SRS Resource type; O; 9.2.73; YES; ignore. >ARP ID; O; 9.2.75; YES; ignore. >LoS/NLoS Information; O; 9.2.77; YES; ignore. >hopping measurement; O; ENUMERATED(single hop, multiple hops, …); YES; ignore.

- As we can see from the above, there are several identified impacts to support RedCap Positioning and some impacts require RAN2 alignment with the RRC definition. It is proposed to update the NRPPa BLCRs as illustrated above and align F1AP accordingly.

- Proposal 1: add support of RedCap positioning in NRPPa and F1AP identified IEs: Requested SRS Transmission Characteristics, Positioning SRS Resource, TRP Measurement Result

# 3.	Conclusions and Proposals

Our observations and proposals are summarized below.

- Proposal 1: add support of RedCap positioning in NRPPa and F1AP identified IEs: Requested SRS Transmission Characteristics,  Positioning SRS Resource, TRP Measurement Result

# 4.	References

R3-240010, LS on Rel-18 higher-layers parameter list