| # 3GPP TSG-RAN WG3 Meeting #123 R3-240744 # Athens, Greece, 26th Feb - 1st March 2024 CR-Form-v12.2. CHANGE REQUEST. 38.401; CR; 0362; rev; -; Current version:; 18.0.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; Radio Access Network; X; Core Network. |
| --- |
| Title: | Correction on 38.401 for Time resilience and URLLC |  |  |  |  |  |  |  |  |  |
| Source to WG: | ZTE, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC-NR-Core |  |  |  |  |  | Date: |  |  | 2024-2-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The wording of step 7 in the procedures of RAN TSS reporting towards the CN was still a bit controversial at RAN3#122, the last meeting of R18 WI. For instance, the step 7 is actually to describe some primary source events, but the new event c) mentions some behaviour of DU when the event happens. This paper provides some rewording to better describe how the DU reports RAN TSS report to the CU, and to make the standard more comprehensive. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The step 7 and step 8 in the procedures of RAN TSS reporting towards the CN is reworded. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description about how the DU reports RAN TSS report to the CU is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.24.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/CR ...CR... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Changes Start

## 8.24	Timing resiliency service

The following clauses describe the overall procedures for RAN Timing Synchronisation Status (TSS) reporting involving gNB-CU/gNB-DU.

### 8.24.1	RAN TSS reporting towards the CN

The signaling flow for RAN TSS reporting towards the CN is shown in Figure 8.24.1-1. This procedure is used when the TSCTSF subscribes to RAN TSS reporting at the AMF as described in TS 23.502 [32].

Figure 8.24.1-1:	RAN TSS reporting towards the CN

0.	The gNB-DU is pre-configured with a threshold for each RAN TSS attribute it supports. The gNB-DU does not report RAN TSS attribute values better than the pre-configured thresholds, i.e. if a RAN TSS attribute has a value better than the pre-configured threshold, the gNB-DU reports the threshold value to the gNB-CU instead.

NOTE 1: 	It is assumed the pre-configured thresholds in the gNB-DU are sufficient to meet UE time sync performance requirement which are configured by the operator.

1.	The AMF requests RAN TSS reporting by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-CU.

2.	The gNB-CU requests RAN TSS reporting from at least one gNB-DU by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-DU.

NOTE 2:	It is up to gNB-CU implementation whether to send the request to all its gNB-DUs or to a particular subset of its gNB-DUs, depending on e.g. network topology.

3.	The gNB-DU replies to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS RESPONSE message.

4.	The gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS RESPONSE message. If the gNB-CU does not receive a successful response from at least one gNB-DU, the gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS FAILURE message and the flow stops at this step.

5. 	Upon sending the response to the gNB-CU, the gNB-DU provides a first RAN TSS report to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 3:	The RAN TSS attributes included in the report is up to gNB-DU implementation.

6.	The gNB-CU sends the TIMING SYNCHRONISATION STATUS REPORT message to the AMF. The message contains the RAN TSS attributes received from the gNB-DU and the RAN TSS Scope IE to indicate whether the scope of the RAN TSS report is “RAN node level” or “cell list level”.

7.	Later, the gNB-DU detects a primary source event:

a)	a RAN TSS attribute cannot meet a pre-configured threshold (i.e. status is degraded);

b)	a RAN TSS attribute meets the pre-configured threshold again (i.e. status is no longer degraded);

c)	a previously reported RAN TSS attribute value can no longer be met.

NOTE 4: Additional primary source events, if any, are up to gNB-DU implementation.

- Upon detecting the primary source event, the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message. If event a) occurred and event b) has not yet been reached, the gNB-DU performs periodic reporting.

9. 	Same as step 6.

End of Changes