3GPP TSG-RAN WG3 Meeting #123	R3-240958

Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0123 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MBS RAN Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell, CATT, ZTE, Samsung, CMCC, Google Inc., NEC, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an agreement achieved in RAN3 for RAN sharing MOCN scenario: - For MOCN scenario, only one set of shared F1-U tunnels is established and kept for MOCN scenario as long as there is one PLMN keeping the MBS service. Currently the F1-U tunnel Not Established IE is introduced per MBS Session, however, in case of location dependent sessions, multiple F1-U tunnels may need to be established. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the F1-U tunnel Not Established IE from the BC Bearer Context To Modify IE to the BC Bearer Context F1-U TNL Info at DU IE, in both, the location independent branch, and per Area Session in the location dependent branch. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource efficiency for RAN sharing for location dependent session not fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.119, 9.3.3.28, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------Start of the First Change---------------

#### 9.3.1.119	BC Bearer Context F1-U TNL Info at DU

This IE contains CU F1-U TNL information for an MBS Session. It may also contain per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  | - |  |
| >location independent |  |  |  |  |  |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  | - |  |
| >>F1-U tunnel Not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish F1-U tunnel for this MBS session. | YES | ignore |
| >location dependent |  |  |  |  |  |  |
| >>Location dependent MBS F1-U Information at DU |  | 1..<maxnoofMBSAreaSessionIDs> |  |  | - |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  | - |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  | - |  |
| >>>F1-U tunnel Not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish F1-U tunnel for this MBS Area session. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

-------------Start of the Next Change---------------

#### 9.3.3.28	BC Bearer Context To Modify

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify | O |  | BC Bearer Context NG-U TNL Info at 5GC 9.3.1.112 |  | - |  |
| BC MRB To Setup List | O |  | BC MRB Setup Configuration 9.3.1.114 |  | - |  |
| BC MRB To Modify List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >BC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.119 |  | - |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 |  | - |  |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. | - |  |
| BC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |

-------------Start of the Next Change---------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

//skip unchanged part

##### -- BCBearerContextToModify

BCBearerContextToModify ::= SEQUENCE {

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC	OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration			OPTIONAL,

bcMRBToModifyList					BCMRBModifyConfiguration			OPTIONAL,

bcMRBToRemoveList					BCMRBRemoveConfiguration			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCMRBModifyConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBModifyConfiguration-Item

BCMRBModifyConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

bcBearerContextF1U-TNLInfoatDU 	BCBearerContextF1U-TNLInfoatDU	OPTIONAL,

mbs-pdcp-config					PDCP-Configuration				OPTIONAL,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List		OPTIONAL,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBModifyConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBModifyConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextF1U-TNLInfoatDU ::= CHOICE {

locationindependent				MBSF1UInformationAtDU,

locationdependent				LocationDependentMBSF1UInformationAtDU,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextF1U-TNLInfoatDU-ExtIEs}}

}

BCBearerContextF1U-TNLInfoatDU-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBRemoveConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MRB-ID

//skip unchanged part

LocationDependentMBSF1UInformationAtCU ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformationAtCU-Item

LocationDependentMBSF1UInformationAtCU-Item ::= SEQUENCE {

mbsAreaSession-ID				MBSAreaSessionID,

mbs-f1u-info-at-CU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformationAtCU-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformationAtCU-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationDependentMBSF1UInformationAtDU ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformationAtDU-Item

LocationDependentMBSF1UInformationAtDU-Item ::= SEQUENCE {

mbsAreaSession-ID				MBSAreaSessionID,

mbs-f1u-info-at-DU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformationAtDU-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformationAtDU-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-F1UTunnelNotEstablished		CRITICALITY		ignore		EXTENSION		F1UTunnelNotEstablished		PRESENCE		optional	},

...

}

LocationDependentMBSNGUInformationAtNGRAN ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSNGUInformationAtNGRAN-Item

LocationDependentMBSNGUInformationAtNGRAN-Item ::= SEQUENCE {

mbsAreaSession-ID							MBSAreaSessionID,

mbsNGUInformationAtNGRAN						MBSNGUInformationAtNGRAN,

iE-Extensions							ProtocolExtensionContainer	{ { LocationDependentMBSNGUInformationAtNGRAN-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSNGUInformationAtNGRAN-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

MBSF1UInformationAtCU ::= SEQUENCE {

mbs-f1u-info-at-CU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformationAtCU-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformationAtCU-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformationAtDU ::= SEQUENCE {

mbs-f1u-info-at-DU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformationAtDU-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformationAtDU-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-F1UTunnelNotEstablished		CRITICALITY		ignore		EXTENSION		F1UTunnelNotEstablished		PRESENCE		optional	},

...

}

MBSNGUInformationAt5GC ::= CHOICE {

multicast 				MBSNGUInformationAt5GC-Multicast,

choice-extension	ProtocolIE-SingleContainer	{{MBSNGUInformationAt5GC-ExtIEs}}

}

MBSNGUInformationAt5GC-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

-------------End of the Changes---------------