3GPP RAN WG3 Meeting #123	R3-240970

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1091 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ECN marking reporting request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, Huawei, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-2-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | “9.3.1.266 ECN Marking or Congestion Information Reporting Request” has three choices in the tabular, but only two in the ASN.1. Interaction issue will happen in this case, e.g., the receiver got the message with the second branch, it will understand as ECN Marking at UPF, but it is actually the Congestion Monitoring Request. |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. To split the choice “eCNMarkingRequest” into two in the ASN.1, in accordance with the tabular. 2 Change the name “congestionMonitoringRequest” to “congestionInformationRequest” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tabular is not aligned with the ASN.1, and the gNB will not perform ECN marking or congestion information reporting per CN request properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1 Update the cover page. |  |  |  |  |  |  |  |  |

### 9.4.5	Information Element Definitions

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

#### -- E

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

ECNMarkingorCongestionInformationReportingRequest ::= CHOICE {

eCNMarkingAtRANRequest			ECNMarkingAtRANRequest,

eCNMarkingAtUPFRequest			ECNMarkingAtUPFRequest,

congestionInformationRequest		CongestionMonitoringRequest,

choice-Extensions				ProtocolIE-SingleContainer { {ECNMarkingorCongestionInformationReportingRequest-ExtIEs} }

}

ECNMarkingorCongestionInformationReportingRequest-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

ECNMarkingAtRANRequest ::= ENUMERATED {ul, dl, both, stop, ...}

ECNMarkingAtUPFRequest ::= ENUMERATED {ul, dl, both, stop, ...}

CongestionInformationRequest	::= ENUMERATED {ul, dl, both, stop, ...}

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////