3GPP TSG-RAN WG3 Meeting #123	                     R3-241035

Athens, Greece, 26th Feb – 1st  Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.482 | CR | 0005 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on E1AP PPID for DTLS over SCTP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split-Core |  |  |  |  |  | Date: |  |  | 2024-2-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Payload Protocol Identifier (PPID) to be used by SCTP for the application layer protocol E1AP should be assigned by IANA. As assigned by IANA, the PPID for DTLS over SCTP should be 69. However, in current specification, PPID for DTLS over SCTP in E1AP is wrongly set to 67. Therefore, the misalignment PPID value for E1AP DTLS over SCTP between E1AP and IANA should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the new reference of IANA (Stream Control Transmission Protocol (SCTP) Parameters). To replace the wrong PPID number with IANA reference. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has functional impact on the signalling transport compared to the previous version of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong Payload Protocol Identifier (PPID) for DTLS over SCTP in E1AP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.462 CR 0021 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This Cat. A CR to TS 37.482 is a mirror CR of previous release of TS 38.462. No REL-17/18 CR to TS 38.462 is needed as TS 38.462 is an empty pointer specification to TS 37.482 since REL-17. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture description".

[3]	IETF RFC 8200 (2017-07): "Internet Protocol, Version 6 (IPv6) Specification".

[4]	IETF RFC 791 (1981-09): "Internet Protocol".

[5]	IETF RFC 2474 (1998-12): "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[6]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[7]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[8]	3GPP TS 38.461: "NG-RAN; E1 layer 1".

[9]	3GPP TS 38.463: "NG-RAN; E1 Application Protocol (E1AP)".

[10]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage-2".

[11]	IETF RFC 6083 (2011-01): "Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[12]	IETF RFC 6335 (2011-08): "Internet Assigned Numbers Authority (IANA) Procedures for the Management of the Service Name and Transport Protocol Port Number Registry".

[13]	IANA: "Stream Control Transmission Protocol (SCTP) Parameters", [https://www.iana.org/assignments/sctp-parameters/sctp-parameters.xhtml#sctp-parameters-25].

Next Change

7	Transport layer

SCTP (IETF RFC 4960 [6]) shall be supported as the transport layer of E1 signalling bearer. The Payload Protocol Identifier to be used by SCTP for the application layer protocol E1AP and for DTLS over SCTP (IETF RFC 6083 [11]) is assigned by IANA  in [13]. The byte order of the ppid shall be big-endian.

End of Change