3GPP TSG-RAN WG3 Meeting #123	R3-241057

# Athens, Greece, 26 February-1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1766 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility Restrictions with NR NTN - X2AP Impacts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone, Thales Alenia Space, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2024-02-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An operator with satellite and terrestrial access should be able to restrict usage of satellite access for specific subscriptions; SA2 has identified some access restriction cases not currently covered. New codepoints need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new codepoints for NR-LEO/MEO/GEO/OTHERSAT to the RAT Restriction Information IE. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible for an operator to restrict access from E-UTRA to NR NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.413 CR 1924 TS 38.413 CR 1109 TS 38.423 CR 1202 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R3-240164: submitted to RAN3 #123 (changes to cover page) |  |  |  |  |  |  |  |  |

START OF CHANGES

### 9.2.3	Handover Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, e.g., handover and CCO, or for SCG selection during dual connectivity operation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.2.4 |  | – |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of "equivalent PLMNs list" as defined in TS 24.301 [14]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the serving PLMN and Equivalent PLMNs. | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| Forbidden TAs |  | 0..<maxnoofEPLMNsPlusOne> |  | intra E-UTRAN roaming restrictions | – |  |
| >PLMN Identity | M |  | 9.2.4 | The PLMN of forbidden TACs | – |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  | – |  |
| >>TAC | M |  | OCTET STRING(2) | The forbidden TAC | – |  |
| Forbidden LAs |  | 0..<maxnoofEPLMNsPlusOne> |  | inter-3GPP RAT roaming restrictions | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| >Forbidden LACs |  | 1..<maxnoofForbLACs> |  |  | – |  |
| >>LAC | M |  | OCTET STRING(2) |  | – |  |
| Forbidden inter RATs | O |  | ENUMERATED(ALL, GERAN, UTRAN, CDMA2000, …,GERAN and UTRAN, CDMA2000 and UTRAN) | inter-3GPP and 3GPP2 RAT access restrictions. "ALL" means that all RATs mentioned in the enumeration of this IE are restricted. | – |  |
| NR restriction in EPS as secondary RAT | O |  | ENUMERATED(NRrestrictedinEPSasSecondaryRAT, …) | Restriction to use NR when the NR is used as secondary RAT in EN-DC. | YES | ignore |
| Core Network Type Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | Includes any of the Serving PLMN or any PLMN of the Equivalent PLMNs listed in the Mobility Restriction List IE for which core network type restriction applies as specified in TS 23.501 [38]. | YES | ignore |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| >Core Network Type | M |  | ENUMERATED (5GCForbidden, …, EPCForbidden) | The indication indicates whether the UE is restricted to connect to 5GC or to EPC for this PLMN. | – |  |
| NR Restriction in 5GS | O |  | ENUMERATED(NRrestrictedin5GS, …) | Restriction to use NR when the NR connects to 5GS. | YES | ignore |
| Last NG-RAN PLMN Identity | O |  | 9.2.4 | Indicates the NG-RAN PLMN from where the UE formerly handed over to EPS and which is preferred in case of subsequent mobility to 5GS. | YES | ignore |
| Unlicensed Spectrum Restriction | O |  | ENUMERATED(UnlicensedRestricted, …) | Restriction to use unlicensed spectrum in the form of LAA or LWA/LWIP or NR-U as described in TS 23.401 [11]. | YES | ignore |
| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.401 [11]. | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| >RAT Restriction Information | M |  | BIT STRING { LEO (0), MEO (1), GEO (2), OTHERSAT (3), NR-LEO (4), NR-MEO (5), NR-GEO (6), NR-OTHERSAT (7)} (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMN Ids. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of equivalent PLMN Ids plus one. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbLACs | Maximum no. of forbidden Location Area Codes. Value is 4096. |

END OF CHANGES