3GPP TSG-RAN WG3 Meeting #123	R3-240609

- Athens, Greece, 26th Feb – 1st Mar, 2024

Agenda Item:	8.1

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on MultipleTrace

Document for:	Approval

# Introduction

CB: # 3_MultipleTrace

- Reply to SA5 based on solution existed in RAN3 already (e.g., Trace failure, Trace reference handling, only immediate MDT configuration needs to be transferred over HO)

- Avoid to bring NBC issue towards R15

(moderator - E///)

Summary of offline disc R3-240814

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## Issue 1: Support of Multiple Traces Forwarding

The first issue to be addressed is which of the following ways forward should be followed by RAN3:

- Support Multiple Trace Forwarding over RAN interfaces for Handover signaling and UE context retrieval

- Maintain legacy functionality and rely on forwarding of a single trace within Handover signalling and UE context retrieval

| Company | Option 1/Option 2 | Comment |
| --- | --- | --- |
| Ericsson | Option 2 | The impact of supporting multiple traces is very high while we do not see gains for that |

Discussion and conclusion:

## Issue 2: Changes to Trace Failure Indication

Current Trace Failure Indication procedure text states the following:

### 8.11.2	Trace Failure Indication

#### 8.11.2.1	General

The purpose of the Trace Failure Indication procedure is to allow the NG-RAN node to inform the AMF that a Trace Start procedure or a Deactivate Trace procedure or an ongoing trace has failed due to an interaction with a handover procedure or due to reception of multiple trace activations while the UE is in RRC-INACTIVE. The procedure uses UE-associated signalling.

#### 8.11.2.2	Successful Operation

Figure 8.11.2.2-1: Trace failure indication

The NG-RAN node initiates the procedure by sending a TRACE FAILURE INDICATION message. Upon reception of the TRACE FAILURE INDICATION message, the AMF shall take appropriate actions based on the failure reason indicated by the Cause IE.

In light of the agreements for Issue 1, is there any need to modify the Trace Failure Indication procedure description?

Companies are invited to comment on possible improvements to the Trace Failure Indication

| Company | Comment |
| --- | --- |
| Ericsson | One possible improvement could be the following The purpose of the Trace Failure Indication procedure is to allow the NG-RAN node to inform the AMF that a Trace Start procedure or a Deactivate Trace procedure or an ongoing trace has failed due to an interaction with a handover procedure or due to reception of multiple trace activations . The procedure uses UE-associated signalling |

Discussion and conclusion:

## Issue 3: In case of partial forwarding of traces, which trace to forward?

In case the legacy functionality is supported or in case only some of the configured traces are forwarded, which traces should be forwarded and which should be failed?

Option 1: Only the most recent trace(s) should be forwarded

Option 2: It is up to implementation which trace is forwarded

Companies are invited to provide inputs on this aspect

| Company | Option 1/Option 2 | Comment |
| --- | --- | --- |
| Ericsson | Option 1 |  |

# Conclusion, Recommendations [if needed]

If needed

# References