3GPP TSG-RAN WG3 #124																			R3-243325

Fukuoka, Japan, 20 - 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1393 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on preamble index allocation for intra-CU LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Lenovo, Verizon, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current F1AP specification, a list of preamble indexes is allocated in the UE-associated signalings, however those values are shared by multiple UEs that are configured with LTM and connected to the serving gNB-DU. With the limited number of preamble indexes serving multiple functions, scalability soon becomes a problem. To solve this issue, the best way is to allow the candidate gNB-DU to provide one preamble index upon request from the source gNB-DU for one UE that is configured with LTM. Therefore, new signalings should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove preamble indexes list from the Early UL Sync Information IE and section 9.3.1.329. - Introduce four new messages, DU-CU Dynamic Resource Allocation Request/Reponse and CU-DU Dynamic Resource Allocation Request/Response to request for preamble index to be allocated for certain UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Preamble index would soon become insufficient. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, 8.3.x1 (new), 8.3.x2 (new), 9.2.2.x1 (new), 9.2.2.x2 (new), 9.2.2.x3 (new), 9.2.2.x4 (new), 9.3.1.328, 9.3.4, 9.4.4, 9.4.5, 9.4.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmission Rev 0: R3-241818 |  |  |  |  |  |  |  |  |

<<<<<< START OF CHANGES>>>>>>

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| Multicast Context Notification | MULTICAST CONTEXT NOTIFICATION INDICATION | MULTICAST CONTEXT NOTIFICATION CONFIRM | MULTICAST CONTEXT NOTIFICATION REFUSE |
| Multicast Common Configuration | MULTICAST COMMON CONFIGURATION REQUEST | MULTICAST COMMON CONFIGURATION RESPONSE | MULTICAST COMMON CONFIGURATION REFUSE |
| DU-CU Dynamic Resource Allocation | DU-CU DYNAMIC RESOURCE ALLOCATION REQUEST | DU-CU DYNAMIC RESOURCE ALLOCATION RESPONSE |  |
| CU-DU Dynamic Resource Allocation | CU-DU DYNAMIC RESOURCE ALLOCATION REQUEST | CU-DU DYNAMIC RESOURCE ALLOCATION RESPONSE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| DU-CU Cell Switch Notification | DU-CU CELL SWITCH NOTIFICATION |
| CU-DU Cell Switch Notification | CU-DU CELL SWITCH NOTIFICATION |
| DU-CU TA Information Transfer | DU-CU TA INFORMATION TRANSFER |
| CU-DU TA Information Transfer | CU-DU TA INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| RACH Indication | RACH INDICATION |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| Mobile IAB F1 Setup Triggering | MIAB F1 SETUP TRIGGERING |
| Mobile IAB F1 Setup Outcome Notification | MIAB F1 SETUP OUTCOME NOTIFICATION |
| Broadcast Transport Resource Request | BROADCAST TRANSPORT RESOURCE REQUEST |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| DU-CU Access And Mobility Indication | DU-CU ACCESS AND MOBILITY INDICATION |

< skip unchanged part>

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

< skip unchanged part>

### 8.3.10	CU-DU Cell Switch Notification

#### 8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

#### 8.3.10.3	Unsuccessful Operation

Not applicable.

#### 8.3.10.4	Abnormal Conditions

Not applicable.

### 8.3.x1	DU-CU Dynamic Resource Allocation

#### 8.3.x1.1	General

The purpose of the DU-CU Dynamic Resource Allocation procedure is to enable the gNB-DU to inform the gNB-CU that a request is initiated for allocation of dynamic resources for the UE. The procedure uses UE-associated signalling.

#### 8.3.x1.2	Successful Operation

Figure 8.3.x1.2-1: DU-CU Dynamic Resource Allocation procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU DYNAMIC RESOURCE ALLOCATION REQUEST message.

Upon reception of the DU-CU DYNAMIC RESOURCE ALLOCATION REQUEST message, the gNB-CU shall, if supported, consider it as a request to allocate the dynamic resource for the UE and reply by sending the DU-CU DYNAMIC RESOURCE ALLOCATION RESPONSE message.

If the Dynamic Resource Allocation Request IE is set to "request for preamble index" in the DU-CU DYNAMIC RESOURCE ALLOCATION REQUEST message, the gNB-CU, shall, if supported, include the Preamble Index Allocated IE within the Dynamic Resource Allocation Response IE in the DU-CU DYNAMIC RESOURCE ALLOCATION RESPONSE message.

#### 8.3.x1.3	Unsuccessful Operation

Not applicable.

#### 8.3.x1.4	Abnormal Conditions

Not applicable.

### 8.3.x2	CU-DU Dynamic Resource Allocation

#### 8.3.x2.1	General

The purpose of the CU-DU Dynamic Resource Allocation procedure is to enable the gNB-CU to inform the gNB-DU that a request is initiated for allocation of dynamic resources for the UE. The procedure uses UE-associated signalling.

#### 8.3.x2.2	Successful Operation

Figure 8.3.x2.2-1: CU-DU Dynamic Resource Allocation procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU DYNAMIC RESOURCE ALLOCATION REQUEST message.

Upon reception of the CU-DU DYNAMIC RESOURCE ALLOCATION REQUEST message, the gNB-DU shall, if supported, consider it as a request to allocate the dynamic resource for the UE and reply by sending the CU-DU DYNAMIC RESOURCE ALLOCATION RESPONSE message.

If the Dynamic Resource Allocation Request IE is set to "request for preamble index" in the CU-DU DYNAMIC RESOURCE ALLOCATION REQUEST message, the gNB-DU, shall, if supported, include the Preamble Index Allocated IE within the Dynamic Resource Allocation Response IE in the CU-DU DYNAMIC RESOURCE ALLOCATION RESPONSE message.

#### 8.3.x2.3	Unsuccessful Operation

Not applicable.

#### 8.3.x2.4	Abnormal Conditions

Not applicable.

< skip unchanged part>

#### 9.2.2.x1	DU-CU Dynamic Resource Allocation Request

This message is sent by the gNB-DU to the gNB-CU to request for dynamic resource information for the UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Target Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| Dynamic Resource Allocation Request | M |  | ENUMERATED (request for preamble index, …) |  | YES | reject |

#### 9.2.2.x2	DU-CU Dynamic Resource Allocation Response

This message is sent by the gNB-CU to inform the gNB-DU about the dynamic resource information for the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Target Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Dynamic Resource Allocation Response |  | 1 |  |  | YES | reject |
| >Preamble Index Allocated | O |  | INTEGER (0..63) |  | - |  |

#### 9.2.2.x3	CU-DU Dynamic Resource Allocation Request

This message is sent by the gNB-CU to the gNB-DU to transfer the request for dynamic resource information for the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Target Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| Dynamic Resource Allocation Request | M |  | ENUMERATED (request for preamble index, …) |  | YES | reject |

#### 9.2.2.x4	CU-DU Dynamic Resource Allocation Response

This message is sent by the gNB-DU to inform the gNB-CU about the dynamic resource information for the UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Target Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Dynamic Resource Allocation Response |  | 1 |  |  | YES | reject |
| >Preamble Index Allocated | O |  | INTEGER (0..63) |  | - |  |

< skip unchanged part>

#### 9.3.1.328	Early UL Sync Configuration

This IE indicates the early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8]. |
| LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |
| >LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

< skip unchanged part>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

F1SetupResponse,

F1SetupFailure,

GNBDUConfigurationUpdate,

GNBDUConfigurationUpdateAcknowledge,

GNBDUConfigurationUpdateFailure,

GNBCUConfigurationUpdate,

GNBCUConfigurationUpdateAcknowledge,

GNBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

GNBDUResourceCoordinationRequest,

GNBDUResourceCoordinationResponse,

PrivateMessage,

UEInactivityNotification,

InitialULRRCMessageTransfer,

SystemInformationDeliveryCommand,

Paging,

Notify,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

PWSCancelRequest,

PWSCancelResponse,

PWSRestartIndication,

PWSFailureIndication,

GNBDUStatusIndication,

RRCDeliveryReport,

UEContextModificationRefuse,

F1RemovalRequest,

F1RemovalResponse,

F1RemovalFailure,

NetworkAccessRateReduction,

TraceStart,

DeactivateTrace,

DUCURadioInformationTransfer,

CUDURadioInformationTransfer,

BAPMappingConfiguration,

BAPMappingConfigurationAcknowledge,

BAPMappingConfigurationFailure,

GNBDUResourceConfiguration,

GNBDUResourceConfigurationAcknowledge,

GNBDUResourceConfigurationFailure,

IABTNLAddressRequest,

IABTNLAddressResponse,

IABTNLAddressFailure,

IABUPConfigurationUpdateRequest,

IABUPConfigurationUpdateResponse,

IABUPConfigurationUpdateFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

AccessAndMobilityIndication,

ReferenceTimeInformationReportingControl,

ReferenceTimeInformationReport,

AccessSuccess,

CellTrafficTrace,

PositioningMeasurementRequest,

PositioningMeasurementResponse,

PositioningMeasurementFailure,

PositioningAssistanceInformationControl,

PositioningAssistanceInformationFeedback,

PositioningMeasurementReport,

PositioningMeasurementAbort,

PositioningMeasurementFailureIndication,

PositioningMeasurementUpdate,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PositioningInformationUpdate,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

BroadcastContextSetupRequest,

BroadcastContextSetupResponse,

BroadcastContextSetupFailure,

BroadcastContextReleaseCommand,

BroadcastContextReleaseComplete,

BroadcastContextReleaseRequest,

BroadcastContextModificationRequest,

BroadcastContextModificationResponse,

BroadcastContextModificationFailure,

MulticastGroupPaging,

MulticastContextSetupRequest,

MulticastContextSetupResponse,

MulticastContextSetupFailure,

MulticastContextReleaseCommand,

MulticastContextReleaseComplete,

MulticastContextReleaseRequest,

MulticastContextModificationRequest,

MulticastContextModificationResponse,

MulticastContextModificationFailure,

MulticastDistributionSetupRequest,

MulticastDistributionSetupResponse,

MulticastDistributionSetupFailure,

MulticastDistributionReleaseCommand,

MulticastDistributionReleaseComplete,

PDCMeasurementInitiationRequest,

PDCMeasurementInitiationResponse,

PDCMeasurementInitiationFailure,

PDCMeasurementReport,

PDCMeasurementTerminationCommand,

PDCMeasurementFailureIndication,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

PosSystemInformationDeliveryCommand,

DUCUCellSwitchNotification,

CUDUCellSwitchNotification,

DUCUTAInformationTransfer,

CUDUTAInformationTransfer,

QoEInformationTransferControl,

RachIndication,

TimingSynchronisationStatusRequest,

TimingSynchronisationStatusResponse,

TimingSynchronisationStatusFailure,

TimingSynchronisationStatusReport,

MIABF1SetupTriggering,

MIABF1SetupOutcomeNotification,

MulticastContextNotificationIndication,

MulticastContextNotificationConfirm,

MulticastContextNotificationRefuse,

MulticastCommonConfigurationRequest,

MulticastCommonConfigurationResponse,

MulticastCommonConfigurationRefuse,

BroadcastTransportResourceRequest,

DUCUAccessAndMobilityIndication,

SRSInformationReservationNotification,

DUCUDynamicResourceAllocationRequest,

DUCUDynamicResourceAllocationResponse,

CUDUDynamicResourceAllocationRequest,

CUDUDynamicResourceAllocationResponse

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-DUCUAccessAndMobilityIndication,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementInitiationRequest,

id-PDCMeasurementInitiationResponse,

id-PDCMeasurementInitiationFailure,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand,

id-DUCUCellSwitchNotification,

id-CUDUCellSwitchNotification,

id-DUCUTAInformationTransfer,

id-CUDUTAInformationTransfer,

id-QoEInformationTransferControl,

id-RachIndication,

id-TimingSynchronisationStatus,

id-TimingSynchronisationStatusReport,

id-MIABF1SetupTriggering,

id-MIABF1SetupOutcomeNotification,

id-MulticastContextNotification,

id-MulticastCommonConfiguration,

id-BroadcastTransportResourceRequest,

id-SRSInformationReservationNotification,

id-DUCUDynamicResourceAllocation,

id-CUDUDynamicResourceAllocation

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

F1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

choice-extension	ProtocolIE-SingleContainer { { F1AP-PDU-ExtIEs} }

}

F1AP-PDU-ExtIEs F1AP-PROTOCOL-IES ::= { -- this extension is not used

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURES F1AP-ELEMENTARY-PROCEDURE ::= {

F1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

F1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

f1Setup							|

gNBDUConfigurationUpdate		|

gNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning				|

pWSCancel						|

gNBDUResourceCoordination		|

f1Removal						|

bAPMappingConfiguration			|

gNBDUResourceConfiguration		|

iABTNLAddressAllocation			|

iABUPConfigurationUpdate		|

resourceStatusReportingInitiation	|

positioningMeasurementExchange	|

tRPInformationExchange			|

positioningInformationExchange	|

positioningActivation			|

e-CIDMeasurementInitiation		|

broadcastContextSetup			|

broadcastContextRelease			|

broadcastContextModification	|

multicastContextSetup			|

multicastContextRelease			|

multicastContextModification	|

multicastDistributionSetup		|

multicastDistributionRelease	|

pDCMeasurementInitiation		|

pRSConfigurationExchange		|

measurementPreconfiguration	|

timingSynchronisationStatus	|

multicastContextNotification	|

multicastCommonConfiguration	|

dUCUDynamicResourceAllocation	|

cUDUDynamicResourceAllocation	,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery					|

dUCUCellSwitchNotification					|

cUDUCellSwitchNotification					|

dUCUTAInformationTransfer					|

cUDUTAInformationTransfer					|

qoEInformationTransferControl					|

rachIndication								|

timingSynchronisationStatusReport				|

mIABF1SetupTriggering						|

mIABF1SetupOutcomeNotification				|

broadcastTransportResourceRequest			|

dUCUAccessAndMobilityIndication			|

sRSInformationReservationNotification,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

reset F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-Reset

CRITICALITY				reject

}

f1Setup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1SetupRequest

SUCCESSFUL OUTCOME		F1SetupResponse

UNSUCCESSFUL OUTCOME	F1SetupFailure

PROCEDURE CODE			id-F1Setup

CRITICALITY				reject

}

gNBDUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBDUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBDUConfigurationUpdate

CRITICALITY				reject

}

gNBCUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBCUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBCUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBCUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBCUConfigurationUpdate

CRITICALITY				reject

}

uEContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextSetupRequest

SUCCESSFUL OUTCOME		UEContextSetupResponse

UNSUCCESSFUL OUTCOME	UEContextSetupFailure

PROCEDURE CODE			id-UEContextSetup

CRITICALITY				reject

}

uEContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

uEContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME	UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uEContextModificationRequired F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequired

SUCCESSFUL OUTCOME		UEContextModificationConfirm

UNSUCCESSFUL OUTCOME	UEContextModificationRefuse

PROCEDURE CODE			id-UEContextModificationRequired

CRITICALITY				reject

}

writeReplaceWarning F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

pWSCancel F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSCancelRequest

SUCCESSFUL OUTCOME		PWSCancelResponse

PROCEDURE CODE			id-PWSCancel

CRITICALITY				reject

}

errorIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

uEContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

initialULRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialULRRCMessageTransfer

PROCEDURE CODE			id-InitialULRRCMessageTransfer

CRITICALITY				ignore

}

dLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLRRCMessageTransfer

PROCEDURE CODE			id-DLRRCMessageTransfer

CRITICALITY				ignore

}

uLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULRRCMessageTransfer

PROCEDURE CODE			id-ULRRCMessageTransfer

CRITICALITY				ignore

}

uEInactivityNotification  F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEInactivityNotification

PROCEDURE CODE			id-UEInactivityNotification

CRITICALITY				ignore

}

gNBDUResourceCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceCoordinationRequest

SUCCESSFUL OUTCOME		GNBDUResourceCoordinationResponse

PROCEDURE CODE			id-GNBDUResourceCoordination

CRITICALITY				reject

}

privateMessage F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

systemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SystemInformationDeliveryCommand

PROCEDURE CODE			id-SystemInformationDeliveryCommand

CRITICALITY				ignore

}

paging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

notify F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Notify

PROCEDURE CODE			id-Notify

CRITICALITY				ignore

}

networkAccessRateReduction F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NetworkAccessRateReduction

PROCEDURE CODE			id-NetworkAccessRateReduction

CRITICALITY				ignore

}

pWSRestartIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

pWSFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

gNBDUStatusIndication 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUStatusIndication

PROCEDURE CODE			id-GNBDUStatusIndication

CRITICALITY				ignore

}

rRCDeliveryReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCDeliveryReport

PROCEDURE CODE			id-RRCDeliveryReport

CRITICALITY				ignore

}

f1Removal F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1RemovalRequest

SUCCESSFUL OUTCOME		F1RemovalResponse

UNSUCCESSFUL OUTCOME	F1RemovalFailure

PROCEDURE CODE			id-F1Removal

CRITICALITY				reject

}

traceStart F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

deactivateTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

dUCURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCURadioInformationTransfer

PROCEDURE CODE			id-DUCURadioInformationTransfer

CRITICALITY				ignore

}

cUDURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDURadioInformationTransfer

PROCEDURE CODE			id-CUDURadioInformationTransfer

CRITICALITY				ignore

}

bAPMappingConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BAPMappingConfiguration

SUCCESSFUL OUTCOME		BAPMappingConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	BAPMappingConfigurationFailure

PROCEDURE CODE			id-BAPMappingConfiguration

CRITICALITY				reject

}

gNBDUResourceConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceConfiguration

SUCCESSFUL OUTCOME		GNBDUResourceConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUResourceConfigurationFailure

PROCEDURE CODE			id-GNBDUResourceConfiguration

CRITICALITY				reject

}

iABTNLAddressAllocation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABTNLAddressRequest

SUCCESSFUL OUTCOME		IABTNLAddressResponse

UNSUCCESSFUL OUTCOME	IABTNLAddressFailure

PROCEDURE CODE			id-IABTNLAddressAllocation

CRITICALITY				reject

}

iABUPConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABUPConfigurationUpdateRequest

SUCCESSFUL OUTCOME		IABUPConfigurationUpdateResponse

UNSUCCESSFUL OUTCOME	IABUPConfigurationUpdateFailure

PROCEDURE CODE			id-IABUPConfigurationUpdate

CRITICALITY				reject

}

resourceStatusReportingInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusRequest

SUCCESSFUL OUTCOME		ResourceStatusResponse

UNSUCCESSFUL OUTCOME	ResourceStatusFailure

PROCEDURE CODE			id-resourceStatusReportingInitiation

CRITICALITY				reject

}

resourceStatusReporting F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

accessAndMobilityIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY				ignore

}

referenceTimeInformationReportingControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReportingControl

PROCEDURE CODE			id-ReferenceTimeInformationReportingControl

CRITICALITY				ignore

}

referenceTimeInformationReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReport

PROCEDURE CODE			id-ReferenceTimeInformationReport

CRITICALITY				ignore

}

accessSuccess F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessSuccess

PROCEDURE CODE			id-accessSuccess

CRITICALITY				ignore

}

cellTrafficTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-cellTrafficTrace

CRITICALITY				ignore

}

positioningAssistanceInformationControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationControl

PROCEDURE CODE			id-PositioningAssistanceInformationControl

CRITICALITY				ignore

}

positioningAssistanceInformationFeedback F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationFeedback

PROCEDURE CODE			id-PositioningAssistanceInformationFeedback

CRITICALITY				ignore

}

positioningMeasurementExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementRequest

SUCCESSFUL OUTCOME		PositioningMeasurementResponse

UNSUCCESSFUL OUTCOME	PositioningMeasurementFailure

PROCEDURE CODE			id-PositioningMeasurementExchange

CRITICALITY				reject

}

positioningMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementReport

PROCEDURE CODE			id-PositioningMeasurementReport

CRITICALITY				ignore

}

positioningMeasurementAbort F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementAbort

PROCEDURE CODE			id-PositioningMeasurementAbort

CRITICALITY				ignore

}

positioningMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementFailureIndication

PROCEDURE CODE			id-PositioningMeasurementFailureIndication

CRITICALITY				ignore

}

positioningMeasurementUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementUpdate

PROCEDURE CODE			id-PositioningMeasurementUpdate

CRITICALITY				ignore

}

tRPInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TRPInformationRequest

SUCCESSFUL OUTCOME		TRPInformationResponse

UNSUCCESSFUL OUTCOME	TRPInformationFailure

PROCEDURE CODE			id-TRPInformationExchange

CRITICALITY				reject

}

positioningInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-PositioningInformationExchange

CRITICALITY				reject

}

positioningActivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningActivationRequest

SUCCESSFUL OUTCOME		PositioningActivationResponse

UNSUCCESSFUL OUTCOME	PositioningActivationFailure

PROCEDURE CODE			id-PositioningActivation

CRITICALITY				reject

}

positioningDeactivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDeactivation

PROCEDURE CODE			id-PositioningDeactivation

CRITICALITY				ignore

}

e-CIDMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-E-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-E-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-E-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-E-CIDMeasurementTermination

CRITICALITY				ignore

}

positioningInformationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-PositioningInformationUpdate

CRITICALITY				ignore

}

broadcastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextSetupRequest

SUCCESSFUL OUTCOME		BroadcastContextSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastContextSetupFailure

PROCEDURE CODE			id-BroadcastContextSetup

CRITICALITY				reject

}

broadcastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseCommand

SUCCESSFUL OUTCOME		BroadcastContextReleaseComplete

PROCEDURE CODE			id-BroadcastContextRelease

CRITICALITY				reject

}

broadcastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseRequest

PROCEDURE CODE			id-BroadcastContextReleaseRequest

CRITICALITY				reject

}

broadcastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextModificationRequest

SUCCESSFUL OUTCOME		BroadcastContextModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastContextModificationFailure

PROCEDURE CODE			id-BroadcastContextModification

CRITICALITY				reject

}

multicastGroupPaging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastGroupPaging

PROCEDURE CODE			id-MulticastGroupPaging

CRITICALITY				ignore

}

multicastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextSetupRequest

SUCCESSFUL OUTCOME		MulticastContextSetupResponse

UNSUCCESSFUL OUTCOME	MulticastContextSetupFailure

PROCEDURE CODE			id-MulticastContextSetup

CRITICALITY				reject

}

multicastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseCommand

SUCCESSFUL OUTCOME		MulticastContextReleaseComplete

PROCEDURE CODE			id-MulticastContextRelease

CRITICALITY				reject

}

multicastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseRequest

PROCEDURE CODE			id-MulticastContextReleaseRequest

CRITICALITY				reject

}

multicastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextModificationRequest

SUCCESSFUL OUTCOME		MulticastContextModificationResponse

UNSUCCESSFUL OUTCOME	MulticastContextModificationFailure

PROCEDURE CODE			id-MulticastContextModification

CRITICALITY				reject

}

multicastDistributionSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionSetupRequest

SUCCESSFUL OUTCOME		MulticastDistributionSetupResponse

UNSUCCESSFUL OUTCOME	MulticastDistributionSetupFailure

PROCEDURE CODE			id-MulticastDistributionSetup

CRITICALITY				reject

}

multicastDistributionRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionReleaseCommand

SUCCESSFUL OUTCOME		MulticastDistributionReleaseComplete

PROCEDURE CODE			id-MulticastDistributionRelease

CRITICALITY				reject

}

pDCMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementInitiationRequest

SUCCESSFUL OUTCOME		PDCMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	PDCMeasurementInitiationFailure

PROCEDURE CODE			id-PDCMeasurementInitiation

CRITICALITY				reject

}

pDCMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementReport

PROCEDURE CODE			id-PDCMeasurementReport

CRITICALITY				ignore

}

pDCMeasurementTerminationCommand F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementTerminationCommand

PROCEDURE CODE			id-PDCMeasurementTerminationCommand

CRITICALITY				ignore

}

pDCMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementFailureIndication

PROCEDURE CODE			id-PDCMeasurementFailureIndication

CRITICALITY				ignore

}

pRSConfigurationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PRSConfigurationRequest

SUCCESSFUL OUTCOME		PRSConfigurationResponse

UNSUCCESSFUL OUTCOME	PRSConfigurationFailure

PROCEDURE CODE			id-pRSConfigurationExchange

CRITICALITY				reject

}

measurementPreconfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementPreconfigurationRequired

SUCCESSFUL OUTCOME		MeasurementPreconfigurationConfirm

UNSUCCESSFUL OUTCOME	MeasurementPreconfigurationRefuse

PROCEDURE CODE			id-measurementPreconfiguration

CRITICALITY				reject

}

measurementActivation		F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivation

PROCEDURE CODE			id-measurementActivation

CRITICALITY				ignore

}

qoEInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransfer

PROCEDURE CODE			id-QoEInformationTransfer

CRITICALITY				ignore

}

posSystemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PosSystemInformationDeliveryCommand

PROCEDURE CODE			id-PosSystemInformationDeliveryCommand

CRITICALITY				ignore

}

dUCUCellSwitchNotification 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUCellSwitchNotification

PROCEDURE CODE			id-DUCUCellSwitchNotification

CRITICALITY				ignore

}

cUDUCellSwitchNotification 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUCellSwitchNotification

PROCEDURE CODE			id-CUDUCellSwitchNotification

CRITICALITY				ignore

}

dUCUTAInformationTransfer 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUTAInformationTransfer

PROCEDURE CODE			id-DUCUTAInformationTransfer

CRITICALITY				ignore

}

cUDUTAInformationTransfer 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUTAInformationTransfer

PROCEDURE CODE			id-CUDUTAInformationTransfer

CRITICALITY				ignore

}

qoEInformationTransferControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransferControl

PROCEDURE CODE			id-QoEInformationTransferControl

CRITICALITY				ignore

}

rachIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RachIndication

PROCEDURE CODE			id-RachIndication

CRITICALITY				ignore

}

timingSynchronisationStatus F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusRequest

SUCCESSFUL OUTCOME		TimingSynchronisationStatusResponse

UNSUCCESSFUL OUTCOME	TimingSynchronisationStatusFailure

PROCEDURE CODE			id-TimingSynchronisationStatus

CRITICALITY				reject

}

timingSynchronisationStatusReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusReport

PROCEDURE CODE			id-TimingSynchronisationStatusReport

CRITICALITY				ignore

}

mIABF1SetupTriggering F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MIABF1SetupTriggering

PROCEDURE CODE			id-MIABF1SetupTriggering

CRITICALITY				ignore

}

mIABF1SetupOutcomeNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MIABF1SetupOutcomeNotification

PROCEDURE CODE			id-MIABF1SetupOutcomeNotification

CRITICALITY				ignore

}

multicastContextNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextNotificationIndication

SUCCESSFUL OUTCOME		MulticastContextNotificationConfirm

UNSUCCESSFUL OUTCOME	MulticastContextNotificationRefuse

PROCEDURE CODE			id-MulticastContextNotification

CRITICALITY				reject

}

multicastCommonConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastCommonConfigurationRequest

SUCCESSFUL OUTCOME		MulticastCommonConfigurationResponse

UNSUCCESSFUL OUTCOME	MulticastCommonConfigurationRefuse

PROCEDURE CODE			id-MulticastCommonConfiguration

CRITICALITY				reject

}

broadcastTransportResourceRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastTransportResourceRequest

PROCEDURE CODE			id-BroadcastTransportResourceRequest

CRITICALITY				reject

}

dUCUAccessAndMobilityIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUAccessAndMobilityIndication

PROCEDURE CODE			id-DUCUAccessAndMobilityIndication

CRITICALITY				ignore

}

sRSInformationReservationNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SRSInformationReservationNotification

PROCEDURE CODE			id-SRSInformationReservationNotification

CRITICALITY				reject

}

dUCUDynamicResourceAllocation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUDynamicResourceAllocationRequest

SUCCESSFUL OUTCOME		DUCUDynamicResourceAllocationResponse

PROCEDURE CODE			id-DUCUDynamicResourceAllocation

CRITICALITY				reject

}

cUDUDynamicResourceAllocation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUDynamicResourceAllocationRequest

SUCCESSFUL OUTCOME		CUDUDynamicResourceAllocationResponse

PROCEDURE CODE			id-CUDUDynamicResourceAllocation

CRITICALITY				reject

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Allowed-to-be-Deactivated-List-Item,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

EUTRANQoS,

ExecuteDuplication,

FullConfiguration,

GNB-CU-MBS-F1AP-ID,

GNB-CU-UE-F1AP-ID,

GNB-DU-MBS-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

LowerLayerPresenceStatusChange,

MBS-Area-Session-ID,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

MulticastMRBs-Setup-Item,

MulticastMRBs-FailedToBeSetup-Item,

MulticastMRBs-ToBeSetupMod-Item,

MulticastMRBs-ToBeModified-Item,

MulticastMRBs-ToBeReleased-Item,

MulticastMRBs-SetupMod-Item,

MulticastMRBs-FailedToBeSetupMod-Item,

MulticastMRBs-Modified-Item,

MulticastMRBs-FailedToBeModified-Item,

BroadcastAreaScope,

NetworkControlledRepeaterAuthorized,

NotificationControl,

NRCGI,

NRPCI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

RequestedSRSTransmissionCharacteristics,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCContainer-RRCSetupComplete,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

SDT-Volume-Threshold,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SNSSAI,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

SupportedUETypeList,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

UEIdentity-List-For-Paging-Item,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

NotificationInformation,

TraceActivation,

TraceID,

Neighbour-Cell-Information-Item,

SymbolAllocInSlot,

NumDLULSymbols,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Address-Info,

BHChannels-ToBeSetup-Item,

BHChannels-Setup-Item,

BHChannels-FailedToBeSetup-Item,

BHChannels-ToBeModified-Item,

BHChannels-ToBeReleased-Item,

BHChannels-ToBeSetupMod-Item,

BHChannels-FailedToBeModified-Item,

BHChannels-FailedToBeSetupMod-Item,

BHChannels-Modified-Item,

BHChannels-SetupMod-Item,

BHChannels-Required-ToBeReleased-Item,

BAPAddress,

BAPPathID,

BAPRoutingID,

BH-Routing-Information-Added-List-Item,

BH-Routing-Information-Removed-List-Item,

Child-Nodes-List,

Child-Nodes-List-Item,

Child-Node-Cells-List,

Child-Node-Cells-List-Item,

Activated-Cells-to-be-Updated-List,

Activated-Cells-to-be-Updated-List-Item,

UL-BH-Non-UP-Traffic-Mapping,

IABTNLAddressesRequested,

IABIPv6RequestType,

IAB-TNL-Addresses-To-Remove-Item,

IABTNLAddress,

IAB-Allocated-TNL-Address-Item,

IABv4AddressesRequested,

TrafficMappingInfo,

UL-UP-TNL-Information-to-Update-List-Item,

UL-UP-TNL-Address-to-Update-List-Item,

DL-UP-TNL-Address-to-Update-List-Item,

NRV2XServicesAuthorized,

LTEV2XServicesAuthorized,

NRUESidelinkAggregateMaximumBitrate,

LTEUESidelinkAggregateMaximumBitrate,

SLDRBs-SetupMod-Item,

SLDRBs-ModifiedConf-Item,

SLDRBID,

SLDRBs-FailedToBeModified-Item,

SLDRBs-FailedToBeSetup-Item,

SLDRBs-FailedToBeSetupMod-Item,

SLDRBs-Modified-Item,

SLDRBs-Required-ToBeModified-Item,

SLDRBs-Required-ToBeReleased-Item,

SLDRBs-Setup-Item,

SLDRBs-ToBeModified-Item,

SLDRBs-ToBeReleased-Item,

SLDRBs-ToBeSetup-Item,

SLDRBs-ToBeSetupMod-Item,

GNBCUMeasurementID,

GNBDUMeasurementID,

RegistrationRequest,

ReportCharacteristics,

CellToReportList,

HardwareLoadIndicator,

CellMeasurementResultList,

ReportingPeriodicity,

TNLCapacityIndicator,

RAReportList,

RLFReportInformationList,

ReportingRequestType,

TimeReferenceInformation,

ConditionalInterDUMobilityInformation,

ConditionalIntraDUMobilityInformation,

TargetCellList,

MDTPLMNList,

PrivacyIndicator,

TransportLayerAddress,

URI-address,

NID,

PosAssistance-Information,

PosBroadcast,

PositioningBroadcastCells,

RoutingID,

PosAssistanceInformationFailureList,

PosMeasurementQuantities,

PosMeasurementResultList,

PosReportCharacteristics,

TRPInformationTypeItem,

TRPInformationItem,

LMF-MeasurementID,

RAN-MeasurementID,

SDT-Termination-Request,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

SRSConfiguration,

TRPList,

E-CID-MeasurementQuantities,

MeasurementPeriodicity,

E-CID-MeasurementResult,

Cell-Portion-ID,

LMF-UE-MeasurementID,

RAN-UE-MeasurementID,

RelativeTime1900,

SystemFrameNumber,

SlotNumber,

AbortTransmission,

TRP-MeasurementRequestList,

MeasurementBeamInfoRequest,

E-CID-ReportCharacteristics,

Extended-GNB-CU-Name,

Extended-GNB-DU-Name,

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

SuccessfulHOReportInformationList,

Coverage-Modification-Notification,

CCO-Assistance-Information,

CellsForSON-List,

IABCongestionIndication,

IABConditionalRRCMessageDeliveryIndication,

F1CTransferPathNRDC,

BufferSizeThresh,

IAB-TNL-Addresses-Exception,

BAP-Header-Rewriting-Added-List-Item,

Re-routingEnableIndicator,

NonF1terminatingTopologyIndicator,

EgressNonF1terminatingTopologyIndicator,

IngressNonF1terminatingTopologyIndicator,

Neighbour-Node-Cells-List,

Neighbour-Node-Cells-List-Item,

NA-Resource-Configuration-List,

NA-Resource-Configuration-Item,

Serving-Cells-List,

Serving-Cells-List-Item,

RBSetConfiguration,

PDCMeasurementPeriodicity,

PDCMeasurementQuantities,

PDCMeasurementResult,

PDCReportType,

RAN-UE-PDC-MeasID,

SCGActivationRequest,

SCGActivationStatus,

TRP-MeasurementUpdateList,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

TRP-PRS-Info-List,

PRS-Measurement-Info-List,

PRSConfigRequestType,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

UEReportingInformation,

PosConextRevIndication,

NRRedCapUEIndication,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

QoEInformation,

CG-SDTQueryIndication,

CG-SDTKeptIndicator,

CG-SDTSessionInfo,

SDTInformation,

FiveG-ProSeAuthorized,

UuRLCChannelToBeSetupList,

UuRLCChannelToBeModifiedList,

UuRLCChannelToBeReleasedList,

UuRLCChannelSetupList,

UuRLCChannelFailedToBeSetupList,

UuRLCChannelModifiedList,

UuRLCChannelFailedToBeModifiedList,

UuRLCChannelRequiredToBeModifiedList,

UuRLCChannelRequiredToBeReleasedList,

PC5RLCChannelToBeSetupList,

PC5RLCChannelToBeModifiedList,

PC5RLCChannelToBeReleasedList,

PC5RLCChannelSetupList,

PC5RLCChannelFailedToBeSetupList,

PC5RLCChannelFailedToBeModifiedList,

PC5RLCChannelRequiredToBeModifiedList,

PC5RLCChannelRequiredToBeReleasedList,

PC5RLCChannelModifiedList,

RemoteUELocalID,

PathSwitchConfiguration,

SidelinkRelayConfiguration,

PagingCause,

PEIPSAssistanceInfo,

UEPagingCapability,

GNBDUUESliceMaximumBitRateList,

MDTPollutedMeasurementIndicator,

UE-MulticastMRBs-ConfirmedToBeModified-Item,

UE-MulticastMRBs-RequiredToBeModified-Item,

UE-MulticastMRBs-RequiredToBeReleased-Item,

UE-MulticastMRBs-Setup-Item,

UE-MulticastMRBs-Setupnew-Item,

UE-MulticastMRBs-ToBeReleased-Item,

UE-MulticastMRBs-ToBeSetup-Item,

UE-MulticastMRBs-ToBeSetup-atModify-Item,

PosMeasurementAmount,

BAP-Header-Rewriting-Removed-List-Item,

SLDRXCycleList,

MDTPLMNModificationList,

ActivationRequestType,

PosMeasGapPreConfigList,

PosMeasurementPeriodicityNR-AoA,

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

ServingCellMO-List-Item,

ServingCellMO-encoded-in-CGC-List,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

DedicatedSIDeliveryIndication,

Configured-BWP-List,

MT-SDT-Information,

LTMInformation-Setup,

LTMConfigurationIDMappingList,

LTMInformation-Modify,

LTMCells-ToBeReleased-List,

LTMConfiguration,

EarlySyncInformation-Request,

EarlySyncInformation,

EarlySyncInformation-List,

LTMCellSwitchInformation,

DUtoCUTAInformation-List,

CUtoDUTAInformation-List,

DeactivationIndication,

RAReportIndicationList,

SuccessfulPSCellChangeReportInformationList,

PathAdditionInformation,

RANTSSRequestType,

RANTimingSynchronisationStatusInfo,

GlobalGNB-ID,

Activated-Cells-Mapping-List-Item,

RRC-Terminating-IAB-Donor-Related-Info,

NCGI-to-be-Updated-List-Item,

Mobile-IAB-MTUserLocationInformation,

TAI,

IndicationMCInactiveReception,

MulticastCU2DURRCInfo,

MulticastDU2CURRCInfo,

MBSMulticastSessionReceptionState,

MulticastCU2DUCommonRRCInfo,

NRA2XServicesAuthorized,

LTEA2XServicesAuthorized,

NReRedCapUEIndication,

NRPaginglongeDRXInformationforRRCINACTIVE,

Cells-With-SSBs-Activated-List,

Recommended-SSBs-for-Paging-List,

S-CPAC-Configuration,

DLLBTFailureInformationRequest,	DLLBTFailureInformationList,

SLPositioning-Ranging-Service-Info,

TimeWindowInformation-SRS-List,

TimeWindowInformation-Measurement-List,

SRSPosRRCInactiveValidityAreaConfig,

PosValidityAreaCellList,

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

DynamicResourceAllocationRequest,

DynamicResourceAllocationResponse

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

id-BroadcastMRBs-SetupMod-Item,

id-BroadcastMRBs-ToBeModified-List,

id-BroadcastMRBs-ToBeModified-Item,

id-BroadcastMRBs-ToBeReleased-List,

id-BroadcastMRBs-ToBeReleased-Item,

id-BroadcastMRBs-ToBeSetup-List,

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-Cells-Allowed-to-be-Deactivated-List,

id-Cells-Allowed-to-be-Deactivated-List-Item,

id-Cells-With-SSBs-Activated-List,

id-Recommended-SSBs-for-Paging-List,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-MBS-F1AP-ID,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-MBS-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-Extended-GNB-CU-Name,

id-Extended-GNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-MBS-Area-Session-ID,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

id-MulticastMRBs-FailedToBeSetupMod-Item,

id-MulticastMRBs-Modified-List,

id-MulticastMRBs-Modified-Item,

id-MulticastMRBs-Setup-List,

id-MulticastMRBs-Setup-Item,

id-MulticastMRBs-SetupMod-List,

id-MulticastMRBs-SetupMod-Item,

id-MulticastMRBs-ToBeModified-List,

id-MulticastMRBs-ToBeModified-Item,

id-MulticastMRBs-ToBeReleased-List,

id-MulticastMRBs-ToBeReleased-Item,

id-MulticastMRBs-ToBeSetup-List,

id-MulticastMRBs-ToBeSetup-Item,

id-MulticastMRBs-ToBeSetupMod-List,

id-MulticastMRBs-ToBeSetupMod-Item,

id-MulticastF1UContext-ToBeSetup-List,

id-MulticastF1UContext-ToBeSetup-Item,

id-MulticastF1UContext-Setup-List,

id-MulticastF1UContext-Setup-Item,

id-MulticastF1UContext-FailedToBeSetup-List,

id-MulticastF1UContext-FailedToBeSetup-Item,

id-BroadcastAreaScope,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-RequestedSRSTransmissionCharacteristics,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCContainer-RRCSetupComplete,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SDT-Termination-Request,

id-SDT-Volume-Threshold,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SNSSAI,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-SupportedUETypeList,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-UEIdentity-List-For-Paging-List,

id-UEIdentity-List-For-Paging-Item,

id-UE-MulticastMRBs-ConfirmedToBeModified-List,

id-UE-MulticastMRBs-ConfirmedToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeModified-List,

id-UE-MulticastMRBs-RequiredToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeReleased-List,

id-UE-MulticastMRBs-RequiredToBeReleased-Item,

id-UE-MulticastMRBs-Setup-List,

id-UE-MulticastMRBs-Setup-Item,

id-UE-MulticastMRBs-Setupnew-List,

id-UE-MulticastMRBs-Setupnew-Item,

id-UE-MulticastMRBs-ToBeReleased-List,

id-UE-MulticastMRBs-ToBeReleased-Item,

id-UE-MulticastMRBs-ToBeSetup-atModify-List,

id-UE-MulticastMRBs-ToBeSetup-atModify-Item,

id-UE-MulticastMRBs-ToBeSetup-List,

id-UE-MulticastMRBs-ToBeSetup-Item,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-NotificationInformation,

id-TraceActivation,

id-TraceID,

id-Neighbour-Cell-Information-List,

id-Neighbour-Cell-Information-Item,

id-SymbolAllocInSlot,

id-NumDLULSymbols,

id-AdditionalRRMPriorityIndex,

id-DUCURadioInformationType,

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Address-Info,

id-BHChannels-ToBeSetup-List,

id-BHChannels-ToBeSetup-Item,

id-BHChannels-Setup-List,

id-BHChannels-Setup-Item,

id-BHChannels-ToBeModified-Item,

id-BHChannels-ToBeModified-List,

id-BHChannels-ToBeReleased-Item,

id-BHChannels-ToBeReleased-List,

id-BHChannels-ToBeSetupMod-Item,

id-BHChannels-ToBeSetupMod-List,

id-BHChannels-FailedToBeSetup-Item,

id-BHChannels-FailedToBeSetup-List,

id-BHChannels-FailedToBeModified-Item,

id-BHChannels-FailedToBeModified-List,

id-BHChannels-FailedToBeSetupMod-Item,

id-BHChannels-FailedToBeSetupMod-List,

id-BHChannels-Modified-Item,

id-BHChannels-Modified-List,

id-BHChannels-SetupMod-Item,

id-BHChannels-SetupMod-List,

id-BHChannels-Required-ToBeReleased-Item,

id-BHChannels-Required-ToBeReleased-List,

id-BAPAddress,

id-ConfiguredBAPAddress,

id-BH-Routing-Information-Added-List,

id-BH-Routing-Information-Added-List-Item,

id-BH-Routing-Information-Removed-List,

id-BH-Routing-Information-Removed-List-Item,

id-UL-BH-Non-UP-Traffic-Mapping,

id-Child-Nodes-List,

id-Activated-Cells-to-be-Updated-List,

id-IABIPv6RequestType,

id-IAB-TNL-Addresses-To-Remove-List,

id-IAB-TNL-Addresses-To-Remove-Item,

id-IAB-Allocated-TNL-Address-List,

id-IAB-Allocated-TNL-Address-Item,

id-IABv4AddressesRequested,

id-TrafficMappingInformation,

id-UL-UP-TNL-Information-to-Update-List,

id-UL-UP-TNL-Information-to-Update-List-Item,

id-UL-UP-TNL-Address-to-Update-List,

id-UL-UP-TNL-Address-to-Update-List-Item,

id-DL-UP-TNL-Address-to-Update-List,

id-DL-UP-TNL-Address-to-Update-List-Item,

id-NRV2XServicesAuthorized,

id-LTEV2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrate,

id-LTEUESidelinkAggregateMaximumBitrate,

id-PC5LinkAMBR,

id-SLDRBs-FailedToBeModified-Item,

id-SLDRBs-FailedToBeModified-List,

id-SLDRBs-FailedToBeSetup-Item,

id-SLDRBs-FailedToBeSetup-List,

id-SLDRBs-Modified-Item,

id-SLDRBs-Modified-List,

id-SLDRBs-Required-ToBeModified-Item,

id-SLDRBs-Required-ToBeModified-List,

id-SLDRBs-Required-ToBeReleased-Item,

id-SLDRBs-Required-ToBeReleased-List,

id-SLDRBs-Setup-Item,

id-SLDRBs-Setup-List,

id-SLDRBs-ToBeModified-Item,

id-SLDRBs-ToBeModified-List,

id-SLDRBs-ToBeReleased-Item,

id-SLDRBs-ToBeReleased-List,

id-SLDRBs-ToBeSetup-Item,

id-SLDRBs-ToBeSetup-List,

id-SLDRBs-ToBeSetupMod-Item,

id-SLDRBs-ToBeSetupMod-List,

id-SLDRBs-SetupMod-List,

id-SLDRBs-FailedToBeSetupMod-List,

id-SLDRBs-SetupMod-Item,

id-SLDRBs-FailedToBeSetupMod-Item,

id-SLDRBs-ModifiedConf-List,

id-SLDRBs-ModifiedConf-Item,

id-gNBCUMeasurementID,

id-gNBDUMeasurementID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReportList,

id-CellMeasurementResultList,

id-HardwareLoadIndicator,

id-ReportingPeriodicity,

id-TNLCapacityIndicator,

id-RAReportList,

id-RLFReportInformationList,

id-ReportingRequestType,

id-TimeReferenceInformation,

id-ConditionalInterDUMobilityInformation,

id-ConditionalIntraDUMobilityInformation,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-TraceCollectionEntityIPAddress,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityURI,

id-ServingNID,

id-PosAssistance-Information,

id-PosBroadcast,

id-PositioningBroadcastCells,

id-RoutingID,

id-PosAssistanceInformationFailureList,

id-PosMeasurementQuantities,

id-PosMeasurementResultList,

id-PosMeasurementPeriodicity,

id-PosReportCharacteristics,

id-TRPInformationTypeListTRPReq,

id-TRPInformationTypeItem,

id-TRPInformationListTRPResp,

id-TRPInformationItem,

id-LMF-MeasurementID,

id-RAN-MeasurementID,

id-SRSType,

id-ActivationTime,

id-AbortTransmission,

id-SRSConfiguration,

id-TRPList,

id-E-CID-MeasurementQuantities,

id-E-CID-MeasurementPeriodicity,

id-E-CID-MeasurementResult,

id-Cell-Portion-ID,

id-LMF-UE-MeasurementID,

id-RAN-UE-MeasurementID,

id-SFNInitialisationTime,

id-SystemFrameNumber,

id-SlotNumber,

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-PosMeasurementPeriodicityExtended,

id-SuccessfulHOReportInformationList,

id-Coverage-Modification-Notification,

id-CCO-Assistance-Information,

id-CellsForSON-List,

id-IABCongestionIndication,

id-IABConditionalRRCMessageDeliveryIndication,

id-F1CTransferPathNRDC,

id-BufferSizeThresh,

id-IAB-TNL-Addresses-Exception,

id-BAP-Header-Rewriting-Added-List,

id-BAP-Header-Rewriting-Added-List-Item,

id-Re-routingEnableIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-IngressNonF1terminatingTopologyIndicator,

id-Neighbour-Node-Cells-List,

id-Serving-Cells-List,

id-MDTPollutedMeasurementIndicator,

id-PDCMeasurementPeriodicity,

id-PDCMeasurementQuantities,

id-PDCMeasurementResult,

id-PDCReportType,

id-RAN-UE-PDC-MeasID,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-TRP-MeasurementUpdateList,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-TRP-PRS-Info-List,

id-PRS-Measurement-Info-List,

id-PRSConfigRequestType,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-UEReportingInformation,

id-PosConextRevIndication,

id-NRRedCapUEIndication,

id-RANUEPagingDRX,

id-CNUEPagingDRX,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-QoEInformation,

id-CG-SDTQueryIndication,

id-CG-SDTKeptIndicator,

id-CG-SDTSessionInfoOld,

id-SDTInformation,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5LinkAMBR,

id-FiveG-ProSeUEPC5AggregateMaximumBitrate,

id-UuRLCChannelToBeSetupList,

id-UuRLCChannelToBeModifiedList,

id-UuRLCChannelToBeReleasedList,

id-UuRLCChannelSetupList,

id-UuRLCChannelFailedToBeSetupList,

id-UuRLCChannelModifiedList,

id-UuRLCChannelFailedToBeModifiedList,

id-UuRLCChannelRequiredToBeModifiedList,

id-UuRLCChannelRequiredToBeReleasedList,

id-PC5RLCChannelToBeSetupList,

id-PC5RLCChannelToBeModifiedList,

id-PC5RLCChannelToBeReleasedList,

id-PC5RLCChannelSetupList,

id-PC5RLCChannelFailedToBeSetupList,

id-PC5RLCChannelModifiedList,

id-PC5RLCChannelFailedToBeModifiedList,

id-PC5RLCChannelRequiredToBeModifiedList,

id-PC5RLCChannelRequiredToBeReleasedList,

id-SidelinkRelayConfiguration,

id-UpdatedRemoteUELocalID,

id-PathSwitchConfiguration,

id-PagingCause,

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-PosMeasurementAmount,

id-BAP-Header-Rewriting-Removed-List,

id-BAP-Header-Rewriting-Removed-List-Item,

id-SLDRXCycleList,

id-ManagementBasedMDTPLMNModificationList,

id-ActivationRequestType,

id-PosMeasGapPreConfigList,

id-PosMeasurementPeriodicityNR-AoA,

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-ServingCellMO-List,

id-ServingCellMO-List-Item,

id-ServingCellMO-encoded-in-CGC-List,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-DedicatedSIDeliveryIndication,

id-Configured-BWP-List,

id-NetworkControlledRepeaterAuthorized,

id-MT-SDT-Information,

id-LTMInformation-Setup,

id-LTMConfigurationIDMappingList,

id-LTMInformation-Modify,

id-LTMCells-ToBeReleased-List,

id-LTMConfiguration,

id-EarlySyncInformation-Request,

id-EarlySyncInformation,

id-EarlySyncInformation-List,

id-LTMCellSwitchInformation,

id-DUtoCUTAInformation-List,

id-CUtoDUTAInformation-List,

id-Source-gNB-DU-ID,

id-DeactivationIndication,

id-RAReportIndicationList,

id-SuccessfulPSCellChangeReportInformationList,

id-PathAdditionInformation,

id-RANTSSRequestType,

id-RANTimingSynchronisationStatusInfo,

id-Target-gNB-ID,

id-Target-gNB-IP-address,

id-Target-SeGW-IP-address,

id-Activated-Cells-Mapping-List,

id-Activated-Cells-Mapping-List-Item,

id-F1SetupOutcome,

id-RRC-Terminating-IAB-Donor-Related-Info,

id-RRC-Terminating-IAB-Donor-gNB-ID,

id-NCGI-to-be-Updated-List,

id-NCGI-to-be-Updated-List-Item,

id-Mobile-IAB-MTUserLocationInformation,

id-IndicationMCInactiveReception,

id-MulticastCU2DURRCInfo,

id-MulticastDU2CURRCInfo,

id-MBSMulticastSessionReceptionState,

id-MulticastCU2DUCommonRRCInfo,

id-NRA2XServicesAuthorized,

id-LTEA2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrateForA2X,

id-LTEUESidelinkAggregateMaximumBitrateForA2X,

id-NReRedCapUEIndication,

id-NRPaginglongeDRXInformationforRRCINACTIVE,

id-Target-F1-Terminating-Donor-gNB-ID,

id-Broadcast-MRBs-Transport-Request-List,

id-Broadcast-MRBs-Transport-Request-Item,

id-S-CPAC-Configuration,

id-DLLBTFailureInformationRequest,

id-DLLBTFailureInformationList,

id-SLPositioning-Ranging-Service-Info,

id-TimeWindowInformation-SRS-List,

id-TimeWindowInformation-Measurement-List,

id-SRSPosRRCInactiveValidityAreaConfig,

id-PosValidityAreaCellList,

id-SRSReservationType,

id-RequestedSRSPreconfigurationCharacteristics-List,

id-SRSPreconfiguration-List,

id-SRSInformation,

id-ValidityAreaSpecificSRSInformation,

id-DynamicResourceAllocationRequest,

id-DynamicResourceAllocationResponse,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofChildIABNodes,

maxnoofServedCellsIAB,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofNeighbourNodeCellsIAB,

maxnoofMRBsforUE,

maxnoofServingCellMOs,

maxnoofLTMCells

FROM F1AP-Constants;

-- **************************************************************

--

#### -- RESET ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

f1-Interface					ResetAll,

partOfF1-Interface				UE-associatedLogicalF1-ConnectionListRes,

choice-extension				ProtocolIE-SingleContainer { { ResetType-ExtIEs} }

}

ResetType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalF1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualF1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalF1-ConnectionItemRes } }

UE-associatedLogicalF1-ConnectionItemRes F1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalF1-ConnectionItem	CRITICALITY reject	TYPE UE-associatedLogicalF1-ConnectionItem	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID													PRESENCE mandatory	}|

{ ID id-UE-associatedLogicalF1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalF1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalF1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualF1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalF1-ConnectionItemResAck } }

UE-associatedLogicalF1-ConnectionItemResAck 	F1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalF1-ConnectionItem	 CRITICALITY ignore 	TYPE UE-associatedLogicalF1-ConnectionItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Error Indication

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ErrorIndicationIEs}},

...

}

ErrorIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory}|

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-CU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-DU-ID						CRITICALITY reject	TYPE GNB-DU-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-Name						CRITICALITY ignore	TYPE GNB-DU-Name						PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE GNB-DU-Served-Cells-List			PRESENCE optional	}|

{ ID id-GNB-DU-RRC-Version				CRITICALITY reject	TYPE RRC-Version						PRESENCE mandatory	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info		PRESENCE optional	}|

{ ID id-BAPAddress						CRITICALITY ignore	TYPE BAPAddress							PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name			CRITICALITY ignore	TYPE Extended-GNB-DU-Name				PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-gNB-ID	CRITICALITY reject	TYPE GlobalGNB-ID					PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation	CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		PRESENCE optional	},

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- F1 Setup Response

--

-- **************************************************************

F1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupResponseIEs} },

...

}

F1SetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-Name						CRITICALITY ignore	TYPE GNB-CU-Name						PRESENCE optional	}|

{ ID id-Cells-to-be-Activated-List		CRITICALITY reject	TYPE Cells-to-be-Activated-List			PRESENCE optional	}|

{ ID id-GNB-CU-RRC-Version				CRITICALITY reject	TYPE RRC-Version						PRESENCE mandatory	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info		PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping	CRITICALITY reject	TYPE UL-BH-Non-UP-Traffic-Mapping		PRESENCE optional	}|

{ ID id-BAPAddress						CRITICALITY ignore	TYPE BAPAddress							PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name			CRITICALITY ignore	TYPE Extended-GNB-CU-Name				PRESENCE optional	}|

{ ID id-NCGI-to-be-Updated-List			CRITICALITY reject	TYPE NCGI-to-be-Updated-List			PRESENCE optional	},

...

}

Cells-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Activated-List-ItemIEs } }

Cells-to-be-Activated-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-Cells-to-be-Activated-List-Item				CRITICALITY reject	TYPE Cells-to-be-Activated-List-Item						PRESENCE mandatory},

...

}

NCGI-to-be-Updated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { NCGI-to-be-Updated-List-ItemIEs } }

NCGI-to-be-Updated-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-NCGI-to-be-Updated-List-Item				CRITICALITY reject	TYPE NCGI-to-be-Updated-List-Item						PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- F1 Setup Failure

--

-- **************************************************************

F1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupFailureIEs} },

...

}

F1SetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE

--

-- **************************************************************

GNBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateIEs} },

...

}

GNBDUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Served-Cells-To-Add-List				CRITICALITY reject	TYPE Served-Cells-To-Add-List								PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List				CRITICALITY reject	TYPE Served-Cells-To-Modify-List							PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List				CRITICALITY reject	TYPE Served-Cells-To-Delete-List							PRESENCE optional	}|

{ ID id-Cells-Status-List						CRITICALITY reject	TYPE Cells-Status-List											PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List					PRESENCE optional	}|

{ ID id-gNB-DU-ID								CRITICALITY reject	TYPE GNB-DU-ID													PRESENCE optional	}|

{ ID id-GNB-DU-TNL-Association-To-Remove-List	CRITICALITY reject	TYPE GNB-DU-TNL-Association-To-Remove-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE Transport-Layer-Address-Info							PRESENCE optional	}|

{ ID id-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Coverage-Modification-Notification					PRESENCE optional	}|

{ ID id-gNB-DU-Name								CRITICALITY ignore	TYPE GNB-DU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name					CRITICALITY ignore	TYPE Extended-GNB-DU-Name										PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-Related-Info	CRITICALITY reject	TYPE RRC-Terminating-IAB-Donor-Related-Info	         PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation    CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		      PRESENCE optional	},

...

}

Served-Cells-To-Add-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Add-ItemIEs } }

Served-Cells-To-Modify-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Modify-ItemIEs } }

Served-Cells-To-Delete-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Delete-ItemIEs } }

Cells-Status-List	::= SEQUENCE (SIZE(0.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Status-ItemIEs } }

Dedicated-SIDelivery-NeededUE-List::= SEQUENCE (SIZE(1.. maxnoofUEIDs))	OF ProtocolIE-SingleContainer { { Dedicated-SIDelivery-NeededUE-ItemIEs } }

GNB-DU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-DU-TNL-Association-To-Remove-ItemIEs } }

Served-Cells-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Add-Item		CRITICALITY reject	TYPE	Served-Cells-To-Add-Item				PRESENCE mandatory	},

...

}

Served-Cells-To-Modify-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Modify-Item			CRITICALITY reject	TYPE		Served-Cells-To-Modify-Item							PRESENCE mandatory	},

...

}

Served-Cells-To-Delete-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Delete-Item				CRITICALITY reject	TYPE		Served-Cells-To-Delete-Item					PRESENCE mandatory	},

...

}

Cells-Status-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Status-Item				CRITICALITY reject	TYPE		Cells-Status-Item					PRESENCE mandatory	},

...

}

Dedicated-SIDelivery-NeededUE-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Dedicated-SIDelivery-NeededUE-Item		CRITICALITY ignore	TYPE	Dedicated-SIDelivery-NeededUE-Item				PRESENCE mandatory	},

...

}

GNB-DU-TNL-Association-To-Remove-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-DU-TNL-Association-To-Remove-Item		CRITICALITY reject	TYPE	 GNB-DU-TNL-Association-To-Remove-Item			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBDUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBDUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List			CRITICALITY reject	TYPE Cells-to-be-Activated-List				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List		CRITICALITY reject	TYPE Cells-to-be-Deactivated-List			PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info			PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping		CRITICALITY reject	TYPE UL-BH-Non-UP-Traffic-Mapping			PRESENCE optional	}|

{ ID id-BAPAddress							CRITICALITY ignore  TYPE BAPAddress									PRESENCE optional }|

{ ID id-CellsForSON-List					CRITICALITY ignore  TYPE CellsForSON-List							PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

GNBDUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateFailureIEs} },

...

}

GNBDUConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

GNB-CU-TNL-Association-To-Add-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Add-ItemIEs } }

GNB-CU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Remove-ItemIEs } }

GNB-CU-TNL-Association-To-Update-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Update-ItemIEs } }

Cells-to-be-Barred-List			::= SEQUENCE(SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-to-be-Barred-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Allowed-to-be-Deactivated-List-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Allowed-to-be-Deactivated-List-Item	CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List-Item	PRESENCE mandatory },

...

}

Cells-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Deactivated-List-Item						CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List-Item					PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Add-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Add-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Remove-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Remove-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Remove-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Update-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Update-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Update-Item			PRESENCE mandatory	},

...

}

Cells-to-be-Barred-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Barred-Item		CRITICALITY ignore	TYPE	 Cells-to-be-Barred-Item				PRESENCE mandatory	},

...

}

Protected-EUTRA-Resources-List ::= SEQUENCE (SIZE(1.. maxCellineNB))	OF ProtocolIE-SingleContainer { { Protected-EUTRA-Resources-ItemIEs } }

Protected-EUTRA-Resources-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Protected-EUTRA-Resources-Item 					CRITICALITY reject 	TYPE Protected-EUTRA-Resources-Item							PRESENCE mandatory},

...

}

Neighbour-Cell-Information-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Neighbour-Cell-Information-ItemIEs } }

Neighbour-Cell-Information-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Neighbour-Cell-Information-Item 					CRITICALITY ignore 	TYPE Neighbour-Cell-Information-Item							PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBCUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBCUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-Failed-to-be-Activated-List			CRITICALITY reject	TYPE Cells-Failed-to-be-Activated-List				PRESENCE optional}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-Setup-List			CRITICALITY ignore	TYPE GNB-CU-TNL-Association-Setup-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-Failed-To-Setup-List	CRITICALITY ignore	TYPE GNB-CU-TNL-Association-Failed-To-Setup-List	PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List				CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info				CRITICALITY ignore	TYPE Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-Cells-With-SSBs-Activated-List				CRITICALITY ignore	TYPE Cells-With-SSBs-Activated-List					PRESENCE optional	},

...

}

Cells-Failed-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Failed-to-be-Activated-List-ItemIEs } }

GNB-CU-TNL-Association-Setup-List ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-Setup-ItemIEs } }

GNB-CU-TNL-Association-Failed-To-Setup-List ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-Failed-To-Setup-ItemIEs } }

Cells-Failed-to-be-Activated-List-ItemIEs F1AP-PROTOCOL-IES		::= {

{ ID id-Cells-Failed-to-be-Activated-List-Item		CRITICALITY reject	TYPE Cells-Failed-to-be-Activated-List-Item		PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-Setup-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-Setup-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-Setup-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-Failed-To-Setup-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-Failed-To-Setup-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-Failed-To-Setup-Item			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

GNBCUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateFailureIEs} },

...

}

GNBCUConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- GNB-DU RESOURCE COORDINATION REQUEST

--

-- **************************************************************

GNBDUResourceCoordinationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{GNBDUResourceCoordinationRequest-IEs}},

...

}

GNBDUResourceCoordinationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-RequestType										CRITICALITY reject	TYPE RequestType										PRESENCE mandatory	}|

{ ID id-EUTRA-NR-CellResourceCoordinationReq-Container	CRITICALITY reject	TYPE EUTRA-NR-CellResourceCoordinationReq-Container	PRESENCE mandatory}|

{ ID id-IgnoreResourceCoordinationContainer				CRITICALITY reject	TYPE IgnoreResourceCoordinationContainer		PRESENCE optional },

...

}

-- **************************************************************

--

##### -- GNB-DU RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

GNBDUResourceCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{GNBDUResourceCoordinationResponse-IEs}},

...

}

GNBDUResourceCoordinationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID										CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-EUTRA-NR-CellResourceCoordinationReqAck-Container	CRITICALITY reject	TYPE EUTRA-NR-CellResourceCoordinationReqAck-Container		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- UE Context Setup ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List								PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE conditional }|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-new-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID											PRESENCE optional	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-BHChannels-ToBeSetup-List				CRITICALITY reject	TYPE BHChannels-ToBeSetup-List						PRESENCE optional	}|

{ ID id-ConfiguredBAPAddress					CRITICALITY reject	TYPE BAPAddress											PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetup-List					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-List							PRESENCE optional	}|

{ ID id-ConditionalInterDUMobilityInformation	CRITICALITY reject	TYPE ConditionalInterDUMobilityInformation		PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE 		MDTPLMNList									PRESENCE optional }|

{ ID id-ServingNID								CRITICALITY reject	TYPE NID												PRESENCE optional }|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-F1CTransferPathNRDC						CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional }|

{ ID id-MDTPollutedMeasurementIndicator			CRITICALITY ignore	TYPE MDTPollutedMeasurementIndicator				PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore	TYPE SCGActivationRequest								PRESENCE optional }|

{ ID id-CG-SDTSessionInfoOld					CRITICALITY ignore	TYPE CG-SDTSessionInfo									PRESENCE optional }|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional }|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitrate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-FiveG-ProSePC5LinkAMBR					CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-UuRLCChannelToBeSetupList				CRITICALITY reject	TYPE UuRLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-PC5RLCChannelToBeSetupList				CRITICALITY reject	TYPE PC5RLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-PathSwitchConfiguration					CRITICALITY ignore	TYPE PathSwitchConfiguration				 			PRESENCE optional	}|

{ ID id-GNBDUUESliceMaximumBitRateList			CRITICALITY ignore  TYPE GNBDUUESliceMaximumBitRateList 				PRESENCE optional }|

{ ID id-MulticastMBSSessionSetupList			CRITICALITY reject	TYPE MulticastMBSSessionList							PRESENCE optional }|

{ ID id-UE-MulticastMRBs-ToBeSetup-List			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-List				PRESENCE optional	}|

{ ID id-ServingCellMO-List						CRITICALITY ignore	TYPE ServingCellMO-List									PRESENCE optional	}|

{ ID id-NetworkControlledRepeaterAuthorized		CRITICALITY ignore	TYPE NetworkControlledRepeaterAuthorized			PRESENCE optional	}|

{ ID id-SDT-Volume-Threshold						CRITICALITY ignore	TYPE SDT-Volume-Threshold								PRESENCE optional  }|

{ ID id-LTMInformation-Setup					CRITICALITY reject	TYPE LTMInformation-Setup								PRESENCE optional	}|

{ ID id-LTMConfigurationIDMappingList			CRITICALITY reject	TYPE LTMConfigurationIDMappingList					PRESENCE optional	}|

{ ID id-EarlySyncInformation-Request			CRITICALITY ignore	TYPE EarlySyncInformation-Request					PRESENCE optional	}|

{ ID id-PathAdditionInformation				CRITICALITY reject	TYPE PathAdditionInformation							PRESENCE optional}|

{ ID id-NRA2XServicesAuthorized					CRITICALITY ignore	TYPE NRA2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEA2XServicesAuthorized				CRITICALITY ignore	TYPE LTEA2XServicesAuthorized								PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrateForA2X		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrateForA2X	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-DLLBTFailureInformationRequest				CRITICALITY ignore	TYPE DLLBTFailureInformationRequest				PRESENCE optional }|

{ ID id-SLPositioning-Ranging-Service-Info		CRITICALITY ignore	TYPE SLPositioning-Ranging-Service-Info						PRESENCE optional	},

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF ProtocolIE-SingleContainer { { Candidate-SpCell-ItemIEs} }

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetup-ItemIEs} }

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetup-ItemIEs} }

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetup-ItemIEs} }

BHChannels-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetup-ItemIEs} }

SLDRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetup-ItemIEs} }

UE-MulticastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeSetup-ItemIEs} }

ServingCellMO-List ::= SEQUENCE (SIZE(1..maxnoofServingCellMOs)) OF ProtocolIE-SingleContainer { { ServingCellMO-List-ItemIEs} }

Candidate-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Candidate-SpCell-Item					CRITICALITY ignore	TYPE Candidate-SpCell-Item						PRESENCE mandatory	},

...

}

SCell-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetup-Item						CRITICALITY ignore	TYPE SCell-ToBeSetup-Item					PRESENCE mandatory	},

...

}

SRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetup-Item		CRITICALITY reject		TYPE SRBs-ToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetup-Item					CRITICALITY reject	TYPE DRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

BHChannels-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetup-Item					CRITICALITY reject	TYPE BHChannels-ToBeSetup-Item					PRESENCE mandatory},

...

}

SLDRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetup-Item					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeSetup-Item			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-Item			PRESENCE mandatory},

...

}

ServingCellMO-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ServingCellMO-List-Item			CRITICALITY reject	TYPE ServingCellMO-List-Item	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT SETUP RESPONSE

--

-- **************************************************************

UEContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupResponseIEs} },

...

}

UEContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-DUtoCURRCInformation					CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE mandatory }|

{ ID id-C-RNTI									CRITICALITY ignore	TYPE C-RNTI											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration								PRESENCE optional	}|

{ ID id-DRBs-Setup-List							CRITICALITY ignore	TYPE DRBs-Setup-List								PRESENCE optional	}|

{ ID id-SRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE SRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-DRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE DRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-SCell-FailedtoSetup-List				CRITICALITY ignore	TYPE SCell-FailedtoSetup-List					PRESENCE optional	}|

{ ID id-InactivityMonitoringResponse			CRITICALITY reject	TYPE InactivityMonitoringResponse				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-SRBs-Setup-List							CRITICALITY ignore	TYPE SRBs-Setup-List								PRESENCE optional	}|

{ ID id-BHChannels-Setup-List					CRITICALITY ignore	TYPE BHChannels-Setup-List						PRESENCE optional	}|

{ ID id-BHChannels-FailedToBeSetup-List			CRITICALITY ignore	TYPE BHChannels-FailedToBeSetup-List			PRESENCE optional	}|

{ ID id-SLDRBs-Setup-List						CRITICALITY ignore	TYPE SLDRBs-Setup-List								PRESENCE optional	}|

{ ID id-SLDRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetup-List				PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID				CRITICALITY reject	TYPE NRCGI											PRESENCE optional}|

{ ID id-SCGActivationStatus						CRITICALITY ignore	TYPE SCGActivationStatus							PRESENCE optional }|

{ ID id-UuRLCChannelSetupList					CRITICALITY ignore	TYPE UuRLCChannelSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelFailedToBeSetupList			CRITICALITY ignore	TYPE UuRLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-PC5RLCChannelSetupList					CRITICALITY ignore	TYPE PC5RLCChannelSetupList						PRESENCE optional}|

{ ID id-PC5RLCChannelFailedToBeSetupList		CRITICALITY ignore	TYPE PC5RLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-ServingCellMO-encoded-in-CGC-List		CRITICALITY ignore	TYPE ServingCellMO-encoded-in-CGC-List			PRESENCE optional}|

{ ID id-UE-MulticastMRBs-Setupnew-List			CRITICALITY reject	TYPE UE-MulticastMRBs-Setupnew-List				PRESENCE optional}|

{ ID id-DedicatedSIDeliveryIndication			CRITICALITY ignore	TYPE DedicatedSIDeliveryIndication				PRESENCE optional}|

{ ID id-Configured-BWP-List						CRITICALITY ignore	TYPE Configured-BWP-List								PRESENCE optional}|

{ ID id-EarlySyncInformation					CRITICALITY ignore	TYPE EarlySyncInformation							PRESENCE optional	}|

{ ID id-LTMConfiguration						CRITICALITY ignore	TYPE LTMConfiguration								PRESENCE optional	}|

{ ID id-S-CPAC-Configuration					CRITICALITY ignore	TYPE S-CPAC-Configuration							PRESENCE optional	},

...

}

DRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Setup-ItemIEs} }

SRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-FailedToBeSetup-ItemIEs} }

DRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-FailedToBeSetup-ItemIEs} }

SCell-FailedtoSetup-List ::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-FailedtoSetup-ItemIEs} }

SRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-Setup-ItemIEs} }

BHChannels-Setup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-Setup-ItemIEs} }

BHChannels-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-FailedToBeSetup-ItemIEs} }

DRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Setup-Item						CRITICALITY ignore	TYPE DRBs-Setup-Item						PRESENCE mandatory},

...

}

SRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-Setup-Item						CRITICALITY ignore	TYPE SRBs-Setup-Item						PRESENCE mandatory},

...

}

SRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-FailedToBeSetup-Item		CRITICALITY ignore		TYPE SRBs-FailedToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE DRBs-FailedToBeSetup-Item			PRESENCE mandatory},

...

}

SCell-FailedtoSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-FailedtoSetup-Item			CRITICALITY ignore	TYPE SCell-FailedtoSetup-Item			PRESENCE mandatory},

...

}

BHChannels-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-Setup-Item						CRITICALITY ignore	TYPE BHChannels-Setup-Item						PRESENCE mandatory},

...

}

BHChannels-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-FailedToBeSetup-Item						CRITICALITY ignore	TYPE BHChannels-FailedToBeSetup-Item		PRESENCE mandatory},

...

}

SLDRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Setup-ItemIEs} }

SLDRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-FailedToBeSetup-ItemIEs} }

SLDRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Setup-Item						CRITICALITY ignore	TYPE SLDRBs-Setup-Item						PRESENCE mandatory},

...

}

SLDRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetup-Item			PRESENCE mandatory},

...

}

UE-MulticastMRBs-Setupnew-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-Setupnew-ItemIEs } }

UE-MulticastMRBs-Setupnew-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-Setupnew-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-Setupnew-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT SETUP FAILURE

--

-- **************************************************************

UEContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupFailureIEs} },

...

}

UEContextSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	}|

{ ID id-Potential-SpCell-List		CRITICALITY ignore	TYPE Potential-SpCell-List		PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID	CRITICALITY reject	TYPE NRCGI						PRESENCE optional},

...

}

Potential-SpCell-List::= SEQUENCE (SIZE(0..maxnoofPotentialSpCells)) OF ProtocolIE-SingleContainer { { Potential-SpCell-ItemIEs} }

Potential-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Potential-SpCell-Item				CRITICALITY ignore	TYPE Potential-SpCell-Item					PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE Context Release Request ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Context Release Request

--

-- **************************************************************

UEContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ UEContextReleaseRequestIEs}},

...

}

UEContextReleaseRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-targetCellsToCancel				CRITICALITY reject	TYPE TargetCellList					PRESENCE optional		}|

{ ID id-LTMCells-ToBeReleased-List		CRITICALITY ignore	TYPE LTMCells-ToBeReleased-List		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- UE Context Release (gNB-CU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMMAND

--

-- **************************************************************

UEContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextReleaseCommandIEs} },

...

}

UEContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY ignore	TYPE RRCContainer					PRESENCE optional	}|

{ ID id-SRBID							CRITICALITY ignore	TYPE SRBID							PRESENCE conditional	}|

{ ID id-oldgNB-DU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID				PRESENCE optional	}|

{ ID id-ExecuteDuplication				CRITICALITY ignore	TYPE ExecuteDuplication				PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest		CRITICALITY ignore	TYPE RRCDeliveryStatusRequest		PRESENCE optional }|

{ ID id-targetCellsToCancel				CRITICALITY reject	TYPE TargetCellList					PRESENCE optional}|

{ ID id-PosConextRevIndication			CRITICALITY reject	TYPE PosConextRevIndication			PRESENCE optional}|

{ ID id-CG-SDTKeptIndicator				CRITICALITY ignore	TYPE CG-SDTKeptIndicator			PRESENCE optional}|

{ ID id-LTMCells-ToBeReleased-List		CRITICALITY ignore	TYPE LTMCells-ToBeReleased-List		PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMPLETE

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextReleaseCompleteIEs} },

...

}

UEContextReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	}|

{ ID id-Recommended-SSBs-for-Paging-List		CRITICALITY ignore	TYPE Recommended-SSBs-for-Paging-List		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	TYPE LowerLayerPresenceStatusChange				PRESENCE optional	}|

{ ID id-BHChannels-ToBeSetupMod-List			CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeModified-List			CRITICALITY reject	TYPE BHChannels-ToBeModified-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeReleased-List			CRITICALITY reject	TYPE BHChannels-ToBeReleased-List					PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetupMod-List				CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeModified-List				CRITICALITY reject	TYPE SLDRBs-ToBeModified-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeReleased-List				CRITICALITY reject	TYPE SLDRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-ConditionalIntraDUMobilityInformation	CRITICALITY reject	TYPE ConditionalIntraDUMobilityInformation		PRESENCE optional}|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore	TYPE SCGIndicator										PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentTwoCarrierListInfo		CRITICALITY ignore	TYPE UplinkTxDirectCurrentTwoCarrierListInfo	PRESENCE optional	}|

{ ID id-IABConditionalRRCMessageDeliveryIndication				CRITICALITY reject	TYPE IABConditionalRRCMessageDeliveryIndication							PRESENCE optional	}|

{ ID id-F1CTransferPathNRDC							CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional }|

{ ID id-MDTPollutedMeasurementIndicator				CRITICALITY ignore	TYPE MDTPollutedMeasurementIndicator				PRESENCE optional }|

{ ID id-SCGActivationRequest						CRITICALITY ignore	TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CG-SDTQueryIndication						CRITICALITY ignore	TYPE CG-SDTQueryIndication							PRESENCE optional	}|

{ ID id-FiveG-ProSeAuthorized						CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional }|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-FiveG-ProSePC5LinkAMBR						CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-UpdatedRemoteUELocalID						CRITICALITY ignore	TYPE RemoteUELocalID				 					PRESENCE optional	}|

{ ID id-UuRLCChannelToBeSetupList					CRITICALITY reject	TYPE UuRLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelToBeModifiedList				CRITICALITY reject	TYPE UuRLCChannelToBeModifiedList					PRESENCE optional}|

{ ID id-UuRLCChannelToBeReleasedList				CRITICALITY reject	TYPE UuRLCChannelToBeReleasedList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeSetupList					CRITICALITY reject	TYPE PC5RLCChannelToBeSetupList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeModifiedList				CRITICALITY reject	TYPE PC5RLCChannelToBeModifiedList				PRESENCE optional}|

{ ID id-PC5RLCChannelToBeReleasedList				CRITICALITY reject	TYPE PC5RLCChannelToBeReleasedList				PRESENCE optional}|

{ ID id-PathSwitchConfiguration						CRITICALITY ignore	TYPE PathSwitchConfiguration				 		PRESENCE optional	}|

{ ID id-GNBDUUESliceMaximumBitRateList				CRITICALITY ignore	TYPE GNBDUUESliceMaximumBitRateList				PRESENCE optional }|

{ ID id-MulticastMBSSessionSetupList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-MulticastMBSSessionRemoveList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-UE-MulticastMRBs-ToBeSetup-atModify-List	CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-atModify-List	PRESENCE optional	}|

{ ID id-UE-MulticastMRBs-ToBeReleased-List			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-List			PRESENCE optional	}|

{ ID id-SLDRXCycleList								CRITICALITY ignore  TYPE SLDRXCycleList 										PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList		CRITICALITY ignore	TYPE MDTPLMNModificationList						PRESENCE optional }|

{ ID id-SDTBearerConfigurationQueryIndication		CRITICALITY ignore	TYPE SDTBearerConfigurationQueryIndication		PRESENCE optional }|

{ ID id-DAPS-HO-Status								CRITICALITY ignore	TYPE DAPS-HO-Status										PRESENCE optional }|

{ ID id-ServingCellMO-List							CRITICALITY ignore	TYPE ServingCellMO-List								PRESENCE optional	}|

{ ID id-UlTxDirectCurrentMoreCarrierInformation	    CRITICALITY ignore	TYPE UlTxDirectCurrentMoreCarrierInformation	PRESENCE optional	}|

{ ID id-CPACMCGInformation							CRITICALITY ignore	TYPE CPACMCGInformation								PRESENCE optional }|

{ ID id-NetworkControlledRepeaterAuthorized			CRITICALITY ignore	TYPE NetworkControlledRepeaterAuthorized			PRESENCE optional	}|

{ ID id-SDT-Volume-Threshold							CRITICALITY ignore	TYPE SDT-Volume-Threshold								PRESENCE optional  }|

{ ID id-LTMInformation-Modify						CRITICALITY reject	TYPE LTMInformation-Modify							PRESENCE optional	}|

{ ID id-LTMConfigurationIDMappingList				CRITICALITY reject	TYPE LTMConfigurationIDMappingList				PRESENCE optional	}|

{ ID id-EarlySyncInformation-Request				CRITICALITY ignore	TYPE EarlySyncInformation-Request					PRESENCE optional	}|

{ ID id-EarlySyncInformation-List					CRITICALITY ignore	TYPE EarlySyncInformation-List						PRESENCE optional	}|

{ ID id-LTMCells-ToBeReleased-List					CRITICALITY ignore	TYPE LTMCells-ToBeReleased-List					PRESENCE optional	}|

{ ID id-PathAdditionInformation					CRITICALITY reject	TYPE PathAdditionInformation								PRESENCE optional}|

{ ID id-NRA2XServicesAuthorized						CRITICALITY ignore	TYPE NRA2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEA2XServicesAuthorized						CRITICALITY ignore	TYPE LTEA2XServicesAuthorized							PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrateForA2X		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrateForA2X	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-DLLBTFailureInformationRequest				CRITICALITY ignore	TYPE DLLBTFailureInformationRequest				PRESENCE optional }|

{ ID id-SLPositioning-Ranging-Service-Info		CRITICALITY ignore	TYPE SLPositioning-Ranging-Service-Info						PRESENCE optional	},

...

}

SCell-ToBeSetupMod-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetupMod-ItemIEs} }

SCell-ToBeRemoved-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeRemoved-ItemIEs} }

SRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetupMod-ItemIEs} }

BHChannels-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetupMod-ItemIEs} }

DRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeModified-ItemIEs} }

BHChannels-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeModified-ItemIEs} }

SRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeReleased-ItemIEs} }

DRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeReleased-ItemIEs} }

BHChannels-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeReleased-ItemIEs} }

UE-MulticastMRBs-ToBeSetup-atModify-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeSetup-atModify-ItemIEs} }

UE-MulticastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeReleased-ItemIEs} }

SCell-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetupMod-Item			CRITICALITY ignore	TYPE SCell-ToBeSetupMod-Item			PRESENCE mandatory	},

...

}

SCell-ToBeRemoved-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeRemoved-Item			CRITICALITY ignore	TYPE SCell-ToBeRemoved-Item			PRESENCE mandatory	},

...

}

SRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE DRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeModified-Item		CRITICALITY reject	TYPE DRBs-ToBeModified-Item			PRESENCE mandatory},

...

}

SRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeReleased-Item	CRITICALITY reject	TYPE SRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

DRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeReleased-Item		CRITICALITY reject	TYPE DRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetupMod-Item		CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeModified-Item		CRITICALITY reject	TYPE BHChannels-ToBeModified-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeReleased-Item		CRITICALITY reject	TYPE BHChannels-ToBeReleased-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetupMod-ItemIEs} }

SLDRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeModified-ItemIEs} }

SLDRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeReleased-ItemIEs} }

SLDRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeModified-Item		CRITICALITY reject	TYPE SLDRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeReleased-Item		CRITICALITY reject	TYPE SLDRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeSetup-atModify-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeSetup-atModify-Item	CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-atModify-Item			PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

UEContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationResponseIEs} },

...

}

UEContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-DUtoCURRCInformation					CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE optional}|

{ ID id-DRBs-SetupMod-List						CRITICALITY ignore	TYPE DRBs-SetupMod-List								PRESENCE optional}|

{ ID id-DRBs-Modified-List						CRITICALITY ignore	TYPE DRBs-Modified-List								PRESENCE optional}|

{ ID id-SRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE SRBs-FailedToBeSetupMod-List				PRESENCE optional	}|

{ ID id-DRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE DRBs-FailedToBeSetupMod-List				PRESENCE optional	}|

{ ID id-SCell-FailedtoSetupMod-List				CRITICALITY ignore	TYPE SCell-FailedtoSetupMod-List				PRESENCE optional	}|

{ ID id-DRBs-FailedToBeModified-List			CRITICALITY ignore	TYPE DRBs-FailedToBeModified-List				PRESENCE optional	}|

{ ID id-InactivityMonitoringResponse			CRITICALITY reject	TYPE InactivityMonitoringResponse				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-C-RNTI									CRITICALITY ignore	TYPE C-RNTI											PRESENCE optional	}|

{ ID id-Associated-SCell-List					CRITICALITY ignore  TYPE Associated-SCell-List						PRESENCE optional	}|

{ ID id-SRBs-SetupMod-List						CRITICALITY ignore	TYPE SRBs-SetupMod-List								PRESENCE optional	}|

{ ID id-SRBs-Modified-List						CRITICALITY ignore	TYPE SRBs-Modified-List								PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration								PRESENCE optional	}|

{ ID id-BHChannels-SetupMod-List				CRITICALITY ignore	TYPE BHChannels-SetupMod-List					PRESENCE optional}|

{ ID id-BHChannels-Modified-List				CRITICALITY ignore	TYPE BHChannels-Modified-List					PRESENCE optional}|

{ ID id-BHChannels-FailedToBeSetupMod-List		CRITICALITY ignore	TYPE BHChannels-FailedToBeSetupMod-List		PRESENCE optional	}|

{ ID id-BHChannels-FailedToBeModified-List		CRITICALITY ignore	TYPE BHChannels-FailedToBeModified-List		PRESENCE optional	}|

{ ID id-SLDRBs-SetupMod-List					CRITICALITY ignore	TYPE SLDRBs-SetupMod-List							PRESENCE optional	}|

{ ID id-SLDRBs-Modified-List					CRITICALITY ignore	TYPE SLDRBs-Modified-List							PRESENCE optional	}|

{ ID id-SLDRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetupMod-List			PRESENCE optional	}|

{ ID id-SLDRBs-FailedToBeModified-List			CRITICALITY ignore	TYPE SLDRBs-FailedToBeModified-List			PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID				CRITICALITY reject	TYPE NRCGI											PRESENCE optional}|

{ ID id-SCGActivationStatus						CRITICALITY ignore	TYPE SCGActivationStatus							PRESENCE optional }|

{ ID id-UuRLCChannelSetupList					CRITICALITY ignore	TYPE UuRLCChannelSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelFailedToBeSetupList			CRITICALITY ignore	TYPE UuRLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-UuRLCChannelModifiedList				CRITICALITY ignore	TYPE UuRLCChannelModifiedList					PRESENCE optional}|

{ ID id-UuRLCChannelFailedToBeModifiedList		CRITICALITY ignore	TYPE UuRLCChannelFailedToBeModifiedList		PRESENCE optional}|

{ ID id-PC5RLCChannelSetupList					CRITICALITY ignore	TYPE PC5RLCChannelSetupList						PRESENCE optional}|

{ ID id-PC5RLCChannelFailedToBeSetupList		CRITICALITY ignore	TYPE PC5RLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-PC5RLCChannelModifiedList				CRITICALITY ignore	TYPE PC5RLCChannelModifiedList					PRESENCE optional}|

{ ID id-PC5RLCChannelFailedToBeModifiedList		CRITICALITY ignore	TYPE PC5RLCChannelFailedToBeModifiedList		PRESENCE optional}|

{ ID id-SDTBearerConfigurationInfo				CRITICALITY ignore	TYPE SDTBearerConfigurationInfo					PRESENCE optional}|

{ ID id-UE-MulticastMRBs-Setup-List				CRITICALITY reject	TYPE UE-MulticastMRBs-Setup-List				PRESENCE optional}|

{ ID id-ServingCellMO-encoded-in-CGC-List		CRITICALITY ignore	TYPE ServingCellMO-encoded-in-CGC-List		PRESENCE optional}|

{ ID id-DedicatedSIDeliveryIndication			CRITICALITY ignore	TYPE DedicatedSIDeliveryIndication				PRESENCE optional}|

{ ID id-Configured-BWP-List						CRITICALITY ignore	TYPE Configured-BWP-List							PRESENCE optional}|

{ ID id-EarlySyncInformation					CRITICALITY ignore	TYPE EarlySyncInformation							PRESENCE optional}|

{ ID id-LTMConfiguration						CRITICALITY ignore	TYPE LTMConfiguration								PRESENCE optional}|

{ ID id-S-CPAC-Configuration					CRITICALITY ignore	TYPE S-CPAC-Configuration							PRESENCE optional	},

...

}

DRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-SetupMod-ItemIEs} }

DRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Modified-ItemIEs } }

SRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-SetupMod-ItemIEs} }

SRBs-Modified-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-Modified-ItemIEs } }

DRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-FailedToBeModified-ItemIEs} }

SRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-FailedToBeSetupMod-ItemIEs} }

DRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-FailedToBeSetupMod-ItemIEs} }

SCell-FailedtoSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-FailedtoSetupMod-ItemIEs} }

BHChannels-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-SetupMod-ItemIEs} }

BHChannels-Modified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-Modified-ItemIEs } }

BHChannels-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-FailedToBeModified-ItemIEs} }

BHChannels-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-FailedToBeSetupMod-ItemIEs} }

Associated-SCell-List ::= SEQUENCE (SIZE(1.. maxnoofSCells)) OF ProtocolIE-SingleContainer { { Associated-SCell-ItemIEs} }

DRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-SetupMod-Item		CRITICALITY ignore		TYPE DRBs-SetupMod-Item		PRESENCE mandatory},

...

}

DRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Modified-Item			CRITICALITY ignore	TYPE DRBs-Modified-Item		PRESENCE mandatory},

...

}

SRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-SetupMod-Item		CRITICALITY ignore		TYPE SRBs-SetupMod-Item		PRESENCE mandatory},

...

}

SRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-Modified-Item			CRITICALITY ignore	TYPE SRBs-Modified-Item		PRESENCE mandatory},

...

}

SRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE SRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE DRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-FailedToBeModified-Item		CRITICALITY ignore	TYPE DRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

SCell-FailedtoSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-FailedtoSetupMod-Item			CRITICALITY ignore	TYPE SCell-FailedtoSetupMod-Item			PRESENCE mandatory},

...

}

Associated-SCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Associated-SCell-Item			CRITICALITY ignore	TYPE Associated-SCell-Item			PRESENCE mandatory},

...

}

BHChannels-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-SetupMod-Item		CRITICALITY ignore		TYPE BHChannels-SetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-Modified-Item		CRITICALITY ignore	TYPE BHChannels-Modified-Item		PRESENCE mandatory},

...

}

BHChannels-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE BHChannels-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-FailedToBeModified-Item		CRITICALITY ignore	TYPE BHChannels-FailedToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-SetupMod-List 			::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-SetupMod-ItemIEs} }

SLDRBs-Modified-List				::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Modified-ItemIEs } }

SLDRBs-FailedToBeModified-List 	::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-FailedToBeModified-ItemIEs} }

SLDRBs-FailedToBeSetupMod-List 	::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-FailedToBeSetupMod-ItemIEs} }

SLDRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-SetupMod-Item		CRITICALITY ignore		TYPE SLDRBs-SetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Modified-Item			CRITICALITY ignore	TYPE SLDRBs-Modified-Item		PRESENCE mandatory},

...

}

SLDRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-FailedToBeModified-Item		CRITICALITY ignore	TYPE SLDRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-Setup-ItemIEs } }

UE-MulticastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-Setup-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-Setup-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

UEContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationFailureIEs} },

...

}

UEContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID		CRITICALITY reject	TYPE NRCGI							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- UE Context Modification Required (gNB-DU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

UEContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequiredIEs} },

...

}

UEContextModificationRequiredIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer		CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer			PRESENCE optional	}|

{ ID id-DUtoCURRCInformation						CRITICALITY reject	TYPE DUtoCURRCInformation								PRESENCE optional}|

{ ID id-DRBs-Required-ToBeModified-List				CRITICALITY reject	TYPE DRBs-Required-ToBeModified-List					PRESENCE optional}|

{ ID id-SRBs-Required-ToBeReleased-List				CRITICALITY reject	TYPE SRBs-Required-ToBeReleased-List					PRESENCE optional}|

{ ID id-DRBs-Required-ToBeReleased-List				CRITICALITY reject	TYPE DRBs-Required-ToBeReleased-List					PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-BHChannels-Required-ToBeReleased-List		CRITICALITY reject	TYPE BHChannels-Required-ToBeReleased-List			PRESENCE optional}|

{ ID id-SLDRBs-Required-ToBeModified-List			CRITICALITY reject	TYPE SLDRBs-Required-ToBeModified-List				PRESENCE optional}|

{ ID id-SLDRBs-Required-ToBeReleased-List			CRITICALITY reject	TYPE SLDRBs-Required-ToBeReleased-List				PRESENCE optional}|

{ ID id-targetCellsToCancel							CRITICALITY reject	TYPE TargetCellList											PRESENCE optional}|

{ ID id-UuRLCChannelRequiredToBeModifiedList		CRITICALITY reject	TYPE UuRLCChannelRequiredToBeModifiedList			PRESENCE optional}|

{ ID id-UuRLCChannelRequiredToBeReleasedList		CRITICALITY reject	TYPE UuRLCChannelRequiredToBeReleasedList			PRESENCE optional}|

{ ID id-PC5RLCChannelRequiredToBeModifiedList		CRITICALITY reject	TYPE PC5RLCChannelRequiredToBeModifiedList			PRESENCE optional}|

{ ID id-PC5RLCChannelRequiredToBeReleasedList		CRITICALITY reject	TYPE PC5RLCChannelRequiredToBeReleasedList			PRESENCE optional}|

{ ID id-UE-MulticastMRBs-RequiredToBeModified-List	CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeModified-List	PRESENCE optional  }|

{ ID id-UE-MulticastMRBs-RequiredToBeReleased-List	CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeReleased-List	PRESENCE optional  }|

{ ID id-LTMCells-ToBeReleased-List					CRITICALITY ignore	TYPE LTMCells-ToBeReleased-List						PRESENCE optional	},

...

}

DRBs-Required-ToBeModified-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Required-ToBeModified-ItemIEs } }

DRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Required-ToBeReleased-ItemIEs } }

SRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-Required-ToBeReleased-ItemIEs } }

BHChannels-Required-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-Required-ToBeReleased-ItemIEs } }

DRBs-Required-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Required-ToBeModified-Item			CRITICALITY reject	TYPE DRBs-Required-ToBeModified-Item		PRESENCE mandatory},

...

}

DRBs-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Required-ToBeReleased-Item			CRITICALITY reject	TYPE DRBs-Required-ToBeReleased-Item		PRESENCE mandatory},

...

}

SRBs-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-Required-ToBeReleased-Item			CRITICALITY reject	TYPE SRBs-Required-ToBeReleased-Item			PRESENCE mandatory},

...

}

BHChannels-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-Required-ToBeReleased-Item			CRITICALITY reject	TYPE BHChannels-Required-ToBeReleased-Item		PRESENCE mandatory},

...

}

SLDRBs-Required-ToBeModified-List::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Required-ToBeModified-ItemIEs } }

SLDRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Required-ToBeReleased-ItemIEs } }

SLDRBs-Required-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Required-ToBeModified-Item			CRITICALITY reject	TYPE SLDRBs-Required-ToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Required-ToBeReleased-Item			CRITICALITY reject	TYPE SLDRBs-Required-ToBeReleased-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-RequiredToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-RequiredToBeModified-ItemIEs} }

UE-MulticastMRBs-RequiredToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-RequiredToBeModified-Item	CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeModified-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-RequiredToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-RequiredToBeReleased-ItemIEs} }

UE-MulticastMRBs-RequiredToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-RequiredToBeReleased-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

UEContextModificationConfirm::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationConfirmIEs} },

...

}

UEContextModificationConfirmIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer		CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer			PRESENCE optional		}|

{ ID id-DRBs-ModifiedConf-List						CRITICALITY ignore	TYPE DRBs-ModifiedConf-List								PRESENCE optional		}|

{ ID id-RRCContainer								CRITICALITY ignore	TYPE RRCContainer											PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		}|

{ ID id-ExecuteDuplication							CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional		}|

{ ID id-ResourceCoordinationTransferInformation		CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation		PRESENCE optional		}|

{ ID id-SLDRBs-ModifiedConf-List					CRITICALITY ignore	TYPE SLDRBs-ModifiedConf-List							PRESENCE optional		}|

{ ID id-UuRLCChannelModifiedList					CRITICALITY reject	TYPE UuRLCChannelModifiedList							PRESENCE optional		}|

{ ID id-PC5RLCChannelModifiedList					CRITICALITY reject	TYPE PC5RLCChannelModifiedList							PRESENCE optional		}|

{ ID id-UE-MulticastMRBs-ConfirmedToBeModified-List	CRITICALITY reject	TYPE UE-MulticastMRBs-ConfirmedToBeModified-List	PRESENCE optional		},

...

}

DRBs-ModifiedConf-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ModifiedConf-ItemIEs } }

DRBs-ModifiedConf-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ModifiedConf-Item		CRITICALITY ignore	TYPE DRBs-ModifiedConf-Item			PRESENCE mandatory},

...

}

SLDRBs-ModifiedConf-List::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ModifiedConf-ItemIEs } }

SLDRBs-ModifiedConf-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ModifiedConf-Item		CRITICALITY ignore	TYPE SLDRBs-ModifiedConf-Item			PRESENCE mandatory},

...

}

UE-MulticastMRBs-ConfirmedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-ConfirmedToBeModified-ItemIEs} }

UE-MulticastMRBs-ConfirmedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ConfirmedToBeModified-Item	CRITICALITY reject	TYPE UE-MulticastMRBs-ConfirmedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

-- UE CONTEXT MODIFICATION REFUSE

--

-- **************************************************************

UEContextModificationRefuse::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRefuseIEs} },

...

}

UEContextModificationRefuseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- WRITE-REPLACE WARNING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Write-Replace Warning Request

--

-- **************************************************************

WriteReplaceWarningRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {WriteReplaceWarningRequestIEs} },

...

}

WriteReplaceWarningRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-PWSSystemInformation 				CRITICALITY reject	TYPE PWSSystemInformation 						PRESENCE mandatory }|

{ ID id-RepetitionPeriod 					CRITICALITY reject	TYPE RepetitionPeriod 							PRESENCE mandatory }|

{ ID id-NumberofBroadcastRequest 			CRITICALITY reject	TYPE NumberofBroadcastRequest 				PRESENCE mandatory }|

{ ID id-Cells-To-Be-Broadcast-List			CRITICALITY reject	TYPE Cells-To-Be-Broadcast-List				PRESENCE optional	},

...

}

Cells-To-Be-Broadcast-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-To-Be-Broadcast-List-ItemIEs } }

Cells-To-Be-Broadcast-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-To-Be-Broadcast-Item		CRITICALITY reject	TYPE	Cells-To-Be-Broadcast-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- Write-Replace Warning Response

--

-- **************************************************************

WriteReplaceWarningResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {WriteReplaceWarningResponseIEs} },

...

}

WriteReplaceWarningResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory	}|

{ ID id-Cells-Broadcast-Completed-List			CRITICALITY reject	TYPE Cells-Broadcast-Completed-List				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List			PRESENCE optional	},

...

}

Cells-Broadcast-Completed-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Broadcast-Completed-List-ItemIEs } }

Cells-Broadcast-Completed-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Broadcast-Completed-Item		CRITICALITY reject	TYPE	Cells-Broadcast-Completed-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PWS CANCEL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS Cancel Request

--

-- **************************************************************

PWSCancelRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {PWSCancelRequestIEs} },

...

}

PWSCancelRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-NumberofBroadcastRequest 				CRITICALITY reject TYPE NumberofBroadcastRequest						PRESENCE mandatory }|

{ ID id-Broadcast-To-Be-Cancelled-List			CRITICALITY reject TYPE Broadcast-To-Be-Cancelled-List				PRESENCE optional	}|

{ ID id-Cancel-all-Warning-Messages-Indicator	CRITICALITY reject TYPE Cancel-all-Warning-Messages-Indicator	PRESENCE optional	}|

{ ID id-NotificationInformation					CRITICALITY reject TYPE NotificationInformation						PRESENCE optional},

...

}

Broadcast-To-Be-Cancelled-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Broadcast-To-Be-Cancelled-List-ItemIEs } }

Broadcast-To-Be-Cancelled-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Broadcast-To-Be-Cancelled-Item		CRITICALITY reject	TYPE	Broadcast-To-Be-Cancelled-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PWS Cancel Response

--

-- **************************************************************

PWSCancelResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {PWSCancelResponseIEs} },

...

}

PWSCancelResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-Cells-Broadcast-Cancelled-List	CRITICALITY reject	TYPE Cells-Broadcast-Cancelled-List	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

Cells-Broadcast-Cancelled-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Broadcast-Cancelled-List-ItemIEs } }

Cells-Broadcast-Cancelled-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Broadcast-Cancelled-Item		CRITICALITY reject	TYPE	Cells-Broadcast-Cancelled-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE Inactivity Notification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Inactivity Notification

--

-- **************************************************************

UEInactivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ UEInactivityNotificationIEs}},

...

}

UEInactivityNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-DRB-Activity-List							CRITICALITY reject	TYPE DRB-Activity-List							PRESENCE mandatory	}|

{ ID id-SDT-Termination-Request						CRITICALITY ignore	TYPE SDT-Termination-Request					PRESENCE optional 	},

...

}

DRB-Activity-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRB-Activity-ItemIEs } }

DRB-Activity-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Activity-Item			CRITICALITY reject	TYPE DRB-Activity-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- Initial UL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- INITIAL UL RRC Message Transfer

--

-- **************************************************************

InitialULRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ InitialULRRCMessageTransferIEs}},

...

}

InitialULRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-NRCGI								CRITICALITY reject	TYPE NRCGI									PRESENCE mandatory	}|

{ ID id-C-RNTI								CRITICALITY reject	TYPE C-RNTI									PRESENCE mandatory	}|

{ ID id-RRCContainer						CRITICALITY reject	TYPE RRCContainer							PRESENCE mandatory	}|

{ ID id-DUtoCURRCContainer					CRITICALITY reject	TYPE DUtoCURRCContainer						PRESENCE optional	}|

{ ID id-SULAccessIndication					CRITICALITY ignore	TYPE SULAccessIndication					PRESENCE optional	}|

{ ID id-TransactionID						CRITICALITY ignore	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-RANUEID								CRITICALITY ignore	TYPE RANUEID								PRESENCE optional	}|

{ ID id-RRCContainer-RRCSetupComplete		CRITICALITY ignore	TYPE RRCContainer-RRCSetupComplete 			PRESENCE optional	}|

{ ID id-NRRedCapUEIndication				CRITICALITY ignore	TYPE NRRedCapUEIndication  					PRESENCE optional	}|

{ ID id-SDTInformation						CRITICALITY ignore	TYPE SDTInformation							PRESENCE optional	}|

{ ID id-SidelinkRelayConfiguration			CRITICALITY ignore	TYPE SidelinkRelayConfiguration			PRESENCE optional	}|

{ ID id-NReRedCapUEIndication					CRITICALITY ignore	TYPE NReRedCapUEIndication  				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- DL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DL RRC Message Transfer

--

-- **************************************************************

DLRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DLRRCMessageTransferIEs}},

...

}

DLRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-oldgNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE optional	}|

{ ID id-SRBID										CRITICALITY reject	TYPE SRBID											PRESENCE mandatory	}|

{ ID id-ExecuteDuplication							CRITICALITY ignore	TYPE ExecuteDuplication							PRESENCE optional}|

{ ID id-RRCContainer								CRITICALITY reject	TYPE RRCContainer									PRESENCE mandatory	}|

{ ID id-RAT-FrequencyPriorityInformation			CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation		PRESENCE optional	}|

{ ID id-RRCDeliveryStatusRequest					CRITICALITY ignore	TYPE RRCDeliveryStatusRequest					PRESENCE optional }|

{ ID id-UEContextNotRetrievable						CRITICALITY reject	TYPE UEContextNotRetrievable					PRESENCE optional }|

{ ID id-RedirectedRRCmessage						CRITICALITY reject	TYPE OCTET STRING									PRESENCE optional }|

{ ID id-PLMNAssistanceInfoForNetShar				CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-new-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex					CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex				PRESENCE optional }|

{ ID id-SRBMappingInfo								CRITICALITY ignore	TYPE UuRLCChannelID					 				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- UL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UL RRC Message Transfer

--

-- **************************************************************

ULRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ULRRCMessageTransferIEs}},

...

}

ULRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-SRBID							CRITICALITY reject	TYPE SRBID							PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY reject	TYPE RRCContainer					PRESENCE mandatory	}|

{ ID id-SelectedPLMNID					CRITICALITY reject	TYPE PLMN-Identity					PRESENCE optional		}|

{ ID id-new-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs F1AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- System Information ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- System information Delivery Command

--

-- **************************************************************

SystemInformationDeliveryCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ SystemInformationDeliveryCommandIEs}},

...

}

SystemInformationDeliveryCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-SItype-List				CRITICALITY reject	TYPE SItype-List					PRESENCE mandatory	}|

{ ID id-ConfirmedUEID 			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Paging PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PagingIEs}},

...

}

PagingIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue	CRITICALITY reject	TYPE UEIdentityIndexValue		PRESENCE mandatory	}|

{ ID id-PagingIdentity			CRITICALITY reject	TYPE PagingIdentity				PRESENCE mandatory	}|

{ ID id-PagingDRX				CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-PagingPriority			CRITICALITY ignore	TYPE PagingPriority				PRESENCE optional	}|

{ ID id-PagingCell-List			CRITICALITY ignore	TYPE PagingCell-list			PRESENCE mandatory	}|

{ ID id-PagingOrigin			CRITICALITY ignore	TYPE PagingOrigin				PRESENCE optional	}|

{ ID id-RANUEPagingDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-CNUEPagingDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-NRPagingeDRXInformation	CRITICALITY ignore	TYPE NRPagingeDRXInformation	PRESENCE optional	}|

{ ID id-NRPagingeDRXInformationforRRCINACTIVE	CRITICALITY ignore	TYPE NRPagingeDRXInformationforRRCINACTIVE	PRESENCE optional	}|

{ ID id-PagingCause			CRITICALITY ignore	TYPE PagingCause				PRESENCE optional	}|

{ ID id-PEIPSAssistanceInfo		CRITICALITY ignore	TYPE PEIPSAssistanceInfo		PRESENCE optional	}|

{ ID id-UEPagingCapability		CRITICALITY ignore	TYPE UEPagingCapability			PRESENCE optional	}|

{ ID id-ExtendedUEIdentityIndexValue	CRITICALITY ignore	TYPE ExtendedUEIdentityIndexValue		PRESENCE optional}|

{ ID id-HashedUEIdentityIndexValue		CRITICALITY ignore	TYPE HashedUEIdentityIndexValue			PRESENCE optional}|

{ ID id-MT-SDT-Information		CRITICALITY ignore	TYPE MT-SDT-Information			PRESENCE optional	}|

{ ID id-NRPaginglongeDRXInformationforRRCINACTIVE		CRITICALITY ignore		TYPE NRPaginglongeDRXInformationforRRCINACTIVE				PRESENCE optional },

...

}

PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF ProtocolIE-SingleContainer { { PagingCell-ItemIEs } }

PagingCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-PagingCell-Item		CRITICALITY ignore	TYPE PagingCell-Item			PRESENCE mandatory}	,

...

}

-- **************************************************************

--

#### -- Notify

--

-- **************************************************************

Notify ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ NotifyIEs}},

...

}

NotifyIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-DRB-Notify-List						CRITICALITY reject	TYPE DRB-Notify-List						PRESENCE mandatory	},

...

}

DRB-Notify-List::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRB-Notify-ItemIEs } }

DRB-Notify-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Notify-Item			CRITICALITY reject	TYPE DRB-Notify-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- NETWORK ACCESS RATE REDUCTION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Network Access Rate Reduction

--

-- **************************************************************

NetworkAccessRateReduction ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ NetworkAccessRateReductionIEs }},

...

}

NetworkAccessRateReductionIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID 					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-UAC-Assistance-Info				CRITICALITY reject	TYPE UAC-Assistance-Info			PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PWS RESTART INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS Restart Indication

--

-- **************************************************************

PWSRestartIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { PWSRestartIndicationIEs} },

...

}

PWSRestartIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NR-CGI-List-For-Restart-List	CRITICALITY reject	TYPE NR-CGI-List-For-Restart-List	PRESENCE mandatory	},

...

}

NR-CGI-List-For-Restart-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { NR-CGI-List-For-Restart-List-ItemIEs } }

NR-CGI-List-For-Restart-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-NR-CGI-List-For-Restart-Item		CRITICALITY reject	TYPE	NR-CGI-List-For-Restart-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PWS FAILURE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS Failure Indication

--

-- **************************************************************

PWSFailureIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { PWSFailureIndicationIEs} },

...

}

PWSFailureIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-PWS-Failed-NR-CGI-List	CRITICALITY reject	TYPE PWS-Failed-NR-CGI-List		PRESENCE optional	},

...

}

PWS-Failed-NR-CGI-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { PWS-Failed-NR-CGI-List-ItemIEs } }

PWS-Failed-NR-CGI-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-PWS-Failed-NR-CGI-Item		CRITICALITY reject	TYPE	PWS-Failed-NR-CGI-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- gNB-DU STATUS INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- gNB-DU Status Indication

--

-- **************************************************************

GNBDUStatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUStatusIndicationIEs} },

...

}

GNBDUStatusIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-GNBDUOverloadInformation		CRITICALITY reject	TYPE GNBDUOverloadInformation		PRESENCE mandatory	}|

{ ID id-IABCongestionIndication			CRITICALITY ignore	TYPE IABCongestionIndication		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- RRC Delivery Report ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- RRC Delivery Report

--

-- **************************************************************

RRCDeliveryReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ RRCDeliveryReportIEs}},

...

}

RRCDeliveryReportIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID	PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID	PRESENCE mandatory	}|

{ ID id-RRCDeliveryStatus	CRITICALITY ignore	TYPE RRCDeliveryStatus	PRESENCE mandatory	}|

{ ID id-SRBID				CRITICALITY ignore	TYPE SRBID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- F1 Removal ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- F1 Removal Request

--

-- **************************************************************

F1RemovalRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ F1RemovalRequestIEs }},

...

}

F1RemovalRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- F1 Removal Response

--

-- **************************************************************

F1RemovalResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ F1RemovalResponseIEs }},

...

}

F1RemovalResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- F1 Removal Failure

--

-- **************************************************************

F1RemovalFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ F1RemovalFailureIEs }},

...

}

F1RemovalFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation				CRITICALITY ignore	TYPE TraceActivation				PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceID						CRITICALITY ignore	TYPE TraceID						PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- CELL TRAFFIC TRACE

--

-- **************************************************************

CellTrafficTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CellTrafficTraceIEs} },

...

}

CellTrafficTraceIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ID id-TraceID							CRITICALITY ignore	TYPE TraceID						PRESENCE mandatory	}|

{ID id-TraceCollectionEntityIPAddress	CRITICALITY ignore	TYPE TransportLayerAddress			PRESENCE mandatory	}|

{ID id-PrivacyIndicator					CRITICALITY ignore	TYPE PrivacyIndicator				PRESENCE optional	}|

{ID id-TraceCollectionEntityURI	CRITICALITY ignore	TYPE URI-address		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- DU-CU Radio Information Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Radio Information Transfer

--

-- **************************************************************

DUCURadioInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCURadioInformationTransferIEs}},

...

}

DUCURadioInformationTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-DUCURadioInformationType		CRITICALITY ignore	TYPE DUCURadioInformationType				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- CU-DU Radio Information Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU Radio Information Transfer

--

-- **************************************************************

CUDURadioInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDURadioInformationTransferIEs}},

...

}

CUDURadioInformationTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-CUDURadioInformationType	CRITICALITY ignore	TYPE CUDURadioInformationType				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- IAB PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- BAP Mapping Configuration ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BAP MAPPING CONFIGURATION

--

-- **************************************************************

BAPMappingConfiguration ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{ {BAPMappingConfiguration-IEs} },

...

}

BAPMappingConfiguration-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID								PRESENCE mandatory}|

{ ID id-BH-Routing-Information-Added-List		CRITICALITY ignore	TYPE	BH-Routing-Information-Added-List	PRESENCE optional}|

{ ID id-BH-Routing-Information-Removed-List		CRITICALITY ignore	TYPE	BH-Routing-Information-Removed-List	PRESENCE optional}|

{ ID id-TrafficMappingInformation				CRITICALITY ignore	TYPE	TrafficMappingInfo						PRESENCE optional}|

{ ID id-BufferSizeThresh						CRITICALITY ignore	TYPE	BufferSizeThresh							PRESENCE optional}|

{ ID id-BAP-Header-Rewriting-Added-List			CRITICALITY ignore	TYPE	BAP-Header-Rewriting-Added-List		PRESENCE optional}|

{ ID id-Re-routingEnableIndicator				CRITICALITY ignore	TYPE	Re-routingEnableIndicator				PRESENCE optional}|

{ ID id-BAP-Header-Rewriting-Removed-List		CRITICALITY ignore	TYPE	BAP-Header-Rewriting-Removed-List	PRESENCE optional},

...

}

BH-Routing-Information-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BH-Routing-Information-Added-List-ItemIEs } }

BH-Routing-Information-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BH-Routing-Information-Removed-List-ItemIEs } }

BH-Routing-Information-Added-List-ItemIEs	F1AP-PROTOCOL-IES ::= {

{ ID id-BH-Routing-Information-Added-List-Item				CRITICALITY ignore	TYPE BH-Routing-Information-Added-List-Item						PRESENCE optional},

...

}

BH-Routing-Information-Removed-List-ItemIEs	F1AP-PROTOCOL-IES ::= {

{ ID id-BH-Routing-Information-Removed-List-Item				CRITICALITY ignore	TYPE BH-Routing-Information-Removed-List-Item						PRESENCE optional},

...

}

BAP-Header-Rewriting-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Added-List-ItemIEs } }

BAP-Header-Rewriting-Added-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BAP-Header-Rewriting-Added-List-Item	CRITICALITY ignore	TYPE BAP-Header-Rewriting-Added-List-Item PRESENCE optional},

...

}

BAP-Header-Rewriting-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Removed-List-ItemIEs } }

BAP-Header-Rewriting-Removed-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BAP-Header-Rewriting-Removed-List-Item	CRITICALITY ignore	TYPE BAP-Header-Rewriting-Removed-List-Item PRESENCE optional},

...

}

-- **************************************************************

--

##### -- BAP MAPPING CONFIGURATION ACKNOWLEDGE

--

-- **************************************************************

BAPMappingConfigurationAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BAPMappingConfigurationAcknowledge-IEs} },

...

}

BAPMappingConfigurationAcknowledge-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE	TransactionID			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE	CriticalityDiagnostics	PRESENCE optional},

...

}

-- **************************************************************

--

##### -- BAP MAPPING CONFIGURATION FAILURE

--

-- **************************************************************

BAPMappingConfigurationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { BAPMappingConfigurationFailureIEs} },

...

}

BAPMappingConfigurationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-DU Configuration ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU RESOURCE CONFIGURATION

--

-- **************************************************************

GNBDUResourceConfiguration ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{{ GNBDUResourceConfigurationIEs}},

...

}

GNBDUResourceConfigurationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-Activated-Cells-to-be-Updated-List		CRITICALITY reject	TYPE Activated-Cells-to-be-Updated-List	PRESENCE optional}|

{ ID id-Child-Nodes-List						CRITICALITY reject	TYPE Child-Nodes-List							PRESENCE optional}|

{ ID id-Neighbour-Node-Cells-List				CRITICALITY reject	TYPE Neighbour-Node-Cells-List				PRESENCE optional}|

{ ID id-Serving-Cells-List						CRITICALITY reject	TYPE Serving-Cells-List							PRESENCE optional},

...

}

-- **************************************************************

--

##### -- GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE

--

-- **************************************************************

GNBDUResourceConfigurationAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { GNBDUResourceConfigurationAcknowledgeIEs} },

...

}

GNBDUResourceConfigurationAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-DU RESOURCE CONFIGURATION FAILURE

--

-- **************************************************************

GNBDUResourceConfigurationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { GNBDUResourceConfigurationFailureIEs} },

...

}

GNBDUResourceConfigurationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- IAB TNL Address Allocation ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- IAB TNL ADDRESS REQUEST

--

-- **************************************************************

IABTNLAddressRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {IABTNLAddressRequestIEs} },

...

}

IABTNLAddressRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-IABv4AddressesRequested				CRITICALITY reject	TYPE IABv4AddressesRequested					PRESENCE optional	}|

{ ID id-IABIPv6RequestType					CRITICALITY reject	TYPE IABIPv6RequestType							PRESENCE optional	}|

{ ID id-IAB-TNL-Addresses-To-Remove-List	CRITICALITY reject	TYPE IAB-TNL-Addresses-To-Remove-List		PRESENCE optional	}|

{ ID id-IAB-TNL-Addresses-Exception			CRITICALITY reject	TYPE IAB-TNL-Addresses-Exception				PRESENCE optional	},

...

}

IAB-TNL-Addresses-To-Remove-List	::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))	OF ProtocolIE-SingleContainer { { IAB-TNL-Addresses-To-Remove-ItemIEs } }

IAB-TNL-Addresses-To-Remove-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-IAB-TNL-Addresses-To-Remove-Item			CRITICALITY reject	TYPE IAB-TNL-Addresses-To-Remove-Item					PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- IAB TNL ADDRESS RESPONSE

--

-- **************************************************************

IABTNLAddressResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {IABTNLAddressResponseIEs} },

...

}

IABTNLAddressResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-IAB-Allocated-TNL-Address-List				CRITICALITY reject	TYPE IAB-Allocated-TNL-Address-List			PRESENCE mandatory	},

...

}

IAB-Allocated-TNL-Address-List ::= SEQUENCE (SIZE(1.. maxnoofTLAsIAB))	OF ProtocolIE-SingleContainer { { IAB-Allocated-TNL-Address-List-ItemIEs } }

IAB-Allocated-TNL-Address-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-IAB-Allocated-TNL-Address-Item			CRITICALITY reject	TYPE IAB-Allocated-TNL-Address-Item					PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- IAB TNL ADDRESS FAILURE

--

-- **************************************************************

IABTNLAddressFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABTNLAddressFailureIEs} },

...

}

IABTNLAddressFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- IAB UP Configuration Update ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- IAB UP Configuration Update Request

--

-- **************************************************************

IABUPConfigurationUpdateRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABUPConfigurationUpdateRequestIEs} },

...

}

IABUPConfigurationUpdateRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory  }|

{ ID id-UL-UP-TNL-Information-to-Update-List	CRITICALITY ignore	TYPE UL-UP-TNL-Information-to-Update-List		PRESENCE optional	}|

{ ID id-UL-UP-TNL-Address-to-Update-List		CRITICALITY ignore	TYPE UL-UP-TNL-Address-to-Update-List				PRESENCE optional	},

...

}

UL-UP-TNL-Information-to-Update-List ::= SEQUENCE (SIZE(1.. maxnoofULUPTNLInformationforIAB))	OF ProtocolIE-SingleContainer { { UL-UP-TNL-Information-to-Update-List-ItemIEs } }

UL-UP-TNL-Information-to-Update-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UL-UP-TNL-Information-to-Update-List-Item	CRITICALITY ignore	TYPE UL-UP-TNL-Information-to-Update-List-Item PRESENCE mandatory },

...

}

UL-UP-TNL-Address-to-Update-List ::= SEQUENCE (SIZE(1.. maxnoofUPTNLAddresses))	OF ProtocolIE-SingleContainer { { UL-UP-TNL-Address-to-Update-List-ItemIEs } }

UL-UP-TNL-Address-to-Update-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UL-UP-TNL-Address-to-Update-List-Item	CRITICALITY ignore	TYPE UL-UP-TNL-Address-to-Update-List-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

-- IAB UP Configuration Update Response

--

-- **************************************************************

IABUPConfigurationUpdateResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABUPConfigurationUpdateResponseIEs} },

...

}

IABUPConfigurationUpdateResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional	}|

{ ID id-DL-UP-TNL-Address-to-Update-List	CRITICALITY reject	TYPE DL-UP-TNL-Address-to-Update-List	PRESENCE optional	},

...

}

DL-UP-TNL-Address-to-Update-List ::= SEQUENCE (SIZE(1.. maxnoofUPTNLAddresses))	OF ProtocolIE-SingleContainer { { DL-UP-TNL-Address-to-Update-List-ItemIEs } }

DL-UP-TNL-Address-to-Update-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DL-UP-TNL-Address-to-Update-List-Item	CRITICALITY ignore	TYPE DL-UP-TNL-Address-to-Update-List-Item	PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- IAB UP Configuration Update Failure

--

-- **************************************************************

IABUPConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABUPConfigurationUpdateFailureIEs} },

...

}

IABUPConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

#### -- MIAB F1 SETUP TRIGGERING PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MIAB F1 SETUP TRIGGERING

--

-- **************************************************************

MIABF1SetupTriggering ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MIABF1SetupTriggeringIEs}},

...

}

MIABF1SetupTriggeringIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-Target-gNB-ID						CRITICALITY reject	TYPE GlobalGNB-ID							PRESENCE mandatory	}|

{ ID id-Target-gNB-IP-address				CRITICALITY ignore	TYPE TransportLayerAddress					PRESENCE optional	}|

{ ID id-Target-SeGW-IP-address				CRITICALITY ignore	TYPE TransportLayerAddress					PRESENCE optional	},

...

}

#### -- MIAB F1 SETUP OUTCOME NOTIFICATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MIAB F1 SETUP OUTCOME NOTIFICATION

--

-- **************************************************************

MIABF1SetupOutcomeNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MIABF1SetupOutcomeNotificationIEs}},

...

}

MIABF1SetupOutcomeNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-F1SetupOutcome						CRITICALITY reject	TYPE F1SetupOutcome								PRESENCE mandatory	}|

{ ID id-Activated-Cells-Mapping-List		CRITICALITY ignore	TYPE Activated-Cells-Mapping-List			PRESENCE optional	}|

{ ID id-Target-F1-Terminating-Donor-gNB-ID	CRITICALITY reject	TYPE GlobalGNB-ID								PRESENCE optional	},

...

}

F1SetupOutcome ::= ENUMERATED {success, failure,...}

Activated-Cells-Mapping-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Activated-Cells-Mapping-List-ItemIEs } }

Activated-Cells-Mapping-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Activated-Cells-Mapping-List-Item	CRITICALITY ignore	TYPE Activated-Cells-Mapping-List-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- Resource Status Reporting Initiation ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Request

--

-- **************************************************************

ResourceStatusRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResourceStatusRequestIEs} },

...

}

ResourceStatusRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE conditional	}|

{ ID id-RegistrationRequest		CRITICALITY ignore	TYPE RegistrationRequest	PRESENCE mandatory	}|

{ ID id-ReportCharacteristics	CRITICALITY ignore	TYPE ReportCharacteristics	PRESENCE conditional	}|

{ ID id-CellToReportList		CRITICALITY ignore	TYPE CellToReportList		PRESENCE optional	}|

{ ID id-ReportingPeriodicity	CRITICALITY ignore	TYPE ReportingPeriodicity	PRESENCE  optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Response

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusResponseIEs} },

...

}

ResourceStatusResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Failure

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusFailureIEs} },

...

}

ResourceStatusFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Resource Status Reporting ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Update

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ResourceStatusUpdateIEs}},

...

}

ResourceStatusUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID			CRITICALITY reject	TYPE GNBCUMeasurementID				PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID			CRITICALITY ignore	TYPE GNBDUMeasurementID			PRESENCE mandatory	}|

{ ID id-HardwareLoadIndicator			CRITICALITY ignore	TYPE HardwareLoadIndicator			PRESENCE optional	}|

{ ID id-TNLCapacityIndicator			CRITICALITY ignore	TYPE TNLCapacityIndicator		PRESENCE optional	}|

{ ID id-CellMeasurementResultList		CRITICALITY ignore	TYPE CellMeasurementResultList	PRESENCE optional	},

...

}

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- REFERENCE TIME INFORMATION REPORTING CONTROL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- REFERENCE TIME INFORMATION REPORTING CONTROL

--

-- **************************************************************

ReferenceTimeInformationReportingControl::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ReferenceTimeInformationReportingControlIEs} },

...

}

ReferenceTimeInformationReportingControlIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-ReportingRequestType		CRITICALITY reject	TYPE ReportingRequestType		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- REFERENCE TIME INFORMATION REPORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- REFERENCE TIME INFORMATION REPORT

--

-- **************************************************************

ReferenceTimeInformationReport::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ReferenceTimeInformationReportIEs} },

...

}

ReferenceTimeInformationReportIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY ignore	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-TimeReferenceInformation	CRITICALITY ignore	TYPE TimeReferenceInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- ACCESS SUCCESS ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

#### -- Access Success

--

-- **************************************************************

AccessSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ AccessSuccessIEs}},

...

}

AccessSuccessIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-NRCGI								CRITICALITY reject	TYPE NRCGI									PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING ASSISTANCE INFORMATION CONTROL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Assistance Information Control

--

-- **************************************************************

PositioningAssistanceInformationControl ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PositioningAssistanceInformationControlIEs}},

...

}

PositioningAssistanceInformationControlIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-PosAssistance-Information		CRITICALITY reject	TYPE PosAssistance-Information		PRESENCE optional}|

{ ID id-PosBroadcast					CRITICALITY reject	TYPE PosBroadcast				PRESENCE optional}|

{ ID id-PositioningBroadcastCells		CRITICALITY reject	TYPE PositioningBroadcastCells		PRESENCE optional}|

{ ID id-RoutingID						CRITICALITY reject	TYPE RoutingID				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING ASSISTANCE INFORMATION FEEDBACK ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Assistance Information Feedback

--

-- **************************************************************

PositioningAssistanceInformationFeedback ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PositioningAssistanceInformationFeedbackIEs}},

...

}

PositioningAssistanceInformationFeedbackIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-PosAssistanceInformationFailureList	CRITICALITY reject	TYPE PosAssistanceInformationFailureList	PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells				PRESENCE optional}|

{ ID id-RoutingID								CRITICALITY reject	TYPE RoutingID									PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITONING MEASUREMENT EXCHANGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Request

--

-- **************************************************************

PositioningMeasurementRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningMeasurementRequestIEs} },

...

}

PositioningMeasurementRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory}|

{ ID id-LMF-MeasurementID						CRITICALITY reject	TYPE LMF-MeasurementID								PRESENCE mandatory}|

{ ID id-RAN-MeasurementID						CRITICALITY reject	TYPE RAN-MeasurementID								PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList				CRITICALITY reject	TYPE TRP-MeasurementRequestList					PRESENCE mandatory}|

{ ID id-PosReportCharacteristics				CRITICALITY reject	TYPE PosReportCharacteristics					PRESENCE mandatory}|

{ ID id-PosMeasurementPeriodicity				CRITICALITY reject	TYPE MeasurementPeriodicity						PRESENCE conditional }|

-- The above IE shall be present if the PosReportCharacteristics IE is set to “periodic” --

{ ID id-PosMeasurementQuantities				CRITICALITY reject	TYPE PosMeasurementQuantities					PRESENCE mandatory}|

{ ID id-SFNInitialisationTime					CRITICALITY ignore	TYPE RelativeTime1900	PRESENCE optional	}|

{ ID id-SRSConfiguration						CRITICALITY ignore	TYPE SRSConfiguration								PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest				CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional	}|

{ ID id-SystemFrameNumber						CRITICALITY ignore	TYPE SystemFrameNumber		PRESENCE optional}|

{ ID id-SlotNumber								CRITICALITY ignore	TYPE SlotNumber				PRESENCE optional}|

{ ID id-PosMeasurementPeriodicityExtended		CRITICALITY reject	TYPE MeasurementPeriodicityExtended			PRESENCE conditional }|

-- The IE shall be present the MeasurementPeriodicity IE is set to the value "extended"

{ ID id-ResponseTime							CRITICALITY ignore	TYPE ResponseTime									PRESENCE optional}|

{ ID id-MeasurementCharacteristicsRequestIndicator			CRITICALITY ignore	TYPE MeasurementCharacteristicsRequestIndicator	PRESENCE optional}|

{ ID id-MeasurementTimeOccasion					CRITICALITY ignore	TYPE MeasurementTimeOccasion	PRESENCE optional	}|

{ ID id-PosMeasurementAmount					CRITICALITY ignore	TYPE PosMeasurementAmount	PRESENCE optional	}|

{ ID id-TimeWindowInformation-Measurement-List		CRITICALITY ignore	TYPE TimeWindowInformation-Measurement-List	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Positioning Measurement Response

--

-- **************************************************************

PositioningMeasurementResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningMeasurementResponseIEs} },

...

}

PositioningMeasurementResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory}|

{ ID id-LMF-MeasurementID						CRITICALITY reject	TYPE LMF-MeasurementID								PRESENCE mandatory}|

{ ID id-RAN-MeasurementID						CRITICALITY reject	TYPE RAN-MeasurementID								PRESENCE mandatory}|

{ ID id-PosMeasurementResultList				CRITICALITY reject	TYPE PosMeasurementResultList					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Positioning Measurement Failure

--

-- **************************************************************

PositioningMeasurementFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningMeasurementFailureIEs} },

...

}

PositioningMeasurementFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID				CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID				CRITICALITY reject	TYPE RAN-MeasurementID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT REPORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Report

--

-- **************************************************************

PositioningMeasurementReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementReportIEs} },

...

}

PositioningMeasurementReportIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID			PRESENCE mandatory	}|

{ ID id-PosMeasurementResultList	CRITICALITY reject	TYPE PosMeasurementResultList	PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT ABORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Abort

--

-- **************************************************************

PositioningMeasurementAbort ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementAbortIEs} },

...

}

PositioningMeasurementAbortIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID				CRITICALITY reject	TYPE LMF-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID				CRITICALITY reject	TYPE RAN-MeasurementID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT FAILURE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Failure Indication

--

-- **************************************************************

PositioningMeasurementFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementFailureIndicationIEs} },

...

}

PositioningMeasurementFailureIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Update

--

-- **************************************************************

PositioningMeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementUpdateIEs} },

...

}

PositioningMeasurementUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-TRP-MeasurementUpdateList	CRITICALITY reject	TYPE TRP-MeasurementUpdateList 	PRESENCE optional}|

{ ID id-MeasurementCharacteristicsRequestIndicator	CRITICALITY ignore	TYPE	MeasurementCharacteristicsRequestIndicator	PRESENCE optional}|

{ ID id-MeasurementTimeOccasion		CRITICALITY ignore	TYPE MeasurementTimeOccasion	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION EXCHANGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- TRP Information Request

--

-- **************************************************************

TRPInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { TRPInformationRequestIEs} },

...

}

TRPInformationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-TRPList								CRITICALITY ignore	TYPE TRPList									PRESENCE optional	}|

{ ID id-TRPInformationTypeListTRPReq		CRITICALITY reject	TYPE TRPInformationTypeListTRPReq			PRESENCE mandatory	},

...

}

TRPInformationTypeListTRPReq ::= SEQUENCE (SIZE(1.. maxnoofTRPInfoTypes)) OF ProtocolIE-SingleContainer { { TRPInformationTypeItemTRPReq } }

TRPInformationTypeItemTRPReq 	F1AP-PROTOCOL-IES ::= {

{ ID id-TRPInformationTypeItem	 CRITICALITY reject		TYPE TRPInformationTypeItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- TRP Information Response

--

-- **************************************************************

TRPInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { TRPInformationResponseIEs} },

...

}

TRPInformationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-TRPInformationListTRPResp		CRITICALITY ignore	TYPE TRPInformationListTRPResp		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

TRPInformationListTRPResp ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF ProtocolIE-SingleContainer { { TRPInformationItemTRPResp } }

TRPInformationItemTRPResp 	F1AP-PROTOCOL-IES ::= {

{ ID id-TRPInformationItem	 CRITICALITY ignore		TYPE TRPInformationItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- TRP Information Failure

--

-- **************************************************************

TRPInformationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { TRPInformationFailureIEs} },

...

}

TRPInformationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION EXCHANGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Information Request

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationRequestIEs} },

...

}

PositioningInformationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional}|

{ ID id-UEReportingInformation					CRITICALITY ignore	TYPE UEReportingInformation							PRESENCE optional}|

{ ID id-SRSPosRRCInactiveQueryIndication		CRITICALITY ignore	TYPE SRSPosRRCInactiveQueryIndication				PRESENCE optional}|

{ ID id-TimeWindowInformation-SRS-List			CRITICALITY ignore	TYPE TimeWindowInformation-SRS-List						PRESENCE optional},

...

}

-- **************************************************************

--

##### -- Positioning Information Response

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationResponseIEs} },

...

}

PositioningInformationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional }|

{ ID id-SRSPosRRCInactiveConfig		CRITICALITY ignore	TYPE SRSPosRRCInactiveConfig	PRESENCE optional}|

{ ID id-SRSPosRRCInactiveValidityAreaConfig		CRITICALITY ignore	TYPE SRSPosRRCInactiveValidityAreaConfig	PRESENCE optional}|

{ ID id-SRSPreconfiguration-List	CRITICALITY ignore	TYPE SRSPreconfiguration-List	PRESENCE optional},

...

}

-- **************************************************************

--

##### -- Positioning Information Failure

--

-- **************************************************************

PositioningInformationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationFailureIEs} },

...

}

PositioningInformationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITIONING ACTIVATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Activation Request

--

-- **************************************************************

PositioningActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationRequestIEs} },

...

}

PositioningActivationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SRSType					CRITICALITY reject	TYPE SRSType					PRESENCE mandatory	}|

{ ID id-ActivationTime			CRITICALITY ignore	TYPE RelativeTime1900		PRESENCE optional	},

...

}

SRSType ::= CHOICE {

semipersistentSRS				SemipersistentSRS,

aperiodicSRS					AperiodicSRS,

choice-extension				ProtocolIE-SingleContainer { { SRSType-ExtIEs} }

}

SRSType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SemipersistentSRS ::= SEQUENCE {

sRSResourceSetID			SRSResourceSetID,

sRSSpatialRelation			SpatialRelationInfo	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SemipersistentSRS-ExtIEs} } OPTIONAL,

...

}

SemipersistentSRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelationPerSRSResource	CRITICALITY ignore	EXTENSION SpatialRelationPerSRSResource PRESENCE optional},

...

}

AperiodicSRS ::= SEQUENCE {

aperiodic					ENUMERATED {true, ...},

sRSResourceTrigger			SRSResourceTrigger		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {AperiodicSRS-ExtIEs} } OPTIONAL,

...

}

AperiodicSRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

##### -- Positioning Activation Response

--

-- **************************************************************

PositioningActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationResponseIEs} },

...

}

PositioningActivationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SystemFrameNumber		CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional }|

{ ID id-SlotNumber				CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional }|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

##### -- Positioning Activation Failure

--

-- **************************************************************

PositioningActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationFailureIEs} },

...

}

PositioningActivationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITIONING DEACTIVATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Deactivation

--

-- **************************************************************

PositioningDeactivation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningDeactivationIEs} },

...

}

PositioningDeactivationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-AbortTransmission		CRITICALITY ignore	TYPE AbortTransmission			PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Information Update

--

-- **************************************************************

PositioningInformationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationUpdateIEs} },

...

}

PositioningInformationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration		CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime	CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional},

...

}

-- **************************************************************

--

-- SRS Information Reservation Notification

--

-- **************************************************************

SRSInformationReservationNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ SRSInformationReservationNotificationIEs}},

...

}

SRSInformationReservationNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-SRSReservationType		CRITICALITY ignore	TYPE SRSReservationType			PRESENCE mandatory	}|

{ ID id-SRSInformation			CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Initiation Request

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID			CRITICALITY reject	TYPE LMF-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID			CRITICALITY reject	TYPE RAN-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-E-CID-ReportCharacteristics		CRITICALITY reject	TYPE E-CID-ReportCharacteristics		PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementPeriodicity	CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional	}|

-- The above IE shall be present if the E-CID-ReportCharacteristics IE is set to “periodic” –-

{ ID id-E-CID-MeasurementQuantities		CRITICALITY reject	TYPE E-CID-MeasurementQuantities	PRESENCE mandatory}|

{ ID id-PosMeasurementPeriodicityNR-AoA	CRITICALITY reject	TYPE PosMeasurementPeriodicityNR-AoA		PRESENCE conditional},

-- The IE shall be present if the E-CID-ReportCharacteristics IE is set to “periodic” and the E-CID-MeasurementQuantities-Item IE in the E-CID-MeasurementQuantities IE is set to the value "angleOfArrivalNR"--

...

}

-- **************************************************************

--

##### -- E-CID Measurement Initiation Response

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

##### -- E-CID Measurement Initiation Failure

--

-- **************************************************************

E-CIDMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementInitiationFailure-IEs}},

...

}

E-CIDMeasurementInitiationFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT FAILURE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Failure Indication

--

-- **************************************************************

E-CIDMeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementFailureIndication-IEs}},

...

}

E-CIDMeasurementFailureIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID		PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID		PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT REPORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Report

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE mandatory }|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT TERMINATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Termination Command

--

-- **************************************************************

E-CIDMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementTerminationCommand-IEs}},

...

}

E-CIDMeasurementTerminationCommand-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP REQUEST

--

-- **************************************************************

BroadcastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupRequestIEs} },

...

}

BroadcastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE	GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID					PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea					CRITICALITY reject 	TYPE	MBS-ServiceArea					PRESENCE optional   }|

{ ID id-MBS-CUtoDURRCInformation			CRITICALITY reject	TYPE	MBS-CUtoDURRCInformation		PRESENCE mandatory	}|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI							PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetup-List		CRITICALITY reject	TYPE	BroadcastMRBs-ToBeSetup-List	PRESENCE mandatory	}|

{ ID id-SupportedUETypeList				CRITICALITY ignore	TYPE	SupportedUETypeList				PRESENCE	optional		}|

{ ID id-AssociatedSessionID				CRITICALITY ignore	TYPE	AssociatedSessionID				PRESENCE optional		},

...

}

BroadcastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetup-ItemIEs} }

BroadcastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	BroadcastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

BroadcastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupResponseIEs} },

...

}

BroadcastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-Setup-List			CRITICALITY reject TYPE BroadcastMRBs-Setup-List				PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeSetup-List	PRESENCE optional	}|

{ ID id-BroadcastAreaScope					CRITICALITY ignore TYPE BroadcastAreaScope						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics					PRESENCE optional	},

...

}

BroadcastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-Setup-ItemIEs} }

BroadcastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeSetup-ItemIEs} }

BroadcastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-Setup-Item			CRITICALITY reject	TYPE BroadcastMRBs-Setup-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeSetup-Item	CRITICALITY ignore	TYPE BroadcastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},	...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP FAILURE

--

-- **************************************************************

BroadcastContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupFailureIEs} },

...

}

BroadcastContextSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT RELEASE COMMAND

--

-- **************************************************************

BroadcastContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextReleaseCommandIEs} },

...

}

BroadcastContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT RELEASE COMPLETE

--

-- **************************************************************

BroadcastContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextReleaseCompleteIEs} },

...

}

BroadcastContextReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT RELEASE REQUEST ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT RELEASE REQUEST

--

-- **************************************************************

BroadcastContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ BroadcastContextReleaseRequestIEs}},

...

}

BroadcastContextReleaseRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

BroadcastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationRequestIEs} },

...

}

BroadcastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional		}|

{ ID id-MBS-CUtoDURRCInformation				CRITICALITY reject	TYPE MBS-CUtoDURRCInformation				PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeSetupMod-List		PRESENCE optional	}|

{ ID id-BroadcastMRBs-ToBeModified-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeModified-List		PRESENCE optional	}|

{ ID id-BroadcastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeReleased-List		PRESENCE optional	}|

{ ID id-SupportedUETypeList					CRITICALITY ignore	TYPE SupportedUETypeList					PRESENCE	optional	},

...

}

BroadcastMRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetupMod-ItemIEs} }

BroadcastMRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeModified-ItemIEs} }

BroadcastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeReleased-ItemIEs} }

BroadcastMRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeModified-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

BroadcastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationResponseIEs} },

...

}

BroadcastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory}|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory}|

{ ID id-BroadcastMRBs-SetupMod-List				CRITICALITY reject TYPE BroadcastMRBs-SetupMod-List					PRESENCE optional}|

{ ID id-BroadcastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeSetupMod-List	PRESENCE optional}|

{ ID id-BroadcastMRBs-Modified-List				CRITICALITY reject TYPE BroadcastMRBs-Modified-List					PRESENCE optional}|

{ ID id-BroadcastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeModified-List	PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-BroadcastAreaScope						CRITICALITY ignore TYPE BroadcastAreaScope								PRESENCE optional},

...

}

BroadcastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-SetupMod-ItemIEs} }

BroadcastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeSetupMod-ItemIEs} }

BroadcastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-Modified-ItemIEs } }

BroadcastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeModified-ItemIEs} }

BroadcastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE BroadcastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE BroadcastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-Modified-Item				CRITICALITY		reject	TYPE BroadcastMRBs-Modified-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE BroadcastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

BroadcastContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationFailureIEs} },

...

}

BroadcastContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BROADCAST TRANSPORT RESOURCE REQUEST ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST TRANSPORT RESOURCE REQUEST

--

-- **************************************************************

BroadcastTransportResourceRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ BroadcastTransportResourceRequestIEs}},

...

}

BroadcastTransportResourceRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Broadcast-MRBs-Transport-Request-List		CRITICALITY reject TYPE Broadcast-MRBs-Transport-Request-List	PRESENCE optional	},

...

}

Broadcast-MRBs-Transport-Request-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { Broadcast-MRBs-Transport-Request-ItemIEs} }

Broadcast-MRBs-Transport-Request-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Broadcast-MRBs-Transport-Request-Item			CRITICALITY reject	TYPE Broadcast-MRBs-Transport-Request-Item			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- Multicast Group Paging ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Multicast Group Paging

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastGroupPagingIEs}},

...

}

MulticastGroupPagingIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID					CRITICALITY reject	TYPE MBS-Session-ID							PRESENCE mandatory	}|

{ ID id-UEIdentity-List-For-Paging-List	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-List		PRESENCE optional		}|

{ ID id-MC-PagingCell-List				CRITICALITY ignore	TYPE MC-PagingCell-list						PRESENCE optional		}|

{ ID id-IndicationMCInactiveReception		CRITICALITY ignore	TYPE IndicationMCInactiveReception		PRESENCE optional		},

...

}

UEIdentity-List-For-Paging-List	 ::= SEQUENCE (SIZE(1.. maxnoofUEIDforPaging)) OF ProtocolIE-SingleContainer { { UEIdentity-List-For-Paging-ItemIEs } }

UEIdentity-List-For-Paging-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentity-List-For-Paging-Item	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-Item 			PRESENCE optional }	,

...

}

MC-PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF ProtocolIE-SingleContainer { { MC-PagingCell-ItemIEs } }

MC-PagingCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MC-PagingCell-Item		CRITICALITY ignore	TYPE MC-PagingCell-Item			PRESENCE mandatory}	,

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP REQUEST

--

-- **************************************************************

MulticastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupRequestIEs}},

...

}

MulticastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE	GNB-CU-MBS-F1AP-ID								PRESENCE mandatory  }|

{ ID id-MBS-Session-ID						CRITICALITY reject	TYPE	MBS-Session-ID									PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE	MBS-ServiceArea									PRESENCE optional   }|

{ ID id-SNSSAI								CRITICALITY reject	TYPE	SNSSAI											PRESENCE mandatory  }|

{ ID id-MulticastMRBs-ToBeSetup-List			CRITICALITY reject	TYPE	MulticastMRBs-ToBeSetup-List					PRESENCE mandatory  }|

{ ID id-MulticastCU2DURRCInfo					CRITICALITY reject	TYPE	MulticastCU2DURRCInfo							PRESENCE optional   }|

{ ID id-MBSMulticastSessionReceptionState		CRITICALITY reject	TYPE	MBSMulticastSessionReceptionState				PRESENCE optional   },

...

}

MulticastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetup-ItemIEs} }

MulticastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	MulticastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

MulticastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupResponseIEs}},

...

}

MulticastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MulticastMRBs-Setup-List			CRITICALITY reject TYPE MulticastMRBs-Setup-List					PRESENCE mandatory	}|

{ ID id-MulticastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetup-List 	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-MulticastDU2CURRCInfo				CRITICALITY reject TYPE MulticastDU2CURRCInfo						PRESENCE optional	},

...

}

MulticastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Setup-ItemIEs} }

MulticastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetup-ItemIEs} }

MulticastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Setup-Item			CRITICALITY reject	TYPE MulticastMRBs-Setup-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE MulticastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP FAILURE

--

-- **************************************************************

MulticastContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupFailureIEs}},

...

}

MulticastContextSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY ignore	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE optional	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE COMMAND

--

-- **************************************************************

MulticastContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseCommandIEs}},

...

}

MulticastContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE COMPLETE

--

-- **************************************************************

MulticastContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseCompleteIEs}},

...

}

MulticastContextReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT RELEASE REQUEST ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE REQUEST

--

-- **************************************************************

MulticastContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseRequestIEs}},

...

}

MulticastContextReleaseRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

MulticastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationRequestIEs}},

...

}

MulticastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea							PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-List		PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeModified-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-List		PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-List		PRESENCE optional  }|

{ ID id-MulticastCU2DURRCInfo					CRITICALITY reject	TYPE	MulticastCU2DURRCInfo					PRESENCE optional  }|

{ ID id-MBSMulticastSessionReceptionState		CRITICALITY reject	TYPE MBSMulticastSessionReceptionState		PRESENCE optional  },

...

}

MulticastMRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetupMod-ItemIEs} }

MulticastMRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeModified-ItemIEs} }

MulticastMRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeModified-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeReleased-ItemIEs} }

MulticastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

MulticastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationResponseIEs}},

...

}

MulticastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-MulticastMRBs-SetupMod-List			CRITICALITY reject TYPE MulticastMRBs-SetupMod-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-Modified-List			CRITICALITY reject TYPE MulticastMRBs-Modified-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeModified-List	PRESENCE optional  }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional  }|

{ ID id-MulticastDU2CURRCInfo					CRITICALITY reject TYPE MulticastDU2CURRCInfo							PRESENCE optional	},

...

}

MulticastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-SetupMod-ItemIEs} }

MulticastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE MulticastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetupMod-ItemIEs} }

MulticastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE MulticastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Modified-ItemIEs } }

MulticastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Modified-Item				CRITICALITY		reject	TYPE MulticastMRBs-Modified-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeModified-ItemIEs} }

MulticastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE MulticastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

MulticastContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationFailureIEs}},

...

}

MulticastContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT NOTIFICATION INDICATION

--

-- **************************************************************

MulticastContextNotificationIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastContextNotificationIndicationIEs}},

...

}

MulticastContextNotificationIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MulticastDU2CURRCInfo				CRITICALITY reject	TYPE MulticastDU2CURRCInfo						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT NOTIFICATION CONFIRM

--

-- **************************************************************

MulticastContextNotificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastContextNotificationConfirmIEs}},

...

}

MulticastContextNotificationConfirmIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT NOTIFICATION REFUSE

--

-- **************************************************************

MulticastContextNotificationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastContextNotificationRefuseIEs}},

...

}

MulticastContextNotificationRefuseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- MULTICAST COMMON CONFIGURATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST COMMON CONFIGURATION REQUEST

--

-- **************************************************************

MulticastCommonConfigurationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastCommonConfigurationRequestIEs}},

...

}

MulticastCommonConfigurationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID											PRESENCE mandatory	}|

{ ID id-MulticastCU2DUCommonRRCInfo				CRITICALITY reject	TYPE MulticastCU2DUCommonRRCInfo						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST COMMON CONFIGURATION RESPONSE

--

-- **************************************************************

MulticastCommonConfigurationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastCommonConfigurationResponseIEs}},

...

}

MulticastCommonConfigurationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST COMMON CONFIGURATION REFUSE

--

-- **************************************************************

MulticastCommonConfigurationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastCommonConfigurationRefuseIEs}},

...

}

MulticastCommonConfigurationRefuseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- MULTICAST DISTRIBUTION SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

MulticastDistributionSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionSetupRequestIEs}},

...

}

MulticastDistributionSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-MulticastF1UContext-ToBeSetup-List	CRITICALITY reject	TYPE MulticastF1UContext-ToBeSetup-List	PRESENCE mandatory	},

...

}

MulticastF1UContext-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF

ProtocolIE-SingleContainer { { MulticastF1UContext-ToBeSetup-ItemIEs} }

MulticastF1UContext-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastF1UContext-ToBeSetup-Item				CRITICALITY		reject	TYPE MulticastF1UContext-ToBeSetup-Item			PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION SETUP RESPONSE

--

-- **************************************************************

MulticastDistributionSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionSetupResponseIEs}},

...

}

MulticastDistributionSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID									PRESENCE mandatory}|

{ ID id-gNB-DU-MBS-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID									PRESENCE mandatory}|

{ ID id-MBSMulticastF1UContextDescriptor			CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor				PRESENCE mandatory}|

{ ID id-MulticastF1UContext-Setup-List				CRITICALITY reject	TYPE MulticastF1UContext-Setup-List					PRESENCE mandatory}|

{ ID id-MulticastF1UContext-FailedToBeSetup-List	CRITICALITY ignore	TYPE MulticastF1UContext-FailedToBeSetup-List		PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional }|

{ ID id-MulticastF1UContextReferenceCU				CRITICALITY reject	TYPE MulticastF1UContextReferenceCU					PRESENCE mandatory},

...

}

MulticastF1UContext-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastF1UContext-Setup-ItemIEs} }

MulticastF1UContext-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastF1UContext-Setup-Item				CRITICALITY		reject	TYPE MulticastF1UContext-Setup-Item			PRESENCE mandatory},

...

}

MulticastF1UContext-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF 
																ProtocolIE-SingleContainer { { MulticastF1UContext-FailedToBeSetup-ItemIEs} }

MulticastF1UContext-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastF1UContext-FailedToBeSetup-Item	CRITICALITY	 ignore	TYPE MulticastF1UContext-FailedToBeSetup-Item	 PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION SETUP FAILURE

--

-- **************************************************************

MulticastDistributionSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionSetupFailureIEs}},

...

}

MulticastDistributionSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY ignore	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE optional	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST DISTRIBUTION RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION RELEASE COMMAND

--

-- **************************************************************

MulticastDistributionReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionReleaseCommandIEs}},

...

}

MulticastDistributionReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION RELEASE COMPLETE

--

-- **************************************************************

MulticastDistributionReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionReleaseCompleteIEs}},

...

}

MulticastDistributionReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- PDC MEASUREMENT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Initiation Request

--

-- **************************************************************

PDCMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PDCMeasurementInitiationRequest-IEs}},

...

}

PDCMeasurementInitiationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID				CRITICALITY reject	TYPE RAN-UE-PDC-MeasID					PRESENCE mandatory	}|

{ ID id-PDCReportType					CRITICALITY reject	TYPE PDCReportType						PRESENCE mandatory	}|

{ ID id-PDCMeasurementPeriodicity		CRITICALITY reject	TYPE PDCMeasurementPeriodicity			PRESENCE conditional	}|

-- The above IE shall be present if the PDCReportType IE is set to “periodic” –-

{ ID id-PDCMeasurementQuantities		CRITICALITY reject	TYPE PDCMeasurementQuantities			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PDC Measurement Initiation Response

--

-- **************************************************************

PDCMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PDCMeasurementInitiationResponse-IEs}},

...

}

PDCMeasurementInitiationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY reject	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	}|

{ ID id-PDCMeasurementResult		CRITICALITY ignore	TYPE PDCMeasurementResult		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PDC Measurement Initiation Failure

--

-- **************************************************************

PDCMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PDCMeasurementInitiationFailure-IEs}},

...

}

PDCMeasurementInitiationFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY ignore	TYPE RAN-UE-PDC-MeasID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- PDC MEASUREMENT REPORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Report

--

-- **************************************************************

PDCMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PDCMeasurementReport-IEs}},

...

}

PDCMeasurementReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY reject	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	}|

{ ID id-PDCMeasurementResult		CRITICALITY ignore	TYPE PDCMeasurementResult		PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- PDC MEASUREMENT TERMINATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Termination

--

-- **************************************************************

PDCMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PDCMeasurementTerminationCommand-IEs} },

...

}

PDCMeasurementTerminationCommand-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY	ignore	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PDC MEASUREMENT FAILURE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Failure Indication

--

-- **************************************************************

PDCMeasurementFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PDCMeasurementFailureIndication-IEs} },

...

}

PDCMeasurementFailureIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY ignore	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PPS CONFIGURATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PRS CONFIGURATION REQUEST

--

-- **************************************************************

PRSConfigurationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PRSConfigurationRequest-IEs}},

...

}

PRSConfigurationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-PRSConfigRequestType	CRITICALITY reject	TYPE PRSConfigRequestType				PRESENCE mandatory }|

{ ID id-PRSTRPList				CRITICALITY ignore	TYPE PRSTRPList							PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- PRS CONFIGURATION RESPONSE

--

-- **************************************************************

PRSConfigurationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ PRSConfigurationResponse-IEs}},

...

}

PRSConfigurationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory}|

{ ID id-PRSTransmissionTRPList	CRITICALITY ignore	TYPE PRSTransmissionTRPList			PRESENCE optional}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PRS CONFIGURATION FAILURE

--

-- **************************************************************

PRSConfigurationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ PRSConfigurationFailure-IEs}},

...

}

PRSConfigurationFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory}|

{ ID id-Cause	CRITICALITY ignore	TYPE Cause	PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT PRECONFIGURATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Preconfiguration Required

--

-- **************************************************************

MeasurementPreconfigurationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ MeasurementPreconfigurationRequired-IEs}},

...

}

MeasurementPreconfigurationRequired-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID	PRESENCE mandatory}|

{ ID id-gNB-DU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID	PRESENCE mandatory}|

{ ID id-TRP-PRS-Info-List	CRITICALITY ignore	TYPE TRP-PRS-Info-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- Positioning Preconfiguration Confirm

--

-- **************************************************************

MeasurementPreconfigurationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementPreconfigurationConfirm-IEs} },

...

}

MeasurementPreconfigurationConfirm-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-PosMeasGapPreConfigList		CRITICALITY ignore	TYPE PosMeasGapPreConfigList		PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional },

...

}

-- **************************************************************

--

##### -- Positioning Preconfiguration Refuse

--

-- **************************************************************

MeasurementPreconfigurationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementPreconfigurationRefuse-IEs} },

...

}

MeasurementPreconfigurationRefuse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- MEASUREMENT ACTIVATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Measurement Activation

--

-- **************************************************************

MeasurementActivation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementActivation-IEs} },

...

}

MeasurementActivation-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-ActivationRequestType		CRITICALITY reject	TYPE ActivationRequestType 			PRESENCE mandatory}|

{ ID id-PRS-Measurement-Info-List	CRITICALITY ignore	TYPE PRS-Measurement-Info-List		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- QOE INFORMATION TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- QoE Information Transfer

--

-- **************************************************************

QoEInformationTransfer ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{QoEInformationTransfer-IEs}},

...

}

QoEInformationTransfer-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-QoEInformation				CRITICALITY ignore	TYPE QoEInformation						PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- POSITIONING SYSTEM INFORMATION DELIVERY ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning System information Delivery Command

--

-- **************************************************************

PosSystemInformationDeliveryCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PosSystemInformationDeliveryCommandIEs}},

...

}

PosSystemInformationDeliveryCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-PosSItypeList			CRITICALITY reject	TYPE PosSItypeList					PRESENCE mandatory	}|

{ ID id-ConfirmedUEID 			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- DU-CU CELL SWITCH NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Cell Switch Notification

--

-- **************************************************************

DUCUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCUCellSwitchNotificationIEs}},

...

}

DUCUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 CRITICALITY ignore	TYPE LTMCellSwitchInformation	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- CU-DU CELL SWITCH NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU Cell Switch Notification

--

-- **************************************************************

CUDUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUCellSwitchNotificationIEs}},

...

}

CUDUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 CRITICALITY ignore	TYPE LTMCellSwitchInformation	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- DU-CU TA INFORMATION TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU TA Information Transfer

--

-- **************************************************************

DUCUTAInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCUTAInformationTransferIEs}},

...

}

DUCUTAInformationTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-DUtoCUTAInformation-List		CRITICALITY ignore	TYPE DUtoCUTAInformation-List				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- CU-DU TA INFORMATION TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU TA Information Transfer

--

-- **************************************************************

CUDUTAInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUTAInformationTransferIEs}},

...

}

CUDUTAInformationTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-CUtoDUTAInformation-List		CRITICALITY ignore	TYPE CUtoDUTAInformation-List				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- QOE INFORMATION TRANSFER CONTROL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- QoE Information Transfer Control

--

-- **************************************************************

QoEInformationTransferControl ::= SEQUENCE {

protocolIEs						ProtocolIE-Container {{QoEInformationTransferControl-IEs}},

...

}

QoEInformationTransferControl-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-DeactivationIndication				CRITICALITY ignore	TYPE DeactivationIndication						PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- RACH Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- RACH Indication

--

-- **************************************************************

RachIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{ RachIndication-IEs}},

...

}

RachIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-RAReportIndicationList				CRITICALITY reject	TYPE RAReportIndicationList						PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Timing Synchronisation Status Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS REQUEST

--

-- **************************************************************

TimingSynchronisationStatusRequest::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TimingSynchronisationStatusRequest-IEs}},

...

}

TimingSynchronisationStatusRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-RANTSSRequestType				CRITICALITY reject	TYPE RANTSSRequestType							PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS RESPONSE

--

-- **************************************************************

TimingSynchronisationStatusResponse::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TimingSynchronisationStatusResponse-IEs}},

...

}

TimingSynchronisationStatusResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS FAILURE

--

-- **************************************************************

TimingSynchronisationStatusFailure::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TimingSynchronisationStatusFailure-IEs}},

...

}

TimingSynchronisationStatusFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Timing Synchronisation Status Reporting Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS REPORT

--

-- **************************************************************

TimingSynchronisationStatusReport::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ TimingSynchronisationStatusReport-IEs}},

...

}

TimingSynchronisationStatusReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-RANTimingSynchronisationStatusInfo				CRITICALITY ignore	TYPE RANTimingSynchronisationStatusInfo							PRESENCE mandatory },

...

}

-- **************************************************************

--

#### --  DU-CU Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Access And Mobility Indication

--

-- **************************************************************

DUCUAccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DUCUAccessAndMobilityIndicationIEs} },

...

}

DUCUAccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-DLLBTFailureInformationList				CRITICALITY ignore	TYPE DLLBTFailureInformationList	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DU-CU DYNAMIC RESOURCE ALLOCATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Dynamic Resource Allocation Request

--

-- **************************************************************

DUCUDynamicResourceAllocationRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCUDynamicResourceAllocationRequestIEs}},

...

}

DUCUDynamicResourceAllocationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-DynamicResourceAllocationRequest	 CRITICALITY ignore	TYPE DynamicResourceAllocationRequest	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DU-CU Dynamic Resource Allocation Response

--

-- **************************************************************

DUCUDynamicResourceAllocationResponse::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCUDynamicResourceAllocationResponseIEs}},

...

}

DUCUDynamicResourceAllocationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-DynamicResourceAllocationResponse	 CRITICALITY ignore	TYPE DynamicResourceAllocationResponse	PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- CU-DU DYNAMIC RESOURCE ALLOCATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU Dynamic Resource Allocation Request

--

-- **************************************************************

CUDUDynamicResourceAllocationRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUDynamicResourceAllocationRequestIEs}},

...

}

CUDUDynamicResourceAllocationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-DynamicResourceAllocationRequest	 CRITICALITY ignore	TYPE DynamicResourceAllocationRequest	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- CU-DU Dynamic Resource Allocation Response

--

-- **************************************************************

CUDUDynamicResourceAllocationResponse::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUDynamicResourceAllocationResponseIEs}},

...

}

CUDUDynamicResourceAllocationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-DynamicResourceAllocationResponse	 CRITICALITY ignore	TYPE DynamicResourceAllocationResponse	PRESENCE mandatory	},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Coverage-Modification-Cause,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-areaScope,

id-IntendedTDD-DL-ULConfig,

id-QosMonitoringRequest,

id-BHInfo,

id-IAB-Info-IAB-DU,

id-IAB-Info-IAB-donor-CU,

id-IAB-Barred,

id-SIB12-message,

id-SIB13-message,

id-SIB14-message,

id-UEAssistanceInformationEUTRA,

id-SL-PHY-MAC-RLC-Config,

id-SL-ConfigDedicatedEUTRA-Info,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRPRACHConfig,

id-TDD-UL-DLConfigCommonNR,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-TSCTrafficCharacteristics,

id-AdditionalPDCPDuplicationTNL-List,

id-RLCDuplicationInformation,

id-AdditionalDuplicationIndication,

id-mdtConfiguration,

id-TraceCollectionEntityURI,

id-NID,

id-NPNSupportInfo,

id-NPNBroadcastInformation,

id-AvailableSNPN-ID-List,

id-SIB10-message,

id-RequestedP-MaxFR2,

id-DLCarrierList,

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-Supported-MBS-FSA-ID-List,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-MBS-Broadcast-NeighbourCellList,

id-PDCPTerminatingNodeDLTNLAddrInfo,

id-ENBDLTNLAddress,

id-PRS-Resource-ID,

id-LocationMeasurementInformation,

id-SliceRadioResourceStatus,

id-CompositeAvailableCapacity-SUL,

id-NR-U,

id-NR-U-Channel-List,

id-MIMOPRBusageInformation,

id-IngressNonF1terminatingTopologyIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-rBSetConfiguration,

id-frequency-Domain-HSNA-Configuration-List,

id-child-IAB-Nodes-NA-Resource-List,

id-Parent-IAB-Nodes-NA-Resource-Configuration-List,

id-uL-FreqInfo,

id-uL-Transmission-Bandwidth,

id-dL-FreqInfo,

id-dL-Transmission-Bandwidth,

id-uL-NR-Carrier-List,

id-dL-NR-Carrier-List,

id-nRFreqInfo,

id-transmission-Bandwidth,

id-nR-Carrier-List,

id-permutation,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-SurvivalTime,

id-PDCMeasurementQuantities-Item,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoAInformation,

id-ARPLocationInfo,

id-ARP-ID,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRPRx-TEGInformation,

id-TRPBeamAntennaInformation,

id-Redcap-Bcast-Information,

id-NR-TADV,

id-SDT-MAC-PHY-CG-Config,

id-CG-SDTindicatorSetup,

id-CG-SDTindicatorMod,

id-SDTRLCBearerConfiguration,

id-SRBMappingInfo,

id-DRBMappingInfo,

id-LastUsedCellIndication,

id-Recommended-SSBs-List,

id-SSBs-withinTheCell-tobe-Activated-List,

id-SIB17-message,

id-MUSIM-GapConfig,

id-SIB20-message,

id-pathPower,

id-DU-RX-MT-RX-Extend,

id-DU-TX-MT-TX-Extend,

id-DU-RX-MT-TX-Extend,

id-DU-TX-MT-RX-Extend,

id-TAINSAGSupportList,

id-SL-RLC-ChannelToAddModList,

id-SIB15-message,

id-InterFrequencyConfig-NoGap,

id-MBSInterestIndication,

id-L571Info,

id-L1151Info,

id-SCS-480,

id-SCS-960,

id-SRSPortIndex,

id-PEISubgroupingSupportIndication,

id-NeedForGapsInfoNR,

id-NeedForGapNCSGInfoNR,

id-NeedForGapNCSGInfoEUTRA,

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-Preconfigured-measurement-GAP-Request,

id-BWP-Id,

id-ExtendedResourceSymbolOffset,

id-MusimCapabilityRestrictionIndication,

id-duplicationIndication,

id-dRB-List,

id-ChannelOccupancyTimePercentageUL,

id-RadioResourceStatusNR-U,

id-FiveG-ProSeLayer2Multipath,

id-FiveG-ProSeLayer2UEtoUERelay,

id-FiveG-ProSeLayer2UEtoUERemote,

id-TSCTrafficCharacteristicsFeedback,

id-RANfeedbacktype,

id-Mobile-TRP-LocationInformation,

id-Mobile-IAB-MT-UE-ID,

id-MobileAccessPointLocation,

id-SIB24-message,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-ERedcap-Bcast-Information,

id-NeedForInterruptionInfoNR,

id-LTMCells-ToBeReleased-Item,

id-SCPAC-Request,

id-MobileIAB-Barred,

id-F1UTunnelNotEstablished,

id-S-CPACLowerLayerReferenceConfigRequest,

id-MusimCandidateBandList,

id-PSIbasedSDUdiscardUL,

id-SIB22-message,

id-U2URLCChannelQoS,

id-SL-PHY-MAC-RLC-ConfigExt,

id-UL-RSCP,

id-BW-Aggregation-Request-Indication,

id-ReportingGranularitykminus1,

id-ReportingGranularitykminus1additionalpath,

id-ReportingGranularitykminus2,

id-ReportingGranularitykminus2additionalpath,

id-ReportingGranularitykminus3,

id-ReportingGranularitykminus3additionalpath,

id-ReportingGranularitykminus4,

id-ReportingGranularitykminus4additionalpath,

id-ReportingGranularitykminus5,

id-ReportingGranularitykminus5additionalpath,

id-ReportingGranularitykminus6,

id-ReportingGranularitykminus6additionalpath,

id-TimingReportingGranularityFactorExtended,

id-PosValidityAreaCellList,

id-SymbolIndex,

id-AggregatedPosSRSResourceIDList,

id-PhaseQuality,

id-PRSBandwidthAggregationRequestIndication,

id-AggregatedPRSResourceSetList,

id-MeasuredFrequencyHops,

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxCellingNBDU,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofslots,

maxnoofNonUPTrafficMappings,

maxnoofServingCells,

maxnoofServedCellsIAB,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofSymbols,

maxnoofDUFSlots,

maxnoofHSNASlots,

maxnoofEgressLinks,

maxnoofMappingEntries,

maxnoofDSInfo,

maxnoofQoSParaSets,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofPhysicalResourceBlocks-1,

maxnoofPRACHconfigs,

maxnoofRAReports,

maxnoofRLFReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationState,

maxnoofCHOcells,

maxnoofMDTPLMNs,

maxnoofCAGsupported,

maxnoofNIDsupported,

maxnoofExtSliceItems,

maxnoofPosMeas,

maxnoofTRPInfoTypes,

maxnoofSRSTriggerStates,

maxnoofSpatialRelations,

maxnoBcastCell,

maxnoofTRPs,

maxnoofAngleInfo,

maxnooflcs-gcs-translation,

maxnoofPath,

maxnoofMeasE-CID,

maxnoofSSBs,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofSuccessfulHOReports,

maxnoofNR-UChannelIDs,

maxServedCellforSON,

maxNeighbourCellforSON,

maxAffectedCells,

maxnoofMBSQoSFlows,

maxnoofMBSFSAs,

maxnoofMBSAreaSessionIDs,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofCellsforMBS,

maxnoofIABCongInd,

maxnoofBHRLCChannels,

maxnoofTLAsIAB,

maxnoofRBsetsPerCell,

maxnoofRBsetsPerCell-1,

maxnoofNeighbourNodeCellsIAB,

maxnoofMeasPDC,

maxnoARPs,

maxnoofULAoAs,

maxNoPathExtended,

maxnoTRPTEGs,

maxFreqLayers,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

maxnoofPRSTRPs,

maxnoofQoEInformation,

maxnoofUuRLCChannels,

maxnoofPC5RLCChannels,

maxnoofSMBRValues,

maxnoofMBSSessionsofUE,

maxnoofSLdestinations,

maxnoofNSAGs,

maxnoofSDTBearers,

maxnoofPosSITypes,

maxnoofMRBs,

maxNrofBWPs,

maxnoofUETypes,

maxnoofLTMCells,

maxnoofJointorDLTCIStates,

maxnoofULTCIStates,

maxnoofLTMgNB-DUs,

maxnoofTAList,

maxnoofDRBs,

maxnoofUEsInQMCTransferControlMessage,

maxnoofUEsforRAReportIndications,

maxnoofSuccessfulPSCellChangeReports,

maxnoofPeriodicities,

maxnoofThresholdMBS-1,

maxMBSSessionsinSessionInfoList,

maxnoofLBTFailureInformation,

maxnoofRSPPQoSFlows,

maxnoVACell,

maxnoAggregatedSRS-Resources,

maxnoAggregatedPosSRSResourceSets,

maxnoAggregatedPosPRSResourceSets,

maxnoofTimeWindowSRS,

maxnoofTimeWindowMea,

maxnoPreconfiguredSRS,

maxnoHopsMinusOne

FROM F1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM F1AP-CommonDataTypes

ProtocolExtensionContainer{},

F1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

#### -- A

AbortTransmission ::= CHOICE {

sRSResourceSetID		SRSResourceSetID,

releaseALL				NULL,

choice-extension		ProtocolIE-SingleContainer { { AbortTransmission-ExtIEs } }

}

AbortTransmission-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AccessPointPosition ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

latitude					INTEGER (0..8388607),

longitude					INTEGER (-8388608..8388607),

directionOfAltitude			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),

uncertaintySemi-major		INTEGER (0..127),

uncertaintySemi-minor		INTEGER (0..127),

orientationOfMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { AccessPointPosition-ExtIEs} } OPTIONAL

}

AccessPointPosition-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Activated-Cells-Mapping-List-Item	::= SEQUENCE {

nRCGIforTargetLogicalDU						NRCGI,

nRCGIforSourceLogicalDU						NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Activated-Cells-Mapping-List-ItemExtIEs } }	OPTIONAL,

...

}

Activated-Cells-Mapping-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Activated-Cells-to-be-Updated-List ::= SEQUENCE (SIZE(1..maxnoofServedCellsIAB)) OF Activated-Cells-to-be-Updated-List-Item

Activated-Cells-to-be-Updated-List-Item ::=	SEQUENCE{

nRCGI								NRCGI,

iAB-DU-Cell-Resource-Configuration-Mode-Info	IAB-DU-Cell-Resource-Configuration-Mode-Info,

iE-Extensions						ProtocolExtensionContainer { { Activated-Cells-to-be-Updated-List-Item-ExtIEs} } OPTIONAL

}

Activated-Cells-to-be-Updated-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ActivationRequestType ::= ENUMERATED {activate, deactivate, ...}

ActiveULBWP  ::= SEQUENCE {

locationAndBandwidth		INTEGER (0..37949,...),

subcarrierSpacing           ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

cyclicPrefix				ENUMERATED {normal, extended},

txDirectCurrentLocation		INTEGER (0..3301,...),

shift7dot5kHz				ENUMERATED {true, ...} OPTIONAL,

sRSConfig					SRSConfig,

iE-Extensions					ProtocolExtensionContainer { { ActiveULBWP-ExtIEs} } OPTIONAL

}

ActiveULBWP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalDuplicationIndication ::= ENUMERATED {

three,

four,

...

}

AdditionalPath-List::= SEQUENCE (SIZE(1..maxnoofPath)) OF AdditionalPath-Item

AdditionalPath-Item ::=SEQUENCE {

relativePathDelay	RelativePathDelay,

pathQuality			TRPMeasurementQuality 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalPath-Item-ExtIEs } }	OPTIONAL

}

AdditionalPath-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MultipleULAoA	CRITICALITY ignore EXTENSION MultipleULAoA		PRESENCE optional}|

{ ID id-pathPower		CRITICALITY ignore EXTENSION UL-SRS-RSRPP		PRESENCE optional},

...

}

ExtendedAdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPathExtended)) OF ExtendedAdditionalPathList-Item

ExtendedAdditionalPathList-Item ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TRPMeasurementQuality	OPTIONAL,

multipleULAoA		MultipleULAoA  			OPTIONAL,

pathPower			UL-SRS-RSRPP			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAdditionalPathList-Item-ExtIEs} } OPTIONAL,

...

}

ExtendedAdditionalPathList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalPDCPDuplicationTNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF AdditionalPDCPDuplicationTNL-Item

AdditionalPDCPDuplicationTNL-Item ::=SEQUENCE {

additionalPDCPDuplicationUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { AdditionalPDCPDuplicationTNL-ItemExtIEs } }	OPTIONAL,

...

}

AdditionalPDCPDuplicationTNL-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BHInfo		CRITICALITY ignore	EXTENSION BHInfo		PRESENCE optional	},

...

}

AdditionalSIBMessageList ::= SEQUENCE (SIZE(1..maxnoofAdditionalSIBs)) OF AdditionalSIBMessageList-Item

AdditionalSIBMessageList-Item ::= SEQUENCE {

additionalSIB			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { AdditionalSIBMessageList-Item-ExtIEs} } OPTIONAL

}

AdditionalSIBMessageList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AffectedCellsAndBeams-List ::= SEQUENCE (SIZE (1.. maxAffectedCells)) OF AffectedCellsAndBeams-Item

AffectedCellsAndBeams-Item::= SEQUENCE {

nRCGI					NRCGI,

affectedSSB-List		AffectedSSB-List OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AffectedCellsAndBeams-Item-ExtIEs} } OPTIONAL,

...

}

AffectedCellsAndBeams-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AffectedSSB-List::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF AffectedSSB-Item

AffectedSSB-Item::= SEQUENCE {

sSB-Index	INTEGER(0..63),

iE-Extensions		ProtocolExtensionContainer { { AffectedSSB-Item-ExtIEs} } OPTIONAL,

...

}

AffectedSSB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggregatedPosSRSResourceIDList ::= SEQUENCE (SIZE(1..maxnoAggregatedSRS-Resources)) OF Aggregated-PosSRS-Resource-ID-Item

Aggregated-PosSRS-Resource-ID-Item ::= SEQUENCE {

positioningSRS		SRSPosResourceID,

iE-Extensions		ProtocolExtensionContainer { { Aggregated-PosSRS-Resource-ID-Item-ExtIEs} } OPTIONAL,

...

}

Aggregated-PosSRS-Resource-ID-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggregatedPosSRSResourceSetList ::= SEQUENCE (SIZE(1..maxnoAggregatedPosSRSResourceSets)) OF AggregatedPosSRSResourceSet-Item

AggregatedPosSRSResourceSet-Item ::= SEQUENCE {

pointA							INTEGER (0..3279165),

nRPCI							NRPCI				OPTIONAL,

posSRSResourceSetID				INTEGER(0..15),

iE-Extensions					ProtocolExtensionContainer { { AggregatedPosSRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

AggregatedPosSRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggregatedPRSResourceSetList ::= SEQUENCE (SIZE(1..maxnoAggregatedPosPRSResourceSets)) OF AggregatedPRSResourceSet-Item

AggregatedPRSResourceSet-Item ::= SEQUENCE {

pointA							INTEGER (0..3279165),

pRSResourceSetID				PRS-Resource-Set-ID,

iE-Extensions					ProtocolExtensionContainer { { AggregatedPRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

AggregatedPRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggressorCellList ::= SEQUENCE (SIZE(1..maxCellingNBDU)) OF AggressorCellList-Item

AggressorCellList-Item ::= SEQUENCE {

aggressorCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { AggressorCellList-Item-ExtIEs } }		OPTIONAL

}

AggressorCellList-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggressorgNBSetID ::= SEQUENCE {

aggressorgNBSetID		GNBSetID,

iE-Extensions	ProtocolExtensionContainer { { AggressorgNBSetID-ExtIEs } }	OPTIONAL

}

AggressorgNBSetID-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AngleMeasurementQuality ::= SEQUENCE {

azimuthQuality	INTEGER(0..255),

zenithQuality	INTEGER(0..255) OPTIONAL,

resolution		ENUMERATED{deg0dot1,...},

iE-Extensions	ProtocolExtensionContainer { { AngleMeasurementQuality-ExtIEs } }	OPTIONAL

}

AngleMeasurementQuality-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoofSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (1..3)

Associated-SCell-Item ::= SEQUENCE {

sCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Associated-SCell-ItemExtIEs } }	OPTIONAL

}

Associated-SCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= OCTET STRING

AvailablePLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF AvailablePLMNList-Item

AvailablePLMNList-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { AvailablePLMNList-Item-ExtIEs} } OPTIONAL

}

AvailablePLMNList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailableSNPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF AvailableSNPN-ID-List-Item

AvailableSNPN-ID-List-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

availableNIDList			BroadcastNIDList,

iE-Extensions				ProtocolExtensionContainer { { AvailableSNPN-ID-List-ItemExtIEs} } OPTIONAL,

...

}

AvailableSNPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow  ::= INTEGER (0..4095, ...)

AreaScope ::= ENUMERATED {true, ...}

AoA-AssistanceInfo ::= SEQUENCE {

angleMeasurement			AngleMeasurementType,

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AoA-AssistanceInfo-ExtIEs } } OPTIONAL,

...

}

AoA-AssistanceInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AngleMeasurementType ::= CHOICE {

expected-ULAoA		Expected-UL-AoA,

expected-ZoA		Expected-ZoA-only,

choice-extension ProtocolIE-SingleContainer { { AngleMeasurementType-ExtIEs } }

}

AngleMeasurementType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AppLayerBufferLevelList ::= OCTET STRING

ARP-ID ::= INTEGER (1..16, ...)

ARPLocationInformation ::= SEQUENCE (SIZE (1..maxnoARPs)) OF ARPLocationInformation-Item

ARPLocationInformation-Item ::= SEQUENCE {

aRP-ID				ARP-ID,

aRPLocationType		ARPLocationType,

iE-Extensions		ProtocolExtensionContainer { { ARPLocationInformation-ExtIEs} } OPTIONAL,

...

}

ARPLocationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ARPLocationType ::= CHOICE {

aRPPositionRelativeGeodetic			RelativeGeodeticLocation,

aRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-SingleContainer { { ARPLocationType-ExtIEs } }

}

ARPLocationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

#### -- B

BAP-Header-Rewriting-Added-List-Item::= SEQUENCE {

ingressBAPRoutingID		BAPRoutingID,

egressBAPRoutingID		BAPRoutingID,

nonF1terminatingTopologyIndicator		NonF1terminatingTopologyIndicator		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { BAP-Header-Rewriting-Added-List-Item-ExtIEs} }	OPTIONAL

}

BAP-Header-Rewriting-Added-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BAP-Header-Rewriting-Removed-List-Item::= SEQUENCE {

ingressBAPRoutingID		BAPRoutingID,

iE-Extensions			ProtocolExtensionContainer { { BAP-Header-Rewriting-Removed-List-Item-ExtIEs} } OPTIONAL

}

BAP-Header-Rewriting-Removed-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BandwidthSRS ::= CHOICE {

fR1								FR1-Bandwidth,

fR2								FR2-Bandwidth,

choice-extension				ProtocolIE-SingleContainer {{ BandwidthSRS-ExtIEs }}

}

BandwidthSRS-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

BAPAddress ::= BIT STRING (SIZE(10))

BAPCtrlPDUChannel ::= ENUMERATED {true, ...}

BAPlayerBHRLCchannelMappingInfo ::= SEQUENCE {

bAPlayerBHRLCchannelMappingInfoToAdd			BAPlayerBHRLCchannelMappingInfoList			OPTIONAL,

bAPlayerBHRLCchannelMappingInfoToRemove			MappingInformationtoRemove					OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { BAPlayerBHRLCchannelMappingInfo-ExtIEs} } OPTIONAL,

...

}

BAPlayerBHRLCchannelMappingInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BAPlayerBHRLCchannelMappingInfoList ::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF BAPlayerBHRLCchannelMappingInfo-Item

BAPlayerBHRLCchannelMappingInfo-Item ::= SEQUENCE {

mappingInformationIndex			MappingInformationIndex,

priorHopBAPAddress				BAPAddress		OPTIONAL,

ingressbHRLCChannelID			BHRLCChannelID		OPTIONAL,

nextHopBAPAddress				BAPAddress		OPTIONAL,

egressbHRLCChannelID			BHRLCChannelID		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { BAPlayerBHRLCchannelMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

BAPlayerBHRLCchannelMappingInfo-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-IngressNonF1terminatingTopologyIndicator	CRITICALITY ignore	EXTENSION IngressNonF1terminatingTopologyIndicator		PRESENCE optional}|

{ ID id-EgressNonF1terminatingTopologyIndicator	CRITICALITY ignore	EXTENSION EgressNonF1terminatingTopologyIndicator		PRESENCE optional},

...

}

BAPPathID ::= BIT STRING (SIZE(10))

BAPRoutingID ::= SEQUENCE {

bAPAddress		BAPAddress,

bAPPathID		BAPPathID,

iE-Extensions	ProtocolExtensionContainer { { BAPRoutingIDExtIEs } }	OPTIONAL

}

BAPRoutingIDExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextF1U-TNLInfo ::= CHOICE {

locationindpendent				MBSF1UInformation,

locationdependent				LocationDependentMBSF1UInformation,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextF1U-TNLInfo-ExtIEs}}

}

BCBearerContextF1U-TNLInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

BitRate ::= INTEGER (0..4000000000000,...)

BearerTypeChange ::= ENUMERATED {true, ...}

BHRLCChannelID ::= BIT STRING (SIZE(16))

BHChannels-FailedToBeModified-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-FailedToBeModified-ItemExtIEs } }	OPTIONAL

}

BHChannels-FailedToBeModified-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-FailedToBeSetup-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-FailedToBeSetup-ItemExtIEs } }	OPTIONAL

}

BHChannels-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-FailedToBeSetupMod-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

BHChannels-FailedToBeSetupMod-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-Modified-Item ::= SEQUENCE {

bHRLCChannelID			BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-Modified-ItemExtIEs } }	OPTIONAL

}

BHChannels-Modified-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-Required-ToBeReleased-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

BHChannels-Required-ToBeReleased-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-Setup-Item ::= SEQUENCE {

bHRLCChannelID							BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-Setup-ItemExtIEs } }	OPTIONAL

}

BHChannels-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-SetupMod-Item ::= SEQUENCE {

bHRLCChannelID							BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-SetupMod-ItemExtIEs } }	OPTIONAL

}

BHChannels-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeModified-Item ::= SEQUENCE {

bHRLCChannelID					BHRLCChannelID,

bHQoSInformation				BHQoSInformation,

rLCmode				RLCMode	OPTIONAL,

bAPCtrlPDUChannel	BAPCtrlPDUChannel		OPTIONAL,

trafficMappingInfo	TrafficMappingInfo		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-ToBeModified-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeReleased-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeSetup-Item ::= SEQUENCE	{

bHRLCChannelID						BHRLCChannelID,

bHQoSInformation					BHQoSInformation,

rLCmode								RLCMode,

bAPCtrlPDUChannel					BAPCtrlPDUChannel		OPTIONAL,

trafficMappingInfo					TrafficMappingInfo		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { BHChannels-ToBeSetup-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeSetupMod-Item ::= SEQUENCE {

bHRLCChannelID				BHRLCChannelID,

bHQoSInformation			BHQoSInformation,

rLCmode				RLCMode,

bAPCtrlPDUChannel	BAPCtrlPDUChannel		OPTIONAL,

trafficMappingInfo	TrafficMappingInfo		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BHChannels-ToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHInfo ::= SEQUENCE {

bAProutingID			BAPRoutingID 	OPTIONAL,

egressBHRLCCHList		EgressBHRLCCHList	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { BHInfo-ExtIEs} } OPTIONAL

}

BHInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NonF1terminatingTopologyIndicator	CRITICALITY ignore	EXTENSION NonF1terminatingTopologyIndicator		PRESENCE optional	},

...

}

BHQoSInformation ::= CHOICE {

bHRLCCHQoS					QoSFlowLevelQoSParameters,

eUTRANBHRLCCHQoS			EUTRANQoS,

cPTrafficType				CPTrafficType,

choice-extension			ProtocolIE-SingleContainer { { BHQoSInformation-ExtIEs} }

}

BHQoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

BHRLCCHList ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF BHRLCCHItem

BHRLCCHItem ::= SEQUENCE {

bHRLCChannelID			BHRLCChannelID,

iE-Extensions			ProtocolExtensionContainer {{BHRLCCHItemExtIEs }}	 OPTIONAL

}

BHRLCCHItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BH-Routing-Information-Added-List-Item ::= SEQUENCE {

bAPRoutingID				BAPRoutingID,

nextHopBAPAddress			BAPAddress,

iE-Extensions				ProtocolExtensionContainer { { BH-Routing-Information-Added-List-ItemExtIEs} }	OPTIONAL

}

BH-Routing-Information-Added-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-NonF1terminatingTopologyIndicator  CRITICALITY ignore EXTENSION   NonF1terminatingTopologyIndicator		PRESENCE optional},

...

}

BH-Routing-Information-Removed-List-Item ::= SEQUENCE {

bAPRoutingID				BAPRoutingID,

iE-Extensions				ProtocolExtensionContainer { { BH-Routing-Information-Removed-List-ItemExtIEs} }	OPTIONAL

}

BH-Routing-Information-Removed-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BPLMN-ID-Info-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNsNR)) OF BPLMN-ID-Info-Item

BPLMN-ID-Info-Item ::= SEQUENCE {

pLMN-Identity-List			AvailablePLMNList,

extended-PLMN-Identity-List	ExtendedAvailablePLMN-List	OPTIONAL,

fiveGS-TAC					FiveGS-TAC					OPTIONAL,

nr-cell-ID					NRCellIdentity,

ranac						RANAC						OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BPLMN-ID-Info-ItemExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject EXTENSION NPNBroadcastInformation		PRESENCE optional},

...

}

ServedPLMNs-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedPLMNs-Item

ServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

iE-Extensions				ProtocolExtensionContainer { { ServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ServedPLMNs-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-TAISliceSupportList			CRITICALITY ignore	EXTENSION SliceSupportList				PRESENCE optional	}|

{ ID id-NPNSupportInfo				CRITICALITY reject	EXTENSION NPNSupportInfo				PRESENCE optional	}|

{ ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional	}|

{ ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION NSAGSupportList				PRESENCE optional},

...

}

BroadcastCAGList ::= SEQUENCE (SIZE(1..maxnoofCAGsupported)) OF CAGID

BroadcastMRBs-FailedToBeModified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-FailedtoBeModified-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-FailedtoBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-FailedToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-FailedToBeSetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-FailedToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-FailedToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-Modified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-Modified-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-Modified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-Setup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-Setup-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-SetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-SetupMod-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-SetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeModified-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters		OPTIONAL,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List	OPTIONAL,

bcBearerCtxtF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfo		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { BroadcastMRBs-ToBeModified-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-ToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeReleased-Item	::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

BroadcastMRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeSetup-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

bcBearerCtxtF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfo	,

iE-Extensions					ProtocolExtensionContainer { { BroadcastMRBs-ToBeSetup-Item-ExtIEs} },

...

}

BroadcastMRBs-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeSetupMod-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

bcBearerCtxtF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfo,

iE-Extensions					ProtocolExtensionContainer { { BroadcastMRBs-ToBeSetupMod-Item-ExtIEs} },

...

}

BroadcastMRBs-ToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastNIDList ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF NID

BroadcastSNPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF BroadcastSNPN-ID-List-Item

BroadcastSNPN-ID-List-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

broadcastNIDList			BroadcastNIDList,

iE-Extensions				ProtocolExtensionContainer { { BroadcastSNPN-ID-List-ItemExtIEs} } OPTIONAL,

...

}

BroadcastSNPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastPNI-NPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofCAGsupported)) OF BroadcastPNI-NPN-ID-List-Item

BroadcastPNI-NPN-ID-List-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

broadcastCAGList			BroadcastCAGList,

iE-Extensions				ProtocolExtensionContainer { { BroadcastPNI-NPN-ID-List-ItemExtIEs} } OPTIONAL,

...

}

BroadcastPNI-NPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastAreaScope ::= CHOICE {

completeSuccess			NULL,

partialSuccess			PartialSuccessCell,

choice-extension		ProtocolIE-SingleContainer { { BroadcastAreaScope-ExtIEs } }

}

BroadcastAreaScope-ExtIEs F1AP-PROTOCOL-IES::={

...

}

BroadcastCellList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF Broadcast-Cell-List-Item

Broadcast-Cell-List-Item ::= SEQUENCE {

cellID				NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Broadcast-Cell-List-ItemExtIEs} } OPTIONAL,

...

}

Broadcast-Cell-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BufferSizeThresh ::= INTEGER(0..16777215)

BurstArrivalTime ::= OCTET STRING

BW-Aggregation-Request-Indication ::= ENUMERATED  {true, ...}

BWP-Id ::= INTEGER (0..4)

BurstArrivalTimeWindow ::= SEQUENCE {

burstArrivalTimeWindowStart				INTEGER (0..640000, ...),

burstArrivalTimeWindowEnd				INTEGER (0..640000, ...),

iE-Extension			ProtocolExtensionContainer { {BurstArrivalTimeWindow-ExtIEs} } OPTIONAL,

...

}

BurstArrivalTimeWindow-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Broadcast-MRBs-Transport-Request-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { {Broadcast-MRBs-Transport-Request-Item-ExtIEs} } OPTIONAL,

...

}

Broadcast-MRBs-Transport-Request-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- C

CAGID ::= BIT STRING (SIZE(32))

Cancel-all-Warning-Messages-Indicator ::= ENUMERATED {true, ...}

Candidate-SpCell-Item ::= SEQUENCE {

candidate-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Candidate-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Candidate-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CapacityValue::= SEQUENCE {

capacityValue				INTEGER (0..100),

sSBAreaCapacityValueList	SSBAreaCapacityValueList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CapacityValue-ExtIEs} } OPTIONAL

}

CapacityValue-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer { { Cause-ExtIEs} }

}

Cause-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

rl-failure-rlc,

unknown-or-already-allocated-gnb-cu-ue-f1ap-id,

unknown-or-already-allocated-gnb-du-ue-f1ap-id,

unknown-or-inconsistent-pair-of-ue-f1ap-id,

interaction-with-other-procedure,

not-supported-qci-Value,

action-desirable-for-radio-reasons,

no-radio-resources-available,

procedure-cancelled,

normal-release,

...,

cell-not-available,

rl-failure-others,

ue-rejection,

resources-not-available-for-the-slice,

amf-initiated-abnormal-release,

release-due-to-pre-emption,

plmn-not-served-by-the-gNB-CU,

multiple-drb-id-instances,

unknown-drb-id,

multiple-bh-rlc-ch-id-instances,

unknown-bh-rlc-ch-id,

cho-cpc-resources-tobechanged,

nPN-not-supported,

nPN-access-denied,

gNB-CU-Cell-Capacity-Exceeded,

report-characteristics-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object,

unknown-bh-address,

unknown-bap-routing-id,

insufficient-ue-capabilities,

scg-activation-deactivation-failure,

scg-deactivation-failure-due-to-data-transmission,

requested-item-not-supported-on-time,

unknown-or-already-allocated-gNB-CU-MBS-F1AP-ID,

unknown-or-already-allocated-gNB-DU-MBS-F1AP-ID,

unknown-or-inconsistent-pair-of-MBS-F1AP-ID,

unknown-or-inconsistent-MRB-ID,

tat-sdt-expiry,

lTM-command-triggered,

sSB-not-available

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...,

unknown-TNL-address-for-IAB,

unknown-UP-TNL-information-for-IAB

}

CellGroupConfig ::= OCTET STRING

CellCapacityClassValue ::= INTEGER (1..100,...)

Cell-Direction ::= ENUMERATED {dl-only, ul-only}

CellMeasurementResultList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CellMeasurementResultItem

CellMeasurementResultItem ::= SEQUENCE {

cellID							NRCGI,

radioResourceStatus				RadioResourceStatus 			OPTIONAL,

compositeAvailableCapacityGroup	CompositeAvailableCapacityGroup	OPTIONAL,

sliceAvailableCapacity			SliceAvailableCapacity 			OPTIONAL,

numberofActiveUEs 				NumberofActiveUEs				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CellMeasurementResultItem-ExtIEs} } OPTIONAL

}

CellMeasurementResultItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NR-U-Channel-List	CRITICALITY ignore	EXTENSION NR-U-Channel-List PRESENCE optional },

...

}

Cell-Portion-ID ::= INTEGER (0..4095,...)

CellsForSON-List ::= SEQUENCE (SIZE(1.. maxServedCellforSON)) OF CellsForSON-Item

CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

neighbourNR-CellsForSON-List					NeighbourNR-CellsForSON-List									OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

CellsForSON-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Failed-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI				NRCGI,

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { { Cells-Failed-to-be-Activated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Failed-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Status-Item ::= SEQUENCE {

nRCGI			NRCGI,

service-status		Service-Status,

iE-Extensions				ProtocolExtensionContainer { { Cells-Status-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Status-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-To-Be-Broadcast-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-To-Be-Broadcast-ItemExtIEs } }	OPTIONAL,

...

}

Cells-To-Be-Broadcast-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Completed-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-Broadcast-Completed-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Completed-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Broadcast-To-Be-Cancelled-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Broadcast-To-Be-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Broadcast-To-Be-Cancelled-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Cancelled-Item ::= SEQUENCE {

nRCGI				NRCGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { { Cells-Broadcast-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Cancelled-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

nRPCI		NRPCI		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Activated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-gNB-CUSystemInformation			CRITICALITY reject	EXTENSION GNB-CUSystemInformation				PRESENCE optional }|

{ ID id-AvailablePLMNList				CRITICALITY ignore	EXTENSION AvailablePLMNList						PRESENCE optional }|

{ ID id-ExtendedAvailablePLMN-List		CRITICALITY ignore	EXTENSION ExtendedAvailablePLMN-List			PRESENCE optional }|

{ ID id-IAB-Info-IAB-donor-CU			CRITICALITY ignore	EXTENSION IAB-Info-IAB-donor-CU					PRESENCE optional}|

{ ID id-AvailableSNPN-ID-List			CRITICALITY ignore	EXTENSION AvailableSNPN-ID-List					PRESENCE optional }|

{ ID id-MBS-Broadcast-NeighbourCellList	CRITICALITY ignore	EXTENSION MBS-Broadcast-NeighbourCellList	PRESENCE optional }|

{ ID id-SSBs-withinTheCell-tobe-Activated-List			CRITICALITY reject	EXTENSION SSBs-toBeActivated-List				PRESENCE optional },

...

}

Cells-With-SSBs-Activated-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF Cells-With-SSBs-Activated-List-Item

Cells-With-SSBs-Activated-List-Item::= SEQUENCE {

nRCGI			 			NRCGI,

sSBs-activated-List 		SSBs-activated-List,

iE-Extensions				ProtocolExtensionContainer { { Cells-With-SSBs-Activated-List-Item-ExtIEs} } OPTIONAL

}

Cells-With-SSBs-Activated-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Allowed-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Cells-Allowed-to-be-Deactivated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-Allowed-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Deactivated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Barred-Item::= SEQUENCE {

nRCGI			NRCGI	,

cellBarred		CellBarred,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Barred-Item-ExtIEs } }	OPTIONAL

}

Cells-to-be-Barred-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-IAB-Barred	CRITICALITY ignore	EXTENSION IAB-Barred		PRESENCE optional }|

{ ID id-MobileIAB-Barred	CRITICALITY ignore	EXTENSION MobileIAB-Barred		PRESENCE optional },

...

}

CellBarred	::=	ENUMERATED {barred, not-barred, ...}

CellSize ::= ENUMERATED {verysmall, small, medium, large, ...}

CellToReportList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CellToReportItem

CellToReportItem ::= SEQUENCE {

cellID		NRCGI,

sSBToReportList		SSBToReportList		 OPTIONAL,

sliceToReportList	SliceToReportList	 OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { CellToReportItem-ExtIEs} } OPTIONAL

}

CellToReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellType ::= SEQUENCE {

cellSize		CellSize,

iE-Extensions		ProtocolExtensionContainer { {CellType-ExtIEs} }	OPTIONAL,

...

}

CellType-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellULConfigured ::=  ENUMERATED {none, ul, sul, ul-and-sul, ...}

CG-SDTQueryIndication ::=  ENUMERATED {true, ...}

CG-SDTKeptIndicator ::= ENUMERATED {true, ...}

CG-SDTindicatorSetup ::= ENUMERATED {true, ...}

CG-SDTindicatorMod ::= ENUMERATED {true, false, ...}

CG-SDTSessionInfo ::= SEQUENCE {

gNB-CU-UE-F1AP-ID					GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID					GNB-DU-UE-F1AP-ID,

iE-Extensions						ProtocolExtensionContainer {{CG-SDTSessionInfo-ExtIEs}}		OPTIONAL,

...

}

CG-SDTSessionInfo-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

ChannelOccupancyTimePercentage ::= INTEGER (0..100,...)

Child-IAB-Nodes-NA-Resource-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-IAB-Nodes-NA-Resource-List-Item

Child-IAB-Nodes-NA-Resource-List-Item::= SEQUENCE {

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID,

nA-Resource-Configuration-List		NA-Resource-Configuration-List 		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Child-IAB-Nodes-NA-Resource-List-Item-ExtIEs} } OPTIONAL

}

Child-IAB-Nodes-NA-Resource-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Child-Node-Cells-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-Node-Cells-List-Item

Child-Node-Cells-List-Item ::=	SEQUENCE{

nRCGI 								NRCGI,

iAB-DU-Cell-Resource-Configuration-Mode-Info 	IAB-DU-Cell-Resource-Configuration-Mode-Info	OPTIONAL,

iAB-STC-Info						IAB-STC-Info			OPTIONAL,

rACH-Config-Common					RACH-Config-Common		OPTIONAL,

rACH-Config-Common-IAB				RACH-Config-Common-IAB	OPTIONAL,

cSI-RS-Configuration				OCTET STRING			OPTIONAL,

sR-Configuration					OCTET STRING			OPTIONAL,

pDCCH-ConfigSIB1					OCTET STRING			OPTIONAL,

sCS-Common							OCTET STRING	OPTIONAL,

multiplexingInfo					MultiplexingInfo	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer {{Child-Node-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Child-Node-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Child-Nodes-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-Nodes-List-Item

Child-Nodes-List-Item ::= SEQUENCE{

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID,

child-Node-Cells-List 	Child-Node-Cells-List	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{Child-Nodes-List-Item-ExtIEs}}		OPTIONAL

}

Child-Nodes-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CHOtrigger-InterDU ::= ENUMERATED {

cho-initiation,

cho-replace,

...

}

CHOtrigger-IntraDU ::= ENUMERATED {

cho-initiation,

cho-replace,

cho-cancel,

...

}

CNSubgroupID ::= INTEGER (0..7, ...)

CNUEPagingIdentity ::= CHOICE {

fiveG-S-TMSI			BIT STRING (SIZE(48)),

choice-extension			ProtocolIE-SingleContainer { { CNUEPagingIdentity-ExtIEs } }

}

CNUEPagingIdentity-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CompositeAvailableCapacityGroup ::= SEQUENCE {

compositeAvailableCapacityDownlink	CompositeAvailableCapacity,

compositeAvailableCapacityUplink 	CompositeAvailableCapacity,

iE-Extensions	ProtocolExtensionContainer { { CompositeAvailableCapacityGroup-ExtIEs} } OPTIONAL

}

CompositeAvailableCapacityGroup-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CompositeAvailableCapacity-SUL		CRITICALITY ignore	EXTENSION CompositeAvailableCapacity		PRESENCE optional	},

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue 	CellCapacityClassValue		OPTIONAL,

capacityValue			CapacityValue,

iE-Extensions	ProtocolExtensionContainer { { CompositeAvailableCapacity-ExtIEs} } OPTIONAL

}

CompositeAvailableCapacity-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CHO-Probability ::= INTEGER (1..100)

ConditionalInterDUMobilityInformation ::= SEQUENCE {

cho-trigger						CHOtrigger-InterDU,

targetgNB-DUUEF1APID			GNB-DU-UE-F1AP-ID							OPTIONAL

-- This IE shall be present if the cho-trigger IE is present and set to "cho-replace" --,

iE-Extensions					ProtocolExtensionContainer { { ConditionalInterDUMobilityInformation-ExtIEs} }	OPTIONAL,

...

}

ConditionalInterDUMobilityInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-EstimatedArrivalProbability		CRITICALITY ignore		EXTENSION CHO-Probability				PRESENCE optional	}|

{ ID id-SCPAC-Request					CRITICALITY reject		EXTENSION SCPAC-Request			PRESENCE optional	}|

{ ID id-S-CPACLowerLayerReferenceConfigRequest	CRITICALITY reject		EXTENSION S-CPACLowerLayerReferenceConfigRequest		PRESENCE optional	},

...

}

ConditionalIntraDUMobilityInformation ::= SEQUENCE {

cho-trigger						CHOtrigger-IntraDU,

targetCellsTocancel				TargetCellList								OPTIONAL,

-- This IE may be present if the cho-trigger IE is present and set to "cho-cancel"

iE-Extensions					ProtocolExtensionContainer { { ConditionalIntraDUMobilityInformation-ExtIEs} }	OPTIONAL,

...

}

ConditionalIntraDUMobilityInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-EstimatedArrivalProbability		CRITICALITY ignore		EXTENSION CHO-Probability				PRESENCE optional	}|

{ ID id-SCPAC-Request					CRITICALITY reject		EXTENSION SCPAC-Request			PRESENCE optional	}|

{ ID id-S-CPACLowerLayerReferenceConfigRequest	CRITICALITY reject		EXTENSION S-CPACLowerLayerReferenceConfigRequest		PRESENCE optional	},

...

}

ConfigRestrictInfoDAPS ::= OCTET STRING

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

Configured-BWP-List ::= SEQUENCE (SIZE(1.. maxNrofBWPs)) OF Configured-BWP-Item

Configured-BWP-Item ::= SEQUENCE {

bWP-Id									BWP-Id,

bWP-Location-and-bandwidth				INTEGER (0..37949),

iE-Extensions					ProtocolExtensionContainer { { Configured-BWP-Item-ExtIEs } }	OPTIONAL,

...

}

Configured-BWP-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CoordinateID ::= INTEGER (0..511, ...)

Coverage-Modification-Notification ::= SEQUENCE {

coverage-Modification-List				Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Coverage-Modification-Item

Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

cellCoverageState					CellCoverageState,

sSBCoverageModificationList			SSBCoverageModification-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Coverage-Modification-Cause	CRITICALITY ignore	EXTENSION CCO-issue-detection		PRESENCE optional },

...

}

CellCoverageState ::= INTEGER (0..63, ...)

CCO-Assistance-Information ::= SEQUENCE {

cCO-issue-detection				CCO-issue-detection		OPTIONAL,

affectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

CCO-issue-detection	::=	ENUMERATED {

coverage,

cell-edge-capacity,

...,

network-energy-saving}

CP-TransportLayerAddress ::= CHOICE {

endpoint-IP-address				TransportLayerAddress,

endpoint-IP-address-and-port	Endpoint-IP-address-and-port,

choice-extension				ProtocolIE-SingleContainer { { CP-TransportLayerAddress-ExtIEs } }

}

CP-TransportLayerAddress-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CPACMCGInformation ::= SEQUENCE {

cpac-trigger					CPAC-trigger,

pscellid						NRCGI,

iE-Extensions					ProtocolExtensionContainer { { CPACMCGInformation-ExtIEs} }	OPTIONAL,

...

}

CPACMCGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CPAC-trigger ::= ENUMERATED {

cpac-preparation,

cpac-executed,

...

}

CPTrafficType ::= INTEGER (1..3,...)

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode														OPTIONAL,

triggeringMessage				TriggeringMessage													OPTIONAL,

procedureCriticality			Criticality															OPTIONAL,

transactionID					TransactionID														OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer {{CriticalityDiagnostics-ExtIEs}}		OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1.. maxnoofErrors)) OF CriticalityDiagnostics-IE-Item

CriticalityDiagnostics-IE-Item ::= SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError 			TypeOfError,

iE-Extensions			ProtocolExtensionContainer {{CriticalityDiagnostics-IE-Item-ExtIEs}}	OPTIONAL,

...

}

CriticalityDiagnostics-IE-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= INTEGER (0..65535, ...)

CUDURadioInformationType ::= CHOICE {

rIM								CUDURIMInformation,

choice-extension				ProtocolIE-SingleContainer { { CUDURadioInformationType-ExtIEs} }

}

CUDURadioInformationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CUDURIMInformation ::= SEQUENCE {

victimgNBSetID			GNBSetID,

rIMRSDetectionStatus	RIMRSDetectionStatus,

iE-Extensions			ProtocolExtensionContainer { { CUDURIMInformation-ExtIEs} }	OPTIONAL

}

CUDURIMInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CUtoDURRCInformation ::= SEQUENCE {

cG-ConfigInfo						CG-ConfigInfo						OPTIONAL,

uE-CapabilityRAT-ContainerList		UE-CapabilityRAT-ContainerList		OPTIONAL,

measConfig							MeasConfig							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CUtoDURRCInformation-ExtIEs} } OPTIONAL,

...

}

CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-HandoverPreparationInformation	CRITICALITY ignore	EXTENSION HandoverPreparationInformation		PRESENCE optional }|

{ ID id-CellGroupConfig					CRITICALITY ignore	EXTENSION CellGroupConfig							PRESENCE optional }|

{ ID id-MeasurementTimingConfiguration	CRITICALITY ignore	EXTENSION MeasurementTimingConfiguration		PRESENCE optional }|

{ ID id-UEAssistanceInformation			CRITICALITY ignore	EXTENSION UEAssistanceInformation					PRESENCE optional }|

{ ID id-CG-Config						CRITICALITY ignore	EXTENSION CG-Config									PRESENCE optional }|

{ ID id-UEAssistanceInformationEUTRA	CRITICALITY ignore	EXTENSION UEAssistanceInformationEUTRA			PRESENCE optional }|

{ ID id-LocationMeasurementInformation	CRITICALITY ignore	EXTENSION LocationMeasurementInformation		PRESENCE optional }|

{ ID id-MUSIM-GapConfig					CRITICALITY reject	EXTENSION MUSIM-GapConfig							PRESENCE optional }|

{ ID id-SDT-MAC-PHY-CG-Config			CRITICALITY ignore	EXTENSION SDT-MAC-PHY-CG-Config						PRESENCE optional }|

{ ID id-MBSInterestIndication			CRITICALITY ignore	EXTENSION MBSInterestIndication						PRESENCE optional }|

{ ID id-NeedForGapsInfoNR				CRITICALITY ignore	EXTENSION NeedForGapsInfoNR							PRESENCE optional }|

{ ID id-NeedForGapNCSGInfoNR			CRITICALITY ignore	EXTENSION NeedForGapNCSGInfoNR						PRESENCE optional }|

{ ID id-NeedForGapNCSGInfoEUTRA			CRITICALITY ignore	EXTENSION NeedForGapNCSGInfoEUTRA					PRESENCE optional }|

{ ID id-ConfigRestrictInfoDAPS			CRITICALITY ignore	EXTENSION ConfigRestrictInfoDAPS					PRESENCE optional }|

{ ID id-Preconfigured-measurement-GAP-Request			CRITICALITY ignore	EXTENSION Preconfigured-measurement-GAP-Request					PRESENCE optional }|

{ ID id-NeedForInterruptionInfoNR		CRITICALITY ignore	EXTENSION NeedForInterruptionInfoNR				PRESENCE optional }|

{ ID id-MusimCapabilityRestrictionIndication		CRITICALITY ignore	EXTENSION MusimCapabilityRestrictionIndication				PRESENCE optional }|

{ ID id-MusimCandidateBandList			CRITICALITY ignore	EXTENSION MusimCandidateBandList					PRESENCE optional },

...

}

CUtoDUTAInformation-List ::= SEQUENCE (SIZE(1.. maxnoofTAList)) OF CUtoDUTAInformation

CUtoDUTAInformation	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

preambleIndex			PreambleIndex,

rA-RNTI					RA-RNTI,

iE-Extensions			ProtocolExtensionContainer { { CUtoDUTAInformationItem-ExtIEs} }	OPTIONAL,

...

}

CUtoDUTAInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CSIResourceConfiguration ::= SEQUENCE  {

cSIResourceConfigToAddModList 			OCTET STRING		OPTIONAL,

cSIResourceConfigToReleaseList			OCTET STRING		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { CSIResourceConfiguration-ExtIEs} }	OPTIONAL

}

CSIResourceConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

DAPS-HO-Status::= ENUMERATED{initiation,... }

DCBasedDuplicationConfigured::= ENUMERATED{true,..., false}

DeactivationIndication ::= CHOICE {

perUE						DeactivationIndicationList,

deactivateAll				NULL,

choice-extension			ProtocolIE-SingleContainer { { DeactivationIndication-ExtIEs} }

}

DeactivationIndication-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DeactivationIndicationList ::= SEQUENCE (SIZE(1..maxnoofUEsInQMCTransferControlMessage)) OF DeactivationIndicationList-Item

DeactivationIndicationList-Item ::= SEQUENCE {

gNB-CU-UE-F1AP-ID						GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID						GNB-DU-UE-F1AP-ID,

iE-Extensions							ProtocolExtensionContainer { { DeactivationIndicationList-Item-ExtIEs} } OPTIONAL,

...

}

DeactivationIndicationList-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Dedicated-SIDelivery-NeededUE-Item ::= SEQUENCE {

gNB-CU-UE-F1AP-ID						GNB-CU-UE-F1AP-ID,

nRCGI									NRCGI,

iE-Extensions							ProtocolExtensionContainer { { DedicatedSIDeliveryNeededUE-Item-ExtIEs} } OPTIONAL,

...

}

DedicatedSIDeliveryNeededUE-Item-ExtIEs F1AP-PROTOCOL-EXTENSION::={

...

}

DedicatedSIDeliveryIndication::= ENUMERATED{true, ...}

DL-PRS ::= SEQUENCE {

prsid 					INTEGER (0..255),

dl-PRSResourceSetID		PRS-Resource-Set-ID,

dl-PRSResourceID		PRS-Resource-ID	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {DL-PRS-ExtIEs} }	OPTIONAL

}

DL-PRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSMutingPattern ::= CHOICE {

two					BIT STRING (SIZE(2)),

four				BIT STRING (SIZE(4)),

six					BIT STRING (SIZE(6)),

eight				BIT STRING (SIZE(8)),

sixteen				BIT STRING (SIZE(16)),

thirty-two			BIT STRING (SIZE(32)),

choice-extension							ProtocolIE-SingleContainer { { DL-PRSMutingPattern-ExtIEs } }

}

DL-PRSMutingPattern-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DLPRSResourceCoordinates ::= SEQUENCE {

listofDL-PRSResourceSetARP		SEQUENCE (SIZE(1.. maxnoofPRS-ResourceSets)) OF DLPRSResourceSetARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceCoordinates-ExtIEs } } OPTIONAL

}

DLPRSResourceCoordinates-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLPRSResourceSetARP ::= SEQUENCE {

dl-PRSResourceSetID				PRS-Resource-Set-ID,

dL-PRSResourceSetARPLocation	DL-PRSResourceSetARPLocation,

listofDL-PRSResourceARP			SEQUENCE (SIZE(1.. maxnoofPRS-ResourcesPerSet)) OF DLPRSResourceARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceSetARP-ExtIEs } } OPTIONAL

}

DLPRSResourceSetARP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceSetARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-SingleContainer { { DL-PRSResourceSetARPLocation-ExtIEs } }

}

DL-PRSResourceSetARPLocation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DLPRSResourceARP ::= SEQUENCE {

dl-PRSResourceID			PRS-Resource-ID,

dL-PRSResourceARPLocation	DL-PRSResourceARPLocation,

iE-Extensions				ProtocolExtensionContainer { { DLPRSResourceARP-ExtIEs } } OPTIONAL

}

DLPRSResourceARP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-SingleContainer { { DL-PRSResourceARPLocation-ExtIEs } }

}

DL-PRSResourceARPLocation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DL-UP-TNL-Address-to-Update-List-Item	::= SEQUENCE {

oldIPAdress						TransportLayerAddress,

newIPAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { DL-UP-TNL-Address-to-Update-List-ItemExtIEs } }	OPTIONAL,

...

}

DL-UP-TNL-Address-to-Update-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLUPTNLInformation-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDLUPTNLInformation)) OF DLUPTNLInformation-ToBeSetup-Item

DLUPTNLInformation-ToBeSetup-Item ::= SEQUENCE {

dLUPTNLInformation	UPTransportLayerInformation	,

iE-Extensions	ProtocolExtensionContainer { { DLUPTNLInformation-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DLUPTNLInformation-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity-Item ::= SEQUENCE {

dRBID			DRBID,

dRB-Activity	DRB-Activity		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity ::= ENUMERATED {active, not-active}

DRBID ::= INTEGER (1..32, ...)

DRBs-FailedToBeModified-Item	::= SEQUENCE {

dRBID		DRBID		,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetup-Item	::= SEQUENCE {

dRBID	DRBID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

dRBID		DRBID	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardUL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardUL		PRESENCE optional	},

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Modified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status							CRITICALITY ignore	EXTENSION RLC-Status									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex								PRESENCE optional }|

{ ID id-TSCTrafficCharacteristicsFeedback	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristicsFeedback		PRESENCE optional	}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus				PRESENCE optional },

...

}

DRBs-ModifiedConf-Item	::= SEQUENCE {

dRBID							DRBID,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ModifiedConf-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ModifiedConf-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional },

...

}

DRB-Notify-Item ::= SEQUENCE {

dRBID			DRBID,

notification-Cause	Notification-Cause,

iE-Extensions	ProtocolExtensionContainer { { DRB-Notify-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Notify-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION QoSParaSetNotifyIndex	PRESENCE optional	}|

{ ID id-TSCTrafficCharacteristicsFeedback	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristicsFeedback	PRESENCE optional	},

...

}

DRBs-Required-ToBeModified-Item	::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional },

...

}

DRBs-Required-ToBeReleased-Item	::= SEQUENCE {

dRBID		DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Setup-Item ::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex									PRESENCE optional }|

{ ID id-TSCTrafficCharacteristicsFeedback	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristicsFeedback			PRESENCE optional	}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus			PRESENCE optional },

...

}

DRBs-SetupMod-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex									PRESENCE optional }|

{ ID id-TSCTrafficCharacteristicsFeedback	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristicsFeedback			PRESENCE optional	}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus	PRESENCE optional },

...

}

DRBs-ToBeModified-Item	::= SEQUENCE {

dRBID						DRBID,

qoSInformation				QoSInformation	OPTIONAL,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List	,

uLConfiguration				ULConfiguration	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DLPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength										PRESENCE optional }|

{ ID id-ULPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength										PRESENCE optional }|

{ID id-BearerTypeChange						CRITICALITY ignore	EXTENSION BearerTypeChange									PRESENCE optional}|

{ ID id-RLCMode								CRITICALITY ignore	EXTENSION RLCMode											PRESENCE optional }|

{ ID id-Duplication-Activation				CRITICALITY reject	EXTENSION DuplicationActivation							PRESENCE optional }|

{ ID id-DC-Based-Duplication-Configured		CRITICALITY reject	EXTENSION DCBasedDuplicationConfigured					PRESENCE optional }|

{ ID id-DC-Based-Duplication-Activation		CRITICALITY reject	EXTENSION DuplicationActivation							PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation						PRESENCE optional}|

{ ID id-TransmissionStopIndicator			CRITICALITY ignore	EXTENSION TransmissionStopIndicator						PRESENCE optional}|

{ ID id-CG-SDTindicatorMod					CRITICALITY reject	EXTENSION CG-SDTindicatorMod								PRESENCE optional },

...

}

DRBs-ToBeReleased-Item	::= SEQUENCE {

dRBID	DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeSetup-Item ::= SEQUENCE	{

dRBID						DRBID,

qoSInformation				QoSInformation,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List	,

rLCMode						RLCMode,

uLConfiguration				ULConfiguration	OPTIONAL,

duplicationActivation		DuplicationActivation	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DC-Based-Duplication-Configured		CRITICALITY reject	EXTENSION DCBasedDuplicationConfigured				PRESENCE optional }|

{ ID id-DC-Based-Duplication-Activation		CRITICALITY reject	EXTENSION DuplicationActivation						PRESENCE optional }|

{ ID id-DLPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE mandatory }|

{ ID id-ULPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-SDTRLCBearerConfiguration			CRITICALITY ignore	EXTENSION SDTRLCBearerConfiguration					PRESENCE optional },

...

}

DRBs-ToBeSetupMod-Item	::= SEQUENCE {

dRBID						DRBID,

qoSInformation				QoSInformation,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List,

rLCMode						RLCMode,

uLConfiguration				ULConfiguration	OPTIONAL,

duplicationActivation		DuplicationActivation	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DC-Based-Duplication-Configured		CRITICALITY reject	EXTENSION DCBasedDuplicationConfigured				PRESENCE optional }|

{ ID id-DC-Based-Duplication-Activation		CRITICALITY reject	EXTENSION DuplicationActivation						PRESENCE optional }|

{ ID id-DLPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE optional }|

{ ID id-ULPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-CG-SDTindicatorSetup				CRITICALITY reject	EXTENSION CG-SDTindicatorSetup						PRESENCE optional },

...

}

DRB-List ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-List-Item

DRB-List-Item ::= SEQUENCE {

dRBID						DRBID,

iE-Extensions				ProtocolExtensionContainer { { DRB-List-Item-ExtIEs} } OPTIONAL

}

DRB-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRXCycle	::= SEQUENCE {

longDRXCycleLength	LongDRXCycleLength,

shortDRXCycleLength		ShortDRXCycleLength	OPTIONAL,

shortDRXCycleTimer	ShortDRXCycleTimer OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { DRXCycle-ExtIEs} } OPTIONAL,

...

}

DRXCycle-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRX-Config ::= OCTET STRING

DRXConfigurationIndicator	::=	ENUMERATED{	release, ...}

DRX-LongCycleStartOffset ::= INTEGER (0..10239)

DSInformationList ::= SEQUENCE (SIZE(0..maxnoofDSInfo)) OF DSCP

DSCP ::= BIT STRING (SIZE (6))

DUtoCURRCContainer ::= OCTET STRING

DUCURadioInformationType ::= CHOICE {

rIM								DUCURIMInformation,

choice-extension				ProtocolIE-SingleContainer { { DUCURadioInformationType-ExtIEs} }

}

DUCURadioInformationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DUCURIMInformation ::= SEQUENCE {

victimgNBSetID				GNBSetID,

rIMRSDetectionStatus		RIMRSDetectionStatus,

aggressorCellList			AggressorCellList,

iE-Extensions				ProtocolExtensionContainer { { DUCURIMInformation-ExtIEs} }		OPTIONAL

}

DUCURIMInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DUF-Slot-Config-Item 	::=	CHOICE {

explicitFormat				ExplicitFormat,

implicitFormat				ImplicitFormat,

choice-extension				ProtocolIE-SingleContainer { { DUF-Slot-Config-Item-ExtIEs} }

}

DUF-Slot-Config-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DUF-Slot-Config-List	::= SEQUENCE (SIZE(1..maxnoofDUFSlots)) OF DUF-Slot-Config-Item

DUFSlotformatIndex ::= INTEGER(0..254)

DUFTransmissionPeriodicity ::= ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ...}

DU-RX-MT-RX ::= ENUMERATED {supported, not-supported }

DU-TX-MT-TX ::= ENUMERATED {supported, not-supported }

DU-RX-MT-TX ::= ENUMERATED {supported, not-supported }

DU-TX-MT-RX ::= ENUMERATED {supported, not-supported }

DU-RX-MT-RX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DU-TX-MT-TX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DU-RX-MT-TX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DU-TX-MT-RX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DUtoCURRCInformation ::= SEQUENCE {

cellGroupConfig		CellGroupConfig,

measGapConfig			MeasGapConfig	OPTIONAL,

requestedP-MaxFR1				OCTET STRING				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { DUtoCURRCInformation-ExtIEs} } OPTIONAL,

...

}

DUtoCURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DRX-LongCycleStartOffset			CRITICALITY ignore	EXTENSION DRX-LongCycleStartOffset				PRESENCE optional }|

{ ID id-SelectedBandCombinationIndex		CRITICALITY ignore	EXTENSION SelectedBandCombinationIndex			PRESENCE optional }|

{ ID id-SelectedFeatureSetEntryIndex		CRITICALITY ignore	EXTENSION SelectedFeatureSetEntryIndex			PRESENCE optional }|

{ ID id-Ph-InfoSCG							CRITICALITY ignore	EXTENSION Ph-InfoSCG								PRESENCE optional }|

{ ID id-RequestedBandCombinationIndex		CRITICALITY ignore	EXTENSION RequestedBandCombinationIndex		PRESENCE optional }|

{ ID id-RequestedFeatureSetEntryIndex		CRITICALITY ignore	EXTENSION RequestedFeatureSetEntryIndex		PRESENCE optional }|

{ ID id-DRX-Config							CRITICALITY ignore	EXTENSION DRX-Config								PRESENCE optional }|

{ ID id-PDCCH-BlindDetectionSCG				CRITICALITY ignore	EXTENSION PDCCH-BlindDetectionSCG				PRESENCE optional }|

{ ID id-Requested-PDCCH-BlindDetectionSCG	CRITICALITY ignore	EXTENSION Requested-PDCCH-BlindDetectionSCG	PRESENCE optional }|

{ ID id-Ph-InfoMCG							CRITICALITY ignore	EXTENSION Ph-InfoMCG								PRESENCE optional }|

{ ID id-MeasGapSharingConfig				CRITICALITY ignore	EXTENSION MeasGapSharingConfig					PRESENCE optional }|

{ ID id-SL-PHY-MAC-RLC-Config				CRITICALITY ignore	EXTENSION SL-PHY-MAC-RLC-Config					PRESENCE optional }|

{ ID id-SL-ConfigDedicatedEUTRA-Info		CRITICALITY ignore	EXTENSION SL-ConfigDedicatedEUTRA-Info			PRESENCE optional }|

{ ID id-RequestedP-MaxFR2					CRITICALITY ignore	EXTENSION RequestedP-MaxFR2							PRESENCE optional }|

{ ID id-SDT-MAC-PHY-CG-Config				CRITICALITY ignore	EXTENSION SDT-MAC-PHY-CG-Config					PRESENCE optional }|

{ ID id-MUSIM-GapConfig						CRITICALITY ignore	EXTENSION MUSIM-GapConfig							PRESENCE optional }|

{ ID id-SL-RLC-ChannelToAddModList			CRITICALITY ignore	EXTENSION SL-RLC-ChannelToAddModList			PRESENCE optional }|

{ ID id-InterFrequencyConfig-NoGap			CRITICALITY ignore	EXTENSION InterFrequencyConfig-NoGap			PRESENCE optional }|

{ ID id-UL-GapFR2-Config					CRITICALITY ignore	EXTENSION UL-GapFR2-Config							PRESENCE optional }|

{ ID id-TwoPHRModeMCG						CRITICALITY ignore	EXTENSION TwoPHRModeMCG								PRESENCE optional }|

{ ID id-TwoPHRModeSCG						CRITICALITY ignore	EXTENSION TwoPHRModeSCG								PRESENCE optional }|

{ ID id-ncd-SSB-RedCapInitialBWP-SDT		CRITICALITY ignore	EXTENSION Ncd-SSB-RedCapInitialBWP-SDT			PRESENCE optional }|

{ ID id-ServCellInfoList					CRITICALITY ignore	EXTENSION ServCellInfoList							PRESENCE optional }|

{ ID id-SL-PHY-MAC-RLC-ConfigExt			CRITICALITY ignore	EXTENSION SL-PHY-MAC-RLC-ConfigExt				PRESENCE optional },

...

}

DUtoCUTAInformation-List ::= SEQUENCE (SIZE(1.. maxnoofTAList)) OF DUtoCUTAInformation

DUtoCUTAInformation	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

preambleIndex			PreambleIndex,

rA-RNTI					RA-RNTI,

sourceGNB-DU-ID 		GNB-DU-ID,

iE-Extensions			ProtocolExtensionContainer { { DUtoCUTAInformationItem-ExtIEs} }	OPTIONAL,

...

}

DUtoCUTAInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DuplicationActivation ::= ENUMERATED{active,inactive,... }

DuplicationIndication ::= ENUMERATED {true, ... , false }

DuplicationState ::= ENUMERATED {

active,

inactive,

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel					INTEGER (1..127),

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

fiveQI								INTEGER (0..255, ...)								OPTIONAL,

delayCritical						ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

-- C-ifGBRflow: This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow 					AveragingWindow										OPTIONAL,

-- C-ifGBRflow: This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Dynamic5QIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetDownlink			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUplink			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

}

DynamicPQIDescriptor	::= SEQUENCE {

resourceType						ENUMERATED {gbr, non-gbr, delay-critical-grb, ...}	OPTIONAL,

qoSPriorityLevel					INTEGER (1..8, ...),

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

averagingWindow 					AveragingWindow										OPTIONAL,

-- C-ifGBRflow: This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { DynamicPQIDescriptor-ExtIEs } } OPTIONAL

}

DynamicPQIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLLBTFailureInformationRequest 	::= ENUMERATED {inquiry, ...}

DLLBTFailureInformationList		::= SEQUENCE (SIZE(1.. maxnoofLBTFailureInformation)) OF DLLBTFailureInformationList-Item

DLLBTFailureInformationList-Item::= SEQUENCE {

uEAssistantIdentifier		GNB-CU-UE-F1AP-ID,

numberOfDLLBTFailures		INTEGER (1..1000,...)							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { DLLBTFailureInformationList-Item-ExtIEs} }	OPTIONAL,

...

}

DLLBTFailureInformationList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DynamicResourceAllocationRequest ::= ENUMERATED {request-for-preamble-index, ...}

DynamicResourceAllocationResponse	::= SEQUENCE {

preambleIndex		PreambleIndex							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { DynamicResourceAllocationResponse-ExtIEs} }	OPTIONAL,

...

}

DynamicResourceAllocationResponse-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

EarlyULSyncConfig ::= SEQUENCE {

rACH 								RACHConfiguration,

lTMgNB-DU-IDsList					LTMgNB-DU-IDsList							OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { EarlyULSyncConfig-ExtIEs} } OPTIONAL,

...

}

EarlyULSyncConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation-Request  ::= SEQUENCE {

requestforRACHConfiguration					RequestforRACHConfiguration									OPTIONAL,

lTMgNB-DU-IDsList							LTMgNB-DU-IDsList											OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { EarlySyncInformation-Request-ExtIEs} } OPTIONAL,

...

}

EarlySyncInformation-Request-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation  ::= SEQUENCE {

tCIStatesConfigurationsList		TCIStatesConfigurationsList,

earlyULSyncConfig				EarlyULSyncConfig									OPTIONAL,

earlyULSyncConfigSUL			EarlyULSyncConfig									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { EarlySyncInformation-ExtIEs} } OPTIONAL,

...

}

EarlySyncInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF EarlySyncInformation-Item

EarlySyncInformation-Item ::= SEQUENCE {

nRCGI				NRCGI,

tCIStatesConfigurationsList		TCIStatesConfigurationsList							OPTIONAL,

earlyULSyncConfig				EarlyULSyncConfig									OPTIONAL,

earlyULSyncConfigSUL			EarlyULSyncConfig									OPTIONAL,

tAAssistanceInfo				TAAssistanceInfo									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { EarlySyncInformation-Item-ExtIEs } }	OPTIONAL,

...

}

EarlySyncInformation-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxnoofMeasE-CID)) OF ProtocolIE-SingleContainer { {E-CID-MeasurementQuantities-ItemIEs} }

E-CID-MeasurementQuantities-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-E-CID-MeasurementQuantities-Item	CRITICALITY reject	TYPE E-CID-MeasurementQuantities-Item		PRESENCE mandatory}

}

E-CID-MeasurementQuantities-Item ::= SEQUENCE {

e-CIDmeasurementQuantitiesValue				E-CID-MeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { E-CID-MeasurementQuantitiesValue-ExtIEs} } OPTIONAL

}

E-CID-MeasurementQuantitiesValue-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasurementQuantitiesValue ::= ENUMERATED {

default,

angleOfArrivalNR,

... ,

timingAdvanceNR

}

E-CID-MeasurementResult ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates 	OPTIONAL,

measuredResults-List		E-CID-MeasuredResults-List 	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL

}

E-CID-MeasurementResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MobileAccessPointLocation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional },

...

}

E-CID-MeasuredResults-List ::= SEQUENCE (SIZE(1..maxnoofMeasE-CID)) OF E-CID-MeasuredResults-Item

E-CID-MeasuredResults-Item ::= SEQUENCE {

e-CID-MeasuredResults-Value 	E-CID-MeasuredResults-Value,

iE-Extensions			ProtocolExtensionContainer {{ E-CID-MeasuredResults-Item-ExtIEs }}	 OPTIONAL

}

E-CID-MeasuredResults-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasuredResults-Value ::= CHOICE {

valueAngleofArrivalNR	UL-AoA,

choice-extension		ProtocolIE-SingleContainer { { E-CID-MeasuredResults-Value-ExtIEs} }

}

E-CID-MeasuredResults-Value-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-NR-TADV			CRITICALITY ignore	TYPE NR-TADV	PRESENCE mandatory	},

...

}

E-CID-ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

EgressBHRLCCHList ::= SEQUENCE (SIZE(1..maxnoofEgressLinks)) OF EgressBHRLCCHItem

EgressBHRLCCHItem ::= SEQUENCE {

nextHopBAPAddress 		BAPAddress,

bHRLCChannelID			BHRLCChannelID,

iE-Extensions			ProtocolExtensionContainer {{EgressBHRLCCHItemExtIEs }}	 OPTIONAL

}

EgressBHRLCCHItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EgressNonF1terminatingTopologyIndicator ::= ENUMERATED {true, ...}

Endpoint-IP-address-and-port ::=SEQUENCE {

endpointIPAddress TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL

}

Endpoint-IP-address-and-port-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-portNumber	CRITICALITY reject	EXTENSION PortNumber		PRESENCE optional},

...

}

EnergyDetectionThreshold ::= INTEGER (-100..-50, ...)

ExtendedAvailablePLMN-List ::= SEQUENCE (SIZE(1..maxnoofExtendedBPLMNs)) OF ExtendedAvailablePLMN-Item

ExtendedAvailablePLMN-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAvailablePLMN-Item-ExtIEs} } OPTIONAL

}

ExplicitFormat ::=	SEQUENCE {

permutation			Permutation,

noofDownlinkSymbols	NoofDownlinkSymbols		OPTIONAL,

noofUplinkSymbols	NoofUplinkSymbols		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExplicitFormat-ExtIEs} } OPTIONAL

}

ExplicitFormat-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedAvailablePLMN-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedServedPLMNs-List ::= SEQUENCE (SIZE(1.. maxnoofExtendedBPLMNs)) OF ExtendedServedPLMNs-Item

ExtendedServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

tAISliceSupportList 		SliceSupportList	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ExtendedServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ExtendedServedPLMNs-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NPNSupportInfo				CRITICALITY reject	EXTENSION NPNSupportInfo				PRESENCE optional	}|

{ ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional	}|

{ ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION NSAGSupportList				PRESENCE optional},

...

}

ExtendedSliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofExtSliceItems)) OF SliceSupportItem

ExtendedUEIdentityIndexValue ::= BIT STRING (SIZE(16))

EUTRACells-List  ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF EUTRACells-List-item

EUTRACells-List-item ::= SEQUENCE {

eUTRA-Cell-ID					EUTRA-Cell-ID,

served-EUTRA-Cells-Information	Served-EUTRA-Cells-Information,

iE-Extensions ProtocolExtensionContainer { { EUTRACells-List-itemExtIEs } }    OPTIONAL

}

EUTRACells-List-itemExtIEs    F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Cell-ID ::= BIT STRING (SIZE(28))

EUTRA-Coex-FDD-Info ::= SEQUENCE {

uL-EARFCN						ExtendedEARFCN					OPTIONAL,

dL-EARFCN						ExtendedEARFCN,

uL-Transmission-Bandwidth		EUTRA-Transmission-Bandwidth	OPTIONAL,

dL-Transmission-Bandwidth		EUTRA-Transmission-Bandwidth,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-Coex-FDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-Coex-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Coex-Mode-Info ::= CHOICE {

fDD		EUTRA-Coex-FDD-Info,

tDD		EUTRA-Coex-TDD-Info,

...

}

EUTRA-Coex-TDD-Info ::= SEQUENCE {

eARFCN							ExtendedEARFCN,

transmission-Bandwidth			EUTRA-Transmission-Bandwidth,

subframeAssignment				EUTRA-SubframeAssignment,

specialSubframe-Info			EUTRA-SpecialSubframe-Info,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-Coex-TDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-Coex-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-CyclicPrefixDL ::= ENUMERATED {

normal,

extended,

...

}

EUTRA-CyclicPrefixUL ::= ENUMERATED {

normal,

extended,

...

}

EUTRA-PRACH-Configuration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							BOOLEAN,

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)		OPTIONAL,

-- C-ifTDD: This IE shall be present if the EUTRA-Mode-Info IE in the Resource Coordination E-UTRA Cell Information IE is set to the value "TDD"

iE-Extensions							ProtocolExtensionContainer { {EUTRA-PRACH-Configuration-ExtIEs} }	OPTIONAL,

...

}

EUTRA-PRACH-Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-SpecialSubframe-Info ::= SEQUENCE {

specialSubframePatterns		EUTRA-SpecialSubframePatterns,

cyclicPrefixDL				EUTRA-CyclicPrefixDL,

cyclicPrefixUL				EUTRA-CyclicPrefixUL,

iE-Extensions				ProtocolExtensionContainer { { EUTRA-SpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-SpecialSubframe-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-SpecialSubframePatterns ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

ssp10,

...

}

EUTRA-SubframeAssignment ::= ENUMERATED {

sa0,

sa1,

sa2,

sa3,

sa4,

sa5,

sa6,

...

}

EUTRA-Transmission-Bandwidth ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

EUTRANQoS	::= SEQUENCE {

qCI								QCI,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

gbrQosInformation				GBR-QosInformation									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { EUTRANQoS-ExtIEs} }	OPTIONAL,

...

}

EUTRANQoS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ENBDLTNLAddress		CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional	},

...

}

ExecuteDuplication ::= ENUMERATED{true,...}

ExtendedEARFCN ::= INTEGER (0..262143)

EUTRA-Mode-Info ::= CHOICE {

eUTRAFDD		EUTRA-FDD-Info,

eUTRATDD		EUTRA-TDD-Info,

choice-extension	ProtocolIE-SingleContainer { { EUTRA-Mode-Info-ExtIEs} }

}

EUTRA-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

EUTRA-NR-CellResourceCoordinationReq-Container	::= OCTET STRING

EUTRA-NR-CellResourceCoordinationReqAck-Container	::= OCTET STRING

EUTRA-FDD-Info ::= SEQUENCE {

uL-offsetToPointA				OffsetToPointA,

dL-offsetToPointA				OffsetToPointA,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-FDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-TDD-Info ::= SEQUENCE {

offsetToPointA					OffsetToPointA,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-TDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EventType ::= ENUMERATED {

on-demand,

periodic,

stop,

...

}

ExtendedPacketDelayBudget ::= INTEGER (1..65535, ..., 65536..109999)

Expected-UL-AoA ::= SEQUENCE {

expected-Azimuth-AoA	Expected-Azimuth-AoA,

expected-Zenith-AoA		Expected-Zenith-AoA		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { Expected-UL-AoA-ExtIEs } }	OPTIONAL,

...

}

Expected-UL-AoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-ZoA-only ::= SEQUENCE {

expected-ZoA-only	Expected-Zenith-AoA,

iE-extensions		ProtocolExtensionContainer { { Expected-ZoA-only-ExtIEs } }	OPTIONAL,

...

}

Expected-ZoA-only-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-Azimuth-AoA ::= SEQUENCE {

expected-Azimuth-AoA-value			Expected-Value-AoA,

expected-Azimuth-AoA-uncertainty	Uncertainty-range-AoA,

iE-Extensions						ProtocolExtensionContainer { { Expected-Azimuth-AoA-ExtIEs } }		OPTIONAL,

...

}

Expected-Azimuth-AoA-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-Zenith-AoA ::= SEQUENCE {

expected-Zenith-AoA-value			Expected-Value-ZoA,

expected-Zenith-AoA-uncertainty		Uncertainty-range-ZoA,

iE-Extensions						ProtocolExtensionContainer { { Expected-Zenith-AoA-ExtIEs } }		OPTIONAL,

...

}

Expected-Zenith-AoA-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-Value-AoA ::= INTEGER (0..3599)

Expected-Value-ZoA ::= INTEGER (0..1799)

ECNMarkingorCongestionInformationReportingRequest ::= CHOICE {

ecnMarking			ECNmarkingRequest,

congestionInformation	CongestionInformationRequest,

choice-extension		ProtocolIE-SingleContainer { { ECNMarkingorCongestionInformationReportingRequest-ExtIEs } }

}

ECNMarkingorCongestionInformationReportingRequest-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ECNmarkingRequest ::= ENUMERATED { ul, dl, both, stop, ... }

CongestionInformationRequest ::= ENUMERATED { ul, dl, both, stop, ... }

ECNMarkingorCongestionInformationReportingStatus ::= ENUMERATED { active, not-active, ...}

#### -- F

F1CPathNSA ::= ENUMERATED {lte, nr, both}

F1CTransferPath ::= SEQUENCE {

f1CPathNSA						F1CPathNSA,

iE-Extensions					ProtocolExtensionContainer { { F1CTransferPath-ExtIEs} } OPTIONAL,

...

}

F1CTransferPath-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

F1CPathNRDC ::= ENUMERATED {mcg, scg, both}

F1CTransferPathNRDC ::= SEQUENCE {

f1CPathNRDC						F1CPathNRDC,

iE-Extensions					ProtocolExtensionContainer { { F1CTransferPathNRDC-ExtIEs} } OPTIONAL,

...

}

F1CTransferPathNRDC-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

F1UTunnelNotEstablished ::= ENUMERATED {

true,

...

}

FDD-Info ::= SEQUENCE {

uL-NRFreqInfo						NRFreqInfo,

dL-NRFreqInfo						NRFreqInfo,

uL-Transmission-Bandwidth			Transmission-Bandwidth,

dL-Transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions						ProtocolExtensionContainer { {FDD-Info-ExtIEs} } OPTIONAL,

...

}

FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ULCarrierList				CRITICALITY ignore	EXTENSION NRCarrierList				PRESENCE optional }|

{ ID id-DLCarrierList				CRITICALITY ignore EXTENSION NRCarrierList				PRESENCE optional },

...

}

FDD-InfoRel16 ::= SEQUENCE {

uL-FreqInfo										FreqInfoRel16													OPTIONAL,

sUL-FreqInfo									FreqInfoRel16													OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {FDD-InfoRel16-ExtIEs} }		OPTIONAL,

...

}

FDD-InfoRel16-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FiveG-ProSeAuthorized ::= SEQUENCE {

fiveG-proSeDirectDiscovery					FiveG-ProSeDirectDiscovery									OPTIONAL,

fiveG-proSeDirectCommunication				FiveG-ProSeDirectCommunication								OPTIONAL,

fiveG-ProSeLayer2UEtoNetworkRelay			FiveG-ProSeLayer2UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer3UEtoNetworkRelay			FiveG-ProSeLayer3UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer2RemoteUE					FiveG-ProSeLayer2RemoteUE									OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { {FiveG-ProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

FiveG-ProSeAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-FiveG-ProSeLayer2Multipath	CRITICALITY ignore	EXTENSION FiveG-ProSeLayer2Multipath		PRESENCE optional		}|

{ ID id-FiveG-ProSeLayer2UEtoUERelay	CRITICALITY ignore	EXTENSION FiveG-ProSeLayer2UEtoUERelay		PRESENCE optional		}|

{ ID id-FiveG-ProSeLayer2UEtoUERemote	CRITICALITY ignore	EXTENSION FiveG-ProSeLayer2UEtoUERemote		PRESENCE optional		},

...

}

FiveG-ProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer3UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2RemoteUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2Multipath ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoUERelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoUERemote ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveQI ::= INTEGER (0..255, ...)

Flows-Mapped-To-DRB-List	::=	SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF Flows-Mapped-To-DRB-Item

Flows-Mapped-To-DRB-Item 	::= SEQUENCE {

qoSFlowIdentifier							QoSFlowIdentifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions							ProtocolExtensionContainer { { Flows-Mapped-To-DRB-ItemExtIEs} } OPTIONAL

}

Flows-Mapped-To-DRB-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoSFlowMappingIndication			PRESENCE optional}|

{ID id-TSCTrafficCharacteristics	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics			PRESENCE optional},

...

}

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw160, bw200}

FR2-Bandwidth ::= ENUMERATED {bw50, bw100, bw200, bw400, ..., bw800, bw1600, bw2000, bw600}

FreqBandNrItem ::= SEQUENCE {

freqBandIndicatorNr 		INTEGER (1..1024,...),

supportedSULBandList		SEQUENCE (SIZE(0..maxnoofNrCellBands)) OF SupportedSULFreqBandItem,

iE-Extensions				ProtocolExtensionContainer { {FreqBandNrItem-ExtIEs} } OPTIONAL,

...

}

FreqBandNrItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

FreqDomainLength ::= CHOICE {

l839							L839Info,

l139							L139Info,

choice-extension				ProtocolIE-SingleContainer { {FreqDomainLength-ExtIEs} }

}

FreqDomainLength-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-L571Info	CRITICALITY reject	TYPE L571Info PRESENCE mandatory}|

{ ID id-L1151Info	CRITICALITY reject	TYPE L1151Info PRESENCE mandatory},

...

}

FreqInfoRel16 ::=  SEQUENCE {

nRARFCN											INTEGER (0..maxNRARFCN)											OPTIONAL,

frequencyShift7p5khz							FrequencyShift7p5khz											OPTIONAL,

carrierList										NRCarrierList													OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { FreqInfoRel16-ExtIEs} }		OPTIONAL,

...

}

FreqInfoRel16-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

FrequencyShift7p5khz ::= ENUMERATED {false, true, ...}

Frequency-Domain-HSNA-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofRBsetsPerCell)) OF Frequency-Domain-HSNA-Configuration-Item

Frequency-Domain-HSNA-Configuration-Item::= SEQUENCE {

rBSetIndex 		    INTEGER (0..maxnoofRBsetsPerCell-1, ...),

frequency-Domain-HSNA-Slot-Configuration-List				Frequency-Domain-HSNA-Slot-Configuration-List,

iE-Extensions		ProtocolExtensionContainer { { Frequency-Domain-HSNA-Configuration-Item-ExtIEs} } OPTIONAL

}

Frequency-Domain-HSNA-Configuration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Frequency-Domain-HSNA-Slot-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF Frequency-Domain-HSNA-Slot-Configuration-Item

Frequency-Domain-HSNA-Slot-Configuration-Item::= SEQUENCE {

slotIndex				INTEGER (0..5119)		OPTIONAL,

hSNADownlink			HSNADownlink 			OPTIONAL,

hSNAUplink				HSNAUplink 				OPTIONAL,

hSNAFlexible			HSNAFlexible 			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { Frequency-Domain-HSNA-Slot-Configuration-Item-ExtIEs } } OPTIONAL

}

Frequency-Domain-HSNA-Slot-Configuration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FullConfiguration ::= ENUMERATED {full, ...}

FlowsMappedToSLDRB-List ::= SEQUENCE (SIZE(1.. maxnoofPC5QoSFlows)) OF FlowsMappedToSLDRB-Item

FlowsMappedToSLDRB-Item ::= SEQUENCE {

pc5QoSFlowIdentifier			PC5QoSFlowIdentifier,

iE-Extensions					ProtocolExtensionContainer { {FlowsMappedToSLDRB-Item-ExtIEs} } OPTIONAL,

...

}

FlowsMappedToSLDRB-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ 	ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

CG-Config ::= OCTET STRING

GeographicalCoordinates ::= SEQUENCE {

tRPPositionDefinitionType	TRPPositionDefinitionType,

dLPRSResourceCoordinates	DLPRSResourceCoordinates	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GeographicalCoordinates-ExtIEs } } OPTIONAL

}

GeographicalCoordinates-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ARPLocationInfo		CRITICALITY ignore EXTENSION ARPLocationInformation	PRESENCE optional},

...

}

GlobalGNB-ID ::= SEQUENCE {

pLMNIdentity		PLMN-Identity,

gNB-ID				GNB-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalGNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalGNB-ID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID ::= CHOICE {

gNB-ID					BIT STRING (SIZE(22..32)),

choice-Extensions		ProtocolIE-SingleContainer { {GNB-ID-ExtIEs} }

}

GNB-ID-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

GNB-CU-MBS-F1AP-ID		::= INTEGER (0..4294967295)

GNBCUMeasurementID ::= INTEGER (0.. 4095, ...)

GNBDUMeasurementID ::= INTEGER (0.. 4095, ...)

GNB-CUSystemInformation::= SEQUENCE {

sibtypetobeupdatedlist	SEQUENCE (SIZE(1.. maxnoofSIBTypes)) OF SibtypetobeupdatedListItem,

iE-Extensions					ProtocolExtensionContainer { { GNB-CUSystemInformation-ExtIEs} } OPTIONAL,

...

}

GNB-CUSystemInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-systemInformationAreaID  CRITICALITY ignore	EXTENSION SystemInformationAreaID PRESENCE optional},

...

}

GNB-CU-TNL-Association-Setup-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-TNL-Association-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-Failed-To-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-TNL-Association-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-To-Add-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-To-Add-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-TNL-Association-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-To-Remove-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressgNBDU	CRITICALITY reject	EXTENSION CP-TransportLayerAddress	PRESENCE optional},

...

}

GNB-CU-TNL-Association-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

tNLAssociationUsage						TNLAssociationUsage OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-To-Update-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-To-Update-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UE-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-Cell-Resource-Configuration	::= SEQUENCE {

subcarrierSpacing				SubcarrierSpacing,

dUFTransmissionPeriodicity		DUFTransmissionPeriodicity	OPTIONAL,

dUF-Slot-Config-List			DUF-Slot-Config-List	OPTIONAL,

hSNATransmissionPeriodicity		HSNATransmissionPeriodicity,

hsNSASlotConfigList				HSNASlotConfigList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-Cell-Resource-Configuration-ExtIEs } } OPTIONAL

}

GNB-DU-Cell-Resource-Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-rBSetConfiguration       CRITICALITY reject	EXTENSION       RBSetConfiguration	PRESENCE optional}|

{ID id-frequency-Domain-HSNA-Configuration-List	 CRITICALITY reject	EXTENSION    Frequency-Domain-HSNA-Configuration-List   PRESENCE optional}|

{ID id-child-IAB-Nodes-NA-Resource-List	CRITICALITY reject	EXTENSION Child-IAB-Nodes-NA-Resource-List    PRESENCE optional}|

{ID id-Parent-IAB-Nodes-NA-Resource-Configuration-List   CRITICALITY reject	EXTENSION  Parent-IAB-Nodes-NA-Resource-Configuration-List  PRESENCE optional},

...

}

GNB-DU-MBS-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-UE-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-ID			::= INTEGER (0..68719476735)

GNB-CU-Name ::= PrintableString(SIZE(1..150,...))

GNB-DU-Name ::= PrintableString(SIZE(1..150,...))

Extended-GNB-CU-Name	 ::= SEQUENCE {

gNB-CU-NameVisibleString		GNB-CU-NameVisibleString					OPTIONAL,

gNB-CU-NameUTF8String			GNB-CU-NameUTF8String						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Extended-GNB-CU-Name-ExtIEs } } OPTIONAL,

...

}

Extended-GNB-CU-Name-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-NameVisibleString ::= VisibleString(SIZE(1..150,...))

GNB-CU-NameUTF8String ::= UTF8String(SIZE(1..150,...))

Extended-GNB-DU-Name	 ::= SEQUENCE {

gNB-DU-NameVisibleString		GNB-DU-NameVisibleString					OPTIONAL,

gNB-DU-NameUTF8String			GNB-DU-NameUTF8String						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Extended-GNB-DU-Name-ExtIEs } } OPTIONAL,

...

}

Extended-GNB-DU-Name-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-NameVisibleString ::= VisibleString(SIZE(1..150,...))

GNB-DU-NameUTF8String ::= UTF8String(SIZE(1..150,...))

GNB-DU-Served-Cells-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

gNB-DU-System-Information	GNB-DU-System-Information	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GNB-DU-Served-Cells-ItemExtIEs} }	OPTIONAL,

...

}

GNB-DU-Served-Cells-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-System-Information ::= SEQUENCE {

mIB-message		MIB-message,

sIB1-message		SIB1-message,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-System-Information-ExtIEs } } OPTIONAL,

...

}

GNB-DU-System-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SIB12-message		CRITICALITY ignore	EXTENSION SIB12-message		PRESENCE optional}|

{ ID id-SIB13-message		CRITICALITY ignore	EXTENSION SIB13-message		PRESENCE optional}|

{ ID id-SIB14-message		CRITICALITY ignore	EXTENSION SIB14-message		PRESENCE optional}|

{ ID id-SIB10-message		CRITICALITY ignore	EXTENSION SIB10-message		PRESENCE optional}|

{ ID id-SIB17-message		CRITICALITY ignore	EXTENSION SIB17-message		PRESENCE optional}|

{ ID id-SIB20-message		CRITICALITY ignore	EXTENSION SIB20-message		PRESENCE optional}|

{ ID id-SIB15-message		CRITICALITY ignore	EXTENSION SIB15-message		PRESENCE optional}|

{ ID id-SIB24-message		CRITICALITY ignore	EXTENSION SIB24-message		PRESENCE optional}|

{ ID id-SIB22-message		CRITICALITY ignore	EXTENSION SIB22-message		PRESENCE optional},

...

}

GNB-DUConfigurationQuery ::= ENUMERATED {true, ...}

GNBDUOverloadInformation ::= ENUMERATED {overloaded, not-overloaded}

GNB-DU-TNL-Association-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress				CP-TransportLayerAddress	,

tNLAssociationTransportLayerAddressgNBCU		CP-TransportLayerAddress		OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { GNB-DU-TNL-Association-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-DU-TNL-Association-To-Remove-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNBDUUESliceMaximumBitRateList::= SEQUENCE (SIZE(1.. maxnoofSMBRValues)) OF GNBDUUESliceMaximumBitRateItem

GNBDUUESliceMaximumBitRateItem::= SEQUENCE {

sNSSAI		SNSSAI,

uESliceMaximumBitRateUL		BitRate,

iE-Extensions				ProtocolExtensionContainer { { GNBDUUESliceMaximumBitRateItem-ExtIEs} } OPTIONAL,

...

}

GNBDUUESliceMaximumBitRateItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-RxTxTimeDiff ::= SEQUENCE {

rxTxTimeDiff			GNBRxTxTimeDiffMeas,

additionalPath-List		AdditionalPath-List		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { GNB-RxTxTimeDiff-ExtIEs} }  OPTIONAL

}

GNB-RxTxTimeDiff-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList		CRITICALITY ignore EXTENSION ExtendedAdditionalPathList 	PRESENCE optional}|

{ ID id-TRPTEGInformation				CRITICALITY ignore EXTENSION TRPTEGInformation			PRESENCE optional },

...

}

GNBRxTxTimeDiffMeas ::= CHOICE {

k0			INTEGER (0.. 1970049),

k1			INTEGER (0.. 985025),

k2			INTEGER (0.. 492513),

k3			INTEGER (0.. 246257),

k4			INTEGER (0.. 123129),

k5			INTEGER (0.. 61565),

choice-extension		ProtocolIE-SingleContainer { { GNBRxTxTimeDiffMeas-ExtIEs } }

}

GNBRxTxTimeDiffMeas-ExtIEs		F1AP-PROTOCOL-IES ::= {

{ID id-ReportingGranularitykminus1 	CRITICALITY ignore TYPE ReportingGranularitykminus1 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus2 	CRITICALITY ignore TYPE ReportingGranularitykminus2 PRESENCE mandatory }|

{ID id-ReportingGranularitykminus3 	CRITICALITY ignore TYPE ReportingGranularitykminus3 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus4 	CRITICALITY ignore TYPE ReportingGranularitykminus4 PRESENCE mandatory }|

{ID id-ReportingGranularitykminus5 	CRITICALITY ignore TYPE ReportingGranularitykminus5 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus6 	CRITICALITY ignore TYPE ReportingGranularitykminus6 PRESENCE mandatory },

...

}

GNBSetID ::= BIT STRING (SIZE(22))

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddress				TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }			OPTIONAL

}

GTPTLA-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GTPTunnel				::= SEQUENCE {

transportLayerAddress		TransportLayerAddress,

gTP-TEID		GTP-TEID,

iE-Extensions				ProtocolExtensionContainer { { GTPTunnel-ExtIEs } } OPTIONAL,

...

}

GTPTunnel-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- H

HandoverPreparationInformation ::= OCTET STRING

HardwareLoadIndicator ::= SEQUENCE {

dLHardwareLoadIndicator			INTEGER (0..100, ...),

uLHardwareLoadIndicator			INTEGER (0..100, ...),

iE-Extensions					ProtocolExtensionContainer { { HardwareLoadIndicator-ExtIEs } } 	OPTIONAL,

...

}

HardwareLoadIndicator-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

HSNASlotConfigList ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF HSNASlotConfigItem

HSNASlotConfigItem 	::=	SEQUENCE {

hSNADownlink			HSNADownlink 		OPTIONAL,

hSNAUplink				HSNAUplink 			OPTIONAL,

hSNAFlexible			HSNAFlexible 		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { HSNASlotConfigItem-ExtIEs } } OPTIONAL

}

HSNASlotConfigItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

HSNADownlink ::= ENUMERATED { hard, soft, notavailable }

HSNAFlexible ::= ENUMERATED { hard, soft, notavailable }

HSNAUplink ::= ENUMERATED { hard, soft, notavailable }

HSNATransmissionPeriodicity ::=	ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, ...}

HashedUEIdentityIndexValue ::= BIT STRING (SIZE(13, ...))

#### -- I

IAB-Barred	::=	ENUMERATED {barred, not-barred, ...}

IABConditionalRRCMessageDeliveryIndication ::= ENUMERATED {true, ...}

IABCongestionIndication ::= SEQUENCE {

iAB-Congestion-Indication-List			IAB-Congestion-Indication-List,

iE-Extensions	ProtocolExtensionContainer { { IAB-Congestion-Indication-List-ExtIEs } } OPTIONAL

}

IAB-Congestion-Indication-List-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Congestion-Indication-List ::= SEQUENCE (SIZE(1..maxnoofIABCongInd)) OF IAB-Congestion-Indication-Item

IAB-Congestion-Indication-Item ::= SEQUENCE {

childNodeIdentifier				BAPAddress,

bHRLCCHList		                BHRLCCHList	    OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IAB-Congestion-Indication-ItemExtIEs } }	OPTIONAL

}

IAB-Congestion-Indication-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Info-IAB-donor-CU ::=	SEQUENCE{

iAB-STC-Info	IAB-STC-Info	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IAB-Info-IAB-donor-CU-ExtIEs } } OPTIONAL

}

IAB-Info-IAB-donor-CU-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Info-IAB-DU ::=	SEQUENCE{

multiplexingInfo		MultiplexingInfo	OPTIONAL,

iAB-STC-Info		IAB-STC-Info	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { IAB-Info-IAB-DU-ExtIEs } } OPTIONAL

}

IAB-Info-IAB-DU-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-MT-Cell-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF IAB-MT-Cell-List-Item

IAB-MT-Cell-List-Item ::= 	SEQUENCE {

nRCellIdentity				NRCellIdentity,

dU-RX-MT-RX					DU-RX-MT-RX,

dU-TX-MT-TX					DU-TX-MT-TX,

dU-RX-MT-TX					DU-RX-MT-TX,

dU-TX-MT-RX					DU-TX-MT-RX,

iE-Extensions				ProtocolExtensionContainer { { IAB-MT-Cell-List-Item-ExtIEs } } OPTIONAL

}

IAB-MT-Cell-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DU-RX-MT-RX-Extend			CRITICALITY ignore	EXTENSION DU-RX-MT-RX-Extend	PRESENCE optional	}|

{ ID id-DU-TX-MT-TX-Extend			CRITICALITY ignore	EXTENSION DU-TX-MT-TX-Extend	PRESENCE optional	}|

{ ID id-DU-RX-MT-TX-Extend			CRITICALITY ignore	EXTENSION DU-RX-MT-TX-Extend	PRESENCE optional	}|

{ ID id-DU-TX-MT-RX-Extend			CRITICALITY ignore	EXTENSION DU-TX-MT-RX-Extend	PRESENCE optional	},

...

}

IAB-MT-Cell-NA-Resource-Configuration-Mode-Info	::=	CHOICE {

fDD		IAB-MT-Cell-NA-Resource-Configuration-FDD-Info,

tDD		IAB-MT-Cell-NA-Resource-Configuration-TDD-Info,

choice-extension			ProtocolIE-SingleContainer { { IAB-MT-Cell-NA-Resource-Configuration-Mode-Info-ExtIEs} }

}

IAB-MT-Cell-NA-Resource-Configuration-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IAB-MT-Cell-NA-Resource-Configuration-FDD-Info ::= SEQUENCE {

gNB-DU-Cell-NA-Resource-Configuration-FDD-UL				GNB-DU-Cell-Resource-Configuration,

gNB-DU-Cell-NA-Resource-Configuration-FDD-DL				GNB-DU-Cell-Resource-Configuration,

uL-FreqInfo							NRFreqInfo					OPTIONAL,

uL-Transmission-Bandwidth			Transmission-Bandwidth		OPTIONAL,

uL-NR-Carrier-List  				NRCarrierList				OPTIONAL,

dL-FreqInfo							NRFreqInfo					OPTIONAL,

dL-Transmission-Bandwidth			Transmission-Bandwidth		OPTIONAL,

dL-NR-Carrier-List  				NRCarrierList				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {IAB-MT-Cell-NA-Resource-Configuration-FDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-MT-Cell-NA-Resource-Configuration-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-MT-Cell-NA-Resource-Configuration-TDD-Info ::= SEQUENCE {

gNB-DU-Cell-NA-Resourc-Configuration-TDD				GNB-DU-Cell-Resource-Configuration,

nRFreqInfo							NRFreqInfo  	OPTIONAL,

transmission-Bandwidth			    Transmission-Bandwidth  	  OPTIONAL,

nR-Carrier-List   		  	        NRCarrierList  	OPTIONAL,

iE-Extensions					    ProtocolExtensionContainer { {IAB-MT-Cell-NA-Resource-Configuration-TDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-MT-Cell-NA-Resource-Configuration-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-STC-Info	::=	SEQUENCE{

iAB-STC-Info-List	IAB-STC-Info-List,

iE-Extensions		ProtocolExtensionContainer { { IAB-STC-Info-ExtIEs } } OPTIONAL

}

IAB-STC-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-STC-Info-List ::= 	SEQUENCE (SIZE(1..maxnoofIABSTCInfo)) OF IAB-STC-Info-Item

IAB-STC-Info-Item::=	SEQUENCE {

sSB-freqInfo						SSB-freqInfo,

sSB-subcarrierSpacing				SSB-subcarrierSpacing,

sSB-transmissionPeriodicity			SSB-transmissionPeriodicity,

sSB-transmissionTimingOffset		SSB-transmissionTimingOffset,

sSB-transmissionBitmap				SSB-transmissionBitmap,

iE-Extensions		ProtocolExtensionContainer { { IAB-STC-Info-Item-ExtIEs } } OPTIONAL

}

IAB-STC-Info-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Allocated-TNL-Address-Item	::= SEQUENCE {

iABTNLAddress				IABTNLAddress,

iABTNLAddressUsage			IABTNLAddressUsage	 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { IAB-Allocated-TNL-Address-Item-ExtIEs } } OPTIONAL

}

IAB-Allocated-TNL-Address-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-DU-Cell-Resource-Configuration-Mode-Info	::=	CHOICE {

fDD		IAB-DU-Cell-Resource-Configuration-FDD-Info,

tDD		IAB-DU-Cell-Resource-Configuration-TDD-Info,

choice-extension			ProtocolIE-SingleContainer { { IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs} }

}

IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IAB-DU-Cell-Resource-Configuration-FDD-Info ::= SEQUENCE {

gNB-DU-Cell-Resource-Configuration-FDD-UL				GNB-DU-Cell-Resource-Configuration,

gNB-DU-Cell-Resource-Configuration-FDD-DL				GNB-DU-Cell-Resource-Configuration,

iE-Extensions					ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-uL-FreqInfo					CRITICALITY reject	EXTENSION	NRFreqInfo					PRESENCE optional}|

{ID id-uL-Transmission-Bandwidth	CRITICALITY reject	EXTENSION	Transmission-Bandwidth		PRESENCE optional}|

{ID id-uL-NR-Carrier-List			CRITICALITY reject	EXTENSION	NRCarrierList				PRESENCE optional}|

{ID id-dL-FreqInfo					CRITICALITY reject	EXTENSION	NRFreqInfo  				PRESENCE optional}|

{ID id-dL-Transmission-Bandwidth	CRITICALITY reject	EXTENSION	Transmission-Bandwidth		PRESENCE optional}|

{ID id-dL-NR-Carrier-List			CRITICALITY reject	EXTENSION	NRCarrierList				PRESENCE optional},

...

}

IAB-DU-Cell-Resource-Configuration-TDD-Info ::= SEQUENCE {

gNB-DU-Cell-Resourc-Configuration-TDD				GNB-DU-Cell-Resource-Configuration,

iE-Extensions					ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-nRFreqInfo				CRITICALITY reject	EXTENSION  NRFreqInfo  				PRESENCE optional}|

{ID id-transmission-Bandwidth	CRITICALITY reject	EXTENSION  Transmission-Bandwidth  	PRESENCE optional}|

{ID id-nR-Carrier-List			CRITICALITY reject	EXTENSION  NRCarrierList  			PRESENCE optional},

...

}

IABIPv6RequestType	 ::= CHOICE {

iPv6Address						IABTNLAddressesRequested,

iPv6Prefix						IABTNLAddressesRequested,

choice-extension				ProtocolIE-SingleContainer { { IABIPv6RequestType-ExtIEs} }

}

IABIPv6RequestType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IABTNLAddress ::= CHOICE {

iPv4Address						BIT STRING (SIZE(32)),

iPv6Address						BIT STRING (SIZE(128)),

iPv6Prefix						BIT STRING (SIZE(64)),

choice-extension				ProtocolIE-SingleContainer { { IABTNLAddress-ExtIEs} }

}

IABTNLAddress-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IABTNLAddressesRequested ::= SEQUENCE {

tNLAddressesOrPrefixesRequestedAllTraffic	INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-C			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-U			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedNoNF1		INTEGER (1..256) 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { IABTNLAddressesRequested-ExtIEs } } OPTIONAL

}

IABTNLAddressesRequested-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-TNL-Addresses-To-Remove-Item ::= SEQUENCE {

iABTNLAddress			IABTNLAddress,

iE-Extensions			ProtocolExtensionContainer { { IAB-TNL-Addresses-To-Remove-Item-ExtIEs} } OPTIONAL

}

IAB-TNL-Addresses-To-Remove-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-TNL-Addresses-Exception ::= 	SEQUENCE {

iABTNLAddressList			IABTNLAddressList,

iE-Extensions				ProtocolExtensionContainer { { IAB-TNL-Addresses-Exception-ExtIEs} } OPTIONAL

}

IAB-TNL-Addresses-Exception-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressList ::= SEQUENCE (SIZE(1.. maxnoofTLAsIAB)) OF IABTNLAddress-Item

IABTNLAddress-Item ::= SEQUENCE {

iABTNLAddress		IABTNLAddress	,

iE-Extensions	ProtocolExtensionContainer { { IABTNLAddress-ItemExtIEs } }	OPTIONAL

}

IABTNLAddress-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressUsage ::= ENUMERATED {

f1-c,

f1-u,

non-f1,

...

}

IABv4AddressesRequested ::= SEQUENCE {

iABv4AddressesRequested			IABTNLAddressesRequested,

iE-Extensions		ProtocolExtensionContainer { { IABv4AddressesRequested-ExtIEs} } OPTIONAL

}

IABv4AddressesRequested-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Mobile-IAB-MTUserLocationInformation ::= SEQUENCE {

nRCGI							NRCGI,

tAI                             TAI,

iE-Extensions					ProtocolExtensionContainer { { Mobile-IAB-MTUserLocationInformation-ExtIEs} }	OPTIONAL

}

Mobile-IAB-MTUserLocationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ImplicitFormat	::= SEQUENCE	{

dUFSlotformatIndex 			DUFSlotformatIndex,

iE-Extensions		ProtocolExtensionContainer { { ImplicitFormat-ExtIEs } } OPTIONAL

}

ImplicitFormat-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IgnorePRACHConfiguration::= ENUMERATED { true,...}

IgnoreResourceCoordinationContainer ::= ENUMERATED { yes,...}

InactivityMonitoringRequest ::= ENUMERATED { true,...}

InactivityMonitoringResponse ::= ENUMERATED { not-supported,...}

IndirectPathAddition ::= SEQUENCE {

targetRelayUEID			BIT STRING(SIZE(24)),

remoteUELocalID			RemoteUELocalID,

iE-Extensions			ProtocolExtensionContainer { { IndirectPathAddition-ExtIEs } }		OPTIONAL,

...

}

IndirectPathAddition-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

InterfacesToTrace ::= BIT STRING (SIZE(8))

IntendedTDD-DL-ULConfig ::= SEQUENCE {

nRSCS						ENUMERATED { scs15, scs30, scs60, scs120,..., scs480, scs960},

nRCP						ENUMERATED { normal, extended,...},

nRDLULTxPeriodicity			ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, ...},

slot-Configuration-List 	Slot-Configuration-List,

iE-Extensions						ProtocolExtensionContainer { {IntendedTDD-DL-ULConfig-ExtIEs} } OPTIONAL

}

InterFrequencyConfig-NoGap ::= ENUMERATED {

true,

...

}

IngressNonF1terminatingTopologyIndicator ::= ENUMERATED {true, ...}

IntendedTDD-DL-ULConfig-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

IndicationMCInactiveReception ::= ENUMERATED {true, ...}

IPHeaderInformation ::= SEQUENCE {

destinationIABTNLAddress			IABTNLAddress,

dsInformationList					DSInformationList	OPTIONAL,

iPv6FlowLabel						BIT STRING (SIZE (20))	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { IPHeaderInformation-ItemExtIEs} } OPTIONAL,

...

}

IPHeaderInformation-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IPtolayer2TrafficMappingInfo ::= SEQUENCE {

iPtolayer2TrafficMappingInfoToAdd					IPtolayer2TrafficMappingInfoList		OPTIONAL,

iPtolayer2TrafficMappingInfoToRemove				MappingInformationtoRemove				OPTIONAL,

iE-Extensions										ProtocolExtensionContainer { { IPtolayer2TrafficMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

IPtolayer2TrafficMappingInfoList ::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF IPtolayer2TrafficMappingInfo-Item

IPtolayer2TrafficMappingInfo-Item ::= SEQUENCE {

mappingInformationIndex		MappingInformationIndex,

iPHeaderInformation			IPHeaderInformation,

bHInfo	 					BHInfo,	iE-Extensions				ProtocolExtensionContainer { { IPtolayer2TrafficMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

IPtolayer2TrafficMappingInfo-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- J

JointorDLTCIStatesConfigurationsList ::= SEQUENCE (SIZE (1.. maxnoofJointorDLTCIStates)) OF JointorDLTCIStatesConfigurations-Item

JointorDLTCIStatesConfigurations-Item::= SEQUENCE {

jointorDLTCIState			OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { JointorDLTCIStatesConfigurations-Item-ExtIEs} } OPTIONAL

}

JointorDLTCIStatesConfigurations-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

JointorDLTCIStateID  ::= OCTET STRING

ULTCIStatesConfigurationsList ::= SEQUENCE (SIZE (1.. maxnoofULTCIStates)) OF ULTCIStatesConfigurations-Item

ULTCIStatesConfigurations-Item::= SEQUENCE {

uLTCIStatesConfigurations	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { ULTCIStatesConfigurations-Item-ExtIEs} } OPTIONAL

}

ULTCIStatesConfigurations-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- K

#### -- L

LTEA2XServicesAuthorized ::= SEQUENCE {

aerialUE			AerialUE														OPTIONAL,

controllerUE 		ControllerUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTEA2XServicesAuthorized-ExtIEs} }		OPTIONAL

}

LTEA2XServicesAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

L139Info ::= SEQUENCE {

prachSCS						ENUMERATED {scs15, scs30, scs60, scs120, ..., scs480, scs960},

rootSequenceIndex			INTEGER (0..137)								OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {L139Info-ExtIEs} } 		OPTIONAL,

...

}

L139Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

L839Info ::= SEQUENCE {

rootSequenceIndex			INTEGER (0..837),

restrictedSetConfig			ENUMERATED {unrestrictedSet, restrictedSetTypeA,

restrictedSetTypeB, ...},

iE-Extension		ProtocolExtensionContainer { {L839Info-ExtIEs} } 		OPTIONAL,

...

}

L839Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

L571Info ::= SEQUENCE {

prachSCSForL571				ENUMERATED { scs30, scs120, ... , scs480},

rootSequenceIndex			INTEGER (0..569),

iE-Extension				ProtocolExtensionContainer { {L571Info-ExtIEs} } 		OPTIONAL,

...

}

L571Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

L1151Info ::= SEQUENCE {

prachSCSForL1151				ENUMERATED { scs15, scs120,...},

rootSequenceIndex				INTEGER (0..1149),

iE-Extension					ProtocolExtensionContainer { {L1151Info-ExtIEs} } 		OPTIONAL,

...

}

L1151Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LastUsedCellIndication ::= ENUMERATED {true, ...}

LCID ::= INTEGER (1..32, ...)

LCS-to-GCS-Translation::= SEQUENCE {

alpha				INTEGER (0..3599),

beta				INTEGER (0..3599),

gamma				INTEGER (0..3599),

iE-Extensions		ProtocolExtensionContainer { { LCS-to-GCS-Translation-ExtIEs} } OPTIONAL,

...

}

LCS-to-GCS-Translation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LCStoGCSTranslationList ::= SEQUENCE (SIZE (1.. maxnooflcs-gcs-translation)) OF LCStoGCSTranslation

LCStoGCSTranslation ::= SEQUENCE {

alpha			INTEGER (0..359),

alpha-fine		INTEGER (0..9)		OPTIONAL,

beta			INTEGER (0..359),

beta-fine		INTEGER (0..9)		OPTIONAL,

gamma			INTEGER (0..359),

gamma-fine		INTEGER (0..9)		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {LCStoGCSTranslation-ExtIEs} } OPTIONAL

}

LCStoGCSTranslation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LMF-MeasurementID ::= INTEGER (1.. 65536, ...)

LMF-UE-MeasurementID ::= INTEGER (1.. 256, ...)

LocationDependentMBSF1UInformation ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformation-Item

LocationDependentMBSF1UInformation-Item ::= SEQUENCE {

mbsAreaSession-ID				MBS-Area-Session-ID,

mbs-f1u-info-at-CU				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformation-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformation-Item-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-F1UTunnelNotEstablished	CRITICALITY		ignore		EXTENSION		F1UTunnelNotEstablished		PRESENCE		optional	},

...

}

LocationMeasurementInformation ::= OCTET STRING

LocationUncertainty ::= SEQUENCE {

horizontalUncertainty		INTEGER (0..255),

horizontalConfidence		INTEGER (0..100),

verticalUncertainty			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { LocationUncertainty-ExtIEs} } OPTIONAL

}

LocationUncertainty-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LongDRXCycleLength ::= 	ENUMERATED

{ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...}

LowerLayerPresenceStatusChange ::= ENUMERATED {

suspend-lower-layers,

resume-lower-layers,

...

}

LoS-NLoSIndicatorHard ::= ENUMERATED {nLoS, loS}

LoS-NLoSIndicatorSoft ::= INTEGER (0..10)

LoS-NLoSInformation ::= CHOICE {

loS-NLoSIndicatorSoft		LoS-NLoSIndicatorSoft,

loS-NLoSIndicatorHard		LoS-NLoSIndicatorHard,

choice-Extension			ProtocolIE-SingleContainer {{ LoS-NLoSInformation-ExtIEs}}

}

LoS-NLoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

LTEUESidelinkAggregateMaximumBitrate ::= SEQUENCE {

uELTESidelinkAggregateMaximumBitrate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {LTEUESidelinkAggregateMaximumBitrate-ExtIEs} } OPTIONAL

}

LTEUESidelinkAggregateMaximumBitrate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTEV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE														OPTIONAL,

pedestrianUE 		PedestrianUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTEV2XServicesAuthorized-ExtIEs} }		OPTIONAL

}

LTEV2XServicesAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMCells-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF ProtocolIE-SingleContainer { { LTMCells-ToBeReleased-ItemIEs} }

LTMCells-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-LTMCells-ToBeReleased-Item		CRITICALITY ignore	TYPE LTMCells-ToBeReleased-Item		PRESENCE mandatory},

...

}

LTMCells-ToBeReleased-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { LTMCells-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

LTMCells-ToBeReleased-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMInformation-Setup ::= SEQUENCE {

lTMIndicator						LTMIndicator,

lTMConfigurationID					LTMConfigurationID,

referenceConfiguration				ReferenceConfiguration								OPTIONAL,

cSIResourceConfiguration			CSIResourceConfiguration							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-Setup-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-Setup-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMConfigurationIDMappingList	::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfigurationIDMapping-Item

LTMConfigurationIDMapping-Item::= SEQUENCE{

lTMCellID			NRCGI,

lTMConfigurationID	LTMConfigurationID 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{ LTMConfigurationIDMapping-Item-ExtIEs}}		OPTIONAL

}

LTMConfigurationIDMapping-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMInformation-Modify	::= SEQUENCE {

lTMIndicator						LTMIndicator,

lTMConfigurationID					LTMConfigurationID,

referenceConfiguration				ReferenceConfiguration									OPTIONAL,

cSIResourceConfiguration 			CSIResourceConfiguration								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-Modify-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-Modify-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMIndicator ::= ENUMERATED {true, ...}

CompleteConfigurationIndicator	::= ENUMERATED {true, ...}

LTMConfigurationID ::=  INTEGER (1..8)

ReferenceConfigurationInformation ::= OCTET STRING

LTMConfiguration	::= SEQUENCE {

sSBInformationItem					SSBInformationItem,

referenceConfigurationInformation 	ReferenceConfigurationInformation			OPTIONAL,

completeConfigurationIndicator		CompleteConfigurationIndicator 				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { LTMConfiguration-ExtIEs } }	OPTIONAL,

...

}

LTMConfiguration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMCellSwitchInformation	::= SEQUENCE {

jointorDLTCIStateID			JointorDLTCIStateID 		OPTIONAL,

uLTCIStateID					ULTCIStateID 				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMCellSwitchInformation-ExtIEs } }	OPTIONAL,

...

}

LTMCellSwitchInformation-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMgNB-DU-IDsList	::= SEQUENCE (SIZE(1..maxnoofLTMgNB-DUs)) OF LTMgNB-DU-IDs-Item

LTMgNB-DU-IDs-Item	::= SEQUENCE{

lTMgNB-DU-ID			GNB-DU-ID,

iE-Extensions			ProtocolExtensionContainer {{ LTMgNB-DU-IDs-Item-ExtIEs}}		OPTIONAL

}

LTMgNB-DU-IDs-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- M

MappingInformationIndex	::= BIT STRING (SIZE (26))

MappingInformationtoRemove	::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF MappingInformationIndex

MaskedIMEISV ::= 	BIT STRING (SIZE (64))

MaxDataBurstVolume  ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaxPacketLossRate ::= INTEGER (0..1000)

MBS-Broadcast-NeighbourCellList ::= OCTET STRING

MBS-Flows-Mapped-To-MRB-List	::=	SEQUENCE (SIZE(1.. maxnoofMBSQoSFlows)) OF MBS-Flows-Mapped-To-MRB-Item

MBS-Flows-Mapped-To-MRB-Item 	::= SEQUENCE {

mBS-QoSFlowIdentifier						QoSFlowIdentifier,

mbs-QoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions								ProtocolExtensionContainer { { MBS-Flows-Mapped-To-MRB-Item-ExtIEs} } OPTIONAL

}

MBS-Flows-Mapped-To-MRB-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformation ::= SEQUENCE {

mbs-f1u-info				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformation-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformation-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-F1UTunnelNotEstablished	CRITICALITY		ignore		EXTENSION		F1UTunnelNotEstablished		PRESENCE		optional	},

...

}

MBSInterestIndication ::= OCTET STRING

MBS-Session-ID ::= SEQUENCE {

tMGI						TMGI,

nID							NID					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Session-ID-ExtIEs} } OPTIONAL,

...

}

MBS-Session-ID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBS-CUtoDURRCInformation		::= SEQUENCE {

mBS-Broadcast-Cell-List		MBS-Broadcast-Cell-List,

mBS-Broadcast-MRB-List		MBS-Broadcast-MRB-List,

iE-Extensions				ProtocolExtensionContainer { { MBS-CUtoDURRCInformation-ExtIEs } } OPTIONAL,

...

}

MBS-CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Broadcast-Cell-Item

MBS-Broadcast-Cell-Item ::= SEQUENCE {

nRCGI						NRCGI,

mtch-neighbourCell			OCTET STRING		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Broadcast-MRB-Item

MBS-Broadcast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastF1UContextDescriptor ::= SEQUENCE {

multicastF1UContextReferenceF1		MulticastF1UContextReferenceF1,

mc-F1UCtxtusage 	ENUMERATED {ptm, ptp, ptp-retransmission, ptp-forwarding, ...},

mbsAreaSession						MBS-Area-Session-ID				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}} OPTIONAL,

...

}

MBSMulticastF1UContextDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-Information ::= SEQUENCE {

mt-SDT-Indicator	MT-SDT-Indicator,

iE-Extensions		ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs } } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-Indicator ::= ENUMERATED {true, ...}

MBSMulticastSessionReceptionState ::= ENUMERATED {start-monitoring-G-RNTI, stop-monitoring-G-RNTI, ...}

MulticastCU2DURRCInfo		::= SEQUENCE {

mBS-Multicast-CU2DU-Cell-List		MBS-Multicast-CU2DU-Cell-List 	OPTIONAL,

mBS-Multicast-MRB-List		MBS-Multicast-MRB-List 					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastCU2DURRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastCU2DURRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-CU2DU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Multicast-CU2DU-Cell-Item

MBS-Multicast-CU2DU-Cell-Item ::= SEQUENCE {

nRCGI									NRCGI,

mbsMulticastRRC-INACTIVEReceptionMode	MBSMulticastRRCINACTIVEReceptionMode	OPTIONAL,

mbsMulticastConfigurationRequest		ENUMERATED {query, ...}					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-CU2DU-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-CU2DU-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastRRCINACTIVEReceptionMode ::= ENUMERATED {activated, deactivated, ...}

MBS-Multicast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Multicast-MRB-Item

MBS-Multicast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCU2DUCommonRRCInfo ::= SEQUENCE {

multicastCommonCU2DUCellList		MulticastCommonCU2DUCellList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {MulticastCU2DUCommonRRCInfo-ExtIEs} } OPTIONAL,

...

}

MulticastCU2DUCommonRRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCommonCU2DUCellList ::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF MulticastCommonCU2DUCell-Item

MulticastCommonCU2DUCell-Item ::= SEQUENCE {

nRCGI									NRCGI,

multicastCommonCu2DUCellInformation		MulticastCommonCu2DUCellInformation,

iE-Extensions				ProtocolExtensionContainer { {MulticastCommonCU2DUCell-Item-ExtIEs} } OPTIONAL,

...

}

MulticastCommonCU2DUCell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCommonCu2DUCellInformation ::= SEQUENCE {

mBSMulticastNeighbourCellListItem	MBSMulticastNeighbourCellListItem		OPTIONAL,

thresholdMBS-ListItem				ThresholdMBS-ListItem					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {MulticastCommonCu2DUCellInformation-ExtIEs} } OPTIONAL,

...

}

MulticastCommonCu2DUCellInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastNeighbourCellListItem ::= CHOICE {

mbsMulticastNeighbourCellListInformationprovided		UpdateMBSMulticastNeighbourCellListInformation,

nombsMulticastNeighbourCellListInformationprovided		NULL,

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastNeighbourCellListItem-ExtIEs} }

}

MBSMulticastNeighbourCellListItem-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ThresholdMBS-ListItem ::= CHOICE {

thresholdMBS-ListInformationprovided		UpdateThresholdMBS-ListInformation,

nothresholdMBSListInformationprovided		NULL,

choice-extension		ProtocolIE-SingleContainer { {ThresholdMBS-ListItem-ExtIEs} }

}

ThresholdMBS-ListItem-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UpdateMBSMulticastNeighbourCellListInformation ::= SEQUENCE {

mbs-NeighbourCellList			OCTET STRING						OPTIONAL,

mbs-MulticastSessionList		MTCH-NeighbourCellSessionList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {UpdateMBSMulticastNeighbourCellListInformation-ExtIEs} } OPTIONAL,

...

}

UpdateMBSMulticastNeighbourCellListInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MTCH-NeighbourCellSessionList ::= SEQUENCE (SIZE(1..maxMBSSessionsinSessionInfoList))	OF MTCH-NeighbourCellSession-Item

MTCH-NeighbourCellSession-Item ::= SEQUENCE {

mbsSessionID						MBS-Session-ID,

mtch-NeighbourCellInformation			MTCH-NeighbourCellInformation,

iE-Extensions				ProtocolExtensionContainer { {MTCH-NeighbourCellSession-Item-ExtIEs} } OPTIONAL,

...

}

MTCH-NeighbourCellSession-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MTCH-NeighbourCellInformation ::= CHOICE {

mtch-NeighbourCellprovided					OCTET STRING,

mtch-NeighbourCellnotprovided				NULL,

choice-extension		ProtocolIE-SingleContainer { {MTCH-NeighbourCellInformation-ExtIEs} }

}

MTCH-NeighbourCellInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UpdateThresholdMBS-ListInformation ::= SEQUENCE {

thresholdMBSList				OCTET STRING				OPTIONAL,

thresholdIndexSessionList		ThresholdIndexSessionList	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {UpdateThresholdMBS-ListInformation-ExtIEs} } OPTIONAL,

...

}

UpdateThresholdMBS-ListInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ThresholdIndexSessionList ::= SEQUENCE (SIZE(1..maxMBSSessionsinSessionInfoList))	OF ThresholdIndexSession-Item

ThresholdIndexSession-Item ::= SEQUENCE {

mbsSessionID						MBS-Session-ID,

thresholdIndexInformation			ThresholdIndexInformation,

iE-Extensions				ProtocolExtensionContainer { {ThresholdIndexSession-Item-ExtIEs} } OPTIONAL,

...

}

ThresholdIndexSession-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ThresholdIndexInformation ::= CHOICE {

thresholdIndexprovided				ThresholdIndex,

thresholdIndexnotprovided			NULL,

choice-extension		ProtocolIE-SingleContainer { {ThresholdIndexInformation-ExtIEs} }

}

ThresholdIndexInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ThresholdIndex ::= INTEGER (0..maxnoofThresholdMBS-1)

MulticastDU2CURRCInfo		::= SEQUENCE {

mBS-Multicast-DU2CU-Cell-List		MBS-Multicast-DU2CU-Cell-List			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastDU2CURRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastDU2CURRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-DU2CU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Multicast-DU2CU-Cell-Item

MBS-Multicast-DU2CU-Cell-Item ::= SEQUENCE {

nRCGI									NRCGI,

mbsMulticastConfigurationResponseInfo	MBSMulticastConfigurationResponseInfo		OPTIONAL,

mbsMulticastConfigurationNotification	MBSMulticastConfigurationNotification								OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-DU2CU-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-DU2CU-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationResponse ::= SEQUENCE {

mbsMulticastConfigurationResponseInfo			MBSMulticastConfigurationResponseInfo		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBSMulticastConfigurationResponse-ExtIEs} }	OPTIONAL,

...

}

MBSMulticastConfigurationResponse-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationResponseInfo ::= CHOICE {

mbsMulticastConfiguration-available			MBSMulticastConfiguration-available,

mbsMulticastConfiguration-notavailable		MBSMulticastConfiguration-notavailable,

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastConfigurationResponseInfo-ExtIEs} }

}

MBSMulticastConfigurationResponseInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MBSMulticastConfiguration-available ::= SEQUENCE {

mBSMulticastConfiguration				OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBSMulticastConfiguration-available-ExtIEs} } OPTIONAL,

...

}

MBSMulticastConfiguration-available-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfiguration-notavailable ::= SEQUENCE {

mBSMulticastConfiguration-notavailable			ENUMERATED {not-available, ...},

iE-Extensions				ProtocolExtensionContainer { { MBSMulticastConfiguration-notavailable-ExtIEs} } OPTIONAL,

...

}

MBSMulticastConfiguration-notavailable-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationNotification ::= SEQUENCE {

mbsMulticastConfigurationNotificationInfo			MBSMulticastConfigurationNotificationInfo		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBSMulticastConfigurationNotification-ExtIEs} }	OPTIONAL,

...

}

MBSMulticastConfigurationNotification-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastConfigurationNotificationInfo ::= CHOICE {

mbsMulticastConfigurationChanged			OCTET STRING,

mbsMulticastConfigurationRemoved			NULL,

choice-extension		ProtocolIE-SingleContainer { {MBSMulticastConfigurationNotificationInfo-ExtIEs} }

}

MBSMulticastConfigurationNotificationInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastF1UContext-ToBeSetup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-DU      UPTransportLayerInformation,

mbsProgressInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-ToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-Setup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-CU      UPTransportLayerInformation,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-Setup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-FailedToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSPTPRetransmissionTunnelRequired ::= ENUMERATED {true,	...}

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBSServiceArea-ExtIEs} }

}

MBSServiceArea-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NRCGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF MBS-ServiceAreaTAIList-Item

MBS-ServiceAreaTAIList-Item ::= SEQUENCE {

plmn-ID							PLMN-Identity,

fiveGS-TAC						FiveGS-TAC,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaTAIList-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaTAIList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID					MBS-Area-Session-ID,

mBS-ServiceAreaInformation 			MBS-ServiceAreaInformation,

iE-Extensions						ProtocolExtensionContainer { { MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MC-PagingCell-Item ::= SEQUENCE {

nRCGI			NRCGI,

iE-Extensions	ProtocolExtensionContainer { { MC-PagingCell-ItemExtIEs } }	OPTIONAL

}

MC-PagingCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MIB-message ::= OCTET STRING

MeasConfig ::= OCTET STRING

MeasGapConfig ::= OCTET STRING

MeasGapSharingConfig ::= OCTET STRING

PosMeasurementAmount ::= ENUMERATED {ma0, ma1, ma2, ma4, ma8, ma16, ma32, ma64}

MeasurementBeamInfoRequest ::= ENUMERATED {true, ...}

MeasurementBeamInfo	 ::= SEQUENCE {

pRS-Resource-ID				PRS-Resource-ID		OPTIONAL,

pRS-Resource-Set-ID			PRS-Resource-Set-ID	OPTIONAL,

sSB-Index					SSB-Index			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MeasurementBeamInfo-ExtIEs} } OPTIONAL

}

MeasurementBeamInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementTimingConfiguration ::= OCTET STRING

MessageIdentifier ::= BIT STRING (SIZE (16))

MeasurementTimeOccasion ::= ENUMERATED {o1, o4, ...}

MeasurementCharacteristicsRequestIndicator ::= BIT STRING (SIZE (16))

MRB-ProgressInformation ::= CHOICE {

pdcp-SN12			INTEGER (0..4095),

pdcp-SN18			INTEGER (0..262143),

choice-extension		ProtocolIE-SingleContainer { { MRB-ProgressInformation-ExtIEs} }

}

MRB-ProgressInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastF1UContextReferenceF1 ::= OCTET STRING (SIZE(4))

MulticastF1UContextReferenceCU ::= OCTET STRING (SIZE(4))

MultipleULAoA ::= SEQUENCE {

multipleULAoA					MultipleULAoA-List,

iE-Extensions					ProtocolExtensionContainer { { MultipleULAoA-ExtIEs} } OPTIONAL,

...

}

MultipleULAoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MultipleULAoA-List ::= SEQUENCE (SIZE(1.. maxnoofULAoAs)) OF MultipleULAoA-Item

MultipleULAoA-Item ::= CHOICE {

uL-AoA		UL-AoA,

ul-ZoA		ZoAInformation,

choice-extension ProtocolIE-SingleContainer { { MultipleULAoA-Item-ExtIEs } }

}

MultipleULAoA-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MDTPollutedMeasurementIndicator	::= ENUMERATED {iDC,no-IDC, ...}

MRB-ID ::= INTEGER (1..512, ...)

MulticastMBSSessionList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MulticastMBSSessionList-Item

MulticastMBSSessionList-Item ::= SEQUENCE {

mbsSessionId				MBS-Session-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMBSSessionList-Item-ExtIEs } } OPTIONAL,

...

}

MulticastMBSSessionList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeModified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedtoBeModified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedtoBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeSetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-Modified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-Modified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-Modified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-Setup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-Setup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-SetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-SetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-SetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeModified-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters		OPTIONAL,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List	OPTIONAL,

mBS-DL-PDCP-SN-Length			PDCPSNLength 					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeModified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-ToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeReleased-Item	::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

MulticastMRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeSetup-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

mBS-DL-PDCP-SN-Length			PDCPSNLength,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeSetup-Item-ExtIEs} },

...

}

MulticastMRBs-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeSetupMod-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

mBS-DL-PDCP-SN-Length			PDCPSNLength,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeSetupMod-Item-ExtIEs} },

...

}

MulticastMRBs-ToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL

}

MultiplexingInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MusimCapabilityRestrictionIndication ::= ENUMERATED {true, ...}

MusimCandidateBandList ::= OCTET STRING

M2Configuration ::= ENUMERATED {true, ...}

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		M5-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M5ReportAmount	CRITICALITY ignore	EXTENSION M5ReportAmount PRESENCE optional },

...

}

M5period ::= ENUMERATED { ms1024, ms2048, ms5120, ms10240, min1, ... }

M5ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M5-Links-to-log	::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		M6-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M6ReportAmount	CRITICALITY ignore	EXTENSION M6ReportAmount PRESENCE optional },

...

}

M6report-Interval ::= ENUMERATED { ms120, ms240, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, ..., ms480}

M6ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M6-Links-to-log	::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		M7-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M7ReportAmount	CRITICALITY ignore	EXTENSION M7ReportAmount PRESENCE optional},

...

}

M7period	::= INTEGER(1..60, ...)

M7ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M7-Links-to-log	::= ENUMERATED {downlink, ...}

MDT-Activation ::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

...

}

MDTConfiguration ::= SEQUENCE {

mdt-Activation				MDT-Activation,

measurementsToActivate		MeasurementsToActivate,

m2Configuration				M2Configuration		OPTIONAL,

--  C-ifM2: This IE shall be present if the Measurements to Activate IE has the second bit set to "1".

m5Configuration				M5Configuration		OPTIONAL,

--  C-ifM5: This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1".

m6Configuration				M6Configuration		OPTIONAL,

--  C-ifM6: This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1".

m7Configuration				M7Configuration		OPTIONAL,

--  C-ifM7: This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1".

iE-Extensions				ProtocolExtensionContainer { { MDTConfiguration-ExtIEs} } OPTIONAL,

...

}

MDTConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTPLMNModificationList ::= SEQUENCE (SIZE(0..maxnoofMDTPLMNs)) OF PLMN-Identity

MeasuredFrequencyHops ::= ENUMERATED {singleHop, multiHop, ...}

MeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOA-Measurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension	ProtocolIE-SingleContainer { { MeasuredResultsValue-ExtIEs } }

}

MeasuredResultsValue-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ZoAInformation	CRITICALITY reject TYPE ZoAInformation	PRESENCE mandatory}|

{ ID id-MultipleULAoA	CRITICALITY reject TYPE MultipleULAoA	PRESENCE mandatory}|

{ ID id-UL-SRS-RSRPP	CRITICALITY reject TYPE UL-SRS-RSRPP	PRESENCE mandatory}|

{ ID id-UL-RSCP			CRITICALITY reject TYPE UL-RSCP			PRESENCE mandatory},

...

}

MeasurementsToActivate ::= BIT STRING (SIZE (8))

Mobile-TRP-LocationInformation ::= SEQUENCE {

location-Information			OCTET STRING,

velocity-Information			OCTET STRING	OPTIONAL,

location-time-stamp				TimeStamp		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Mobile-TRP-LocationInformation-ExtIEs} } OPTIONAL,

...

}

Mobile-TRP-LocationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Mobile-IAB-MT-UE-ID ::= OCTET STRING

MUSIM-GapConfig ::= OCTET STRING

MobileIAB-Barred	::=	ENUMERATED {barred, not-barred, ...}

#### -- N

NRA2XServicesAuthorized ::= SEQUENCE {

aerialUE			AerialUE														OPTIONAL,

controllerUE		ControllerUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NRA2XServicesAuthorized-ExtIEs} }	OPTIONAL

}

NRA2XServicesAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AerialUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

ControllerUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

N3CIndirectPathAddition::= SEQUENCE {

targetRelayUEID			GNB-DU-UE-F1AP-ID,

iE-Extensions			ProtocolExtensionContainer { { N3CIndirectPathAddition-ExtIEs } }		OPTIONAL,

...

}

N3CIndirectPathAddition-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NA-Resource-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofHSNASlots)) OF NA-Resource-Configuration-Item

NA-Resource-Configuration-Item ::= SEQUENCE {

nADownlink					NADownlink 	    OPTIONAL,

nAUplink					NAUplink 	    OPTIONAL,

nAFlexible					NAFlexible 	    OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { NA-Resource-Configuration-Item-ExtIEs} } OPTIONAL

}

NA-Resource-Configuration-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NADownlink ::= ENUMERATED { true, false, ...}

NAFlexible ::= ENUMERATED { true, false, ...}

NAUplink ::= ENUMERATED { true, false, ...}

Ncd-SSB-RedCapInitialBWP-SDT ::= OCTET STRING

NetworkControlledRepeaterAuthorized ::= ENUMERATED { authorized, not-authorized, ...}

NCGI-to-be-Updated-List-Item ::= SEQUENCE {

oLDNCGI		NRCGI,

nEWNCGI		NRCGI,

iE-Extensions				ProtocolExtensionContainer { { NCGI-to-be-Updated-List-ItemExtIEs} }	OPTIONAL,

...

}

NCGI-to-be-Updated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Neighbour-Node-Cells-List ::= SEQUENCE (SIZE(1..maxnoofNeighbourNodeCellsIAB)) OF Neighbour-Node-Cells-List-Item

Neighbour-Node-Cells-List-Item ::= SEQUENCE{

nRCGI			 								NRCGI,

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID 			OPTIONAL,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID 			OPTIONAL,

peer-Parent-Node-Indicator						ENUMERATED {true, ...} 	OPTIONAL,

iAB-DU-Cell-Resource-Configuration-Mode-Info	IAB-DU-Cell-Resource-Configuration-Mode-Info 	OPTIONAL,

iAB-STC-Info									IAB-STC-Info	OPTIONAL,

rACH-Config-Common								RACH-Config-Common	OPTIONAL,

rACH-Config-Common-IAB							RACH-Config-Common-IAB	OPTIONAL,

cSI-RS-Configuration							OCTET STRING	OPTIONAL,

sR-Configuration								OCTET STRING	OPTIONAL,

pDCCH-ConfigSIB1								OCTET STRING	OPTIONAL,

sCS-Common										OCTET STRING	OPTIONAL,

iE-Extensions									ProtocolExtensionContainer {{Neighbour-Node-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Neighbour-Node-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeedforGap::= ENUMERATED {true, ...}

NeedForGapsInfoNR ::= OCTET STRING

NeedForGapNCSGInfoNR ::= OCTET STRING

NeedForGapNCSGInfoEUTRA ::= OCTET STRING

NeedForInterruptionInfoNR ::= OCTET STRING

Neighbour-Cell-Information-Item ::= SEQUENCE {

nRCGI				NRCGI,

intendedTDD-DL-ULConfig		IntendedTDD-DL-ULConfig OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Neighbour-Cell-Information-ItemExtIEs } }	OPTIONAL

}

Neighbour-Cell-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourNR-CellsForSON-List ::= SEQUENCE (SIZE(1.. maxNeighbourCellforSON)) OF NeighbourNR-CellsForSON-Item

NeighbourNR-CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

nR-ModeInfoRel16								NR-ModeInfoRel16												OPTIONAL,

sSB-PositionsInBurst							SSB-PositionsInBurst											OPTIONAL,

nRPRACHConfig									NRPRACHConfig													OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { NeighbourNR-CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourNR-CellsForSON-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NGRANHighAccuracyAccessPointPosition ::= SEQUENCE {

latitude					INTEGER (-2147483648.. 2147483647),

longitude					INTEGER (-2147483648.. 2147483647),

altitude					INTEGER (-64000..1280000),

uncertaintySemi-major		INTEGER (0..255),

uncertaintySemi-minor		INTEGER (0..255),

orientationOfMajorAxis		INTEGER (0..179),

horizontalConfidence		INTEGER (0..100),

uncertaintyAltitude			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NGRANHighAccuracyAccessPointPosition-ExtIEs} } OPTIONAL

}

NGRANHighAccuracyAccessPointPosition-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NID ::= BIT STRING (SIZE(44))

NonF1terminatingTopologyIndicator ::= ENUMERATED {

true,

...

}

NR-CGI-List-For-Restart-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { NR-CGI-List-For-Restart-ItemExtIEs } }	OPTIONAL,

...

}

NR-CGI-List-For-Restart-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NrofSymbolsExtended ::=  ENUMERATED {n8, n10, n12, n14, ...}

NR-PRSBeamInformation ::= SEQUENCE {

nR-PRSBeamInformationList		NR-PRSBeamInformationList,

lCStoGCSTranslationList 		LCStoGCSTranslationList		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NR-PRSBeamInformation-ExtIEs } } OPTIONAL

}

NR-PRSBeamInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-PRSBeamInformationList ::= SEQUENCE (SIZE(1.. maxnoofPRS-ResourceSets)) OF NR-PRSBeamInformationItem

NR-PRSBeamInformationItem ::= SEQUENCE {

pRSResourceSetID	PRS-Resource-Set-ID,

pRSAngleList		PRSAngleList,

iE-Extensions		ProtocolExtensionContainer { { NR-PRSBeamInformationItem-ExtIEs } } OPTIONAL

}

NR-PRSBeamInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-TADV ::= INTEGER (0.. 7690)

NReRedCapUEIndication ::= ENUMERATED {true, ...}

ERedcap-Bcast-Information ::= BIT STRING(SIZE(8))

NRRedCapUEIndication ::= ENUMERATED {true, ...}

NRPagingeDRXInformation ::= SEQUENCE {

nrpaging-eDRX-Cycle-Idle		NRPaging-eDRX-Cycle-Idle,

nrpaging-Time-Window			NRPaging-Time-Window												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {NRPagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Idle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4,

hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NRPaging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...,

s17, s18, s19, s20, s21,

s22, s23, s24, s25, s26,

s27, s28, s29, s30, s31, s32

}

NRPagingeDRXInformationforRRCINACTIVE ::= SEQUENCE {

nrpaging-eDRX-Cycle-Inactive		NRPaging-eDRX-Cycle-Inactive,

iE-Extensions			ProtocolExtensionContainer { { NRPagingeDRXInformationforRRCINACTIVE-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformationforRRCINACTIVE-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Inactive ::= ENUMERATED {

hfquarter, hfhalf, hf1,

...

}

NRPaginglongeDRXInformationforRRCINACTIVE ::= SEQUENCE {

nRPaging-long-eDRX-Cycle-Inactive		NRPaging-long-eDRX-Cycle-Inactive,

nRPaging-Time-Window-Inactive			NRPaging-Time-Window-Inactive,

iE-Extensions							ProtocolExtensionContainer { { NRPaginglongeDRXInformationforRRCINACTIVE-ExtIEs} }	OPTIONAL,

...

}

NRPaginglongeDRXInformationforRRCINACTIVE-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-long-eDRX-Cycle-Inactive ::= ENUMERATED {

hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, ...

}

NRPaging-Time-Window-Inactive ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

s17, s18, s19, s20, s21, s22,

s23, s24, s25, s26, s27, s28, s29,

s30, s31, s32, ...

}

NonDynamic5QIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			INTEGER (1..127)				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NonDynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

NonDynamic5QIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDownlink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional	}|

{ ID id-CNPacketDelayBudgetUplink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional	},

...

}

NonDynamicPQIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			INTEGER (1..8, ...)				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NonDynamicPQIDescriptor-ExtIEs } } OPTIONAL

}

NonDynamicPQIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NonUPTrafficType ::=	ENUMERATED {ue-associated, non-ue-associated, non-f1, bap-control-pdu,...}

NoofDownlinkSymbols	::= INTEGER (0..14)

NoofUplinkSymbols	::= INTEGER (0..14)

Notification-Cause ::= ENUMERATED {fulfilled, not-fulfilled, ...}

NotificationControl ::= ENUMERATED {active, not-active, ...}

NotificationInformation ::= SEQUENCE {

message-Identifier	MessageIdentifier,

serialNumber		SerialNumber,

iE-Extensions	ProtocolExtensionContainer { { NotificationInformationExtIEs} } OPTIONAL,

...

}

NotificationInformationExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

NPNBroadcastInformation ::= CHOICE {

sNPN-Broadcast-Information					NPN-Broadcast-Information-SNPN,

pNI-NPN-Broadcast-Information				NPN-Broadcast-Information-PNI-NPN,

choice-extension					ProtocolIE-SingleContainer { {NPNBroadcastInformation-ExtIEs} }

}

NPNBroadcastInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

NPN-Broadcast-Information-SNPN ::= SEQUENCE {

broadcastSNPNID-List		BroadcastSNPN-ID-List,

iE-Extension				ProtocolExtensionContainer { {NPN-Broadcast-Information-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-SNPN-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NPN-Broadcast-Information-PNI-NPN ::= SEQUENCE {

broadcastPNI-NPN-ID-Information		BroadcastPNI-NPN-ID-List,

iE-Extension							ProtocolExtensionContainer { {NPN-Broadcast-Information-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-PNI-NPN-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NPNSupportInfo ::= CHOICE {

sNPN-Information		NID,

choice-extension		ProtocolIE-SingleContainer { { NPNSupportInfo-ExtIEs } }

}

NPNSupportInfo-ExtIEs		F1AP-PROTOCOL-IES ::= {

...

}

NRCarrierList ::= SEQUENCE (SIZE(1..maxnoofNRSCSs)) OF NRCarrierItem

NRCarrierItem ::= SEQUENCE {

carrierSCS						NRSCS,

offsetToCarrier					INTEGER (0..2199, ...),

carrierBandwidth				INTEGER (0..maxnoofPhysicalResourceBlocks, ...),

iE-Extension					ProtocolExtensionContainer { {NRCarrierItem-ExtIEs} } 				OPTIONAL,

...

}

NRCarrierItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRFreqInfo ::=  SEQUENCE {

nRARFCN			INTEGER (0..maxNRARFCN),

sul-Information	SUL-Information		OPTIONAL,

freqBandListNr	SEQUENCE (SIZE(1..maxnoofNrCellBands)) OF FreqBandNrItem,

iE-Extensions	ProtocolExtensionContainer { { NRFreqInfoExtIEs} } OPTIONAL,

...

}

NRFreqInfoExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },

...

}

NRCGI ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

nRCellIdentity			NRCellIdentity,

iE-Extensions			ProtocolExtensionContainer { {NRCGI-ExtIEs} } OPTIONAL,

...

}

NRCGI-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-Mode-Info ::= CHOICE {

fDD		FDD-Info,

tDD		TDD-Info,

choice-extension			ProtocolIE-SingleContainer { { NR-Mode-Info-ExtIEs} }

}

NR-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-NR-U		CRITICALITY ignore	TYPE NR-U-Channel-Info-List PRESENCE mandatory},

...

}

NR-ModeInfoRel16 ::= CHOICE {

fDD												FDD-InfoRel16,

tDD												TDD-InfoRel16,

choice-extension								ProtocolIE-SingleContainer { { NR-ModeInfoRel16-ExtIEs} }

}

NR-ModeInfoRel16-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

NRPRACHConfig ::= SEQUENCE {

ulPRACHConfigList			NRPRACHConfigList									OPTIONAL,

sulPRACHConfigList			NRPRACHConfigList									OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NRPRACHConfig-ExtIEs} } 	OPTIONAL,

...

}

NRPRACHConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRCellIdentity ::= BIT STRING (SIZE(36))

NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242, nrb15}

NRPCI ::= INTEGER(0..1007)

NRPRACHConfigList ::= SEQUENCE (SIZE(0..maxnoofPRACHconfigs)) OF NRPRACHConfigItem

NRPRACHConfigItem ::= SEQUENCE {

nRSCS					NRSCS,

prachFreqStartfromCarrier	INTEGER (0..maxnoofPhysicalResourceBlocks-1, ...),

prachFDM					ENUMERATED {one, two, four, eight, ...},

prachConfigIndex			INTEGER (0..255, ..., 256..262),

ssb-perRACH-Occasion		ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen, ...},

freqDomainLength			FreqDomainLength,

zeroCorrelZoneConfig		INTEGER (0..15),

iE-Extension		ProtocolExtensionContainer { { NRPRACHConfigItem-ExtIEs} } 		OPTIONAL,

...

}

NRPRACHConfigItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ..., scs480, scs960}

NRUERLFReportContainer ::= OCTET STRING

NR-U-Channel-Info-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Info-Item

NR-U-Channel-Info-Item ::= SEQUENCE {

nr-U-channel-ID			INTEGER(1.. maxnoofNR-UChannelIDs,...),

nR-ARFCN				INTEGER (0..maxNRARFCN),

bandwidth				ENUMERATED{mHz-10,mHz-20,mHz-40, mHz-60, mHz-80,..., mHz-100},

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Info-List-ExtIEs } }		OPTIONAL,

...

}

NR-U-Channel-Info-List-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						INTEGER(1..maxnoofNR-UChannelIDs),

channelOccupancyTimePercentageDL		ChannelOccupancyTimePercentage,

energyDetectionThreshold				EnergyDetectionThreshold,

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Item-ExtIEs} } OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore EXTENSION ChannelOccupancyTimePercentage PRESENCE optional}|

{ ID id-RadioResourceStatusNR-U					CRITICALITY ignore EXTENSION RadioResourceStatusNR-U PRESENCE optional},

...

}

NumberofActiveUEs ::= INTEGER(0..16777215, ...)

NumberOfBroadcasts ::= INTEGER (0..65535)

NumberofBroadcastRequest ::= INTEGER (0..65535)

NumberOfTRPRxTEG ::= ENUMERATED {two, three, four, six, eight, ...}

NumberOfTRPRxTxTEG ::= ENUMERATED {wo, three, four, six, eight, ...}

NumDLULSymbols ::= SEQUENCE {

numDLSymbols	INTEGER (0..13, ...),

numULSymbols	INTEGER (0..13, ...),

iE-Extensions			ProtocolExtensionContainer { { NumDLULSymbols-ExtIEs} } OPTIONAL

}

NumDLULSymbols-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-permutation		CRITICALITY ignore	EXTENSION Permutation	    PRESENCE optional },

...

}

NRV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE														OPTIONAL,

pedestrianUE 		PedestrianUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NRV2XServicesAuthorized-ExtIEs} }	OPTIONAL

}

NRV2XServicesAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRUESidelinkAggregateMaximumBitrate ::= SEQUENCE {

uENRSidelinkAggregateMaximumBitrate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {NRUESidelinkAggregateMaximumBitrate-ExtIEs} } OPTIONAL

}

NRUESidelinkAggregateMaximumBitrate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NZP-CSI-RS-ResourceID::= INTEGER  (0..191)

N6JitterInformation ::= SEQUENCE {

n6JitterLowerBound		INTEGER (-127..127),

n6JitterUpperBound		INTEGER (-127..127),

iE-Extensions			ProtocolExtensionContainer { { N6JitterInformationExtIEs } }	OPTIONAL,

...}

N6JitterInformationExtIEs   F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- O

OffsetToPointA	::= INTEGER (0..2199,...)

OnDemandPRS-Info ::= SEQUENCE {

onDemandPRSRequestAllowed				BIT STRING (SIZE (16)),

allowedResourceSetPeriodicityValues		BIT STRING (SIZE (24))	OPTIONAL,

allowedPRSBandwidthValues				BIT STRING (SIZE (64))	OPTIONAL,

allowedResourceRepetitionFactorValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedResourceNumberOfSymbolsValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedCombSizeValues					BIT STRING (SIZE (8))	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { OnDemandPRS-Info-ExtIEs} } OPTIONAL,

...

}

OnDemandPRS-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PathAdditionInformation ::= CHOICE {

indirectPathAddition			IndirectPathAddition,

directPathAddition			NULL,

n3C-indirectPathAddition		N3CIndirectPathAddition,

choice-extension				ProtocolIE-SingleContainer { { PathAdditionInformation-ExtIEs} }

}

PathAdditionInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PagingCell-Item ::= SEQUENCE {

nRCGI		NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { PagingCell-ItemExtIEs } }	OPTIONAL

}

PagingCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-LastUsedCellIndication					CRITICALITY ignore 	EXTENSION LastUsedCellIndication						PRESENCE optional }|

{	ID id-PEISubgroupingSupportIndication			CRITICALITY ignore	EXTENSION PEISubgroupingSupportIndication			PRESENCE optional }|

{	ID id-Recommended-SSBs-List						CRITICALITY ignore	EXTENSION Recommended-SSBs-List						PRESENCE optional },

...

}

Recommended-SSBs-List ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF RecommendedSSBItem-List-Item

RecommendedSSBItem-List-Item::= SEQUENCE {

sSB-Index			SSB-Index,

iE-Extensions		ProtocolExtensionContainer { { RecommendedSSBItem-List-Item-ExtIEs} } OPTIONAL

}

RecommendedSSBItem-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingIdentity ::=	CHOICE {

rANUEPagingIdentity	RANUEPagingIdentity,

cNUEPagingIdentity	CNUEPagingIdentity,

choice-extension			ProtocolIE-SingleContainer { { PagingIdentity-ExtIEs } }

}

PagingCause ::= ENUMERATED { voice,	...}

PagingIdentity-ExtIEs F1AP-PROTOCOL-IES::= {

...

}

PagingOrigin ::= ENUMERATED { non-3gpp,	...}

PagingPriority ::= ENUMERATED { priolevel1, priolevel2, priolevel3, priolevel4, priolevel5, priolevel6, priolevel7, priolevel8,...}

ParentTImeSource ::= ENUMERATED {synce, ptp, gnss, atomicclock, terrestrialradio, serialtimecode, ntp, handset, other, ...}

PEIPSAssistanceInfo ::= SEQUENCE {

cNSubgroupID		CNSubgroupID,

iE-Extensions	ProtocolExtensionContainer { { PEIPSAssistanceInfo-ExtIEs } }	OPTIONAL

}

PEIPSAssistanceInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativePathDelay ::= CHOICE {

k0					INTEGER (0..16351),

k1					INTEGER (0..8176),

k2					INTEGER (0..4088),

k3					INTEGER (0..2044),

k4					INTEGER (0..1022),

k5					INTEGER (0..511),

choice-extension			ProtocolIE-SingleContainer { { RelativePathDelay-ExtIEs } }

}

RelativePathDelay-ExtIEs F1AP-PROTOCOL-IES ::= {

{ID id-ReportingGranularitykminus1additionalpath 	CRITICALITY ignore TYPE ReportingGranularitykminus1AdditionalPath PRESENCE mandatory}|

{ID id-ReportingGranularitykminus2additionalpath 	CRITICALITY ignore TYPE ReportingGranularitykminus2AdditionalPath PRESENCE mandatory }|

{ID id-ReportingGranularitykminus3additionalpath 	CRITICALITY ignore TYPE ReportingGranularitykminus3AdditionalPath PRESENCE mandatory}|

{ID id-ReportingGranularitykminus4additionalpath 	CRITICALITY ignore TYPE ReportingGranularitykminus4AdditionalPath PRESENCE mandatory }|

{ID id-ReportingGranularitykminus5additionalpath 	CRITICALITY ignore TYPE ReportingGranularitykminus5AdditionalPath PRESENCE mandatory}|

{ID id-ReportingGranularitykminus6additionalpath 	CRITICALITY ignore TYPE ReportingGranularitykminus6AdditionalPath PRESENCE mandatory },

...

}

Parent-IAB-Nodes-NA-Resource-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF Parent-IAB-Nodes-NA-Resource-Configuration-Item

Parent-IAB-Nodes-NA-Resource-Configuration-Item::= SEQUENCE {

nADownlink					NADownlink 	    OPTIONAL,

nAUplink					NAUplink 	    OPTIONAL,

nAFlexible					NAFlexible 	    OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Parent-IAB-Nodes-NA-Resource-Configuration-Item-ExtIEs} } OPTIONAL

}

Parent-IAB-Nodes-NA-Resource-Configuration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PartialSuccessCell ::= SEQUENCE {

broadcastCellList		BroadcastCellList,

iE-Extensions				ProtocolExtensionContainer { { PartialSuccessCell-ExtIEs} } OPTIONAL,

...

}

PartialSuccessCell-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PathlossReferenceInfo ::= SEQUENCE {

pathlossReferenceSignal			PathlossReferenceSignal,

iE-Extensions					ProtocolExtensionContainer { {PathlossReferenceInfo-ExtIEs} }	OPTIONAL

}

PathlossReferenceInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PathlossReferenceSignal ::= CHOICE {

sSB										SSB,

dL-PRS									DL-PRS,

choice-extension						ProtocolIE-SingleContainer {{PathlossReferenceSignal-ExtIEs }}

}

PathlossReferenceSignal-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PathSwitchConfiguration ::= SEQUENCE {

targetRelayUEID			BIT STRING(SIZE(24)),

remoteUELocalID			RemoteUELocalID,

t420					ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

iE-Extensions			ProtocolExtensionContainer { { PathSwitchConfiguration-ExtIEs } }		OPTIONAL,

...

}

PathSwitchConfiguration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5QoSFlowIdentifier ::= INTEGER (1..2048)

PC5-QoS-Characteristics ::= CHOICE {

non-Dynamic-PQI				NonDynamicPQIDescriptor,

dynamic-PQI					DynamicPQIDescriptor,

choice-extension			ProtocolIE-SingleContainer { { PC5-QoS-Characteristics-ExtIEs } }

}

PC5-QoS-Characteristics-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PC5QoSParameters	::= SEQUENCE {

pC5-QoS-Characteristics				PC5-QoS-Characteristics,

pC5-QoS-Flow-Bit-Rates				PC5FlowBitRates				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PC5QoSParameters-ExtIEs } }	OPTIONAL,

...

}

PC5QoSParameters-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5FlowBitRates ::= SEQUENCE {

guaranteedFlowBitRate		BitRate,

maximumFlowBitRate			BitRate,

iE-Extensions				ProtocolExtensionContainer { { PC5FlowBitRates-ExtIEs } }	OPTIONAL,

...

}

PC5FlowBitRates-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelID ::= INTEGER (1..512, ...)

PC5RLCChannelQoSInformation ::= CHOICE {

pC5RLCChannelQoS				QoSFlowLevelQoSParameters,

pC5ControlPlaneTrafficType		ENUMERATED {srb1,srb2,...},

choice-extension				ProtocolIE-SingleContainer { { PC5RLCChannelQoSInformation-ExtIEs} }

}

PC5RLCChannelQoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

{	ID id-U2URLCChannelQoS		CRITICALITY reject TYPE PC5QoSParameters		PRESENCE mandatory},

...

}

PC5RLCChannelToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelToBeSetupItem

PC5RLCChannelToBeSetupItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

pC5RLCChannelQoSInformation		PC5RLCChannelQoSInformation,

rLCMode							RLCMode,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelToBeModifiedItem

PC5RLCChannelToBeModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

pC5RLCChannelQoSInformation		PC5RLCChannelQoSInformation			OPTIONAL,

rLCMode							RLCMode			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelToBeReleasedItem

PC5RLCChannelToBeReleasedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelSetupList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelSetupItem

PC5RLCChannelSetupItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelSetupItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelFailedToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelFailedToBeSetupItem

PC5RLCChannelFailedToBeSetupItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

cause							Cause					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelFailedToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelFailedToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelModifiedItem

PC5RLCChannelModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelFailedToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelFailedToBeModifiedItem

PC5RLCChannelFailedToBeModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

cause							Cause	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelFailedToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelFailedToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelRequiredToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelRequiredToBeModifiedItem

PC5RLCChannelRequiredToBeModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelRequiredToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelRequiredToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelRequiredToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelRequiredToBeReleasedItem

PC5RLCChannelRequiredToBeReleasedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelRequiredToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelRequiredToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCCH-BlindDetectionSCG ::= OCTET STRING

PDCMeasurementPeriodicity ::= ENUMERATED

{ms80, ms120, ms160, ms240, ms320, ms480, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ...}

PDCMeasurementQuantities ::= SEQUENCE (SIZE (1.. maxnoofMeasPDC)) OF ProtocolIE-SingleContainer { {PDCMeasurementQuantities-ItemIEs} }

PDCMeasurementQuantities-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-PDCMeasurementQuantities-Item	CRITICALITY reject	TYPE PDCMeasurementQuantities-Item		PRESENCE mandatory}

}

PDCMeasurementQuantities-Item ::= SEQUENCE {

pDCmeasurementQuantitiesValue				PDCMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { PDCMeasurementQuantitiesValue-ExtIEs} } OPTIONAL

}

PDCMeasurementQuantitiesValue-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasurementQuantitiesValue ::= ENUMERATED {

nr-pdc-tadv,

gNB-rx-tx,

...

}

PDCMeasurementResult ::= SEQUENCE {

pDCMeasuredResultsList			PDCMeasuredResultsList,

iE-Extensions					ProtocolExtensionContainer { { PDCMeasurementResult-ExtIEs} } OPTIONAL

}

PDCMeasurementResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasuredResultsList ::= SEQUENCE (SIZE(1..maxnoofMeasPDC)) OF PDCMeasuredResults-Item

PDCMeasuredResults-Item ::= SEQUENCE {

pDCMeasuredResults-Value	PDCMeasuredResults-Value,

iE-Extensions				ProtocolExtensionContainer {{ PDCMeasuredResults-Item-ExtIEs }}	 OPTIONAL

}

PDCMeasuredResults-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasuredResults-Value ::= CHOICE {

pDC-TADV-NR				PDC-TADV-NR,

pDC-RxTxTimeDiff		PDC-RxTxTimeDiff,

choice-extension		ProtocolIE-SingleContainer { { PDCMeasuredResults-Value-ExtIEs} }

}

PDCMeasuredResults-Value-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PDCReportType ::= ENUMERATED {

onDemand,

periodic,

...

}

PDC-RxTxTimeDiff ::= INTEGER (0..61565, ...)

PDC-TADV-NR ::= INTEGER (0..62500, ...)

PDCP-SN ::= INTEGER (0..4095)

PDCPSNLength	::= ENUMERATED { twelve-bits,eighteen-bits,...}

PDUSessionID ::= INTEGER (0..255)

PEISubgroupingSupportIndication ::= ENUMERATED {true, ...}

ReportingPeriodicityValue ::= INTEGER (0..512, ...)

Periodicity ::= INTEGER (0..640000, ...)

PeriodicitySRS ::= ENUMERATED { ms0p125, ms0p25, ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms4, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ...}

PeriodicityList ::= SEQUENCE (SIZE(1.. maxnoSRS-ResourcePerSet)) OF PeriodicityList-Item

PeriodicityList-Item ::= SEQUENCE {

periodicitySRS				PeriodicitySRS,

iE-Extensions				ProtocolExtensionContainer { { PeriodicityList-ItemExtIEs} } OPTIONAL

}

PeriodicityList-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PeriodicityBound ::= SEQUENCE {

periodicityLowerBound					Periodicity,

periodicityUpperBound					Periodicity,

iE-Extensions				ProtocolExtensionContainer { {PeriodicityBound-ExtIEs} } OPTIONAL,

...

}

PeriodicityBound-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AllowedPeriodicityList ::= SEQUENCE (SIZE(1..maxnoofPeriodicities)) OF Periodicity

PeriodicityRange ::= CHOICE {

periodicityBound				PeriodicityBound,

periodicityList					AllowedPeriodicityList,

choice-extensions				ProtocolIE-SingleContainer { {PeriodicityRange-ExtIEs} }

}

PeriodicityRange-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

Permutation ::= ENUMERATED {dfu, ufd, ...}

Ph-InfoMCG  ::= OCTET STRING

Ph-InfoSCG  ::= OCTET STRING

PLMN-Identity ::= OCTET STRING (SIZE(3))

PlayoutDelayForMediaStartup ::= OCTET STRING

PortNumber ::= BIT STRING (SIZE (16))

PosAssistance-Information ::= OCTET STRING

PosAssistanceInformationFailureList ::= OCTET STRING

PosBroadcast ::= ENUMERATED {

start,

stop,

...

}

PosConextRevIndication ::= ENUMERATED {true, ...}

PositioningBroadcastCells ::= SEQUENCE (SIZE (1..maxnoBcastCell)) OF NRCGI

PosMeasGapPreConfigList ::= SEQUENCE {

posMeasGapPreConfigToAddModList				OCTET STRING					OPTIONAL,

posMeasGapPreConfigToReleaseList			OCTET STRING					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { PosMeasGapPreConfigList-ExtIEs} } OPTIONAL

}

PosMeasGapPreConfigList-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementPeriodicity ::= ENUMERATED

{ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, ..., ms20480, ms40960, extended }

MeasurementPeriodicityExtended ::= ENUMERATED {ms160, ms320, ms1280, ms2560, ms61440, ms81920,	ms368640, ms737280, ms1843200,	...}

PosMeasurementPeriodicityNR-AoA ::= ENUMERATED {

ms160,

ms320,

ms640,

ms1280,

ms2560,

ms5120,

ms10240,

ms20480,

ms40960,

ms61440,

ms81920,

ms368640,

ms737280,

ms1843200,

...

}

PosMeasurementQuantities ::= SEQUENCE (SIZE(1.. maxnoofPosMeas)) OF PosMeasurementQuantities-Item

PosMeasurementQuantities-Item ::= SEQUENCE {

posMeasurementType					PosMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PosMeasurementQuantities-ItemExtIEs} } OPTIONAL

}

PosMeasurementQuantities-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional},

...

}

PosMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoofPosMeas)) OF PosMeasurementResultItem

PosMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				MeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TRPMeasurementQuality	OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { PosMeasurementResultItemExtIEs } }	OPTIONAL

}

PosMeasurementResultItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ARP-ID				CRITICALITY ignore EXTENSION ARP-ID 					PRESENCE optional}|

{ ID id-SRSResourcetype		CRITICALITY ignore EXTENSION SRSResourcetype 			PRESENCE optional}|

{ ID id-LoS-NLoSInformation	CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional }|

{ ID id-Mobile-TRP-LocationInformation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation		PRESENCE optional }|

{ ID id-AggregatedPosSRSResourceIDList CRITICALITY ignore EXTENSION AggregatedPosSRSResourceIDList PRESENCE optional }|

{ ID id-MeasuredFrequencyHops			CRITICALITY ignore EXTENSION MeasuredFrequencyHops PRESENCE optional },

...

}

PosMeasurementResultList ::= SEQUENCE (SIZE(1.. maxNoOfMeasTRPs)) OF PosMeasurementResultList-Item

PosMeasurementResultList-Item ::= SEQUENCE {

posMeasurementResult			PosMeasurementResult,

tRPID							TRPID,

iE-Extensions					ProtocolExtensionContainer { { PosMeasurementResultList-ItemExtIEs} } OPTIONAL

}

PosMeasurementResultList-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRCGI	CRITICALITY ignore EXTENSION NRCGI		PRESENCE optional },

...

}

PosMeasurementType ::= ENUMERATED {

gnb-rx-tx,

ul-srs-rsrp,

ul-aoa,

ul-rtoa,

... ,

multiple-ul-aoa,

ul-srs-rsrpp,

ul-rscp

}

PosReportCharacteristics ::= ENUMERATED {

ondemand,

periodic,

...

}

PosResourceSetType  ::= CHOICE {

periodic			PosResourceSetTypePR,

semi-persistent		PosResourceSetTypeSP,

aperiodic			PosResourceSetTypeAP,

choice-extension	ProtocolIE-SingleContainer {{ PosResourceSetType-ExtIEs }}

}

PosResourceSetType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PosResourceSetTypePR ::= SEQUENCE {

posperiodicSet		ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypePR-ExtIEs} }	OPTIONAL

}

PosResourceSetTypePR-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeSP ::= SEQUENCE {

possemi-persistentSet		ENUMERATED{true, ...},

iE-Extensions				ProtocolExtensionContainer { { PosResourceSetTypeSP-ExtIEs} }	OPTIONAL

}

PosResourceSetTypeSP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeAP ::= SEQUENCE {

sRSResourceTrigger-List		INTEGER(1..3),

iE-Extensions				ProtocolExtensionContainer { { PosResourceSetTypeAP-ExtIEs} }	OPTIONAL

}

PosResourceSetTypeAP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosSItypeList ::= SEQUENCE (SIZE(1.. maxnoofPosSITypes)) OF PosSItype-Item

PosSItype-Item ::= SEQUENCE {

posItype			PosSItype   ,

iE-Extensions	ProtocolExtensionContainer { { PosSItype-ItemExtIEs } } OPTIONAL

}

PosSItype-ItemExtIEs    F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosSItype ::= INTEGER (1..32, ...)

PosSRSResourceID-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourcePerSet)) OF SRSPosResourceID

PosSRSResource-Item ::= SEQUENCE {

srs-PosResourceId				SRSPosResourceID,

transmissionCombPos				TransmissionCombPos,

startPosition					INTEGER (0..13),

nrofSymbols						ENUMERATED {n1, n2, n4, n8, n12},

freqDomainShift					INTEGER (0..268),

c-SRS							INTEGER (0..63),

groupOrSequenceHopping			ENUMERATED { neither, groupHopping, sequenceHopping },

resourceTypePos					ResourceTypePos,

sequenceId						INTEGER (0.. 65535),

spatialRelationPos				SpatialRelationPos 	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PosSRSResource-Item-ExtIEs} }	OPTIONAL

}

PosSRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-TxHoppingConfiguration	CRITICALITY ignore	EXTENSION TxHoppingConfiguration	PRESENCE optional},

...

}

PosSRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF PosSRSResource-Item

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceID-List			PosSRSResourceID-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL

}

PosSRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AggregatedPosSRSResourceSetList	CRITICALITY ignore EXTENSION AggregatedPosSRSResourceSetList	PRESENCE optional},

...

}

PosValidityAreaCellList ::= SEQUENCE (SIZE(1.. maxnoVACell)) OF PosValidityAreaCellList-Item

PosValidityAreaCellList-Item ::= SEQUENCE {

nRCGI						NRCGI,

nRPCI						INTEGER (0..1007) 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { PosValidityAreaCellList-Item-ExtIEs } }	OPTIONAL

}

PosValidityAreaCellList-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourceSets)) OF PosSRSResourceSet-Item

PrimaryPathIndication ::= ENUMERATED {

true,

false,

...

}

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

Preconfigured-measurement-GAP-Request ::= ENUMERATED {true, ...}

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

ProtectedEUTRAResourceIndication		::= OCTET STRING

Protected-EUTRA-Resources-Item ::= SEQUENCE {

spectrumSharingGroupID					SpectrumSharingGroupID,

eUTRACells-List		EUTRACells-List,

iE-Extensions		ProtocolExtensionContainer { { Protected-EUTRA-Resources-ItemExtIEs } }	OPTIONAL

}

Protected-EUTRA-Resources-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSConfiguration ::= SEQUENCE {

pRSResourceSet-List			PRSResourceSet-List,

iE-Extensions				ProtocolExtensionContainer { { PRSConfiguration-ExtIEs } }	OPTIONAL

}

PRSConfiguration-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSInformationPos  ::= SEQUENCE {

pRS-IDPos					INTEGER(0..255),

pRS-Resource-Set-IDPos		INTEGER(0..7),

pRS-Resource-IDPos			INTEGER(0..63)	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { PRSInformationPos-ExtIEs} } OPTIONAL

}

PRSInformationPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRS-Measurement-Info-List ::= SEQUENCE (SIZE(1..maxFreqLayers)) OF PRS-Measurement-Info-List-Item

PRS-Measurement-Info-List-Item ::= SEQUENCE {

pointA						INTEGER (0..3279165),

measPRSPeriodicity			ENUMERATED {ms20, ms40, ms80, ms160, ...},

measPRSOffset				INTEGER (0..159, ...),

measurementPRSLength		ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

iE-Extensions				ProtocolExtensionContainer { { PRS-Measurement-Info-List-Item-ExtIEs} } OPTIONAL,

...

}

PRS-Measurement-Info-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Potential-SpCell-Item ::= SEQUENCE {

potential-SpCell-ID			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Potential-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Potential-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSAngleList ::= SEQUENCE (SIZE(1.. maxnoofPRS-ResourcesPerSet)) OF PRSAngleItem

PRSAngleItem ::= SEQUENCE {

nR-PRS-Azimuth			INTEGER (0..359),

nR-PRS-Azimuth-fine		INTEGER (0..9) OPTIONAL,

nR-PRS-Elevation		INTEGER (0..180) OPTIONAL,

nR-PRS-Elevation-fine	INTEGER (0..9) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { PRSAngleItem-ItemExtIEs } }	OPTIONAL

}

PRSAngleItem-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-PRS-Resource-ID		CRITICALITY ignore EXTENSION PRS-Resource-ID		PRESENCE optional },

...

}

PRSConfigRequestType ::= ENUMERATED {configure, off, ...}

PRSMuting::= SEQUENCE {

pRSMutingOption1			PRSMutingOption1		OPTIONAL,

pRSMutingOption2			PRSMutingOption2		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { PRSMuting-ExtIEs} } OPTIONAL

}

PRSMuting-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption1 ::= SEQUENCE {

mutingPattern					DL-PRSMutingPattern,

mutingBitRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf8,...},

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption1-ExtIEs} } OPTIONAL

}

PRSMutingOption1-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption2 ::= SEQUENCE {

mutingPattern					DL-PRSMutingPattern,

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption2-ExtIEs} } OPTIONAL

}

PRSMutingOption2-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRS-Resource-ID ::= INTEGER (0..63)

PRSResource-List::= SEQUENCE (SIZE (1..maxnoofPRSresources)) OF PRSResource-Item

PRSResource-Item  ::= SEQUENCE {

pRSResourceID			PRS-Resource-ID,

sequenceID				INTEGER(0..4095),

rEOffset				INTEGER(0..11,...),

resourceSlotOffset		INTEGER(0..511),

resourceSymbolOffset	INTEGER(0..12),

qCLInfo					PRSResource-QCLInfo		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { PRSResource-Item-ExtIEs} } OPTIONAL

}

PRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedResourceSymbolOffset		CRITICALITY ignore EXTENSION ExtendedResourceSymbolOffset 	PRESENCE optional}|

{ID id-AggregatedPRSResourceSetList	CRITICALITY ignore EXTENSION AggregatedPRSResourceSetList	PRESENCE optional},

...

}

PRSBandwidthAggregationRequestIndication ::= ENUMERATED{true, ...}

ExtendedResourceSymbolOffset ::= INTEGER (0..13,...)

PRSResource-QCLInfo  ::= CHOICE {

qCLSourceSSB		PRSResource-QCLSourceSSB,

qCLSourcePRS		PRSResource-QCLSourcePRS,

choice-extension	ProtocolIE-SingleContainer { { PRSResource-QCLInfo-ExtIEs } }

}

PRSResource-QCLInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PRSResource-QCLSourceSSB ::= SEQUENCE {

pCI-NR				INTEGER(0..1007),

sSB-Index 			SSB-Index OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PRSResource-QCLSourceSSB-ExtIEs} } OPTIONAL,

...

}

PRSResource-QCLSourceSSB-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-QCLSourcePRS ::= SEQUENCE {

qCLSourcePRSResourceSetID		PRS-Resource-Set-ID,

qCLSourcePRSResourceID 			PRS-Resource-ID OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSResource-QCLSourcePRS-ExtIEs} } OPTIONAL

}

PRSResource-QCLSourcePRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRS-Resource-Set-ID ::= INTEGER(0..7)

PRSResourceSet-List ::= SEQUENCE (SIZE (1.. maxnoofPRSresourceSets)) OF PRSResourceSet-Item

PRSResourceSet-Item ::= SEQUENCE {

pRSResourceSetID				PRS-Resource-Set-ID,

subcarrierSpacing				ENUMERATED{kHz15, kHz30, kHz60, kHz120, ...},

pRSbandwidth					INTEGER(1..63),

startPRB						INTEGER(0..2176),

pointA							INTEGER (0..3279165),

combSize						ENUMERATED{n2, n4, n6, n12, ...},

cPType							ENUMERATED{normal, extended, ...},

resourceSetPeriodicity			ENUMERATED{n4,n5,n8,n10,n16,n20,n32,n40,n64,n80,n160,n320,n640,n1280,n2560,n5120,n10240,n20480,n40960, n81920,..., n128, n256, n512},

resourceSetSlotOffset			INTEGER(0..81919,...),

resourceRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf6,rf8,rf16,rf32,...},

resourceTimeGap					ENUMERATED{tg1,tg2,tg4,tg8,tg16,tg32,...},

resourceNumberofSymbols			ENUMERATED{n2,n4,n6,n12,...,n1},

pRSMuting						PRSMuting 		OPTIONAL,

pRSResourceTransmitPower		INTEGER(-60..50),

pRSResource-List				PRSResource-List,

iE-Extensions					ProtocolExtensionContainer { { PRSResourceSet-Item-ExtIEs} } OPTIONAL

}

PRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffIndication ::= CHOICE {

pRSTransmissionOffPerTRP				NULL,

pRSTransmissionOffPerResourceSet		PRSTransmissionOffPerResourceSet,

pRSTransmissionOffPerResource			PRSTransmissionOffPerResource,

choice-extension		ProtocolIE-SingleContainer { { PRSTransmissionOffIndication-ExtIEs } }

}

PRSTransmissionOffIndication-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PRSTransmissionOffPerResource ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSets)) OF PRSTransmissionOffPerResource-Item

PRSTransmissionOffPerResource-Item  ::= SEQUENCE {

pRSResourceSetID								PRS-Resource-Set-ID,

pRSTransmissionOffIndicationPerResourceList		SEQUENCE (SIZE(1.. maxnoofPRSresources)) OF PRSTransmissionOffIndicationPerResource-Item,

iE-Extensions		ProtocolExtensionContainer { { PRSTransmissionOffPerResource-Item-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffPerResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffIndicationPerResource-Item  ::= SEQUENCE {

pRSResourceID		PRS-Resource-ID,

iE-Extensions		ProtocolExtensionContainer { { PRSTransmissionOffIndicationPerResource-Item-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffIndicationPerResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffInformation ::= SEQUENCE {

pRSTransmissionOffIndication	PRSTransmissionOffIndication,

iE-Extensions					ProtocolExtensionContainer { { PRSTransmissionOffInformation-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffPerResourceSet ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSets)) OF PRSTransmissionOffPerResourceSet-Item

PRSTransmissionOffPerResourceSet-Item  ::= SEQUENCE {

pRSResourceSetID		PRS-Resource-Set-ID,

iE-Extensions			ProtocolExtensionContainer { { PRSTransmissionOffPerResourceSet-Item-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffPerResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PWS-Failed-NR-CGI-Item ::= SEQUENCE {

nRCGI				NRCGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { { PWS-Failed-NR-CGI-ItemExtIEs } }	OPTIONAL,

...

}

PWS-Failed-NR-CGI-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PWSSystemInformation ::= SEQUENCE {

sIBtype 				SIBType-PWS,

sIBmessage				OCTET STRING,

iE-Extensions			ProtocolExtensionContainer { { PWSSystemInformationExtIEs } }	OPTIONAL,

...

}

PWSSystemInformationExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-NotificationInformation		CRITICALITY ignore	EXTENSION NotificationInformation		PRESENCE optional}|

{ ID id-AdditionalSIBMessageList	CRITICALITY reject	EXTENSION AdditionalSIBMessageList		PRESENCE optional},

...

}

PrivacyIndicator ::= ENUMERATED {immediate-MDT,	logged-MDT,	...}

PRS-ID ::= INTEGER(0..255)

PRSTRPList ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF PRSTRPItem

PRSTRPItem ::= SEQUENCE {

tRP-ID		TRPID,

requestedDLPRSTransmissionCharacteristics	RequestedDLPRSTransmissionCharacteristics 		OPTIONAL,

-- The IE shall be present if the PRS Configuration Request Type IE is set to “configure” --

pRSTransmissionOffInformation		PRSTransmissionOffInformation				OPTIONAL,

-- The IE shall be present if the PRS Configuration Request Type IE is set to “off” --

iE-Extensions	ProtocolExtensionContainer { { PRSTRPItem-ExtIEs} } OPTIONAL,

...

}

PRSTRPItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RequestedDLPRSTransmissionCharacteristics ::= SEQUENCE {

requestedDLPRSResourceSet-List		RequestedDLPRSResourceSet-List,

numberofFrequencyLayers			INTEGER(1..4)										OPTIONAL,

startTimeAndDuration			StartTimeAndDuration								OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { RequestedDLPRSTransmissionCharacteristics-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSTransmissionCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-PRSBandwidthAggregationRequestIndication CRITICALITY ignore EXTENSION PRSBandwidthAggregationRequestIndication	PRESENCE optional},

...

}

RequestedDLPRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSets)) OF RequestedDLPRSResourceSet-Item

RequestedDLPRSResourceSet-Item ::= SEQUENCE {

pRSbandwidth					INTEGER(1..63) OPTIONAL,

combSize						ENUMERATED{n2, n4, n6, n12, ...}		OPTIONAL,

resourceSetPeriodicity			ENUMERATED{n4,n5,n8,n10,n16,n20,n32,n40,n64,n80,n160,n320,n640,n1280,n2560,n5120,n10240,n20480,n40960, n81920,..., n128, n256, n512}	OPTIONAL,

resourceRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf6,rf8,rf16,rf32,...}		OPTIONAL,

resourceNumberofSymbols			ENUMERATED{n2,n4,n6,n12,...,n1}						OPTIONAL,

requestedDLPRSResource-List		RequestedDLPRSResource-List							OPTIONAL,

resourceSetStartTimeAndDuration	StartTimeAndDuration								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { RequestedDLPRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RequestedDLPRSResource-List::= SEQUENCE (SIZE (1..maxnoofPRSresources)) OF RequestedDLPRSResource-Item

RequestedDLPRSResource-Item  ::= SEQUENCE {

qCLInfo					PRSResource-QCLInfo		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedDLPRSResource-Item-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionTRPList ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF PRSTransmissionTRPItem

PRSTransmissionTRPItem ::= SEQUENCE {

tRP-ID				TRPID,

pRSConfiguration	PRSConfiguration,

iE-Extensions			ProtocolExtensionContainer { { PRSTransmissionTRPItem-ExtIEs} } OPTIONAL,

...

}

PRSTransmissionTRPItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PreambleIndex ::= INTEGER(0..63)

PDUSetQoSParameters ::= SEQUENCE {

ulPDUSetQoSInformation					PDUSetQoSInformation	OPTIONAL,

dlPDUSetQoSInformation					PDUSetQoSInformation	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUSetQoSParameters-ExtIEs } }	OPTIONAL

}

PDUSetQoSParameters-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDUSetQoSInformation	::= SEQUENCE {

pduSetDelayBudget						ExtendedPacketDelayBudget		OPTIONAL,

pduSetErrorRate							PacketErrorRate					OPTIONAL,

pduSetIntegratedHandlingInformation		ENUMERATED {true, false, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUSetQoSInformation-ExtIEs } }	OPTIONAL

}

PDUSetQoSInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PSIbasedSDUdiscardUL ::= ENUMERATED {start, stop, ...}

#### -- Q

QCI ::= INTEGER (0..255)

QoEInformation ::= SEQUENCE {

qoEInformationList		QoEInformationList,

iE-Extensions			ProtocolExtensionContainer { { QoEInformation-ExtIEs} } OPTIONAL

}

QoEInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

QoEInformationList ::= SEQUENCE (SIZE(1.. maxnoofQoEInformation)) OF QoEInformationList-Item

QoEInformationList-Item ::= SEQUENCE {

qoEMetrics			QoEMetrics	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { QoEInformationList-Item-ExtIEs} } 	OPTIONAL

}

QoEInformationList-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-dRB-List CRITICALITY ignore EXTENSION DRB-List PRESENCE optional},

...

}

QoEMetrics ::= SEQUENCE {

appLayerBufferLevelList				AppLayerBufferLevelList  OPTIONAL,

playoutDelayForMediaStartup			PlayoutDelayForMediaStartup OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { QoEMetrics-ExtIEs} } OPTIONAL,

...

}

QoEMetrics-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				NonDynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer { { QoS-Characteristics-ExtIEs } }

}

QoS-Characteristics-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

QoSFlowIdentifier ::= INTEGER (0..63)

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation				OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionID								CRITICALITY ignore	EXTENSION PDUSessionID				PRESENCE optional}|

{ ID id-ULPDUSessionAggregateMaximumBitRate			CRITICALITY ignore	EXTENSION BitRate						PRESENCE optional}|

{ ID id-QosMonitoringRequest						CRITICALITY ignore	EXTENSION QosMonitoringRequest		PRESENCE optional}|

{ ID id-PDCPTerminatingNodeDLTNLAddrInfo			CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional	}|

{ ID id-PDUSetQoSParameters							CRITICALITY	ignore	EXTENSION PDUSetQoSParameters		PRESENCE optional},

...

}

QoSFlowMappingIndication ::= ENUMERATED {ul,dl,...}

QoSInformation	::=	CHOICE {

eUTRANQoS					EUTRANQoS,

choice-extension			ProtocolIE-SingleContainer { { QoSInformation-ExtIEs} }

}

QoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

{	ID id-DRB-Information		CRITICALITY ignore TYPE DRB-Information		PRESENCE mandatory},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both, ..., stop}

QoSParaSetIndex ::= INTEGER (1..8, ...)

QoSParaSetNotifyIndex ::= INTEGER (0..8, ...)

#### -- R

RACH-Config-Common	::= OCTET STRING

RACH-Config-Common-IAB	::= OCTET STRING

Range ::= ENUMERATED {m50, m80, m180, m200, m350, m400, m500, m700, m1000, ...}

RAReportContainer::= OCTET STRING

RAReportList	::= SEQUENCE (SIZE(1.. maxnoofRAReports)) OF RAReportItem

RAReportItem	::= SEQUENCE {

rAReportContainer				RAReportContainer,

uEAssitantIdentifier			GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { RAReportItem-ExtIEs} }	OPTIONAL,

...

}

RAReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RAReportIndicationList ::= SEQUENCE (SIZE(1..maxnoofUEsforRAReportIndications)) OF RAReportIndicationList-Item

RAReportIndicationList-Item ::= SEQUENCE {

gNB-CU-UE-F1AP-ID					GNB-CU-UE-F1AP-ID,

iE-Extensions						ProtocolExtensionContainer { { RAReportIndicationList-Item-ExtIEs} } OPTIONAL,

...

}

RAReportIndicationList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RadioResourceStatus ::= SEQUENCE {

sSBAreaRadioResourceStatusList		SSBAreaRadioResourceStatusList,

iE-Extensions	ProtocolExtensionContainer { { RadioResourceStatus-ExtIEs} } OPTIONAL

}

RadioResourceStatus-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceRadioResourceStatus		CRITICALITY ignore	EXTENSION SliceRadioResourceStatus		PRESENCE optional	}|

{ ID id-MIMOPRBusageInformation			CRITICALITY ignore	EXTENSION MIMOPRBusageInformation			PRESENCE optional	},

...

}

RadioResourceStatusNR-U ::= SEQUENCE {

dl-Total-PRB-usage 	INTEGER (0..100),

ul-Total-PRB-usage 	INTEGER (0..100),

iE-Extensions					ProtocolExtensionContainer { { RadioResourceStatusNR-U-ExtIEs} }	OPTIONAL,

...

}

RadioResourceStatusNR-U-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MIMOPRBusageInformation ::= SEQUENCE {

dl-GBR-PRB-usage-for-MIMO		INTEGER (0..100),

ul-GBR-PRB-usage-for-MIMO		INTEGER (0..100),

dl-non-GBR-PRB-usage-for-MIMO	INTEGER (0..100),

ul-non-GBR-PRB-usage-for-MIMO 	INTEGER (0..100),

dl-Total-PRB-usage-for-MIMO 	INTEGER (0..100),

ul-Total-PRB-usage-for-MIMO 	INTEGER (0..100),

iE-Extensions					ProtocolExtensionContainer { { MIMOPRBusageInformation-ExtIEs} }	OPTIONAL,

...

}

MIMOPRBusageInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RANfeedbacktype ::= CHOICE {

proactive					RANfeedbacktype-proactive,

reactive					RANfeedbacktype-reactive,

choice-extensions			ProtocolIE-SingleContainer { {RANfeedbacktype-ExtIEs} }

}

RANfeedbacktype-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RANfeedbacktype-proactive ::= SEQUENCE {

burstArrivalTimeWindow	BurstArrivalTimeWindow,

periodicityRange		PeriodicityRange	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {RANfeedbacktype-proactive-ExtIEs} }	OPTIONAL,

...

}

RANfeedbacktype-proactive-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RANfeedbacktype-reactive ::= SEQUENCE {

capabilityForBATAdaptation	ENUMERATED {true, ...},

iE-Extension			ProtocolExtensionContainer { {RANfeedbacktype-reactive-ExtIEs} }	OPTIONAL,

...

}

RANfeedbacktype-reactive-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RANTSSRequestType ::= ENUMERATED {start, stop, ...}

RANTimingSynchronisationStatusInfo ::= SEQUENCE {

synchronisationstate				ENUMERATED {locked, holdover, freeRun, ...}      OPTIONAL,

traceabletoUTC						ENUMERATED { true, false, ...}                  OPTIONAL,

traceabletoGNSS						ENUMERATED { true, false, ...}                  OPTIONAL,

clockFrequencyStability				BIT STRING (SIZE(16))                           OPTIONAL,

clockAccuracy						ClockAccuracy                                   OPTIONAL,

parentTimeSource					ParentTImeSource                                OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { RANTimingSynchronisationStatusInfo-ExtIEs} }	OPTIONAL,

...

}

RANTimingSynchronisationStatusInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ClockAccuracy ::= CHOICE {

clockAccuracyValue		INTEGER (1..40000000, ...),

clockAccuracyIndex		INTEGER (32..47, ...),

choice-Extensions		ProtocolIE-SingleContainer { { ClockAccuracy-ExtIEs} }

}

ClockAccuracy-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RANAC ::= INTEGER (0..255)

RAN-MeasurementID ::= INTEGER (1.. 65536, ...)

RAN-UE-MeasurementID ::= INTEGER (1.. 256, ...)

RAN-UE-PDC-MeasID ::= INTEGER (1..16, ...)

RANUEID ::= OCTET STRING (SIZE (8))

RANUEPagingIdentity ::= SEQUENCE	{

iRNTI						BIT STRING (SIZE(40)),

iE-Extensions				ProtocolExtensionContainer { { RANUEPagingIdentity-ExtIEs } }	OPTIONAL}

RANUEPagingIdentity-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RAT-FrequencyPriorityInformation::= CHOICE {

eNDC		SubscriberProfileIDforRFP,

nGRAN		RAT-FrequencySelectionPriority,

choice-extension			ProtocolIE-SingleContainer { { RAT-FrequencyPriorityInformation-ExtIEs} }

}

RAT-FrequencyPriorityInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RAT-FrequencySelectionPriority::= INTEGER (1.. 256, ...)

RBSetConfiguration ::= SEQUENCE {

subcarrierSpacing				SubcarrierSpacing,

rBSetSize						RBSetSize,

nUmberRBsets					INTEGER(1..maxnoofRBsetsPerCell),

iE-Extensions					ProtocolExtensionContainer { { RBSetConfiguration-ExtIEs} } OPTIONAL

}

RBSetConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RBSetSize ::=	ENUMERATED { rb2, rb4, rb8, rb16, rb32, rb64}

Re-routingEnableIndicator ::= ENUMERATED {

true,

false,

...

}

Recommended-SSBs-for-Paging-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF Recommended-SSBs-for-Paging-List-Item

Recommended-SSBs-for-Paging-List-Item::= SEQUENCE {

nRCGI			 			NRCGI,

sSBs-forPaging-List 		SSBs-forPaging-List,

iE-Extensions				ProtocolExtensionContainer { { Recommended-SSBs-for-Paging-List-Item-ExtIEs} } OPTIONAL

}

Recommended-SSBs-for-Paging-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Redcap-Bcast-Information ::= BIT STRING(SIZE(8))

RedCapIndication ::= ENUMERATED {true, ...}

Reestablishment-Indication	::=	ENUMERATED  {

reestablished,

...

}

ReferencePoint ::= CHOICE {

coordinateID					CoordinateID,

referencePointCoordinate		AccessPointPosition,

referencePointCoordinateHA		NGRANHighAccuracyAccessPointPosition,

choice-Extension				ProtocolIE-SingleContainer { { ReferencePoint-ExtIEs} }

}

ReferencePoint-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ReferenceSFN ::= INTEGER (0..1023)

ReferenceSignal ::= CHOICE {

nZP-CSI-RS								NZP-CSI-RS-ResourceID,

sSB										SSB,

sRS										SRSResourceID,

positioningSRS							SRSPosResourceID,

dL-PRS									DL-PRS,

choice-extension						ProtocolIE-SingleContainer {{ReferenceSignal-ExtIEs }}

}

ReferenceSignal-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RA-RNTI ::= INTEGER (0..65535, ...)

ReferenceConfiguration ::= CHOICE {

rEQUESTforLowerLayerConfiguration			RequestforLowerLayerConfiguration,

referenceConfiguration					ReferenceConfigurationInformation,

choice-extension ProtocolIE-SingleContainer { { ReferenceConfiguration-ExtIEs } }

}

ReferenceConfiguration-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RelativeCartesianLocation ::= SEQUENCE {

xYZunit						ENUMERATED {mm, cm, dm, ...},

xvalue						INTEGER (-65536..65535),

yvalue						INTEGER (-65536..65535),

zvalue						INTEGER (-32768..32767),

locationUncertainty			LocationUncertainty,

iE-Extensions				ProtocolExtensionContainer { { RelativeCartesianLocation-ExtIEs} } OPTIONAL

}

RelativeCartesianLocation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativeGeodeticLocation ::= SEQUENCE {

milli-Arc-SecondUnits		ENUMERATED {zerodot03, zerodot3, three, ...},

heightUnits					ENUMERATED {mm, cm, m, ...},

deltaLatitude				INTEGER (-1024.. 1023),

deltaLongitude				INTEGER (-1024.. 1023),

deltaHeight					INTEGER (-1024.. 1023),

locationUncertainty			LocationUncertainty,

iE-extensions				ProtocolExtensionContainer {{RelativeGeodeticLocation-ExtIEs }} OPTIONAL

}

RelativeGeodeticLocation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RemoteUELocalID ::= INTEGER (0..255, ...)

ReferenceTime ::= OCTET STRING

RegistrationRequest ::= ENUMERATED{start, stop, add, ...}

ReportCharacteristics ::= BIT STRING (SIZE(32))

ReportingGranularitykminus1 ::= INTEGER(0..3940097)

ReportingGranularitykminus2 ::= INTEGER(0..7880193)

ReportingGranularitykminus3 ::= INTEGER(0..15760385)

ReportingGranularitykminus4 ::= INTEGER(0..31520769)

ReportingGranularitykminus5 ::= INTEGER(0..63041537)

ReportingGranularitykminus6 ::= INTEGER(0..126083073)

ReportingGranularitykminus1AdditionalPath ::= INTEGER(0..32701)

ReportingGranularitykminus2AdditionalPath ::= INTEGER(0..65401)

ReportingGranularitykminus3AdditionalPath ::= INTEGER(0..130801)

ReportingGranularitykminus4AdditionalPath ::= INTEGER(0..261601)

ReportingGranularitykminus5AdditionalPath ::= INTEGER(0..523201)

ReportingGranularitykminus6AdditionalPath ::= INTEGER(0..1046401)

ReportingPeriodicity ::= ENUMERATED{ms500, ms1000, ms2000, ms5000, ms10000, ...}

RequestedBandCombinationIndex ::= OCTET STRING

RequestedFeatureSetEntryIndex ::= OCTET STRING

RequestedP-MaxFR2 ::= OCTET STRING

Requested-PDCCH-BlindDetectionSCG ::= OCTET STRING

RequestedSRSPreconfigurationCharacteristics-List ::= SEQUENCE (SIZE (1.. maxnoPreconfiguredSRS)) OF RequestedSRSPreconfigurationCharacteristics-Item

RequestedSRSPreconfigurationCharacteristics-Item ::= SEQUENCE {

requestedSRSTransmissionCharacteristics 	RequestedSRSTransmissionCharacteristics,

iE-Extensions			ProtocolExtensionContainer {{ RequestedSRSPreconfigurationCharacteristics-Item-ExtIEs}}	OPTIONAL,

...

}

RequestedSRSPreconfigurationCharacteristics-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions		INTEGER (0..500, ...)		OPTIONAL,

-- The IE shall be present if the Resource Type IE is set to “periodic” --

resourceType				ENUMERATED  {periodic, semi-persistent, aperiodic,...},

bandwidthSRS				BandwidthSRS,

sRSResourceSetList 			SRSResourceSetList				OPTIONAL,

sSBInformation				SSBInformation				OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedSRSTransmissionCharacteristics-ExtIEs} } OPTIONAL

}

RequestedSRSTransmissionCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SrsFrequency		CRITICALITY ignore EXTENSION SrsFrequency		PRESENCE optional }|

{ ID id-BW-Aggregation-Request-Indication		CRITICALITY ignore EXTENSION BW-Aggregation-Request-Indication	PRESENCE optional }|

{ ID id-PosValidityAreaCellList					CRITICALITY ignore EXTENSION PosValidityAreaCellList			PRESENCE optional }|

{ ID id-ValidityAreaSpecificSRSInformation		CRITICALITY ignore EXTENSION ValidityAreaSpecificSRSInformation	PRESENCE optional },

...

}

RequestType	::= ENUMERATED {offer, execution, ...}

ResourceCoordinationEUTRACellInfo ::= SEQUENCE {

eUTRA-Mode-Info 						EUTRA-Coex-Mode-Info,

eUTRA-PRACH-Configuration 				EUTRA-PRACH-Configuration,

iE-Extensions	ProtocolExtensionContainer { { ResourceCoordinationEUTRACellInfo-ExtIEs } }	OPTIONAL,

...

}

ResourceCoordinationEUTRACellInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-IgnorePRACHConfiguration		CRITICALITY reject EXTENSION IgnorePRACHConfiguration		PRESENCE optional },

...

}

ResourceCoordinationTransferInformation ::= SEQUENCE {

meNB-Cell-ID								EUTRA-Cell-ID,

resourceCoordinationEUTRACellInfo		ResourceCoordinationEUTRACellInfo	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { ResourceCoordinationTransferInformation-ExtIEs } }	OPTIONAL,

...

}

ResourceCoordinationTransferInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordinationTransferContainer ::= OCTET STRING

ResourceMapping ::= SEQUENCE {

startPosition			INTEGER (0..13),

nrofSumbols				ENUMERATED {n1, n2, n4, n8, n12},

iE-Extensions		ProtocolExtensionContainer { { ResourceMapping-ExtIEs} }	OPTIONAL,

...

}

ResourceMapping-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetType  ::= CHOICE {

periodic			ResourceSetTypePeriodic,

semi-persistent		ResourceSetTypeSemi-persistent,

aperiodic			ResourceSetTypeAperiodic,

choice-extension				ProtocolIE-SingleContainer {{ ResourceSetType-ExtIEs }}

}

ResourceSetType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResourceSetTypePeriodic ::= SEQUENCE {

periodicSet			ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypePeriodic-ExtIEs} }	OPTIONAL

}

ResourceSetTypePeriodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeSemi-persistent ::= SEQUENCE {

semi-persistentSet	ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypeSemi-persistent-ExtIEs} }	OPTIONAL

}

ResourceSetTypeSemi-persistent-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeAperiodic ::= SEQUENCE {

sRSResourceTrigger-List 	INTEGER(1..3),

slotoffset					INTEGER(0..32),

iE-Extensions				ProtocolExtensionContainer { { ResourceSetTypeAperiodic-ExtIEs} }	OPTIONAL

}

ResourceSetTypeAperiodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RepetitionFactorExtended ::=  ENUMERATED {n3, n5, n6, n7, n8, n10, n12, n14, ...}

RepetitionPeriod ::= INTEGER (0..131071, ...)

ReportingRequestType ::= SEQUENCE {

eventType						EventType,

reportingPeriodicityValue						ReportingPeriodicityValue		OPTIONAL,

-- C-ifEventTypeisPeriodic: This IE shall be present if the Event Type IE is set to "periodic" in the Event Type IE.

iE-Extensions					ProtocolExtensionContainer { {ReportingRequestType-ExtIEs} }	OPTIONAL

}

ReportingRequestType-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceType ::= CHOICE {

periodic			ResourceTypePeriodic,

semi-persistent		ResourceTypeSemi-persistent,

aperiodic			ResourceTypeAperiodic,

choice-extension				ProtocolIE-SingleContainer {{ ResourceType-ExtIEs }}

}

ResourceType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodic ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset				INTEGER(0..2559, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypePeriodic-ExtIEs} }	OPTIONAL

}

ResourceTypePeriodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistent ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset				INTEGER(0..2559, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistent-ExtIEs} }	OPTIONAL

}

ResourceTypeSemi-persistent-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodic ::= SEQUENCE {

aperiodicResourceType	   ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodic-ExtIEs} }	OPTIONAL

}

ResourceTypeAperiodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypePos ::= CHOICE {

periodic			ResourceTypePeriodicPos,

semi-persistent		ResourceTypeSemi-persistentPos,

aperiodic			ResourceTypeAperiodicPos,

choice-extension	ProtocolIE-SingleContainer {{ ResourceTypePos-ExtIEs }}

}

ResourceTypePos-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodicPos ::= SEQUENCE {

periodicity			SRS-Periodicity,

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypePeriodicPos-ExtIEs} }	OPTIONAL

}

ResourceTypePeriodicPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistentPos ::= SEQUENCE {

periodicity			SRS-Periodicity,

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistentPos-ExtIEs} }	OPTIONAL

}

ResourceTypeSemi-persistentPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodicPos ::= SEQUENCE {

slotOffset			INTEGER (0..32),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodicPos-ExtIEs} }	OPTIONAL

}

ResourceTypeAperiodicPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationInformation ::= SEQUENCE {

rLCDuplicationStateList 			RLCDuplicationStateList,

primaryPathIndication			PrimaryPathIndication	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RLCDuplicationInformation-ExtIEs} }	OPTIONAL

}

RLCDuplicationInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationStateList	::= SEQUENCE (SIZE(1..maxnoofRLCDuplicationState)) OF RLCDuplicationState-Item

RLCDuplicationState-Item ::=SEQUENCE {

duplicationState		DuplicationState,

iE-Extensions	ProtocolExtensionContainer { {RLCDuplicationState-Item-ExtIEs } }	OPTIONAL,

...

}

RLCDuplicationState-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCFailureIndication ::= SEQUENCE {

assocatedLCID				LCID,

iE-Extensions				ProtocolExtensionContainer { {RLCFailureIndication-ExtIEs} } OPTIONAL

}

RLCFailureIndication-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCMode ::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication 	Reestablishment-Indication,

iE-Extensions				ProtocolExtensionContainer { { RLC-Status-ExtIEs } } OPTIONAL,

...

}

RLC-Status-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLFReportInformationList	::= SEQUENCE (SIZE(1.. maxnoofRLFReports)) OF RLFReportInformationItem

RLFReportInformationItem	::= SEQUENCE {

nRUERLFReportContainer		NRUERLFReportContainer,

uEAssitantIdentifier		GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { RLFReportInformationItem-ExtIEs} }	OPTIONAL,

...

}

RLFReportInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RIMRSDetectionStatus ::= ENUMERATED {rs-detected, rs-disappeared, ...}

RRCContainer ::= OCTET STRING

RRCContainer-RRCSetupComplete ::= OCTET STRING

RRCDeliveryStatus ::= SEQUENCE	{

delivery-status 			PDCP-SN,

triggering-message			PDCP-SN,

iE-Extensions				ProtocolExtensionContainer { { RRCDeliveryStatus-ExtIEs } }	OPTIONAL}

RRCDeliveryStatus-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RRCDeliveryStatusRequest ::= ENUMERATED {true, ...}

RRCReconfigurationCompleteIndicator	::= ENUMERATED {

true,

...,

failure

}

RRC-Terminating-IAB-Donor-Related-Info	::= SEQUENCE {

rRC-TerminatingIAB-Donor-gNB-ID 		GlobalGNB-ID,

mobileIAB-MT-BAP-Address              	BAPAddress,

iE-Extensions					ProtocolExtensionContainer { { RRC-Terminating-IAB-Donor-Related-Info-ExtIEs} }	OPTIONAL,

...

}

RRC-Terminating-IAB-Donor-Related-Info-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RRC-Version ::= SEQUENCE	{

latest-RRC-Version			BIT STRING (SIZE(3)),

iE-Extensions				ProtocolExtensionContainer { { RRC-Version-ExtIEs } }	OPTIONAL}

RRC-Version-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-latest-RRC-Version-Enhanced		CRITICALITY ignore EXTENSION OCTET STRING (SIZE(3))		PRESENCE optional },

...

}

RoutingID ::= OCTET STRING

ResponseTime ::= SEQUENCE {

time      			INTEGER (1..128,...),

timeUnit	ENUMERATED {second, ten-seconds, ten-milliseconds,...},

iE-Extensions		ProtocolExtensionContainer { { ResponseTime-ExtIEs} }	OPTIONAL,

...

}

ResponseTime-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RACHConfiguration ::= OCTET STRING

RequestforRACHConfiguration  ::= ENUMERATED {true, ...}

RequestforLowerLayerConfiguration::= ENUMERATED {true, ...}

RxTxTimingErrorMargin ::= ENUMERATED {tc0dot5, tc1, tc2, tc4, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc64, tc80, tc96, tc128, ...}

#### -- S

SCell-FailedtoSetup-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-FailedtoSetupMod-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeRemoved-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeRemoved-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeRemoved-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeSetup-Item ::= SEQUENCE {

sCell-ID			NRCGI	,

sCellIndex			SCellIndex,

sCellULConfigured		CellULConfigured 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ServingCellMO		CRITICALITY ignore	EXTENSION ServingCellMO		PRESENCE optional	},

...

}

SCell-ToBeSetupMod-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

sCellIndex			SCellIndex,

sCellULConfigured		CellULConfigured 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ServingCellMO		CRITICALITY ignore	EXTENSION ServingCellMO		PRESENCE optional	},

...

}

SCellIndex ::=INTEGER (1..31, ...)

SCGActivationRequest ::= ENUMERATED {activate-scg, deactivate-scg, ...}

SCGActivationStatus ::= ENUMERATED {scg-activated, scg-deactivated, ...}

SCGIndicator	::=	ENUMERATED{released, ...}

SCPAC-Request ::= ENUMERATED {initiation, ...}

S-CPAC-Configuration	::= SEQUENCE {

referenceConfigurationInformation 	ReferenceConfigurationInformation			OPTIONAL,

completeConfigurationIndicator		CompleteConfigurationIndicator 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { S-CPAC-Configuration-ExtIEs } }	OPTIONAL,

...

}

S-CPAC-Configuration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

S-CPACLowerLayerReferenceConfigRequest	::=	ENUMERATED{true, ...}

SCS-480	::= INTEGER(0..319)

SCS-960	::= INTEGER(0..639)

SCS-SpecificCarrier ::=					SEQUENCE {

offsetToCarrier						INTEGER (0..2199,...),

subcarrierSpacing					ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

carrierBandwidth					INTEGER (1..275,...),

iE-Extensions						ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL

}

SCS-SpecificCarrier-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerConfigurationQueryIndication ::=	ENUMERATED {true, ...}

SDTBearerConfigurationInfo ::= SEQUENCE {

sDTBearerConfig-List			SDTBearerConfig-List,

iE-Extensions					ProtocolExtensionContainer { { SDTBearerConfigurationInfo-ExtIEs } } OPTIONAL

}

SDTBearerConfigurationInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerConfig-List ::= SEQUENCE (SIZE(1..maxnoofSDTBearers)) OF SDTBearerConfig-List-Item

SDTBearerConfig-List-Item ::= SEQUENCE{

sDTBearerType 							SDTBearerType,

sDTRLCBearerConfiguration				SDTRLCBearerConfiguration,

iE-Extensions			ProtocolExtensionContainer {{ SDTBearerConfig-List-Item-ExtIEs}}		OPTIONAL

}

SDTBearerConfig-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerType ::= CHOICE {

sRB					SRBID,

dRB					DRBID,

choice-extension				ProtocolIE-SingleContainer {{ SDTBearerType-ExtIEs }}

}

SDTBearerType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SDT-MAC-PHY-CG-Config ::= OCTET STRING

SDTInformation ::= SEQUENCE {

sdtIndicator					ENUMERATED {true,...},

sdtAssistantInformation			ENUMERATED {singlepacket, multiplepackets,...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SDTInformation-ExtIEs } } OPTIONAL

}

SDTInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTRLCBearerConfiguration ::= OCTET STRING

SDT-Termination-Request	::= ENUMERATED {radio-link-problem, normal, ...,sdt-volume-threshold-crossed}

SDT-Volume-Threshold ::= INTEGER(1.. 192000,...)

Search-window-information ::= SEQUENCE {

expectedPropagationDelay		INTEGER (-3841..3841,...),

delayUncertainty				INTEGER (1..246,...),

iE-Extensions					ProtocolExtensionContainer { { Search-window-information-ExtIEs } } OPTIONAL

}

Search-window-information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SerialNumber ::= BIT STRING (SIZE (16))

SIBType-PWS ::=INTEGER (6..8, ...)

SelectedBandCombinationIndex ::= OCTET STRING

SelectedFeatureSetEntryIndex ::= OCTET STRING

CG-ConfigInfo ::= OCTET STRING

ServCellInfoList ::= OCTET STRING

ServCellIndex ::= INTEGER (0..31, ...)

ServingCellMO ::= INTEGER (1..64, ...)

ServingCellMO-List-Item ::= SEQUENCE {

servingCellMO					ServingCellMO,

sSB-Frequency					INTEGER (0..3279165),

iE-Extensions					ProtocolExtensionContainer { { ServingCellMO-List-Item-ExtIEs } } OPTIONAL

}

ServingCellMO-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ServingCellMO-encoded-in-CGC-List ::= SEQUENCE (SIZE(1.. maxNrofBWPs)) OF ServingCellMO-encoded-in-CGC-Item

ServingCellMO-encoded-in-CGC-Item ::= SEQUENCE {

servingCellMO					ServingCellMO,

iE-Extensions					ProtocolExtensionContainer { { ServingCellMO-encoded-in-CGC-Item-ExtIEs } }	OPTIONAL,

...

}

ServingCellMO-encoded-in-CGC-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BWP-Id	CRITICALITY ignore	EXTENSION BWP-Id	PRESENCE optional },

...

}

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional }|

{	ID id-ERedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information	PRESENCE optional }

,

...

}

Serving-Cells-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF Serving-Cells-List-Item

Serving-Cells-List-Item ::= SEQUENCE{

nRCGI 								NRCGI,

iAB-MT-Cell-NA-Resource-Configuration-Mode-Info       IAB-MT-Cell-NA-Resource-Configuration-Mode-Info		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{Serving-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Serving-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Supported-MBS-FSA-ID-List::= SEQUENCE (SIZE(1.. maxnoofMBSFSAs)) OF MBS-FrequencySelectionArea-Identity

MBS-FrequencySelectionArea-Identity::= OCTET STRING (SIZE(3))

SFN-Offset ::= SEQUENCE {

sFN-Time-Offset					BIT STRING (SIZE(24)),

iE-Extensions		ProtocolExtensionContainer { {SFN-Offset-ExtIEs} } OPTIONAL,

...

}

SFN-Offset-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Add-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

gNB-DU-System-Information	GNB-DU-System-Information	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Add-ItemExtIEs} }	OPTIONAL,

...

}

Served-Cells-To-Add-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Delete-Item ::= SEQUENCE {

oldNRCGI					NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Delete-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Delete-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Modify-Item ::= SEQUENCE {

oldNRCGI					NRCGI							,

served-Cell-Information		Served-Cell-Information		,

gNB-DU-System-Information	GNB-DU-System-Information 	OPTIONAL	,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Modify-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Modify-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-EUTRA-Cells-Information::= SEQUENCE {

eUTRA-Mode-Info						EUTRA-Mode-Info,

protectedEUTRAResourceIndication	ProtectedEUTRAResourceIndication,

iE-Extensions						ProtocolExtensionContainer { {Served-EUTRA-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-EUTRA-Cell-Information-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Service-State ::= ENUMERATED {

in-service,

out-of-service,

...

}

Service-Status ::= SEQUENCE {

service-state				Service-State,

switchingOffOngoing			ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Service-Status-ExtIEs } }	OPTIONAL,

...

}

Service-Status-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativeTime1900 ::= 	BIT STRING (SIZE (64))

ShortDRXCycleLength ::=  ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ...}

ShortDRXCycleTimer ::= INTEGER (1..16)

SIB1-message ::= OCTET STRING

SIB10-message ::= OCTET STRING

SIB12-message ::= OCTET STRING

SIB13-message ::= OCTET STRING

SIB14-message ::= OCTET STRING

SIB15-message ::= OCTET STRING

SIB17-message ::= OCTET STRING

SIB20-message ::= OCTET STRING

SIB24-message ::= OCTET STRING

SIB22-message ::= OCTET STRING

SItype ::= INTEGER (1..32, ...)

SItype-List ::= SEQUENCE (SIZE(1.. maxnoofSITypes)) OF SItype-Item

SItype-Item ::= SEQUENCE {

sItype		SItype	,

iE-Extensions	ProtocolExtensionContainer { { SItype-ItemExtIEs } }	OPTIONAL

}

SItype-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SibtypetobeupdatedListItem ::= SEQUENCE {

sIBtype 			INTEGER (2..32,...),

sIBmessage			OCTET STRING,

valueTag			INTEGER (0..31,...),

iE-Extensions		ProtocolExtensionContainer { { SibtypetobeupdatedListItem-ExtIEs } }	OPTIONAL,

...

}

SibtypetobeupdatedListItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-areaScope	CRITICALITY ignore	EXTENSION	AreaScope	PRESENCE optional},

...

}

SidelinkRelayConfiguration ::= SEQUENCE {

gNB-DU-UE-F1APIDofRelayUE			GNB-DU-UE-F1AP-ID,

remoteUELocalID						RemoteUELocalID,

sidelinkConfigurationContainer		SidelinkConfigurationContainer		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { SidelinkRelayConfiguration-ExtIEs } }		OPTIONAL,

...

}

SidelinkRelayConfiguration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SidelinkConfigurationContainer ::= OCTET STRING

SLDRBID ::= INTEGER (1..512, ...)

SLDRBInformation ::= SEQUENCE {

sLDRB-QoS				PC5QoSParameters,

flowsMappedToSLDRB-List	FlowsMappedToSLDRB-List,

...

}

SLDRBs-FailedToBeModified-Item	::= SEQUENCE {

sLDRBID		SLDRBID		,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-FailedToBeModified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-FailedToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-FailedToBeSetup-Item	::= SEQUENCE {

sLDRBID	SLDRBID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-FailedToBeSetup-ItemExtIEs } }		OPTIONAL

}

SLDRBs-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

sLDRBID			SLDRBID	,

cause			Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

SLDRBs-FailedToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Modified-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Modified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Modified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ModifiedConf-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ModifiedConf-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ModifiedConf-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Required-ToBeModified-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Required-ToBeModified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Required-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Required-ToBeReleased-Item	::= SEQUENCE {

sLDRBID		SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Required-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Setup-Item ::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Setup-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-SetupMod-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-SetupMod-ItemExtIEs } }	OPTIONAL

}

SLDRBs-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ToBeModified-Item	::= SEQUENCE {

sLDRBID						SLDRBID,

sLDRBInformation			SLDRBInformation		OPTIONAL,

rLCMode						RLCMode					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeModified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-duplicationIndication  CRITICALITY ignore EXTENSION   DuplicationIndication		PRESENCE optional},

...

}

SLDRBs-ToBeReleased-Item	::= SEQUENCE {

sLDRBID			SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ToBeSetup-Item ::= SEQUENCE	{

sLDRBID						SLDRBID,

sLDRBInformation			SLDRBInformation,

rLCMode						RLCMode,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-duplicationIndication  CRITICALITY ignore EXTENSION   DuplicationIndication		PRESENCE optional},

...

}

SLDRBs-ToBeSetupMod-Item	::= SEQUENCE {

sLDRBID						SLDRBID,

sLDRBInformation			SLDRBInformation,

rLCMode						RLCMode			OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-duplicationIndication  CRITICALITY ignore EXTENSION   DuplicationIndication		PRESENCE optional},

...

}

SLDRXCycleList ::= SEQUENCE (SIZE(1.. maxnoofSLdestinations)) OF SLDRXCycleItem

SLDRXCycleItem ::= SEQUENCE {

rXUEID					BIT STRING (SIZE(24)),

sLDRXInformation		SLDRXInformation,

iE-Extensions			ProtocolExtensionContainer { { SLDRXCycleItem-ExtIEs } }	OPTIONAL,

...

}

SLDRXCycleItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRXInformation    ::= CHOICE {

sLDRXCycle			SLDRXCycleLength,

nosLDRX				SLDRXConfigurationIndicator,

choice-extension	ProtocolIE-SingleContainer { { SLDRXInformation-ExtIEs} }

}

SLDRXCycleLength ::= ENUMERATED{ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...}

SLDRXConfigurationIndicator ::= ENUMERATED{ release, ...}

SLDRXInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SL-PHY-MAC-RLC-Config ::= OCTET STRING

SL-PHY-MAC-RLC-ConfigExt ::= OCTET STRING

SL-RLC-ChannelToAddModList::= OCTET STRING

SL-ConfigDedicatedEUTRA-Info ::= OCTET STRING

SliceAvailableCapacity ::= SEQUENCE {

sliceAvailableCapacityList	SliceAvailableCapacityList,

iE-Extensions				ProtocolExtensionContainer { { SliceAvailableCapacity-ExtIEs} } OPTIONAL

}

SliceAvailableCapacity-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceAvailableCapacityList ::= SEQUENCE (SIZE(1.. maxnoofBPLMNsNR)) OF SliceAvailableCapacityItem

SliceAvailableCapacityItem ::= SEQUENCE {

pLMNIdentity					PLMN-Identity,

sNSSAIAvailableCapacity-List	SNSSAIAvailableCapacity-List,

iE-Extensions	ProtocolExtensionContainer { { SliceAvailableCapacityItem-ExtIEs} } OPTIONAL

}

SliceAvailableCapacityItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIAvailableCapacity-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIAvailableCapacity-Item

SNSSAIAvailableCapacity-Item ::= SEQUENCE {

sNSSAI		SNSSAI,

sliceAvailableCapacityValueDownlink	INTEGER (0..100)	OPTIONAL,

sliceAvailableCapacityValueUplink	INTEGER (0..100)	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SNSSAIAvailableCapacity-Item-ExtIEs } }	OPTIONAL

}

SNSSAIAvailableCapacity-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceRadioResourceStatus ::= SEQUENCE {

sliceRadioResourceStatus	SliceRadioResourceStatus-List,

iE-Extensions				ProtocolExtensionContainer { { SliceRadioResourceStatus-ExtIEs} } OPTIONAL

}

SliceRadioResourceStatus-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNsNR)) OF SliceRadioResourceStatus-Item

SliceRadioResourceStatus-Item::= SEQUENCE {

pLMNIdentity					PLMN-Identity,

sNSSAIRadioResourceStatus-List	SNSSAIRadioResourceStatus-List,

iE-Extensions					ProtocolExtensionContainer { { SliceRadioResourceStatus-Item-ExtIEs} } OPTIONAL

}

SliceRadioResourceStatus-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIRadioResourceStatus-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIRadioResourceStatus-Item

SNSSAIRadioResourceStatus-Item ::= SEQUENCE {

sNSSAI		SNSSAI,

sNSSAIdlGBRPRBusage			INTEGER (0..100),

sNSSAIulGBRPRBusage			INTEGER (0..100),

sNSSAIdlNonGBRPRBusage		INTEGER (0..100),

sNSSAIulNonGBRPRBusage		INTEGER (0..100),

sNSSAIdlTotalPRBallocation	INTEGER (0..100),

sNSSAIulTotalPRBallocation	INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { SNSSAIRadioResourceStatus-Item-ExtIEs } }	OPTIONAL

}

SNSSAIRadioResourceStatus-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SliceSupportItem

SliceSupportItem ::= SEQUENCE {

sNSSAI	SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { SliceSupportItem-ExtIEs } }	OPTIONAL

}

SliceSupportItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceToReportList ::= SEQUENCE (SIZE(1.. maxnoofBPLMNsNR)) OF SliceToReportItem

SliceToReportItem ::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions				ProtocolExtensionContainer { { SliceToReportItem-ExtIEs} } OPTIONAL

}

SliceToReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SlotNumber ::= INTEGER (0..79)

SLPositioning-Ranging-Service-Info ::= SEQUENCE{

sLPositioning-Ranging-Authorized		SLPositioning-Ranging-Authorized,

rSPP-transport-QoS-parameters			RSPP-transport-QoS-parameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SLPositioning-Ranging-Service-Info-ExtIEs} }	OPTIONAL,

...

}

SLPositioning-Ranging-Service-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLPositioning-Ranging-Authorized ::= ENUMERATED {

authorized,

not-authorized,

...

}

RSPP-transport-QoS-parameters ::= SEQUENCE {

rSPPQoSFlowList				RSPPQoSFlowList,

rSPPLinkAggregateBitRates	BitRate												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { RSPP-transport-QoS-parameters-ExtIEs} }	OPTIONAL,

...

}

RSPP-transport-QoS-parameters-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RSPPQoSFlowList ::= SEQUENCE (SIZE(1..maxnoofRSPPQoSFlows)) OF RSPPQoSFlowItem

RSPPQoSFlowItem ::= SEQUENCE {

pQI					FiveQI,

rSPPFlowBitRates	RSPPFlowBitRates											OPTIONAL,

range				Range														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { RSPPQoSFlowItem-ExtIEs} }	OPTIONAL,

...

}

RSPPQoSFlowItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RSPPFlowBitRates ::= SEQUENCE {

guaranteedFlowBitRate		BitRate,

maximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { { RSPPFlowBitRates-ExtIEs} }	OPTIONAL,

...

}

RSPPFlowBitRates-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAI-Item

SNSSAI-Item ::= SEQUENCE {

sNSSAI		SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-Item-ExtIEs } }	OPTIONAL

}

SNSSAI-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Slot-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofslots)) OF Slot-Configuration-Item

Slot-Configuration-Item ::= SEQUENCE {

slotIndex				INTEGER (0..5119, ...),

symbolAllocInSlot		SymbolAllocInSlot,

iE-Extensions			ProtocolExtensionContainer { { Slot-Configuration-ItemExtIEs } }	OPTIONAL

}

Slot-Configuration-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SlotOffsetForRemainingHopsList ::= SEQUENCE (SIZE (1..maxnoHopsMinusOne)) OF SlotOffsetForRemainingHopsItem

SlotOffsetForRemainingHopsItem ::= SEQUENCE {

slotOffsetRemainingHops			SlotOffsetRemainingHops,

iE-Extensions					ProtocolExtensionContainer { { SlotOffsetForRemainingHopsItem-ExtIEs} } OPTIONAL,

...

}

SlotOffsetForRemainingHopsItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SlotOffsetRemainingHops ::= CHOICE {

aperiodic			SlotOffsetRemainingHopsAperiodic,

semi-persistent		SlotOffsetRemainingHopsSemiPersistent,

periodic			SlotOffsetRemainingHopsPeriodic,

choice-extension		ProtocolIE-SingleContainer {{ SlotOffsetRemainingHops-ExtIEs }}

}

SlotOffsetRemainingHops-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SlotOffsetRemainingHopsAperiodic ::= SEQUENCE {

slotOffset			INTEGER (1..32)			OPTIONAL,

startPosition		INTEGER (0..13)			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SlotOffsetRemainingHopsAperiodic-ExtIEs} }	OPTIONAL,

...

}

SlotOffsetRemainingHopsAperiodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SlotOffsetRemainingHopsSemiPersistent ::= SEQUENCE {

sRSperiodicity		SRS-Periodicity,

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { SlotOffsetRemainingHopsSemiPersistent-ExtIEs} }	OPTIONAL,

...

}

SlotOffsetRemainingHopsSemiPersistent-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SlotOffsetRemainingHopsPeriodic ::= SEQUENCE {

sRSperiodicity		SRS-Periodicity,

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { SlotOffsetRemainingHopsSemiPeriodic-ExtIEs} }	OPTIONAL,

...

}

SlotOffsetRemainingHopsSemiPeriodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI ::= SEQUENCE {

sST			OCTET STRING (SIZE(1)),

sD			OCTET STRING (SIZE(3)) 	OPTIONAL	,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-ExtIEs } }	OPTIONAL

}

SNSSAI-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialDirectionInformation ::= SEQUENCE {

nR-PRSBeamInformation			NR-PRSBeamInformation,

iE-Extensions					ProtocolExtensionContainer { { SpatialDirectionInformation-ExtIEs } } OPTIONAL

}

SpatialDirectionInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationInfo ::= SEQUENCE {

spatialRelationforResourceID					SpatialRelationforResourceID,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationInfo-ExtIEs} }	OPTIONAL

}

SpatialRelationInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationforResourceID ::= SEQUENCE (SIZE(1..maxnoofSpatialRelations)) OF SpatialRelationforResourceIDItem

SpatialRelationforResourceIDItem ::= SEQUENCE {

referenceSignal		ReferenceSignal,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationforResourceIDItem-ExtIEs} }	OPTIONAL

}

SpatialRelationforResourceIDItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPerSRSResource ::= SEQUENCE {

spatialRelationPerSRSResource-List	SpatialRelationPerSRSResource-List,

iE-Extensions		ProtocolExtensionContainer { { SpatialRelationPerSRSResource-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationPerSRSResource-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPerSRSResource-List::= SEQUENCE(SIZE (1.. maxnoSRS-ResourcePerSet)) OF SpatialRelationPerSRSResourceItem

SpatialRelationPerSRSResourceItem ::= SEQUENCE {

referenceSignal		ReferenceSignal,

iE-Extensions		ProtocolExtensionContainer { { SpatialRelationPerSRSResourceItem-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationPerSRSResourceItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPos ::= CHOICE {

sSBPos					SSB,

pRSInformationPos		PRSInformationPos,

choice-extension		ProtocolIE-SingleContainer {{ SpatialInformationPos-ExtIEs }}

}

SpatialInformationPos-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SpectrumSharingGroupID ::= INTEGER (1..maxCellineNB)

SRBID ::= INTEGER (0..3, ..., 4 | 5)

SRBs-FailedToBeSetup-Item	::= SEQUENCE {

sRBID		SRBID	,

cause		Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-FailedToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

sRBID		SRBID		,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-FailedToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Modified-Item ::= SEQUENCE {

sRBID							SRBID,

lCID							LCID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Modified-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Required-ToBeReleased-Item	::= SEQUENCE {

sRBID	SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Required-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Setup-Item ::= SEQUENCE {

sRBID							SRBID,

lCID								LCID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-SetupMod-Item ::= SEQUENCE {

sRBID						SRBID,

lCID							LCID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-ToBeReleased-Item	::= SEQUENCE {

sRBID		SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-ToBeSetup-Item ::= SEQUENCE {

sRBID	 SRBID	,

duplicationIndication	DuplicationIndication	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDuplicationIndication	CRITICALITY ignore	EXTENSION AdditionalDuplicationIndication		PRESENCE optional	}|

{ ID id-SDTRLCBearerConfiguration		CRITICALITY ignore	EXTENSION SDTRLCBearerConfiguration				PRESENCE optional }|

{ ID id-SRBMappingInfo					CRITICALITY ignore	EXTENSION UuRLCChannelID							PRESENCE optional	},

...

}

SRBs-ToBeSetupMod-Item	::= SEQUENCE {

sRBID	SRBID,

duplicationIndication	DuplicationIndication	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDuplicationIndication	CRITICALITY ignore	EXTENSION AdditionalDuplicationIndication		PRESENCE optional	}|

{ ID id-SRBMappingInfo					CRITICALITY ignore	EXTENSION UuRLCChannelID							PRESENCE optional	}|

{ ID id-CG-SDTindicatorSetup			CRITICALITY reject	EXTENSION CG-SDTindicatorSetup						PRESENCE optional },

...

}

SRSCarrier-List ::= SEQUENCE (SIZE(1.. maxnoSRS-Carriers)) OF SRSCarrier-List-Item

SRSCarrier-List-Item ::= SEQUENCE {

pointA							INTEGER (0..3279165),

uplinkChannelBW-PerSCS-List		UplinkChannelBW-PerSCS-List,

activeULBWP						ActiveULBWP,

pci								NRPCI		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSCarrier-List-Item-ExtIEs } } OPTIONAL

}

SRSCarrier-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSConfig  ::= SEQUENCE {

sRSResource-List			SRSResource-List 		OPTIONAL,

posSRSResource-List			PosSRSResource-List 	OPTIONAL,

sRSResourceSet-List			SRSResourceSet-List 	OPTIONAL,

posSRSResourceSet-List		PosSRSResourceSet-List 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SRSConfig-ExtIEs } } OPTIONAL

}

SRSConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSConfiguration ::= SEQUENCE {

sRSCarrier-List		SRSCarrier-List,

iE-Extensions		ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL

}

SRSConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SrsFrequency ::= INTEGER (0..3279165)

SRSPortIndex ::= ENUMERATED {id1000, id1001, id1002, id1003,...}

SRSPosResourceID ::= INTEGER (0..63)

SRSPreconfiguration-List ::= SEQUENCE (SIZE (1.. maxnoPreconfiguredSRS)) OF SRSPreconfiguration-Item

SRSPreconfiguration-Item ::= SEQUENCE {

sRSPosRRCInactiveValidityAreaConfig		SRSPosRRCInactiveValidityAreaConfig,

posValidityAreaCellList 				PosValidityAreaCellList,

iE-Extensions			ProtocolExtensionContainer {{ SRSPreconfiguration-Item-ExtIEs}}			OPTIONAL,

...

}

SRSPreconfiguration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSResource::= SEQUENCE {

sRSResourceID					SRSResourceID,

nrofSRS-Ports					ENUMERATED {port1, ports2, ports4},

transmissionComb				TransmissionComb,

startPosition					INTEGER (0..13),

nrofSymbols						ENUMERATED {n1, n2, n4},

repetitionFactor				ENUMERATED {n1, n2, n4},

freqDomainPosition				INTEGER (0..67),

freqDomainShift					INTEGER (0..268),

c-SRS							INTEGER (0..63),

b-SRS							INTEGER (0..3),

b-hop							INTEGER (0..3),

groupOrSequenceHopping			ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType					ResourceType,

sequenceId						INTEGER (0..1023),

iE-Extensions					ProtocolExtensionContainer { { SRSResource-ExtIEs } } OPTIONAL

}

SRSResource-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-nrofSymbolsExtended			CRITICALITY ignore EXTENSION NrofSymbolsExtended			PRESENCE optional}|

{ ID id-repetitionFactorExtended		CRITICALITY ignore EXTENSION RepetitionFactorExtended 	PRESENCE optional}|

{ ID id-startRBHopping			CRITICALITY ignore EXTENSION StartRBHopping 			PRESENCE optional}|

{ ID id-startRBIndex			CRITICALITY ignore EXTENSION StartRBIndex 			PRESENCE optional},

...

}

SRSResourceID ::= INTEGER (0..63)

SRSResourceID-List::= SEQUENCE (SIZE (1..maxnoSRS-ResourcePerSet)) OF SRSResourceID

SRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-Resources)) OF SRSResource

SRSResourceSet::= SEQUENCE {

sRSResourceSetID				SRSResourceSetID,

sRSResourceID-List				SRSResourceID-List,

resourceSetType					ResourceSetType,

iE-Extensions					ProtocolExtensionContainer { { SRSResourceSet-ExtIEs } } OPTIONAL

}

SRSResourceSet-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceSetID ::= INTEGER (0..15, ...)

SRSResourceSetList ::= SEQUENCE (SIZE(1.. maxnoSRS-ResourceSets)) OF SRSResourceSetItem

SRSResourceSetItem ::= SEQUENCE {

numSRSresourcesperset		INTEGER (1..16, ...)	OPTIONAL,

periodicityList				PeriodicityList			OPTIONAL,

spatialRelationInfo			SpatialRelationInfo		OPTIONAL,

pathlossReferenceInfo		PathlossReferenceInfo	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRSResourceSetItemExtIEs } }	OPTIONAL

}

SRSResourceSetItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelationPerSRSResource	CRITICALITY ignore	EXTENSION SpatialRelationPerSRSResource PRESENCE optional},

...

}

SRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-ResourceSets)) OF SRSResourceSet

SRSResourceTrigger ::= SEQUENCE {

aperiodicSRSResourceTriggerList					AperiodicSRSResourceTriggerList,

iE-Extensions		ProtocolExtensionContainer { {SRSResourceTrigger-ExtIEs} }	OPTIONAL

}

SRSResourceTrigger-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSResourcetype ::= SEQUENCE {

sRSResourceTypeChoice					SRSResourceTypeChoice,

iE-Extensions		ProtocolExtensionContainer { { SRSResourcetype-ExtIEs} }	OPTIONAL,

...

}

SRSResourcetype-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSPortIndex			CRITICALITY ignore EXTENSION SRSPortIndex	PRESENCE optional },

...

}

SRSResourceTypeChoice ::= CHOICE {

sRSResourceInfo				SRSInfo,

posSRSResourceInfo			PosSRSInfo,

choice-extension			ProtocolIE-SingleContainer { { SRSResourceTypeChoice-ExtIEs} }

}

SRSResourceTypeChoice-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SRSInfo ::= SEQUENCE {

sRSResource			SRSResourceID,

...

}

SRS-Periodicity ::= ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920, ..., slot128, slot256, slot512, slot20480}

SRSPosRRCInactiveConfig ::= OCTET STRING

SRSPosRRCInactiveValidityAreaConfig ::= OCTET STRING

SRSPosRRCInactiveQueryIndication ::= ENUMERATED {true, ...}

PosSRSInfo ::= SEQUENCE {

posSRSResourceID		SRSPosResourceID,

...

}

SRSReservationType ::= ENUMERATED {reserve, release, ...}

SSB ::= SEQUENCE {

pCI-NR				NRPCI,

ssb-index			SSB-Index	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SSB-ExtIEs} }	OPTIONAL

}

SSBCoverageModification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF SSBCoverageModification-Item

SSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

sSBCoverageState				SSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { SSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

SSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBCoverageState ::= INTEGER (0..15, ...)

SSB-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSB-freqInfo ::= INTEGER (0..maxNRARFCN)

SSB-Index ::= INTEGER(0..63)

SSB-subcarrierSpacing ::=  ENUMERATED {kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, ...}

SSB-transmissionPeriodicity	::= ENUMERATED {sf10, sf20, sf40, sf80, sf160, sf320, sf640, ...}

SSB-transmissionTimingOffset ::= INTEGER (0..127, ...)

SSB-transmissionBitmap ::= CHOICE {

shortBitmap			BIT STRING (SIZE (4)),

mediumBitmap		BIT STRING (SIZE (8)),

longBitmap			BIT STRING (SIZE (64)),

choice-extension	ProtocolIE-SingleContainer { { SSB-transmisisonBitmap-ExtIEs} }

}

SSB-transmisisonBitmap-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SSBAreaCapacityValueList ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF	SSBAreaCapacityValueItem

SSBAreaCapacityValueItem ::= SEQUENCE {

sSBIndex				INTEGER(0..63),

sSBAreaCapacityValue	INTEGER (0..100),

iE-Extensions			ProtocolExtensionContainer { { SSBAreaCapacityValueItem-ExtIEs} } OPTIONAL

}

SSBAreaCapacityValueItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBAreaRadioResourceStatusList::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF	SSBAreaRadioResourceStatusItem

SSBAreaRadioResourceStatusItem::= SEQUENCE {

sSBIndex					INTEGER(0..63),

sSBAreaDLGBRPRBusage		INTEGER (0..100),

sSBAreaULGBRPRBusage		INTEGER (0..100),

sSBAreaDLnon-GBRPRBusage	INTEGER (0..100),

sSBAreaULnon-GBRPRBusage	INTEGER (0..100),

sSBAreaDLTotalPRBusage		INTEGER (0..100),

sSBAreaULTotalPRBusage		INTEGER (0..100),

dLschedulingPDCCHCCEusage	INTEGER (0..100)		OPTIONAL,

uLschedulingPDCCHCCEusage	INTEGER (0..100) 		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SSBAreaRadioResourceStatusItem-ExtIEs} } OPTIONAL

}

SSBAreaRadioResourceStatusItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBInformation ::= SEQUENCE {

sSBInformationList	SSBInformationList,

iE-Extensions		ProtocolExtensionContainer { { SSBInformation-ExtIEs } }	OPTIONAL

}

SSBInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBInformationList ::= SEQUENCE (SIZE(1.. maxnoofSSBs)) OF SSBInformationItem

SSBInformationItem ::= SEQUENCE {

sSB-Configuration	SSB-TF-Configuration,

pCI-NR				NRPCI,

iE-Extensions		ProtocolExtensionContainer { { SSBInformationItem-ExtIEs } }	OPTIONAL

}

SSBInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSB-PositionsInBurst ::= CHOICE {

shortBitmap						BIT STRING (SIZE (4)),

mediumBitmap					BIT STRING (SIZE (8)),

longBitmap						BIT STRING (SIZE (64)),

choice-extension				ProtocolIE-SingleContainer { {SSB-PositionsInBurst-ExtIEs} }

}

SSB-PositionsInBurst-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SSBs-activated-List ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSB-Index

SSBs-forPaging-List ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSB-Index

SSBs-toBeActivated-List ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSB-Index

SSB-TF-Configuration ::= SEQUENCE {

sSB-frequency				INTEGER (0..3279165),

sSB-subcarrier-spacing		ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, ..., kHz480, kHz960},

-- The value kHz60 is not supported in this version of the specification.

sSB-Transmit-power			INTEGER (-60..50),

sSB-periodicity				ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, ...},

sSB-half-frame-offset		INTEGER(0..1),

sSB-SFN-offset				INTEGER(0..15),

sSB-position-in-burst		SSB-PositionsInBurst		OPTIONAL,

sFNInitialisationTime		RelativeTime1900		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SSB-TF-Configuration-ExtIEs} } OPTIONAL

}

SSB-TF-Configuration-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBToReportList ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSBToReportItem

SSBToReportItem ::= SEQUENCE {

sSBIndex					INTEGER(0..63),

iE-Extensions				ProtocolExtensionContainer { { SSBToReportItem-ExtIEs} } OPTIONAL

}

SSBToReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

StartRBIndex  ::= CHOICE{

freqScalingFactor2   INTEGER(0..1),

freqScalingFactor4   INTEGER(0..3),

choice-extension	 ProtocolIE-SingleContainer { { StartRBIndex-ExtIEs} }

}

StartRBIndex-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

StartRBHopping  ::= ENUMERATED {enable}

StartTimeAndDuration ::= SEQUENCE {

startTime			RelativeTime1900				OPTIONAL,

duration			INTEGER (0..90060, ...)			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { StartTimeAndDuration-ExtIEs } }	OPTIONAL,

...

}

StartTimeAndDuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SUL-Information ::= SEQUENCE {

sUL-NRARFCN							INTEGER (0..maxNRARFCN),

sUL-transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions						ProtocolExtensionContainer { { SUL-InformationExtIEs} } OPTIONAL,

...

}

SUL-InformationExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },

...

}

SubcarrierSpacing ::=	ENUMERATED { kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, ...}

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

SuccessfulHOReportInformationList::= SEQUENCE (SIZE(1.. maxnoofSuccessfulHOReports)) OF SuccessfulHOReportInformation-Item

SuccessfulHOReportInformation-Item ::= SEQUENCE {

successfulHOReportContainer				OCTET STRING,

iE-Extensions	ProtocolExtensionContainer { { SuccessfulHOReportInformation-Item-ExtIEs } }	OPTIONAL

}

SuccessfulHOReportInformation-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SuccessfulPSCellChangeReportInformationList::= SEQUENCE (SIZE(1.. maxnoofSuccessfulPSCellChangeReports)) OF SuccessfulPSCellChangeReportInformation-Item

SuccessfulPSCellChangeReportInformation-Item ::= SEQUENCE {

successfulPSCellChangeReportContainer				OCTET STRING,

iE-Extensions	ProtocolExtensionContainer { { SuccessfulPSCellChangeReportInformation-Item-ExtIEs } }	OPTIONAL

}

SuccessfulPSCellChangeReportInformation-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SULAccessIndication ::= ENUMERATED {true,...}

SupportedSULFreqBandItem ::= SEQUENCE {

freqBandIndicatorNr 			INTEGER (1..1024,...),

iE-Extensions					ProtocolExtensionContainer { { SupportedSULFreqBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULFreqBandItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SupportedUETypeList ::= SEQUENCE (SIZE(1.. maxnoofUETypes)) OF SupportedUETypeList-Item

SupportedUETypeList-Item ::= SEQUENCE {

supportedUEtype				ENUMERATED {non-redcap-ue, redcap-ue, ...},

iE-Extensions				ProtocolExtensionContainer { { SupportedUETypeList-Item-ExtIEs } } OPTIONAL,

...

}

SupportedUETypeList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SurvivalTime ::= INTEGER (0.. 1920000,...)

SymbolAllocInSlot ::= CHOICE {

all-DL					NULL,

all-UL					NULL,

both-DL-and-UL			NumDLULSymbols,

choice-extension		ProtocolIE-SingleContainer { { SymbolAllocInSlot-ExtIEs } }

}

SymbolAllocInSlot-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SymbolIndex ::= INTEGER (0..13)

SystemFrameNumber ::= INTEGER (0..1023)

SystemInformationAreaID ::=BIT STRING (SIZE (24))

#### -- T

TAI ::= SEQUENCE {

pLMN-Identity				         PLMN-Identity,

fiveGS-TAC					         FiveGS-TAC,

iE-Extensions					ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL,

...

}

TAI-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TAAssistanceInfo ::=  ENUMERATED{zero, ...}

FiveGS-TAC ::= OCTET STRING (SIZE(3))

Configured-EPS-TAC ::= OCTET STRING (SIZE(2))

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {

target-cell								NRCGI,

iE-Extensions							ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL

}

TargetCellList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NSAGSupportList ::= SEQUENCE (SIZE(1.. maxnoofNSAGs)) OF NSAGSupportItem

NSAGSupportItem ::= SEQUENCE {

nSAG-ID					NSAG-ID,

nSAGSliceSupport		ExtendedSliceSupportList,

iE-Extensions			ProtocolExtensionContainer { {NSAGSupportItem-ExtIEs} }	OPTIONAL,

...

}

NSAGSupportItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NSAG-ID ::= INTEGER (0..255, ...)

TCIStatesConfigurationsList	::= SEQUENCE {

jointorDLTCIStatesConfigurationsList	JointorDLTCIStatesConfigurationsList		OPTIONAL,

uLTCIStatesConfigurationsList			ULTCIStatesConfigurationsList				OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { TCIStatesConfigurationsList-ExtIEs} }	OPTIONAL,

...

}

TCIStatesConfigurationsList-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TAValue ::= INTEGER (0..4095)

TDD-Info ::= SEQUENCE {

nRFreqInfo						NRFreqInfo,

transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions					ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-IntendedTDD-DL-ULConfig	CRITICALITY ignore	EXTENSION	IntendedTDD-DL-ULConfig	PRESENCE optional}|

{ID id-TDD-UL-DLConfigCommonNR	CRITICALITY ignore	EXTENSION TDD-UL-DLConfigCommonNR	PRESENCE optional }|

{ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList				PRESENCE optional },

...

}

TDD-InfoRel16 ::= SEQUENCE {

tDD-FreqInfo									FreqInfoRel16													OPTIONAL,

sUL-FreqInfo									FreqInfoRel16													OPTIONAL,

tDD-UL-DLConfigCommonNR							TDD-UL-DLConfigCommonNR											OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {TDD-InfoRel16-ExtIEs} }		OPTIONAL,

...

}

TDD-InfoRel16-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TDD-UL-DLConfigCommonNR ::= OCTET STRING

TRPTEGInformation ::= CHOICE {

rxTx-TEG			RxTxTEG,

rx-TEG				RxTEG,

choice-extension		ProtocolIE-SingleContainer { { TRPTEGInformation-ExtIEs} }

}

TRPTEGInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RxTxTEG ::= SEQUENCE {

tRP-RxTx-TEGInformation			TRP-RxTx-TEGInformation,

tRP-Tx-TEGInformation			TRP-Tx-TEGInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { RxTxTEG-ExtIEs } }		OPTIONAL,

...

}

RxTxTEG-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RxTEG ::= SEQUENCE {

tRP-Rx-TEGInformation		TRP-Rx-TEGInformation,

tRP-Tx-TEGInformation		TRP-Tx-TEGInformation,

iE-Extensions			ProtocolExtensionContainer { { RxTEG-ExtIEs } }		OPTIONAL,

...

}

RxTEG-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeReferenceInformation ::= SEQUENCE {

referenceTime					ReferenceTime,

referenceSFN					ReferenceSFN,

uncertainty						Uncertainty,

timeInformationType				TimeInformationType,

iE-Extensions		ProtocolExtensionContainer { {TimeReferenceInformation-ExtIEs} }	OPTIONAL

}

TimeReferenceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeInformationType ::= ENUMERATED {localClock}

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL

}

TimeStamp-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SymbolIndex	 CRITICALITY ignore	EXTENSION SymbolIndex  	PRESENCE optional },

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-SingleContainer { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCS-480		CRITICALITY reject	TYPE SCS-480 PRESENCE mandatory}|

{ ID id-SCS-960		CRITICALITY reject	TYPE SCS-960 PRESENCE mandatory},

...

}

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TimingErrorMargin ::= ENUMERATED {m0Tc, m2Tc, m4Tc, m6Tc, m8Tc, m12Tc, m16Tc, m20Tc, m24Tc, m32Tc, m40Tc, m48Tc, m56Tc, m64Tc, m72Tc, m80Tc, ...}

TimingMeasurementQuality ::= SEQUENCE {

measurementQuality		INTEGER(0..31),

resolution				ENUMERATED{m0dot1, m1, m10, m30, ...},

iE-Extensions			ProtocolExtensionContainer { { TimingMeasurementQuality-ExtIEs} }	OPTIONAL

}

TimingMeasurementQuality-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimingReportingGranularityFactorExtended ::=INTEGER(-6..-1,...)

TimeWindowInformation-Measurement-List ::= SEQUENCE (SIZE (1.. maxnoofTimeWindowMea)) OF TimeWindowInformation-Measurement-Item

TimeWindowInformation-Measurement-Item ::= SEQUENCE {

timeWindowDurationMeasurement		TimeWindowDurationMeasurement,

timeWindowType						ENUMERATED {single, periodic, ...},

timeWindowPeriodicityMeasurement	TimeWindowPeriodicityMeasurement		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeWindowInformation-Measurement-Item-ExtIEs} }	OPTIONAL,

...}

TimeWindowInformation-Measurement-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeWindowInformation-SRS-List ::= SEQUENCE (SIZE (1.. maxnoofTimeWindowSRS)) OF TimeWindowInformation-SRS-Item

TimeWindowInformation-SRS-Item ::= SEQUENCE {

timeWindowStartSRS					TimeWindowStartSRS,

timeWindowDurationSRS				TimeWindowDurationSRS,

timeWindowType						ENUMERATED {single, periodic, ...},

timeWindowPeriodicitySRS			TimeWindowPeriodicitySRS				OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeWindowInformation-SRS-ExtIEs} }	OPTIONAL,

...

}

TimeWindowInformation-SRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeWindowDurationMeasurement ::= CHOICE {

durationSlots		ENUMERATED {n1, n2, n4, n6, n8, n12, n16, ...},

choice-extension		ProtocolIE-SingleContainer { { TimeWindowDurationMeasurement-ExtIEs} }

}

TimeWindowDurationMeasurement-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TimeWindowDurationSRS ::= CHOICE {

durationSymbols		ENUMERATED {n1, n2, n4, n8, n12, ...},

durationSlots		ENUMERATED {n1, n2, n4, n6, n8, n12, n16, ...},

choice-extension		ProtocolIE-SingleContainer { { TimeWindowDurationSRS-ExtIEs} }

}

TimeWindowDurationSRS-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TimeWindowPeriodicityMeasurement ::= ENUMERATED {ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms40960, ms61440, ms81920, ms368640, ms737280, ms1843200, ...}

TimeWindowPeriodicitySRS ::= ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot625, ms1, ms1dot25, ms2, ms2dot5, ms4, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ...}

TimeWindowStartSRS ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotNumber				SlotNumber,

symbolIndex				SymbolIndex,

iE-Extension			ProtocolExtensionContainer { { TimeWindowStartSRS-ExtIEs} }	OPTIONAL,

...

}

TimeWindowStartSRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TMGI ::=  OCTET STRING (SIZE(6))

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TNLCapacityIndicator::= SEQUENCE {

dLTNLOfferedCapacity		INTEGER (1.. 16777216,...),

dLTNLAvailableCapacity		INTEGER (0.. 100,...),

uLTNLOfferedCapacity		INTEGER (1.. 16777216,...),

uLTNLAvailableCapacity		INTEGER (0.. 100,...),

iE-Extensions	ProtocolExtensionContainer { { TNLCapacityIndicator-ExtIEs} } OPTIONAL

}

TNLCapacityIndicator-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TraceActivation ::= SEQUENCE {

traceID								TraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL

}

TraceActivation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-mdtConfiguration			CRITICALITY ignore	EXTENSION	MDTConfiguration		PRESENCE optional}|

{ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URI-address				PRESENCE optional	},

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TraceID ::= OCTET STRING (SIZE(8))

TrafficMappingInfo	::= CHOICE {

iPtolayer2TrafficMappingInfo					IPtolayer2TrafficMappingInfo,

bAPlayerBHRLCchannelMappingInfo					BAPlayerBHRLCchannelMappingInfo,

choice-extension								ProtocolIE-SingleContainer { { TrafficMappingInfo-ExtIEs} }

}

TrafficMappingInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TransportLayerAddress		::= BIT STRING (SIZE(1..160, ...))

TransactionID				::= INTEGER (0..255, ...)

Transmission-Bandwidth ::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { { Transmission-Bandwidth-ExtIEs} } OPTIONAL,

...

}

Transmission-Bandwidth-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionComb ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

choice-extension				ProtocolIE-SingleContainer { { TransmissionComb-ExtIEs} }

}

TransmissionComb-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-transmissionCombn8	CRITICALITY reject TYPE TransmissionCombn8 PRESENCE mandatory},

...

}

TransmissionCombn8 ::= SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5),

iE-Extensions			   ProtocolExtensionContainer { { TransmissionCombn8-ExtIEs} } OPTIONAL

}

TransmissionCombn8-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionCombPos ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

n8    SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5)

},

choice-extension				ProtocolIE-SingleContainer { { TransmissionCombPos-ExtIEs} }

}

TransmissionCombPos-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TransmissionStopIndicator ::= ENUMERATED {true, ... }

Transport-UP-Layer-Address-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Address-Info-To-Add-Item

Transport-UP-Layer-Address-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Address-Info-To-Add-ItemExtIEs } }	OPTIONAL

}

Transport-UP-Layer-Address-Info-To-Add-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Address-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Address-Info-To-Remove-Item

Transport-UP-Layer-Address-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Address-Info-To-Remove-ItemExtIEs } }	OPTIONAL

}

Transport-UP-Layer-Address-Info-To-Remove-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionActionIndicator ::= ENUMERATED {stop, ..., restart }

TRPBeamAntennaInformation ::= SEQUENCE {

choice-TRP-Beam-Antenna-Info-Item		Choice-TRP-Beam-Antenna-Info-Item								,

iE-Extensions			ProtocolExtensionContainer {{ TRPBeamAntennaInformation-ExtIEs}}				OPTIONAL,

...

}

TRPBeamAntennaInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Choice-TRP-Beam-Antenna-Info-Item ::= CHOICE {

reference					TRPID,

explicit					TRP-BeamAntennaExplicitInformation,

noChange					NULL,

choice-extension			ProtocolIE-SingleContainer { { Choice-TRP-Beam-Info-Item-ExtIEs } }

}

Choice-TRP-Beam-Info-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRP-BeamAntennaExplicitInformation ::= SEQUENCE {

trp-BeamAntennaAngles					TRP-BeamAntennaAngles,

lcs-to-gcs-translation	 				LCS-to-GCS-Translation										OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRP-BeamAntennaExplicitInformation-ExtIEs}}				OPTIONAL,

...

}

TRP-BeamAntennaExplicitInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-BeamAntennaAngles ::= SEQUENCE (SIZE (1.. maxnoAzimuthAngles)) OF TRP-BeamAntennaAnglesList-Item

TRP-BeamAntennaAnglesList-Item ::= SEQUENCE {

trp-azimuth-angle					INTEGER (0..359),

trp-azimuth-angle-fine				INTEGER (0..9)	OPTIONAL,

trp-elevation-angle-list			SEQUENCE (SIZE (1.. maxnoElevationAngles)) OF TRP-ElevationAngleList-Item,

iE-Extensions			ProtocolExtensionContainer {{ TRP-BeamAntennaAnglesList-Item-ExtIEs}}			OPTIONAL,

...

}

TRP-BeamAntennaAnglesList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-ElevationAngleList-Item ::= SEQUENCE {

trp-elevation-angle					INTEGER (0..180),

trp-elevation-angle-fine			INTEGER (0..9)	OPTIONAL,

trp-beam-power-list					SEQUENCE (SIZE (2..maxNumResourcesPerAngle)) OF TRP-Beam-Power-Item,

iE-Extensions			ProtocolExtensionContainer {{ TRP-ElevationAngleList-Item-ExtIEs}}			OPTIONAL,

...

}

TRP-ElevationAngleList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-Beam-Power-Item ::= SEQUENCE {

pRSResourceSetID				PRS-Resource-Set-ID													OPTIONAL,

pRSResourceID					PRS-Resource-ID,

relativePower					INTEGER (0..30), --negative value

relativePowerFine				INTEGER (0..9)			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRP-Beam-Power-Item-ExtIEs}}					OPTIONAL,

...

}

TRP-Beam-Power-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPID ::= INTEGER (0.. maxnoofTRPs, ...)

TRPInformation ::= SEQUENCE {

tRPID							TRPID,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL

}

TRPInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Mobile-IAB-MT-UE-ID			CRITICALITY reject EXTENSION Mobile-IAB-MT-UE-ID		PRESENCE optional},

--This IE shall be present if the TRP type IE is set to the value "mobile-trp"

...

}

TRPInformationItem ::= SEQUENCE {

tRPInformation					TRPInformation,

iE-Extensions					ProtocolExtensionContainer { { TRPInformationItem-ExtIEs } }	OPTIONAL

}

TRPInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPInformationTypeItem ::= ENUMERATED {

nrPCI,

nG-RAN-CGI,

arfcn,

pRSConfig,

sSBConfig,

sFNInitTime,

spatialDirectInfo,

geoCoord,

...,

trp-type,

ondemandPRS,

trpTxTeg,

beam-antenna-info,

mobile-trp-location-info

}

TRPInformationTypeResponseList ::= SEQUENCE (SIZE(1.. maxnoofTRPInfoTypes)) OF TRPInformationTypeResponseItem

TRPInformationTypeResponseItem ::= CHOICE {

pCI-NR								NRPCI,

nG-RAN-CGI							NRCGI,

nRARFCN								INTEGER (0..maxNRARFCN),

pRSConfiguration					PRSConfiguration,

sSBinformation						SSBInformation,

sFNInitialisationTime				RelativeTime1900,

spatialDirectionInformation			SpatialDirectionInformation,

geographicalCoordinates				GeographicalCoordinates,

choice-extension					ProtocolIE-SingleContainer { { TRPInformationTypeResponseItem-ExtIEs} }

}

TRPInformationTypeResponseItem-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TRPType						CRITICALITY reject		TYPE TRPType					PRESENCE mandatory }|

{ ID id-OnDemandPRS	 				CRITICALITY reject		TYPE OnDemandPRS-Info			PRESENCE mandatory}|

{ ID id-TRPTxTEGAssociation			CRITICALITY reject		TYPE TRPTxTEGAssociation		PRESENCE mandatory}|

{ ID id-TRPBeamAntennaInformation	CRITICALITY reject		TYPE TRPBeamAntennaInformation	PRESENCE mandatory }|

{ ID id-Mobile-TRP-LocationInformation		CRITICALITY ignore		TYPE Mobile-TRP-LocationInformation		PRESENCE mandatory },

...

}

TRPList ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF TRPListItem

TRPListItem ::= SEQUENCE {

tRPID							TRPID,

iE-Extensions					ProtocolExtensionContainer { { TRPListItem-ExtIEs } }	OPTIONAL

}

TRPListItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-PRSBandwidthAggregationRequestIndication CRITICALITY ignore EXTENSION PRSBandwidthAggregationRequestIndication	PRESENCE optional},

...

}

TRPMeasurementQuality ::= SEQUENCE {

tRPmeasurementQuality-Item 	TRPMeasurementQuality-Item,

iE-Extensions				ProtocolExtensionContainer { {TRPMeasurementQuality-ExtIEs} } OPTIONAL

}

TRPMeasurementQuality-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuality-Item ::= CHOICE {

timingMeasurementQuality	TimingMeasurementQuality,

angleMeasurementQuality		AngleMeasurementQuality,

choice-extension			ProtocolIE-SingleContainer { { TRPMeasurementQuality-Item-ExtIEs } }

}

TRPMeasurementQuality-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

{ID id-PhaseQuality				CRITICALITY ignore TYPE PhaseQuality		PRESENCE mandatory},

...

}

PhaseQuality ::= SEQUENCE {

phaseQualityIndex			INTEGER(0..179),

phaseQualityResolution		ENUMERATED {deg0dot1, deg1, ...},

iE-Extensions	ProtocolExtensionContainer { { PhaseQuality-ExtIEs } }	OPTIONAL

}

PhaseQuality-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-MeasurementRequestList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementRequestItem

TRP-MeasurementRequestItem ::= SEQUENCE {

tRPID							TRPID,

search-window-information		Search-window-information	OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { TRP-MeasurementRequestItem-ExtIEs } } OPTIONAL

}

TRP-MeasurementRequestItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRCGI					CRITICALITY ignore EXTENSION NRCGI					PRESENCE optional }|

{ ID id-AoA-SearchWindow		CRITICALITY ignore EXTENSION AoA-AssistanceInfo		PRESENCE optional }|

{ ID id-NumberOfTRPRxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTEG		PRESENCE optional }|

{ ID id-NumberOfTRPRxTxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTxTEG		PRESENCE optional },

...

}

TRP-PRS-Info-List ::= SEQUENCE (SIZE(1.. maxnoofPRSTRPs)) OF TRP-PRS-Info-List-Item

TRP-PRS-Info-List-Item ::= SEQUENCE {

tRP-ID				TRPID,

nR-PCI				NRPCI,

cGI-NR				NRCGI					OPTIONAL,

pRSConfiguration							PRSConfiguration,

iE-Extensions	ProtocolExtensionContainer { { TRP-PRS-Info-List-Item-ExtIEs} } OPTIONAL,

...

}

TRP-PRS-Info-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPPositionDefinitionType ::= CHOICE {

direct		TRPPositionDirect,

referenced	TRPPositionReferenced,

choice-extension							ProtocolIE-SingleContainer { { TRPPositionDefinitionType-ExtIEs } }

}

TRPPositionDefinitionType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRPPositionDirect ::= SEQUENCE {

accuracy	TRPPositionDirectAccuracy,

iE-extensions		ProtocolExtensionContainer { { TRPPositionDirect-ExtIEs } }	OPTIONAL

}

TRPPositionDirect-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPPositionDirectAccuracy ::= CHOICE {

tRPPosition				AccessPointPosition,

tRPHAposition			NGRANHighAccuracyAccessPointPosition,

choice-extension		ProtocolIE-SingleContainer { { TRPPositionDirectAccuracy-ExtIEs } }

}

TRPPositionDirectAccuracy-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRPPositionReferenced ::= SEQUENCE {

referencePoint					ReferencePoint,

referencePointType				TRPReferencePointType,

iE-extensions					ProtocolExtensionContainer { { TRPPositionReferenced-ExtIEs } } 	OPTIONAL

}

TRPPositionReferenced-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPReferencePointType ::= CHOICE {

tRPPositionRelativeGeodetic			RelativeGeodeticLocation,

tRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-SingleContainer { { TRPReferencePointType-ExtIEs } }

}

TRPReferencePointType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRP-Rx-TEGInformation ::= SEQUENCE {

tRP-Rx-TEGID					INTEGER (0..31),

tRP-Rx-TimingErrorMargin		TimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-Rx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-Rx-TEGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-RxTx-TEGInformation ::= SEQUENCE {

tRP-RxTx-TEGID					INTEGER (0..255),

tRP-RxTx-TimingErrorMargin		RxTxTimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-RxTx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-RxTx-TEGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-Tx-TEGInformation ::= SEQUENCE {

tRP-Tx-TEGID					INTEGER (0..7),

tRP-Tx-TimingErrorMargin		TimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-Tx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-Tx-TEGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPTxTEGAssociation ::= SEQUENCE (SIZE(1.. maxnoTRPTEGs)) OF TRPTEG-Item

TRPTEG-Item ::= SEQUENCE {

tRP-Tx-TEGInformation		TRP-Tx-TEGInformation,

dl-PRSResourceSetID			PRS-Resource-Set-ID,

dl-PRSResourceID-List	SEQUENCE (SIZE(1.. maxnoofPRS-ResourcesPerSet)) OF DLPRSResourceID-Item OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { TRPTEGItem-ExtIEs } } OPTIONAL,

...

}

TRPTEGItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLPRSResourceID-Item ::= SEQUENCE {

dl-PRSResourceID		PRS-Resource-ID,

iE-Extensions			ProtocolExtensionContainer { { DLPRSResource-Item-ExtIEs} }	OPTIONAL,

...

}

DLPRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Transport-Layer-Address-Info ::= SEQUENCE {

transport-UP-Layer-Address-Info-To-Add-List		Transport-UP-Layer-Address-Info-To-Add-List							OPTIONAL,

transport-UP-Layer-Address-Info-To-Remove-List	Transport-UP-Layer-Address-Info-To-Remove-List					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs } }								OPTIONAL

}

Transport-Layer-Address-Info-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPType ::= ENUMERATED {

prsOnlyTP,

srsOnlyRP,

tp,

rp,

trp,

...,

mobile-trp

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime													OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SurvivalTime	CRITICALITY ignore	EXTENSION SurvivalTime	PRESENCE optional }|

{ ID id-RANfeedbacktype	CRITICALITY ignore	EXTENSION RANfeedbacktype		PRESENCE optional}|

{ ID id-N6JitterInformation	CRITICALITY ignore	EXTENSION N6JitterInformation	PRESENCE optional },

...

}

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDL		TSCAssistanceInformation								OPTIONAL,

tSCAssistanceInformationUL		TSCAssistanceInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} }	OPTIONAL,

...

}

TSCTrafficCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCTrafficCharacteristicsFeedback ::= SEQUENCE {

tSCFeedbackInformationDL		TSCFeedbackInformation								OPTIONAL,

tSCFeedbackInformationUL		TSCFeedbackInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TSCTrafficCharacteristicsFeedback-ExtIEs} }	OPTIONAL,

...

}

TSCTrafficCharacteristicsFeedback-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCFeedbackInformation ::= SEQUENCE {

burstArrivalTimeOffset							INTEGER (-640000..640000, ...),

adjustedPeriodicity								Periodicity							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TSCFeedbackInformation-ExtIEs} }	OPTIONAL,

...

}

TSCFeedbackInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-MeasurementUpdateList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementUpdateItem

TRP-MeasurementUpdateItem ::= SEQUENCE {

tRP-ID						TRPID,

aoA-window-information		AoA-AssistanceInfo	OPTIONAL,

iE-extensions				ProtocolExtensionContainer { { TRP-MeasurementUpdateItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementUpdateItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NumberOfTRPRxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTEG		PRESENCE optional }|

{ ID id-NumberOfTRPRxTxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTxTEG		PRESENCE optional },

...

}

TwoPHRModeMCG ::= ENUMERATED {enabled, ...}

TwoPHRModeSCG ::= ENUMERATED {enabled, ...}

TxHoppingConfiguration ::= SEQUENCE {

overlapValue						ENUMERATED {rb0, rb1, rb2, rb4},

numberOfHops						INTEGER (1..6),

slotOffsetForRemainingHopsList		SlotOffsetForRemainingHopsList,

iE-extensions		ProtocolExtensionContainer { { TxHoppingConfiguration-ExtIEs } }	OPTIONAL,

...

}

TxHoppingConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- U

UAC-Assistance-Info ::= SEQUENCE {

uACPLMN-List		UACPLMN-List,

iE-Extensions		ProtocolExtensionContainer { { UAC-Assistance-InfoExtIEs} } OPTIONAL

}

UAC-Assistance-InfoExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UACPLMN-List ::= SEQUENCE (SIZE(1..maxnoofUACPLMNs)) OF UACPLMN-Item

UACPLMN-Item::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

uACType-List				UACType-List,	iE-Extensions		ProtocolExtensionContainer { { UACPLMN-Item-ExtIEs} } OPTIONAL

}

UACPLMN-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NID	CRITICALITY ignore	EXTENSION NID	PRESENCE optional },

...

}

UACType-List ::= SEQUENCE (SIZE(1..maxnoofUACperPLMN)) OF UACType-Item

UACType-Item::= SEQUENCE {

uACReductionIndication 		UACReductionIndication,

uACCategoryType				UACCategoryType,

iE-Extensions		ProtocolExtensionContainer { { UACType-Item-ExtIEs } } OPTIONAL

}

UACType-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UACCategoryType ::= CHOICE {

uACstandardized				UACAction,

uACOperatorDefined			UACOperatorDefined,

choice-extension			ProtocolIE-SingleContainer { { UACCategoryType-ExtIEs } }

}

UACCategoryType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UACOperatorDefined ::= SEQUENCE {

accessCategory					INTEGER (32..63,...),

accessIdentity					BIT STRING (SIZE(7)),

iE-Extensions		ProtocolExtensionContainer { { UACOperatorDefined-ExtIEs} } OPTIONAL

}

UACOperatorDefined-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UACAction ::= ENUMERATED {

reject-non-emergency-mo-dt,

reject-rrc-cr-signalling,

permit-emergency-sessions-and-mobile-terminated-services-only,

permit-high-priority-sessions-and-mobile-terminated-services-only,

...

}

UACReductionIndication ::= INTEGER (0..100)

UE-associatedLogicalF1-ConnectionItem ::= SEQUENCE {

gNB-CU-UE-F1AP-ID		GNB-CU-UE-F1AP-ID	 OPTIONAL,

gNB-DU-UE-F1AP-ID		GNB-DU-UE-F1AP-ID	 OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UE-associatedLogicalF1-ConnectionItemExtIEs} } OPTIONAL,

...

}

UEAssistanceInformation ::= OCTET STRING

UEAssistanceInformationEUTRA ::= OCTET STRING

UE-associatedLogicalF1-ConnectionItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-CapabilityRAT-ContainerList::= OCTET STRING

UEContextNotRetrievable ::= ENUMERATED {true, ...}

UEIdentityIndexValue ::= CHOICE {

indexLength10			BIT STRING (SIZE (10)),

choice-extension		ProtocolIE-SingleContainer { {UEIdentityIndexValueChoice-ExtIEs} }

}

UEIdentityIndexValueChoice-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UEIdentity-List-For-Paging-Item		::= SEQUENCE {

uEIdentityIndexValue						UEIdentityIndexValue,

pagingDRX									PagingDRX		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UEIdentity-List-For-Paging-Item-ExtIEs} } OPTIONAL

}

UEIdentity-List-For-Paging-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-ConfirmedToBeModified-Item::= SEQUENCE {

mRB-ID						MRB-ID,

mrb-type-reconfiguration	MBSPTPRetransmissionTunnelRequired			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-MulticastMRBs-ConfirmedToBeModified-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ConfirmedToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-RequiredToBeModified-Item::= SEQUENCE {

mRB-ID							MRB-ID,

mrb-type-reconfiguration		ENUMERATED {true, ...}							OPTIONAL,

mrb-reconfigured-RLCtype		ENUMERATED {

rlc-um-ptp,

rlc-am-ptp,

rlc-um-dl-ptm,

two-rlc-um-dl-ptp-and-dl-ptm,

three-rlc-um-dl-ptp-ul-ptp-dl-ptm,

two-rlc-am-ptp-um-dl-ptm,

...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UE-MulticastMRBs-RequiredToBeModified-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-RequiredToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MulticastF1UContextReferenceCU				CRITICALITY reject	EXTENSION MulticastF1UContextReferenceCU					PRESENCE optional},

...

}

UE-MulticastMRBs-RequiredToBeReleased-Item::= SEQUENCE {

mRB-ID					MRB-ID,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-RequiredToBeReleased-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-RequiredToBeReleased-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-Setup-Item	::= SEQUENCE {

mRB-ID							MRB-ID,

multicastF1UContextReferenceCU	MulticastF1UContextReferenceCU,

iE-Extensions					ProtocolExtensionContainer { { UE-MulticastMRBs-Setup-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-Setupnew-Item	::= SEQUENCE {

mRB-ID							MRB-ID,

multicastF1UContextReferenceCU	MulticastF1UContextReferenceCU,

iE-Extensions					ProtocolExtensionContainer { { UE-MulticastMRBs-Setupnew-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-Setupnew-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-ToBeReleased-Item	::= SEQUENCE {

mRB-ID					MRB-ID,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-ToBeReleased-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ToBeReleased-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-ToBeSetup-Item	::= SEQUENCE {

mRB-ID									MRB-ID,

mbsPTPRetransmissionTunnelRequired		MBSPTPRetransmissionTunnelRequired			OPTIONAL,

mbsPTPForwardingRequiredInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-ToBeSetup-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Source-MRB-ID		CRITICALITY ignore EXTENSION MRB-ID		PRESENCE optional },

...

}

UE-MulticastMRBs-ToBeSetup-atModify-Item	::= SEQUENCE {

mRB-ID									MRB-ID,

mbsPTPRetransmissionTunnelRequired		MBSPTPRetransmissionTunnelRequired			OPTIONAL,

mbsPTPForwardingRequiredInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-ToBeSetup-atModify-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ToBeSetup-atModify-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UEPagingCapability ::= SEQUENCE {

iNACTIVEStatePODetermination						ENUMERATED {supported, ...} 	OPTIONAL,

iE-Extension				ProtocolExtensionContainer { { UEPagingCapability-ExtIEs} } 		OPTIONAL,

...

}

UEPagingCapability-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RedCapIndication		CRITICALITY ignore 	EXTENSION RedCapIndication		PRESENCE optional },

...

}

UEReportingInformation::= SEQUENCE {

reportingAmount				ENUMERATED {ma0, ma1, ma2, ma4, ma8, ma16, ma32, ma64},

reportingInterval			ENUMERATED {none, one, two, four, eight, ten, sixteen, twenty, thirty-two, sixty-four, ...},

iE-extensions			ProtocolExtensionContainer { { UEReportingInformation-ExtIEs } }	OPTIONAL,

...

}

UEReportingInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UlTxDirectCurrentMoreCarrierInformation::= OCTET STRING

UL-AoA ::= SEQUENCE {

azimuthAoA					INTEGER (0..3599),

zenithAoA					INTEGER (0..1799)		OPTIONAL,

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-AoA-ExtIEs } }	OPTIONAL,

...

}

UL-AoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-BH-Non-UP-Traffic-Mapping ::= SEQUENCE {

uL-BH-Non-UP-Traffic-Mapping-List			UL-BH-Non-UP-Traffic-Mapping-List,

iE-Extensions	ProtocolExtensionContainer { { UL-BH-Non-UP-Traffic-Mapping-ExtIEs } } OPTIONAL

}

UL-BH-Non-UP-Traffic-Mapping-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-BH-Non-UP-Traffic-Mapping-List ::= SEQUENCE (SIZE(1..maxnoofNonUPTrafficMappings)) OF UL-BH-Non-UP-Traffic-Mapping-Item

UL-BH-Non-UP-Traffic-Mapping-Item ::= SEQUENCE {

nonUPTrafficType				NonUPTrafficType,

bHInfo						BHInfo,

iE-Extensions					ProtocolExtensionContainer { { UL-BH-Non-UP-Traffic-Mapping-ItemExtIEs } }	OPTIONAL

}

UL-BH-Non-UP-Traffic-Mapping-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ULConfiguration ::= SEQUENCE	{

uLUEConfiguration		ULUEConfiguration,

iE-Extensions	ProtocolExtensionContainer { { ULConfigurationExtIEs } }	OPTIONAL,

...

}

ULConfigurationExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-GapFR2-Config ::= OCTET STRING

UL-RTOA-Measurement ::= SEQUENCE {

uL-RTOA-MeasurementItem		UL-RTOA-MeasurementItem,

additionalPath-List			AdditionalPath-List OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UL-RTOA-Measurement-ExtIEs } }	OPTIONAL

}

UL-RTOA-Measurement-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList	PRESENCE optional}|

{ ID id-TRPRx-TEGInformation		CRITICALITY ignore EXTENSION TRP-Rx-TEGInformation		PRESENCE optional},

...

}

UL-RTOA-MeasurementItem ::= CHOICE {

k0					INTEGER (0..1970049),

k1					INTEGER (0..985025),

k2					INTEGER (0..492513),

k3					INTEGER (0..246257),

k4					INTEGER (0..123129),

k5					INTEGER (0..61565),

choice-extension			ProtocolIE-SingleContainer { { UL-RTOA-MeasurementItem-ExtIEs } }

}

UL-RTOA-MeasurementItem-ExtIEs F1AP-PROTOCOL-IES ::= {

{ID id-ReportingGranularitykminus1 	CRITICALITY ignore TYPE ReportingGranularitykminus1 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus2 	CRITICALITY ignore TYPE ReportingGranularitykminus2 PRESENCE mandatory },

...

}

UL-SRS-RSRP ::= INTEGER (0..126)

UL-SRS-RSRPP ::= SEQUENCE {

firstPathRSRPP				INTEGER (0..126),

iE-extensions			ProtocolExtensionContainer { { UL-SRS-RSRPP-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-RSRPP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-RSCP	::= SEQUENCE {

uLRSCP					INTEGER (0..3599),

iE-extensions			ProtocolExtensionContainer { { UL-RSCP-ExtIEs } }	OPTIONAL,

...

}

UL-RSCP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ULUEConfiguration ::= ENUMERATED {no-data, shared, only, ...}

UL-UP-TNL-Information-to-Update-List-Item	::= SEQUENCE {

uLUPTNLInformation		UPTransportLayerInformation,

newULUPTNLInformation	UPTransportLayerInformation		OPTIONAL,

bHInfo	BHInfo,

iE-Extensions	ProtocolExtensionContainer { { UL-UP-TNL-Information-to-Update-List-ItemExtIEs } }	OPTIONAL,

...

}

UL-UP-TNL-Information-to-Update-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-UP-TNL-Address-to-Update-List-Item	::= SEQUENCE {

oldIPAdress						TransportLayerAddress,

newIPAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { UL-UP-TNL-Address-to-Update-List-ItemExtIEs } }	OPTIONAL,

...

}

UL-UP-TNL-Address-to-Update-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

ULUPTNLInformation-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofULUPTNLInformation)) OF ULUPTNLInformation-ToBeSetup-Item

ULUPTNLInformation-ToBeSetup-Item ::=SEQUENCE {

uLUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { ULUPTNLInformation-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

ULUPTNLInformation-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BHInfo			CRITICALITY ignore	EXTENSION BHInfo			PRESENCE optional	}|

{ ID id-DRBMappingInfo	CRITICALITY ignore	EXTENSION UuRLCChannelID	PRESENCE optional	},

...

}

Uncertainty ::= INTEGER (0..32767, ...)

UplinkChannelBW-PerSCS-List ::= SEQUENCE (SIZE (1..maxnoSCSs)) OF SCS-SpecificCarrier

UplinkTxDirectCurrentListInformation ::= OCTET STRING

UplinkTxDirectCurrentTwoCarrierListInfo ::= OCTET STRING

ULTCIStateID  ::= OCTET STRING

UPTransportLayerInformation		::= CHOICE {

gTPTunnel		GTPTunnel,

choice-extension			ProtocolIE-SingleContainer { { UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

URI-address ::= VisibleString

Uncertainty-range-AoA ::= INTEGER (0..3599)

Uncertainty-range-ZoA ::= INTEGER (0..1799)

UuRLCChannelID ::= INTEGER (1..32)

UuRLCChannelQoSInformation ::= CHOICE {

uuRLCChannelQoS					QoSFlowLevelQoSParameters,

uuControlPlaneTrafficType		ENUMERATED {srb0,srb1,srb2,...},

choice-extension		ProtocolIE-SingleContainer { { UuRLCChannelQoSInformation-ExtIEs} }

}

UuRLCChannelQoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UuRLCChannelToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelToBeSetupItem

UuRLCChannelToBeSetupItem ::= SEQUENCE {

uuRLCChannelID					UuRLCChannelID,

uuRLCChannelQoSInformation		UuRLCChannelQoSInformation,

rLCMode							RLCMode,

iE-Extensions					ProtocolExtensionContainer { { UuRLCChannelToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelToBeModifiedItem

UuRLCChannelToBeModifiedItem ::= SEQUENCE {

uuRLCChannelID					UuRLCChannelID,

uuRLCChannelQoSInformation		UuRLCChannelQoSInformation			OPTIONAL,

rLCMode							RLCMode			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UuRLCChannelToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelToBeReleasedItem

UuRLCChannelToBeReleasedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelSetupList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelSetupItem

UuRLCChannelSetupItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelSetupItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelFailedToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelFailedToBeSetupItem

UuRLCChannelFailedToBeSetupItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

cause					Cause	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelFailedToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelFailedToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelModifiedItem

UuRLCChannelModifiedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelFailedToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelFailedToBeModifiedItem

UuRLCChannelFailedToBeModifiedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

cause					Cause	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelFailedToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelFailedToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelRequiredToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelRequiredToBeModifiedItem

UuRLCChannelRequiredToBeModifiedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelRequiredToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelRequiredToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelRequiredToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelRequiredToBeReleasedItem

UuRLCChannelRequiredToBeReleasedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelRequiredToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelRequiredToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- V

VictimgNBSetID ::= SEQUENCE {

victimgNBSetID		GNBSetID,

iE-Extensions		ProtocolExtensionContainer { { VictimgNBSetID-ExtIEs } }		OPTIONAL

}

VictimgNBSetID-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

VehicleUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

#### -- V

ValidityAreaSpecificSRSInformation ::= SEQUENCE {

transmissionCombPos				TransmissionCombPos 	OPTIONAL,

resourceMapping 				ResourceMapping			OPTIONAL,

freqDomainShift					INTEGER (0..268)		OPTIONAL,

c-SRS							INTEGER (0..63)			OPTIONAL,

resourceTypePos					ResourceTypePos			OPTIONAL,

sequenceIDPos					INTEGER (0..65535)		OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { ValidityAreaSpecificSRSInformation-ExtIEs } }	OPTIONAL,

...

}

ValidityAreaSpecificSRSInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- W

#### -- X

#### -- Y

#### -- Z

ZoAInformation ::= SEQUENCE {

zenithAoA					INTEGER (0..1799),

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { ZoAInformation-ExtIEs } }	OPTIONAL,

...

}

ZoAInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

END

-- ASN1STOP

### 9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

F1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0..65535),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..65535)

ProtocolIE-ID		::= INTEGER (0..65535)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome }

END

-- ASN1STOP

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel								ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

id-GNBDUStatusIndication 					ProcedureCode ::= 24

id-RRCDeliveryReport		 				ProcedureCode ::= 25

id-F1Removal								ProcedureCode ::= 26

id-NetworkAccessRateReduction				ProcedureCode ::= 27

id-TraceStart								ProcedureCode ::= 28

id-DeactivateTrace							ProcedureCode ::= 29

id-DUCURadioInformationTransfer				ProcedureCode ::= 30

id-CUDURadioInformationTransfer				ProcedureCode ::= 31

id-BAPMappingConfiguration					ProcedureCode ::= 32

id-GNBDUResourceConfiguration				ProcedureCode ::= 33

id-IABTNLAddressAllocation					ProcedureCode ::= 34

id-IABUPConfigurationUpdate					ProcedureCode ::= 35

id-resourceStatusReportingInitiation		ProcedureCode ::= 36

id-resourceStatusReporting					ProcedureCode ::= 37

id-accessAndMobilityIndication				ProcedureCode ::= 38

id-accessSuccess							ProcedureCode ::= 39

id-cellTrafficTrace 						ProcedureCode ::= 40

id-PositioningMeasurementExchange			ProcedureCode ::= 41

id-PositioningAssistanceInformationControl	ProcedureCode ::= 42

id-PositioningAssistanceInformationFeedback	ProcedureCode ::= 43

id-PositioningMeasurementReport				ProcedureCode ::= 44

id-PositioningMeasurementAbort				ProcedureCode ::= 45

id-PositioningMeasurementFailureIndication	ProcedureCode ::= 46

id-PositioningMeasurementUpdate				ProcedureCode ::= 47

id-TRPInformationExchange					ProcedureCode ::= 48

id-PositioningInformationExchange			ProcedureCode ::= 49

id-PositioningActivation					ProcedureCode ::= 50

id-PositioningDeactivation					ProcedureCode ::= 51

id-E-CIDMeasurementInitiation				ProcedureCode ::= 52

id-E-CIDMeasurementFailureIndication		ProcedureCode ::= 53

id-E-CIDMeasurementReport					ProcedureCode ::= 54

id-E-CIDMeasurementTermination				ProcedureCode ::= 55

id-PositioningInformationUpdate				ProcedureCode ::= 56

id-ReferenceTimeInformationReport			ProcedureCode ::= 57

id-ReferenceTimeInformationReportingControl	ProcedureCode ::= 58

id-BroadcastContextSetup					ProcedureCode ::= 59

id-BroadcastContextRelease					ProcedureCode ::= 60

id-BroadcastContextReleaseRequest			ProcedureCode ::= 61

id-BroadcastContextModification				ProcedureCode ::= 62

id-MulticastGroupPaging						ProcedureCode ::= 63

id-MulticastContextSetup					ProcedureCode ::= 64

id-MulticastContextRelease					ProcedureCode ::= 65

id-MulticastContextReleaseRequest			ProcedureCode ::= 66

id-MulticastContextModification				ProcedureCode ::= 67

id-MulticastDistributionSetup				ProcedureCode ::= 68

id-MulticastDistributionRelease				ProcedureCode ::= 69

id-PDCMeasurementInitiation					ProcedureCode ::= 70

id-PDCMeasurementReport						ProcedureCode ::= 71

id-PDCMeasurementInitiationRequest			ProcedureCode ::= 72

id-PDCMeasurementInitiationResponse			ProcedureCode ::= 73

id-PDCMeasurementInitiationFailure			ProcedureCode ::= 74

id-pRSConfigurationExchange					ProcedureCode ::= 75

id-measurementPreconfiguration				ProcedureCode ::= 76

id-measurementActivation					ProcedureCode ::= 77

id-QoEInformationTransfer					ProcedureCode ::= 78

id-PDCMeasurementTerminationCommand			ProcedureCode ::= 79

id-PDCMeasurementFailureIndication 			ProcedureCode ::= 80

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

id-DUCUCellSwitchNotification				ProcedureCode ::= 82

id-CUDUCellSwitchNotification				ProcedureCode ::= 83

id-DUCUTAInformationTransfer				ProcedureCode ::= 84

id-CUDUTAInformationTransfer				ProcedureCode ::= 85

id-QoEInformationTransferControl			ProcedureCode ::= 86

id-RachIndication							ProcedureCode ::= 87

id-TimingSynchronisationStatus				ProcedureCode ::= 88

id-TimingSynchronisationStatusReport		ProcedureCode ::= 89

id-MIABF1SetupTriggering					ProcedureCode ::= 90

id-MIABF1SetupOutcomeNotification			ProcedureCode ::= 91

id-MulticastContextNotification 			ProcedureCode ::= 92

id-MulticastCommonConfiguration				ProcedureCode ::= 93

id-BroadcastTransportResourceRequest		ProcedureCode ::= 94

id-DUCUAccessAndMobilityIndication			ProcedureCode ::= 95

id-SRSInformationReservationNotification	ProcedureCode ::= 96

id-DUCUDynamicResourceAllocation 			ProcedureCode ::= x1

id-CUDUDynamicResourceAllocation 			ProcedureCode ::= x2

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofSSBarea							INTEGER ::=64

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRAReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofSuccessfulHOReports				INTEGER ::= 64

maxnoofNR-UChannelIDs					INTEGER ::= 16

maxServedCellforSON						INTEGER ::= 256

maxNeighbourCellforSON					INTEGER ::= 32

maxAffectedCells						INTEGER ::= 32

maxnoofMRBs								INTEGER ::= 32

maxnoofMBSQoSFlows						INTEGER ::= 64

maxnoofMBSFSAs 							INTEGER ::= 256

maxnoofUEIDforPaging 					INTEGER ::= 4096

maxnoofCellsforMBS						INTEGER ::= 512

maxnoofTAIforMBS						INTEGER ::= 512

maxnoofMBSAreaSessionIDs				INTEGER ::= 256

maxnoofMBSServiceAreaInformation		INTEGER ::= 256

maxnoofIABCongInd						INTEGER ::= 1024

maxnoofNeighbourNodeCellsIAB			INTEGER ::= 1024

maxnoofRBsetsPerCell					INTEGER ::= 8

maxnoofRBsetsPerCell-1					INTEGER ::= 7

maxnoofMeasPDC							INTEGER ::= 16

maxnoARPs								INTEGER ::=	16

maxnoofULAoAs							INTEGER ::= 8

maxNoPathExtended						INTEGER ::= 8

maxnoTRPTEGs							INTEGER ::= 8

maxFreqLayers							INTEGER ::= 4

maxNumResourcesPerAngle					INTEGER ::= 24

maxnoAzimuthAngles						INTEGER ::= 3600

maxnoElevationAngles					INTEGER ::= 1801

maxnoofPRSTRPs							INTEGER ::= 256

maxnoofQoEInformation					INTEGER ::= 16

maxnoofUuRLCChannels					INTEGER ::= 32

maxnoofPC5RLCChannels					INTEGER ::= 512

maxnoofSMBRValues						INTEGER ::= 8

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

maxnoofUETypes							INTEGER ::= 8

maxnoofLTMCells							INTEGER	::= 8

maxnoofJointorDLTCIStates				INTEGER	::= 128

maxnoofULTCIStates						INTEGER	::= 64

maxnoofTAList							INTEGER	::= 8

maxnoofLTMgNB-DUs						INTEGER	::= 8

maxnoofUEsInQMCTransferControlMessage	INTEGER ::= 512

maxnoofUEsforRAReportIndications		INTEGER ::= 64

maxnoofSuccessfulPSCellChangeReports	INTEGER ::= 64

maxnoofPeriodicities					INTEGER ::= 8

maxnoofThresholdMBS-1					INTEGER ::= 7

maxMBSSessionsinSessionInfoList			INTEGER ::= 1024

maxnoofLBTFailureInformation			INTEGER ::= 64

maxnoofRSPPQoSFlows						INTEGER ::= 2048

maxnoVACell								INTEGER ::= 32

maxnoAggregatedSRS-Resources			INTEGER ::= 3

maxnoAggregatedPosSRSResourceSets		INTEGER ::= 48

maxnoAggregatedPosPRSResourceSets		INTEGER ::= 3

maxnoofTimeWindowSRS					INTEGER ::= 16

maxnoofTimeWindowMea					INTEGER ::= 16

maxnoPreconfiguredSRS					INTEGER ::= 16

maxnoHopsMinusOne						INTEGER ::= 5

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container	ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-NotficationControl								ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-MeasGapSharingConfig								ProtocolIE-ID ::= 238

id-systemInformationAreaID							ProtocolIE-ID ::= 239

id-areaScope										ProtocolIE-ID ::= 240

id-RRCContainer-RRCSetupComplete					ProtocolIE-ID ::= 241

id-TraceActivation									ProtocolIE-ID ::= 242

id-TraceID											ProtocolIE-ID ::= 243

id-Neighbour-Cell-Information-List					ProtocolIE-ID ::= 244

id-SymbolAllocInSlot								ProtocolIE-ID ::= 246

id-NumDLULSymbols									ProtocolIE-ID ::= 247

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 248

id-DUCURadioInformationType							ProtocolIE-ID ::= 249

id-CUDURadioInformationType 						ProtocolIE-ID ::= 250

id-AggressorgNBSetID								ProtocolIE-ID ::= 251

id-VictimgNBSetID									ProtocolIE-ID ::= 252

id-LowerLayerPresenceStatusChange					ProtocolIE-ID ::= 253

id-Transport-Layer-Address-Info						ProtocolIE-ID ::= 254

id-Neighbour-Cell-Information-Item					ProtocolIE-ID ::= 255

id-IntendedTDD-DL-ULConfig							ProtocolIE-ID ::= 256

id-QosMonitoringRequest								ProtocolIE-ID ::= 257

id-BHChannels-ToBeSetup-List						ProtocolIE-ID ::= 258

id-BHChannels-ToBeSetup-Item						ProtocolIE-ID ::= 259

id-BHChannels-Setup-List							ProtocolIE-ID ::= 260

id-BHChannels-Setup-Item							ProtocolIE-ID ::= 261

id-BHChannels-ToBeModified-Item						ProtocolIE-ID ::= 262

id-BHChannels-ToBeModified-List						ProtocolIE-ID ::= 263

id-BHChannels-ToBeReleased-Item						ProtocolIE-ID ::= 264

id-BHChannels-ToBeReleased-List						ProtocolIE-ID ::= 265

id-BHChannels-ToBeSetupMod-Item						ProtocolIE-ID ::= 266

id-BHChannels-ToBeSetupMod-List						ProtocolIE-ID ::= 267

id-BHChannels-FailedToBeModified-Item				ProtocolIE-ID ::= 268

id-BHChannels-FailedToBeModified-List				ProtocolIE-ID ::= 269

id-BHChannels-FailedToBeSetupMod-Item				ProtocolIE-ID ::= 270

id-BHChannels-FailedToBeSetupMod-List				ProtocolIE-ID ::= 271

id-BHChannels-Modified-Item							ProtocolIE-ID ::= 272

id-BHChannels-Modified-List							ProtocolIE-ID ::= 273

id-BHChannels-SetupMod-Item							ProtocolIE-ID ::= 274

id-BHChannels-SetupMod-List							ProtocolIE-ID ::= 275

id-BHChannels-Required-ToBeReleased-Item			ProtocolIE-ID ::= 276

id-BHChannels-Required-ToBeReleased-List			ProtocolIE-ID ::= 277

id-BHChannels-FailedToBeSetup-Item					ProtocolIE-ID ::= 278

id-BHChannels-FailedToBeSetup-List					ProtocolIE-ID ::= 279

id-BHInfo											ProtocolIE-ID ::= 280

id-BAPAddress										ProtocolIE-ID ::= 281

id-ConfiguredBAPAddress								ProtocolIE-ID ::= 282

id-BH-Routing-Information-Added-List				ProtocolIE-ID ::= 283

id-BH-Routing-Information-Added-List-Item			ProtocolIE-ID ::= 284

id-BH-Routing-Information-Removed-List				ProtocolIE-ID ::= 285

id-BH-Routing-Information-Removed-List-Item			ProtocolIE-ID ::= 286

id-UL-BH-Non-UP-Traffic-Mapping						ProtocolIE-ID ::= 287

id-Activated-Cells-to-be-Updated-List				ProtocolIE-ID ::= 288

id-Child-Nodes-List									ProtocolIE-ID ::= 289

id-IAB-Info-IAB-DU									ProtocolIE-ID ::= 290

id-IAB-Info-IAB-donor-CU							ProtocolIE-ID ::= 291

id-IAB-TNL-Addresses-To-Remove-List					ProtocolIE-ID ::= 292

id-IAB-TNL-Addresses-To-Remove-Item					ProtocolIE-ID ::= 293

id-IAB-Allocated-TNL-Address-List					ProtocolIE-ID ::= 294

id-IAB-Allocated-TNL-Address-Item					ProtocolIE-ID ::= 295

id-IABIPv6RequestType								ProtocolIE-ID ::= 296

id-IABv4AddressesRequested							ProtocolIE-ID ::= 297

id-IAB-Barred										ProtocolIE-ID ::= 298

id-TrafficMappingInformation						ProtocolIE-ID ::= 299

id-UL-UP-TNL-Information-to-Update-List				ProtocolIE-ID ::= 300

id-UL-UP-TNL-Information-to-Update-List-Item		ProtocolIE-ID ::= 301

id-UL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 302

id-UL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 303

id-DL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 304

id-DL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 305

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= 306

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= 307

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 308

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 309

id-SIB12-message									ProtocolIE-ID ::= 310

id-SIB13-message									ProtocolIE-ID ::= 311

id-SIB14-message									ProtocolIE-ID ::= 312

id-SLDRBs-FailedToBeModified-Item					ProtocolIE-ID ::= 313

id-SLDRBs-FailedToBeModified-List					ProtocolIE-ID ::= 314

id-SLDRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 315

id-SLDRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 316

id-SLDRBs-Modified-Item								ProtocolIE-ID ::= 317

id-SLDRBs-Modified-List								ProtocolIE-ID ::= 318

id-SLDRBs-Required-ToBeModified-Item				ProtocolIE-ID ::= 319

id-SLDRBs-Required-ToBeModified-List				ProtocolIE-ID ::= 320

id-SLDRBs-Required-ToBeReleased-Item				ProtocolIE-ID ::= 321

id-SLDRBs-Required-ToBeReleased-List				ProtocolIE-ID ::= 322

id-SLDRBs-Setup-Item								ProtocolIE-ID ::= 323

id-SLDRBs-Setup-List								ProtocolIE-ID ::= 324

id-SLDRBs-ToBeModified-Item							ProtocolIE-ID ::= 325

id-SLDRBs-ToBeModified-List							ProtocolIE-ID ::= 326

id-SLDRBs-ToBeReleased-Item							ProtocolIE-ID ::= 327

id-SLDRBs-ToBeReleased-List							ProtocolIE-ID ::= 328

id-SLDRBs-ToBeSetup-Item							ProtocolIE-ID ::= 329

id-SLDRBs-ToBeSetup-List							ProtocolIE-ID ::= 330

id-SLDRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 331

id-SLDRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 332

id-SLDRBs-SetupMod-List								ProtocolIE-ID ::= 333

id-SLDRBs-FailedToBeSetupMod-List					ProtocolIE-ID ::= 334

id-SLDRBs-SetupMod-Item								ProtocolIE-ID ::= 335

id-SLDRBs-FailedToBeSetupMod-Item					ProtocolIE-ID ::= 336

id-SLDRBs-ModifiedConf-List							ProtocolIE-ID ::= 337

id-SLDRBs-ModifiedConf-Item							ProtocolIE-ID ::= 338

id-UEAssistanceInformationEUTRA						ProtocolIE-ID ::= 339

id-PC5LinkAMBR										ProtocolIE-ID ::= 340

id-SL-PHY-MAC-RLC-Config							ProtocolIE-ID ::= 341

id-SL-ConfigDedicatedEUTRA-Info						ProtocolIE-ID ::= 342

id-AlternativeQoSParaSetList						ProtocolIE-ID ::= 343

id-CurrentQoSParaSetIndex							ProtocolIE-ID ::= 344

id-gNBCUMeasurementID								ProtocolIE-ID ::= 345

id-gNBDUMeasurementID								ProtocolIE-ID ::= 346

id-RegistrationRequest								ProtocolIE-ID ::= 347

id-ReportCharacteristics							ProtocolIE-ID ::= 348

id-CellToReportList									ProtocolIE-ID ::= 349

id-CellMeasurementResultList						ProtocolIE-ID ::= 350

id-HardwareLoadIndicator							ProtocolIE-ID ::= 351

id-ReportingPeriodicity								ProtocolIE-ID ::= 352

id-TNLCapacityIndicator								ProtocolIE-ID ::= 353

id-CarrierList										ProtocolIE-ID ::= 354

id-ULCarrierList									ProtocolIE-ID ::= 355

id-FrequencyShift7p5khz								ProtocolIE-ID ::= 356

id-SSB-PositionsInBurst								ProtocolIE-ID ::= 357

id-NRPRACHConfig									ProtocolIE-ID ::= 358

id-RAReportList										ProtocolIE-ID ::= 359

id-RLFReportInformationList							ProtocolIE-ID ::= 360

id-TDD-UL-DLConfigCommonNR							ProtocolIE-ID ::= 361

id-CNPacketDelayBudgetDownlink						ProtocolIE-ID ::= 362

id-ExtendedPacketDelayBudget						ProtocolIE-ID ::= 363

id-TSCTrafficCharacteristics						ProtocolIE-ID ::= 364

id-ReportingRequestType								ProtocolIE-ID ::= 365

id-TimeReferenceInformation							ProtocolIE-ID ::= 366

id-CNPacketDelayBudgetUplink						ProtocolIE-ID ::= 369

id-AdditionalPDCPDuplicationTNL-List				ProtocolIE-ID ::= 370

id-RLCDuplicationInformation						ProtocolIE-ID ::= 371

id-AdditionalDuplicationIndication					ProtocolIE-ID ::= 372

id-ConditionalInterDUMobilityInformation			ProtocolIE-ID ::= 373

id-ConditionalIntraDUMobilityInformation			ProtocolIE-ID ::= 374

id-targetCellsToCancel								ProtocolIE-ID ::= 375

id-requestedTargetCellGlobalID						ProtocolIE-ID ::= 376

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 377

id-TraceCollectionEntityIPAddress 					ProtocolIE-ID ::= 378

id-PrivacyIndicator									ProtocolIE-ID ::= 379

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 380

id-mdtConfiguration									ProtocolIE-ID ::= 381

id-ServingNID										ProtocolIE-ID ::= 382

id-NPNBroadcastInformation							ProtocolIE-ID ::= 383

id-NPNSupportInfo									ProtocolIE-ID ::= 384

id-NID												ProtocolIE-ID ::= 385

id-AvailableSNPN-ID-List							ProtocolIE-ID ::= 386

id-SIB10-message									ProtocolIE-ID ::= 387

id-DLCarrierList									ProtocolIE-ID ::= 389

id-ExtendedTAISliceSupportList					ProtocolIE-ID ::= 390

id-RequestedSRSTransmissionCharacteristics			ProtocolIE-ID ::= 391

id-PosAssistance-Information						ProtocolIE-ID ::= 392

id-PosBroadcast										ProtocolIE-ID ::= 393

id-RoutingID										ProtocolIE-ID ::= 394

id-PosAssistanceInformationFailureList				ProtocolIE-ID ::= 395

id-PosMeasurementQuantities							ProtocolIE-ID ::= 396

id-PosMeasurementResultList							ProtocolIE-ID ::= 397

id-TRPInformationTypeListTRPReq						ProtocolIE-ID ::= 398

id-TRPInformationTypeItem							ProtocolIE-ID ::= 399

id-TRPInformationListTRPResp						ProtocolIE-ID ::= 400

id-TRPInformationItem								ProtocolIE-ID ::= 401

id-LMF-MeasurementID								ProtocolIE-ID ::= 402

id-SRSType											ProtocolIE-ID ::= 403

id-ActivationTime									ProtocolIE-ID ::= 404

id-AbortTransmission								ProtocolIE-ID ::= 405

id-PositioningBroadcastCells						ProtocolIE-ID ::= 406

id-SRSConfiguration									ProtocolIE-ID ::= 407

id-PosReportCharacteristics							ProtocolIE-ID ::= 408

id-PosMeasurementPeriodicity						ProtocolIE-ID ::= 409

id-TRPList											ProtocolIE-ID ::= 410

id-RAN-MeasurementID								ProtocolIE-ID ::= 411

id-LMF-UE-MeasurementID								ProtocolIE-ID ::= 412

id-RAN-UE-MeasurementID								ProtocolIE-ID ::= 413

id-E-CID-MeasurementQuantities						ProtocolIE-ID ::= 414

id-E-CID-MeasurementQuantities-Item					ProtocolIE-ID ::= 415

id-E-CID-MeasurementPeriodicity						ProtocolIE-ID ::= 416

id-E-CID-MeasurementResult							ProtocolIE-ID ::= 417

id-Cell-Portion-ID									ProtocolIE-ID ::= 418

id-SFNInitialisationTime							ProtocolIE-ID ::= 419

id-SystemFrameNumber								ProtocolIE-ID ::= 420

id-SlotNumber										ProtocolIE-ID ::= 421

id-TRP-MeasurementRequestList						ProtocolIE-ID ::= 422

id-MeasurementBeamInfoRequest						ProtocolIE-ID ::= 423

id-E-CID-ReportCharacteristics						ProtocolIE-ID ::= 424

id-ConfiguredTACIndication							ProtocolIE-ID ::= 425

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 426

id-Extended-GNB-DU-Name								ProtocolIE-ID ::= 427

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-PDCPTerminatingNodeDLTNLAddrInfo					ProtocolIE-ID ::= 436

id-ENBDLTNLAddress									ProtocolIE-ID ::= 437

id-PosMeasurementPeriodicityExtended				ProtocolIE-ID ::= 438

id-PRS-Resource-ID									ProtocolIE-ID ::= 439

id-LocationMeasurementInformation					ProtocolIE-ID ::= 440

id-SliceRadioResourceStatus							ProtocolIE-ID ::= 441

id-CompositeAvailableCapacity-SUL					ProtocolIE-ID ::= 442

id-SuccessfulHOReportInformationList				ProtocolIE-ID ::= 443

id-NR-U-Channel-List								ProtocolIE-ID ::= 444

id-NR-U												ProtocolIE-ID ::= 445

id-Coverage-Modification-Notification				ProtocolIE-ID ::= 446

id-CCO-Assistance-Information						ProtocolIE-ID ::= 447

id-Neighbor-node-CCO-Assistance-Information-List	ProtocolIE-ID ::= 448

id-CellsForSON-List									ProtocolIE-ID ::= 449

id-MIMOPRBusageInformation							ProtocolIE-ID ::= 450

id-gNB-CU-MBS-F1AP-ID								ProtocolIE-ID ::= 451

id-gNB-DU-MBS-F1AP-ID								ProtocolIE-ID ::= 452

id-MBS-Area-Session-ID								ProtocolIE-ID ::= 453

id-MBS-CUtoDURRCInformation							ProtocolIE-ID ::= 454

id-MBS-Session-ID									ProtocolIE-ID ::= 455

id-SNSSAI											ProtocolIE-ID ::= 456

id-MBS-Broadcast-NeighbourCellList					ProtocolIE-ID ::= 457

id-BroadcastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 458

id-BroadcastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 459

id-BroadcastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 460

id-BroadcastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 461

id-BroadcastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 462

id-BroadcastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 463

id-BroadcastMRBs-Modified-List						ProtocolIE-ID ::= 464

id-BroadcastMRBs-Modified-Item						ProtocolIE-ID ::= 465

id-BroadcastMRBs-Setup-List							ProtocolIE-ID ::= 466

id-BroadcastMRBs-Setup-Item							ProtocolIE-ID ::= 467

id-BroadcastMRBs-SetupMod-List						ProtocolIE-ID ::= 468

id-BroadcastMRBs-SetupMod-Item						ProtocolIE-ID ::= 469

id-BroadcastMRBs-ToBeModified-List					ProtocolIE-ID ::= 470

id-BroadcastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 471

id-BroadcastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 472

id-BroadcastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 473

id-BroadcastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 474

id-BroadcastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 475

id-BroadcastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 476

id-BroadcastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 477

id-Supported-MBS-FSA-ID-List						ProtocolIE-ID ::= 478

id-UEIdentity-List-For-Paging-List 					ProtocolIE-ID ::= 479

id-UEIdentity-List-For-Paging-Item 					ProtocolIE-ID ::= 480

id-MBS-ServiceArea									ProtocolIE-ID ::= 481

id-MulticastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 482

id-MulticastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 483

id-MulticastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 484

id-MulticastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 485

id-MulticastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 486

id-MulticastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 487

id-MulticastMRBs-Modified-List						ProtocolIE-ID ::= 488

id-MulticastMRBs-Modified-Item						ProtocolIE-ID ::= 489

id-MulticastMRBs-Setup-List							ProtocolIE-ID ::= 490

id-MulticastMRBs-Setup-Item							ProtocolIE-ID ::= 491

id-MulticastMRBs-SetupMod-List						ProtocolIE-ID ::= 492

id-MulticastMRBs-SetupMod-Item						ProtocolIE-ID ::= 493

id-MulticastMRBs-ToBeModified-List					ProtocolIE-ID ::= 494

id-MulticastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 495

id-MulticastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 496

id-MulticastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 497

id-MulticastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 498

id-MulticastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 499

id-MulticastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 500

id-MulticastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 501

id-MBSMulticastF1UContextDescriptor					ProtocolIE-ID ::= 502

id-MulticastF1UContext-ToBeSetup-List				ProtocolIE-ID ::= 503

id-MulticastF1UContext-ToBeSetup-Item				ProtocolIE-ID ::= 504

id-MulticastF1UContext-Setup-List					ProtocolIE-ID ::= 505

id-MulticastF1UContext-Setup-Item					ProtocolIE-ID ::= 506

id-MulticastF1UContext-FailedToBeSetup-List			ProtocolIE-ID ::= 507

id-MulticastF1UContext-FailedToBeSetup-Item			ProtocolIE-ID ::= 508

id-IABCongestionIndication							ProtocolIE-ID ::= 509

id-IABConditionalRRCMessageDeliveryIndication		ProtocolIE-ID ::= 510

id-F1CTransferPathNRDC								ProtocolIE-ID ::= 511

id-BufferSizeThresh 								ProtocolIE-ID ::= 512

id-IAB-TNL-Addresses-Exception						ProtocolIE-ID ::= 513

id-BAP-Header-Rewriting-Added-List					ProtocolIE-ID ::= 514

id-BAP-Header-Rewriting-Added-List-Item				ProtocolIE-ID ::= 515

id-Re-routingEnableIndicator						ProtocolIE-ID ::= 516

id-NonF1terminatingTopologyIndicator				ProtocolIE-ID ::= 517

id-EgressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 518

id-IngressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 519

id-rBSetConfiguration 								ProtocolIE-ID ::= 520

id-frequency-Domain-HSNA-Configuration-List			ProtocolIE-ID ::= 521

id-child-IAB-Nodes-NA-Resource-List					ProtocolIE-ID ::= 522

id-Parent-IAB-Nodes-NA-Resource-Configuration-List	ProtocolIE-ID ::= 523

id-uL-FreqInfo										ProtocolIE-ID ::= 524

id-uL-Transmission-Bandwidth						ProtocolIE-ID ::= 525

id-dL-FreqInfo										ProtocolIE-ID ::= 526

id-dL-Transmission-Bandwidth						ProtocolIE-ID ::= 527

id-uL-NR-Carrier-List								ProtocolIE-ID ::= 528

id-dL-NR-Carrier-List								ProtocolIE-ID ::= 529

id-nRFreqInfo										ProtocolIE-ID ::= 530

id-transmission-Bandwidth							ProtocolIE-ID ::= 531

id-nR-Carrier-List									ProtocolIE-ID ::= 532

id-Neighbour-Node-Cells-List						ProtocolIE-ID ::= 533

id-Serving-Cells-List								ProtocolIE-ID ::= 534

id-permutation										ProtocolIE-ID ::= 535

id-MDTPollutedMeasurementIndicator					ProtocolIE-ID ::= 536

id-M5ReportAmount 									ProtocolIE-ID ::= 537

id-M6ReportAmount 									ProtocolIE-ID ::= 538

id-M7ReportAmount 									ProtocolIE-ID ::= 539

id-SurvivalTime										ProtocolIE-ID ::= 540

id-PDCMeasurementPeriodicity						ProtocolIE-ID ::= 541

id-PDCMeasurementQuantities							ProtocolIE-ID ::= 542

id-PDCMeasurementQuantities-Item					ProtocolIE-ID ::= 543

id-PDCMeasurementResult								ProtocolIE-ID ::= 544

id-PDCReportType									ProtocolIE-ID ::= 545

id-RAN-UE-PDC-MeasID								ProtocolIE-ID ::= 546

id-SCGActivationRequest								ProtocolIE-ID ::= 547

id-SCGActivationStatus								ProtocolIE-ID ::= 548

id-PRSTRPList										ProtocolIE-ID ::= 549

id-PRSTransmissionTRPList							ProtocolIE-ID ::= 550

id-OnDemandPRS										ProtocolIE-ID ::= 551

id-AoA-SearchWindow									ProtocolIE-ID ::= 552

id-TRP-MeasurementUpdateList						ProtocolIE-ID ::= 553

id-ZoAInformation									ProtocolIE-ID ::= 554

id-ResponseTime										ProtocolIE-ID ::= 555

id-ARPLocationInfo									ProtocolIE-ID ::= 556

id-ARP-ID											ProtocolIE-ID ::= 557

id-MultipleULAoA									ProtocolIE-ID ::= 558

id-UL-SRS-RSRPP										ProtocolIE-ID ::= 559

id-SRSResourcetype									ProtocolIE-ID ::= 560

id-ExtendedAdditionalPathList						ProtocolIE-ID ::= 561

id-LoS-NLoSInformation								ProtocolIE-ID ::= 562

id-NumberOfTRPRxTEG									ProtocolIE-ID ::= 564

id-NumberOfTRPRxTxTEG								ProtocolIE-ID ::= 565

id-TRPTxTEGAssociation								ProtocolIE-ID ::= 566

id-TRPTEGInformation								ProtocolIE-ID ::= 567

id-TRPRx-TEGInformation								ProtocolIE-ID ::= 568

id-TRP-PRS-Info-List								ProtocolIE-ID ::= 569

id-PRS-Measurement-Info-List						ProtocolIE-ID ::= 570

id-PRSConfigRequestType								ProtocolIE-ID ::= 571

id-MeasurementTimeOccasion							ProtocolIE-ID ::= 573

id-MeasurementCharacteristicsRequestIndicator		ProtocolIE-ID ::= 574

id-UEReportingInformation							ProtocolIE-ID ::= 575

id-PosConextRevIndication							ProtocolIE-ID ::= 576

id-TRPBeamAntennaInformation						ProtocolIE-ID ::= 577

id-NRRedCapUEIndication 							ProtocolIE-ID ::= 578

id-Redcap-Bcast-Information							ProtocolIE-ID ::= 579

id-RANUEPagingDRX									ProtocolIE-ID ::= 580

id-CNUEPagingDRX									ProtocolIE-ID ::= 581

id-NRPagingeDRXInformation							ProtocolIE-ID ::= 582

id-NRPagingeDRXInformationforRRCINACTIVE			ProtocolIE-ID ::= 583

id-NR-TADV											ProtocolIE-ID ::= 584

id-QoEInformation									ProtocolIE-ID ::= 585

id-CG-SDTQueryIndication							ProtocolIE-ID ::= 586

id-SDT-MAC-PHY-CG-Config							ProtocolIE-ID ::= 587

id-CG-SDTKeptIndicator								ProtocolIE-ID ::= 588

id-CG-SDTindicatorSetup								ProtocolIE-ID ::= 589

id-CG-SDTindicatorMod								ProtocolIE-ID ::= 590

id-CG-SDTSessionInfoOld								ProtocolIE-ID ::= 591

id-SDTInformation									ProtocolIE-ID ::= 592

id-SDTRLCBearerConfiguration						ProtocolIE-ID ::= 593

id-FiveG-ProSeAuthorized							ProtocolIE-ID ::= 594

id-FiveG-ProSeUEPC5AggregateMaximumBitrate			ProtocolIE-ID ::= 595

id-FiveG-ProSePC5LinkAMBR	 						ProtocolIE-ID ::= 596

id-SRBMappingInfo									ProtocolIE-ID ::= 597

id-DRBMappingInfo									ProtocolIE-ID ::= 598

id-UuRLCChannelToBeSetupList						ProtocolIE-ID ::= 599

id-UuRLCChannelToBeModifiedList						ProtocolIE-ID ::= 600

id-UuRLCChannelToBeReleasedList						ProtocolIE-ID ::= 601

id-UuRLCChannelSetupList							ProtocolIE-ID ::= 602

id-UuRLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 603

id-UuRLCChannelModifiedList							ProtocolIE-ID ::= 604

id-UuRLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 605

id-UuRLCChannelRequiredToBeModifiedList				ProtocolIE-ID ::= 606

id-UuRLCChannelRequiredToBeReleasedList				ProtocolIE-ID ::= 607

id-PC5RLCChannelToBeSetupList						ProtocolIE-ID ::= 608

id-PC5RLCChannelToBeModifiedList					ProtocolIE-ID ::= 609

id-PC5RLCChannelToBeReleasedList					ProtocolIE-ID ::= 610

id-PC5RLCChannelSetupList							ProtocolIE-ID ::= 611

id-PC5RLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 612

id-PC5RLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 613

id-PC5RLCChannelRequiredToBeModifiedList			ProtocolIE-ID ::= 614

id-PC5RLCChannelRequiredToBeReleasedList			ProtocolIE-ID ::= 615

id-PC5RLCChannelModifiedList						ProtocolIE-ID ::= 616

id-SidelinkRelayConfiguration						ProtocolIE-ID ::= 617

id-UpdatedRemoteUELocalID							ProtocolIE-ID ::= 618

id-PathSwitchConfiguration							ProtocolIE-ID ::= 619

id-PagingCause										ProtocolIE-ID ::= 620

id-MUSIM-GapConfig									ProtocolIE-ID ::= 621

id-PEIPSAssistanceInfo								ProtocolIE-ID ::= 622

id-UEPagingCapability								ProtocolIE-ID ::= 623

id-LastUsedCellIndication							ProtocolIE-ID ::= 624

id-SIB17-message									ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-SIB20-message									ProtocolIE-ID ::= 627

id-UE-MulticastMRBs-ToBeReleased-List				ProtocolIE-ID ::= 628

id-UE-MulticastMRBs-ToBeReleased-Item				ProtocolIE-ID ::= 629

id-UE-MulticastMRBs-ToBeSetup-List					ProtocolIE-ID ::= 630

id-UE-MulticastMRBs-ToBeSetup-Item					ProtocolIE-ID ::= 631

id-MulticastMBSSessionSetupList						ProtocolIE-ID ::= 632

id-MulticastMBSSessionRemoveList					ProtocolIE-ID ::= 633

id-PosMeasurementAmount								ProtocolIE-ID ::= 634

id-SDT-Termination-Request							ProtocolIE-ID ::= 635

id-pathPower										ProtocolIE-ID ::= 636

id-DU-RX-MT-RX-Extend								ProtocolIE-ID ::= 637

id-DU-TX-MT-TX-Extend								ProtocolIE-ID ::= 638

id-DU-RX-MT-TX-Extend								ProtocolIE-ID ::= 639

id-DU-TX-MT-RX-Extend								ProtocolIE-ID ::= 640

id-BAP-Header-Rewriting-Removed-List				ProtocolIE-ID ::= 641

id-BAP-Header-Rewriting-Removed-List-Item			ProtocolIE-ID ::= 642

id-SLDRXCycleList									ProtocolIE-ID ::= 643

id-TAINSAGSupportList								ProtocolIE-ID ::= 644

id-SL-RLC-ChannelToAddModList						ProtocolIE-ID ::= 645

id-BroadcastAreaScope								ProtocolIE-ID ::= 646

id-ManagementBasedMDTPLMNModificationList 			ProtocolIE-ID ::= 647

id-SIB15-message									ProtocolIE-ID ::= 648

id-ActivationRequestType							ProtocolIE-ID ::= 649

id-PosMeasGapPreConfigList							ProtocolIE-ID ::= 650

id-InterFrequencyConfig-NoGap						ProtocolIE-ID ::= 651

id-MBSInterestIndication							ProtocolIE-ID ::= 652

id-UE-MulticastMRBs-ConfirmedToBeModified-List		ProtocolIE-ID ::= 653

id-UE-MulticastMRBs-ConfirmedToBeModified-Item		ProtocolIE-ID ::= 654

id-UE-MulticastMRBs-RequiredToBeModified-List		ProtocolIE-ID ::= 655

id-UE-MulticastMRBs-RequiredToBeModified-Item		ProtocolIE-ID ::= 656

id-UE-MulticastMRBs-RequiredToBeReleased-List		ProtocolIE-ID ::= 657

id-UE-MulticastMRBs-RequiredToBeReleased-Item		ProtocolIE-ID ::= 658

id-L571Info											ProtocolIE-ID ::= 659

id-L1151Info										ProtocolIE-ID ::= 660

id-SCS-480											ProtocolIE-ID ::= 661

id-SCS-960											ProtocolIE-ID ::= 662

id-SRSPortIndex										ProtocolIE-ID ::= 663

id-PEISubgroupingSupportIndication					ProtocolIE-ID ::= 664

id-NeedForGapsInfoNR								ProtocolIE-ID ::= 665

id-NeedForGapNCSGInfoNR								ProtocolIE-ID ::= 666

id-NeedForGapNCSGInfoEUTRA							ProtocolIE-ID ::= 667

id-procedure-code-668-not-to-be-used				ProtocolIE-ID ::= 668

id-procedure-code-669-not-to-be-used				ProtocolIE-ID ::= 669

id-procedure-code-670-not-to-be-used				ProtocolIE-ID ::= 670

id-Source-MRB-ID									ProtocolIE-ID ::= 671

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-RedCapIndication									ProtocolIE-ID ::= 673

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList									ProtocolIE-ID ::= 707

id-DedicatedSIDeliveryIndication					ProtocolIE-ID ::= 708

id-Configured-BWP-List								ProtocolIE-ID ::= 709

id-Preconfigured-measurement-GAP-Request			ProtocolIE-ID ::= 710

id-BWP-Id											ProtocolIE-ID ::= 711

id-NetworkControlledRepeaterAuthorized				ProtocolIE-ID ::= 712

id-MT-SDT-Information								ProtocolIE-ID ::= 713

id-ExtendedResourceSymbolOffset						ProtocolIE-ID ::= 714

id-NeedForInterruptionInfoNR						ProtocolIE-ID ::= 715

id-SDT-Volume-Threshold								ProtocolIE-ID ::= 716

id-SupportedUETypeList								ProtocolIE-ID ::= 717

id-MusimCapabilityRestrictionIndication				ProtocolIE-ID ::= 718

id-duplicationIndication							ProtocolIE-ID ::= 719

id-LTMInformation-Setup								ProtocolIE-ID ::= 720

id-LTMConfigurationIDMappingList 					ProtocolIE-ID ::= 721

id-LTMInformation-Modify							ProtocolIE-ID ::= 722

id-LTMCells-ToBeReleased-List						ProtocolIE-ID ::= 723

id-LTMCells-ToBeReleased-Item						ProtocolIE-ID ::= 724

id-LTMConfiguration									ProtocolIE-ID ::= 725

id-EarlySyncInformation-Request						ProtocolIE-ID ::= 726

id-EarlySyncInformation								ProtocolIE-ID ::= 727

id-EarlySyncInformation-List						ProtocolIE-ID ::= 728

id-LTMCellSwitchInformation							ProtocolIE-ID ::= 729

id-DUtoCUTAInformation-List							ProtocolIE-ID ::= 730

id-Source-gNB-DU-ID									ProtocolIE-ID ::= 731

id-dRB-List											ProtocolIE-ID ::= 732

id-DeactivationIndication							ProtocolIE-ID ::= 733

id-RAReportIndicationList							ProtocolIE-ID ::= 734

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= 735

id-SuccessfulPSCellChangeReportInformationList		ProtocolIE-ID ::= 736

id-RadioResourceStatusNR-U							ProtocolIE-ID ::= 737

id-FiveG-ProSeLayer2Multipath						ProtocolIE-ID ::= 738

id-FiveG-ProSeLayer2UEtoUERelay						ProtocolIE-ID ::= 739

id-FiveG-ProSeLayer2UEtoUERemote					ProtocolIE-ID ::= 740

id-PathAdditionInformation							ProtocolIE-ID ::= 741

id-Recommended-SSBs-List							ProtocolIE-ID ::= 742

id-Recommended-SSBs-for-Paging-List					ProtocolIE-ID ::= 743

id-SSBs-withinTheCell-tobe-Activated-List			ProtocolIE-ID ::= 744

id-Cells-With-SSBs-Activated-List					ProtocolIE-ID ::= 745

id-Cells-Allowed-to-be-Deactivated-List				ProtocolIE-ID ::= 746

id-Cells-Allowed-to-be-Deactivated-List-Item		ProtocolIE-ID ::= 747

id-Coverage-Modification-Cause						ProtocolIE-ID ::= 748

id-RANTSSRequestType								ProtocolIE-ID ::= 749

id-RANTimingSynchronisationStatusInfo				ProtocolIE-ID ::= 750

id-TSCTrafficCharacteristicsFeedback				ProtocolIE-ID ::= 751

id-RANfeedbacktype									ProtocolIE-ID ::= 752

id-Mobile-TRP-LocationInformation					ProtocolIE-ID ::= 753

id-Mobile-IAB-MT-UE-ID								ProtocolIE-ID ::= 754

id-Target-gNB-ID									ProtocolIE-ID ::= 755

id-Target-gNB-IP-address							ProtocolIE-ID ::= 756

id-Target-SeGW-IP-address							ProtocolIE-ID ::= 757

id-Activated-Cells-Mapping-List						ProtocolIE-ID ::= 758

id-Activated-Cells-Mapping-List-Item				ProtocolIE-ID ::= 759

id-F1SetupOutcome									ProtocolIE-ID ::= 760

id-RRC-Terminating-IAB-Donor-Related-Info			ProtocolIE-ID ::= 761

id-RRC-Terminating-IAB-Donor-gNB-ID					ProtocolIE-ID ::= 762

id-NCGI-to-be-Updated-List							ProtocolIE-ID ::= 763

id-NCGI-to-be-Updated-List-Item						ProtocolIE-ID ::= 764

id-Mobile-IAB-MTUserLocationInformation				ProtocolIE-ID ::= 765

id-MobileAccessPointLocation						ProtocolIE-ID ::= 766

id-AssociatedSessionID								ProtocolIE-ID ::= 767

id-IndicationMCInactiveReception					ProtocolIE-ID ::= 768

id-MulticastCU2DURRCInfo							ProtocolIE-ID ::= 769

id-MBSMulticastSessionReceptionState				ProtocolIE-ID ::= 770

id-F1UTunnelNotEstablished							ProtocolIE-ID ::= 771

id-MulticastDU2CURRCInfo							ProtocolIE-ID ::= 772

id-SIB24-message										ProtocolIE-ID ::= 773

id-MulticastCU2DUCommonRRCInfo						ProtocolIE-ID ::= 774

id-PDUSetQoSParameters								ProtocolIE-ID ::= 775

id-N6JitterInformation								ProtocolIE-ID ::= 776

id-ECNMarkingorCongestionInformationReportingRequest	ProtocolIE-ID ::= 777

id-ECNMarkingorCongestionInformationReportingStatus	ProtocolIE-ID ::= 778

id-NRA2XServicesAuthorized							ProtocolIE-ID ::= 779

id-LTEA2XServicesAuthorized							ProtocolIE-ID ::= 780

id-NRUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 781

id-LTEUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 782

id-NReRedCapUEIndication 							ProtocolIE-ID ::= 783

id-ERedcap-Bcast-Information 						ProtocolIE-ID ::= 784

id-NRPaginglongeDRXInformationforRRCINACTIVE		ProtocolIE-ID ::= 785

id-SCPAC-Request									ProtocolIE-ID ::= 786

id-Target-F1-Terminating-Donor-gNB-ID				ProtocolIE-ID ::= 787

id-MobileIAB-Barred									ProtocolIE-ID ::= 788

id-Broadcast-MRBs-Transport-Request-List 			ProtocolIE-ID ::= 789

id-Broadcast-MRBs-Transport-Request-Item			ProtocolIE-ID ::= 790

id-S-CPACLowerLayerReferenceConfigRequest			ProtocolIE-ID ::= 791

id-S-CPAC-Configuration								ProtocolIE-ID ::= 792

id-MusimCandidateBandList							ProtocolIE-ID ::= 793

id-DLLBTFailureInformationRequest					ProtocolIE-ID ::= 794

id-DLLBTFailureInformationList						ProtocolIE-ID ::= 795

id-PSIbasedSDUdiscardUL								ProtocolIE-ID ::= 796

id-SIB22-message 									ProtocolIE-ID ::= 797

id-CUtoDUTAInformation-List							ProtocolIE-ID ::= 798

id-U2URLCChannelQoS								ProtocolIE-ID ::= 799

id-SL-PHY-MAC-RLC-ConfigExt							ProtocolIE-ID ::= 800

id-SLPositioning-Ranging-Service-Info				ProtocolIE-ID ::= 801

id-TimeWindowInformation-SRS-List					ProtocolIE-ID ::= 802

id-TimeWindowInformation-Measurement-List			ProtocolIE-ID ::= 803

id-UL-RSCP											ProtocolIE-ID ::= 804

id-BW-Aggregation-Request-Indication				ProtocolIE-ID ::= 805

id-ReportingGranularitykminus1						ProtocolIE-ID ::= 806

id-ReportingGranularitykminus2						ProtocolIE-ID ::= 807

id-ReportingGranularitykminus1additionalpath		ProtocolIE-ID ::= 808

id-ReportingGranularitykminus2additionalpath		ProtocolIE-ID ::= 809

id-TimingReportingGranularityFactorExtended			ProtocolIE-ID ::= 810

id-SRSPosRRCInactiveValidityAreaConfig				ProtocolIE-ID ::= 811

id-PosValidityAreaCellList							ProtocolIE-ID ::= 812

id-SRSReservationType							ProtocolIE-ID ::= 813

id-SymbolIndex										ProtocolIE-ID ::= 814

id-PRSBandwidthAggregationRequestIndication			ProtocolIE-ID ::= 815

id-AggregatedPosSRSResourceIDList					ProtocolIE-ID ::= 816

id-AggregatedPRSResourceSetList						ProtocolIE-ID ::= 817

id-PhaseQuality										ProtocolIE-ID ::= 818

id-MeasuredFrequencyHops							ProtocolIE-ID ::= 819

id-TxHoppingConfiguration							ProtocolIE-ID ::= 820

id-ReportingGranularitykminus3						ProtocolIE-ID ::= 821

id-ReportingGranularitykminus4						ProtocolIE-ID ::= 822

id-ReportingGranularitykminus5						ProtocolIE-ID ::= 823

id-ReportingGranularitykminus6						ProtocolIE-ID ::= 824

id-ReportingGranularitykminus3additionalpath		ProtocolIE-ID ::= 825

id-ReportingGranularitykminus4additionalpath		ProtocolIE-ID ::= 826

id-ReportingGranularitykminus5additionalpath		ProtocolIE-ID ::= 827

id-ReportingGranularitykminus6additionalpath		ProtocolIE-ID ::= 828

id-AggregatedPosSRSResourceSetList					ProtocolIE-ID ::= 829

id-RequestedSRSPreconfigurationCharacteristics-List	ProtocolIE-ID ::= 830

id-SRSPreconfiguration-List							ProtocolIE-ID ::= 831

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-DynamicResourceAllocationRequest					ProtocolIE-ID ::= xx1

id-DynamicResourceAllocationResponse				ProtocolIE-ID ::= xx2

END

-- ASN1STOP

### 9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

F1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

Presence,

PrivateIE-ID,

ProtocolExtensionID,

ProtocolIE-ID

FROM F1AP-CommonDataTypes

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs

FROM F1AP-Constants;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

F1AP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 					UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

F1AP-PROTOCOL-IES-PAIR ::= CLASS {

&id					ProtocolIE-ID 				UNIQUE,

&firstCriticality	Criticality,

&FirstValue,

&secondCriticality	Criticality,

&SecondValue,

&presence			Presence

}

WITH SYNTAX {

ID				&id

FIRST CRITICALITY		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

F1AP-PROTOCOL-EXTENSION ::= CLASS {

&id				ProtocolExtensionID			UNIQUE,

&criticality	Criticality,

&Extension,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

EXTENSION		&Extension

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

F1AP-PRIVATE-IES ::= CLASS {

&id				PrivateIE-ID,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {F1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer {F1AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {F1AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				F1AP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		F1AP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			F1AP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {F1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {F1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					F1AP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	F1AP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			F1AP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	F1AP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			F1AP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {F1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {F1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					F1AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			F1AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		F1AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {F1AP-PRIVATE-IES : IEsSetParam } ::=

SEQUENCE (SIZE (1.. maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {F1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id					F1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality			F1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value				F1AP-PRIVATE-IES.&Value				({IEsSetParam}{@id})

}

END

-- ASN1STOP

<<<<<< END OF CHANGE >>>>