3GPP TSG-RAN WG3 #124                                                 R3-24xxxx

Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	11.2

Source:	CATT

Title:	TP to TR 38.743 on support of AI/ML enabled slicing

Document for:	Discussion

Introduction

The contribution provides TP toward TR 38.473 based on the following conclusion made during offline discussion:

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

TP for TR38.743

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Finer granularity UE performance feedback for those UEs handed over from the source gNB.FFS on the exact granularity of UE performance feedback.

#### 4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhanced existing procedure to collect predicted information between gNBs:

  - Predicted radio resource status per slice

  - Predicted slice available capacity

  - Finer granularity UE performance feedback. FFS on the exact granularity of UE performance feedback.