3GPP TSG-RAN WG3 Meeting #124	R3-243862

Fukuoka, Japan, 20-24 May, 2024

Agenda Item:	12.2

Source:	ZTE

Title:	(TP to TR 38.799) on other issues

Document for:	other

1	Introduction

This contribution is to provide TP to TR 38.799 on miscellaneous issues according to the following CB:

CB: # WAB

- Resolve the FFS captured above

- Converge on the TPs below, where agreements taken above will be captured. If any more agreements are taken, they can be included in the TPs below:

  - TP for Architecture (Nokia)

  - TP for Integration procedure (Huawei)

  - TP for Authorization (CATT)

  - TP for Mobility (Ericsson)

  - TP for miscellaneous issues (ZTE)

    - WAB configuration

    - Etc

- SA2 reply LS (Qualcomm)

(Moderator – Docomo)

Summary of offline disc in R3-243844

Annex. TP for TR 38.799 V0.0.1

Start of Change

4.3	Operational aspects

Editor Note:

- Impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations).

- Inter-gNB- and gNB-to-CN signalling to address the support of WAB.

### 4.3.x IP address allocation for WAB-node

A WAB-MT may obtain IP address(es) as a normal UE. The WAB-MT may deliver the allocated IP address(es) to the co-located WAB-gNB, which is used by the WAB-gNB for traffic exchange via the backhaul.

Alternatively, the WAB-gNB may obtain dedicated IP address(es) from operator. In this case, separate IP addresses are used by the WAB-gNB and co-located WAB-MT. A tunnel (e.g. based on IPsec or L2TP) could be established to transfer the WAB traffic by implementation. If a tunnel is established, a gateway may be deployed to terminate the tunnel.

### 4.3.y	TAC/RANAC (re-)configuration for WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be same as or different than the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcasted by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

- 4.3.z Resource multiplexing

Resource multiplexing for WAB node can be supported for the in-band deployment scenario. Resource multiplexing for WAB node mechanism considers the R16/17 IAB resource multiplexing mechanism as baseline. In order to achieve the resource mltiplexing, the BH gNB needs to be aware of the co-location of a WAB-MT and WAB-gNB.

End of Change